3GPP TSG RAN WG1 #106-e		R1-2108330

e-Meeting, August 16th – 27th, 2021

Title: 	Discussion on RAN2 LS on resource reselection trigger sl-reselectAfter

Source: 	ZTE, Sanechips

Agenda item:	5

Document for:	Discussion and decision

Introduction

In R1-2106406(R2-2106625), RAN2 raised two questions on LS on resource reselection trigger sl-reselectAfter. In this contribution, we share our views on these questions.

Discussions

resource reselection trigger

In NR V2X, resource reselection trigger sl-reselectAfter is specified by RAN2 which is similar to LTE V2X for mode 2 UE.

As cited by RAN2, in NR V2X, when HARQ feedback is enabled, and an ACK has been received, some of the resources for retransmission may not be used. To better interpret the relationship between these unused resources and the consecutive unused transmission opportunities, the following questions are provided in RAN2 incoming LS[1]:

Question 1: In NR V2X, whether “unused retransmission opportunities in case of HARQ feedback is enabled” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection? Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

In RAN1’s perspective, resource reselection trigger condition is used for a grant, which includes all of the selected resources of the grant. Thus, the resource reselection trigger sl-reselectAfter should be treated per grant, i.e. only in case of all of the resources of a grant are not used, the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant should be increased by 1. When the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant is equal to sl-ReselectAfter, the Tx resource (re-)selection should be triggered.

The resource reselection trigger “sl-reselectAfter” should be treated per grant, only in case of all of the resources of a grant are not used, the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant should be increased by 1.

In some cases, the resource for initial transmission is used, but the retransmission opportunities are not used due to HARQ feedback. There may be doubts about the reliability of those unused retransmission resources. In those cases, those unused retransmission resources could be re-sensed by pre-emption or re-evaluation.

There is no critial issue for the case that the unused retransmission opportunities in case of HARQ feedback is enabled are not counted towards “consecutive unused transmission opportunities” to trigger resource reselection.

Based on above analysis, we provide the following proposal to reply the LS:

For Q1, “unused retransmission opportunities in case of HARQ feedback is enabled” shall NOT be counted towards “consecutive unused transmission opportunities” to trigger resource reselection., only in case of all of the resources of a grant are not used, the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant should be increased by 1.

- For Q2, only if all of the resources of grant, including already and not yet indicated in SCI are unused, it shall be counted one time of “consecutive unused transmission opportunities” to trigger resource reselection.

Conclusion

This contribution concludes with the following proposals:

Proposal 1:	The resource reselection trigger “sl-reselectAfter” should be treated per grant, only in case of all of the resources of a grant are not used, the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant should be increased by 1.

Proposal 2:	For Q1, “unused retransmission opportunities in case of HARQ feedback is enabled” shall NOT be counted towards “consecutive unused transmission opportunities” to trigger resource reselection., only in case of all of the resources of a grant are not used, the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant should be increased by 1.

For Q2, only if all of the resources of grant, including already and not yet indicated in SCI are unused, it shall be counted one time of “consecutive unused transmission opportunities” to trigger resource reselection.

References

- R1-2106406(R2-2106625), LS on resource reselection trigger sl-reselectAfter, RAN2.