3GPP TSG RAN WG1 Meeting #106-e	R1- 2108358

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 3)

Document for:	Discussion and Decision

# Introduction

# Summary of CSI enhancement for MTRP

Proposal 13: For CSI measurement associated with a CSI-ReportingConfig for NC-JT, support following Alt:

- Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMRs configured within CMR pairs for NCJT measurement hypothesis

Supporting companies (16): Motorola, Futurewei, QC, Spreadtrum, MediaTek, LGE, Samsung, Intel, CATT, Ericsson, ZTE, Oppo, NEC, MTK, NTT DoComo, Fraunhofer

Alt 3 (Variation 1): For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis per CMR pair using CMRs configured for NCJT measurement hypothesis

Supporting companies: CMCC

Alt 3 (Variation 2): For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMRs configured within CMR pairs for NCJT measurement hypothesis

- For FR2, M = Ks, if CMR sharing is enabled, M=Ks-2N if CMR sharing is disabled,

- For FR1, CMR sharing is enabled by default and gNB configures the value of M, with 0<=M<=Ks

Supporting companies: Nokia

Alt 4: CMR sharing between single-TRP measurement hypothesis and NCJT measurement hypothesis is realized by configuring the same value of CMR ID for single-TRP CMR and NCJT CMR pair.

Supporting companies: Vivo

Proposal 25: To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis:

- Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214.

- Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214.

- Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

# Summary of CSI enhancement for FDD

Proposal 4: For Rel-17 PS codebook, following values of R are supported:

- R = 1 and

- At most one value from {2, D* NPRBSB}

  - FFS: which one is to be decided in RAN1 106bis if support, and applicable conditions, e.g. only when Mv=2

  - D is the density of CSI-RS in frequency domain and NPRBSB is the CQI subband size in PRBs

  - Note that this R is optional

Proposal 5-1: For Rel-17 PS codebook

- Wf  OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3

- Support pmiReportingFormat = WB [if N3=1]

- FFS: the case when no SB size is configured (from RAN1#105-e agreement)

Remove the 2nd sub-bullet (10): Motorola, Fraunhofer, LG, Ericsson, QC, Sony, Intel, Nokia, Vivo, NTT DoCoMo,

Keep the 2nd sub-bullet: Samsung, CATT, ZTE

Proposal X (Updated agreement)

Following working assumption is confirmed:

- At least for rank 1 and 2, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1.

- FFS other restrictions, e.g. value(s) of N, if the value of N3 is small

- FFS other restrictions, e.g. when the number of CSI-RS ports is small

# Appendix

Agreement

Following working assumption is confirmed:

- At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1.

- FFS other restrictions, e.g. value(s) of N, if the value of N3 is small

- FFS other restrictions, e.g. when the number of CSI-RS ports is small

Conclusion

For Rel-17 PS codebook, there is no consensus on the support of Mv>2 for Wf.

Agreement

For Rel-17 PS codebook, the reserved state for reference amplitude is to be reserved as Rel-16 PS codebook.

Agreement

For Rel-17 PS codebook, support reporting of the position, [il*, fl*], of the strongest coefficient (SCI) of layer l, using ceil(log2(K1*Mv)) bits

Agreement

For Rel-17 PS codebook, support layer-common port selection for rank 2.

Agreement

Support parameter combinations represented by (alpha, Mv, beta) with K1 = alpha*P for Rel-17 PS codebook

- The candidate values of alpha are {1/2, 3/4, 1}

- Note that exact parameter combination will be discussed from RAN1 106bis:

  - based on trade-off among UPT performance, feedback overhead, and complexity

  - based on all supported ranks

  - Limit total number of parameter combinations comparable to Rel-16 eType II

- Mv={1, 2} and beta = {[1/4], 1/2, 3/4, 1} are from previous agreements

Agreement

For Rel-17 PS codebook with Rank 2, support layer-specific bitmap for indicating non-zero coefficient selection of W2.

Agreement

Support rank 3 and 4 for Rel-17 PS codebook with following:

- Supporting ranks 3 and 4 is optional with separate UE capability (same as Rel-16 PS codebook)

- The maximal CSI overhead of rank 3 and 4 is comparable to rank 2

  - FFS: use a smaller K1 (or alpha) or beta for ranks 3 and 4, or limit the maximum number of non-zero coefficients across all layers to 2K0 and per layer to K0 with the same beta

- FFS: limit Mv=1 for ranks 3 and 4 PMI

Conclusion:

Default value of Ks, max can be discussed later with Rel-17 MIMO UE capability.

Agreement

For CSI measurement associated to a NCJT measurement hypothesis in Rel-17, the maximal number of total transmission layers is up to 4 layers.

Agreement

For the UE configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis (i.e. Option 1), the bitwidth associated to X+1 CRI(s) are given as following:

- Ceil(log2(N)) for X=0

- Ceil(log2(N)) in CSI associated with NCJT measurement hypothesis and Ceil(log2(M1+M2)) in CSI associated with Single-TRP measurement hypothesis for X=1

- Ceil(log2(N))  in CSI associated with NCJT measurement hypothesis and Ceil(log2(M1))  and  Ceil(log2(M2)) in CSI associated with Single-TRP measurement hypothesis for X=2

- Note that M1 (M1<=K1) and M2 (M2<=K2) is the number of CMRs configured for Single-TRP measurement hypothesis in the first and second CMR groups respectively in a CMR measurement set.

Agreement

For the UE be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses (i.e. Option 2),

- Alt 1: the first M1+M2 codepoints of CRI corresponds to M1+M2 CMRs for Single-TRP measurement hypothesis and the second N codepoints corresponds to N CMR pairs for NC-JT measurement hypothesis.

- Note that M1M1 (M1<=K1) and M2 (M2<=K2) is the number of CMRs configured for Single-TRP measurement hypothesis in the first and second CMR groups respectively in a CMR measurement set.

Agreement

For CSI measurement associated with a CSI-ReportingConfig for NC-JT, study following restriction(s) for two CMRs within the same CMR pair configured for NCJT measurement hypothesis:

- FFS: two resources are restricted within the same DL slot

- FFS: two resources are restricted with the same CDRX active time

Agreement

For a CMR pair configured for a NCJT measurement hypothesis, study following Alternatives:

- Alt 1: a separate powerControlOffset (Pc ratio) shall be configured for the NCJT measurement hypothesis by re-defining such Pc ratio as 10log10(P_PDSCH/P_CSIRS) dB, whereas

  - P_PDSCH is the energy of PDSCH ports with a same TCI state as the CMR on one subcarrier of one OFDM symbol

  - P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol

- Alt 2: re-interpret two Pc ratios configured for the CMR pair for the NCJT measurement hypothesis, FFS detailed impact of specification

- Alt 3: No change to definition or configuration of Pc ratio

- Note that other solutions are not excluded.

Agreement

For CSI computation delay requirement associated with a CSI-ReportingConfig for a NCJT measurement hypothesis, study following alternatives:

- Alt1: introducing new/relaxed values on Z and Z’, FFS exact values or other conditions

- Alt2: No changes of values on Z and Z’

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT measurement hypothesis, study whether to support non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" in Rel-17

- Related details, if needed, are to be discussed in RAN1#106bis.

- Interested companies are encouraged to share details and related specification impact if support

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction by selecting at most one alternative from the following in RAN1#106bis-e:

- Alt 1: One RI restriction is configured per CodebookConfig, whereas the RI restriction is applied to both Single-TRP and NCJT measurement hypotheses.

  - If rank restriction of X is configured, reported rank is X for a Single-TRP measurement hypothesis and sum of two reported ranks is X for a Multi-TRP measurement hypothesis.

- Alt 2: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to one CMR group in a CMR resource set respectively, i.e. per TRP.

  - If rank restriction of (X, Y) is configured, reported rank is X for the CMR in the first CMR group and Y for the CMR in the second CMR group, regardless single-TRP and NCJT measurement hypotheses.

- Alt 3: Multiple RI restrictions can be configured per CodebookConfig, whereas RI restriction is applied to per each CMR in CMR pair for NCJT and per each CMR for Single-TRP.

- Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses.

  - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses.

- Alt 5: Three RI restrictions can be configured per CodebookConfig, whereas two RI restrictions are applied to two CMR groups in a CMR resource set respectively for Single-TRP measurement hypothesis, and the third one is applied to all NCJT measurement hypotheses.

  - If rank restriction of (X1, X2, Y) is configured, reported rank is X1, X2 for each CMR group respectively for single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses.

- Alt 6: Switch between Alt 4 and Alt 5 where gNB can configure via RRC signaling which alternative to use

Note that if none of above Alternatives is agreed in Rel-17, RI restriction is only applied for Single-TRP measurement hypotheses and no RI restriction is applied for Multi-TRP measurement hypotheses.

Agreement

At least for rank 1/2 and Mv > 1, for relationship between N and Mv, support following alternative

- Alt 2-1: N >= Mv, Wf is layer-common and reported by UE for N>Mv.

  - For Mv=2, N=2 and one value from {3, 4, 5}

    - RAN1 to select one value from {3, 4, 5} in RAN1#106bis-e

  - FFS: how to report Wf in terms of reporting mechanism and associated bits when Mv=2 and N=one value from {3, 4, 5}

Note: Wf is layer-common for N=Mv

Note: For all alternatives, a layer-common window/set of size N is configured.

Agreement

If a bitmap for indicating non-zero coefficients can be absent, down-select one Alt from the following for Rel-17 PS codebook:

- Alt 1: At least for rank 1 PMI, the bitmap of indicating non-zero coefficients is not needed if Mv=1 and Beta=1.

  - FFS the need for Mv>1 and/or Beta<1

- Alt 2: For rank 1 /2 PMI, the bitmap(s) of indicating non-zero coefficients for corresponding layer(s) is absent if reported KNZ=K1*Mv*rank

  - Where KNZ is the number of non-zero coefficients

- Alt 3: In addition to Alt 2, additional field is reported by UE to inform whether the bitmap of indicating non-zero coefficients for specific layer is absent if rank>1.

- Alt 4: The bitmap of indicating non-zero coefficients is not needed if the number of coefficients is sufficiently small, i.e. K1Mv ≤ δ

Note: If none of above Alternative is agreed in RAN1#106bis-e, the bitmap for indicating non-zero coefficient is always present by default.