3GPP TSG RAN WG1 #106-e	R1-2108571

e-Meeting, August 16th – 27th, 2021

Source:	moderator (vivo)

Title:	Feature lead summary#3 on Enhancements on Multi-TRP inter-cell operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Introduction

In this contribution, contributions submitted in AI 8.1.2.2 are summarized. In section 2, the points raised in the contributions are listed.

Updated   Possible Agreement

- Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs  per CC is denoted X and can be reported as a UE capability

  - For the report value of X, multiple candidate values including 1 is supported.

    - FFS : Which values to support other than 1. [If RAN1 cannot converge on other values, only X=1 will be supported.]

    - Values larger than 7 are precluded

    - RAN1 needs to agree on value(s) of X other than 1

  - Down-select one of the following alternatives:

    - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity

    - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB

  - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States

Proposal3-2:

- For inter-cell mTRP, one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex

  - FFS: Inter-cell + intra-cell mTRP

  - FFS : The association between PCI and CORESETPoolIndex when F f ast switching between intra-cell mTRP and inter-cell mTRP , where one PCI associated with activated TCI states can be associated with more than one CORESETPoolIndex and one CORESETPoolIndex can be associated with only one PCI associated with activated TCI states

Updated Proposal 7-3

- For a CSI-RS QCLed with a neighboring cell SSB , the CSI-RS EPRE transmit power is calculated based on powerControlOffsetSS and the SSB transmission power in the neighboring cell information.

And, I would also like to start discussion LS for RAN2.

LS to RAN2

1. Overall Description:

For inter-cell multi TRP operation in Rel-17 FeMIMO, RAN1 has reached following agreements in past few meetings:

Agreement from RAN1#103-e

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

Agreements from RAN1#104-e

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

       FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

       SSB time domain position

       SSB transmission periodicity

       SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreements from RAN1#104b-e

Agreement

       For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

o   The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

o   Applicable at least for non-cross carrier QCL indication

  FFS: Cross carrier scheduling QCL indication

       RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

       Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view

Agreements from RAN1#106-e

Agreement

Introduce a new RRC indicator/signaling (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with, where the new indicator/signaling is not the exact PCI value

       Detailed signaling design is up to RAN2

[outcome of above possible agreements to be added]

RAN1 is aware of the terminology “non-serving cell” is not proper as mentioned in respective LS in R2-2106787, the terminology “non-serving cell”  means  “cell having TRP with different PCI”.

2. Actions:

To RAN2

ACTION:

RAN1 would like to respectfully request RAN2 to take the above agreements into account while designing necessary signaling.

| Company | comments |
| --- | --- |
| Apple | Regarding the FFS point of proposal 3.2, I suggest the following wording to avoid debating too much on details for a FFS. I guess the switching can be based on RRC or MAC CE, and we can discuss the details later. BTW, the bracket around “PDSCH” may need to be removed as we suggested before. Thank you. Proposal3-2: - For inter-cell mTRP , one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS: details on switching between intra-cell mTRP and inter-cell mTRP - FFS : Inter-cell + intra-cell mTRP - FFS : The association between PCI and CORESETPoolIndex when F f ast switching between intra-cell mTRP and inter-cell mTRP , where one PCI associated with activated TCI states can be associated with more than one CORESETPoolIndex and one CORESETPoolIndex can be associated with only one PCI associated with activated TCI states |
| Lenovo | We share the similar view with Dalin on the inter-cell mTRP and intra-cell in Jialing’s scenario, i.e., - inter-cell mTRP , i.e. CORESETPoolIndex 0 associated with TRP 0 of cell 0, and CORESETPoolIndex 1 associated with TRP 2 of cell 1, or - intra-cell mTRP , i.e. CORESETPoolIndex 0 associated with TRP 0 of cell 0, and CORESETPoolIndex 1 associated with TRP 1 of cell 0, or - intra-cell mTRP , i.e. CORESETPoolIndex 0 associated with TRP 2 of cell 1, and CORESETPoolIndex 1 associated with TRP 3 of cell 1. We are fine to take intra-cell mTRP as FFS in proposal 3-2. However, I can’t understand the meaning of “fast switching”, does it means the switching between inter-cell mTRP and intra-cell mTRP is done via DCI? If not we suggest to only say ‘switching between intra-cell mTRP and inter-cell mTRP’. Proposal3-2: - For inter-cell mTRP , one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS : Inter-cell + intra-cell mTRP - FFS : The association between PCI and CORESETPoolIndex when F f ast switching between intra-cell mTRP and inter-cell mTRP , where one PCI associated with activated TCI states can be associated with more than one CORESETPoolIndex and one CORESETPoolIndex can be associated with only one PCI associated with activated TCI states |
| Samsung | For the possible agreement, we incorporate the FL’s clarification in the first main bullet (in blue). Updated Possible Agreement - Rel. 17 inter-cell MTRP , the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability - For the report value of X, multiple candidate values including 1 is supported. - FFS : Which values to support other than 1. [If RAN1 cannot converge on other values, only X=1 will be supported.] - Values larger than 7 are precluded - RAN1 needs to agree on value(s) of X other than 1 - Down-select one of the following alternatives: - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB - The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States |
| ZTE | Regarding proposal 3.2, although the part of FFS is deviated from the above goal, we can live with it in principle. For the sake of progress, we suggest: Proposal3-2: - For inter-cell mTRP, one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS: Inter-cell + intra-cell mTRP - FFS: The association between PCI and CORESETPoolIndex when Ffast switching between intra-cell mTRP and inter-cell mTRP, where one PCI associated with activated TCI states can be associated with more than one CORESETPoolIndex and one CORESETPoolIndex can be associated with only one PCI associated with activated TCI states |
| LG | In the following example, you mentioned, are you saying that the new RRC indicator will be used in the same way as CORESET pool index in order to support R16 MDCI features (such as out of order, or separate A/N) between cell 0 and cell 1? If yes, we are not sure it is in the scope of this AI because it is about extension of MDCI based MTRP operation up to 4 TRPs. |
| Futurewei | - Cell 0: TRP 0 and TRP 1 use CORESETPoolIndexes 0 and 1, respectively - Cell 1: TRP 2 and TRP 3 use CORESETPoolIndexes 0 and 1, respectively - Cell 0 and Cell 1 are differentiated by the new RRC indicator/signaling To allow this, ‘and one CORESETPoolIndex can be associated with only one PCI associated with activated TCI states’ will need to be removed for now but can be discussed later. Would this be acceptable? Proposal3-2: - For inter-cell mTRP, one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS: Inter-cell + intra-cell mTRP - FFS: Fast switching between intra-cell mTRP and inter-cell mTRP, where one PCI associated with activated TCI states can be associated with more than one CORESETPoolIndex |
| Samsung | - Cell 0: TRP 0 and TRP 1 use CORESETPoolIndexes 0 and 1, respectively - Cell 1: TRP 2 and TRP 3 use CORESETPoolIndexes 2 and 3, respectively the intra-cell MTRP and inter-cell MTRP in our mind is: at the same time, the UE either supports - inter-cell mTRP, i.e. CORESETPoolIndex 0 associated with TRP 0 of cell 0, and CORESETPoolIndex 1 associated with TRP 2 of cell 1, or - intra-cell mTRP, i.e. CORESETPoolIndex 0 associated with TRP 0 of cell 0, and CORESETPoolIndex 1 associated with TRP 1 of cell 0, or - intra-cell mTRP, i.e. CORESETPoolIndex 0 associated with TRP 2 of cell 1, and CORESETPoolIndex 1 associated with TRP 3 of cell 1. We are fine to keep both interpretations open in the FFS. We would like to ask for the following question again to get a better understanding. Regarding the Updated Possible Agreement, based on the FL’s explanations, RAN1 needs to have convergence on the other value(s) than 1, which will affect the UE cap reporting design and also the network flexibility. Is this a common/correct understanding? Regarding proposal 3-2, we are generally fine with Xi’s version, and support studying the switching. To avoid misinterpretation, we suggest the following addition to the FFS, which is basically Alt.2 in the previous FL’s proposal. Proposal3-2: - For inter-cell mTRP, one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS: Inter-cell + intra-cell mTRP - FFS: Fast switching between intra-cell mTRP and inter-cell mTRP, where one PCI associated with activated TCI states can be associated with more than one CORESETPoolIndex and one CORESETPoolIndex can be associated with only one PCI associated with activated TCI states |
| ZTE | Regarding proposal 3.2, although we could live with the FFS part as Xi suggested, we prefer to remove it now and further discuss fast switching between intra/inter-cell MTRP in Rel-18 & beyond (if any), esp. considering that there is little remaining budget in Rel-17. Proposal3-2: - For inter-cell mTRP, one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS: Inter-cell + intra-cell mTRP |
| Samsung | We know that RRC reconfig. would result in larger latency. What we have commented before was that even with this latency, the RRC reconfiguration would work just fine for, e.g., two additional PCIs with hexagonal layout, and in this case, dynamic activation/deactivation would not bring extra benefit. Unless in FL’s mind, even in the aforementioned deployment scenario, fast switching between additional PCIs is still needed, e.g., less than a couple of tens of ms? We would appreciate it if the FL can clarify and elaborate. We also understand Ericsson’s deployment scenario, where dense additional PCIs are implemented and load balancing could be critical. For progress and to be clear, we are fine with the deletion/removal of the red text ([If RAN1 cannot converge on other values, only X=1 will be supported.]) from the possible agreement. We just would like to have a common/better understanding of the proposal (specifically the first bullet), e.g., whether it should be specific to deployment scenario, to avoid any confusion. |
| Huawei | Thanks for the continued discussions. As mentioned before, similar to ZTE/FutureWei, we support intra-cell + inter-cell mTRP operation. Based on the comments thus far, we understand several companies do not want to consider this case in R17. To find a middle ground here, as mentioned by Yushu, we suggest changing the FFS point as “FFS: Fast switching between intra-cell mTRP and inter-cell mTRP operation”. Hope it is acceptable. Proposal3-2: - For inter-cell mTRP, one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH is associated with another CORESETPoolIndex - FFS: Inter-cell + intra-cell mTRP - FFS: Fast switching between intra-cell mTRP and inter-cell mTRP |
| CATT | We share similar view as Xiaomi, Lenovo/Motorola Mobility and Apple that inter+intra-cell case is out of scope. Furthermore, for M-DCI based multi-TRP transmission, DMRS CDM group from different TRPs should not overlap with each other no matter the two PDSCHs are overlapped or not. If scheduling of 3 PDSCHs is supported, only DMRS configuration type 2 can be used, since we can have 3 DMRS CDM groups only for type 2 DMRS. |
| xiaomi | For proposal3-2, we prefer that the FFS part should be removed. In Rel16, for multi-TRP, the CORESETPoolIndex is used to inform the UE whether to receive multiple PDCCHs from multi-TRPs and the allowed values of CORESETPoolIndex are 0 and 1, which means that only two TRPs are support. For inter-cell multi-TRP, we think that only two TRPs, one from serving cell and one from non-serving cell, should be considered too. We are open to discuss “intra-cell + inter-cell multi-TRP”, but not in Rel17. |
| Ericsson | Sorry for repeating myself, the benefit of X>1 is less RRC signalling, less overhead, less PDCCH load, lower latency, to make the inter-cell mTRP operation faster to allow fast load balancing and fast interference management. If Samsung would like to rely on the slow RRC signaling for this in their networks, it is still possible to configure a single additional PCI, but please don’t restrict the deployment intentions with fast inter-cell management that has been pointed out as beneficial by both Ericsson and Huawei. You mention the flexibility comes “at the cost of something else”, can you elaborate what the cost is? UE chipset vendors are ok with larger values of X, at least if SSBs are aligned. |

Previous agreements

RAN1 #102-e:

Agreement

Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation.

- Details on configuration of non-serving cell RS;

- Allowed source and target RS types for RS transmitted from the non-serving cell TRP ;

- Allowed QCL types for RS transmitted from the non-serving cell TRP ;

- Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any;

- Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels;

Other details not precluded.

RAN1#103-e:

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

For future meetings

Consider rate matching behavior related to non-serving cell SSB.

RAN1#104-e:

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.

RAN1#104b-e:

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specification point of view

Agreement

For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e

- Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex

- Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex

- Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex

Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e

Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e