3GPP TSG RAN WG1 #106-e	R1-21xxxxx

e-Meeting, August 16th – 27th, 2021

Agenda item:		5.1

Source:	Moderator (China Telecom)

Title:	[106-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching

Document for:		Discussion

Introduction

In RAN #89 e-meeting, a new Rel-17 WID of “RF requirements enhancement for NR frequency range 1 (FR1)” [1] was approved and was revised in RAN #91 e-meeting [2], including following objectives.

- Specify UE requirements to enable Tx switching between different cases across carriers based on SUL and NR inter-band uplink CA for UE supporting maximum two concurrent transmissions

  - Specify UE requirements to enable Tx switching between cases

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

  - Specify UE requirements to enable Tx switching between cases, where 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL and band B is a non-SUL band

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

and

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

Note 1:  Only addressing the case of co-located and synchronized network deployment for the two UL carriers.

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA.

Note 3:  The UE is configured with two different uplink carrier frequencies.

This contribution is a summary of the following email discussion:

[106-e-NR-R17-TxSwitching-01] Email discussion on RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) – Jianchi (China Telecom)

- 1st check point: August 19

- 2nd check point: August 25

- 3rd check point: August 27

Email discussion (1st round)

2Tx-2Tx switching between two uplink carriers

- 2.1.1	Determination of the state of Tx chains for UL CA option 2

In RAN1 #105e, it was discussed how to handle the case that the state of Tx chains after Tx switching may not be unique for UL CA option 2 and the following agreement was achieved.

Agreement:

- For a UE configured with 2Tx-2Tx UL Tx switching between two uplink carriers and configured with UL CA Option 2, if the state of Tx chains after UL Tx switching is not unique, a rule to determine the state of Tx chains after Tx switching is to be specified.

  - FFS: The state of Tx chains with the most of Tx chains on the most important uplink carrier is assumed, e.g. the carrier with uplinkTxSwitchingPeriodLocation configured as false.

R1-2106500 proposed: If the state of Tx chains after UL Tx switching is not unique, the state of Tx chains with the most of Tx chains on the most important uplink carrier is assumed, e.g. the one carrier with uplinkTxSwitchingPeriodLocation configured as false. R1-2106729, R1-2107122, R1-2107211 proposed if the state of Tx chains after Tx switching is not unique, the state of Tx chains supporting 2Tx transmission on one carrier is assumed. R1-2107970 proposed if the state of Tx chains after Tx switching is not unique, 1Tx on carrier 1 and 1Tx on carrier 2 is assumed. R1-2107308 mentioned several options to handle this issue, and prefer that if the state of Tx chains after Tx switching is not unique, 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

- Prioritize one carrier in the sense that allow at least one Tx chain on that carrier and two Tx chains when possible

  - One example is to prioritize Pcell or Spcell which would be configured with UCI and other important channels.

- Prioritize one switching case

  - One example is to prioritize Case 1 which is more balanced for both carriers.

- Define rules with pre-conditions

  - One example is to define per channel rules

    - if the scheduling on target cell is PUSCH, it prefers Case 3 -> Case 2 with 0P+1P, and Case 2 -> Case 3 with 1P+0P

    - if the scheduling on target cell is PRACH/PUCCH, it prefers Case 3 -> Case 1, and Case 2 -> Case 1, as single port should be sufficient for those channels and the rest port could be on another carrier for future Tx.

Based on companies’ contributions, following proposal is proposed:

Proposal 1: Down select one of the following options:

- Option 1: If the state of Tx chains after UL Tx switching is not unique, the state of Tx chains with the most of Tx chains on the most important uplink carrier is assumed, e.g. the one carrier with uplinkTxSwitchingPeriodLocation configured as false.

  - Support: Huawei, HiSilicon

- Option 2: If the state of Tx chains after UL Tx switching is not unique, the state of Tx chains supporting 2Tx transmission on one carrier is assumed.

  - Support: ZTE, China Telecom, OPPO

- Option 3: If the state of Tx chains after UL Tx switching is not unique, 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

  - Support: Qualcomm, vivo

Companies are encouraged to provide views on the above options.

| Company | Views |
| --- | --- |
| CATT | We are fine with Option 2. |
| ZTE | We support Option2. For Option1, RAN1 may need to further discuss the definition of “most important uplink carrier”, which is time consuming since different companies may have different understandings on this issue, especially considering the different band combinations. Regarding the carrier with uplinkTxSwitchingPeriodLocation configured as false, it may also be the important carrier. For example. FDD+TDD CA, the TDD carrier is configured uplinkTxSwitchingPeriodLocation as false, but this doesn’t mean FDD carrier is less important. The reason is mainly because that TDD carrier has less UL symbols. Between Option2 and Option3, Option2 allows UE to transmit both 1-port and 2-port transmission without unnecessary UL Tx switching, which can improve the overall system performance. Thus, Option2 is preferred. |

- 2.1.2	TP for UL CA option 1

FL comments: Regarding the TP corresponding to the agreed switching mechanism for 2Tx-2Tx UL Tx switching between two uplink carriers for UL CA option 1. The latest proposal in RAN1 #105e is as follows:

Proposal 2: Adopt the following TP to TS 38.214 in principle.

- Note: whether new UE capability “BandCombination-UplinkTxSwitch-R17” will be introduced is up to RAN2.

- FFS potential new RRC parameters.

<Unchanged parts are omitted – 38.214> - 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch [or BandCombination-UplinkTxSwitch-R17] for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-T0ffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. <Unchanged parts are omitted – 38.214>

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| ZTE | There are still some issues not clear from our perspective. The above TP assumes that Rel-16 and Rel-17 UL Tx switching share the same RRC configuration parameter uplinkTxSwitchingOption, which hasn’t been explicitly discussed yet. We prefer to explicitly discuss this issue or add a tentative RRC parameter in the TP for now. Also, the newly added bullet in the above TP is only for Rel-17 UL Tx switching, but not for Rel-16. We prefer to make this clear. Thus, the following is proposed from our perspective. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL' or configured with [RRC_R17_CA Option1_2carrier], when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with [RRC_R17_CA Option1_2carrier] or [RRC_R17_CA Option2_2carrier], when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. |

- 2.1.3	TP for UL CA option 2 (on hold)

R1-2106729 proposed TP for UL CA option 2.

FL comments: It is still under discussion how to handle the case if the state of Tx chains after UL Tx switching is not unique for UL CA option 2. This sub-section is tentatively on hold.

Uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

In RAN1 #104b-e, the following conclusion was reached.

Conclusion:

- For uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B,

  - If the state of Tx chains is 1Tx on Band A and 1Tx on Band B, 1Tx is available simultaneously on both uplink carriers on band B for a UE.

  - If the state of Tx chains is 0Tx on Band A and 2Tx on Band B, 2Tx are available simultaneously on both uplink carriers on band B for a UE.

In RAN1 #105e, the basic principle for uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B was discussed. The latest proposal was as follows:

- For inter-band UL-CA and SUL, for Rel-17 1Tx-2Tx/2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the contiguous uplink carriers on band B should be considered as a single uplink carrier for the purpose of UL Tx switching, i.e.

  - FFS: with respect to the determination of uplink switching triggering, the presence of transmission occasion on any one uplink carrier on Band B is equivalent to the presence of transmission occasion on any other uplink carrier(s) on Band B.

  - FFS: no uplink switching is triggered if the presence of transmission occasion is on one uplink carrier on Band B and the preceding uplink transmission occasion is on other uplink carrier(s) on Band B.

  - FFS: In evaluating the antenna ports for determination of UL Tx switching, the larger ports number among the scheduling for CC2 and CC3 on band B is used.

In this meeting, R1-2106500, R1-2107308 continued to propose the basic mechanism. R1-2106729, R1-2106925, R1-2107122, R1-2107211, R1-2107388, R1-2107970 proposed the detailed switching mechanism for SUL, UL CA option 1 and option 2. R1-2106500, R1-2106729 provided TPs.

FL comments: Considering the situation of the discussion in RAN1 #105e and it seems the majority support to discuss the detailed switching mechanism, suggest to discuss the detailed switching mechanism in this meeting. The corresponding TPs can be discussed later.

Proposal 3:

- For SUL and UL CA option 1, if 1Tx-2Tx UL Tx switching or 2Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured, the switching period is only applicable when the UL transmissions are switched between band A and band B.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| ZTE | Ok with the above proposal. |

Proposal 4:

- For inter-band UL CA, if 1Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1 Tx on band A and 1Tx on band B, the next UL transmission has a 2-port transmission on at least one carrier on band B.

    - If the current state of Tx chains is 0 Tx on band A and 2Tx on band B, the next UL transmission has a 1-port transmission on the carrier on band A.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| ZTE | Ok with the above proposal. |

Proposal 5:

- For inter-band UL CA, if 2Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1Tx on band A and 1Tx on band B, the next UL transmission has a 2-port transmission on the carrier on band A or at least one carrier on band B.

    - If the current state of Tx chains is 0Tx on band A and 2Tx on band B, the next UL transmission has a 1-port or 2-port transmission on the carrier on band A.

    - If the current state of Tx chains is 2Tx on band A and 0Tx on band B, the next UL transmission has a 1-port or 2-port transmission on at least one carrier on band B.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| ZTE | Ok with the proposal. |

Operation with downgraded MIMO setting and/or CA setting

R1-2106500 proposed that if UE support UL Tx switching with two contiguous carriers on Band B, the UE can be configured and operated with one carrier on Band B as a downgraded UL Tx switching and had the following proposal.

Proposal 6: Confirm the following,

- If UE support UL Tx switching with two contiguous carriers on Band B, the UE can be configured and operated with one carrier on Band B as a downgraded UL Tx switching.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal |
| ZTE | First of all, our first preference is still to discuss this issue under UE feature discussion. If majority companies prefer to discuss it now, then we have the following detailed comments. The main bullet is not clear. We are not sure why we use “confirm” here and what’s to be confirmed. We propose to update the proposal as following. Proposal: If UE support UL Tx switching with two contiguous carriers on Band B, the UE can be configured and operated with UL Tx switching with one carrier on Band B as a downgraded UL Tx switching. |

R1-2106500 proposed the number of ports of configured SRS resources on an uplink completely determines the maximum Tx chain required on the uplink, which can also easily differentiate 2Tx-2Tx from 1Tx-2Tx, and had the following proposal.

Proposal 7:

- For a UE configured with UL Tx switching via uplinkTxSwitching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

- 2Tx-2Tx switching mode: when the maximum number is 2 for both uplinks configured with uplinkTxSwitching

- 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

- the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

  - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We are fine with FL proposal. |
| ZTE | We propose to introduce a RRC parameter to explicitly differentiate 1Tx-2Tx vs 2Tx-2Tx switching mode if necessary. The above proposal 7 may not work in the following two cases 1) if non-codebook bases transmission is configured, number of SRS ports cannot determine whether it is 1Tx or 2Tx since each SRS is always 1 port. Two 1-port SRS resources are used for two ports PUSCH transmission. 2) if different maximum number of ports are configured for the two carriers on Band B. For example, 2-ports is configured on carrier1 and carrier3, while only 1 port is configured on carrier2. Besides, UE may need to check all the SRS configurations for all the carriers. An explicit RRC signalling can avoid such cross-carrier checking. |

1-port transmission via DCI format 0_1 for UL CA option 2

This issue was intensively discussed in Rel-16. Many compromised proposals were discussed but unfortunately no consensus was reached. In RAN1 #104b-e, RAN1 #105-e, RAN1 #106-e, some companies raised this issue in Rel-17 again. Based on the discussion in RAN1 #105-e, we can focus on the following two alternatives.

Alt 1: supported by ZTE, Qualcomm

- For UL CA option 2, DCI format 0_1 can be used to schedule a UL transmission on carrier 2 when nrofSRS-Ports is configured as 2 antenna ports and state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2.

  - It’s up to implementation how DCI format 0_1 to be used.

Alt 2: supported by Huawei, HiSilicon, CATT, OPPO

- 1-port transmission via DCI format 0_1 for UL CA option 2 is not considered for Rel-17 Tx switching.

| Company | Views |
| --- | --- |
| CATT | We prefer to Alt.2 because it is out of scope in R17 WID. |
| ZTE | In Rel-17 UL Tx switching, all carriers can support 1-port and 2-port UL transmission. In case the state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2, if nrofSRS-Ports is configured as 2, only fallback DCI can be used to schedule 1-port PUSCH transmission, which is too restrictive. For this issue, Alt.1 is to allow DCI format 0_1 to schedule 1-port PUSCH in this case. We are also open to other solutions as long as it can reduce the restriction mentioned above. |

Back-to-back switching with SRS switching

R1-2107308 mentioned that in Rel-16 UL Tx switching, UE is restricted to support one switch per one slot. However, the switching location could be anywhere inside the slot. For example, if the switch is triggered by SRS transmission, the switching location could be in the middle or even later part of the slot. Therefore, if there is an expected switch on the SRS transmission carrier, there would be two switches in 14 consecutive symbols even these two switches still belong to two slots. Now, when we consider SRS carrier switching and if the UL Tx switching is triggered by SRS carrier switching which means there would be 4 switches (2 for SRS and 2 for UL Tx switch) in 14 consecutive symbols! From UE implementation perspective, we definitely want to avoid this case as too many symbols are costed as switch gap.

UL

DL

DL

Tx Switch

CC1

UL

CC2

DL

DL

CC3

RF tuning

UL

4 switches within 14 consecutive symbols

SRS

RF tuning

SRS

Tx switch

Figure: illustrative figure on 4 switches in 14 consecutive symbols

Proposal 8:

- When SRS carrier switching is configured, a maximum of 3 switches (2 for SRS and 1 for UL Tx switching) are supported in 14 consecutive symbols corresponding to the SCS of SRS.

  - Note: it is applicable to both Rel-16 UL Tx switching and Rel-17 UL Tx switching.

Companies are encouraged to provide views on the above proposal.

| Company | Views |
| --- | --- |
| CATT | We think the proponent to clarify it for Rel-17 WID in RAN plenary because we can’t find out any objective of Rel17 WID related to this proposal. This proposal is related to SRS carrier switching together with UL Tx switching in Rel-16 and first of all it is better to discussion about it in Rel-16 AI. In our understanding, gNB can schedule UE for SRS transmission based on UE capability on SRS-SwitchingTimeNR. So as shown in the above figure, UE can execute SRS transmission on CC2and CC3 and then go back to CC1. So the motivation isn’t clear to us. |

CA based SRS carrier switching

FL comments: This issue is discussed in AI 7.2.12.

Agreements at RAN1#105-e

Agreements:

- For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused.

Agreements:

- For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on.

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

Agreements:

- For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2.

    - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1.

    - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

- Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control.

Agreement:

- For a UE configured with 2Tx-2Tx UL Tx switching between two uplink carriers and configured with UL CA Option 2, if the state of Tx chains after UL Tx switching is not unique, a rule to determine the state of Tx chains after Tx switching is to be specified.

  - FFS: The state of Tx chains with the most of Tx chains on the most important uplink carrier is assumed, e.g. the carrier with uplinkTxSwitchingPeriodLocation configured as false.

Agreements at RAN1#104b-e

Agreements:

- For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Agreements:

- For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Conclusion:

- For uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B,

  - If the state of Tx chains is 1Tx on Band A and 1Tx on Band B, 1Tx is available simultaneously on both uplink carriers on band B for a UE.

  - If the state of Tx chains is 0Tx on Band A and 2Tx on Band B, 2Tx are available simultaneously on both uplink carriers on band B for a UE.

Agreement:

- Send LS to RAN4 asking following question:

  - Question: For UL Tx switching in a band pair of a band combination, whether or not the switching time reported by a UE for 2Tx-2Tx switching can be different from that reported by the UE for 1Tx-2Tx switching.

Agreement:

For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+(0P+0P) |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |

Agreement:

For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |
| Case 3 | 2T+0T | 2P+(0P+0P), 1P+(0P+0P) |

Agreement:

For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P) |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |

Agreement:

For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P) |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |
| Case 3 | 2T+0T | 2P+(0P+0P), 1P+(0P+0P) |

Conclusion:

- For uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, whether Tx switching between 2Tx on Band A and 1Tx on Band A+1Tx on Band B for UL CA option 1 and SUL is included in WID could be clarified by RAN plenary or RAN4.

References

- RP-202088, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17, Huawei, HiSilicon, China Telecom, RAN #89e, Sep. 2020.

- RP-210899, Revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN #91e, Mar. 2021.

- R4-2107847, Reply LS on Rel-17 uplink Tx switching, RAN4, China Telecom, RAN4 #99-e, 19 - 27 May, 2021

- R1-2106500, Discussions on enhancements for UL Tx switching, Huawei, HiSilicon, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2106729, Discussion on Rel-17 UL Tx switching, ZTE, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2106925, Discussion on Rel-17 enhancements for UL TX switching, CATT, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107122, Discussion on Rel-17 uplink Tx switching, China Telecom, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107211, Discussion on Rel-17 Tx Switching enhancement, OPPO, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107308, Discussion on Rel-17 UL switching, Qualcomm Incorporated, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107388, Discussion on Rel-17 UL Tx switching, CMCC, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107970, Discussion on Rel-17 Tx switching enhancements, vivo, RAN1 #106-e, August 16th – 27th, 2021.