3GPP TSG RAN WG1 #106-e   	 	  	                 R1-210xxxx

e-Meeting, August 16th – 27th, 2021

Agenda item: 7.2.6

Source: Moderator (vivo)

Title: Summary of [106-e-NR-eMIMO-04]

Document for: Discussion and Decision

# Introduction

This contribution summaries discussion of email thread [106-e-NR-eMIMO-04] MB.3 (conflict between default and updated spatial relation for multi-CC) triggered by the draft CR in [1].

There are views expressed during the preparation phase that default spatial relation is defined/configured per CC, and including a CC with a default spatial relation in a CC list is an error case.

Two cases exist for simultaneous configuration of CC_list and default beam operation.

- Case1: Simultaneous configuration of enableDefaultBeamPL-ForSRS and simultaneousTCI-UpdateList1/ simultaneousTCI-UpdateList2;

- Case2: Simultaneous configuration of enableDefaultBeamPL-ForSRS and simultaneousSpatial-UpdatedList1/simultaneousSpatial-UpdatedList

We may need to first clarify understanding on current specification before further discussion.

# Views on Case1

Please share your views for case1 in the following table.

| Questions for view sharing: - Do you think it is allowed by current specification to “simultaneously configure enableDefaultBeamPL-ForSRS and simultaneousTCI-UpdateList1/simultaneousTCI-UpdateList2” for a UE? - If answer to above is no, please clarify why this is not allowed? |  |
| --- | --- |
| Apple | - Yes. The two RRC parameters can be simultaneously configured. But default beam is a per-CC feature and simultaneous spatial relation info update is a cross-CC feature. |

# Views on Case2

Please share your views for case2 in the following table.

| Questions for view sharing: - Do you think it is necessary to refine current specification to “simultaneously configure enableDefaultBeamPL-ForSRS and simultaneousSpatial-UpdatedList1/simultaneousSpatial-UpdatedList” for a UE? - If answer to above is no, please clarify your rationale. |  |
| --- | --- |
| Apple | - No. - In our view, specification change should provide justification. |

# Conclusion

TBD

# References

[1] R1-2107988	Draft CR on spatial relation update across CCs for SRS	vivo