3GPP TSG RAN WG1 #106-e   	 	  	                 R1-210xxxx

e-Meeting, August 16th – 27th, 2021

Agenda item: 7.2.6

Source: Moderator (vivo)

Title: Summary of [106-e-NR-eMIMO-04]

Document for: Discussion and Decision

- Introduction

This contribution summaries discussion of email thread [106-e-NR-eMIMO-04] MB.3 (conflict between default and updated spatial relation for multi-CC) triggered by the draft CR in [1].

There are views expressed during the preparation phase that default spatial relation is defined/configured per CC, and including a CC with a default spatial relation in a CC list is an error case.

Two cases exist for simultaneous configuration of CC_list and default beam operation.

- Case1: Simultaneous configuration of enableDefaultBeamPL-ForSRS and simultaneousTCI-UpdateList1/ simultaneousTCI-UpdateList2;

- Case2: Simultaneous configuration of enableDefaultBeamPL-ForSRS and simultaneousSpatial-UpdatedList1/simultaneousSpatial-UpdatedList

We may need to first clarify understanding on current specification before further discussion.

- Views on Case1

Please share your views for case1 in the following table.

| Questions for view sharing: - Do you think it is allowed by current specification to “simultaneously configure enableDefaultBeamPL-ForSRS and simultaneousTCI-UpdateList1/simultaneousTCI-UpdateList2” for a UE? - If answer to above is no, please clarify why this is not allowed? |  |
| --- | --- |
| Apple | - Yes. The two RRC parameters can be simultaneously configured. But default beam is a per-CC feature and simultaneous spatial relation info update is a cross-CC feature. |
| DOCOMO | - Yes. In that case, SRS spatial relation in each CC is derived from PDCCH TCI state in each CC, and PDCCH TCI state in each CC is derived from simultaneousTCI-UpdateList1/simultaneousTCI-UpdateList2. |
| Samsung | - Yes. We have same view with Apple for two RRC parameters. |
| Ericsson | Yes, the two RRC parameters can be simultaneously configured. The case with undefined UE behavior is if enableDefaultBeamPL-ForSRS is configured, and an explicit update of the spatialRelationInfo is performed. |
| Lenovo/MotM | Yes. If both parameters are configured, the spatial relation for SRS is determined by the PDCCH TCI state in each CC, and the PDCCH TCI state in each CC is determined by simultaneousTCI-UpdateList1/simultaneousTCI-UpdateList2. |

- Views on Case2

Please share your views for case2 in the following table.

| Questions for view sharing: - Do you think it is necessary to refine current specification to “simultaneously configure enableDefaultBeamPL-ForSRS and simultaneousSpatial-UpdatedList1/simultaneousSpatial-UpdatedList” for a UE? - If answer to above is no, please clarify your rationale. |  |
| --- | --- |
| Apple | - No. - In our view, specification change should provide justification. |
| DOCOMO | - No, it is not necessary. - We don’t see any benefit to support this, compared to the feature we already have (i.e. enableDefaultBeamPL-ForSRS + simultaneousTCI-UpdateList1/ simultaneousTCI-UpdateList2 in section 2). |
| Samsung | - No. - For changing the SRS default beam, it is enough to use case1 which is already defined in the current spec. |
| Ericsson | - No - The specification is correct. |
| Lenovo/MotM | - No. - simultaneousTCI-UpdateList1/ simultaneousTCI-UpdateList2 are configured for simultaneous MAC CE based SRS spatial relation update across CCs, the CC configured with enableDefaultBeamPL-ForSRS shall not be contained in either CC list. |

- Conclusion

TBD

- References

[1] R1-2107988	Draft CR on spatial relation update across CCs for SRS	vivo