Agreement (RAN#104-e) For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where - W1 is a free selection matrix, with identity matrix as special configuration - FFS polarization-common/specific selection - Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1 - At least one value of Mv>1 is supported - Decide on the value(s) of Mv, e.g. Mv=2, in RAN1# 104bis-e - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters - FFS candidate value(s) of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf - Wf can be turned off by gNB. When turned off, Wf is an all-one vector (FFS; the length of all-one vector) - FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain

Comment based on agreement:

- according to the above agreement, when Wf is OFF, Wf is an all-one vector

- a single precoding matrix indicated by the PMI for all SBs or for the entire CSI reporting band

- WB pmi

- (A) When cqi format = WB, both cqi and pmi are essentially WB  freq granularity of CSI = WB

Spec related issue 1:

- (B) If pmi format is not provided (similar to R16), then otherwise (yellow highlighted) of the following text from 214 spec applies (SB freq granularity).

- (B) is contradictory to (A) which is based on agreement, when Wf OFF (WB pmi) and cqi format = WB.

- The correct text is 1st subbullet (green highlighted).

- Note: this is also an issue in R16 codebook also since Mv=1 is also possible there, though such cases can be argued to be corner cases.

- Pmi format = WB is straightforward solution to avoid the contradictory specification

Section 5.2.1.4, 38.214

A CSI Reporting Setting is said to have a wideband frequency-granularity if

-	reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or

-	reportQuantity is set to 'cri-RI-i1' or

-	reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or

-	reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR'

otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity.

Spec related issue 2:

- N3 is defined as the total number of precoding matrices (cyan text).

- If N3 >1, we can’t say “N3 precoding matrices” for Mv=1 since there is only one precoding matrix indicated by the PMI.

- We have to say one of the following:

  - A single precoding matrix is indicated by the PMI

  - N3 precoding matrices indicated by the PMI, but they are the same when Mv=1

Section 5.2.2.2.5, 38.214

The parameter R is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of configured subbands in csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows

Proposal 5-1: For Rel-17 PS codebook

- Wf  OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3

- Support pmiReportingFormat = WB [if N3=1]

- FFS: the case when no SB size is configured (from RAN1#105-e agreement)

- For Wf OFF, one of the following is included in the codebook description

  - Alt1: a single precoding matrix is indicated by the PMI

  - Alt2: N3 precoding matrices indicated by the PMI, but they are the same when Mv=1