Proposed agreement

- For DL multi-stream evaluations, a UE is declared as a satisfied UE if each stream meets the PER and PDB requirements, i.e., more than a certain percentage of packets are successfully transmitted within a given air interface PDB.

Proposed agreement

For Option 2 (video + audio/data) of evaluation of DL two streams that is an optional evaluation scenario, the audio/data flow is modelled as follows:

- A stream aggregating streams of audio and data

  - Periodicity: 10ms

  - Data rate: 0.756 Mbps/s or 1.12 Mbps

  - Packet size: determined by periodicity and data rate

  - PDB: 30ms (baseline).  Other values can be optionally evaluated.

  - PER: 1% (baseline). Other values can be optionally evaluated.

Proposed agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- The main objective of evaluating this option is to study the impact on capacity from different PDB and PER values for I-frame and P-frame.  It is not intended to directly compare capacity results (i.e., capacity numbers) for cases with two-stream modelling and those for cases with single-stream modelling.

- Alpha value: 2.0.  Other values, e.g., 1.5 or 3.0 can be optionally evaluated

- [PER_I, PER_P, PDB_I, PDB_P]

  - Alternative 1: define a limited number of common study cases in addition to a reference case

    - For DL

      - Reference case:

        - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 10ms, 10ms] for AR/VR

        - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 15ms, 15ms] for CG

      - Common study case

        - [PER_I, PER_P, PDB_I, PDB_P] = [0.5 %, 5 %, 17ms, 9ms] for AR/VR

        - [PER_I, PER_P, PDB_I, PDB_P] = [0.5 %, 5 %, 20ms, 14ms] for CG

      - Other cases, i.e., other values of [PER_I, PER_P, PDB_I, PDB_P] can be evaluated.

    - For UL AR aggregating streams of scene, video, data, and audio

      - Reference case:

        - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 30ms, 30ms]

      - Common study case

        - [PER_I, PER_P, PDB_I, PDB_P] = [0.5 %, 5 %, 40ms, 27ms]

      - Other cases, i.e., other values of [PER_I, PER_P, PDB_I, PDB_P] can be evaluated.

  - Alternative 2: List a full set of cases from which a company can choose cases to evaluate on their own. For instance,

    - For DL

      - AR/VR

        - Case 1: [PER_I, PER_P, PDB_I, PDB_P] = [1%, 1%, 10ms, 10ms]

        - Case 2: [PER_I, PER_P, PDB_I, PDB_P] = [0.5%, 5%, 10ms, 10ms]

        - Case 3: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 17ms, 9ms]

        - Case 4: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 10ms, 10ms]

        - Case 5: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 17ms, 10ms]

        - Case 6: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 17ms, 10ms]

        - …

      - CG

        - PER_I, PER_P: same as PER_I, PER_P of AR/VR, respectively

        - PDB_I, PDB_P: 5ms longer than PDB_I, PDB_P of AR/VR, respectively

        - …

    - For UL AR aggregating streams of scene, video, data, and audio

      - AR/VR

        - Case 1: [PER_I, PER_P, PDB_I, PDB_P] = [1%, 1%, 30ms, 30ms]

        - Case 2: [PER_I, PER_P, PDB_I, PDB_P] = [0.5%, 5%, 30ms, 30ms]

        - Case 3: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 40ms, 27ms]

        - Case 4: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 30ms, 30ms]

        - Case 5: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1%, 40ms, 30ms]

        - Case 6: [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 5%, 40ms, 30ms]

        - …

  - Alternative 3: No definition of a set of values of [PER_I, PER_P, PDB_I, PDB_P] to be evaluated.  Instead, it is left to companies which values to be evaluated.