3GPP TSG RAN meeting #93e										RP-211946

Electronic Meeting, Sep. 13-17, 2021

## Status Report to TSG

Agenda item:			10.4.2

| WI / SI Name | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Study Item: No | Study Item: No |  | Study Item: No |
| Acronym | LTE_terr_bcast_bands_part1 |  |  |  |  |
| Unique ID | 911020 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211144 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2021 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 90% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |  |
| --- | --- | --- | --- |
| Rapporteur | Name | Roland Beutler | Alberto RICO ALVARINO |
|  | Company | EBU | Qualcomm Incorporated |
|  | Email | Roland.Beutler@swr.de | albertor@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

The following agreements were reached in RAN1#106-e:

Agreement:

For supporting 6/7/8MHz PMCH:

- The bandwidth of CAS (system bandwidth indicated in MIB) is set to 25PRBs (5MHz).

  - FFS: whether it can be set to 6 and/or 15 PRBs.

- The bandwidth for PMCH (NRBPMCH) is indicated by another parameter in system information if the center frequency of bandwidth for CAS and bandwidth for PMCH is aligned.

  - FFS: Details

Agreement:

It is supported that the center frequency of system bandwidth and center frequency of PMCH bandwidth are aligned

- FFS: Other cases (non-aligned center frequencies)

Agreement:

For PMCH allocation of 6/7/8 MHz on MBMS-dedicated cells, mapping the MBSFN reference signal to REs is based on the PMCH bandwidth of 6/7/8 MHz instead of .

Agreement:

For PMCH allocation of 6/7/8 MHz on MBMS-dedicated cells, the TBS determination is based on the PMCH bandwidth of 6/7/8 MHz instead of .

Conclusion:

It is RAN1’s understanding that 15kHz SCS is currently supported for MBMS dedicated cells.

- RAN1 to further discuss how to handle 15kHz SCS for 6/7/8 MHz PMCH bandwidth

Agreement:

The following PMCH bandwidth values are supported:

- 8MHz: NRBPMCH=40

- 7MHz: NRBPMCH=35

- 6MHz: NRBPMCH=30

Agreement:

The signalling of PMCH bandwidth is to be selected to be one of the following:

- Alt 1: Per cell

- Alt 2: Per MBSFN area

Agreement:

For CAS bandwidth values, 25PRBs (5MHz) and 15 PRBs (3MHz) are supported.

Conclusion:

Non-aligned PMCH bandwidth and system bandwidth is not supported (this resolves an FFS item in a prior agreement).

Agreement:

For 6/7/8MHz PMCH bandwidth with 15 kHz SCS:

- Alt 1: The control region in MBSFN subframes with 15 kHz SCS has the same bandwidth as CAS. The UE is not required to process the control region.

- Alt 2: The MBSFN subframes with 15 kHz SCS do not have control region.

- Alt 3: The MBSFN subframes with 15 kHz SCS have a control region but its content is not defined

Agreement:

The signalling of PMCH bandwidth is per MBSFN area.

Agreement:

The UE is not required to process the control region in MBSFN subframes for 6/7/8MHz PMCH bandwidth with 15kHz SCS

- Note: This does not preclude future removal of the control region after completion of this WI.

- Note: The agreement is made with the intention to capture the agreement with minimal impact to the specifications.

#### 2.1.2	Remaining Open issues

  - Approve CRs for the corresponding agreements

## 2.2	RAN2

#### 2.2.1	Agreements

None

#### 2.2.2	Remaining Open issues

Approve changes to 36.331 to introduce signalling related to PMCH bandwidth

## 2.3	RAN3

#### 2.3.1	Agreements

None

#### 2.3.2	Remaining Open issues

Approve changes to 36.443 to introduce signalling related to PMCH bandwidth

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

Not applicable

## 4.	References

R1-2106762	Work Plan for New Bands and Bandwidth Allocation for LTE based 5G Terrestrial Broadcast			Qualcomm Incorporated, EBU

R1-2106560	On PMCH allocation of 6/7/8 MHz and corresponding reference signals	Huawei, HiSilicon

R1-2106752	Discussion on PMCH allocation and corresponding MBSFN reference signals	ZTE

R1-2106763	PMCH allocation of 6/7/8MHz	Qualcomm Incorporated

R1-2108211	Feature lead summary on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast		Moderator (Qualcomm)

R1-2108324	Feature lead summary #2 on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast		Moderator (Qualcomm)

R1-2108360	Feature lead summary #3 on New bands and bandwidth allocation for LTE based 5G terrestrial broadcast 	Moderator (Qualcomm)

R1-2106753	Considerations on bandwidth changing in MBSFN subframe for 15KHz subcarrier spacing	ZTE

R1-2107685	Numerologies supported for MBMS-dedicated cell	Huawei, HiSilicon