3GPP TSG-RAN WG1 Meeting #106-e	R1- 2108220

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.4.1

Source:	Moderator (Ericsson)

Title:	Feature lead summary#1 on timing relationship enhancements

Document for:	Discussion

Introduction

A study item on solutions for NR to support non-terrestrial networks (NTN) was completed in Rel-16 [1]. The Rel-17 work item on solutions for NR to support NTN was approved at RAN#86 and the work item description is updated in [2]. One objective is to specify timing relationship enhancements for NTN. The last feature summary from RAN1#105-e on this topic can be found in [3].

In this contribution, we summarize the related issues and proposals based on the contributions submitted to RAN1#105-e under agenda item 8.4.1 [4] – [32].

There are in total 14 issues summarized in this contribution.

- Companies are encouraged to provides views on the following issues by filling in comments in the provided tables:

  - Issue #1, Issue #2, Issue #3, Issue #5, Issue #7, Issue #9, Issue #10, Issue #13, Issue #14

- Companies are encouraged to have offline discussions on the following issues:

  - Issue #4, Issue #6, Issue #8, Issue #11, Issue #12

1	Issue #1: K_offset update

1.1	Background

At RAN1#106-e, many companies provide views on K_offset update after initial access.

[Huawei, HiSilicon]

Proposal 4: K_offset update after initial access is supported via MAC-CE.

Proposal 6: Differential indication with granularity of one slot are adopted for both UE-specific K_offset update and TA report to reduce the signaling overhead.

[vivo]

Proposal 3：Support both MAC-CE and RRC reconfiguration for the update of K_offset.

[Spreadtrum]

Proposal 3: The method to avoid frequent K_offset update with dedicated RRC signaling/MAC CE should be considered.

[Zhejiang Lab]

Proposal 3: Both cell/beam specific and UE specific updating of K_offset should be supported as follows,

- For cell/beam specific K_offset updating, K_offset can be broadcasted in system information;

- For UE specific K_offset updating, the following cases should be considered,

  - if UE location is available to the gNB, UE specific K_offset can be configured by gNB without any reporting from UE;

  - if UE location is not available to the gNB, UE specific K_offset can be derived from common TA and UE specific TA, which requires UE specific TA reporting.

Proposal 4: For updating K_offset after initial access, both RRC reconfiguration and MAC CE should be supported and offset values of the first K_offset value can be signaled for update to save signaling overhead.

[Baicells]

Proposal 1: For clarity, we propose to explicitly state UE-specific K_offset, beam-specific K_offset, or cell-specific K_offset when discussing K_offset.

Proposal 2: For GEO, UE-specific K_offset can be updated by RRC reconfiguration.

Proposal 3-1: For LEO, determine the update rate of UE-specific K_offset according to (1) orbit height, (2) SCS, (3)whether or not the network dynamically compensates the drifting part of the feeder link.

Proposal 3-2: For LEO, update UE-specific K_offset by RRC reconfiguration or MC CE according to system configurations.

Proposal 4: UE calculates UE-specific K_offset autonomously according to UE’s TA, using a formula such as: UE-specific K_offset = ceiling (TA_ue/slot duration).

Proposal 5: UE acquires from the network the location information of the gNB and the distance information between gNB and RP (or TA_network) to calculate TA_ue, and then get UE-specific K_offset according to Proposal 4. If TA_network does not change, UE can update UE-specific K_offset autonomously.

[Samsung]

Proposal 1: The update of K_offset value after initial access is done by the combination of RRC configuration and MAC CE. MAC indicates only one of the multiple values configured by the gNB.

[CATT]

Proposal 4: Both MAC CE and RRC reconfiguration to indicate K_offset can be supported.

[NEC]

Proposal 3: Support UE based triggering for K_offset update after initial access.

[CAICT]

Proposal 1: Support three options for Koffset updating: RRC reconfiguration, MAC CE, RRC reconfiguration and MAC CE in the specification.

[Hyundai Motors]

Proposal 1: A new RRC signalling should be considered to provide update frequency of K_offset.

Proposal 2: Even after the initial access, cell-specific or beam-specific K_offset values provided by SIBs can be used to update K_offset.

Proposal 3: SIB of the target cell provides the handover UE with its cell-specific K_offset after connection to the target cell.

Proposal 4: The serving cell provides the handover UE with the cell-specific K_offset of the target cell using RRC reconfiguration during handover procedure.

[OPPO]

Proposal 2: Support UE requesting K offset update to the network in an event triggered manner.

[Qualcomm]

Proposal 1: Support updating K_offset after initial access at least by MAC-CE

- FFS: details of signaling

[CMCC]

Proposal 1: Regarding UE-specific K_offset update after initial access, both MAC CE option and RRC reconfiguration option can be supported.

Proposal 2: If further down-selection is needed, MAC CE option is preferred for universality.

[Panasonic]

Proposal 2: Support both RRC reconfiguration and MAC CE for the update of Koffset. Relative indication should be used for MAC CE.

[LGE]

Proposal 4: Support at least MAC-CE based K_offset update after initial access.

Proposal 5: Apply new K_offset value X slot/symbol after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception.

[Intel]

Proposal 2:

- At least RRC-based K_offset update after initial access shall be supported

  - MAC CE-based K_offset update can be additionally considered for non-GEO scenario

[Ericsson]

Proposal 1: Koffset is updated by Koffset+ΔKoffset, where ΔKoffset is configured after initial access and is zero if not configured.

Proposal 2: For updating K_offset after initial access, support both MAC CE option and RRC reconfiguration option.

[Apple]

Proposal 2: For signaling of updating Koffset after initial access, both RRC reconfiguration and MAC CE are supported.

[ZTE]

Proposal-2: Updating K_offset via MAC CE only should be supported.

Proposal-3: Signalling of the adjustment value of K_offset is preferred via MAC CE.

[NTT DOCOMO]

Proposal 3: A RRC parameter to configure UE-specific K_offset.

- If this parameter is provided, the UE uses the parameter as K_offset.

- Otherwise, the UE uses K_offset provided in initial access.

[Xiaomi]

Proposal 3: Both RRC and MAC CE are supported to update the K_offset.

[Lenovo, Motorola Mobility]

Proposal 1: Support MAC CE signaling to update K-offset after initial access.

Proposal 3: Update of K-offset can be indicated by a drift rate or by indication of a coordinate of a position.

[ITL]

Proposal 1. It is preferred to adopt MAC CE signaling for updating K_offset value after initial access.

Proposal 2. gNB controlled and UE-initiated mechanisms can be supported for update of K_offset value.

[InterDigital]

Proposal-4: MAC-CE is used for updating K-offset value after initial access

Proposal-5: RRC is not used additionally for updating K-offset value after initial access

[Fraunhofer IIS, Fraunhofer HHI]

Proposal 5: RAN1 to discuss first the scope of Koffset update a) whether or not UE specific Koffset is supported, or b) only cell-specific and beam-specific Koffset update is supported, or c) both options a) and b) are supported.

[Nokia, Nokia Shanghai Bell]

Proposal 6: RAN1 to adopt MAC-CE as the baseline mechanism for K_offset update.

The main discussion point is about using MAC CE or both RRC reconfiguration and MAC CE to update K_offset.

The table below presents a summary of the proposed design options and the corresponding proponents.

| Design option | Proponent(s) |
| --- | --- |
| Both RRC reconfiguration and MAC CE | [14] sources: [vivo, Spreadtrum, Zhejiang Lab, Baicells, Samsung, CATT, CAICT, CMCC, Panasonic, LGE, Intel, Ericsson, Apple, Xiaomi] |
| MAC CE | [7] sources: [Huawei/HiSilicon, Qualcomm, ZTE, Lenovo/Motorola Mobility, ITL, InterDigital, Nokia/Nokia Shanghai Bell] |
| RRC reconfiguration | [2] sources: [Hyundai Motors, NTT DOCOMO] |

Based on companies’ positions, Moderator holds the view that supporting both RRC reconfiguration and MAC CE to update UE specific K_offset is a reasonable middle ground, and hope that it is an acceptable compromise to the group.

1.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 1.2 (Moderator):

UE can be provided by network with a UE-specific K_offset in RRC reconfiguration. The UE-specific K_offset can be updated by MAC CE.

| Company | Comments |
| --- | --- |
| Apple | Support the proposal. When Koffset is updated frequently (e.g., in LEO scenario), MAC CE is suitable due to its lower latency. When Koffset is updated infrequently (e.g., in GEO scenario), RRC reconfiguration can be used. |
| Zhejiang Lab | Support the proposal. Both MAC CE and RRC should be supported depending on circumstances. |
| FGI | Support. However, since no clear action time for RRC reconfiguration is defined in the current spec, this makes impact on Initial proposal 3.2. |
| Xiaomi | Although we share the spirit that Koffset can be updated by RRC or MAC CE. We have one clarification question on the RRC reconfiguration as RRC reconfiguration happens after the RRC connection is established. Does it preclude that the Koffset can be updated during the initial access such as in MSG4. |
| ZTE | Following the agreement in GTW session. Only focus on the MAC CE based solution. |
| Intel | The following agreement was made during the GTW. Agreement: - The UE-specific K_offset can be provided and updated by network with MAC CE. - FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration - FFS: Details on whether and how the two solutions work together |
| OPPO | agree |
| Nokia, Nokia Shanghai Bell | No need for having RRC reconfiguration – we would prefer just use MAC-CE for the updating of the K_offset. In our preference the FFS is simply dropped, but for now we are OK with the agreement. |

2	Issue #2: K_offset value determination

2.1	Background

At RAN1#106-e, many companies provide views on K_offset configuration.

[Huawei, HiSilicon]

Proposal 1: For determination of cell-specific K_offset in system information, K_offset is equal to the sum of two offset values

- The first offset value is equal to common TA signaled in system information

- The second offset is signaled in system information and covers the maximum service link RTD within the cell.

[vivo]

Proposal 2: Support to signaling one offset value for K_offset (Option 1) in system information.

[Spreadtrum]

Proposal 1: One offset value indicated by system information for K_offset is cover the RTT of service link plus the RTT between serving satellite and reference point.

[Zhejiang Lab]

Proposal 2:  The initial value(s) of K_offset should be chosen considering the worst case, i.e., cell edge UE and the K_offset value(s) should depend on numerology and satellite type.

[Baicells]

Proposal 7: For determination of cell-specific K_offset, support as many as possible options to facilitate different system deployments:

- Option 1: Signal 1 value as the cell-specific K_offset. It is an upper limit value to cover all possible TA in the cell during the total period of a feeder link connection, so that it does not need update during this feederlink connection.

- Option 2: Signal 2 values. Value1 is to cover TA_servicelink_max. Value2 is to cover TA_common. Cell-specific K_offset is the sum of Value1 and Value2.

- Option 3: Signal 3 values. Value1 is TA_servicelink_max. Value2 is gNB’s location. Value3 is TA_network. UE calculates TA_feederlink base on value2 and ephemeris information. Then UE calculate cell-specific K_offset using the formula:
cell-specific K_offset = ceiling ( (TA_servicelink_max + TA_feederlink -TA_network) / slot duration )

Where,

TA_servicelink_max is the TA to cover the maximum RTT of the servicelink in a cell.

TA_feederlink is the TA to cover the feeder link.

TA_common is the TA to cover the RTT between RP and satellite.

TA_network is the TA compensated by the network.

[Sony]

Proposal 1: Part of K_offset value should be implicitly derived by calculation at the UE from the .

Proposal 2: RAN1 should support to signal a first offset value and a second offset value for determination of cell-specific K_offset in system information.

[Samsung]

Proposal 2: Configure only a single value for K_offset.

[CATT]

Proposal 1: For determination of cell-specific K_offset in system information, signaling two offset values is preferred.

Proposal 2: Indicating cell-specific K_offset by SIB1 to UE is supported.

Proposal 3: TA should be reported in Msg3, and signal UE_specific K_offset in Msg4.

[NEC]

Proposal 2: Support explicit signaling of K_offset used in initial access in system information.

[MediaTek]

Proposal 4: Signal one offset value for K_offset and TA report are supported.

[OPPO]

Proposal 4: decide whether beam-specific K offset is supported in system information before selecting the K offset indication option 1 vs. option 2.

Proposal 5: decide whether common TA can be used to determine the K offset before selecting the K offset indication option 1 vs. option 2.

[CMCC]

Proposal 5: For determination of cell-specific K_offset in system information, Option 2 (K_offset is equal to the sum of the two indicated offset values) should be supported, wherein,

- K_offset (in unit of slot) = offset_1 + offset_2, where offset_2 is explicitly indicated in system information, and offset_1 is implicitly determined by common TA (NTA,common) as following.

offset1=NTA,common∙Tc2-μ×10-3

[LGE]

Proposal 1: Support explicit signaling of K_offset.

Proposal 2: For determination of cell-specific K_offset in system information, support signaling one offset value for K_offset (Option 1).

[Ericsson]

Proposal 7	For determination of cell-specific K_offset in system information, signal one offset value for K_offset.

Proposal 8	For determination of K_offset value, down-select one option: Option 1 – signal one K_offset value with a unit of ms; Option 2 – signal one K_offset value with a unit of slot associated with a reference numerology.

[Intel]

Proposal 3:

- Indication of K_offset is done using two values for K_offset determination: K_offset_1 and K_offset_2

- K_offset, Common TA and K_mac are determined based on K_offset_1 and K_offset_2 values and value of bit a indicated together with K_offset_1 and K_offset_2

  - If a = 0,

    - K_offset = K_offset_1 + K_offset_2,

    - Common TA = K_offset_2, K_mac = 0

  - If a = 1,

    - K_offset = K_offset_1,

    - Common TA = 0, K_mac = K_offset_2

[Apple]

Proposal 1: Support to signal a single offset value for cell specific Koffset.

[ZTE]

Proposal-1: Option-1 is preferred with single value to cover the RTT of service link plus the RTT between serving satellite and reference point.

[NTT Docomo]

Proposal 1: K_offset is signaled in SIB1 or in SIB following SIB1.

Proposal 4: For determination of cell-specific K_offset in system information, signal one offset value for K_offset.

[Xiaomi]

Proposal 2: It is slightly preferred to signal one single value to determine the cell-specific K_offset in system information.

[Lenovo, Motorola Mobility]

Proposal 2: Support K-offset indication with one values. The value corresponding to RTT between UE and reference point.

[ITL]

Proposal 4. Single one offset value for K_offset can be adopted.

[InterDigital]

Proposal-1: K-offset value is independently determined/indicated from common TA in the system information (Alt-1)

Proposal-2: a single K-offset value is signaled in SIB (Option-1)

[Fraunhofer IIS, Fraunhofer HHI]

Proposal 1: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., RRC timers T300, T301, T319, and T310.

Proposal 2: Common Timing Advance should be used for determination of the first offset value, capturing the RTT of the satellite to RP, in Option-2 to reduce signaling overhead and avoid duplicate signaling.

Proposal 3: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., “ra-ContentionResolutionTimer” and common TA for option-2.

Proposal 4: RAN1 to down-select Option-2 for determination of the value of initial Koffset.

[Nokia, Nokia Shanghai Bell]

Proposal 3: RAN 1 to adopt one single offset value for K_offset to be signaled.

Proposal 4: RAN 1 to consider implicit signalling of differential K_offset in the time/frequency values of the UL scheduling in the RAR as an alternative to explicit NR-beam level signalling in the SI.

Proposal 5: RAN 1 to consider implicit signalling of differential K_offset in the temporary C-RNTI in RAR as an alternative to explicit NR-beam level signalling in the SI.

The main discussion point is about selection between the two options agreed at RAN1#104bis-e to determine K_offset value: signal one offset value vs. signal two offset values. The table below presents a summary of the proposed design options and the corresponding proponents.

| Design option | Proponent(s) |
| --- | --- |
| Option 1: Signal one offset value for K_offset | [16] sources: [vivo, Spreadtrum, Baicells, Samsung, NEC, MediaTek, LGE, Ericsson, Apple, ZTE, NTT Docomo, Xiaomi, Lenovo/Motorola Mobility, ITL, InterDigital, Nokia/Nokia Shanghai Bell] |
| Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values | [7] sources: [Huawei/HiSilicon, Baicells, Sony, CATT, CMCC, Intel, Fraunhofer IIS/Fraunhofer HHI] |

Recall the discussions at the previous meeting, which led to the following recommendation.

Moderator recommendation on Issue #2:

Companies are encouraged to rethink about their position and come back to this issue at the next RAN1 meeting, taking the following into account.

- The signaling overhead saving in Option 2 vs. Option 1 is only about 1 bit.

- The ~1-bit saving might appear free, but comes at the cost of many disadvantages (more complexity for UE, more specification impact, more sources of inaccuracies, scheduling restriction, etc.)

- 1 bit saving in higher layer signaling is negligible.

It is good to see that now companies are more converging on this issue, with many more companies supporting Option 1 (Signal one offset value for K_offset).

Besides the limited signaling overhead saving, indeed Option 2 is unnecessarily complicated. For example, the following observations are made in [Apple]’s contribution:

[Apple]

The signaling of RTT between satellite and timing reference point is still in discussion under AI 8.4.2. Although a common TA broadcast is supported, whether or not to broadcast common TA drift rate and its high order derivatives is still open. If the common TA drift rate and its high order derivatives are broadcasted, then Koffset1 may be maintained by UE, based on broadcasted common TA parameters. This is not desirable since it is not guaranteed that the same Koffset1 is maintained by all UEs. Furthermore, the specification impact and UE complexity of Option 2 is higher than Option 1.

Therefore, in Moderator’s view,

- Making K_offset dependent on common TA is a too much optimization for a basic scheduling parameter as fundamental as K_offset.

- It is error prone. Think about the misalignment between gNB and UE, the time-varying delay for a message to traverse from gNB to UE, the burden that the network needs to have in order to track the time-varying K_offset, etc.

The following figure provides an illustration of the complications of making K_offset dependent on common TA, which may be time varying.

Therefore, it is much more robust that network just signals one value to UE and ask UE to use it for K_offset. And if necessary, network can send an updated value for UE to use as time goes.

Also, it would be good to start discussing the unit and value range for K_offset.

2.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 2.2 (Moderator):

- If the value of K_offset depends on time-varying common TA, do you agree that there would be a misalignment between gNB and UE about the value of K_offset? And why?

- For determination of K_offset value, which option do you think is better? And why?

  - Option 1: milliseconds

  - Option 2: slots for a given subcarrier spacing in a frequency range

- What is the range of values that should be supported for K_offset?

| Company | Comments |
| --- | --- |
| Apple | For 1), we think the misalignment between gNB and UE about K_offset may exist. For example, UE may derive the common TA as 4.4999 ms and gNB may derive the common TA as 4.5000 ms. With the rounding rule, the Koffset derived by gNB could be one larger than the Koffset derived by UE. In another example, UE updates the common TA from 4.4990 ms to 4.5000 ms during its uplink transmissions. gNB is unsure which K_offset value is used by the UE in its uplink transmissions. For 2). We sligtly prefer Option 2. If the K_offset is in unit of milliseconds while the sub-carrier spacing is larger than 15 kHz, then some slot numbers cannot be indicated by K_offset value. For 3). We think the range of K_offset should be large enough to cover the UE-gNB RTT, which is upper bounded by 542 ms according to 38.821. Hence, the value range of K_offset is [0, 542] ms (or, in equivalent slots). Some optimization based on scenarios (e.g., LEO, GEO) could be considered. For example, the value range of K_offset could be [0, 26] ms (or, in equivalent slots) in case of GEO-600. |
| Zhejiang Lab | I guess the focus should be 2) and 3). For 2), we support option 2 since it is more clear. For 3), we think different value range can be defined for different scenario. For example, one for LEO and one for GEO. |
| FGI | - YES - Option2 (that fits K1 and K2) - Range of UE’s TA (which shall be defined as well) |
| Xiaomi | For 1), we are discussing the value determination of cell-specific Koffset. In our understanding, signaling one value or two value depending on the common TA may not lead to misalignment issue between gNB and UE For 2), we don’t have a strong view. Either option is fine to us. As the DL/UL operation is numerology dependent, if option 2 is used, a reference numerology needs to be defined For 3), the range of Koffset is up to gNB’s configuration as it is dependent on the scenarios such as GEO/LEO and RP location. The value range can be large enough to cover the UE-gNB RTT. For example, the value range can be up to [0, 540]ms for GEO case in some cases. |
| ZTE | For Q1, Yes, if K_offset(in fact only partial K_offset) is associated to common TA, different UEs may calculate the value of K_offset in different time instant based on the common TA drift rate, that leads to different results of configuration of first value of K_offset which are supposed to be common. Besides, there would be misalignment between gNB and UE if they derive the first value of K_offset at different time instant, respectively. For Q2, Both are fine. And configurable granularity is preferred, because proper unit of K_offset corresponding to different scenarios may reduce signalling overhead. For Q3, the range of K_offset is related to the value range of RTT in specific scenarios. E.g., for GEO, assume the max RTT is 541.46 ms (service and feeder links) , the value range of K_offset could be 238 ms~542 ms. |
| Intel | - In our view cell-specific K_offset should be based only on constant Common TA indicated in the SI and any additional parameters (e.g. Common TA drift rate) shall be ignored for K_offset determination. Thus, any issue with time-varying Common TA can be avoided. - Option 1 and Option 2 are similar since translation of milliseconds to slots is trivial to specify - We prefer to have full range of K_offset from 0 to maximum value (according to the max RTT). In order to decrease the overhead value of K_offset can be downsampled especially for higher SCS. Also, regarding the benefit from dividing K_offset into two parts, the overhead savings is more than 1 bit if K_mac is considered jointly with Common TA and K_offset. Since those 3 parameters are related to feeder link delay it is redundant to do separate indication. We propose to consider the following solution which enables deployments with aligned DL-UL timing and not aligned DL-UL timing at the gNB. - Indication of K_offset is done using two values for K_offset determination: K_offset_1 and K_offset_2 - K_offset, Common TA and K_mac are determined based on K_offset_1 and K_offset_2 values and value of bit a indicated together with K_offset_1 and K_offset_2 - If a = 0, - K_offset = K_offset_1 + K_offset_2, - Common TA = K_offset_2, K_mac = 0 - If a = 1, - K_offset = K_offset_1, Common TA = 0, K_mac = K_offset_2 |
| Nokia, Nokia Shanghai Bell | As a starting point, we do not support UE autonomous update of K_offset values. The K_offset value is expected to be broadcast by the gNB, and potentially updated on a per-need and per-UE basis. There is no need for defining a UE autonomous behaviour for the K_offset value. We think that there may be potential misalignment – for the transition period – if the K_offset depends on the common TA. For the second part of the question we are a bit uncertain if this question is related to the actual indication of the K_offset (the granularity of the parameter) or for the UE autonomous determination of K_offset value. For the granularity of the parameter, the slot unit seems reasonable. For the third question the range of values would vary by the deployment scenario, such that LEO, MEO and GEO would most likely have a need for different value ranges. |

3	Issue #3: K_offset usage

3.1	Background

At RAN1#106-e, many companies provide views on K_offset update after initial access.

[Baicells]

Proposal 8-1: Explicitly state cell-specific K_offset instead of ‘The K_offset value signaled in system information’.

Proposal 8-2：In RAN1, we suggest to use the specific types of K_offset, i.e. cell-specific K_offset, beam-specific K_offset or UE-specific K_offset, when discussing its usage.

Proposal 8-3：Cell-specific, beam-specific and UE-specific K_offset need to be maintained separately so that it can be assigned, updated and used independently.

Proposal 9-1：Cell/Beam specific K_offset configured in system information or msg2 should be used for timing relationships related to random access.

Proposal 9-2：If UE is provided with UE-specific K_offset, it should be used for timing relationships that not related to random access processes.

Proposal 9-3：If UE-specific K_offset is aging, cell/beam specific value should be used instead of it.

Proposal 9-4：When beam/UE specific K_offset is not available, cell-specific value can be applied for all the timing relationships.

[Sony]

Proposal 3: When enhancing relationships by K_offset extension, apply the extension before the TA.

[Samsung]

Proposal 5: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

[CAICT]

Proposal 2: Use cell-specific Koffset for the timing relationships related to fallback DCI formats and use updated Koffset for the timing relationships related to non-fallback DCI formats.

[FGI, Asia Pacific Telecom, III, ITRI]

Proposal 6	Support the transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Proposal 7	Support the transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Proposal 8	No additional transmission timings related to fallback DCI formats are needed if the MAC CE option is supported for updating K_offset after initial access.

[Panasonic]

Proposal 3: For PUSCH scheduled by DCI 0_0 and HARQ-ACK to PDSCH scheduled by DCI 1_0, Koffset value signaled in system information should be used.

[Ericsson]

Proposal 3	The K_offset value signaled in system information is always used for the transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI and C-RNTI.

Proposal 4	The K_offset value signaled in system information is always used for the transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI and C-RNTI

Proposal 5	The K_offset value signaled in system information is always used for any fallback DCI 1_0 / 0_0 scheduled transmission timing in which K_offset is applicable.

[Ericsson] – cont’d

Proposal 6	Clarify how K_offset is used in each timing relationship as follows:

- For the transmission timing of DCI scheduled PUSCH (including CSI on PUSCH), the slot allocated for the PUSCH is n⋅2μPUSCH2μPDCCH+K2+Koffset.

- For the transmission timing of RAR grant scheduled PUSCH, the UE transmits the PUSCH in slot n + K2 +Δ+Koffset.

- For the transmission timing of HARQ-ACK on PUCCH, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot n+K1+Koffset.

- For the CSI reference resource timing, the CSI reference resource is given in the downlink slot n-nCSIref-Koffset.

- For the transmission timing of aperiodic SRS, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot n∙2μSRSμPDCCH+k+Koffset.

[Apple]

Proposal 5: If a UE is provided with a Koffset value beyond system information, then that Koffset value is used for the transmission timing of HARQ-ACK on PUCCH to Msg4/MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Proposal 6: If a UE is provided with a Koffset value beyond system information, then that Koffset value is used for the transmission timings scheduled by fallback DCI formats.

[ZTE]

Proposal-7: It is preferred that initial K_offset can be applied for following timing relationships:

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

Proposal-8: If there is signaling of K_offset conveyed in Msg2 (e.g., beam-specific value), the K_offset value can be applied in the transmission timing of HARQ-ACK on PUCCH to Msg4.

Proposal-9: No need to define the limitation that only cell-specific K_offset is used for transmission scheduled by fallback DCI formats.

[ITL]

Proposal 3. It is agreeable to use cell-specific K_offset value for the case where fallback DCI is used.

[Nokia, Nokia Shanghai Bell]

Proposal 1: For Fallback DCI formats, the cell-specific K_offset, which is broadcasted in SI, should be used.

The proposals mainly focus on addressing the FFS’s in the following agreement made at RAN1#105-e:

Agreement:

The K_offset value signaled in system information is always used for

- The transmission timing of RAR / fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

FFS: how to treat additional transmission timings related to fallback DCI formats

FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

Since the last FFS is related to beam-specific K_offset, the support of which is still pending, the following discussions focus on the first three FFS’s.

3.1.1	On the “FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI” and the “FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI”

The table below presents a summary of the proposed design options and the corresponding proponents.

| - The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI - The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI | Proponent(s) |
| --- | --- |
| Always use the cell-specific K_offset (i.e., the K_offset value signaled in system information) | [8] sources: [Baicells, CAICT, FGI/Asia Pacific Telecom/III/ITRI, Panasonic, Ericsson, ZTE, ITL, Nokia/Nokia Shanghai Bell] |
| UE-specific K_offset if provided (otherwise, use the cell-specific K_offset) | [1] sources: [Apple] |

Several observations are in order.

- Based on companies’ positions, many hold the view that the cell-specific K_offset (i.e., the K_offset value signaled in system information) should always be used when the CRC of the DCI format 1_0 is scrambled by C-RNTI for the transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH, which is same behavior as when the CRC of the DCI format 1_0 is scrambled by TC-RNTI.

- Similarly, many hold the view that the cell-specific K_offset (i.e., the K_offset value signaled in system information) should always be used when the CRC of the DCI format 1_0 is scrambled by C-RNTI for the transmission timing of HARQ-ACK on PUCCH to MsgB, which is same behavior as when the CRC of the DCI format 1_0 is scrambled by MsgB-RNTI.

In Moderator’s view, having consistent behavior can simplify implementation, as proposed by many companies. That said, it would be good to collect views from more companies.

3.1.2	On the “FFS: how to treat additional transmission timings related to fallback DCI formats”

The table below presents a summary of the proposed design options and the corresponding proponents.

| How to treat additional transmission timings related to fallback DCI formats | Proponent(s) |
| --- | --- |
| Always use the cell-specific K_offset (i.e., the K_offset value signaled in system information) | [5] sources: [CAICT, Panasonic, Ericsson, ITL, Nokia/Nokia Shanghai Bell] |
| UE-specific K_offset if provided (otherwise, use the cell-specific K_offset) | [4] sources: [Baicells, FGI/Asia Pacific Telecom/III/ITRI, Apple, ZTE] |

It can be seen that the views are more polarized in this case. Thus, it would be good to collect views from more companies.

3.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 3.2 (Moderator):

- On the transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI, which option do you think is better? And why?

  - Option 1: Always use the cell-specific K_offset (i.e., the K_offset value signaled in system information)

  - Option 2: UE-specific K_offset if provided (otherwise, use the cell-specific K_offset)

- On the transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI, which option do you think is better? And why?

  - Option 1: Always use the cell-specific K_offset (i.e., the K_offset value signaled in system information)

  - Option 2: UE-specific K_offset if provided (otherwise, use the cell-specific K_offset)

- On the “FFS: how to treat additional transmission timings related to fallback DCI formats”, which option do you think is better? And why?

  - Option 1: Always use the cell-specific K_offset (i.e., the K_offset value signaled in system information)

  - Option 2: UE-specific K_offset if provided (otherwise, use the cell-specific K_offset)

Note: how to update this formulation with beam-specific K_offset can be discussed later if beam-specific K_offset is agreed to be supported.

| Company | Comments |
| --- | --- |
| Apple | For 1) and 2), we prefer Option 2, since it achieves higher scheduling efficiency. For 3), we prefer Option 2. In many cases, UE only needs to monitor fallback DCIs. Using a UE-specific K_offset has the advanage of efficient scheduling. Considering the number of times that UE receiving fallback DCIs for uplink scheduling, it is preferred to support Option 2. |
| Zhejiang Lab | For all situations, we prefer option 2 since UE-specific K_offset should always be used if provided. |
| FGI | Q1) Option 1 (cell-specific) Q2) Option 1 (cell-specific) Q3) Option 1 (cell-specific) For 1) and 2) It seems less problematic if CBRA in RRC_IDLE/INACTIVE/CONNECTED and CFRA in RRC_CONNECTED only use cell-specific K_offset. For 3), to support UE-specific K_offset in RRC reconfiguration, option 1 may be better to handle the ambitious period of 10ms. |
| Xiaomi | For 1) and 2) we prefer option 2. For 3), we prefer option 2, as long as UE-specific Koffset is provided, it should be used. As in NTN scenario, due to strictly link budget, fallback DCI is mostly used to gurantee the control information reliability. In this sense, option 2 is preferred to acheive higher efficiency. |
| ZTE | For Q1, for this case, in our view, only Option-1 shoud be consdiered since the contention resolution related behavior occurs during the initial access. In this way, the UE specific value is not available. For Q2, similar to Q1, option 2 may not work, specifically, assume updated K_offset is indicated in MsgB, the value can’t be applied for NACK reporting since the value can’t be got when transport block in MsgB isn’t detected. For Q3, option 2 is more reasonable, it has been agreed that when UE is not provided updated information, the initial K_offset value can be used. |
| Intel | We don’t see significant issue for Option 2, Option 2 is more efficient from the delay perspective. |
| OPPO | We think the question 1) and 2) are not valid questions. There does not exisit contention resolution PDSCH scheudled by DCI format 1_0 with CRC C-RNTI. At least from the current spec, the contention resolution PDSCH is always scheduled by DCI format 1_0 with CRC TC-RNTI. Our proposal is that - For any uplink transmisisons within RACH procedure, apply a cell-specific K offset. - For any uplink transmission after RACH procedure, apply a UE-specific K offset if no UE-specific K offset, apply a cell-specific K offset |
| Nokia, Nokia Shanghai Bell | On aspect (1), we are of the opinoin that the contention resolution is part of the initial access procedure. For such case, the gNB will have had the chance to update the UE specific K_offset value. Hence, the gNB would need to assume that UE applies the cell-specific K_offset value. On aspect (2), the same argumentation would apply, so cell-specific K_offset would need to be applied here as well. On aspect (3), the gNB would use fallback DCI formats when there is a risk of the UE not being able to understand the other configured DCI formats. Hence, the UE should not be expected to have any understanding of any configured UE specific K_offset. |
| NTT DOCOMO | For 1) and 2), we prefer Option1, since it’s better to have the same mechansim with the PUCCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI. |

4	Issue #4: Beam-specific K_offset in initial access

4.1	Background

At RAN1#106-e, several companies provide proposals on this topic:

Proposals that support introducing beam specific Koffset

[vivo]

Proposal 1: Support beam-specific K_offset for initial access.

[Spreadtrum]

Proposal 2: Beam-specific values of K_offset configuration for initial access should be supported.

[Zhejiang Lab]

Proposal 1: Per beam K_offset configuration should be supported and for the case of implicit configuration derived from per cell common TA, the difference between the per cell K_offset and the per beam K_offset can be signaled in the system information to reduce the signaling overhead, i.e., in the second offset value.

[Baicells]

Proposal 6：Support beam-specific K_offset to to achieve better efficiency in UL response or achieve better efficiency in signaling overhead.

Proposal 10-3: In order to support beam-specific K_offset, we suggest to support beam-specific system information, which carries beam-specific K_offset value and is broadcast in each beam.

Proposal 10-4: Temporary C-RNTI in msg2 is suitable to carry beam-specific K_ offset, due to its large range and thus flexibility.

Proposal 10-5: Support beam-specific K_ offset in initial access, using one or more modes from Mode 1 to Mode 4 elaborated above.

[CMCC]

Proposal 6: gNB has the flexibility of configuring cell-specific or beam specific value of K_offset.

- Beam specific SIB can be supported, i.e., different beam specific SIB may carry different beam specific values (e.g., K_offset).

[LGE]

Proposal 3: Support beam (group)-specific K_offset signaling in addition to cell-specific K_offset in initial access.

[Intel]

Proposal 1: Support beam specific K_offset configured in system information for initial access

- Support indication of K_offset difference between adjacent beams with up to X bits per beam (e.g. X = 2)

- K_offset for all beams should be indicated in the SI transmitted in every beam

[Xiaomi]

Proposal 1: Beam-specific K_offset configuration during the initial access should be supported.

[Lenovo, Motorola Mobility]

Proposal 4: Support indication of beam specific K-offset.

Proposal 5: The beam specific K-offset can be indicated by an associated RS explicitly or implicitly.

[InterDigital]

Proposal-6: beam-specific K-offset indication is also supported optionally

Proposals that do no support introducing beam specific Koffset

[Samsung]

Proposal 4: Only Cell-specific K_offset in initial access is supported.

[NEC]

Proposal 1: Support cell specific K_offset value only configured in system information for use in initial access.

[FGI, Asia Pacific Telecom, III, ITRI]

Proposal 9	Postpone the discussion on beam-specific K_offset update for waiting for more progress on the association between satellite beams and BWPs.

[Panasonic]

Proposal 1: Beam specific Koffset is not necessary.

[NTT DOCOMO]

Proposal 2: K_offset in initial access is a cell-specific parameter. Beam-specific K_offset is not supported.

[ITL]

Proposal 5. Cell-specific K_offset is only supported in initial access procedure.

[Nokia, Nokia Shanghai Bell]

Proposal 2: For initial access, only cell-specific K_offset is provided.

Proposals on how to support beam specific Koffset (if supported)

[Huawei, HiSilicon]

Proposal 2: For determination of beam specific K_offset used in initial access if supported, K_offset is equal to the sum of two offset values

- The first offset value is equal to common TA signaled in system information

- The second offset is signaled in Msg2 and covers the maximum service link RTD within the beam.

Proposal 3: If a UE is provided with a beam-specific K_offset value, the beam-specific K_offset value will be used for

- The transmission timing of RAR/fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

This issue has been discussed at 4 RAN1 meetings with several rounds of email discussion and debated at GTW session. The pros and cons of supporting beam specific K_offset configured in system information and used in initial access are clear to the group – same comments have been made by both sides over the meetings.

In fact, given the views expressed at RAN1#104-e, and RAN1#104bis-e, it was recommended at the two RAN1 meetings that the proponents to offline discuss with other companies to make progress.

However, the proponents had not brought to the Moderator’s attention whether there had been such offline discussion, and if yes, what the outcome was.

Further, at RAN1#105-e, another round of views from companies were collected but convergence turned out to be not possible.

Reading through the submitted contributions, the status does not change much compared to where we have been in over the past several RAN1 meetings.

Therefore, Moderator would like to continue to encourage the proponents of beam-specific K_offset to offline convince the other camp to make progress and let Moderator know if there is a possibility for potential consensus.

5	Issue #5: MAC CE timing relationships

5.1	Background

At RAN1#106-e, several companies provide proposals on this topic:

[Spreadtrum]

Proposal 4: Time-varying value of unalignment should be supported.

[CATT]

Proposal 9: Updating the common delay, K_mac, feeder link RTT with feeder link RTT drift is supported.

Proposal 10: Updating the feeder link RTT and feeder link RTT shift with MAC CE is supported.

[FGI, Asia Pacific Telecom, III, ITRI]

Proposal 1 For K_mac used in initial access, the information of K_mac shall be carried explicitly in SIB1, if NTN-SIB is not supported.

Proposal 2 Updating K_mac after initial access shall be supported to benefit DRX operation, at least for quasi-earth-fixed cells.

Proposal 3 MAC CE shall be considered to update K_mac after initial access to prevent an ambiguous period due to an unclear action time for applying an RRC reconfiguration.

Proposal 4 For an uplink configuration, if a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on the uplink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ, where µ is the SCS configuration for the PUCCH.

[CMCC]

Proposal 7: The K_mac value provided by network can be fixed.

Note: The K_mac is used to compensate the fixed unalignment caused by the distance between NTN GW and gNB in Scenario 2-b (RU located at gateway, with gateway and gNB located away from each other).

[Panasonic]

Proposal 5: Update of K_mac should be supported. The same signaling mechanism as K_offset should be used.

[Apple]

Proposal 7: The scheduling offset Kmac is carried in system information, in the unit of milli-seconds or in the unit of slots for a given subcarrier spacing in a frequency range.

[ZTE]

Proposal 10: Update K_mac via MAC CE can be supported.

The main theme of these proposals is about whether and how to update K_mac.

- [Spreadtrum, CATT, FGI/Asia Pacific Telecom/III/ITRI, Panasonic, ZTE] propose that K_mac update is supported.

- [CMCC] propose that K_mac value provided by network can be fixed.

Besides, [FGI/Asia Pacific Telecom/III/ITRI, Apple] propose that the information of K_mac is carried in system information.

- [Apple] further proposes that the unit of K_mac is milliseconds or slots for a given subcarrier spacing in a frequency range.

5.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 5.2 (Moderator):

- The information of K_mac is carried in system information.

- For the unit of K_mac, down-select one option from below:

  - Option 1: milliseconds

  - Option 2: slots for a given subcarrier spacing in a frequency range

- How to update the information of K_mac?

  - Option 1: No update (fixed in system information)

  - Option 2: System information update

  - Option 3: UE specific RRC

  - Option 4: MAC CE

    - Should the update of K_mac via MAC CE be considered as downlink configuration update or uplink configuration update? This question is relevant because the MAC CE application timing requires K_mac for downlink configuration but not for uplink configuration.

| Company | Comments |
| --- | --- |
| Apple | We support 1). The K_mac needs to be carried in system information since it is used in determining the RAR window starting time in the initial access procedure. For 2), we slightly prefer Option 2. If the K_mac is in unit of milliseconds while the sub-carrier spacing is larger than 15 kHz, then some slot numbers cannot be indicated by K_mac value. For 3). we think the K_mac should be cell-specific as it is common to all UEs. Hence, Option 3 and Option 4 are not efficient and may lead to different UEs having different K_mac values. We are fine with the update of K_mac, considering the feeder link RTT (and the RTT between timing reference point and gateway/gNB) may be time-varying. Hence, we support Option 2. |
| FGI | Support. Q2) Option 2 (slot) Q3) Option 2/3/4 (better to align with how to update common TA) |
| Xiaomi | Generally we think the Kmac could follow the Koffset design. For 1) we support the proposal. For 2), no strong view. Follow the same principle with it in Koffset design. For 3), we support option 2 as K_mac is a cell-specific parameter. |
| ZTE | In general, we prefer to reuse the design as K_offset to simplify the changes on spec. Regarding the update of K_mac, certainly that should be supported due to the various RTT in feeder link. We are open for both Option-2 and Option 4. More specifically, W.r.t the detaield signaling, since this parameter is cell specific one, Option-2 can also be acceptable. For option 4, for this MAC CE for update/adjustment of K_mac, it can be considered like original downlink configuration is still applicable before the confirmation of a new value of K_mac. |
| Intel | - Support. Furthermore, as it is proposed in our tdoc, K_mac, K_offset and Common TA can be determined together (by using one bitfield and additional bit A) since they are related to feeder link. - Both Option 1 and Option 2 can be considered - For simplicity Option 1 can be considered. In our view if Option 1 is supported then Option 2 is also supported by default. |
| Nokia, Nokia Shanghai Bell | Aspect (1): OK Aspect (2): OK to have the options for now, but our preference would be to have option 2. Aspect (3): In principle option 1 and 2 are supposed to be jointly operating. As with any other system configuration parameter, it is possible to change it. Any such change should happen through system information update procedure. Our preference would be to have option1+2, and we have not seen any solid argumentation as to why it should be possible to update this value on a per-UE basis. K_mac was introduced because potential UL-DL timing misalignment at gNB, so it is a generic problem that affects all UEs equally. |
| NTT DOCOMO | 1) K_mac may be used for the stating RAR window, so that it is carried in system information. 3) We may consider with the discussion of the feeder link timing drift compensated by the UE under 8.4.2. |

6	Issue #6: Exceptional MAC CE timing relationships

6.1	Background

At RAN1#106-e, only two companies provide proposals on this topic:

[Huawei, HiSilicon]

Proposal 8: The MAC CE action timing for the aperiodic CSI Trigger State subselection indication and updating the spatial relation of the aperiodic SRS are for the CSI request and SRS triggering respectively.

[Panasonic]

Proposal 7: The following alternatives for the timing definition of Aperiodic CSI trigger state subselection MAC CE action timing should be discussed.

Alt1: new subselection is applied for the CSI report after the UL related MAC CE action timing

Alt2: new subselection is applied for the CSI request after the DL related MAC CE action timing

Proposal 8: Aperiodic CSI trigger state subselection MAC CE should be reflected for the transmission of the CSI report after the UL related MAC CE action timing, i.e. slot n+3Nslotsubframe,µ.

Proposal 9: AP SRS spatial relation Indication MAC CE should be reflected for the SRS transmission after the MAC CE action timing, i.e. slot n+3Nslotsubframe,µ. No specification modification would be necessary because in the current specification the applied timing is defined as “for SRS transmission”.

At RAN1#104-e, RAN1#104bis-e, RAN1#105-e, this issue was discussed. Based on the submitted contributions at RAN1#106-e, the interest in this topic is quite low.

Given (1) the low interest in this topic and (2) discussions happened at the last 3 meetings already, it does not seem helpful to spend online/email effort discussing this topic again.

Given this situation, Moderator would like to recommend the proponents to offline discuss with other companies to make progress and let Moderator know if there is a possibility for potential consensus.

7	Issue #7: On K1 range extension

7.1	Background

At RAN1#106-e, several companies provide proposals on this topic:

[Huawei, HiSilicon]

Proposal 9: K1 indication can be enhanced without impact on the size of DCI by re-interpreting PDSCH-to-HARQ_feedback timing indicator field.

[vivo]

Proposal 4: Support to extend the size of the PDSCH-to-HARQ_feedback timing indicator field up to 4.

[Samsung]

Proposal 7: Do not change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

[CATT]

Proposal 11: Extend K1 range without changing the DCI with 8 different K1 candidates, and dynamically configure the list of K1 values corresponding to the DCI size.

Proposal 12: In case of K1 indication extension for larger HARQ process, utilize existing 3 bits without changing the DCI and the highest bit of the HARQ process ID to extend the K1 range to support 16 different K1 candidates.

[NEC]

Proposal 4: The size of the PDSCH-to-HARQ_feedback timing indicator field in DCI is not changed when the range of the K1 value is extended from (0..15) to (0..31).

[CAICT]

Proposal 4: Enhance the HARQ-ACK timing indication without extending the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Proposal 5: Configure two sets of candidate K1 values. The slot index of scheduled PDSCH is used to decide one candidate K1 set.

[CMCC]

Proposal 8: If increased K1 value range in DCI is supported, Option 4 (the size of the PDSCH-to-HARQ_feedback timing indicator field is 0, 1, 2, 3, or 4 bits in non-fallback DCI 1_1/1_2) is preferred for less spec impact.

[LGE]

Proposal 6: Do not increase the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- For non-fallback DCI, increase the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31).

- For fallback DCI, consider introducing fixed or configurable offset.

[Ericsson]

Proposal 9	Increase the maximum number of entries in the higher layer parameter dl-DataToUL-ACK from 8 to 16.

Proposal 10	In non-fallback DCI 1_1/1_2, the size of the PDSCH-to-HARQ_feedback timing indicator field is 0, 1, 2, 3, or 4 bits, depending on the number of entries in the higher layer parameter dl-DataToUL-ACK (which is proposed to be increased up to 16).

[Apple]

Proposal 9: The K1 range extension does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

[ZTE]

Proposal 14: Indication of the extended K1 value via enhanced DCI should be supported.

[NTT DOCOMO]

Proposal 5: Keep the K1/K2 range for paired spectrum.

Proposal 6: A new RRC parameter (e.g., dl-DataToUL-ACK-r17) is adopted at least for DCI format 1_1.

Proposal 7: Keep the field size for K1 indication in DCI.

[Xiaomi]

Proposal 4: The bit-length of PDSCH-to-HARQ_feedback timing indicator field in the fallabck DCI is kept unchanged.

[ITL]

Proposal 6. Followings on K1 range extension issue are proposed:

- It is not supported to extend the K1 range for FDD

- DCI field range related to the K1 range extension should not be increased.

In summary:

- [Samsung, NEC, LGE, Apple, NTT DOCOMO, Xiaomi, ITL] propose not to change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- [Huawei/HiSi, vivo, CATT, CAICT, CMCC, Ericsson, ZTE] hold the view that enhancement can be considered to accommodate more flexible scheduling.

  - [Huawei/HiSi] proposes to reinterpret the PDSCH-to-HARQ_feedback timing indicator field in DCI to accommodate the increased K1 value range.

  - [vivo, CMCC, Ericsson] consider increasing the max # of RRC configured K1 values from 8 to 16, for which the size of the PDSCH-to-HARQ_feedback timing indicator field is 0, 1, 2, 3, or 4 bits in non-fallback DCI 1_1/1_2.

    - [Ericsson] points out that increasing the maximum number of K1 values that can be configured to a UE from 8 to 16 will not only benefit NTN but also offer significant deployment flexibility for terrestrial 5G NR networks.

  - [CATT] proposes to utilize existing 3 bits in DCI and the highest bit of the HARQ process ID to extend the K1 range to support 16 different K1 candidates.

  - [CAICT] propose to configure two sets of K1 values and use slot index of scheduled PDSCH to signal which K1 set is used.

  - [ZTE] proposes that indication of the extended K1 value via enhanced DCI should be supported.

Based on the proposals submitted at this RAN1#106-e, it appears that the group is still not converging on this issue.

Recall that at the end of RAN1#105-e, Moderator made the following recommendation. But the input on this is limited in the submitted contributions. In Moderator’s view, to make further progress in this topic, it is worthwhile asking companies to provide views on whether it is necessary to address the following scenario.

Companies are encouraged provide input to the next RAN1 meeting on whether it is necessary to address the following scenario.

[ZTE]:

As the example shown above, for the scheduling with larger HARQ process number, since currently the DCI field (3 bits) can only support 8 different K1 candidates, the flexibility of scheduling is quite limited. So, there is need to further enhance it, e.g., extending the value range with 4 bits or other solutions.

7.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 7.2 (Moderator):

Is it necessary to address the following scenario? And why?

[ZTE]:

As the example shown above, for the scheduling with larger HARQ process number, since currently the DCI field (3 bits) can only support 8 different K1 candidates, the flexibility of scheduling is quite limited. So, there is need to further enhance it, e.g., extending the value range with 4 bits or other solutions.

| Company | Comments |
| --- | --- |
| Apple | We do not think it is necessary to address the scenario. With the support of 8 different K1 candidates, we do not lose a lot on scheduling, which is aligned with terrestrial network. |
| FGI | Not essential in Rel-17. Rel-18 may have a new SI for HAPS, which may be a better place to handle this enhancement. |
| Xiaomi | We think it is a valid scenario as we already done some enhancements such as the HARQ process number extension in other agenda. Regarding the enhancement, we prefer not to change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI. For the fallback DCI, some implicit method can be used. For non-fallback DCI, extend the value range in dl-DataToUL-ACK can beconsidered. Although teh flexibility is lost to some extend, it is good enough make the system work for this special case. |
| ZTE | Yes, We are supportive to enhance the indication in DCI with enhanced flexiblity for scheduling. In this way, the performance for TDD scenario can be improved for both DL and UL. |
| OPPO | The current spec can already address this issue. Note that R16 defines a inapplicable K1, when the UE receives an inapplicable K1, the UE will report the HARQ-ACK in a PUCCH slot determined by an applicable K1 from the next DCI. Thus, we don’t think that we need additioal enhancement to address this issue. |
| Nokia, Nokia Shanghai Bell | No need to enhance. |
| NTT DOCOMO | We think it’s not necessary to address the scenario, since current 8 K1 values are sufficient to supprot multiplexing up to 8 HARQ-ACH bits on a single PUCCH as well as that for TN. |

8	Issue #8: Configured grant type 1 timing relationship

8.1	Background

At RAN1#106-e, three companies provide proposals on this topic:

[Huawei, HiSilicon]

Proposal 7: By extending the range of timeReferenceSFN-r16, there can be sufficient scheduling flexibility to fulfil the timing relationship for configured grant type 1.

[Samsung]

Proposal 6: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

[Panasonic]

Proposal 10: Koffset is not necessary for type 1 configured grant.

All the proposals are not in favor of introducing K_offset for configured grant type 1. Therefore, in Moderator’s view, there is no need to discuss this issue further at RAN1#106-e.

9	Issue #9: Start of RAR window

9.1	Background

At RAN1#106-e, several companies provide proposals on this topic:

[CATT]

Proposal 8: Estimate the UE-gNB RTT with the equation UE_RTT = UE-satellite RTT(service link RTT)+feeder link RTT, where feeder link RTT= common delay+K_mac+dela_T*feeder link drift.

[FGI, Asia Pacific Telecom, III, ITRI]

Proposal 5	LS to RAN2 that the estimated UE-gNB RTT provides a slot-level accuracy and may have issues to support DRX operations, e.g., offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT.

[Apple]

Proposal 8: In estimating the UE-gNB RTT for RAR window offset, set NTA to 0 and set NTA,  offset based on the signaled value or default value for a given frequency range.

[ZTE]

Proposal 11: Determining start of Msg2/MsgB RAR window considering following options:

- Option 1: Introducing an offset of UE specific RTT

  - Note: UE may need to start monitoring earlier

- Option 2: Introducing an offset of Minimum RTT

[ITL]

Proposal 7. The RAR window starting time is determined based on UE-specific RTT. Also, consider how the K_offset is used for deriving the UE-specific RTT.

Note that this issue has been largely addressed already by the agreement made in RAN1#105-e:

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac.  How to treat NTA and NTA,offset can be further discussed.

Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

From the agreement, the remaining issue is about how to treat NTA and NTA,offset.

The above agreement says that the estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. According to the agreement below:

- NTA=0 in the TA for PRACH transmission. Thus, it is natural that the NTA should be set to 0 in estimating the UE-gNB RTT.

- The effect of NTA,offset is included in the TA for PRACH transmission. Thus, it is natural that the NTA,offset is also included in estimating the UE-gNB RTT.

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

Where:

- is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- is UE self-estimated TA to pre-compensate for the service link delay.

- is network-controlled common TA, and may include any timing offset considered necessary by the network.

- with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- is a fixed offset used to calculate the timing advance.

Note-1: Definition of  is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3:  is the common timing offset X as agreed in RAN1 #103-e.

9.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 9.2 (Moderator):

In the estimate of UE-gNB RTT, which is equal to the sum of UE’s TA and K_mac, for delaying the starts of ra-ResponseWindow and msgB-ResponseWindow, the UE’s TA is equal to TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc with NTA=0.

| Company | Comments |
| --- | --- |
| Apple | Agree. Without the MAC CE TA command information, UE could by default assume N_TA= 0. |
| FGI | Support. N_TA = 0 is meaningful because the current spec only defines UE assumes N_TA = 0 for PRACH transmission. See TS 38.211, Clause 5.3.2. |
| ZTE | Fine with the meaning of the proposal, but it seems there no new information added in this proposal. |
| Intel | Agree |
| OPPO | It is still vague. Note that the NTA, UE-specific+NTA,common is timing varying. We need to clarify at what reference time UE shall determine these two values. |
| Nokia, Nokia Shanghai Bell | We are OK with this proposal. If clarification related to the reference time is needed, we could use the “time when the signals are supposed to arrive at the gNB”. |
| NTT DOCOMO | We support the proposal. |

10	Issue #10: PDCCH ordered PRACH

10.1	Background

At RAN1#106-e, several companies provide proposals on this topic:

[Huawei, HiSilicon]

Proposal 10: For PDCCH ordered PRACH, K_offset can be introduced for PRACH occasion selection.

[Spreadtrum]

Proposal 5: Introducing K_offset to enhance the timing relationship of PDCCH ordered PRACH should be supported.

[NEC]

Proposal 5: An additional timing offset for PDCCH ordered PRACH is supported.

[CAICT]

Proposal 3: Introduce a timing offset explicitly or implicitly in PDCCH ordered RACH to align the understanding of “next available mapping cycle in a SSB-RO association period after the PDCCH order” for gNB and UE, and thus to reduce the blind detection.

[OPPO]

Proposal 6: the UE behavior after receiving PDCCH order should be clarified in RAN1 meeting.

[FGI, Asia Pacific Telecom, III, ITRI]

Proposal 12 On the timing relationship of PDCCH ordered PRACH, Option 1: Introduce K_offset to enhance the timing relationship of PDCCH ordered PRACH shall be supported.

[CMCC]

Proposal 9: Introduce K_offset to enhance the timing relationship of PDCCH ordered PRACH.

[Panasonic]

Proposal 6: Cell specific Koffset should be used to determine RO for PDCCH order RACH.

[LGE]

Proposal 7: For RACH procedure triggered by PDCCH order in Rel-17 NTN, define timing offset in addition to minimum gap, NT,2+ ∆BWPSwitching+∆Delay+Tswitch.

[Intel]

Proposal 4:

- PDCCH ordered PRACH should be supported for NTN without blind detection at the gNB

  - Alt. 1: PRACH occasion is determined at the gNB based on UE-specific TA reported by the UE

  - Alt. 2: UE selects PRACH occasion based on slot offset K_offset

[ZTE]

Proposal 12: For PDCCH ordered PRACH, using K_offset for determination of RO can be considered.

[NTT DOCOMO]

Proposal 8: K_offset is introduced for PDCCH ordered PRACH.

[InterDigital]

Proposal-7: introduce K-offset for PDCCH ordered PRACH

[Nokia, Nokia Shanghai Bell]

Proposal 9: The common K_offset value shall be used also for the PDCCH ordered RACH.

It has also been brought to Moderator’s attention that [CAICT] has been kindly coordinating the offline discussion, which results in this converging status.

From the proposals, it can be seen that only [OPPO] raises another issue about PDCCH ordered PRACH, while all the other companies are fine / support to introduce K_offset for PDCCH ordered PRACH.

The issue raised by [OPPO] is about the TA value that the UE should apply upon receiving a PDCCH order.

In existing NR, UE applies NTA=0 in PRACH transmission as seen in the following specification clauses:

TS 38.133:

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus .

TS 38.211:

The starting position tstartRA of the PRACH preamble in a subframe (for ) or in a 60 kHz slot (for ) is given by

[Equations omitted]

where

-	the subframe or 60 kHz slot is assumed to start at  ;

-	a timing advance value NTA=0 shall be assumed;

In NR NTN, the Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by the following agreement. Note that UE applies NTA=0 in PRACH transmission, which is consistent with existing PDCCH ordered PRACH.

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

Where:

- is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- is UE self-estimated TA to pre-compensate for the service link delay.

- is network-controlled common TA, and may include any timing offset considered necessary by the network.

- with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- is a fixed offset used to calculate the timing advance.

Note-1: Definition of  is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3:  is the common timing offset X as agreed in RAN1 #103-e.

After discussions at several meetings, now it is common understanding that simply relying on “the next available PRACH occasion” would require blind detection at gNB side. As illustrated in the figure below, UE 1 would transmit PRACH in PRACH occasion 3 and UE 2 would transmit PRACH in PRACH occasion 4, causing some blind detection burden at the gNB side.

To resolve this issue, cell-specific K_offset may be used. In particular, for a PDCCH ordered PRACH received in downlink slot n, UE may select “the next available PRACH occasion” after uplink slot “n + K_offset”. As illustrated in the figure below, both UE1 and UE2 would transmit PRACH in PRACH occasion 5, removing the blind detection burden at the gNB side.

10.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 10.2 (Moderator):

For random access procedure initiated by a PDCCH order received in downlink slot n, UE determines the next available PRACH occasion after uplink slot n+Koffset to transmit the ordered PRACH, where Koffset is given by the value signaled in system information.

Note: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc, where NTA=0 is assumed for PDCCH ordered PRACH.

| Company | Comments |
| --- | --- |
| FGI | Support. This is beneficial for UE’s processing time an gNB’s decoding complexity. Agree N_TA = 0 for the start the RAR window. Note: both cell-specific and UE-specific K_offset shall be feasible. |
| ZTE | Support with modification as following deletion, because K_offset can also be an updated K_offset. where Koffset is given by the value signaled in system information. |
| Intel | Support |
| OPPO | What is the reason that the UE shall apply this K offset? There is not a specific RO resource for the UE to transmit, the UE will select a suitable RO accoridng to whether or not it is ready to transmit. Thus, the motivation of adding K offset is not justfied. |
| Nokia, Nokia Shanghai Bell | We are OK with this proposal. |
| OPPO2 | In today’s GTW, due to the limited time, we probably didn’t make our view clear enough. Since the intention is to resolve the gNB complexity issue (blind detection), we should also consider the following cases. - After UE receives a PDCCH order and it implies that the UE’s UL synchronization is lost, whether the UE shall re-read the SIB for obtaining an updated ephemeris or common TA before transmitting a PRACH. - After UE receives a PDCCH order, if the ephemeris timer is expired, the UE shall read the SIB to update the ephemeris, shall the gNB expects that the UE will not transmit the PRACH before the upcoming SIB to reduce the gNB complexity issue? |
| NTT DOCOMO | We support the proposal. |

11	Issue #11: SFI timing relationship

11.1	Background

At RAN1#106-e, there is only one proposal on this topic:

[Xiaomi]:

Proposal 5: The enhancement on the SFI timing relationship is not supported

Therefore, in Moderator’s view, there is no need to discuss this issue further at RAN1#106-e.

12	Issue #12: Timing of preamble retransmission

12.1	Background

At RAN1#106-e, there is only one proposal on this topic:

[ZTE]

Proposal 13: No timing relationship enhancement is needed for preamble retransmission.

Therefore, in Moderator’s view, there is no need to discuss this issue further at RAN1#106-e.

13	Issue #13: Beam failure recovery timing relationship

13.1	Background

At RAN1#106-e, several companies provide proposals on this topic:

[Huawei, HiSilicon]

Proposal 11: The timing relationship for beam failure recovery needs to be enhanced with timing offset of K_mac, i.e. a UE monitors PDCCH from slot n+4+K_mac within a window configured by BeamFailureRecoveryConfig.

[vivo]

Proposal 5: Support to introduce the K_mac for enhance the timing relationship of PDCCH monitor in beam failure recovery.

Proposal 6: Not support to enhance the timing enhancement of beam reset for PUCCH in beam failure recovery.

[Apple]

Proposal 10: RAN1 to enhance the timing relationship on the beam failure recovery response window offset, by introducing Kmac.

Proposal 11: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

[NTT DOCOMO]

Proposal 9: Extension of BFR timer is not necessary for NTN.

[Lenovo, Motorola Mobility]

Proposal 6: The timing relationship between PRACH transmission in beam failure recovery and start of PDCCH monitoring is 4+K_mac.

Proposal 7: K_offset is added to the delay between PDCCH reception and PUCCH/PUSCH transmission in beam failure revoery. Cell-specific K_offset can be used.

There are two main discussion points under this topic, which are described below respectively.

13.1.1	Delay between PRACH and start of PDCCH monitoring

The delay between PRACH transmission and start of PDCCH monitoring is described in TS 38.213:

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index  provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot  within a window configured by BeamFailureRecoveryConfig.

The “slot n” is about PRACH transmission and hence naturally refers to uplink slot. The slot “n+4” is about PDCCH reception and hence naturally refers to downlink slot.

Because the downlink and uplink frame timing may not be aligned at the gNB side, there may be a causality issue if the slot “n+4” is reused. For example, as illustrated in the figure below, the gNB may not have received PRACH in slot “n+4” and thus cannot send a PDCCH response.

Therefore, as several companies have correctly proposed, the slot “n+4” needs to be extended to the slot “n + K_mac + 4”, as illustrated by the slot marked by green color below.

13.1.2	Delay between PDCCH reception and application of new PUCCH beam

The delay between PDCCH reception and application of new PUCCH beam is described in TS 38.213. Below is one example clause:

For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in Clause 7.2.1 with , , and

Note that in this case, there is no explicit slot numbering, which is unlike the previous case on the delay between PRACH and start of PDCCH monitoring.

Therefore, there may be two interpretations on the “28 symbols”, as illustrated in the figure below.

- Interpretation 1: “28 symbols” is the absolute time between the time UE receives PDCCH and the time UE applies new PUCCH beam

  - In this case, K_offset enhancement is not needed

- Interpretation 2: for a PDCCH reception in downlink slot n, “28 symbols” is the time between UE’s uplink slot n and the time UE applies new PUCCH beam

  - In this case, K_offset enhancement is needed

In Moderator’s view, Interpretation 1 is more reasonable, whereas Interpretation 2 implicitly adds slot numbering which is not present in the existing specification clause.

13.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 13.2 (Moderator):

On beam failure recovery procedure:

- Do you agree that the slot “n + 4” need to be revised to the slot “n + K_mac + 4” for the delay between PRACH and start of PDCCH monitoring? And why?

- On the “28 symbols” delay between PDCCH reception and application of new PUCCH beam, which of the following is your interpretation? And why?

  - Interpretation 1: “28 symbols” is the absolute time between the time UE receives PDCCH and the time UE applies new PUCCH beam

    - In this case, K_offset enhancement is not needed

  - Interpretation 2: for a PDCCH reception in downlink slot n, “28 symbols” is the time between UE’s uplink slot n and the time UE applies new PUCCH beam

    - In this case, K_offset enhancement is needed

| Company | Comments |
| --- | --- |
| Apple | We agree with the first bullet. Considering the case where downlink and uplink frame timings are not aligned at gNB. The beam failure recovery request has to reach gNB before it can send back beam failure recovery response. Hence, the additional Kmac needs to be added to enhance the time relationship on the beam failure recovery response window offset. This is similar to RAR window starting time, where Kmac is used to capture the RTT between timing reference point and gNB. For the second bullet, we think in interpretation 1, “28 symbols” is the absolute time between UE receives PDCCH and the time UE applies new PUCCH beam, gNB does not know the timing of receiving using the new beam. This is because the UE-gNB RTT is unknown at gNB side, as shown in the following figure. Here, to align the timing between gNB and UE, we think the introduction of Koffset is needed. Instead of interpretation 2 of the existing specifications, we think the timing alignment between gNB and UE could be addressed in terms of interpretation 2. Regarding the comment that no PUCCH transmissions during the time when UE sends beam failure recovery request and UE receives beam failure recovery response, we think UE may still have PUCCH transmissions, e.g., periodic CSI reporting, etc. |
| FGI | Interpretation 1 (K_offset enhancement is not needed) seems reasonable. |
| ZTE | For Q1, Yes, in this case, K_mac should be added. Basically the gNB DL timeline is advanced with value of K_mac, so gNB needs to delay the transmission of PDCCH to compensate it, e.g., transmit at slot n+4+K_mac after receiving PRACH at gNB UL slot n. For Q2, we prefer interpretation 1. According to current design of beam failure recovery, no additional transmission will occur within duration by using the old beam. Then, there will be no ambiguity issue. Moreover, from implementation perspective, the changes of UE’s beam has less impact on the performance in NTN since the footprint of Tx beam is pretty large. |
| Intel | - Agree We agree with interpretation 1. Regarding the issue described in the above comment from Apple, we agree that there is some uncertainty at the gNB side, so solution which is based on K_offset can be considered to solve it (so the gNB knows the exact slot for reception with new beam). |
| Nokia, Nokia Shanghai Bell | On aspect (1): We agree – the K_mac is introduced to compensate/offset the UL and DL slots. This should also apply for the beam failure recovery procedure. On aspect (2), we would have the understanding according to interpretation 2, since we need to accommodate the timing offsets for the long propagation delays as experienced in the NTN scenarios. |

14	Issue #14: UE reporting of information about the UE specific TA pre-compensation

14.1	Background

In RAN2 LS on LS on TA pre-compensation (R1-2104230), one of the requests is about UE reporting of information about the UE specific TA pre-compensation:

3) RAN2 respectfully requests RAN1 to provide input on the exact content and frequency of UE reporting of information about the UE specific TA pre-compensation at least for uplink scheduling adaptation.

Note that RAN2 made the following agreement at RAN2#114-e:

Agreement:

- If enabled by the network, the UE reports information about UE specific TA pre-compensation at the random access procedure (MSGA/MSG3 or MSG5) using a MAC CE. Actual content is FFS and also depends on further RAN1 input (we can revise this whole agreement if RAN1 come to a different conclusion in terms of what needs to be conveyed to the NW)

At RAN1#105-e, RAN1 held initial discussions, leading to the following Moderator recommendation:

Moderator recommendation on TA reporting:

Companies are encouraged to provide input on this issue at the next RAN1 meeting, taking into account the questions asked by RAN2 LS:

- At least for uplink scheduling adaptation, what is the exact content of UE reporting of information about the UE specific TA pre-compensation?

  - Option 1: UE specific TA: [Apple, MediaTek, CATT, Lenovo/MM(?)]

  - Option 2: Full TA: [ZTE, Lenovo/MM(?)]

  - Option 3: UE location: [Panasonic]

  - Option 4: Difference between UE-specific K_offset and cell-specific K_offset: [CMCC]

  - Other option(s)?

- At least for uplink scheduling adaptation, how frequent is the UE reporting of information about the UE specific TA pre-compensation?

  - Option 1: Event triggered: [Apple, Panasonic, LG, Qualcomm, Xiaomi, CMCC, CATT]

  - Option 2: Network request: [ZTE, Qualcomm]

  - Option 3: Periodic: [Xiaomi, Lenovo/MM]

  - Other option(s)?

At RAN1#106-e, several companies provide proposals on this topic:

[Huawei, HiSilicon]

Proposal 5: UE-specific TA related information is reported in Msg3 or MsgA for update of K_offset.

Proposal 6: Differential indication with granularity of one slot are adopted for both UE-specific K_offset update and TA report to reduce the signaling overhead.

[Samsung]

Proposal 3: UE’s estimated TA value is reported to gNB by MAC CE.

[CATT]

Proposal 5: On UE_specific TA reporting, both event triggered based reporting and periodic reporting can be supported for different scenarios.

Proposal 6: Using RRC signaling to report TA can be supported.

Proposal 7: Reporting TA change between current TA and previous TA is preferred.

[MediaTek]

Proposal 1: The UE-specific TA report is reported by UE when its UE-specific TA has changed by no more than half subframe duration.

Proposal 2: Support UE-specific TA report by the UE via MAC CE.

Proposal 3: Support Option 1 Event triggered (i.e. autonomous decision by the UE to send the TA report) and Option 2: Network request for UE-specific TA report.

[OPPO]

Proposal 1: Support UE reporting the service link RTT to the network during initial access.

Proposal 3: Support network requesting UE to report service link RTT in connected phase.

[FGI, Asia Pacific Telecom, III, ITRI]

Proposal 10	TA reporting at least for Option 1: UE-specific TA and for Option 2: Full TA shall include UL timing adjustment contributed by the gradual timing adjustment requirements.

Proposal 11	TA reporting shall at least support Option 2: NW request as a baseline, and the support of Option 1: event-triggered shall be considered if further enhancement is needed.

[Qualcomm]

Proposal 2: Support UE specific TA report by MAC-CE

- FFS: details of signaling

Proposal 3: Support periodic TA report.

[CMCC]

Proposal 3: Regarding TA report, option 5 can be supported.

- Option 5: Difference between the last applied K_offset (e.g., cell-specific K_offset or UE-specific K_offset indicated by the network) and one new K_offset suggested by UE.

Proposal 4: TA report is supported with at least one of following information:

- Option 5: Difference between the last applied K_offset (e.g., cell-specific K_offset or UE-specific K_offset indicated by the network) and one new K_offset suggested by UE.

- Option 4: Difference between UE-specific K_offset and cell-specific K_offset.

- Option 3: UE location.

[Panasonic]

Proposal 4: In order to determine UE specific Koffset, UE location report should be utilized if available. If it is not available, UE report of  RTT value with slot level granularity should be specified.

[Intel]

Proposal 5: Consider UE-specific TA reporting from the UE to the gNB with slot granularity

- Reporting of UE-specific TA should be controlled by the gNB

[Apple]

Proposal 3: Support UE reports the preferred differentiated UE specific Koffset for the purpose of uplink scheduling adaptation.

Proposal 4: Support at least event-triggered UE reporting for the purpose of uplink scheduling adaptation.

[ZTE]

Proposal-4: Full TA reporting is preferred to enable the updates of K_offset.

Proposal-5: For the reporting mechanism, Option 3 is preferred with simple periodic reporting.

Proposal-6: For 2-step RACH, a refined value of K_offset can be directly configured for a UE if TA is reported via Msg-A PUSCH.

[InterDigital]

Proposal-3: a UE report UE-specific TA value for K-offset update in RRC connected status if requested by the network

[Fraunhofer IIS, Fraunhofer HHI]

Proposal 6: For updating Koffset in a UE specific manner, UE reports the differential value of UE specific TA.

Proposal 7: RAN1 to discuss the granularity of the differential UE specific TA report.

Proposal 8: RAN1 to support both event triggered and network request  UE TA report.

[Nokia, Nokia Shanghai Bell]

Proposal 7: RAN 1 to consider alternatives to minimize the TA reporting.

Proposal 8: RAN 1 to consider both trigger event on UE side and network request as mechanisms to trigger UE coarse reporting of TA.

About the exact content of UE reporting of information about the UE specific TA pre-compensation, the table below presents a summary of the proposed design options and the corresponding proponents.

| At least for uplink scheduling adaptation, what is the exact content of UE reporting of information about the UE specific TA pre-compensation? | Proponent(s) |
| --- | --- |
| Option 1: UE specific TA | [6] sources: [OPPO, FGI/Asia Pacific Telecom/III/ITRI, Qualcomm, Panasonic, Intel, InterDigital] |
| Option 2: Full TA | [2] sources: [FGI/Asia Pacific Telecom/III/ITRI, ZTE] |
| Option 3: UE location | [2] sources: [CMCC, Panasonic] |
| Option 4: Difference between UE-specific K_offset and cell-specific K_offset | [1] sources: [CMCC] |
| Option 5: Difference between the last applied K_offset (e.g., cell-specific K_offset or UE-specific K_offset indicated by the network) and one new K_offset suggested by UE | [2] sources: [CMCC, Apple] |
| Option 6: Differential indication | [3] sources: [Huawei/HiSilicon, CATT, Fraunhofer IIS/Fraunhofer HHI] |

About the frequency of the UE reporting of the UE specific TA pre-compensation, the table below presents a summary of the proposed design options and the corresponding proponents.

| At least for uplink scheduling adaptation, how frequent is the UE reporting of information about the UE specific TA pre-compensation? | Proponent(s) |
| --- | --- |
| Option 1: Event triggered | [6] sources: [CATT, MediaTek, FGI/Asia Pacific Telecom/III/ITRI, Apple, Fraunhofer IIS/Fraunhofer HHI, Nokia/Nokia Shanghai Bell] |
| Option 2: Network request | [6] sources: [MediaTek, OPPO, FGI/Asia Pacific Telecom/III/ITRI, InterDigital, Fraunhofer IIS/Fraunhofer HHI, Nokia/Nokia Shanghai Bell] |
| Option 3: Periodic | [3] sources: [CATT, Qualcomm, ZTE] |

Several observations can be made in order.

- For uplink scheduling adaptation, network needs to configure UE specific K_offset

- UE specific K_offset can be determined if UE reports its TA information

- Whether UE reports full TA, UE specific TA, difference between K_offset values, or difference between TA values is detailed signaling design. It can be left to RAN2 to decide. To reply to RAN2’s LS, it is sufficient to provide one example that, e.g., receives the majority support

- What matters more from RAN1’s perspective is the granularity of the reported TA information. Several companies propose to have slot-level granularity

- On the frequency of reporting, Option 1: Event triggered, and Option 2: Network request receive more support than Option 3: Periodic

- A different alternative is to report UE location, which can reduce the frequency required for reporting but may lead to some privacy concern (a topic beyond RAN1 scope)

Given that RAN2 has been discussing this topic for relatively long already and has made good progress, it would be preferred to let RAN2 to converge further, rather than RAN1 start to duplicate the discussion which might interfere with RAN2 discussion.

Therefore, Moderator suggests that we focus on answering RAN2’s questions and recommends that we make a conclusion as follows, which can then be used as a basis to draft a reply LS to RAN2.

Proposed conclusion:

At least for uplink scheduling adaptation, the content of UE reporting of information about the UE specific TA pre-compensation should enable the network to configure UE-specific K_offset. Such information can be based on, e.g., UE specific TA, and has slot-level granularity.

Regarding the frequency of the UE reporting of information about the UE specific TA pre-compensation, it can be, e.g., event triggered or upon network request. The triggering event may be, e.g., the change of UE specific TA exceeds

Detailed signaling design is up to RAN2.

14.2	Company views

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 14.2 (Moderator):

Make the following as a conclusion for answering the RAN2’s question on the UE reporting of information about the UE specific TA pre-compensation:

At least for uplink scheduling adaptation, the content of UE reporting of information about the UE specific TA pre-compensation should enable the network to configure UE-specific K_offset. Such information can be based on, e.g., UE specific TA, and has slot-level granularity.

Regarding the frequency of the UE reporting of information about the UE specific TA pre-compensation, it can be, e.g., event triggered or upon network request. The triggering event may be, e.g., the change of UE specific TA exceeding a configured threshold.

Detailed signaling design is up to RAN2.

| Company | Comments |
| --- | --- |
| Apple | We are supportive to the proposal in general. The only comment we have is to remove the example of (e.g.,UE specific TA), since the detailed signaling design is up to RAN2, and we do not want to impact the RAN2 direction of the signaling design. |
| FGI | Support. RAN1 only needs to define 1) content, 2) granularity, and 3) usage. |
| Xiaomi | We support the proposal in general. |
| ZTE | In our view, we need to conclude on the detailed solution firstly before provide the general descriptio with certain examples. It will mislead the discussion in other WGs. W.r.t the technica part, we share the views that the applied TA by UE (i.e., full TA including both feederlink and service link) for UL transmission should be reported to UE and from signaling overhead perspective, the granularity can be enlarged. For the frequency on reporting, the basic principle from RAN1 perspective is tha t such report should be done to enable the correct scheduling by gNB. In this wya, either the network request or periodic report can be considered with clear benefits on the signalling overhead and specific efforts. W.r.t the event triggered based solution, we have strong concerns on this approach since in NTN case, the updates of such value may be needed for a group of UE simulataneously. In this case, the unexpected load on signalling will occur and the system performance will degraded. |
| Intel | Support |
| OPPO | We are fine with the initial proposal. |
| Nokia, Nokia Shanghai Bell | We are OK with this proposal for conclusion. |

References

TR 38.821, Solutions for NR to support non-terrestrial networks

RP-211557, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #92-e, June 2021.

R1-2106325, “Feature lead summary#5 on timing relationship enhancements,” Moderator (Ericsson), RAN1#105-e, May 2021.

R1-2106482, Discussion on timing relationship enhancements for NTN, Huawei, HiSilicon

R1-2106591, Discussion on timing relationship enhancements for NR-NTN, vivo

R1-2106701, Discussion on timing relationship enhancements for NTN, Spreadtrum Communications

R1-2106723, Timing relationship enhancements for NTN, Zhejiang Lab

R1-2106754, Discussion on timing relationship enhancement for NTN, Baicells

R1-2106805, Calculation and application of timing relationship offsets, Sony

R1-2106884, Timing relationship enhancements for NTN, Samsung

R1-2106967, Further discussion on timing relationship enhancements for NTN, CATT

R1-2107014, Discussion on timing relationship enhancements for NTN, NEC

R1-2107064, Timing relationship enhancements for NR-NTN, MediaTek Inc.

R1-2107167, Timing relationship enhancements to support NTN, CAICT

R1-2107194, Discussion on timing relationship enhancements for NTN, Hyundai Motors

R1-2107243, Discusson on timing relationship enhancement, OPPO

R1-2107287, Timing relationship enhancements in NTN, FGI, Asia Pacific Telecom, III, ITRI

R1-2107341, Enhancements on Timing Relationship for NTN, Qualcomm Incorporated

R1-2107399, Discussion on timing relationship enhancements for NTN, CMCC

R1-2107468, Timing relationship for NTN, Panasonic Corporation

R1-2107538, Discussions on timing relationship enhancements in NTN, LG Electronics

R1-2107588, On timing relationship enhancements for NTN, Intel Corporation

R1-2107636, On timing relationship enhancements for NTN, Ericsson

R1-2107736, On Timing Relationship Enhancements for NR NTN, Apple

R1-2107775, Discussion on timing relationship for NR-NTN, ZTE

R1-2107855, Discussion on timing relationship enhancements for NTN, NTT DOCOMO, INC.

R1-2107918, Discussion on the timing relationship enhancement for NTN, Xiaomi

R1-2107944, Discussion on NTN timing relationship, Lenovo, Motorola Mobility

R1-2107992, Timing relationship enhancements for NTN, ITL

R1-2108031, Timing relationship enhancement for NTN, InterDigital, Inc.

R1-2108070, Discussion on Timing Relationship Enhancements for NTN, Fraunhofer IIS, Fraunhofer HHI

R1-2108090, Time relation aspects for NR over NTN, Nokia, Nokia Shanghai Bell

Appendix I: RAN1 agreements on timing relationship

RAN1#102-e:

Agreement:

- Introduce K_offset to enhance the following timing relationships:

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH).

  - The transmission timing of RAR grant scheduled PUSCH.

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

- Note: Additional timing relationships that require K_offset of the same or different values can be further identified.

Agreement:

For K_offset used in initial access, the information of K_offset is carried in system information.

- FFS implicit and/or explicit signaling of K_offset in system information.

- FFS a cell specific K_offset value used in all beams of a cell and/or each beam in a cell uses a beam-specific K_offset value.

- FFS whether/how to update K_offset after initial access.

RAN1#103-e:

Agreement:

Introduce K_offset (may or may not be the same as the K_offset value in other timing relationships) to enhance the timing relationship of HARQ-ACK on PUCCH to MsgB.

Agreement:

- For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported.

- FFS: Beam specific K_offset configured in system information and used in initial access.

Working Assumption:

K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH.

Conclusion:

The agreement made at RAN1#102-e about introducing K_offset in the transmission timing of RAR grant scheduled PUSCH is also applicable to fallbackRAR scheduled PUSCH.

Agreement:

Denote by K_mac a scheduling offset other than K_offset:

- If downlink and uplink frame timing are aligned at gNB:

  - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

  - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

- If downlink and uplink frame timing are not aligned at gNB:

  - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is needed.

  - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

- Note: This does not preclude identifying exceptional MAC CE timing relationship(s) that may or may not require K_mac.

RAN1#104-e:

Agreement:

Confirm the following working assumption:

K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH.

Agreement:

Update of K_offset after initial access is supported

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

RAN1#104bis-e:

Agreement:

For updating K_offset after initial access, at least one of the following options is supported:

- Option 1: RRC reconfiguration

- Option 2: MAC CE

FFS: Other options

Agreement:

- For determination of cell-specific K_offset in system information, down-select one option from below:

  - Option 1: Signal one offset value for K_offset

    - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point

  - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values

    - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Agreement:

Confirm the following working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

Agreement:

When UE is not provided with K_offset value other than the one signaled in system information, the K_offset value signaled in system information is used for all timing relationships that require K_offset enhancement.

Agreement:

UE can be provided by network with a K_mac value.

- When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

RAN1#105-e:

Agreement:

If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH.

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac.  How to treat NTA and NTA,offset can be further discussed.

Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

Agreement:

The K_offset value signaled in system information is always used for

- The transmission timing of RAR / fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

FFS: how to treat additional transmission timings related to fallback DCI formats

FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported