3GPP TSG RAN WG1 #106-e			    R1-210xxxx

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.8.2

Source: 	Moderator (Qualcomm)

Title: 	FL summary of PUCCH coverage enhancement

Document for:	Discussion/Decision

# Introduction

In this document, a summary of companies’ proposals for PUCCH coverage enhancement is provided.

# Dynamic PUCCH repetition factor indication

## Scope of dynamic PUCCH repetition factor indication

Regarding whether dynamic PUCCH repetition factor indication should be applied to semi-static PUCCH, there are diverged views based on submitted contribution from companies.

Whether to support dynamic PUCCH repetition factor indication to P/SP CSI or SR?

- Support: QC, ETRI, Ericsson

  - Rationale: The motivation for PUCCH enhancement from the study item phase was for CSI (which is the coverage bottleneck), so dynamic repetition for PUCCH should support at least CSI.

- Not support: HW/HiSi, ZTE, CATT, Panasonic, OPPO

  - Rationale: As the P/SP CSI payload size is static, gNB can set static repetition factor accordingly.

Whether to support dynamic PUCCH repetition factor indication to HARQ-ACK for SPS PDSCH?

- Support: ZTE, QC, ETRI

  - Rationale: the same PUCCH repetition faction indication mechanism as for dynamic HARQ-ACK can be reused

- Not support: CATT

  - Rationale: for semi-static UCI, any repetition parameters of the corresponding PUCCH resource can only be semi-statically configured by RRC.

Companies are welcome to provide comments/views to this topic.

| Company name | Comments |
| --- | --- |

## Dynamic PUCCH repetition factor indication scheme

### Confirm the working assumption

In RAN1 #105e, the following working assumption was agreed.

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

- Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor.

  - FFS: RRC signaling enhancement details

Based on the proposals in contributions submitted by companies, majority companies want to confirm the working assumption. However, R1-2106905 raised a concern on the working assumption. The concern is that, given a PRI of 3 bits which can indicate 8 resources, if each resource is associated with 4 numbers of repetitions, that effectively only leave a residual 1 PRI bit for PUCCH resource indication.

FL Question 1: Do you view the issue raise in R1-2106905 as a critical issue? What is the solution to solve the issue?

Companies are welcome to provide answer the question in the table below.

| Company name | Answer/Comment |
| --- | --- |

### Applicability of dynamic PUCCH repetition factor indication

One discussion point on dynamic PUCCH repetition factor indication is whether apply this feature to all PUCCH format. Based on proposals from contributions submitted by companies, the views are the following.

- Dynamic PUCCH repetition factor indication only applies to PUCCH format 1, 3, 4:

  - Supported by: Nokia, CATT, Oppo

- Dynamic PUCCH repetition factor indication only applies to all PUCCH formats

  - Supported by: QC, Ericsson, Panasonic

Based on the agreements made in RAN1 #105e under IIoT/URLLC WI, dynamic PUCCH repetition is applied to short PUCCH format 0 and 2.

Agreement:

- Support sub-slot-based PUCCH repetition for HARQ-ACK based on the Rel.16 PUCCH procedure for slot-based PUCCH applied to sub-slot-based PUCCH.

  - Note: The intention is to take the Rel.16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary.

  - FFS whether or not there is any restriction for the applicability of sub-slot-based PUCCH repetition for HARQ-ACK

  - Dynamic repetition indication is supported also for sub-slot-based PUCCH in Rel.17.

    - FFS: If the method to be specified in CovEnh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed.

Agreement:

  - Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot-based PUCCH repetition.

    - FFS: Support for slot-based PUCCH repetition

Apparently, dynamic repetition factor indication should also be applied to long PUCCH format 1,3,4, as this is the intention anyway for this WI. Furthermore, based on companies’ input in the contributions, majority companies support dynamic repetition factor indication for long PUCCH format 1,3,4.

With the above, FL has the following proposal.

FL Proposal 1: Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4.

Companies are welcome to provide comments to the above FL proposal in the following table.

| Company name | Comment |
| --- | --- |

### Interaction between dynamic repetition factor indication and semi-static repetition factor indication

When both the legacy (semi-static) PUCCH repetition factor nrofSlots and the new (dynamic) PUCCH repetition factor are configured to a UE, one discussion point is how does UE handle the interaction between these two features.

A few companies submitted proposals to address this issue. The proposals are listed as below.

R1-2106658: For interaction between the RRC configured repetition factor nrofSlots and the dynamically indicated repetition factor for PUCCH, the following procedure applies:

- For a PUCCH format 1, 3 and 4 with associated scheduling DCI, the dynamically indicated PUCCH repetition factor (if configured) applies and overrides the RRC configured repetition factor nrofSlots.

- For a PUCCH format 1, 3 and 4 without associated scheduling DCI, the RRC configured repetition factor nrofSlots applies.

R1-2107142: If both static PUCCH repetition factor and dynamic PUCCH repetition factor are configured to UE, UE should apply dynamic PUCCH repetition factor which is similar to dynamic PUSCH repetition factor overwriting static PUSCH repetition factor.

R1-2107551: It is necessary to discuss the relationship between the PUCCH repetition number indicated dynamically and the PUCCH repetition number indicated in the PUCCH-FormatConfig.

R1-2107802: the number of repetitions NPUCCHrepeat for a PUCCH transmission corresponding to a PUCCH resource and a PUCCH format is determined as:

    - if the new repetition parameter is configured on the PUCCH resource, NPUCCHrepeat is equal to the new repetition parameter;

    - elseif nrofSlots is configured on the PUCCH format, NPUCCHrepeat is equal to nrofSlots;

    - otherwise, NPUCCHrepeat=1.

The proposals are essentially aligned, despite the wording difference among them. Based on the above proposals, A FL proposal is made as below.

FL Proposal 2: the number of repetitions NPUCCHrepeat for a PUCCH transmission corresponding to a PUCCH resource and a PUCCH format is determined as:

    - if a new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication is configured on the PUCCH resource, NPUCCHrepeat is equal to the new repetition parameter;

    - elseif nrofSlots is configured on the PUCCH format, NPUCCHrepeat is equal to nrofSlots;

    - otherwise, NPUCCHrepeat=1.

Companies are welcome to provide comments to the above FL proposal in the following table.

| Company name | Comment |
| --- | --- |

### Number of repetitions allowed

Regarding the number of repetitions allowed to be configured with this feature of dynamic PUCCH repetition indication, a few proposals are proposed, and they are summarized as below.

- R1-2106905: Support Up to 32 repetitions

- R1-2107802, R1-2107259: Support 1, 2, 3, 8 repetitions

- R1-2107259: Support 16 repetitions

- R1-2107653: Support at least up to 8 repetitions

The views are still diverged on this issue. Companies are welcome to provide comments to the issue in the following table.

| Company name | Comment |
| --- | --- |

### Details of repetition factor indication configuration and interpretation

Regarding the details of how to configure the repetition factor and how to interpret the repetition factor. Two companies provided two proposals.

R1-2106498: Single PUCCH resource can be referred by multiple PUCCH resource sets with different PUCCH repetition factor.

R1-2107362: Support different interpretations of a single dynamic indication of PUCCH repetition factor, for different PUCCH formats and UCI sizes, or different PUCCH resource sets.

Companies are welcome to provide comments to the two proposals in the following table.

| Company name | Comment |
| --- | --- |

## Other proposals

There are a few other proposals mentioned in submitted contributions to this agenda. FL’s initial assessment is that the discussion of those proposals can be deprioritized, comparing to proposals in Section 2.1 and 2.2.

R1-2106991: It is not necessary to introduce new candidate values for dynamic PUCCH repetition.

R1-2107756: Specify conditions under which a PUCCH with dynamic indication of repetition number may overlap with another PUCCH repetitions without dynamic indication of repetitions.

R1-2107756: Support the working assumption as an agreement, and combine existing mechanisms based on PRI, NCCE and nCCE,0 to indicate the PUCCH resource with repetition factor within a PUCCH resource set up to 64 resources

- FFS: indication of repetition factor for a FM1 PUCCH provided by pucch-ResourceCommon

R1-2107562: Periodic or semi-persistent CSI can use the same basic mechanism as dynamically indicated PUCCH resource

- DCI can update a PUCCH resource used for P/SP-CSI reporting

- P/SP-CSI reporting resources can be selected by DL or UL grants

- PUCCH resources used in periodic and/or semi-persistent CSI reporting can be indicated via DCI in at least a DL grant

  - FFS: if UL grants can also be used for this purpose

- The dynamic PUCCH repetition mechanism should be applied to all PUCCH formats and all UCI types including A-CSI.

R1-2107551: The following methods to configure PUCCH repetition for the UE without dedicated PUCCH resource configuration should be studied.

- PUCCH repetition is indicated by using repetition number of PUSCH.

- PUCCH repetition is indicated by PRI and/or system information.

- Introduce a PUCCH resource set with repetition number.

R1-2107362: Support enhancing RRC signaling to allow dynamic indication of frequency hopping for PUCCH repetition via indication of PUCCH resource.

R1-2107362: Also using other properties of PDCCH (e.g. PDCCH aggregation level), in addition to PRI and starting CCE index, to indicate the PUCCH resource.cation of PUCCH resource.

R1-2107362: Support implicit indication of PUCCH repetition factor based on beam selection.

Companies are welcome to provide comments to the above proposals in the following table.

| Company name | Comment |
| --- | --- |

# DMRS bundling across PUCCH repetitions

The second objective of this agenda item is to “specify mechanism to support DMRS bundling across PUCCH repetitions.” Under this objective, a few topics are addressed in companies’ contributions. The topics are summarized as below.

## Use cases

In the LS R1-2104119 sent to RAN4, the following use cases were agreed.

For PUCCH repetitions, the following use cases are considered in RAN1. Among the following cases, RAN1 suggest RAN4 to prioritize the study on use case 3, 4a, 4b, and 5b for PUCCH repetitions.

   Use case 1: back-to-back PUCCH repetitions within one slot.

   Use case 2: non-back-to-back PUCCH repetitions within one slot.

‐   Use case 2a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use case 2b: other uplink transmissions in the middle of two PUCCH repetitions

   Use case 3: back-to-back PUCCH repetitions across consecutive slots.

   Use case 4: non-back-to-back PUCCH repetitions across consecutive slots.

‐   Use 4a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use 4b: other uplink transmissions in the middle of two PUCCH repetitions

   Use case 5: PUCCH repetitions across non-consecutive slots.

‐   Use case 5a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use case 5b: other uplink transmissions in the middle of two PUCCH repetitions

Note: RAN1 assumes “back-to-back PUCCH repetitions” has zero gap in-between adjacent PUCCH repetitions.

Note: intervening “other uplink transmissions” can be either on the same component carrier or a different component carrier.

In the contributions submitted to this meeting, there are proposals to further prioritize several use cases for PUCCH repetitions.

R1-2106614: Regarding the use cases for PUCCH DMRS bundling, we have following proposal:

- Use cases 3 and 4a should be supported

- Use cases 4b and 5b should be deprioritized.

R1-2106742: Decide whether to support Use case 4b/5a/5b for PUCCH repetitions depending on RAN4 further decision.

R1-2106905: Support use cases 3,4,5 for DM-RS bundling for PUCCH repetitions.

R1-2107126: At least support Use case 3 and Use case 4a for PUCCH repetitions with DMRS bundling.

R1-2107362 : Support the following use cases:

- Use case 3: back-to-back PUCCH repetitions across consecutive slots.

- Use case 4: non-back-to-back PUCCH repetitions across consecutive slots.

‐   Use 4a: no uplink transmission in the middle of two PUCCH repetitions

Based on the above proposal, it seems at least reasonable to prioritize to study case 3 and 4a in RAN.

FL Proposal 3: For DMRS bundling for PUCCH repetitions, RAN1 at least prioritize to study use cases 3 and 4a.

Companies are welcome to provide comments to the above FL proposal in the following table.

| Company name | Comment |
| --- | --- |

## DMRS bundling scheme and signalling

In RAN1 104-e, the following agreements were made.

Agreements:

Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

- FFS: the configuration is per UE or per PUCCH resource.

- FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling

- FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

In RAN1 104-e, the following agreements were made.

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements.

- Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

Based on the above agreement. There are a few open issues for further study.

### Time domain window design details

There are a few key questions RAN1 needs to answer to complete the design of time domain window for DMRS bundling for PUCCH repetition.

Question 1: Allow only one single window or allow one or more windows across all repetitions of a PUCCH?

Question 2: If multiple windows are allowed, allow only a common window duration for all windows or allow different window durations for different windows?

Question 3: How to determine the duration of a window?

Question 4: How to determine the start of a window?

The same set of questions can be raised for PUSCH repetitions. A common design is desired for PUCCH/PUSCH repetitions. Based on views expressed in companies’ contributions, majority companies want to wait for the decision of time domain window design for PUSCH repetition and reuse it for PUCCH. Therefore, in this meeting, before progress made on PUSCH time domain window design, we will focus on signaling aspects of time domain window for PUCCH.

### Signalling to support the time domain window

Question 1: the RRC configuration for PUCCH repetition is per UE or per PUCCH resource?

Companies’ views submitted in the contributions are the following:

- Per UE: Nokia, CATT

  - Rationale: 1) the quality of channel condition is common for all PUCCH formats; 2) This additional flexibility with per PUCCH resource configuration is not justified since we see no disadvantage for a UE to have the DMRS bundling constantly activated each time PUCCH repetitions are scheduled. 3) RRC signalling overhead is high with per PUCCH configuration.

- Per PUCCH resource: QC, DCM, Apple

  - Rationale: 1) Due to the differences in the number and span of PUCCH repetitions across different PUCCH resources, it is preferred to configure parameters for DMRS bundling for each PUCCH resource separately; 2) with per PUCCH resource configuration, it is allowed to dynamically activate or deactivate the DMRS bundling feature by selecting a specific PUCCH resource through the PRI field in the scheduling DCI.

Since only a few companies provided input to this question in the contributions. FL would like to collect more input before we make a decision on this open issue.

FL Question 2: the RRC configuration for PUCCH repetition is per UE or per PUCCH resource?

Companies are welcome to provide answer to the above question in the following table. It is also recommended to provide some justifications for your answer, i.e., why per UE configuration is preferred or vise versa.

| Company name | Answer/Comment |
| --- | --- |

Question 2: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling?

Companies’ views submitted in the contributions are the following:

- Not needed: HW/HiSi, Nokia, Lenovo,

- Needed: Spreadtrum, Samsung, Interdigital

Since only a few companies provided input to this question in the contributions. FL would like to collect more input before we make a decision on this open issue.

FL Question 3: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling? If yes, what are the signaling design details.

Companies are welcome to provide answer to the above question in the following table.

| Company name | Answer/Comment |
| --- | --- |

Regarding the details of dynamic signaling, if needed, there is proposal from R1-2107653.

- Support a grant-type dependent index which indicates to the UE which PUCCH repetitions to bundle

Companies are welcome to provide comments to the above proposal in the following table.

| Company name | Comment |
| --- | --- |

## Inter slot freq hopping enhancement with DMRS bundling

In RAN1 104e, the following agreements were made under AI 8.8.2.

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling.

- FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16.

- Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible

In RAN1 104bis-e, the following agreements were made under AI 8.8.1.3. Since RAN1 should trive for common design between PUCCH and PUSCH repetition. The following agreement should be taken into account for the design of PUCCH repetition.

Agreements (RAN1#104-bis-e):

For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

- Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

- Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

  - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

  - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

There are three key questions RAN1 need to answer to complete the design for this topic.

Question 1: how to determine the bundle size (time domain hopping interval) for PUCCH/PUSCH

Question 2: whether the bundle size (time domain hopping interval) equals to the size of time domain window

Question 3: What is the interaction between the determination of time domain hopping interval determination and the determination of time domain window for DMRS bundling? In other words, when the two features, DMRS bundling and frequency hopping, are enabled simultaneously, a UE should determine the hopping intervals first or determine the window(s) for DMRS bundling first?

The first two questions are related to the design of hopping interval. Again, it is desired to have a unified design between PUCCH and PUSCH. Therefore, we will wait for progress in 8.8.1.3 and reuse the design in 8.8.1.2 for 8.8.2.

The third question is on the interaction between the two features of DMRS bundling and frequency hopping. FL would like to start some discussion on this topic. Apparently, there are at least two options as below.

- Option 1: A UE determines the time window(s) for DMRS bundling first. After the time window(s) for DMRS bundling is determined, the UE determines the hopping interval(s) and corresponding hop position for each hopping interval.

- Option 2: A UE determines the hopping intervals first. After the hoping intervals are determined, the UE determines the time window(s) for DMRS bundling.

There maybe other options/solutions that FL missed. Companies are welcome to provide input on this topic.

FL Question 4: What is the interaction between the determination of time domain hopping interval determination and the determination of time domain window for DMRS bundling? In other words, when the two features, DMRS bundling and frequency hopping, are enabled simultaneously, a UE should determine the hopping intervals first or determine the window(s) for DMRS bundling first?

Companies are welcome to provide answer to the above question in the following table.

| Company name | Answer/Comment |
| --- | --- |

## Other proposals

R1-2107562 proposed the following to study the technique of relative phase correction at gNB. This proposal has been discussed in RAN1 #105e without conclusion. We can continue the discussion in this meeting.

- Further study the benefit of gNB estimated inter-slot relative phase correction for PUCCH, addressing how frequency selective such phase corrections would need to be for UEs and/or conditions that do not sufficiently support maintaining inter-slot relative phase.

  - Consider operation with and without frequency hopping and with and without transparent transmit diversity.

Companies are welcome to provide comments to the above proposal in the following table.

| Company name | Comment |
| --- | --- |

Regarding the issue of PUCCH with repetition/DMRS bundling overlap with other PUCCH without repetition/DMRS bundling, R1-2107756 proposed the following proposals.

- Specify conditions under which a PUCCH with dynamic indication of repetition number may overlap with another PUCCH repetitions without dynamic indication of repetitions.

- If DMRS bundling is supported, specify conditions under which phase continuity is kept for a PUCCH with DMRS bundling overlapping in one (or more) occasions with a second PUCCH without DMRS bundling.

Companies are welcome to provide comments to the above proposal in the following table.

| Company name | Comment |
| --- | --- |

# Power control and TA with PUCCH repetitions

R1-2106905 mentioned the power control issue with PUCCH repetition. Given that UE can not adjust Tx power during the time window (for DMRS bundling), how to handle a received TPC with action time falls into the time window? On high level, two options can be considered. Option 1 is discarding this TPC. Option 2 is deferring the action time of this TPC to the time window boundary. Besides the options, R1-2107756 also proposed another solution “Proposal 6: For a PUCCH (or PUSCH) repetition with DMRS bundling, only TPC indicated by a unicast DCI is applied, i.e. TPC on GC-DCI 2-2 is ignored” and “Proposal 7: Unicast DCI with a TPC command implicitly indicates that DMRS bundling is off, from the occasion that new TPC is applied”.

A similar issue needs to be considered on how to handle a received TA command with action time falls into a time window (for DMRS bundling)

FL Question 5: What is the UE behavior for a received TA command or power control command whose appliance time falls into a time window for DMRS bundling for PUCCH/PUSCH repetition.

Companies are welcome to provide answers to the above question in the following table.

| Company name | Answer/Comment |
| --- | --- |

# References

| R1-2106498 | Discussion on PUCCH coverage enhancement | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2106614 | Discussion on PUCCH enhancements | vivo |
| R1-2106658 | PUCCH coverage enhancements | Nokia, Nokia Shanghai Bell |
| R1-2106712 | Discussion on PUCCH enhancements | Spreadtrum Communications |
| R1-2106742 | Discussion on coverage enhancements for PUCCH | ZTE |
| R1-2106905 | PUCCH enhancements | Samsung |
| R1-2106991 | Discussion on PUCCH enhancement | CATT |
| R1-2107118 | Discussion on PUCCH enhancement for NR coverage enhancement | Panasonic Corporation |
| R1-2107126 | Discussion on PUCCH enhancements | China Telecom |
| R1-2107142 | Discussion on PUCCH enhancements | NEC |
| R1-2107193 | Enhancements for PUCCH repetition | Lenovo, Motorola Mobility |
| R1-2107259 | PUCCH enhancements for coverage | OPPO |
| R1-2107362 | PUCCH enhancements | Qualcomm Incorporated |
| R1-2107420 | Discussion on PUCCH enhancements | CMCC |
| R1-2107477 | PUCCH enhancements | ETRI |
| R1-2107551 | Discussions on coverage enhancement for PUCCH | LG Electronics |
| R1-2107562 | PUCCH Dynamic Repetition and DMRS Bundling | Ericsson |
| R1-2107605 | Discussion on PUCCH enhancements | Intel Corporation |
| R1-2107653 | Discussions on PUCCH enhancements | InterDigital, Inc. |
| R1-2107756 | PUCCH coverage enhancement | Apple |
| R1-2107802 | PUCCH coverage enhancement | Sharp |
| R1-2107875 | PUCCH enhancements | NTT DOCOMO, INC. |
| R1-2107938 | Discussion on PUCCH enhancements | Xiaomi |