3GPP TSG RAN WG1 Meeting #106bis-e	R1-2108767

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon

Title:	Initial access signals and channels for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss our views on initial access including SS/PBCH blocks, discovery burst transmission window (DBTW), CORESET#0 multiplexing patterns and PRACH configuration.

SS/PBCH blocks transmission

SSB pattern

Agreement (RAN1 #106-e)[1]:

For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

In RAN1#106-e, the following agreement on SSB pattern have been reached.

Number of candidate SSB position within half frame: For operation without shared spectrum, Lmax=64 is sufficient for both 480 kHz and 960 kHz SCS to support at most 64 beams according to the WID. For operation with shared spectrum, additional candidate SSB positions is beneficial to deal with a LBT failure prior to SSB transmission in 480 kHz or 960 kHz SCS when Contention Exempt Short Control Signaling is not applicable.  Lmax=128 is a reasonable choice considering the tradeoff between increased channel access opportunities and increased candidate SSB index to be indicated in PBCH. On the other hand, the working assumption on support of at most 64 candidate SSB position for 120 kHz SCS in RAN1#106 does not imply there is less chance to have LBT failure in 60GHz unlicensed band. It is mostly due to large standard impact to introduce additional SSB candidates for 120 kHz SCS. We think the working assumption should be confirmed.

Proposal 1: For SSB with 120 kHz SCS, confirm the working assumption on 64 candidate SSBs within a half frame. For SSB with 480 kHz and 960 kHz SCS, 64 candidate SSBs is sufficient for operation without shared spectrum while 128 candidate SSBs should be supported for operation with shared spectrum.

UL slots within SSB burst: Similar to Case D for 120 kHz SSB, it may be reasonable to skip some slots within SSB burst to facilitate UL transmission. However, even with additional SSB candidate indexes for operation with shared spectrum, the total length of SSB burst may be much shorter than that in Case D for 120kHz SCS and, hence, it may not be well-justified to reserve some slots within such a short SSB burst for UL transmission. To balance these opposing observations, we first note that, in Case D for 120 kHz, 8 slot indexes Aμ=A3={8-9, 18-19, 28-29, 38-39} within a half frame are reserved for UL transmission. These slots correspond to slots A5={32-39, 72-79, 112-119, 152-159} within a half frame in 480 kHz and slots A6={64-79, 144-159, 224-239,304-319} within a half frame in 960 kHz. When designing SSB pattern for 480 kHz and 960 kHz SSBs, slots in A5 and A6 may be reserved for UL transmission, respectively. However, for operations without shared spectrum whereLmax=Lmax=64, 64 SSB candidates can be mapped to the first 32 slots 0-31 in 480 kHz or 960 kHz without an overlap with any slot in A5 or A6, respectively. As such, there is no need to reserve UL slots within the SSB burst. For operations with shared spectrum where Lmax=128, 128 SSB candidates can be mapped to slots 0-63 in 960 kHz without any overlap with A6. Therefore, there is no need to reserve UL slots within the SSB bust in this case either. In turn, 128 SSB candidates would occupy 64 slots in 480 kHz which would have overlap with slots 32-39 in A5. As such, we propose to map SSB candidates in 480kHz for operations with shared spectrum on the first 64 available slots 0,1,…,31, 40, 41,…71.

Figure 1  SSB pattern for different SCSs

Figure 1 shows our proposed SSB patterns for 120 kHz, 480 kHz and 960 kHz in operations with shared and without shared spectrum.

Proposal 2: Support following patterns for SSB with 480 kHz and 960 kHz SCS:

- For operations without shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31) for both 480 kHz and 960 kHz SCS.

- For operations with shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31,40,…,71) for 480 kHz SCS;

  - {2,9}+14n, (n=0,1,2,…,63) for 960 kHz SCS.

Since we propose the number of candidate SSBs in 480 kHz and 960 kHz SCS to be Lmax=128 for operation with shared spectrum, one more bit is required to indicate the candidate SSB index compared to Rel-15 FR2. We propose to borrow the 4th LSB of SFN in the PBCH payload to indicate the 7th bit of the candidate SSB index in operation with shared spectrum. In turn, we can transfer the 4th LSB of SFN to MIB payload. Note that if the 4th LSB of SFN is indicated in the MIB payload, the periodicity of update in MIB payload is still kept to 80 ms as in Rel-15.

Proposal 3: For operation with shared spectrum and for 480 kHz and 960 kHz SSBs, indicate the 7th bit of the candidate SSB index by borrowing the 4th LSB of SFN in the PBCH payload. Indicate the 4th LSB of SFN with spare bit in MIB payload.

Discovery burst transmission window

In RAN1#106-e, RAN1 conclude to continue discussions to develop solutions for supporting DBTW. Moreover, if DBTW is supported, similar DBTW lengths of {0.5, 1, 2, 3, 4, 5} msec as Rel-16 NR-U for 120 kHz SCS was agreed.

During initial access, UE determines the candidate SSB index from the DMRS sequence of PBCH and the PBCH payload in FR2 and the half frame indicator from the PBCH payload. This information along with the pre-defined location of each candidate SSB index within the half frame enables UE to determine the radio frame boundary. The corresponding system frame number (SFN) is also obtained from the PBCH and MIB payloads. In operation with shared spectrum, the transmission of SSBs in some candidate locations may be dropped due to a LBT failure. This increases the UE initial access latency if there is no more opportunity to transmit the dropped SSB within a DBTW. Note that, to our understanding, contention exempt short control signaling for operation with shared spectrum is not supported in all regions (e.g., Japan). Therefore, even if the duty cycle of the 480/960 kHz SSB burst is less than 10% within every 100 ms, LBT is mandated in these regions.

Observation 1: Contention exempt short control signaling for operation with shared spectrum is not supported in all regions. Therefore, even if the duty cycle of the 480/960 kHz SSB burst is less than 10% within every 100 ms, LBT must be used prior to 480/960 kHz SSB burst transmission in these regions.

Therefore, to facilitate transmitting the dropped SSBs due to LBT failure, we propose to support DBTW for all numerologies in Rel-17. It should be clarified that DBTW is still possible for SSB of 120 kHz SCS with the previous working assumption of 64 candidate SSB positions, in case the transmitted SSBs from a gNB is smaller than 64. In NR-U Rel-16, the parameters of NSSBQCL=1,2,4,8 is used for UE to determine the candidate SSB positions with same QCL assumption in the DBTW. Value of NSSBQCL  is indicated in MIB which allows the UE during initial access to only monitor the RMSI PDCCHs and RMSI PDSCHs associated with the candidate SSB indexes with the same QCL assumption as the detected SSB. Value of NSSBQCL is also indicated through cell specific or dedicated RRC signaling to facilitate intra and inter cell RRM measurement. In FR2-2, the value range of NSSBQCL should be scaled up considering that at most 64 beams are supported. Moreover, smaller values of NSSBQCL such as 16 and 32 can also be considered catering to gNB with less beams.

Proposal 4: Support discovery burst transmission window (DBTW) for all three numerologies in shared spectrum in 52.6GHz to 71GHz. At least  NSSBQCL=16,32,64 should be indicated in MIB for all three numerologies.

In NRU Rel-16, the DBTW length is indicated in SIB1 to facilitate UE to reduce the number of detection of candidate SSB position and the associated Type0 PDCCH monitoring occasion if co-channel interference is not severe. Although it is possible to thoroughly disable DBTW through MIB implicitly or explicitly (to be discussed later in the section) for the scenario where LBT is not required, finer adjustment of DBTW length in SIB1 as in NRU Rel-16 is still beneficial when DBTW is enabled and NSSBQCL is small.  For example, if  NSSBQCL=16, half of the SSB measurement can be saved if DBTW length of 2.5ms is indicated compared to the case that DBTW length is 5ms for 120 kHz. Note that, when we increase the maximum number of candidate SSB indices to 128 for 480kHz (960kHz) SCS in operation with shared spectrum, the maximum number of slots covering all candidate SSB positions and reserved UL slots in-between is 72 (64)  which equals to 2.25ms (1ms). The configurable DBTW lengths smaller than the above maximum lengths may be obtained by scaling down the supported DBTW lengths of {4, 3, 1, 1, 0.5} for 120 kHz SCS by a factor of 4 (8) for 480 (960) kHz.

Proposal 5: Configure DBTW length in SIB1 for operation with shared spectrum in 52.6GHz to 71GHz with the following values:

- 480 kHz SCS: {72, 32, 24, 16, 8, 4} slots = {2.25, 1, 0.75, 0.5, 0.25, 0.125} ms

- 960 kHz SCS: {64, 32, 24, 16, 8, 4} slots = {1, 0.5, 0.375, 0.25, 0.125, 0.0625} ms

- No indication for licensed and unlicensed operation in MIB

  - Whether and/or how LBT/No-LBT is indicated is separately discussed

- Use of LBT is not indicated in MIB.

  - FFS where and how this is indicated, e.g. SIB1

- For both licensed or unlicensed operation and with or without LBT, support the same DCI size for:

  - DCI format 1_0 scrambled with SI-RNTI monitored in a common search space

    - Note: existing bit padding/truncation rules are assumed to be applied for DCI format 0_0 monitored in common search space.

  - FFS for other cases

There have been long debates in the last four meetings on whether and how operations with/without shared spectrum, LBT on/off, and DBTW enable/disable should be indicated to the UE if DBTW is supported. In RAN1#106-e, the Proposal 1.1-2E) in Feature Leader Summary for initial access in RAN1#106-e [1] has been discussed but without consensus.

There is no ambiguity whether the carrier is operated with licensed band or unlicensed band from the regulatory perspective. Indication of licensed or unlicensed operation in MIB is redundant at least for non-roaming UE as it can be derived directly from subscriber (e.g. SIM card). Moreover, the difference between licensed and unlicensed operations is mainly due to the possible use of LBT for channel access in the unlicensed operation. However, 1) LBT may not be mandatory in all unlicensed bands in all regions; and 2) If the ambiguity in the size of DCI format 1-0 scrambled with SI-RNTI is resolved for the cases of LBT  ON/OFF, LBT ON/OFF indication can be deferred to SIB1.  Therefore, we do not see any reason to additionally indicate the licensed or unlicensed operation in MIB.

Proposal 6: No indication for licensed or unlicensed operation is required in MIB.

The initial access UE is only required to monitor DCI format 1-0 scrambled with SI-RNTI in Type0 PDCCH CSS before UE acquires SIB1. The field of “ChannelAccess-CPext” is not necessary at this stage with either LBT ON or LBT OFF mode. Thus, as discussed earlier, indication of LBT ON/OFF in MIB is not necessary from the perspective of determination of DCI field. After UE acquires SIB1 and before RRC connection is set up, the “ChannelAccess-CPext” field in DCI format 0-0/1-0 scrambled with TC_RNTI for msg3/msg4 and in RAR UL grant should be added for UE operating in LBT ON mode. According to TS38.213, a UE only expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. Adopting different DCI sizes before SIB1 acquisition and after SIB1 acquisition for UE operating with LBT ON mode may consume precious quote of DCI size in blind detection. So we propose to have common DCI size for DCI format 1-0 and DCI format 0-0 for both LBT ON and OFF modes.

Proposal 7: Use of LBT should be indicated in SIB1 to help UE determine the existence of “ChannelAccess-CPext” field in DCI format 1-0/0-0. Common DCI size should be assumed for DCI format 1-0/0-0 in CSS no matter LBT is ON or OFF.

There have also been long discussions on whether and how DBTW enabled/disabled should be indicated during initial access in operation with shared spectrum. First, we need to clarify what exactly DBTW enabled/disabled means. In our view, DBTW disabled means that there is one and only one candidate SSB index corresponding to each SSB index within the half frame that includes SSBs. That is, if a SSB index is not transmitted due to a LBT failure, there is no opportunity to transmit that SSB index within the same half frame. In turns, DBTW enabled means that there is more than one candidate SSB indexes for at least one SSB index within the half frame that includes SSBs. That is, for those SSB indexes that have more than one corresponding candidate SSB indexes, if the SSB index is not transmitted due to a LBT failure, there is at least one more opportunity to transmit that SSB index within the same half frame.

Observation 5: DBTW enabled/disabled can be defined as follows:

- DBTW disabled means that, within DBTW in the half frame that include SSBs, there is one and only one candidate SSB index corresponding to each SSB index.

- DBTW enabled means that, within DBTW in the half frame that include SSBs, there is more than one candidate SSB indexes for at least one SSB index.

Given above clarification of the definition of DBTW enabled/disabled, let us investigate how a UE would know DBTW is enabled/disabled in Rel-16 NR-U. In fact, DBTW enabled/disabled is never explicitly indicated to the UE in Rel-16 NR-U. In Rel-16, UE can infer whether DBTW is enabled/disabled only after reading SIB1 by comparing the maximum number of transmitted SSB indexes NSSBQCL (acquired from MIB payload) with the DBTW length (DiscoveryBurst-WindowLength provided in SIB1). If DBTW length that is configured in SIB1 is such that DBTW can include more than NSSBQCL candidate SSB indexes, UE can infer that DBTW is enabled. In turn, if DBTW length that is configured in SIB1 is such that DBTW cannot include more than NSSBQCL candidate SSB indexes, UE can infer that DBTW is disabled. For instance, in Case A SSB pattern for 15 kHz SCS, if NSSBQCL=8 and DBTW length in SIB1 is configured as 0.5, 1, 2, 3, or 4 ms, UE can infer that DBTW is disabled while if NSSBQCL=8 and DBTW length in SIB1 is configured to 5 ms, UE can infer that DBTW is enabled. Finally, note that, according to [4], if DBTW is not provided, the UE assumes that DBTW length is a half frame (includes all candidate SSB positions). As 5ms can accommodate 10 candidate SSB indexes in Case A and 20 candidate SSB indexes in Case C and, further, maximum supported SSB indexes NSSBQCL in Rel-16 is equal to 8, therefore, prior to reading SIB1, UE assumes that DBTW is enabled.

Observation 6: Following observations can be made regarding DBTW enabled/disabled indication in Rel-16 NR-U:

- DBTW enabled/disabled is not explicitly indicated to the UE.

- DBTW enabled/disabled is implicitly indicated to the UE in SIB1: UE can infer whether DBTW is enabled/disabled only after reading SIB1 by comparing the maximum number of transmitted SSB indexes NSSBQCL (acquired from MIB payload) with the DBTW length (provided in SIB1).

- When DBTW length is not provided, UE assumes that DBTW length is a half frame (includes all candidate SSB positions). Therefore, UE assumes that DBTW is enabled prior to reading SIB1.

In NR-U Rel-16, UE is always assuming LBT is ON and DBTW is ON by default. However, in FR2-2, it is possible that LBT is not mandatory for unlicensed band operation in certain regions. Letting UE know DBTW is OFF before UE acquires SIB1 could be beneficial to reduce blind detection of Type0 PDCCH CSS associated with candidate SSB position not transmitted at all. For example, UE may not need to detect monitoring occasions of Type0 PDCCH CSS associated with candidate SSB indexes in the range of 64 – 127 if DBTW is OFF for 480/960 kHz SCS. If there are 64 candidate SSB positions, indicating NSSBQCL=64 is equivalent to disable the DBTW because there is no additional candidate SSB position with same QCL in the window. However, if the number of candidate SSB positions is larger than 64, a separate entry of  NSSBQCL should be reserved for disabling DBTW.

Proposal 7: UE should be able to identify DBTW enable/disable before acquiring SIB1. A specific entry of NSSBQCL in MIB should be reserved to disable DBTW if the number of candidate SSB positions is larger than 64.

UE assumption regarding ssb-PositionsInBurst in SIB1

ssb-PositionsInBurst is provided in SIB1 to indicate time domain positions of the transmitted SSBs in a SSB burst. In Rel-16 NR-U, ssb-PositionsInBurst is comprised of a single 8-bit bitmap inOneGroup. If MSB k (k>=1) of inOneGroup is set to 1, UE assumes that one or more SSBs within DBTW and with the candidate SSB index corresponding to the SSB index k-1 may be transmitted. In turn, If MSB k of ssb- inOneGroup is set to 0, UE assumes that the SSBs with the candidate SSB indexes corresponding to the SSB index k-1 are not transmitted.  Further, “The UE assumes that a bit at position k > NSSBQCL is 0” [7]. Above interpretations of the bits of ssb-PositionsInBurst cannot be directly carried over in 60 GHz shared spectrum:  In Rel-16 NR-U, maximum number of transmitted SSB indexes is 8 which is equal to the size of the single bitmap inOneGroup in ssb-PositionsInBurst. In turn, in 60 GHz shared spectrum, following the FR2 convention, ssb-PositionsInBurst is comprised of two 8-bit bitmaps of groupPresense and inOneGroup while the maximum number of the transmitted SSB indexes is 64. To clarify UE interpretation of ssb-PositionsInBurst bits for operation with shared spectrum in 60 GHz where ssb-PositionsInBurst is configured with both inOneGroup and groupPresense bitmaps, if MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense are set to 1, UE should assume that SSBs within DBTW with candidate SS/PBCH block index(es) corresponding to SSB index equal to k-1+m-1×8 may be transmitted; otherwise, the UE should assume that the SSB(s) are not transmitted.

Proposal 8: In operation with shared spectrum in 60 GHz, for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst:

- if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted;

- if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

Further, it should be understood that, regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

Proposal 9: Regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense in ssb-PositionsInBurst configured in SIB1, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

CORESET#0/Type0-PDCCH common search space design

It was agreed in RAN 92-e that, for all three supported SCSs of 120, 480, and 960 kHz, Type0-PDCCH and the corresponding SSB have the same SCS and, further, SSB-CORESET#0 multiplexing pattern 1 is prioritized [3]. In addition, the following agreements regarding CORESET#0/Type0-PDCCH for 120 kHz SCS has been reached:

Agreement (RAN1 #104-e): For CORESET#0 and Type0-PDCCH search space configured in MIB: - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz. - FFS: Supporting additional values - FFS: Supported values for SSB to CORESET#0 offset RBs Agreement (RAN1 #106-e): For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - Support the following set of parameters. SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols. 1; 24; 2. 1; 48; 1. 1; 48; 2. - Note: the number of entries corresponding the same {mux pattern, number of RB, number of symbol} tuple (listed above) will depend on required RB offsets that needs to be supported based on channel and sync raster design. FFS: addition other set of parameters

In particular, above agreement states that for {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, both multiplexing Pattern 1 and multiplexing Pattern 3 are supported. For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz and {960, 960} kHz, we believe that supporting multiplexing Pattern 1 is sufficient. As such, we propose the following.

Proposal 10:  For CORESET for Type0-PDCCH in 52.6GHz to 71GHz spectrum, support the following:

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, support multiplexing pattern 1 and multiplexing pattern 3 as per Agreement in RAN1 104-e.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz, support multiplexing pattern 1 only.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {960, 960} kHz, support multiplexing pattern 1 only.

CORESET#0 configuration for 120 kHz SCS

In TS38.213, the set of resource blocks and slot symbols of COREST for Type0-PDCCH search space set are defined in Table 13-8 for  {SS/PBCH block, PDCCH} SCS equal to {120, 120} kHz. As agreed in RAN1 104-e, the same CORESET#0 BWs should also be reused in 52.6GHz to 71GHz spectrum. According to FCC regulation for operation with shared spectrum in the US, the peak transmitter conducted output power will be penalized if the transmission bandwidth is smaller than 100MHz due to the power spectrum density limit (7dBm/MHz). In order to make full use of the transmit power, CORESET#0 with 96 PRB could also be considered with multiplexing pattern 1. CORESET0 with 96 PRB allows gNB to transmit a PDCCH with the aggregation level of 16. It should be noted that 96 PRB for CORESET#0 had been discussed in Rel-15 but was not adopted because of the limited available bandwidth at that time.  In addition, NsymbCORESET  = {1, 2} have already been supported in Rel-15 for NRBCORESET=48. We propose to support both these number of symbols when NRBCORESET is equal to 96 RB.

Proposal 11: For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, in addition to the supported values of (NRBCORESET, NsymbCORESET ) from Rel-15, support NRBCORESET= 96 with NsymbCORESET={1,2} for multiplexing pattern 1.

In Rel-15/Rel-16, each CORESET#0 configuration index is associated with a RB offset from the smallest RB index of the CORESET#0 to the smallest RB index of the common RB overlapping with the first RB of the corresponding SSB. Although the exact values of supported RB offsets from CORESET#0 to SSB in 52.6GHz to 71GHz spectrum maybe restricted by the synch raster and channel raster design in FR2-2, RAN1 can still discuss and agree on the design principles and nominal values of RB offsets from CORESET#0 to SSB. Assuming that there will not be any restriction imposed by the synch raster and channel raster design in FR2-2 on RB offsets from CORESET#0 to SSB, the RB offsets defined in Rel-15/Rel-16 FR2 for the supported (Mux pattern, NRBCORESET, NsymbCORESET ) tuples can be inherited without any modification. In addition, for CORESET0 with 48 and 96 PRB with multiplexing pattern 1, it would be beneficial for gNB to transmit portion of discovery burst (e.g. SSB, Type0-PDCCH, RMSI PDSCH) with the same beam and as compact as possible in time and frequency domain to save LBT and beam switching overheads.  Considering only frequency domain resource allocation type 1 is allowed for RMSI PDSCH, aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for PDSCH carrying SIB1. Similar concept has been adopted in NR-U Rel-16 due to the same reason.

Proposal 12: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={120, 120} kHz:

- For CORESET#0 with 24 RBs and 48 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: additional RB offsets values of 0 and 28 RBs can be considered for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets values of 0 and 76 RBs can be considered for multiplexing pattern 1.

- Note: All above RB offsets are nominal and may need to be modified after finalizing synch raster and channel raster design in FR2-2.

Figure 2 Example of RB-level offset configuration for {SS/PBCH block, PDCCH}={120, 120} kHz when 96 RB CORESET0 is configured.

The new entries (red rows) for CORESET0 configuration discussed above can be added in table 13-8 of TS38.213 as follows:

Table 1 Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 5 | 3 | 24 | 2 | 24 |
| 6 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 7 | 3 | 48 | 2 | 48 |
| 8 | 1 | 48 | 1 | 0 |
| 9 | 1 | 48 | 1 | 28 |
| 10 | 1 | 48 | 2 | 0 |
| 11 | 1 | 48 | 2 | 28 |
| 12 | 1 | 96 | 1 | 0 |
| 13 | 1 | 96 | 1 | 76 |
| 14 | 1 | 96 | 2 | 0 |
| 15 | 1 | 96 | 2 | 76 |

CORESET#0 configuration for 480 kHz and 960 kHz SCS

As the beam switching delay may be too large to be absorbed in CP in 480/960 kHz SCS, it is critical to have the possibility of transmitting SSB and its associated RMSI PDCCH/PDSCH without a need for a switching gap. Aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for PDSCH carrying SIB1. Aligning the lowest (highest) RBs of SSB and CORESET#0 in the frequency domain is already supported in Rel-15/16 for 24 RB CORESET#0 and multiplexing pattern 1 in {SSB, CORESET#0} ={120, 120} kHz, which can be directly reused for 480 kHz and 960 kHz SCS. For 48 RB CORESET#0, at least the RB offset of 0 and 28 RBs should be supported. An example is shown in the following figure.

Figure 3 Example of RB-level offset configuration for {SS/PBCH block, PDCCH}={480, 480} kHz and {960, 960} kHz when 48 RB CORESET0 is configured.

Proposal 13: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

- For CORESET#0 with 24 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: In addition to the offset of 14 RBs already supported in Rel-16, additional values of 0 and28 RBs can be considered for multiplexing pattern 1.

The set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz and {960, 960} kHz SCS is provided in the following table.

Table 2   Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz and {960, 960} kHz for operation with share spectrum

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |

Type0-PDCCH common search space configuration

According to TS38.213 section 13, the associated Type0-PDCCH monitoring occasions corresponding to a SS/PBCH block index i are included in two consecutive slots starting from slot n0. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. Parameters O and M are provided in Table 13-12 of 38.213 (copied in below Table 3) for all combinations of {SSB, CORESET#0/Type0-PDCCH} numerologies supported in FR2-1, including {240, 120}, {120, 120}, {240, 60}, and {120, 60} kHz.

Table 3	Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-1

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 2 | 2.5 | 1 | 1 | 0 |
| 3 | 2.5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 6 | 0 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 7 | 2.5 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 8 | 5 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 9 | 7.5 | 1 | 1 | 0 |
| 10 | 7.5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 11 | 7.5 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 12 | 0 | 1 | 2 | 0 |
| 13 | 5 | 1 | 2 | 0 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

In contrary to FR2-1, only the same numerology between SSB and CORESET0 is supported in FR2-2. As such, once the numerology is determined from the detected SSB, the useful values of O and the first symbol index of monitoring occasions for Type0-PDCCH CSS set are limited. Thus, as discussed in the following lines, a bit of searchSpaceZero can be saved for other purposes.

- Offset value O

The parameter O is used to determine the offset between Type0-PDCCH CSS MO associated with the candidate SSB index 0 in the unit of msec from the beginning of the frame.  In FR2-1, the subset of O=0, 2.5, 5 is used when SSB burst set is located in the 1st half frame. gNB uses O=2.5(5) to configure the  Type0-PDCCH CSS monitoring window after the whole associated 240kHz (120kHz) SSB burst set is transmitted.  Similarly, when SSB burst set is in the 2nd half frame, the subset of  O=0, 5,7.5 is used. The value of  O= 7.5 (0) is configured to shift the Type0-PDCCH CSS monitoring window after the whole associated 240kHz (120kHz) SSB burst set  is transmitted. An example is illustrated in Figure 4.

Figure 4	Examples of SSB/COREST0 multiplexing pattern 1

For 120 kHz SCS in FR2-2, the O values of {2.5, 7.5} are not necessary because they are used for mix numerology configuration for SSBs with 240 kHz SCS in FR2-1. The O values for the 480 kHz and 960 kHz SSB burst set could be derived by scaling the values for 120 kHz SSB burst set with scaling factor of X=12μ-3 when DBTW is OFF. If DBTW is ON and assuming at most 128 candidate SSB position for 480 kHz and 960 kHz SSB burst set, the scaling factor is X=12μ-4.  For SSB burst set transmitted in the 1st half frame, the O values of 0, 5X should be used. For SSB burst set transmitted in the 1st half frame, the O values of 5, 5+5X should be used.

- First symbol index

It is agreed in RAN1#104bis and RAN1#106 that existing SSB pattern case D will be reused in FR2-2 for 120 kHz SSB. By comparing the candidate SSB positions, it is observed that some of Type0-PDCCH monitoring occasions starting at OS#7 collide with non-associated SSBs when the SSB burst set and Type0-PDCCH CSS are located in the same half frame as plotted in Figure 5.  Such a combination is usually not configured due to its low efficiency for SSB measurement, because UE will assume the SSB collide with type0 PDCCH monitoring occasion is not transmitted.

Figure 5	SSB/Type0-PDCCH CSS with index 1/5 in Table 5

For 480kHz and 960kHz SCS, the Type0-PDCCH CSS configuration indexes {6,7,8,11} in Table 13-12 of 38.213 with monitoring occasions starting symbols of {0, if i is even} and {NsymbCORESET, if i is odd} may not work because CP is too short to accommodate potential beam switching gap between two monitoring occasions transmitted with different beams. These configuration indexes are even more problematic if  Type0-PDCCH CSS monitoring window and SSB burst set start in the same half-frame. In such a case, since we agreed that “first symbols of the candidate SSB have index {2, 9} + 14*n”, to accommodate both Type0-PDCCHs and the first candidate SSB in the slot,  NsymbCORESET can only be equal to one. Moreover, gNB should perform two beam switching between the Type0-PDCCHs in OS#0 and OS#1 and the Type0-PDCCH in OS#1 and the SSB candidate in OS#2. On the other hand, since the first symbols of the candidate SSB in a slot have index {2, 9}, the usage of  configurations {1, 3, 5, 10} with monitoring occasions starting symbols of {0, if i is even} and {7 if i is odd} to support COREST0 with 2 OS is very well justified

Based on the above discussion, we propose to update the parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 for FR2-2 in the following tablea.

Table 4  Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-2 when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 5 | 1 | 1 | 0 |
| 2 | 0 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 3 | 5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 4 | 0 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 6 | 0 | 1 | 2 | 0 |
| 7 | 5 | 1 | 2 | 0 |

Table 5 Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-2 when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz or {960, 960} kHz

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 2 | 5X | 1 | 1 | 0 |
| 3 | 5X | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 6 | 5+5X | 1 | 1 | 0 |
| 7 | 5+5X | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 8 | 0 | 1 | 2 | 0 |
| 9 | 5 | 1 | 2 | 0 |
| 10-15 | Reserved |  |  |  |

Proposal 14: The parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 listed in Table 4 and Table 5 should be supported. For 480kHz and 960 kHz SCS, the scaling factor X in Table 5 is 12μ-3 when DBTW is OFF and 12μ-4 when DBTW is ON.

- Note: DBTW OFF is indicated in MIB using a value of NSSBQCL

Offset between off-sync raster SSB and CORESET#0

In Rel-16 NRU, off-synch raster SSBs may configure CORESET#0 to support ANR on SCells. When a UE is configured to report the CGI associated with an off-synch raster SSB, the UE finds the frequency offset from CORESET#0 to the off-synch raster SSB according to a sum of the following first offset and the second offset:

- First offset: Provided in Table 13-1A and Table 13-4A defined in [4][2], and

- Second offset: Determined as the offset from a smallest RB index of the common RB overlapping with the first RB of the off-synch raster SSB indicated in the measurement configuration to a smallest RB index of the common RB overlapping with the first RB of a SSB hypothetically located at the GSCN of a synch raster entry where the single synch raster entry is located in the same channel as the SSB used for the shared spectrum channel access procedure [4].

Above offsets are defined with respect to the SCS of the CORESET#0, which for operation with shared spectrum, is the same as the SCS of the corresponding SSB. The frequency location of the CORESET#0 associated with the off-synch raster SSB can be unambiguously determined from the above sum since only a single synch raster entry is located in the same channel as the configured off-synch raster SSB. The reason for the one to one correspondence between synch raster entry and the channel in Rel-16 NRU is that, in Rel-16 NRU:

1) Each channel has a 20 MHz nominal channel bandwidth as defined in [5] and;

2) Synch raster entries in unlicensed bands are allocated roughly 20 MHz apart such that one and only one on-synch raster SSB can be fully contained in each channel.

However, in 60GHz unlicensed spectrum, neither of the above two conditions are met: First, regulations do not define a fixed nominal channel bandwidth [6]. Second, RAN4 has not determined the synch raster locations yet. Further, given the flexible nominal channel bandwidth in 60GHz unlicensed spectrum, it seems impossible to guarantee only one on-synch raster SSB per channel. Above discussion hints to the conclusion that the same method as in Rel-16 NRU cannot be used to find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum. However, without having to wait for RAN4 to allocate the synch rasters, above issue may be independently resolved in RAN1 by uniquely determining the hypothetical on-synch raster SSB that serves as the reference for the above second offset in case multiple synch rasters (and hence, multiple hypothetical on-synch raster SSBs) are included in a channel bandwidth.

Proposal 15: To find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum, RAN1 should uniquely determine the hypothetical on-synch raster SSB that serves as the reference for the offset to the off-synch raster SSB in case more than one synch rasters are included in a channel bandwidth.

Fields in PBCH payload and MIB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

In RAN1#104bis-e, the following agreement was reached in which some restrictions are set on the bit size of PBCH and MIB.

According to the discussion in previous sections, several features might have impact on the PBCH payload and MIB content. For example, the bit#6 (subCarrierSpacingCommon) and bit#19 (pdcch-configSIB1) can be re-interpreted to indicate NSSBQCL. For 480kHz and 960kHz SCS SSB with DBTW is ON, the bit#23 (4th LSB of SFN) could be used to indicate the 7th bit of candidate SSB index if 128 candidate SSB positions are supported. The 4th LSB of SFN can take the spare bit in bit#22.

Proposal 16: the MIB content and PBCH payload in Table 6 and Table 7 should be supported for 120 kHz, 480 kHz and 960 kHz SSB.

Table 6 MIB and PBCH payload bit allocation for 120kHz SCS SSB

| bit |  | FR2-1 | FR2-2 |
| --- | --- | --- | --- |
|  |  | 120kHz | 120kHz |
| MIB | 0 | 10 - 5 MSB of SFN | 10 - 5 MSB of SFN |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  | 5 |  |  |
|  | 6 | subCarrierSpacingCommon | 1 bit for NSSBQCL (sec 2.2) |
|  | 7 | ssb-SubcarrierOffset | ssb-SubcarrierOffset |
|  | 8 |  |  |
|  | 9 |  |  |
|  | 10 |  |  |
|  | 11 | dmrs-TypeA-Position | dmrs-TypeA-Position |
|  | 12 | pdcch-ConfigSIB1 /controlResourceSetZero | controlResourceSetZero (Sec 3.1 Table 1) |
|  | 13 |  |  |
|  | 14 |  |  |
|  | 15 |  |  |
|  | 16 | pdcch-ConfigSIB1 /searchSpaceZero | searchSpaceZero (Sec 3.3 Table 4) |
|  | 17 |  |  |
|  | 18 |  |  |
|  | 19 |  | 1 bit for NSSBQCL (sec 2.2) |
|  | 20 | cellBarred | cellBarred |
|  | 21 | intraFreqReselection | intraFreqReselection |
|  | 22 | spare | Spare bit |
| PBCH payload | 23 | 4th LSB of SFN | 4th LSB of SFN |
|  | 24 | 3th LSB of SFN | 3th LSB of SFN |
|  | 25 | 2th LSB of SFN | 3th LSB of SFN |
|  | 26 | 1th LSB of SFN | 3th LSB of SFN |
|  | 27 | half frame indication | half frame indication |
|  | 28 | 6th bit of candi. SSB index | 6th bit of candi. SSB index |
|  | 29 | 5th bit of candi. SSB index | 5th bit of candi. SSB index |
|  | 30 | 4th bit of candi. SSB index | 4th bit of candi. SSB index |

Table 7 MIB and PBCH payload bit allocation 480kHz and 960kHz SCS SSB

| bit |  | FR2-1 | FR2-2 |  |
| --- | --- | --- | --- | --- |
|  |  | 120kHz | 480kHz and 960kHz |  |
|  |  |  | DBTW OFF | DBTW ON |
| MIB | 0 | 10 - 5 MSB of SFN | 10 - 5 MSB of SFN |  |
|  | 1 |  |  |  |
|  | 2 |  |  |  |
|  | 3 |  |  |  |
|  | 4 |  |  |  |
|  | 5 |  |  |  |
|  | 6 | subCarrierSpacingCommon | 1 bit for NSSBQCL (sec 2.2) |  |
|  | 7 | ssb-SubcarrierOffset | ssb-SubcarrierOffset |  |
|  | 8 |  |  |  |
|  | 9 |  |  |  |
|  | 10 |  |  |  |
|  | 11 | dmrs-TypeA-Position | dmrs-TypeA-Position |  |
|  | 12 | pdcch-ConfigSIB1 /controlResourceSetZero | controlResourceSetZero (sec 3.2 Table 2) |  |
|  | 13 |  |  |  |
|  | 14 |  |  |  |
|  | 15 |  | searchSpaceZero (Sec 3.3 Table 5) |  |
|  | 16 | pdcch-ConfigSIB1 /searchSpaceZero |  |  |
|  | 17 |  |  |  |
|  | 18 |  |  |  |
|  | 19 |  | 1 bit for NSSBQCL (sec 2.2) |  |
|  | 20 | cellBarred | cellBarred |  |
|  | 21 | intraFreqReselection | intraFreqReselection |  |
|  | 22 | spare | Spare bit | 4th LSB of SFN |
| PBCH payload | 23 | 4th LSB of SFN | 4th LSB of SFN | 7th bit of candi. SSB index (sec 2.1) |
|  | 24 | 3th LSB of SFN | 3th LSB of SFN |  |
|  | 25 | 2th LSB of SFN | 3th LSB of SFN |  |
|  | 26 | 1th LSB of SFN | 3th LSB of SFN |  |
|  | 27 | half frame indication | half frame indication |  |
|  | 28 | 6th bit of candi. SSB index | 6th bit of candi. SSB index |  |
|  | 29 | 5th bit of candi. SSB index | 5th bit of candi. SSB index |  |
|  | 30 | 4th bit of candi. SSB index | 4th bit of candi. SSB index |  |

Random access design

PRACH sequence

In RAN1#104-e, it was agreed to support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2. In RAN1#106-e, it was further agreed not to support PRACH length L=571, 1151 for 960 kHz PRACH and at least L =1151 for 480 kHz PRACH. It is still pending whether to support L=571 for 480 kHz PRACH. It is observed that PRACH length L= 139 in 480 kHz SCS only occupies 66.7MHz. Therefore, UE may not make full use of the allowed conducted output power according to FCC regulation due to emission bandwidth of less than 100 MHz. we propose to support  L=571 for 480 kHz SCS as well as L=139, considering potential coverage enhancement with marginal standard effort.

Proposal 17: Additionally support L=571 for 480 kHz PRACH.

RACH occasions configuration

Agreement:

- For 480 and 960kHz PRACH:

  - The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

Agreement:

For 480 and 960kHz PRACH:

- At least the same RO density in time domain (i.e. number of specified RO per reference slot according the PRACH configuration index) as for 120kHz PRACH in FR2 is supported

  - FFS: Support gap between consecutive ROs in time domain and the details to derive the gap

Agreement:

For 480 and 960kHz PRACH,

- When a PRACH slot can contain all time domain PRACH occasions corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching,

  - and when number of PRACH slots in a reference slot is 1,

    - nslotRA=[7] for 480kHz and nslotRA=[15] for 960kHz PRACH

  - and when the number of PRACH slots in a reference slot is 2,

    - nslotRA=[3,7] for 480kHz and nslotRA=[7,15] for 960kHz PRACH

- FFS: nslotRA values, when a PRACH slot cannot contain all time domain PRACH occasions, corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching.

- FFS: whether to allow for additional nslotRA values if the maximum that can be configured for the number of FD RO’s is less than 8 (due to BW limitation)

In RAN1#106-e, RAN1 agreed that

As discussed earlier, according to the RAN4 LS reply [8], beam switch time at gNB is [59ns]. Moreover, up to 200ns beam switch time at the UE side is suggested in ongoing discussions in RAN4 [9]. Comparing these values with the 73ns (146ns) CP length of 960kHz (480kHz) OFDM symbols, introducing a beam switching gap symbol for both UL and DL signals/channels seem required.  Although the CP lengths of different PRACH formats are longer than that of an OFDM symbol, such larger CP lengths are mainly devised to additionally cope with the timing uncertainty that can be up to the maximum round-trip delay in the cell. In particular, the CP of Format A1 is 146ns (292ns) in 960kHz (480kHz) and the CP of Format B1 is 110ns (220ns) in 960kHz (480kHz). Such CP lengths are not enough to accommodate up to 200ns of beam switch time in addition to round-trip time delay, DL time synchronization, and channel dispersion effects. Therefore, we propose to support a gap symbol between consecutive ROs for 480 kHz and 960 kHz PRACH configurations for Format A1, B1, and A1/B1.

Proposal 18: For 480 kHz and 960 kHz PRACH, support one gap symbol between consecutive ROs in time domain at least for Formats A1, B1, and A1/B1.

In the PRACH configuration table Table 6.3.3.2-4 of 38.211, the triples of l0,NtRA,slot,NdurRA   determine the position of ROs in a PRACH slot. In Rel-15/16, the TDMed RO in a PRACH slot is continuous in time domain. l0 is the starting symbol of the first RO in the PRACH slot. NtRA,slot is the number of time-domain PRACH occasions within the PRACH slot. NdurRA  is PRACH duration in the unit of symbol. Due to the tedious discussion on PRACH configuration table in Rel-15, it would be beneficial to reuse the existing PRACH configuration table. From the triples of l0,NtRA,slot,NdurRA   in the configuration table for PRACH format A1, B1 and A1/B1, it can be observed that ROs usually occupy either almost whole PRACH slot or only the 2nd half of a PRACH slot, because the downlink symbols are usually at beginning of the slot. Similar characteristic should be maintained when the ROs need to be shifted due to a gap insertion. Thus, backward shifting ROs within the PRACH slot is the preferred method since forward shifting of ROs within the PRACH slot inevitably shifts some ROs to the beginning of the next slot. Let us denote

NgapRA=1Format A1, B1, A1/B10Otherwise           (1)

as the gap length between two consecutive ROs in the unit of OFDM symbol. Then, the first symbol for each RO in the reference slot can be derived from the following equation (2).

l=l0-NtRA,slot-1NgapRA+ntRANdurRA+NgapRA+14nslotRA             (2)

Note that, the values of l0, NtRA,slot, NdurRA in equation (2) are reused from Table 6.3.3.2-4 of 38.213 for all PRACH configuration indexes. If NgapRA = 0, then equation (2) reduces to the same equation for l as in Clause 5.3.2 of 38.213.

If NgapRA = 1, the actual starting symbol of the 1st RO in a PRACH slot is shifted backward from the original indicated position l0 by NtRA,slot-1NgapRA=NtRA,slot-1 symbols and the first symbols of the subsequent ROs will be placed at ntRANdurRA+NgapRA= ntRANdurRA+1 symbols after the first symbol of the 1st RO for ntRA=1,…, NtRA,slot-1. Note also that when l0-NtRA,slot-1NgapRA= l0-NtRA,slot-1≥0, the 1st RO  is still within the specified PRACH slot nslotRA in a 60kHz reference slot. However, when l0-NtRA,slot-1 <0, the first symbol of the 1st RO will be at the preceding slot nslotRA-1 (to be specific, at symbol l0-NtRA,slot-1+14  of the preceding slot nslotRA-1 ).

In particular, for the configuration indexes of {12, 14, 15 19-21, 24, 25, 28} with triples of l0,NtRA,slot,NdurRA=7,3,2 for PRACH format A1 and configuration indexes of {95, 97, 98, 102, 104, 105, 107, 108, 111, 209, 210, 213, 216} with triples of l0,NtRA,slot,NdurRA  =8,3,2 for PRACH format B1 and A1/B1, the actual starting point of 1st RO is still within the PRACH slot nslotRAin a 60kHz reference slot because l0-NtRA,slot-1NgapRA= l0-NtRA,slot-1≥0.  An example is shown in Figure 6.

Figure 6: Consecutive ROs with gaps for PRACH format B1 or A1/B1 with configuration triples of l0,NtRA,slot,NdurRA  =8,3,2

In turn, for all the remaining configuration indexes of {0-11, 13, 16-18, 22, 23, 26, 27} for PRACH format A1 with l0,NtRA,slot,NdurRA  =0,6,2 and {89-94, 96, 99-101, 103, 106, 109, 110, 202-208, 211, 212, 214, 215, 217-219} for PRACH format B1 or A1/B1 with  l0,NtRA,slot,NdurRA  =2,6,2, some ROs that would be originally at beginning of the PRACH slot nslotRA if there was no gap, are moved to the end of the preceding slot nslotRA-1 after the gap is inserted.  Note that there is no ROs that straddle between slots nslotRA-1 and nslotRA for any of the above configuration indexes. An example is also shown in Figure 7.

Figure 7 Consecutive ROs with gaps for PRACH format A1 with configuration triples of l0,NtRA,slot,NdurRA  =0,6,2

Proposal 19: For PRACH format A1, B1 and A1/B1, the first symbols for each RO in a reference slot can be derived using equation (2) if a gap symbol between consecutive ROs is introduced. The same triples of l0,NtRA,slot,NdurRA   in Table 6.3.3.2-4 of 38.211 are reused.

- Note: Equation (2) guarantees that no RO straddles between slots nslotRA-1 and nslotRA.

Resolving RA-RNTI/ MsgB-RNTI overflow issue

When a random access preamble is transmitted, UE shall monitor RAR with RA-RNTI provided by

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id	(3)

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The maximum value of t_id is 79 as there are 80 slots per system frame in 120 kHz SCS. With the support of 480kHz and 960kHz PRACH in FR2-2, the maximum number of slots per system frame increases to 640 slots. A straightforward modification of above RA-RNTI formula would then be

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 640 × f_id + 14 × 640 × 8 × ul_carrier_id. 	(4)

However, such modification would result in the maximum value of RA-RNTI exceeding the highest possible value of RNTI, i.e. 2^16-1.

Recall that we have agreed in RAN1 106-e that when the number of PRACH slots in a reference slot is 1, nslotRA=[7] for 480kHz and nslotRA=[15] for 960kHz PRACH and when the number of PRACH slots in a reference slot is 2, nslotRA=[3,7] for 480kHz and nslotRA=[7,15] for 960kHz PRACH. Assuming that the values in brackets are confirmed, the index of any two PRACH slots within a system frame differs at least by the value of 4 (8) in 480 kHz (960 kHz) or, in a more compact form, by the value of 2μ-3for μ=5, 6.

As discussed in Section 5.2, for RO configurations without a gap, or RO configurations with an inserted gap symbol and with l0,NtRA,slot,NdurRA  =8,3,2 or 7,3,2, all ROs are still maintained in the original PRACH slot nslotRA. Since the index of any two PRACH slots within a system frame differs at least by the value of 2μ-3, gNB can compress the index of the slot of the received RO in a system frame (tid) in the RA-RNTI formula to floor(tid2μ-3) without any loss of information and UE can recover the index of the PRACH slot from

RA-RNTI = 1+s_id+14×floor(tid2μ-3)+14×80×f_id+14×80×8×ul_carrier_id	(5)

In turn, as discussed in Section 5.2, for the RO configurations with an inserted gap symbol and with l0,NtRA,slot,NdurRA  =0,6,2 or 2,6,2, some ROs that would have been  in slot nslotRA of the reference slot if there were no gap, would be located in the preceding slot nslotRA-1. In such a case, this ambiguity between the two consecutive slots can be resolved by introducing 1 bit in DCI 1-0 scrambled by RA-RNTI/MsgB-RNTI to indicate whether the received RO belongs to the “original” PRACH slot (slot nslotRA within the reference slot) or in its immediately preceding slot.

Proposal 20: For the same RO density per reference slot as 120 KHz PRACH, the RA-RNTI corresponding to 480 kHz and 960 kHz ROs can be generated according to equation (5) by compressing the t_id to tid2μ-3.

- When some ROs are backward shifted to the immediately preceding slot of the specified slot nslotRA due to the use of a gap symbol between consecutive ROs, support a 1 bit indication field in the DCI scheduling RAR/MsgB to resolve the PRACH slot ambiguity.

RAR window

In Rel-16 NR-U, RAR window was enlarged to the maximum of 40 ms to cope with the possible latency due to the LBT failure prior to msg2 transmission at gNB. In 2-step RACH, the same maximum length of 40 ms was also used to take into account the additional PUSCH processing delay at gNB as gNB needs to decode UE’s PUSCH appended to MsgA prior to sending MsgB. Issues of LBT failure prior to msg2 transmission at gNB for operation with shared spectrum and PUSCH processing delay at gNB prior to sending MsgB for both operations with and without shared spectrum should also be taken into account in 52.6GHz to 71GHz spectrum. Therefore, we propose to also enlarge ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum to the maximum of 40 ms. In this case, similar to Rel-16, the two bits in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI) should also be used to indicate the two LSBs of SFN at which the gNB has received msg1 (MsgA).

Proposal 21: Support maximum of 40 ms for ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum. Support indicating two LSBs of SFN at which gNB has received msg1 (MsgA) in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI).

Conclusions

In this paper, we discuss the issues related to initial access. The following observations and proposals are made.

Proposal 1: For SSB with 120 kHz SCS, confirm the working assumption on 64 candidate SSBs within a half frame. For SSB with 480 kHz and 960 kHz SCS, 64 candidate SSBs is sufficient for operation without shared spectrum while 128 candidate SSBs should be supported for operation with shared spectrum.

Proposal 2: Support following patterns for SSB with 480 kHz and 960 kHz SCS:

- For operations without shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31) for both 480 kHz and 960 kHz SCS.

- For operations with shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31,40,…,71) for 480 kHz SCS;

  - {2,9}+14n, (n=0,1,2,…,63) for 960 kHz SCS.

Proposal 3: For operation with shared spectrum and for 480 kHz and 960 kHz SSBs, indicate the 7th bit of the candidate SSB index by borrowing the 4th LSB of SFN in the PBCH payload. Indicate the 4th LSB of SFN with spare bit in MIB payload.

Observation 1: Contention exempt short control signaling for operation with shared spectrum is not supported in all regions. Therefore, even if the duty cycle of the 480/960 kHz SSB burst is less than 10% within every 100 ms, LBT must be used prior to 480/960 kHz SSB burst transmission in these regions.

Proposal 4: Support discovery burst transmission window (DBTW) for all three numerologies in shared spectrum in 52.6GHz to 71GHz. At least  NSSBQCL=16,32,64 should be indicated in MIB for all three numerologies.

Proposal 5: Configure DBTW length in SIB1 for operation with shared spectrum in 52.6GHz to 71GHz with the following values:

- 480 kHz SCS: {72, 32, 24, 16, 8, 4} slots = {2.25, 1, 0.75, 0.5, 0.25, 0.125} ms

- 960 kHz SCS: {64, 32, 24, 16, 8, 4} slots = {1, 0.5, 0.375, 0.25, 0.125, 0.0625} ms

Proposal 6: No indication for licensed or unlicensed operation is required in MIB.

Proposal 7: Use of LBT should be indicated in SIB1 to help UE determine the existence of “ChannelAccess-CPext” field in DCI format 1-0/0-0. Common DCI size should be assumed for DCI format 1-0/0-0 in CSS no matter LBT is ON or OFF.

Observation 5: DBTW enabled/disabled can be defined as follows:

- DBTW disabled means that, within DBTW in the half frame that include SSBs, there is one and only one candidate SSB index corresponding to each SSB index.

- DBTW enabled means that, within DBTW in the half frame that include SSBs, there is more than one candidate SSB indexes for at least one SSB index.

Observation 6: Following observations can be made regarding DBTW enabled/disabled indication in Rel-16 NR-U:

- DBTW enabled/disabled is not explicitly indicated to the UE.

- DBTW enabled/disabled is implicitly indicated to the UE in SIB1: UE can infer whether DBTW is enabled/disabled only after reading SIB1 by comparing the maximum number of transmitted SSB indexes NSSBQCL (acquired from MIB payload) with the DBTW length (provided in SIB1).

- When DBTW length is not provided, UE assumes that DBTW length is a half frame (includes all candidate SSB positions). Therefore, UE assumes that DBTW is enabled prior to reading SIB1.

Proposal 7: UE should be able to identify DBTW enable/disable before acquiring SIB1. A specific entry of NSSBQCL in MIB should be reserved to disable DBTW if the number of candidate SSB positions is larger than 64.

Proposal 8: In operation with shared spectrum in 60 GHz, for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst:

- if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted;

- if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

Proposal 9: Regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense in ssb-PositionsInBurst configured in SIB1, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

Proposal 10:  For CORESET for Type0-PDCCH in 52.6GHz to 71GHz spectrum, support the following:

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, support multiplexing pattern 1 and multiplexing pattern 3 as per Agreement in RAN1 104-e.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz, support multiplexing pattern 1 only.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {960, 960} kHz, support multiplexing pattern 1 only.

Proposal 11: For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, in addition to the supported values of (NRBCORESET, NsymbCORESET ) from Rel-15, support NRBCORESET= 96 with NsymbCORESET={1,2} for multiplexing pattern 1.

Proposal 12: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={120, 120} kHz:

- For CORESET#0 with 24 RBs and 48 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: additional RB offsets values of 0 and 28 RBs can be considered for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets values of 0 and 76 RBs can be considered for multiplexing pattern 1.

- Note: All above RB offsets are nominal and may need to be modified after finalizing synch raster and channel raster design in FR2-2.

Proposal 13: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

- For CORESET#0 with 24 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: In addition to the offset of 14 RBs already supported in Rel-16, additional values of 0 and28 RBs can be considered for multiplexing pattern 1.

Proposal 14: The parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 listed in Table 4 and Table 5 should be supported. For 480kHz and 960 kHz SCS, the scaling factor X in Table 5 is 12μ-3 when DBTW is OFF and 12μ-4 when DBTW is ON.

- Note: DBTW OFF is indicated in MIB using a value of NSSBQCL

Proposal 15: To find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum, RAN1 should uniquely determine the hypothetical on-synch raster SSB that serves as the reference for the offset to the off-synch raster SSB in case more than one synch rasters are included in a channel bandwidth.

Proposal 16: the MIB content and PBCH payload in Table 6 and Table 7 should be supported for 120 kHz, 480 kHz and 960 kHz SSB.

Proposal 17: Additionally support L=571 for 480 kHz PRACH.

Proposal 18: For 480 kHz and 960 kHz PRACH, support one gap symbol between consecutive ROs in time domain at least for Formats A1, B1, and A1/B1.

Proposal 19: For PRACH format A1, B1 and A1/B1, the first symbols for each RO in a reference slot can be derived using equation (2) if a gap symbol between consecutive ROs is introduced. The same triples of l0,NtRA,slot,NdurRA   in Table 6.3.3.2-4 of 38.211 are reused.

- Note: Equation (2) guarantees that no RO straddles between slots nslotRA-1 and nslotRA.

Proposal 20: For the same RO density per reference slot as 120 KHz PRACH, the RA-RNTI corresponding to 480 kHz and 960 kHz ROs can be generated according to equation (5) by compressing the t_id to tid2μ-3.

- When some ROs are backward shifted to the immediately preceding slot of the specified slot nslotRA due to the use of a gap symbol between consecutive ROs, support a 1 bit indication field in the DCI scheduling RAR/MsgB to resolve the PRACH slot ambiguity.

Proposal 21: Support maximum of 40 ms for ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum. Support indicating two LSBs of SFN at which gNB has received msg1 (MsgA) in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI).

- References

- Chair’s Notes RAN1#106-e

- R1-2108589, “Summary #6 of email discussion on initial access aspect of NR extension up to 71 GHz”, Moderator (Intel Corporation)

- RP-211584, “Revised WID:  Extending current NR operation to 71 GHz” RAN #92-e, Qualcomm, Intel.

- TS 38.213.

- ETSI EN 301 893.

- ETSI EN 302 567.

- TS 38.331.

- R4-2107985, LS on 60GHz time-related issues, Apple

- R4-2107972, WF on 60 GHz Time-related issues, Apple