3GPP TSG RAN WG1 Meeting #106bis-e	R1-2108776

E-meeting, October 11th – 19th, 2021

Title:	Draft reply LS on SCell dropping issue of CA

Response to:	R1-2108701/R4-2114950

Release:	Rel-17

Work Item:	NR_RF_FR1_enh-Core

Source:	Huawei [RAN WG1]

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

#### Name:	Frank Long

E-mail Address:	frank.longyi@huawei.com

Attachments:	-

1. Overall Description:

RAN1 thanks RAN4 for the LS on SCell dropping issue of CA. Regarding the questions raised in the LS, RAN1 provides answers as follows.

Question 1: Whether UE drop Scell power according to the priority rule defined in 38.213 is considered as an issue from RAN1 perspective.

Answer to Q1: RAN1 only focuses on the priority order of uplink transmissions, which will result in the issue of UE SCell dropping. However, the issue cannot be avoided by a gNB scheduling due to lack of real-time PHR information of UEs. The issue is expected to be resolved.

Question 2: Whether UE drop Scell power according to the priority rule defined in 38.213 has been addressed from 16 or 17? If not, what expected solution(s) are?

Answer to Q2: No, additional power balance mechanism for PCell and SCell can be introduced to prevent SCell dropping.

Question 3: If the problem above is solved in RAN4 specifications with solution by higher layer configuration, e.g. introduce additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping (see e.g. R4-2112826 or R4-2114551 for details), is there any expected RAN1 spec impact or possible conflict with UE behaviour defined in RAN1 specifications?

Answer to Q3: If the additional UE-specific configuration should be enabled/disabled by a MAC-CE, which slot should be referred to when a MAC CE command becomes effective should be specified from RAN1 perspective.

2. Actions:

To: RAN4

ACTION: 	RAN1 respectfully requests RAN4 to take the above responses into account in their further work.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#107	11 - 19 November 2021   	Online