3GPP TSG-RAN WG1 Meeting #106bis-e	Tdoc R1-2108830

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.1.2

Source:	Ericsson

Title:	CSI Feedback Enhancements for IIoT/URLLC

Document for:	Discussion

# Introduction

Rel-17 IIoT/URLLC working has been approved and scope was revised recently [1]. Among other objectives, the first objective says:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering:

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

At RAN1#106-e meeting the following was agreed:

Agreement

For subband CQI reporting with more than 2 bits per subband

- Support 4-bits CQI only

Agreement

For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration.

- This feature is subject to UE capability

- FFS: Whether wideband CQI report can be omitted

In this paper we present our view on the open issue for CSI feedback enhancements and suggest text changes in RAN1 specifications.

# Discussion

If UE is configured to report 4-bit sub-band CQI (SB-CQI) the open issue is whether UE additionally reports wide-band CQI (WB-CQI). A CQI value deduced by UE is with respect to CSI reference resource, which in frequency domain is the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. According to our understanding this means that a SB-CQI index for a sub-band n is with respect to a PDSCH transport block (TB) transmitted on the physical resource blocks corresponding the sub-band n while a WB-CQI index is with respect to a TB transmitted over all sub-bands. Also, the wideband CQI and sub-band CQI may be conditioned on different precoding matrices W(i). Hence, even if the radio quality is precisely the same over all sub-band the SB-CQI indices and the WB-CQI index may differ due to that the TB sizes differ. We believe that such a difference depends on UE implementation capability/performance and may hence be different for different UEs.

Even if radio quality is same over all sub-bands, SB-CQI and WB-CQI indices may differ due to different TB sizes reflecting the CQI indices, different precoding matrices, etc. Further, the difference may depend on UE implementation capability/performance.

Based on our observation we believe it is beneficial to report WB-CQI in addition to 4-bit SB-CQI since this will increase gNB knowledge how quality measures of individual sub-bands are averaged to a quality over multiple sub-bands. Thus, even when the 4-bit SB-CQI is reported, WB-CQI should not be removed from the mapping order of CSI fields in the CSI report. We therefore propose:

The UE report WB-CQI in addition to the 4-bit SB-CQI.

In the sections below, we provide detailed specification changes to enable the 4-bit subband CQI reporting.

# Proposed changes to 38.212

In our opinion only straightforward changes is required to implement support for 4-bit SB-CQI into 38.212 specification [2]. The word “differential” should be removed and the “hard-coded” values for number of bits for SB-CQI need to be a variable, e.g. nSB-CQI. For clarity reasons we are open to add a sentence clarifying that if nSB-CQI has the value 2, then sub-band CQI is a sub-band differential CQI.

Below we highlight in red the changes we believe is required:

======================== Start of changes to 38.212 ========================

6.3.1.1.2	CSI only

< Unchanged text is omitted >

The bitwidth for PMI with 1 CSI-RS port is 0.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel is provided in Tables 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | 0 |  |  |  |  |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2nSB-CQI | 2nSB-CQI | 2nSB-CQI | 2nSB-CQI | 2nSB-CQI |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2nSB-CQI |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214].  is the value of the rank. nSB-CQI is equal to <cqi-BitsPerSubband> if provided, otherwise nSB-CQI=2. The value of  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for RI/LI/CQI/CRI of codebookType= typeI-MultiPanel is provided in Table 6.3.1.1.2-4.

Table 6.3.1.1.2-4: RI, LI, CQI, and CRI of codebookType=typeI-MultiPanel

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2nSB-CQI |
| CRI |  |

where  is the number of allowed rank indicator values according to Clause 5.2.2.2.2 [6, TS 38.214],  is the value of the rank, and  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. nSB-CQI is equal to <cqi-BitsPerSubband> if provided, otherwise nSB-CQI=2.

The bitwidth for RI/LI/CQI of codebookType= typeII or codebookType=typeII-PortSelection is provided in Table 6.3.1.1.2-5.

Table 6.3.1.1.2-5: RI, LI, and CQI of codebookType=typeII or typeII-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2nSB-CQI |
| Indicator of the number of non-zero wideband amplitude coefficients for layer |  |

where  is the number of allowed rank indicator values according to Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and  is the value of the rank. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. nSB-CQI is equal to <cqi-BitsPerSubband> if provided, otherwise nSB-CQI=2.

< Unchanged text is omitted >

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.1.1.2-10: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

Note:	Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0.

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

< Unchanged text is omitted>

##### 6.3.2.1.2	CSI

< Unchanged text is omitted>

The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR reporting is provided in Table 6.3.1.1.2-8A. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, CRI/SINR, or SSBRI/SINR reporting.

Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4/6, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ as in Table 6.3.2.1.2-8, if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.2.1.2-5: Mapping order of CSI fields of one CSI report, CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

Note:	Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0.

< Unchanged text is omitted>

The bitwidth for RI/CQI of codebookType= typeII-r16 or codebookType=typeII-PortSelection-r16 is provided in Table 6.3.2.1.2-8.

Table 6.3.2.1.2-8: RI and CQI of codebookType=typeII-r16 or typeII-PortSelection-r16

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2nSB-CQI |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5 and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. nSB-CQI is equal to <cqi-BitsPerSubband> if provided, otherwise nSB-CQI=2.

======================== End of changes to 38.212 ========================

# Proposed changes to 38.214

38.214 [3] need to clarify that if UE is configured with 4-bit SB-CQI, then reported SB-CQI is an index from the configured CQI table. Below we highlight in red the changes we believe is required:

======================== Start of changes to 38.214 ========================

< Unchanged text is omitted>

#### 5.2.2.1	Channel quality indicator (CQI)

The CQI indices and their interpretations are given in Table 5.2.2.1-2 or Table 5.2.2.1-4 for reporting CQI based on QPSK, 16QAM and 64QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-3 for reporting CQI based on QPSK, 16QAM, 64QAM and 256QAM.

< Unchanged text is omitted>

If <cqi-BitsPerSubband> is provided with a value 4, then the reported sub-band CQI is the CQI index in Table 5.2.2.1-2 (‘table1’), Table 5.2.2.1-3 (‘table2’) or Table 5.2.2.1-4 (‘table3’) as configured by the higher layer parameter cqi-Table in CSI-ReportConfig. Otherwise, the reported sub-band CQI is a 2-bit sub-band differential CQI.

For each sub-band index s, a 2-bit sub-band differential CQI is defined as:

-	Sub-band Offset level (s) = sub-band CQI index (s) - wideband CQI index.

The mapping from the 2-bit sub-band differential CQI values to the offset level is shown in Table 5.2.2.1-1

Table 5.2.2.1-1: Mapping sub-band differential CQI value to offset level

| Sub-band differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | ≥ 2 |
| 3 | ≤-1 |

< Unchanged text is omitted>

======================== End of changes to 38.214 ========================

# Conclusion

Based on the discussions above, the following observations were made:

Observation 1	Even if radio quality is same over all sub-bands, SB-CQI and WB-CQI indices may differ due to different TB sizes reflecting the CQI indices, different precoding matrices, etc. Further, the difference may depend on UE implementation capability/performance.

Furthermore, the following proposals were made:

Proposal 1	UE reports WB-CQI together with the 4-bit SB-CQI.

# References

RP-210854, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia. RAN#91e, March 2021.

3GPP TS 38.212, Multiplexing and channel coding, v16.7.0

3GPP TS 38.214, Physical layer procedures for data, v16.7.0