3GPP TSG RAN WG1 Meeting #106bis-e	R1-2108858

e-Meeting, October 11th – 19th, 2021

Source: 	ZTE

Title: 	Discussion on UE features for enhanced IIoT and URLLC

Agenda item:	8.17.3

Document for:	Discussion/Decision

Introduction

Analysis on UE features for enhanced IIoT and URLLC are presented. The discussion is based on [1].

Discussion

Retransmission of cancelled HARQ

Index 25-4:

Index 25-4 is for One-shot HARQ ACK feedback triggered by DCI format 1_2. The components consists of two parts:

1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH

2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value.

RAN1 has the agreement that “Support Rel-16 Type 3 HARQ-ACK CB triggering using DCI format 1_2 in Rel-17 for a UE supporting DCI format 1_2”. But there is no clear agreement on supporting feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH or without scheduling a PDSCH. Although it is well known that feedback of Rel-16 type 3 HARQ-ACK codebook has been supported, triggered by a DCI 1_1 scheduling a PDSCH or without scheduling a PDSCH, it is natural to copy the behaviour to new DCI format 1_2. But should we explicitly indicate this in the table of UE feature after we have achieved the consensus or agreement. By now we suggest removing the description of DCI 1_2 can scheduling a PDSCH or without scheduling a PDSCH.

Proposal 1: The following adjustment is proposed for 25-4.

1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value

Index 25-6 and 25-7:

The two indices are separately for the Enhanced type 3 HARQ-ACK codebook feedback and Triggered HARQ-ACK codebook re-transmission. The common things are both supporting DCI format 1_2 and two HARQ-ACK codebooks / PUCCH config. The feature of supporting DCI format 1_2 is 11-1 and the feature of supporting two HARQ-ACK codebooks / PUCCH config is 11-4. So we are wondering why we not include 11-1 and 11-4 both in the prerequisite feature column of index 25-6 and 25-7.

Proposal 2: Include 11-1 and 11-4 both in the prerequisite feature group column of index 25-6 and 25-7.

Also in 25-7, the component of description for Triggered HARQ-ACK codebook re-transmission is not clear. The “earlier PUCCH slot” is not clear to aim the cancelled HARQ-ACK codebook, below adjustment can clarity the retransmission is for the cancelled HARQ-ACK codebook, the cancellation of the HARQ-ACK codebook is due to various reasons, such as conflicts with the HP channel or Dl symbols, or cancellation based on CI. So we propose:

Proposal 3: The following adjustment is proposed for component of 25-7.

- 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot of the cancelled HARQ-ACK based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) - 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4)

PUCCH carrier switching

Index 25-9 and 25-10:

The type definition from moderator on index 25-9 and 25-10 is “Per UE”. But we think “Per BC” is better for the two feature group. For a certain UE, not all the band combination is suitable for PUCCH switching, for example, one UE could switch between carrier 1 and carrier 2, but carrier 3 is not allowed for switching. So the feature of supporting Semi-static PUCCH carrier switching or dynamic PUCCH carrier switching seems more suitable as Per BC definition.

Proposal 4: The type of the feature group 25-9 and 25-10 is proposed to change to Per BC.

UE Initiating a semi-static channel occupancy

Index 25-12 and 25-13:

For index 25-12, moderator has raised a question on whether this feature group should be optional. From our understanding, in Rel-16, the prerequisite feature group 10-1a “UL channel access for semi-static channel access mode” is optional, so index 25-12 should also be an optional UE feature. In addition, the description of “[This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode]” seems simply copied from Rel-16 NRU UE feature, but the scenarios may need to be restricted in Rel-17 unlicensed band URLLC. Another simpler way is to remove the description in [], since a UE without such feature group could still work and there is no such basic operation description in all other Rel-17 URLLC feature groups.

Regarding index 25-13, according to the previous agreement, in semi-static channel access mode, for a UE supporting UE-initiated COT, by default an FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT, so index 25-12 should be the prerequisite feature group of index 25-13.

Proposal 5: The following modifications are proposed for feature group 25-12 and 25-13:

- Remove the description in [] of “Mandatory/Optional” of FG 25-12

- Add 25-12 in the “Prerequisite feature groups” description of FG 25-13.

Intra-UE Multiplexing/Prioritization

Index 25-17

For the usage of parentheses in component of 25-17, we should keep the commonality on each part of the components. So we propose to remove the parentheses for item 1 and 2.

Proposal 6: The following adjustment is proposed for component of 25-17.

1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI.

Simultaneous x-CC PUCCH/PUSCH transmissions for inter-band CA

Index 25-18:

This feature group is supporting parallel PUCCH and PUSCH transmission across CCs in inter-band CA. The type of this feature group is proposed to “Per BC”. The reason is similar with proposal 4 as not all band combination is supporting parallel PUCCH and PUSCH transmission.

Proposal 7: The type of the feature group 25-18 is proposed to change to Per BC.

Conclusion

Proposal 1: The following adjustment is proposed for 25-4.

1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value

Proposal 2: Include 11-1 and 11-4 both in the prerequisite feature group column of index 25-6 and 25-7.

Proposal 3: The following adjustment is proposed for component of 25-7.

- 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot of the cancelled HARQ-ACK based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) - 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4)

Proposal 4: The type of the feature group 25-9 and 25-10 is proposed to change to Per BC.

Proposal 5: The following modifications are proposed for feature group 25-12 and 25-13:

- Remove the description in [] of “Mandatory/Optional” of FG 25-12

Add 25-12 in the “Prerequisite feature groups” description of FG 25-13.

Proposal 6: The following adjustment is proposed for component of 25-17.

1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI.

Proposal 7: The type of the feature group 25-18 is proposed to change to Per BC.

Reference

R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.)