3GPP TSG RAN WG1 #106bis-e		R1-2108869

e-Meeting, October 10th – 19th, 2021

Title: 	Initial evaluation results for XR

Source: 	CEWiT, IITM, IITB, Saankhya Labs

Agenda item:	8.14.1

Document for:          Discussion and decision

# Introduction

In the previous RAN1 meetings, from #103-e to 106-e, many agreements were made on the XR traffic model, evaluation methodologies and simulation assumptions. Based on these agreements, we perform system level simulations for single stream XR.

In this contribution, we present our initial performance evaluation results for XR.

# Simulation Assumptions, Traffic Model and KPI

In this section we summarize the simulation assumptions, traffic mode and KPI for XR capacity evaluation.

## Simulation Assumptions

In the RAN1#106-e meeting[2], agreements were made to capture the initial performance evaluation results for capacity.

Agreement

- For capturing the observations for capacity for XR, following aspects are considered.

  - Baseline capacity performance

  - Various parameters/modeling on capacity performance, including the capacity performance with different assumptions, capacity performance with multi-stream traffic model, etc.

  - FFS: Potential enhancement on capacity performance

- For capturing the observations for various parameters/modeling on capacity performance, the following could be considered

  - Data-rate on capacity

  - PDB/PER on capacity

  - Multi-stream traffic on capacity

  - Jitter impact on capacity

  - Etc.

FFS: For capturing the observations from the evaluation results for potential enhancement on capacity performance

Based on the above agreement, we perform evaluation of the baseline XR capacity for Downlink. The baseline evaluation parameters for single stream DL video were agreed in RAN1#105-e[3].  For our simulations we consider DL AR/VR and CG video stream with data rate of 30Mbps at 60 fps. The agreed PDB value for AR/VR is 10ms and for CG the PDB value is 15ms. The baseline traffic models are summarised in Table 1.

Table 1 Summary of baseline single stream model

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |

## Traffic Model

The various parameters for the packet generation were agreed in RAN1#104bis-e meeting[4]. The packet size follows a Truncated Gaussian distribution. The mean packet size is derived based on the date rate and the fps values.  Other required packet size parameters can be calculated using the mean value. For our evaluation purpose, we use consider the [STD, Max, Min] to be [10.5, 150, 50] % of the mean packet size.

The arrival time of the packets is quasi-periodic due the presence of jitter.  For jitter we consider the baseline option with mean 0ms, STD 2ms and range [-4, 4]ms. The detailed traffic model parameters are given in Table 2.

Table 2 Single Stream DL XR Traffic model parameters

| Traffic model Parameter | CG | VR/AR |
| --- | --- | --- |
| Data rate | baseline: 30Mbps | baseline: 30Mbps |
| PDB | baseline: 15ms | baseline: 10ms |
| Frame per second | baseline: 60fps |  |
| Packet size | Truncated Gaussian distribution for packet size baseline: [STD, Max, Min]: [10.5, 150, 50] % of Mean packet size |  |
|  | Parameter | Value (Bytes) |
|  | Mean | 65536 |
|  | STD | 6881 |
|  | Max | 98304 |
|  | Min | 32768 |
| Jitter | J is drawn from a truncated Gaussian distribution baseline: Mean: 0 ms; STD: 2 ms; Range: [-4, 4] ms |  |

## KPI for XR Capacity Evaluation

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

According to RAN1#103-e meeting[6] the system capacity is defined as,

For our evaluation we consider the value of X = 90, that is the system capacity is maximum number of users with at least 90 % of users being satisfied.

For calculating the percentage of satisfied users, we use the RAN1 agreement made in RAN1#104bis-e meeting [4] that states that a user is declared satisfied if more that 99 % of packets are successfully derived with the given air interface PDB of 10ms for AR/VR and 15ms for CG applications.  Here the 99% is the baseline value that was agreed in the meeting.

## Other Simulation Assumptions

While performing the system level simulations for XR capacity evaluation for FR1, we assume that users are distributed unevenly across the cells. This results in an uneven load across the cells, but the average load across the simulation remains fixed.

Another factor to consider is the synchronization of arrival time of packets for different users or the traffic arrival offset. In our simulations we consider the traffic arrival offset to be zero, that is, initially, the packet arrival is synchronized for different users.

Also, the scheduler we consider is the SU-MIMO PF scheduler.

# Initial Evaluation Results

In this section we present our initial evaluation results for FR1 downlink XR capacity in Dense Urban and Urban Macro scenarios. The simulations assumptions are summarized in Appendix A.

For all the figures, the x-axis denotes the average number of users per cell and the y-axis denotes the percentage of satisfied users per cell. We have performed evaluations for average user per cell = {2, 4, 6, 8 10}.

## Capacity Results for Dense Urban

Figure 1 shows the percentage of satisfied users per cell for Dense Urban scenario for AR/VR and CG applications.

Figure 1 Average users per cell vs Percentage of satisfied users for FR1 Dense Urban scenario

From the figure, we can observe that for Dense Urban scenario, the system capacity for single stream AR/VR is 4 users per cell and single stream CG is 5 users per cell. For AR/VR, with 4 users per cell, 90.48% of users are satisfied. In case of CG, with 5 users per cell, 93.65% of users are satisfied.

Observation 1: For FR1 Dense Urban scenario, the system capacity for AR/VR, with PDB 10ms, is 4 users per cell for single stream XR video with 30Mbps data rate at 60fps.

Observation 2: For FR1 Dense Urban scenario, the system capacity for CG, with PDB 15ms, is 5 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

## Capacity Results for Urban Macro

Figure 2 shows the percentage of satisfied users per cell for Urban Macro scenario for AR/VR and CG applications.

Figure 2 Average users per cell vs Percentage of satisfied users for FR1 Urban Macro scenario

From the figure, we can observe that for Urban Macro scenario, the system capacity for single stream AR/VR is 2 users per cell and single stream CG is 4 users per cell. For AR/VR, with 2 users per cell, 97.62% of users are satisfied. For CG, with 4 users per cell, 90.48% of users are satisfied.

Observation 3: For FR1 Urban Macro scenario, the system capacity for AR/VR, with PDB 10ms, is 2 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

Observation 4: For FR1 Urban Macro scenario, the system capacity for CG, with PDB 15ms, is 4 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

# Conclusions

In this contribution we presented our initial evaluation results for XR. Based on our system level simulation we have the following observations.

Observation 1: For FR1 Dense Urban scenario, the system capacity for AR/VR, with PDB 10ms, is 4 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

Observation 2: For FR1 Dense Urban scenario, the system capacity for CG, with PDB 15ms, is 5 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

Observation 3: For FR1 Urban Macro scenario, the system capacity for AR/VR, with PDB 10ms, is 2 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

Observation 4: For FR1 Urban Macro scenario, the system capacity for CG, with PDB 15ms, is 4 users per cell for single stream XR video traffic with 30Mbps data rate at 60fps.

# References

RP-201145, Revised SID on XR Evaluations for NR, RAN meeting #88e, June 29 – July 3, 2020

Chairman notes, RAN1#106-e, Aug, 2021.

Chairman notes, RAN1#105-e, May, 2021.

Chairman notes, RAN1#104b-e, April, 2021.

Chairman notes, RAN1#104-e, February, 2021.

Chairman notes, RAN1#103-e, November, 2020.

# Appendix A: System Simulation Parameters

| Parameter | Value |
| --- | --- |
| Deployment | Dense urban with single layer of Marco layer refers to TR 38.913 Urban Macro refers to TR 38.913 |
| Channel model | For Dense urban: - Uma refers to TR 38.901 For Urban Macro: - Uma refers to TR 38.901 |
| Layout | For Dense urban: - 21 cells with wraparound, ISD = 200m For Urban Macro: - 21 cells with wraparound, ISD = 500m |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| System bandwidth | Baseline: 100 MHz |
| TDD configuration | Option 1: DDDSU (S: 10D:2F:2U) |
| BS Tx power | 49 dBm per 20 MHz |
| BS antenna parameters | For Dense Urban/Urban Macro scenario: - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) - (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna parameters | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| BS height | 25 m |
| UE height | For Dense Urban/Urban Macro scenario: - Outdoor UEs: 1.5 m - Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | Omni-directional, 0 dBi |
| Noise figure | BS: 5 dB, UE: 9dB |
| Downtilt | For Dense urban: - 12 degree For Urban Macro: - 6 degree |
| UE distribution | For Dense Urban/Urban Macro scenario: - 80% indoor, 20% outdoor |
| UE speed | 3 km/h |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| MCS | Up to 256QAM |
| Transmission scheme | Closed Loop MIMO |
| Scheduler | SU-MIMO PF |
| CSI acquisition | Realistic |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 |
| gNB processing delay | 3 Slots |
| PDCCH overhead | 1/7 |
| DMRS overhead | 2 symbols |
| Target BLER | 10% |
| Max HARQ transmission | 3 |