3GPP TSG RAN WG1 Meeting #106bis-e                                                                R1-2108875

e-Meeting, October 11th – 19th, 2021

Source:              ZTE

Title:                   Enhancements on SRS flexibility, coverage and capacity

Agenda Item:     8.1.3

Document for:   Discussion and Decision

Introduction

In RAN1#106-e, the following agreements [1] were achieved in the scope of Rel-17 FeMIMO SRS enhancement [2].

Agreement #1

Confirm the following WA:

For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple   candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Agreement #2

Support start RB location (Noffset) hopping in different SRS frequency hopping periods for RPFS and at least periodic/semi-persistent SRS, where Noffset is the start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs.

- For a given SRS transmission occasion, Noffset=kF+khopping mod PFPFmSRS, BSRS , where khopping is same for all SRS occasions within a legacy FH period but changes across legacy FH periods, kF and PF are at least configured by RRC signaling (kF = {0, 1, …, PF-1}).

- Support at least one pattern for khopping in time domain, FFS detailed pattern

- Note: the legacy FH period is the period to sound the full SRS hopping bandwidth across the different subbands of mSRS, BSRS RBs each.

- This start RB location hopping is enabled or disabled by RRC signaling.

- FFS whether MAC CE or DCI can be additionally used

- When this start RB location hopping is disabled,khopping khopping is fixed to be 0 for all SRS symbols

- This start RB location hopping is UE optional.

- FFS whether start RB location hopping is also applicable on SRS occasion(s) within one FH period (e.g., when R>1) and/or on aperiodic SRS, if so, how

Agreement #3

For aperiodic xTyR antenna switching SRS, where xTyR is from {1T6R, 1T8R, 2T6R, 2T8R, 4T8R}, support all the non-zero integer values N<=N_max except N=1 for 1T8R

- For each xTyR configuration, UE does not expect multiple SRS resource sets are configured or triggered in one slot

- UE does not expect that the OFDM symbols contained in one SRS resource set exceed UE capability on which OFDM symbols can be used for SRS taking guard period into account

Agreement #4

Support Opt. 2: Reference slot is the slot indicated by the legacy triggering offset.

- If DCI is transmitted in slot n, and k is the legacy triggering offset, reference slot is slot n+k.

- Note: the legacy triggering offset can be 0, if slotOffset is absent.

Conclusion #1

MAC CE for t value update in Rel-17 is not supported.

Agreement #5

For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS.

- Note: the two SP-SRS resource sets are not activated at the same time

- For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS

- Applies for all supported xTyR where y<=8

- For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources.

This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS

Agreement #6

- Support 4T6R SRS antenna switching in Rel-17.

Agreement #7

For RPFS SRS sequence generation, support

- Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence.

Agreement #8

For SRS increased repetitions in Rel-17, support the following configurations, and no other values are supported.

- (N_symbol, R) = {(8, 1), (8, 2), (8, 4), (8, 8), (12, 1), (12, 2), (12, 3), (12, 4), (12, 6), (12, 12), (10, 1), (10, 2), (10, 5), (10,10), (14, 1), (14, 2), (14, 7), (14, 14)}

- Note: N_symbol SRS symbols are adjacent in a slot.

Agreement #9

- On the presence of guard symbols in Rel-17 for SRS antenna switching, down-select one of the following

  - Alt 1-0: Guard symbols are always-on, which is same as Rel-15

  - Alt 1-1: Guard symbols are configurable subject to UE capability

- On whether to introduce guard symbols between SRS resource sets for antenna switching, down-select one of the following

  - Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set

  - Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots

- Note: Rel-15 guard period symbols are supported if none of the above enhancements is agreed

Agreement #10

For Comb-8 SRS in Rel-17, down-select one of the following in RAN1#106bis-e

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

In this contribution, we discuss further technical details on the SRS enhancements starting from the agreements we have made so far.

Enhancements on aperiodic SRS triggering

NR Rel-15 supports gNB to trigger aperiodic SRS resource set(s) with DCI, to enable dynamic UL/DL channel measurement. However, the current specification support for this feature is quite restrictive. The restriction will lead to PDCCH congestion, unnecessary DCI/RS overhead and/or unnecessary UE power consumption. It’s needed to enhance network’s flexibility when triggering the aperiodic SRS.

More flexible determination of SRS triggering offset

In previous RAN1 meetings, the general framework of SRS triggering offset enhancement has been decided. To complete this feature, the following remaining issues need to be solved.

- SRS collision handling among multiple CCs.

- Remaining details on DCI indication of the offset t values.

SRS collision among multiple CCs

A set of UE feature groups 22-5a, 5b, 5c and 5d have been introduced in Rel-15. These FGs indicate whether simultaneous transmission between SRS for antenna switching and SRS for CB/NCB/BM/AS is supported for intra-band CA or inter-band CA cases. For UE without such capabilities, UE is not able to support simultaneous transmission between SRS in different CCs. In Rel-15, it is a hard limit as there is no rule for UE to handle the case for SRS collision in multiple CCs. Once gNB configure or trigger such collided SRS in different CCs, neither of the SRS in multiple CCs can be transmitted, as UE assumes it as an error case.

The current mechanism to determine the available slot of aperiodic SRS transmission may cause higher chance of SRS collision among different CCs. For example, in Fig. 2, aperiodic SRS in two different CCs collides in time based on available slot rule, so SRS in neither CC can be transmitted. However, if aperiodic SRS in CC1 is triggered to be transmitted in an unavailable slot based on Rel-15 method, SRS in CC1 cannot be transmitted while SRS in CC2 can be transmitted. This issue is more severe in intra-band CC, where slot format configuration should be same for different CCs. Hence to avoid all the SRS transmissions in different CCs are omitted, the hard limit for SRS simultaneous transmission in multiple CCs causes large restriction on gNB’s scheduling.

Fig. 1 An example of SRS collision in multiple CCs in the case of determining available slots to transmit SRS

Observation 1: The current mechanism to determine the available slot of aperiodic SRS transmission may cause higher chance of SRS collision among different CCs.

- The current UE capability definition only depends on gNB’s configuration to avoid SRS collision among different CCs, which causes large restriction gNB’s scheduling.

- Once collision happens, none of the SRS in these CCs can be transmitted.

A simple solution to solve such issue is to define a rule to prioritize SRS transmission in one CC and omit SRS in other CCs. Then SRS transmission in one CC can be reserved, which is better than the hard limit from NW scheduling perspective. Further, it is better to inherit the per-symbol collision handling rule in Rel-15, i.e., collision handling happens only on the overlapping symbols.

Proposal 1: Order SRS in different CCs based on a priority rule, and drop the SRS transmission with lower priority in the overlapping symbol(s) when collision happens for UEs not supporting simultaneous SRS transmission among different CCs.

DCI indication of t

A unified approach for scheduling DCI and non-scheduling DCI to indicate t has been agreed in Agreement #1. Specifically, the following has been decided.

- A list of up to 4 t values can be configured for each SRS resource set.

- Add a new configurable DCI field “SRS offset indication” (SOI) to indicate t for both scheduling DCI and non-scheduling DCI. The bit width of SOI is up to 2 bits.

Hence it is straight-forward that for a DCI that triggers multiple SRS resource sets, if the SOI field in this DCI has B<=2 bits, and it indicates a b-th state, the b-th t value configured in the list of t is used for each of the triggered SRS resource sets, where 1<=b<=2B. Further, as different SRS resource sets may be configured with different sizes of t list based on gNB’s implementation, the bit width of SOI field should depend on the maximum number of t values configured for any of the SRS resource sets. If the indicated state by SOI exceeds the number of t values configured for an SRS resource set, it is natural to use a simple rule to determine the used t value, e.g., use the last t value configured for this SRS resource set. Hence, we have the following proposal.

Proposal 2: For the newly added DCI field “SRS offset indication” (SOI) to indicate the value of t, the bit width of SOI is B<=2 bits if a maximum number of 2B t values are configured for any of the SRS resource sets.

- For each of the triggered SRS resource sets, if SOI field in the same DCI indicates a b-th state, the min(b, bS)-th t value configured in the list of t is used, where 1<=b<=2B, and bS is the number of t values configured for this resource set.

For non-scheduling DCI, it is straight-forward to re-purpose fields other the SRS request for t indication, so that DCI overhead does not need to be increased. The most straight-forward approach is to reuse the TDRA field, as each TDRA table entry can be associated with one or more t values in the configured list of t for each SRS resource set. Therefore, if gNB choose not to configure SOI to save DCI overhead, re-pursing some of the existing DCI field in non-scheduling DCI can still indicate the t values as mentioned above.

Proposal 3: If SOI field is not configured, the value of t can also be indicated by repurposing TDRA field in non-scheduling DCI (DCI format 0_1/0_2 without data and without CSI request).

Enhancements on SRS antenna switching

The general framework to support configuration of SRS antenna switching with {1T6R, 1T8R, 2T6R, 2T8R, 4T8R} has been established in previous RAN1 meetings. Further, the supported number of resource sets has been decided for each xTyR with aperiodic, periodic or semi-persistent time-domain type. The detailed configurations to support {1T6R, 1T8R, 2T6R, 2T8R, 4T8R} antenna switch can be described as follows based on the agreements so far.

- Aperiodic SRS

  - Support the following number of resource sets for {1T6R, 1T8R, 2T6R, 2T8R, 4T8R}. gNB can configure the number of resources in each resource set flexibly.

    - {1T6R}: 1, 2, 3 sets, where the total number of resources is 6, and each resource has 1 port

    - {1T8R}: 2, 3, 4 sets, where the total number of resources is 8, and each resource has 1 port

    - {2T6R}: 1, 2, 3 sets, where the total number of resources is 3, and each resource has 2 ports

    - {2T8R}: 1, 2, 3, 4 sets, where the total number of resources is 4, and each resource has 2 ports

    - {4T8R}: 1, 2 sets, where the total number of resources is 2, and each resource has 4 ports

- Periodic/semi-persistent SRS

  - Support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS

    - For xTyR (except for 4T6R), each resource has x ports and each resource set has y/x resources

The major remaining details include the following issues

- How to support 4T6R;

- Whether to support any enhancement for guard symbols.

Support of 4T6R

The ultimate goal of support 4T6R is to enable the UE implementation of having 4 transmission antennas and 6 receive antennas. Hence the configuration just needs to achieve such goal, and some over-design which causes higher RS overhead is not desirable.

Based on the above principle, we support the following configurations for 4T6R.

- Support 2 resources for 4T6R, where the first resource has 4 ports and the second resource has 2 ports.

  - For aperiodic SRS, the 2 resources can be configured in 1 set or 2 sets

  - For periodic/semi-persistent SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS, where each resource set contains the 2 resources.

Proposal 4: Support 2 resources for 4T6R, where the first resource has 4 ports and the second resource has 2 ports.

  - For aperiodic SRS, the 2 resources can be configured in 1 set or 2 sets

  - For periodic/semi-persistent SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS, where each resource set contains the 2 resources.

Guard symbols

Two aspects are related with possible enhancement for guard symbols, where the first is whether to support configurable presence of guard symbols, and the second is whether to further introduce guard symbols between resource sets.

For the first issue, as whether to have guard symbols between antenna switching resources really depends on UE implementation. If UE is capable of sending two SRS resources for antenna switching in two adjacent symbols without degrading antenna switching performance, the specification should allow the possibility of configuring the presence of guard symbols, so that gNB does not need to waste valuable air interface resources if guard symbols are not needed by UE.

Proposal 5: On the presence of guard symbols in Rel-17 for SRS antenna switching, support Alt 1-1: Guard symbols are configurable subject to UE capability.

The next issue is whether to introduce guard symbols between SRS resource sets. It has been agreed in Agreement #3 that UE does not expect multiple SRS resource sets are configured or triggered in one slot. Hence multiple SRS resource sets should be located in multiple slots. This issue is equivalent to whether to introduce guard symbols between SRS in two adjacent slots.

- If SRS can only be transmitted in the last 6 OFDM symbols in one slot as in Rel-15, there is no need to have guard symbols between SRS in two adjacent slots.

- If SRS can be transmitted in all OFDM symbols in one slot, it may need guard symbols if the SRS resources in two SRS resource sets are configured in adjacent symbols across two slots. Similarly as discussed for the previous issue, the appearance of the guard symbols in two adjacent slots should be configurable subject to UE capability.

Proposal 6: For guard symbols between two SRS resource sets

- If UE is only capable of transmitting SRS in the last 6 OFDM symbols in one slot, support Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set.

- If UE is capable of transmitting SRS in all symbols in one slot, support gNB to configure guard symbols between the two SRS resource sets if the two sets locate in two adjacent slots, subject to UE capability.

Enhancements on SRS coverage and capacity

Three schemes have been agreed in RAN1#104e to enhance SRS coverage and capacity in Rel-17, i.e., increase repetitions, RB-level partial frequency sounding (RPFS) and comb-8. Further, most of the details for these three schemes have been decided until RAN1#106-e. We discuss the remaining details to complete these three features in this section.

RB-level partial frequency sounding (RPFS)

Restriction on 1PFmSRS, BSRS values

One remaining issue is whether any restricted on 1PFmSRS, BSRS values is needed for Rel-17 RPFS as discussed since RAN1#104bis-e. One restriction RAN1 has agreed is new sequences or new sequence length shall be precluded for RPFS. For the agreed PF values, even PF = 4, this restriction can be satisfied when comb 2 is configured as the minimum length of SRS sequence supported by the current specification is 6. Hence there is no need to introduce any other restriction on 1PFmSRS, BSRS values. Note that 1PFmSRS, BSRS being an integer value does not bring any restriction as non-integer values do not work here. Further, if we restrict 1PFmSRS, BSRS as multiples of 4, the supported new patterns will be very limited as mSRS, BSRS has to be multiples of 8. The configurations to support mSRS, BSRS being multiples of 8 is just a few. Hence such restriction will defeat the benefit of introducing RPFS.

Proposal 7: For the restriction on 1PFmSRS, BSRS values, support Alt 1, i.e., no restriction other than it is an integer value.

Hopping pattern for start RB location hopping

Per Agreement #2, start RB location hopping can be enabled for RPFS under legacy FH period level. One remaining issue is the pattern of khopping in time domain.

For PF = 2, khopping={0,1} is the only possible pattern, where  khopping=0 for all the symbols in the n-th legacy FH period, khopping=1 for all the symbols in the (n+1)-th legacy FH period and n={1, 2, 3, …}.

For PF = 4, we think the best pattern is khopping={0, 2, 1, 3} so that the RB locations in the first two FH periods are distributed equally in each subband as shown in Fig. 2. This is beneficial to the performance of channel estimation interpolation after gNB receiving the first two legacy FH periods. This design principle is same as the legacy FH pattern.

Fig. 2 Pattern {0, 2, 1, 3} for khopping in time domain

Based on the above discussion, we have the following proposal.

Proposal 8: Support the following pattern of khopping in time domain when the start location hopping is enabled.

- For PF = 2, khopping={0,1}, where khopping=0 for all the symbols in the n-th legacy FH period, khopping=1 for all the symbols in the (n+1)-th legacy FH period and n={1, 2, 3, …}.

- For PF = 2, khopping={0, 2, 1, 3}, where khopping=0 for all the symbols in the n-th legacy FH period, khopping=2 for all the symbols in the (n+1)-th legacy FH period, khopping=1 for all the symbols in the (n+2)-th legacy FH period, khopping=3 for all the symbols in the (n+3)-th legacy FH period and n={1, 2, 3, …}.

Start RB location hopping for aperiodic SRS

Another remaining issue for start RB location hopping is whether it can be applied for aperiodic SRS. The benefit of having start RB location hopping is also valid for aperiodic SRS. The agreed approach based on legacy FH period level also works for aperiodic SRS.

- In some cases, e.g., the whole SRS BW contains two legacy frequency hops for 4 or more symbols in one SRS resource, multiple legacy FH periods still exists for aperiodic SRS. For these cases, the same start RB location hopping mechanism in Agreement #2 can be reused for aperiodic SRS.

- In the case that only one legacy FH hopping period exists in one SRS resources, no start RB location hopping will be applied. The start RB location equals to the configured kF for all the symbols in the aperiodic SRS resource.

Proposal 9: For aperiodic SRS, support the same start RB location hopping mechanism agreed for periodic and semi-persistent SRS.

Comb-8

For Comb-8, the remaining issues are not many since it has already been supported for positioning in Rel-16. One issue is the maximum number of CSs supported for Comb-8 as discussed in Agreement #10, where the following two alternatives have been identified.

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

We think the benefit of Alt 2 in terms of SRS capacity compared with Alt 1 is not clear as most of the supported SRS patterns will result in SRS sequences shorter than 12 for Comb-8. Further, Alt 2 needs to introduce more implementation and specification complexity to define rules for applicable CSs when SRS sequence is shorter than 12. Hence we have the following proposal.

For Alt 1, the remaining issue is how to support 4 ports. We can use the 8 comb offset values to distribute the two pairs of SRS ports. The first pair of ports can locate in comb offset value n1, and the second pair of ports can locate in comb offset value n2. Based on this approach, gNB can configure two comb offset values and two CS values for a 4-port resource with comb 8. Then the 4 ports can be implemented by similar CS allocation equation for two ports with high flexibility. The change on the current CS allocation equation is minimal.

Proposal 10: The maximum number of CSs for Comb-8 is 6.

- For a 4-port SRS resource, gNB configures two comb offset values and two CS values.

  - The first pair of ports locate in the first configured comb offset according to the first CS value, and the second pair of ports locate in the second comb offset according to the second CS value.

Conclusion

In this contribution, we discuss the remaining issues on SRS enhancements for Rel-17. We have the following observation and proposals.

Observation 1: The current mechanism to determine the available slot of aperiodic SRS transmission may cause higher chance of SRS collision among different CCs.

- The current UE capability definition only depends on gNB’s configuration to avoid SRS collision among different CCs, which causes large restriction gNB’s scheduling.

- Once collision happens, none of the SRS in these CCs can be transmitted.

Proposal 1: Order SRS in different CCs based on a priority rule, and drop the SRS transmission with lower priority in the overlapping symbol(s) when collision happens for UEs not supporting simultaneous SRS transmission among different CCs.

Proposal 2: For the newly added DCI field “SRS offset indication” (SOI) to indicate the value of t, the bit width of SOI is B<=2 bits if a maximum number of 2B t values are configured for any of the SRS resource sets.

- For each of the triggered SRS resource sets, if SOI field in the same DCI indicates a b-th state, the min(b, bS)-th t value configured in the list of t is used, where 1<=b<=2B, and bS is the number of t values configured for this resource set.

Proposal 3: If SOI field is not configured, the value of t can also be indicated by repurposing TDRA field in non-scheduling DCI (DCI format 0_1/0_2 without data and without CSI request).

Proposal 4: Support 2 resources for 4T6R, where the first resource has 4 ports and the second resource has 2 ports.

  - For aperiodic SRS, the 2 resources can be configured in 1 set or 2 sets

  - For periodic/semi-persistent SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS, where each resource set contains the 2 resources.

Proposal 5: On the presence of guard symbols in Rel-17 for SRS antenna switching, support Alt 1-1: Guard symbols are configurable subject to UE capability.

Proposal 6: For guard symbols between two SRS resource sets

- If UE is only capable of transmitting SRS in the last 6 OFDM symbols in one slot, support Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set.

- If UE is capable of transmitting SRS in all symbols in one slot, support gNB to configure guard symbols between the two SRS resource sets if the two sets locate in two adjacent slots, subject to UE capability.

Proposal 7: For the restriction on 1PFmSRS, BSRS values, support Alt 1, i.e., no restriction other than it is an integer value.

Proposal 8: Support the following pattern of khopping in time domain when the start location hopping is enabled.

- For PF = 2, khopping={0,1}, where khopping=0 for all the symbols in the n-th legacy FH period, khopping=1 for all the symbols in the (n+1)-th legacy FH period and n={1, 2, 3, …}.

- For PF = 2, khopping={0, 2, 1, 3}, where khopping=0 for all the symbols in the n-th legacy FH period, khopping=2 for all the symbols in the (n+1)-th legacy FH period, khopping=1 for all the symbols in the (n+2)-th legacy FH period, khopping=3 for all the symbols in the (n+3)-th legacy FH period and n={1, 2, 3, …}.

Proposal 9: For aperiodic SRS, support the same start RB location hopping mechanism agreed for periodic and semi-persistent SRS.

Proposal 10: The maximum number of CSs for Comb-8 is 6.

- For a 4-port SRS resource, gNB configures two comb offset values and two CS values.

  - The first pair of ports locate in the first configured comb offset value, and the second pair of ports locate in the second comb offset value.

References

[1] Chairman notes, RAN1#106-e

[2] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung