3GPP TSG RAN WG1 #106b-e                                              R1-2108885

e-Meeting, October 11th - 19th, 2021

Source:         ZTE

Title:            UE features for NR positioning enhancements

Agenda item:    8.17.5

Document for:	Discussion and Decision

Introduction

We have made some progresses over the past several RAN1 meetings according to the WID agreed in RAN#91e meeting [1]. As usual, we need to define new UE features for the enhancements that are approved in Rel-17 NR positioning. In this contribution, we provide our preliminary views on the UE features for Rel-17 NR positioning based on the features list provided by WI rapporteurs in R1-2108679 [2].

UE features for Rel-17 NR positioning

2.1 Mitigation of UE Rx/Tx timing delays

| 27-x1 | Mitigation of UE Rx timing delays | The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values for DL TDOA and/or Multi-RTT positioning |
| --- | --- | --- |

For the mitigation of UE RX timing delays, the following aspects should be considered for the maximum number of UE-RxTEG supported by UE.

- DL measurements from different bands may experience different timing error since the group delay is frequency dependent. Hence, this UE feature should be provided per band.

- The number of the UE-RxTEG should be considered from both frequency domain and spatial domain, e.g., UE can support up to 4 positioning frequency layers and 4 ports. So, the candidate values can be {1, 2, 4, 8, 16}

- Value=1 is to indicate that the timing error differences between all DL measurements are within a certain margin

- Some of the Rel-16 UE features are defined per positioning method (e.g. max number of DL PRS Resources). To follow the same principle, we propose to have separate values for DL TDOA and/or Multi-RTT positioning.

Proposal 1: For the mitigation of UE RX timing delays, maximum number of UE-RxTEG supported by UE is defined as following,

- This UE feature is provided per band.

- The candidate values are {1, 2, 4, 8, 16}

- Value=1 is to indicate that the timing error differences between all DL measurements are within a certain margin

- Have separate values for DL TDOA and/or Multi-RTT positioning

| 27-x2 | Mitigation of UE Tx timing delays | The maximum number of UE-TxTEG per UE, which is supported and reported by UE for UL TDOA and/or Multi-RTT positioning FFS; the values (>1). FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FF: whether a UE supports different values for UL TDOA and/or Multi-RTT positioning |
| --- | --- | --- |

Similarly, for the mitigation of UE TX timing delays, the following aspects should be considered for the maximum number of UE-TxTEGs supported by UE.

- UL transmissions from different bands may experience different timing errors since the group delay is frequency dependent. Hence, this UE feature should at least be provided per band.

- The number of the UE-TxTEG should be considered from both frequency domain and spatial domain, e.g., UE can support up to 8 contiguous CCs per band and up to 4 ports per UE. So, the candidate values can be {1, 2, 4, 8, 16, 32}

- Value=1 is to indicate that the timing error differences between all UL transmissions are within a certain margin

- The positioning SRS is irrelevant to which positioning method is used. So, there is no need to have separate values for DL TDOA and/or Multi-RTT positioning.

Proposal 2: For the mitigation of UE TX timing delays, maximum number of UE-RxTEG supported by UE is defined as following,

- This UE feature is provided per band

- The candidate values are {1, 2, 4, 8, 16, 32}

- Value=1 is to indicate that the timing error differences between all UL transmissions are within a certain margin

- There is no need to have separate values for DL TDOA and/or Multi-RTT positioning.

| - x3 | Mitigation of UE RxTx timing delays | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning FFS; the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated |
| --- | --- | --- |

For the mitigation of UE Rx+TX timing delays, the following aspects should be considered for the maximum number of UE-RxTxTEG supported by UE.

- DL measurements and UL transmissions from different bands may experience different timing errors since the group delay is frequency dependent. So, this UE feature should be provided per band.

- The candidate values should take into account DL measurements and UL transmissions, which at least should be larger than the number of UE-RxTEG and UE-TxTEG. So, the candidate values can be {1, 2, 4, 8, 16, 32, 64, [128]}

- Value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin

Proposal 3: For the mitigation of UE RX+TX timing delays, maximum number of UE-RxTxTEG supported by UE is defined as followings,

- This UE feature is provided per band

- The candidate values are {1, 2, 4, 8, 16, 32, 64, [128]}

- Value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin

2.2 DL-AOD enhancements

| 27-z1 | Support of PRS RSRP of the first path | UE’s capability to support providing the PRS RSRP of the first path |
| --- | --- | --- |
| 27-z2 | Support of DL PRS RSRP reporting for more than 8 measurements. | UE capability to support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Limit the maximum number of DL PRS RSRP associated with the same Rx beam index |

On the details of the above 27-z1 and 27-z2, we have the following suggestion.

Proposal 4: Support to keep 27-z1 and 27-z2 with the following details,

- 27-z1 and 27-z2 are provided per UE

- The candidate values of 27-z2 are {12, 16}

- Need for location server to know if these two features are supported.

2.3 Latency reduction

| 27-u1 | Support of M-sample measurements. | The capability to support providing a measurement based on measuring M samples (instances) of a DL PRS resource set M=[1, 4]. FFS: other values. If the UE does not provide the capability, the UE is assume to support M=4 only. |
| --- | --- | --- |

Proposal 5: Support to keep 27-u1 with the following details

- This UE feature is provided per UE

- The candidate values are {1, 2, 3, 4}

- Need for location server to know if the feature is supported.

For 27-u5, we suggest the following update to clarify that DL PRS with higher priority for PRS measurement should be contained in a PRS processing priority window. Further, within a PRS processing window, UE measurement should be inside the active DL BWP with PRS having the same numerology as the active DL BWP.

Proposal 6: Update 27-u5 based on the following

| 27-u5 | PRS measurement outside MG and in a PRS processing priority window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing priority window. Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 2B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2C refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note 1: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. Note 2: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP |
| --- | --- | --- |

- This UE feature is provided per UE

- Need for location server to know if the feature is supported.

- FFS: Whether this feature should also be provided to gNB

On 27-u6: DL PRS processing capability

As shown in the formula in TS 38.133 for measurement period of a location information report, the component Tlast is the measurement duration for the last PRS sample, including the sampling time and processing time,

Tlast = Ti + Tavailable_PRS,i

The Tlast is to consider the cases that PRS resources from different sets are not concentrated on the same MG instance or PRS resources appear in the end of the processing window. This component leads to additional latency for the sampling and processing of the last PRS sample. As we are trying to reduce the latency as much as possible, it’s not acceptable to take additional time after the end of the PRS processing window. We propose not to reuse the UE DL PRS processing capability inside measurement gap as shown below,

| 27-u6 | PRS Processing Capability outside MG | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |
| --- | --- | --- |

In order to reduce the latency for DL PRS measurement in the PRS processing window, the location information report should be ready right after the end of the PRS processing window. That is, UE has to finish all the DL PRS receiving and computation in the PRS processing window to make full use of its resources. For Type 1 PRS processing capability showing in the Figure 1 below, a PRS processing window is divided into PRS buffering window and PRS computation window. UE is only expected to receive the DL PRS in the PRS buffering window. Then, based on the buffered DL PRS, UE can compute/process the DL PRS in PRS computation window to get ready for a location information report by the end of PRS processing window as shown in Fig. 1. According to this understanding, UE has to report its capability with at least one combination of {R, P} considering the following.

- A PRS processing window is divided into PRS buffering window and PRS computation window. The PRS computation window starts right after the end of the PRS buffering window. UE is only expected to receive the DL PRS in the PRS buffering window.

- UE shall take P msec of time (the length of PRS computation window) to process up to N msec of symbols containing PRS resources expected to be received by the UE in the PRS buffering window

Fig. 1 Type 1 PRS processing capability

There could be another UE implementation (Type 2 PRS processing capability). UE may not need to buffer all the DL PRS before starting processing the DL PRS. That is, UE can do DL PRS receiving and processing simultaneously as shown in Fig. 2. Therefore, UE only needs to reserve enough time to process the latest DL PRS resource used for the location information report, which is quite similar to the CSI reference resource defined for CSI report. PRS computation time (T) is understood by the following,

- A time span (the value of N shown below) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window

- The value of N is not expected to be smaller than the PRS computation time (T) .

Fig. 2 Type 2 PRS processing capability

Proposal 7: For the UE capability design for DL PRS measurements in a PRS processing window, at least consider one of the following Types,

- Type 1 PRS processing capability: UE reports its capability with at least one combination of {R, P},

- A PRS processing window is divided into PRS buffering window and PRS computation window. The PRS computation window starts right after the end of the PRS buffering window. UE is only expected to receive the DL PRS in the PRS buffering window.

- UE shall take P msec of time (the length of PRS computation window) to process up to R msec of symbols containing PRS resources expected to be received by the UE in the PRS buffering window

- Type 2 PRS processing capability: UE reports its capability of PRS computation time (T)

- A time span (N) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window

- The value of N is not expected to be smaller than the PRS computation time (T).

2.4 NLOS and Multipath mitigation

| 27-v1 | LOS/NLOS Indicator | UE’s capability to support reporting LoS/NLoS indicator to LMF for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning. FFS: whether to have separate capability component for RSTD and UE Rx-Tx time difference measurements. |
| --- | --- | --- |

We have the following suggestions on 27-v1.

Proposal 8：Support to keep 27-v1 with the following details.

- This UE feature is provided per UE.

- Have separate capability components for RSTD and UE Rx-Tx time difference measurements.

- Need for location server to know if the feature is supported.

2.5 On-demand PRS

| 27-w1 | Support of on-demand PRS | UE’s capability to support UE-initiated on-demand PRS. |
| --- | --- | --- |

Due to the feature of UE initiated on-demand PRS contains two aspects and they can work independently based on UE’s implementations. Hence we have the following proposal.

Proposal 9：For UE’s capability reporting to support UE-initiated on-demand PRS

- Separate the UE features for UE-initiated on-demand PRS on pre-configured DL PRS and UE-initiated on-demand PRS to request the parameters of DL PRS

- These two UE features are provided per UE.

- Need for location server to know if these two feature is supported.

- Based on the above understanding, we suggest to revise 27-w1 to the following.

| 27-w1-1 | Support of on-demand PRS on pre-configured DL PRS | UE’s capability to support UE-initiated on-demand PRS on pre-configured DL PRS. |
| --- | --- | --- |
| 27-w1-2 | Support of on-demand PRS to request the parameters of DL PRS | UE’s capability to support UE-initiated on-demand PRS to request the parameters of DL PRS |

Conclusions

In this contribution, we discuss the UE features for Rel-17 NR positioning, the following proposals are proposed,

Proposal 1: For the mitigation of UE RX timing delays, maximum number of UE-RxTEG supported by UE is defined as following,

- This UE feature is provided per band.

- The candidate values are {1, 2, 4, 8, 16}

- Value=1 is to indicate that the timing error differences between all DL measurements are within a certain margin

- Have separate values for DL TDOA and/or Multi-RTT positioning

Proposal 2: For the mitigation of UE TX timing delays, maximum number of UE-RxTEG supported by UE is defined as following,

- This UE feature is provided per band

- The candidate values are {1, 2, 4, 8, 16, 32}

- Value=1 is to indicate that the timing error differences between all UL transmissions are within a certain margin

- There is no need to have separate values for DL TDOA and/or Multi-RTT positioning.

Proposal 3: For the mitigation of UE RX+TX timing delays, maximum number of UE-RxTxTEG supported by UE is defined as followings,

- This UE feature is provided per band

- The candidate values are {1, 2, 4, 8, 16, 32, 64, [128]}

- Value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin

Proposal 4: Support to keep 27-z1 and 27-z2 with the following details,

- 27-z1 and 27-z2 are provided per UE

- The candidate values of 27-z2 are {12, 16}

- Need for location server to know if these two features are supported.

Proposal 5: Support to keep 27-u1 with the following details

- This UE feature is provided per UE

- The candidate values are {1, 2, 3, 4}

- Need for location server to know if the feature is supported.

Proposal 6: Update 27-u5 based on the following

| 27-u5 | PRS measurement outside MG and in a PRS processing priority window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing priority window. Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 2B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2C refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note 1: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. Note 2: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP |
| --- | --- | --- |

- This UE feature is provided per UE

- Need for location server to know if the feature is supported.

- FFS: Whether this feature should also be provided to gNB

Proposal 7: For the UE capability design for DL PRS measurements in a PRS processing window, at least consider one of the following Types,

- Type 1 PRS processing capability: UE reports its capability with at least one combination of {R, P},

- A PRS processing window is divided into PRS buffering window and PRS computation window. The PRS computation window starts right after the end of the PRS buffering window. UE is only expected to receive the DL PRS in the PRS buffering window.

- UE shall take P msec of time (the length of PRS computation window) to process up to R msec of symbols containing PRS resources expected to be received by the UE in the PRS buffering window

- Type 2 PRS processing capability: UE reports its capability of PRS computation time (T)

- A time span (N) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window

- The value of N is not expected to be smaller than the PRS computation time (T).

Proposal 8：Support to keep 27-v1 with the following details.

- This UE feature is provided per UE.

- Have separate capability components for RSTD and UE Rx-Tx time difference measurements.

- Need for location server to know if the feature is supported.

Proposal 9：For UE’s capability reporting to support UE-initiated on-demand PRS

- Separate the UE features for UE-initiated on-demand PRS on pre-configured DL PRS and UE-initiated on-demand PRS to request the parameters of DL PRS

- These two UE features are provided per UE.

- Need for location server to know if these two feature is supported.

- Based on the above understanding, we suggest to revise 27-w1 to the following.

| 27-w1-1 | Support of on-demand PRS on pre-configured DL PRS | UE’s capability to support UE-initiated on-demand PRS on pre-configured DL PRS. |
| --- | --- | --- |
| 27-w1-2 | Support of on-demand PRS to request the parameters of DL PRS | UE’s capability to support UE-initiated on-demand PRS to request the parameters of DL PRS |

References

- RP-210903 Revised WID on NR Positioning Enhancements, Intel Corporation, CATT

- R1-2108679 Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.)