3GPP TSG RAN WG1 #106b-e                                                     R1-2108909

E-Meeting, October 11th – 19th, 2021

Agenda Item:	8.4.1

Source:	Spreadtrum Communications

Title:	Discussion on timing relationship enhancements for NTN

Document for:	Discussion and decision

Introduction

In this contribution, we share our views on timing relationship enhancements for NR NTN.

K_offset in initial access

K_offset value determination

In RAN 104b_e meeting, the following agreement on cell-specific K_offset in system information had been achieved [1].

Agreement: - For determination of cell-specific K_offset in system information, down-select one option from below: - Option 1: Signal one offset value for K_offset - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

For Option 1, one offset value indicated by system information for K_offset is expected to cover the RTT of service link plus the RTT between serving satellite and reference point. It is a very simple way of indicating cell-specific K_offset in system information and can provide a certain configuration flexibility. Furthermore, Option 1 would be sufficient for both DL-UL alignment and DL-UL non-alignment cases.

For Option 2, cell-specific K_offset is determined by two offset values, the first offset value is determined by common TA, and the second offset value is expected to cover RTT of service link winch is indicated by system information. In RAN1#105e, the following FL recommendation is made [2].

Moderator recommendation on Issue #2: Companies are encouraged to rethink about their position and come back to this issue at the next RAN1 meeting, taking the following into account. - The signaling overhead saving in Option 2 vs. Option 1 is only about 1 bit. - The ~1-bit saving might appear free, but comes at the cost of many disadvantages (more complexity for UE, more specification impact, more sources of inaccuracies, scheduling restriction, etc.) - 1 bit saving in higher layer signaling is negligible.

The signaling overhead saving in Option 2 compared with Option 1 is only about 1 bit, 1 bit saving in higher layer signaling is negligible. Furthermore, in Option 2, different UEs may calculate the value of K_offset in different time instant based on the common TA drift rate that leads to different results of configuration of first value of K_offset which are supposed to be common. Therefore, there would be a misalignment between gNB and UE about the value of K_offset.

In our view, one offset value indicated by system information for K_offset is cover the RTT of service link plus the RTT between serving satellite and reference point should be supported.

Proposal 1: One offset value indicated by system information for K_offset is cover the RTT of service link plus the RTT between serving satellite and reference point.

The range of K_offset

In RAN1#106, the following agreement on the range of K_offset had been achieved [3].

Agreement: For defining value range(s) of K_offset, down-select one option from below: - Option 1: One value range of K_offset covering all scenarios. - Option 2: Different value ranges of K_offset for different scenarios.

The value of K_offset depends on the maximum possible TA compensation value in the cell or beam coverage area. TA compensation value is related to the altitude of the satellite and the position of the reference point. For GEO, assume the max RTT between UE and gNB is 541.46 ms, the value range of K_offset could be 238 ms~542 ms. In our view, different value range can be defined based on different scenarios, for example, GEO, MEO, or LEO-600, or LEO-1200. Therefore, we have the following proposal:

Proposal 2: Different value ranges of K_offset for different scenarios should be supported.

The unit of K_offset

In RAN1#106, the following agreement on the unit of K_offset had been achieved [3].

Agreement: The unit of K_offset is number of slots for a given subcarrier spacing. - FFS: one subcarrier spacing value or different subcarrier spacing values for different scenarios.

Considering that the value range of K_offset is different in different scenarios, if all scenarios use the same unit of K_offset, the signaling overhead for K_offset indication of different scenarios will be different. In our view, different subcarrier spacing values for different scenarios to determine K_offset should be supported to maintain the same overhead for K_offset indication.

Proposal 3: Different subcarrier spacing values for different scenarios to determine K_offset should be supported.

Beam-specific K_offset in initial access

In RAN1#103e, it was agreed that at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported. Beam specific K_offset configured in system information and used in initial access is FFS [4].

Agreement: - For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported. - FFS: Beam specific K_offset configured in system information and used in initial access.

Since a cell in the NTN scenario may contain multiple beams and the coverage area of the cell is relatively large, the cell-specific value of K_offset configuration will bring more serious end-to-end latency. In this case, beam-specific values of K_offset configuration can achieve a good trade-off between end-to-end latency and signaling overhead.

Proposal 4: Beam-specific values of K_offset configuration for initial access should be supported.

K_offset update

In RAN1#106, it has agreed that the UE-specific K_offset can be provided and updated by network with MAC CE and the necessity of the RRC reconfiguration method for updating UE-specific K_offset need to FFS [3].

Agreement: - The UE-specific K_offset can be provided and updated by network with MAC CE. - FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration - FFS: Details on whether and how the two solutions work together

Considering that it has agreed that the UE-specific K_offset can be provided and updated by network with MAC CE, we prefer MAC CE only solution for a common framework for different scenarios as anyway MAC CE can work.

Proposal 5: RRC reconfiguration based solution for K_offset update is not needed.

MAC CE timing relationships

In RAN1#106_e, the following agreement on MAC CE timing relationships has been achieved [3].

Agreement: The information of K_mac is carried in system information.

In this section, we share our further views on MAC CE timing relationships.

Update the information of K_mac

The K_mac represents the RTT between the RP and gNB. Based on the loaction of the reference point, there are the following 3 case.

- Case 1: When the reference point is configured at the satellite, the value of unalignment will be equal to the feeder link RTT, i.e., the value of K_mac  is equal to the feeder link RTT.

- Case 2: When the reference point is configured at feeder link, the value of unalignment will be equal to the RTT between the RP and gNB, i.e., the value of K_mac  is equal to the RTT between the RP and gNB, which is smaller than the feeder link RTT.

- Case 3: When the reference point is configured at service link, the value of unalignment will be equal to the RTT between the RP and the satellite plus the feeder link RTT, i.e., the value of K_mac is equal to the RTT between the RP and the satellite plus the feeder link RTT, which is larger than the feeder link RTT.

For Case 3, if the reference point is configured at service link, negative common TA signalling need to be introduced which should be avoided. Reference point at service link should be deprioritized. The maximum value of unalignment is equal to the feeder link RTT, where the reference point is configured at the satellite. Since the TA values over service link and feeder link will change with the movement of the satellite, an update mechanism for the value of K_mac need to be supported. In our view, K_mac is cell-specific and can be update by the system information.

Proposal 6: K_mac can be update by the system information.

The unit of K_mac

In RAN1#106_e, the following agreement on the unit of K_mac had been achieved [3].

Agreement: The unit of K_mac is number of slots for a given subcarrier spacing. FFS: one subcarrier spacing value or different subcarrier spacing values for different scenarios.

Considering that the value range of K_mac is different in different scenarios, if all scenarios use the same unit of K_mac, the signaling overhead for K_mac indication of different scenarios will be different. In our view, different subcarrier spacing values for different scenarios to determine K_mac should be supported to maintain the same overhead for K_mac indication.

Proposal 7: Different subcarrier spacing values for different scenarios to determine K_mac should be supported.

PDCCH ordered PRACH

In RAN1#106_e, the follow agreement on PDCCH ordered PRACH had been achieved [3].

Agreement: For random access procedure initiated by a PDCCH order received in downlink slot n, UE determines the next available PRACH occasion after uplink slot n+Koffset to transmit the ordered PRACH. - Note: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc, where NTA=0 is assumed for PDCCH ordered PRACH. - FFS: Which value of Koffset should be applied - FFS: Whether the n+Koffset timing relationship is impacted by UE behavior within or after the validity duration.

Regarding of which value of Koffset should be applied, our view is that either cell specific K_offset or UE specific K_offset can be applied to enhance the timing of PDCCH ordered PRACH. When the UE specific K_offset is configured, UE should apply the UE-specific K_offset to determine the next available PRACH occasion, otherwise, the cell-specific K_offset should be applied.

Proposal 8: Either cell specific K_offset or UE specific K_offset can be applied to enhance the timing of PDCCH ordered PRACH.

Conclusion

As summary, we have the following proposals.

Proposal 1: One offset value indicated by system information for K_offset is cover the RTT of service link plus the RTT between serving satellite and reference point.

Proposal 2: Different value ranges of K_offset for different scenarios should be supported.

Proposal 3: Different subcarrier spacing values for different scenarios to determine K_offset should be supported.

Proposal 4: Beam-specific values of K_offset configuration for initial access should be supported.

Proposal 5: RRC reconfiguration based solution for K_offset update is not needed.

Proposal 6: K_mac can be update by the system information.

Proposal 7: Different subcarrier spacing values for different scenarios to determine K_mac should be supported.

Proposal 8: Either cell specific K_offset or UE specific K_offset can be applied to enhance the timing of PDCCH ordered PRACH.

Reference

- 3GPP RAN1#104b_e Chairman’s Notes, April 12 - April 20, 2021.

- R1-2106325, “Feature lead summary#5 on timing relationship enhancements”, Moderator (Ericsson).

- 3GPP RAN1#106_e Chairman’s Notes, August 16th – 27th, 2021.

- 3GPP RAN1#103_e Chairman’s Notes, October 26 - November 13, 2020.