3GPP TSG RAN WG1 #106b-e                                                      R1-2108911

E-Meeting, October 11th – 19th, 2021

Agenda Item:	8.4.3

Source:	Spreadtrum Communications

Title:	Discussion on enhancements on HARQ for NTN

Document for:	Discussion and decision

Introduction

In RAN#86, Rel-17 Work Item on Solutions for NR to support non-terrestrial networks was approved with the following RAN1 objectives

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified:

- Timing relationship enhancements[RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

In this paper, we discuss our views on enhancements on HARQ.

Enhancement on HARQ process number

In RAN1#106e, the follow agreements on HARQ process ID indication has been achieved [1].

Agreement: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-1/1-1 when the maximum supported HARQ processes number is configured as 32. Agreement: For enhancement on the HARQ process indication, one of following options for DCI 0-0/1-0 can be considered: - Option 2: Reusing one bit from other bit field - Option 4: No enhancement

Considering that fallback DCI is temporarily used in initial access procedure and RRC reconfiguration procedure, 16 HARQ processes are enough for fallback DCI 0_0/1_0. In our view, no enhancement is needed for DCI 0-0/1-0.

Proposal 1: For the HARQ process indication, no enhancement is needed for DCI 0-0/1-0.

HARQ codebook enhancement

- Type-1 HARQ codebook

For Type-1 HARQ codebook, the following agreement had been achieved [1].

Agreement: - For Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, one of following options should be supported: - Option-1: The UE will report NACK only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH - Option-2: The UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH - FFS: Other cases, e.g., if only DCI carrying feedback-disabled HARQ process is detected by UE Agreement: For Type-1 HARQ codebook, if only DCI carrying feedback-disabled HARQ process is detected by UE, one of following options should be supported: - Option-1: The UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE - Option-2: The UE should skip the codebook feedback at least when the feedback is carried by PUCCH - FFS: the case that feedback is carried by PUSCH.

For Type-1 HARQ codebook, since type-1 HARQ-ACK codebook is semi-static, UE does not need to reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback. Furthermore, due to the potential misdetection of DCI, there is potential ambiguity at gNB side to interpret the feedback. In our view, no enhancement is needed for the Type-1 codebook. Therefore, for Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, the UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH.

Proposal 2: For Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, The UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH.

For Type-1 HARQ codebook, the occasions when only feedback-disabled HARQ processes are carried by DCI are rare. There is no need to introduce a “codebook dropping” rule for this case. Therefore, if only DCI carrying feedback-disabled HARQ process is detected by UE, the UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE.

Proposal 3: If only DCI carrying feedback-disabled HARQ process is detected by UE, the UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE.

- Type-2 HARQ codebook

In RAN1#106_e, the following agreement on Type-2 codebook in NTN had been achieved [1].

Agreement: For the DCI of PDSCH with feedback-disabled HARQ processes, only one of following is supported for Type-2 codebook: - Option-1: The C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE for type 2 codebook generation. - Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

For Option-1, the values of C-DAI and T-DAI can be used as assistance information for UE to know the miss detection of PDCCH scheduling the HARQ feedback enabled PDSCH for some cases. Misalignment between gNB and UE due to misdetection can be avoided by Option 1. Therefore, we have the following proposal:

Proposal 4: For the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE for type 2 codebook generation.

- Type-3 HARQ codebook

Type-3 HARQ codebook is already supported for licensed spectrum. We first need to clarify the use case Type-3 HARQ codebook in NTN scenario. In our view, if Type-3 HARQ codebook is supported in NTN, reduce codebook size with HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes should be considered.

Proposal 5: Enhancement for Type-3 HARQ codebook should be considered, if it is supported in NTN.

Conclusion

As summary, we have the following proposals.

Proposal 1: For the HARQ process indication, no enhancement is needed for DCI 0-0/1-0.

Proposal 2: For Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, The UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH.

Proposal 3: If only DCI carrying feedback-disabled HARQ process is detected by UE, the UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE.

Proposal 4: For the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE for type 2 codebook generation.

Proposal 5: Enhancement for Type-3 HARQ codebook should be considered, if it is supported in NTN.

Reference

- 3GPP RAN1#106_e Chairman’s Notes, August 16th – 27th, 2021.