3GPP TSG RAN WG1 #106bis-e                           	                   R1-2108933

e-Meeting,  October 11th – 19th, 2021

Agenda Item:	8.17.6

Source:	Spreadtrum Communications

Title:	UE features for RedCap

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss the RAN1 related UE features for RedCap.

# Feature group x-1: Reduced bandwidth

For RedCap UE, the maximum bandwidth is reduced. The related objectives in the WID [1] are listed as follows.

- Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

These are baseline capability of the maximum bandwidth for RedCap UEs.

## Components of FG x-1

Based on the above, the components of FG x-1 are listed below.

- The maximum channel bandwidth 20MHz for FR1

- The maximum channel bandwidth 100MHz for FR2

The notes for FG x-1 are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 1: Define FG x-1 for the maximum bandwidth for RedCap UEs.

# SSB presence in a UE-specific RRC configured BWP

It is proposed in [2] to define two capabilities for operation with or without SSB in a UE-specific BWP similar as FG 6-1 and FG 6-1a. Operation without SSB in a UE-specific BWP needs the RedCap UEs to retune RF autonomously for AGC/sync/measurement, and thus it is a separate capability.

FG 6-1 and FG 6-1a are listed as follows.

FG 6-1 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell FG 6-1a BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

The FG 6-1 like for RedCap UEs could be defined on top of FG 6-1.

BW of a UE-specific RRC configured BWP includes BW of CORESET associated to CSS (if CORESET associated to CSS is present) and SSB, where CSS is SIB1, OSI, RAR or paging CSS

The FG 6-1a like for RedCap UEs could be defined on top of FG 6-1a:

BW of UE-specific RRC configured BWP may not include BW of the CORESET associated to SIB1, OSI, RAR or paging CSS (if SIB, OSI, RAR or paging CSS is present) and SSB

## Components of FG x-2 for operation with SSB

Based on the above, the components of FG x-2 on top of FG 6-1 are listed below.

- BW of a UE-specific RRC configured BWP includes BW of CORESET associated to CSS (if CORESET associated to CSS is present) and SSB, where CSS is SIB1, OSI, RAR or paging CSS

The notes for FG x-2 are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 2: Define FG x-2 on top of FG 6-1 for the operation with SSB in a UE-specific BWP for RedCap UEs.

## Components of FG x-2a for operation without SSB

Based on the above, the components of FG x-2a on top of FG 6-1a are listed below.

- BW of UE-specific RRC configured BWP may not include BW of CORESET associated to CSS (if CORESET associated to CSS is present) and SSB, where CSS is SIB1, OSI, RAR or paging CSS

The notes for FG x-2a are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 3: Define FG x-2a on top of FG 6-1a for the operation without SSB in a UE-specific BWP for RedCap UEs.

## FGs like FG 6-2/6-3/6-4

The above FG x-2/x-2a is like FG 6-1/6-1a respectively to solve SSB presence issue. Like FG 6-2/6-3/6-4 (shown in Appendix A.1), some FGs can be defined if there is progress.

Proposal 4: Define FGs like FG 6-2/6-3/6-4 for further progress.

# Feature group x-3: Reduced to 1 Rx antenna/MIMO layer

For RedCap UE, the minimum number of Rx antennas and the maximum number of DL MIMO layers can be reduced to 1 or 2.

The related objectives in the WID [1] are listed as follows.

- Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

As agreed in the WI phase, the number of Rx antenna for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. When the parameter maxNumberMIMO-LayersPDSCH is absent, the UE support 1Rx and 1 MIMO layer.

On the other hand, there is no consensus to specify solutions to reduced PDCCH blocking caused by the large AL due to the reduced number of Rx antennas. Hence, the compact DCI formats (i.e. DCI 0_2/1_2) are still optional. In other words, DCI format 0_0/1_0 and DCI format 0_1/1_1 are mandatory for RedCap UEs as legacy.

## Components of FG x-3 for 1 Rx antenna

Based on the above, the component of FG x-3 for 1 Rx antenna are listed below.

- 1 Rx antenna in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antennas

- 1 Rx antenna in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 4 Rx antennas

- 1 MIMO layer

The notes for FG x-3 are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 5: Define FG x-3 for 1 Rx antenna/MIMO layer for RedCap UEs.

## Components of FG x-3a for 2 Rx antennas

Based on the above, the component of FG x-3a for 2 RX antennas are listed below.

- 2 Rx antennas in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 4 Rx antennas

- 2 Rx antennas in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antennas

- Up to 2 MIMO layers

The notes for FG x-3a are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 6: Define FG x-3a for 2 Rx antenna/MIMO layer for RedCap UEs.

# Feature group x-4: HD-FDD

RedCap UE can optionally support type A HD-FDD. The related objectives in the WID [1] are listed as follows.

- Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.

As discussed in the WI phase, there are two capabilities for RedCap to support HD-FDD, the first one is support of the DL-to-UL and UL-to-DL switching times, and the other is support of DL/UL collision solutions.

For switching times, according to RAN4’s confirmation [3], the existing switching times for FR1 for UE not capable of full duplex in TS 38.211, Table 4.3.2-3 can be reused for HD-FDD switching time.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3. Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx Transition time; FR1; FR2. NTx-Rx; 25600; 13792. NRx-Tx; 25600; 13792.

For collision solutions, RAN1 specified solutions for 7 potential collisions as follows.

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

- Case 8: Dynamic or semi-static DL vs. valid RO

- Case 9: Collision due to direction switching

Some of the above cases can be solved by current specs (in TS 38.213), while some cases need new solutions and new solutions are under discussion.

## Components of FG x-4

Based on the above, the component of FG x-4 are listed below.

- HD-FDD. The possible collision handling is TBD

The notes for FG x-4 are listed below.

- HD-FDD is FDD only feature

- No FR1/FR2 differentiation

Proposal 7: Define FG x-4 for HD-FDD for RedCap UEs.

# Feature group x-5: Relaxed modulation order

RedCap UE can optionally support DL 256QAM. The related objectives in the WID [1] are listed as follows.

- Relaxed maximum modulation order: - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE. - No other relaxations of maximum modulation order are specified for a RedCap UE.

During the WI phase, RAN1 focused on the MCS/CQI tables. We list some current conclusions for MCS/CQI tables initially.

- MCS tables

  - 64QAM MCS tables are mandatory.

  - 256QAM MCS tables are optionally supported according to 256QAM for PDSCH.

  - 64QAM low SE MCS table for PDSCH (Table 5.1.3.1-3 in TS 38.214) is optionally supported.

  - 64QAM low SE MCS tables for PUSCH (Table 5.1.3.1-3 in TS 38.214 for UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) are optionally supported.

- CQI tables

  - “CQI table 1” (Table 5.2.2.1-2 in TS 38.214) that corresponds to MCS Table 5.1.3.1-1 in TS 38.214 is mandatory.

  - “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) are optionally supported (256QAM MCS table).

  - “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) are optionally supported (64QAM low SE MCS table).

## Components of FG x-5

Based on the above, the component of FG x-5 are listed below.

- 256QAM for DL. The MCS/CQI tables related to 256QAM are optional.

The notes for FG x-5 are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 8: Define FG x-5 for relaxed modulation order for RedCap UEs.

# Feature group x-6: Early indication

The functionality of early indication for RedCap UEs is required to specify and the related objectives in the WID [1] are listed as follows.

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

In the RAN1#106-e meeting, the following agreement was achieved on early indication in Msg1 for 4-step RACH:

Agreement Confirm the following working assumption with the modifications in red: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled via SIB - FFS how to support enable/disable the early indication - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported) - separate PRACH resource - PRACH preamble partitioning - FFS: whether/how to address RA-RNTI overlapping issue - FFS the possibility of supporting Msg3 for the early indication

RAN2#115-e meeting has achieved an agreement on early indication for RedCap UEs in Msg3:

Agreements online: 1. A Msg3 early identification based on dedicated LCID is supported (if SA3 confirms there is no problem)

## Components of FG x-6

Based on the above, the components of FG x-6 are listed below.

- Early indication in Msg1

- Early indication in Msg3 based on dedicated LCID

The notes for FG x-6 are listed below.

- No FDD/TDD differentiation

- No FR1/FR2 differentiation

Proposal 9: Define FG x-6 for early indication for RedCap UEs.

# Conclusion

We have the following proposals.

Proposal 1: Define FG x-1 for the maximum bandwidth for RedCap UEs.

Proposal 2: Define FG x-2 on top of FG 6-1 for the operation with SSB in a UE-specific BWP for RedCap UEs.

Proposal 3: Define FG x-2a on top of FG 6-1a for the operation without SSB in a UE-specific BWP for RedCap UEs.

Proposal 4: Define FGs like FG 6-2/6-3/6-4 for further progress.

Proposal 5: Define FG x-3 for 1 Rx antenna/MIMO layer for RedCap UEs.

Proposal 6: Define FG x-3a for 2 Rx antenna/MIMO layer for RedCap UEs.

Proposal 7: Define FG x-4 for HD-FDD for RedCap UEs.

Proposal 8: Define FG x-5 for relaxed modulation order for RedCap UEs.

Proposal 9: Define FG x-6 for early indication for RedCap UEs.

The UE features for RedCap UEs can be summarized in the following table.

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. RedCap | x-1 | Maximum channel bandwidth | - The maximum channel bandwidth 20MHz for FR1. - The maximum channel bandwidth 100MHz for FR2. |  |  |  | No | Yes |  | Mandatory with capability signaling |
|  | x-2 | Basic BWP operation with restriction | BW of a UE-specific RRC configured BWP includes BW of CORESET associated to CSS (if CORESET associated to CSS is present) and SSB, where CSS is SIB1, OSI, RAR or paging CSS | 6-1 |  |  | No | No |  | Mandatory without capability signaling |
|  | x-2a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of CORESET associated to CSS (if CORESET associated to CSS is present) and SSB, where CSS is SIB1, OSI, RAR or paging CSS | 6-1a |  |  | No | No |  | Optional with capability signaling |
|  | x-3 | Antennaand MIMO layer | - 1 Rx antenna in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antennas - 1 Rx antenna in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 4 Rx antennas. - 1 MIMO layer |  | maxNumberMIMO-LayersPDSCH |  | No | No |  | Mandatory with capability signaling |
|  | x-3a | Antennaand MIMO layer | - 2 Rx antennas in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antennas. - 2 Rx antennas in the frequency bands where a legacy NR UE is required to be equipped with a minimum of 4 Rx antennas - Up to 2 MIMO layers |  | maxNumberMIMO-LayersPDSCH |  | Yes | No |  | Optional with capability signaling |
|  | x-3 | HD-FDD | HD-FDD. The possible collision handling is TBD |  |  |  | Yes (only applicable for FDD) | No |  | Optional with capability signalling. |
|  | x-4 | 256QAM for PDSCH | 256QAM for PDSCH |  |  |  | No | No | The MCS/CQI tables related to 256QAM are optional. | Optional with capability signalling |
|  | x-5 | Early indication | - Early indication in Msg1 - Early indication in Msg3 based on dedicated LCID |  |  |  | No | No |  | Mandatory without capability signalling |

# Reference

- RP-211574, “Revised WID on support of reduced capability NR devices”, 14th - 18th June 2021, RAN#92e.

- R1-2108913, “Discussion on aspects related to reduced maximum UE bandwidth”, Spreadtrum, RAN1#106bis-e, 11th – 19th October, 2021.

- R4-2114996, “Reply LS to Half-duplex FDD switching for RedCap UE”, 16th - 27th August 2021. RAN4#100e.

- TR 38.822 V16.0.0, “User Equipment (UE) feature list”, June 2021.

# A  Appendix

## A.1 FG 6-1/6-1a/6-2/6-3/6-4 in TR 38.822 [4]

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | n/a | This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signalling |
|  | 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | bwp-WithoutRestriction | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signalling |
|  | 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto2 in bwp-SameNumerology |  | Optional with capability signalling |
|  | 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-SameNumerology |  | Optional with capability signalling |
|  | 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-DiffNumerology |  | Optional with capability signalling |