3GPP TSG RAN WG1 #106bis-e                                                                          R1-2108934

e-Meeting, October 11th – 19th, 2021

Title:	Discussion on the initial access aspects for 52.6 to 71 GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.1

Document for:	Discussion and Decision

Introduction

In the RAN1 #106 e-meeting, the following agreements related to initial access were further reached [1]. Agreement:

- For 480 and 960kHz PRACH:

  - The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRAnslotRA , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

Agreement:

- For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, X} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- Alt 1: X = 8

- Alt 2: X = 9

Working assumption:

For 120kHz SSB, the number of candidates SSBs in a half frame is 64.

Agreement:

For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

Agreement:

For DBTW with 120kHz SCS (if supported), support DBTW lengths {0.5, 1, 2, 3, 4, 5} msec

- Note: this should be the same as Rel-16 NR-U DBTW lengths.

Agreement:

For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz,

- Support the following set of parameters.

| SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols |
| --- | --- | --- |
| 1 | 24 | 2 |
| 1 | 48 | 1 |
| 1 | 48 | 2 |

  - Note: the number of entries corresponding the same {mux pattern, number of RB, number of symbol} tuple (listed above) will depend on required RB offsets that needs to be supported based on channel and sync raster design.

- FFS: addition other set of parameters

Agreement:

Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH.

Agreement:

For 480 and 960kHz PRACH:

- At least the same RO density in time domain (i.e. number of specified RO per reference slot according the PRACH configuration index) as for 120kHz PRACH in FR2 is supported

  - FFS: Support gap between consecutive ROs in time domain and the details to derive the gap

Agreement:

For 480 and 960kHz PRACH,

- When a PRACH slot can contain all time domain PRACH occasions corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching,

  - and when number of PRACH slots in a reference slot is 1,

    - nslotRA=[7] for 480kHz and nslotRA=[15] for 960kHz PRACH

  - and when the number of PRACH slots in a reference slot is 2,

    - nslotRA=[3,7] for 480kHz and nslotRA=[7,15] for 960kHz PRACH

- FFS: nslotRA values, when a PRACH slot cannot contain all time domain PRACH occasions, corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching.

- FFS: whether to allow for additional nslotRA values if the maximum that can be configured for the number of FD RO’s is less than 8 (due to BW limitation)

In this contribution, we give our views on initial access aspects for NR between 52.6 GHz ~ 71 GHz, including SSB, Type0-PDCCH, PRACH, etc.

Discussion on initial access aspects

  - 2.1 SS/PBCH block and other initial access signals/channels

### 2.1.1 SSB pattern design for 480/960 kHz SCS

In addition to 120 kHz SCS that is supported in both initial and non-initial access cases, RAN1#104bis e-meeting has agreed to support 480 kHz and 960 kHz SCS for the SSB for the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH. After several rounds of discussion, RAN#92 e-meeting has finally agreed to support 480 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with some constraints. It can be said that the SCSs of SSB are finally determined.

RAN1 has agreed that SSB pattern in slots with 120 kHz SCS reuses the pattern of Case D and the number of candidates SSBs in a half frame is still 64. RAN1 has also agreed that first symbols of the candidate SSB have index {2, 9} + 14*n for 480 kHz and 960 kHz SCS, but has not determined the exact values of ‘n’ for each SCS. In order to avoid prolonged occupation by SSBs and leave time gaps between SSBs for the transmission of uplink and urgent services, some blank slots between multiple SSBs should be left. In addition, for shared spectrum operation, more than 64 candidate SSBs in a half frame can be defined for increasing the transmission opportunities of SSBs.

As shown in Figure 2.1.1-1, the following design can be considered for 480/960kHz SCS SSB: First symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- If DBTW (Discovery burst transmission window) is not supported or DBTW is disabled

  - For 480kHz SCS, the 64 candidate SSBs are located in 32 slots, with 2  slots spacing between every 8 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37.

  - For 960kHz SCS, the 64 candidate SSBs are located in 32 slots, with 4  slots spacing between every 16 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35.

- If DBTW is supported and it is enabled

  - Additional 64 candidate SSB can be defined after the above original 64 candidate SSBs in the half frame

Figure 2.1.1-1: Candidate SSBs with SCS 480/960 kHz in a half frame

Proposal 1: The following design of candidate SSBs with SCS 480/960 kHz in a half frame can be considered: First symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- If DBTW is not supported or DBTW is disabled

  - For 480kHz SCS, the 64 candidate SSBs are located in 32 slots, with 2  slots spacing between every 8 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37

  - For 960kHz SCS, the 64 candidate SSBs are located in 32 slots, with 4  slots spacing between every 16 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35

- If DBTW is supported and it is enabled

  - Additional 64 candidate SSB can be defined after the above original 64 candidate SSBs in the half frame

### 2.1.2 Discovery burst transmission window

For operation with shared spectrum channel access of NR 52.6~71 GHz, RAN1 #104bis e-meeting has agreed to support discovery burst (DB) and define the DB same as in Clause 4.0 of TS 37.213 in Rel-16. The Rel-16 discovery burst includes at least an SSB and may also include RMSI-CORESET, RMSI-PDSCH and/or NZP CSI-RS. In order to increase the transmission opportunities of discovery burst, it is also necessary to support configuring a discovery burst transmission window (DBTW).

In RAN1 #104 e-meeting, it has agreed that SSB transmission with LBT is supported at least when the conditions for contention exempt short control signaling based SSB transmission is not met. Wherein, Short control signaling is not a global unified solution and can only be applied in some region (e.g., Japan) where LBT is mandatory. Therefore, supporting DBTW is not only the requirement of 120 kHz SSB SCS, but also the requirement of other SSB SCSs if they are agreed to be supported.

Proposal 2: Discovery burst transmission window (DBTW) should be supported for all approved SSB SCS in FR2-2, including 120 kHz, 480 kHz and 960 kHz.

RAN1 has also agreed that PBCH payload size and the number of PBCH DMRS sequences is no greater than or the same as for FR2, if DBTW is supported. In order to reduce the impact of standardization caused by indicating candidate SSB indices, the number of candidate SSBs defined in the half-frame can be limited to 128 or kept unchanged (maintain 64) for 480/960 kHz SSB SCS.

Proposal 3: In order to reduce the impact of standardization caused by indicating candidate SSB indices, the maximum number of candidate SSBs defined in the half-frame can be kept unchanged (maintain 64) or limited to 128 for 480/960 kHz SSB SCS.

In order to reduce the number of bits indicating NSSBQCL value, four candidate values for NSSBQCL are preferred, such as {8,16,32,64}. NSSBQCL can be indicated via MIB (and/or SIB1), but whether it is feasible depends on the design of other aspects, e.g. multiplexing of SSB and CORESET#0, subcarrier level offset.

In Rel-16 NR-U, a UE derives the QCL relation between candidate SSBs according to the value of  NDM−RSPBCHmodNSSBQCL, where NDM−RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SSB. If the value of  NDM−RSPBCHmodNSSBQCL is same among the SSBs, the UE assumes that the SSBs in a serving cell that are within a same DBTW or across DBTWs are QCLed. However, in Rel-17 above 52.6GHz, the value of NSSBQCL can be larger than 8, e.g. it can be equal to 16, 32 or 64, and the number of PBCH DMRS sequences is the same as for FR2. The UE can not acquire the QCL relation if reusing Rel-16 NR-U mechanism, i.e according to the value of NDM−RSPBCHmodNSSBQCL. In fact, in Rel-16 NR-U WI phase, RAN1 has discussed another method to derive the QCL relation, that is relying on the values of (candidate SSB index  i  mod NSSBQCL). This method is more effective for Rel-17 above 52.6GHz.

Proposal 4: Four candidate values {8,16,32,64} for NSSBQCL are preferred from the perspective of reducing bit overhead.

Observation 1: For Rel-17 above 52.6GHz, the UE can not derive the QCL relation between candidate SSBs if it reuses Rel-16 NR-U mechanism, i.e. according to the value of  NDM−RSPBCHmodNSSBQCL.

Proposal 5: For Rel-17 above 52.6GHz, it is recommended that the UE derives the QCL relation between candidate SSBs by the value of i modNSSBQCL , where i is the candidate SSB index.

For LBT exempt operation and overlapping licensed/unlicensed bands, we can eliminate the effects of DBTW through configuration implementation. Enable/disable of DBTW can be indicated by comparing the value of  NSSBQCL in MIB and DBTW length after UE reads SIB1 or by comparing the value of  NSSBQCL in MIB and default DBTW length of 5 ms before UE reads SIB1. For example, for 120 kHz SSB SCS, the gNB can indicate that the maximum number of beams NSSBQCL = 64 and the length of DBTW = 5 ms if LBT exempt operation for SSB is allowed.

Proposal 6: Enable/disable of DBTW can be implicitly indicated by comparing the value of  NSSBQCL in MIB and DBTW length, and explicit signaling is not needed for this purpose.

### 2.1.3 Multiplexing of SSB and Type0-PDCCH

The association of SSB, Type0-PDCCH and RMSI should be studied for better initial access. Rel-15 FR2 supports SSB with SCS 120/240 kHz and Type0-PDCCH with SCS 60/120 kHz. The multiplexing of SSB and Type0-PDCCH includes three types of patterns: Pattern 1 (TDM), Pattern 2 (FDM with different SCSs) and Pattern 3 (FDM with same SCS), as shown in Figure 2.1.3-1. The frequency range of CORESET#0 of Pattern 1 needs to contain SSB. O can be taken as {0, 2.5, 5, 7.5}, which means that the SFN boundary can be offset by SSB with 0, 2.5, 5 or 7.5 ms. The CORESET#0 and SSB of Pattern 2 and 3 are FDMed in frequency domain, and the detection position of Type0-PDCCH is always located in the same slot or previous slot of the associated SSB.

Figure 2.1.3-1: The multiplexing patterns of SSB and CORESET#0

There are three supported SCS pairs for multiplexing between SSB and Type0-PDCCH suggested to be considered as follows. In RAN1#104 e-meeting, RAN1 has already agreed that below three supported SCS pairs for multiplexing between SSB and Type0-PDCCH can be supported if 480/960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported. RAN#92 e-meeting has decided to support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960 kHz SCS SSB, and only support 1  CORESET#0/Type0-PDCCH SCS for each SSB SCS as below:

- The multiplexing pattern of (SSB, CORESET#0) with SCS (120 kHz, 120 kHz)

- The multiplexing pattern of (SSB, CORESET#0) with SCS (480 kHz, 480 kHz)

- The multiplexing pattern of (SSB, CORESET#0) with SCS (960 kHz, 960 kHz)

Since the SCS of SSB and CORESET#0 in each SCS pair is always the same for FR 2-2, above Pattern 1 and Pattern 3 can be used for multiplexing between SSB and CORESET#0. For the multiplexing between the new SSB pattern of 480/960kHz and CORESET#0, Pattern 3 can be also considered as shown in Figure 2.1.3-2 in addition to Pattern 1, the first symbol index(s) of the corresponding CORESET#0 of the SSB i (i=2k, i=2k+1) can be 2, 9, respectively.  and . Wherein  and  are the SFN and slot index within a frame of the CORESET#0 based on SCS of the CORESET#0 and  and  are the SFN and slot index based on SCS of the CORESET#0, respectively, where the SSB with index  overlaps in time with system frame  and slot .

Figure 2.1.3-2: Multiplexing pattern 3 between SSB and CORESET#0

Proposal 7: In addition to multiplexing pattern 1, multiplexing pattern 3 for three approved SCS combinations of SSB and Type0-PDCCH can also be considered in FR2-2.

- (SSB, Type0-PDCCH): SCS (120 kHz, 120 kHz)

- (SSB, Type0-PDCCH): SCS (480 kHz, 480 kHz)

- (SSB, Type0-PDCCH): SCS (960 kHz, 960 kHz)

In RAN1 #104 e-meeting, it has been agreed to support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz. As RAN4 has agreed in the reply LS to RAN1 that the minimum CBW for 120 kHz SCS is 100 MHz, some companies thought it is not necessary to support 24 PRBs CORESET#0 for both multiplexing pattern 1 and 3 no longer. However, we don’t think the motivation is enough. For example, in Table 13-6 of Clause 13 in TS 38.213, Rel-15 NR supports 24 PRBs CORESET#0 with 30 kHz SCS for multiplexing pattern 1 while minimum CBW is 40 MHz. For multiplexing pattern 3, the CBW of SSB and CORESET#0 with 24 PRBs could be larger than that of multiplexing pattern 1. Therefore, it is not necessary to exclude 24 PRBs CORESEET#0 in both multiplexing pattern 1 and pattern 3.

Proposal 8: For {SSB, CORESET# for Type0-PDCCH} SCS = {120, 120} kHz, even though RAN4 has agreed the minimum CBW is increased to 100 MHz, at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 should still be supported.

For {SSB, CORESET#0 for Type0-PDCCH} SCS equals to {480, 480} kHz and {960, 960} kHz, RAN1 has already agreed to support the following set of parameters.

| SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols |
| --- | --- | --- |
| 1 | 24 | 2 |
| 1 | 48 | 1 |
| 1 | 48 | 2 |

As we suggested above, multiplexing pattern 3 should also be considered for three approved SCS combinations of SSB and Type0-PDCCH. Thus the number of entries (except “Offset”) for {120, 120}kHz in Table 13 of TS 38.213 can be reused for {480, 480} kHz and {960, 960} kHz, as shown in the Table below.

| SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols |
| --- | --- | --- |
| 3 | 24 | 2 |
| 3 | 48 | 2 |

Proposal 9: For {SSB, CORESET#0 for Type0-PDCCH} SCS= {480, 480} kHz and {960, 960} kHz, RAN1 should also support the following set of parameters  of ‘controlResourceSetZero’ configuration.

| SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols |
| --- | --- | --- |
| 3 | 24 | 2 |
| 3 | 48 | 2 |

  - 2.2 PRACH

2.2.1 Sequence length

According to the PRACH related aspects in [2], 120 kHz SCS PRACH with sequence length 139, 571 and 1151 has already been supported in initial and/or non-initial use cases. RAN1#106 e-meeting has agreed that the PRACH length L=571 and 1151 are not supported for 960kHz SCS PRACH, Meanwhile, the PRACH length L=1151 is not supported for 480kHz SCS PRACH. Whether to support PRACH with the sequence length 571 for 480 kHz SCS is still under discussion.

In US, “The 500 mW peak transmitter output limit applies to transmitters with an emission bandwidth of at least 100 MHz and is reduced for systems that employ narrower bandwidths.” and “Transmitters with an emission bandwidth of less than 100 MHz must limit their peak transmitter conducted output power to the product of 500 mW times their emission bandwidth divided by 100 MHz.”, the following Table 2.1.1-1 is the US regulation requirements excerpt from 38.807.

Table 2.1.1-1: USA regulatory requirements for frequency between 52.6GHz and 71GHz

| Frequency band [GHz] | Power/Magnetic Field Requirements | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- |
| 57 – 71 | Max avg, EIRP 40dBm Max peak EIRP 43dBm If emission-BW is less than 100 MHz, max peak conducted output power is {500mW × emission-BW / 100MHz} [Note 1] Otherwise, max peak conducted output power is 500mW | Equipment other than fixed outdoor. | Unlicensed. |
| Note 1: Emission bandwidth is defined as the instantaneous frequency range occupied by a steady state radiated signal with modulation, outside which the radiated power spectral density never exceeds 6 dB below the maximum radiated power spectral density in the band, as measured with a 100kHz resolution bandwidth spectrum analyzer. The center frequency must be stationary during the measurement interval, even if not stationary during normal operation (e.g., for frequency hopping devices). Peak transmitter conducted output power shall be measured with an RF detector that has a detection bandwidth that encompasses the 57-71GHz band and that has a video bandwidth of at least 10MHz. |  |  |  |

Based on the above regulatory requirements, if we want to reach max peak conducted output power (e.g., 500mW), the supported RACH length 139 does not meet this requirements. So from the perspective of avoiding power reduction, we think that the sequence length 571 for 480KHz PRACH SCS is needed.

Proposal 10: Support sequence length 571 for 480KHz PRACH SCS for 52.6 to 71 GHz.

2.2.2 RO configuration

In RAN1#106-e meeting, RO configuration including RO density and reference slot has been discussed when the SCS for PRACH equals 960KHz or 480KHz. Besides, RO configuration is related to RA-RNTI calculation, so it should be prioritized.

On whether gap is necessary to be introduced between ROs, this issue has been extensively discussed in Rel-17 above 52.6GHz and it was also been discussed in Rel-16 NR-U to resolve resource collision issue but no consensus. wherein, omni-directional beam is used for sensing/transmission in Rel-16 NR-U and operation frequency band is below 7GHz. But in Rel-17 above 52.6GHz, directional narrow beam is used for transmission and reception, this beam characteristic naturally helps to alleviate the issue of the resource collision. Therefore, there is no strong need to introduce the LBT gap for 480KHz and 960KHz. For beam switching gap, the potential issue is gNB RX beam switching only. TR 38.817-02 has also captured simulation results that to prevent degradation of system performance, switching time must be less than 80% of the CP length. For 960 kHz SCS NCP, this results in approximately 59 ns time window. Additionally, as shown in the Table 6.3.3.1-2 of TS 38.211, the PRACH CP is at least 1.5 times longer than the NCP. So it is also unnecessary to introduce the beam switching time between ROs.

- On whether to allow for additional nslotRA values if the maximum that can be configured for the number of FD RO’s is less than 8 (due to BW limitation). We think it is not necessary since above 52.6GH is target for the small cell. The similar issue has also been discussed in Rel-16 NR-U when PRACH sequence is 571 or 1151 and FD ROs number is less than 8, but it is not enhanced.

- Proposal 11: For 480kHz and 960kHz, it is unnecessary to introduce gap between ROs for LBT and/or beam switching.

Proposal 12: For 480kHz and 960kHz, it is unnecessary to allow for additional nslotRA values if the maximum that can be configured for the number of FD ROs is less than 8.

2.2.3 RA-RNTI

The RA-RNTI associated with the PRACH occasion in which the preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

The calculation above uses t_id < 80 which assumes a system frame and a maximum SCS of 120 kHz (80 slots in a frame). For higher SCSs (480 and/or 960 kHz), the number of slots in a frame is larger than 80 (320 slots for SCS = 480 kHz and 640 slots for SCS = 960 kHz). Hence, keeping the equation unchanged (with t_id ≥ 80 slots) will make the value of RA-RNTI exceed the range (0, 65535).

In addition, in NR Rel-16, MsgB RAR window and NR-U RAR window were extended to 40 ms by including the 2-bit LSB of the SFN corresponding to the PRACH occasion which is used to transmit the preamble of MsgA and Msg1.

As discussed in last meeting, there were eight solutions to resolve this issue [1] that fall into three categories. In this paper, we will analyze the options for RA-RNTI enhancement.

- Plain Modulus Category

    - Option 1)

      - RA−RNTI=1+sid+14∙tid+14∙max80,Nslotframe,u∙fid+14∙max80,Nslotframe,u∙8∙ulcarrier−id mod (216)

- PRACH Sub-segmentation Method Category

    - Option 2)

      - RA−RNTI=1+sid+14∙(tid)mod 80+14∙80∙fid+14∙80∙8∙ulcarrier−id

      - The same PRACH slot location in each 120kHz slot duration

    - Option 3)

      - Segment the PRACH into N segments

      - RA−RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier−id

      - tid is the index of the PRACH slot that contains the PRACH occasion in a segment.

      - In DCI: RA-indication = Segment index

    - Option 4)

      - Segment the PRACH into N segments

      - RA−RNTI=1+sid+14×tid+14×80*2u−3×fid+14×80*2u−3×8×ulcarrieridmod215

      - In DCI: RA−indication=1+sid+14×tid+14×80*2u−3×fid+14×80*2u−3×8×ulcarrierid/215

    - Option 5)

      - Segment the PRACH into N segments

      - RA−RNTI=1+sid+14∙(tid)mod M+14∙80∙fid+14∙M∙8∙ulcarrier−id

      - In DCI: RA−indication=tid/M

    - Option 6)

      - RA−RNTI=1+sid+14∙(tid)mod 160+14∙160∙fid+14∙160∙8∙ulcarrier−id

      - In DCI: RA−indication=tid/160

- Compressing some indices Category (may require a matching RO configuration to work properly)

    - Option 7)

      - RA−RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier−id

      - tid is the index of the first 120kHz slot that contains the PRACH occasion in a system frame.

      - sid is the index of the first OFDM symbol of the PRACH occasion based on the value of μ specified in clause 5.3.2 of TS 38.211.

    - Option 8)

      - RA-RNTI = 1 + s_id + 14 × floor(t_id / max1, 2μ−3) + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id,

      - t_id is based on the value of μ specified in clause 5.3.2 of TS 38.211.

Option 1 may need contention solution and change the equation of RA-RNTI calculation.

Option2, Option7 and Option8 may need to compress some PRACH slot indices, which may require a matching RO configuration to work properly, in other word, these options are suit for low RO density case without increasing the signaling overhead. Option 2 may need to be revised as: Non-overlapping PRACH slot location in each segment(80 slots). That means, (t_id mod 80) can refer to the unique PRACH slot, if the RO density is kept the same as 120kHz, one of the special case is that all of the PRACH slots are located in one of the segment. Option 2 doesn’t need additional signalling.

Option 3 can reuse the same RA-RNTI formula in NR Rel-15/16, divide the system frame into N segments (each segment is 80 slots using the used SCS), and signal the segment index that transmit the preamble in the DCI. For example, for 960 kHz PRACH SCS, there would be 640 slots in a system frame. Then it can be divided into 8 segments, each segment including 80 slots. Therefore, a 3 bits indication can be included in the DCI to indicate in which segment the PRACH transmits. Similarly, for SCS 480 kHz, 2 bits (corresponding to 4 segments) is needed in the DCI.

For Option 4, 5, 6, they are suitable for any RO density, but the RA-RNTI equation needs to be updated, and the signaling overhead is depending on the number of the segments.

Form requirement aspect, the same RO density as the 120kHz in Rel-15/16 is sufficient for above 52.6GHz, and thus Option 2, Option 7 or Option 8 are suitable, while Option 2 or 7 is preferable over option 8 due to lower complexity. From the spec impact aspect, option3 is much simpler and the similar principle has been adopted for the extended RAR window for NR-U or MsgB in Rel-16. Therefore, we prefer to make further down-selection among Option 2, Option 3 and Option7.

Proposal 13: For higher PRACH SCS (480 and/or 960 kHz), consider the following options for further down-selection of RA-RNTI enhancements:

- Option 2)

- Segment the PRACH into N segments

- RA−RNTI=1+sid+14∙(tid)mod 80+14∙80∙fid+14∙80∙8∙ulcarrier−id

- Non-overlapping PRACH slot location in each segment(80 slots)

- Option 3)

- Segment the PRACH into N segments

- RA−RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier−id

- tid is the index of the PRACH slot that contains the PRACH occasion in a segment.

- In DCI: RA-indication = Segment index

- Option 7)

- RA−RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier−id

- tid is the index of the first 120kHz slot that contains the PRACH occasion in a system frame.

- sid is the index of the first OFDM symbol of the PRACH occasion based on the value of μ specified in clause 5.3.2 of TS 38.211.

  - 2.3 RRC parameters

In RAN plenary #92 e-meeting, RAN1 has agreed only one CORESTE#0 SCS supported for each SSB SCS, i.e., (120, 120) kHz, (480, 480) kHz and (960, 960) kHz. In other words, FR2-2 only support CORESTE#0 SCS same as SSB SCS. Therefore, it is not necessary to use subCarrierSpacingCommon in MIB to indicate the SCS of CORESET#0, SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. Although the new function of subCarrierSpacingCommon has not been determined, it is clear that its field description should be revised for FR2-2.

Agreement in RAN#92-e, June 14-18, 2021 Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/ Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting - Only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960). - Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Proposal 14: The existing parameter subCarrierSpacingCommon in MIB should be captured into Rel-17 RRC parameter table, as it will no longer be used to indicate the SCS of CORESET#0 in FR2-2.

Conclusion

In this contribution, we discuss initial access aspects in NR above 52.6 GHz band operation and have the following observation and proposals.

Observation 1: For Rel-17 above 52.6GHz, the UE can not derive the QCL relation between candidate SSBs if it reuses Rel-16 NR-U mechanism, i.e. according to the value of  NDM−RSPBCHmodNSSBQCL.

Proposal 1: The following design of candidate SSBs with SCS 480/960 kHz in a half frame can be considered: First symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- If DBTW is not supported or DBTW is disabled

  - For 480kHz SCS, the 64 candidate SSBs are located in 32 slots, with 2  slots spacing between every 8 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37

  - For 960kHz SCS, the 64 candidate SSBs are located in 32 slots, with 4  slots spacing between every 16 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35

- If DBTW is supported and it is enabled

  - Additional 64 candidate SSB can be defined after the above original 64 candidate SSBs in the half frame

Proposal 2: Discovery burst transmission window (DBTW) should be supported for all approved SSB SCS in FR2-2, including 120 kHz, 480 kHz and 960 kHz.

Proposal 3: In order to reduce the impact of standardization caused by indicating candidate SSB indices, the maximum number of candidate SSBs defined in the half-frame can be kept unchanged (maintain 64) or limited to 128 for 480/960 kHz SSB SCS.

Proposal 4: Four candidate values {8,16,32,64} for NSSBQCL are preferred from the perspective of reducing bit overhead.

Proposal 5: For Rel-17 above 52.6GHz, it is recommended that the UE derives the QCL relation between candidate SSBs by the value of i modNSSBQCL , where i is the candidate SSB index.

Proposal 6: Enable/disable of DBTW can be implicitly indicated by comparing the value of  NSSBQCL in MIB and DBTW length, and explicit signaling is not needed for this purpose.

Proposal 7: In addition to multiplexing pattern 1, multiplexing pattern 3 for three approved SCS combinations of SSB and Type0-PDCCH can also be considered in FR2-2.

- (SSB, Type0-PDCCH): SCS (120 kHz, 120 kHz)

- (SSB, Type0-PDCCH): SCS (480 kHz, 480 kHz)

- (SSB, Type0-PDCCH): SCS (960 kHz, 960 kHz)

Proposal 8: For {SSB, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz, even though RAN4 has agreed the minimum CBW is increased to 100 MHz, at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 should still be supported.

Proposal 9: For {SSB, CORESET#0 for Type0-PDCCH} SCS= {480, 480} kHz and {960, 960} kHz, RAN1 should also support the following set of parameters  of ‘controlResourceSetZero’ configuration.

| SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols |
| --- | --- | --- |
| 3 | 24 | 2 |
| 3 | 48 | 2 |

Proposal 10: Support sequence length 571 for 480KHz PRACH SCS for 52.6 to 71 GHz.

- Proposal 11: For 480kHz and 960kHz, it is unnecessary to introduce gap between ROs for LBT and/or beam switching.

Proposal 12: For 480kHz and 960kHz,   it is unnecessary to allow for additional nslotRA values if the maximum that can be configured for the number of FD ROs is less than 8

Proposal 13: For higher PRACH SCS (480 and/or 960 kHz), consider the following options for further down-selection of RA-RNTI enhancements:

- Option 2)

- Segment the PRACH into N segments

- RA−RNTI=1+sid+14∙(tid)mod 80+14∙80∙fid+14∙80∙8∙ulcarrier−id

- Non-overlapping PRACH slot location in each segment(80 slots)

- Option 3)

- Segment the PRACH into N segments

- RA−RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier−id

- tid is the index of the PRACH slot that contains the PRACH occasion in a segment.

- In DCI: RA-indication = Segment index

- Option 7)

- RA−RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier−id

- tid is the index of the first 120kHz slot that contains the PRACH occasion in a system frame.

- sid is the index of the first OFDM symbol of the PRACH occasion based on the value of μ specified in clause 5.3.2 of TS 38.211.

Proposal 14: The existing parameter subCarrierSpacingCommon in MIB should be captured into Rel-17 RRC parameter table, as it will no longer be used to indicate the SCS of CORESET#0 in FR2-2.

Reference

- Chair's Notes RAN1#106-e 8.2 eom, 2021-08

- Draft_Minutes_report_RAN1#104-e_v030, 2021-02