3GPP TSG RAN WG1 #106bis-e                                                R1-2108957

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	Further discussion on MTRP CSI and Partial reciprocity

Agenda Item:	8.1.4

Document for:	Discussion and Decision

Introduction

Significant progress on MTRP CSI enhancement and partial reciprocity were achieved in previous meetings [1].

In this paper, our views on remaining issues, further potential enhancements, and evaluation results on CSI enhancements for MTRP/panel transmission and partial reciprocity are provided.

CSI enhancement for MTRP

In previous meetings, as shown in Figure 1, two configuration methods with one reporting setting for NCJT, i.e., Cat1, and multiple reporting settings for NCJT, i.e., Cat2, were proposed to enhance CSI feedback for MTRP transmission. A single reporting setting for NCJT has been agreed for S-DCI based MTRP transmission and multiple reporting settings for NCJT are agreed as a working assumption for M-DCI based MTRP transmission.

Two configuration methods for MTRP transmission

In this paper, we will focus on the remaining issues of RAN1#106-e, Cat2 for non-ideal backhaul based NCJT and the design of RRC signaling for Cat1.

Remaining issues of Cat1

CSI measurement enhancement and CSI framework for MTRP

Following agreements were reached on this issue in the previous meeting.

Conclusion: Default value of Ks, max can be discussed later with Rel-17 MIMO UE capability. Agreement For CSI measurement associated to a NCJT measurement hypothesis in Rel-17, the maximal number of total transmission layers is up to 4 layers. Agreement For CSI measurement associated with a CSI-ReportingConfig for NCJT, study following restriction(s) for two CMRs within the same CMR pair configured for NCJT measurement hypothesis: - FFS: two resources are restricted within the same DL slot - FFS: two resources are restricted with the same CDRX active time Agreement For a CMR pair configured for a NCJT measurement hypothesis, study following Alternatives: - Alt 1: a separate powerControlOffset (Pc ratio) shall be configured for the NCJT measurement hypothesis by re-defining such Pc ratio as 10log10(P_PDSCH/P_CSIRS) dB, whereas - P_PDSCH is the energy of PDSCH ports with a same TCI state as the CMR on one subcarrier of one OFDM symbol - P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol - Alt 2: re-interpret two Pc ratios configured for the CMR pair for the NCJT measurement hypothesis, FFS detailed impact of specification - Alt 3: No change to definition or configuration of Pc ratio - Note that other solutions are not excluded. Agreement For CSI computation delay requirement associated with a CSI-ReportingConfig for a NCJT measurement hypothesis, study following alternatives: - Alt1: introducing new/relaxed values on Z and Z’, FFS exact values or other conditions - Alt2: No changes of values on Z and Z’ Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT measurement hypothesis, study whether to support non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" in Rel-17 - Related details, if needed, are to be discussed in RAN1#106bis. - Interested companies are encouraged to share details and related specification impact if support Agreement For CSI measurement associated with a CSI-ReportConfig for NCJT, support following Alt: - Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMRs configured within CMR pairs for NCJT measurement hypothesis Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction by selecting at most one alternative from the following in RAN1#106bis-e: - Alt 1: One RI restriction is configured per CodebookConfig, whereas the RI restriction is applied to both Single-TRP and NCJT measurement hypotheses. - If rank restriction of X is configured, reported rank is X for a Single-TRP measurement hypothesis and sum of two reported ranks is X for a Multi-TRP measurement hypothesis. - Alt 2: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to one CMR group in a CMR resource set respectively, i.e. per TRP. - If rank restriction of (X, Y) is configured, reported rank is X for the CMR in the first CMR group and Y for the CMR in the second CMR group, regardless single-TRP and NCJT measurement hypotheses. - Alt 3: Multiple RI restrictions can be configured per CodebookConfig, whereas RI restriction is applied to per each CMR in CMR pair for NCJT and per each CMR for Single-TRP. - Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses. - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. - Alt 5: Three RI restrictions can be configured per CodebookConfig, whereas two RI restrictions are applied to two CMR groups in a CMR resource set respectively for Single-TRP measurement hypothesis, and the third one is applied to all NCJT measurement hypotheses. - If rank restriction of (X1, X2, Y) is configured, reported rank is X1, X2 for each CMR group respectively for single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. - Alt 6: Switch between Alt 4 and Alt 5 where gNB can configure via RRC signaling which alternative to use Note that if none of above Alternatives is agreed in Rel-17, RI restriction is only applied for Single-TRP measurement hypotheses and no RI restriction is applied for Multi-TRP measurement hypotheses.

For CSI measurement enhancement and CSI framework, we have discussed many issues about the CMR configuration for Cat1. The remaining issues are divided into two aspects, i.e., CMR/IMR configuration and CSI measurement.

CMR/IMR configuration

For CMR configuration, it is agreed that a UE determines CMR pairs to calculate the NCJT hypothesis from two CMR groups in a CMR set, and CMRs in each CMR group can be used for both NCJT and STRP measurement hypotheses at least for FR1. Besides, we also reach some consensuses in the number of CMR pairs and The configuration method for NCJT CMR pair(s) and STRP CMR(s).

For the configuration method for NCJT CMR pair(s) and STRP CMR(s), three alternatives, i.e., Alt.2, Alt.3, and Alt.4, were proposed in RAN1#105-e and Alt.3 was agreed in RAN1#106-e, where the default maximum number of CMRs will be influenced by the CMR sharing’s state. Besides, the reporting mechanisms, i.e., Option1 with X=0, 1, 2 and Option2, also affect the default maximum number of CMRs. The minimum requirement of the CMR quantity configured in the CMR set for the reporting mechanisms is summarized in the following table.

The minimum requirement on the number of configured CMRs for reporting mechanisms

| Reporting mechanism | CMR sharing enabled | CMR sharing disabled |
| --- | --- | --- |
| Option1 with X=0 | - | 2 CMRs |
| Option1 with X=1 (optional feature) | 2 CMRs | 3 CMRs |
| Option1 with X=2 (optional feature) | 2 CMRs | 4 CMRs |
| Option2 | 2 CMRs | 3 CMRs |

From Table 1, two CMRs are needed for most reporting mechanisms with CMR sharing enabled. Therefore, we prefer value 2 as the default maximum number of CMR for MTRP CSI measurement.

The default maximum number of CMR is 2 with Rel-17 MIMO UE capability for MTRP CSI measurement.

For two CMRs within the same CMR pair configured for the NCJT measurement hypothesis, some companies point that the performance of the receiver will decrease due to random phase rotations at the receiver when there is an uplink transmission between the two occasions of the CMRs within the same CMR pair. In the last meeting, following restriction(s) for two CMRs within the same CMR pair configured for NCJT measurement hypothesis were proposed

FFS: two resources are restricted within the same DL slot

FFS: two resources are restricted with the same CDRX active time

We are fine with the latter restriction but not the first one, as it is too restrictive so that the network has to configure the slot as a DL slot before it transmits CSI-RS for enhanced MTRP CSI feedback in dynamic TDD systems. Actually, avoiding uplink symbols between two CMRs within the same CMR pair can be assured and implemented by network and thus a relaxed restriction can be made.

For CSI measurement associated with a CSI-ReportingConfig for NCJT, following restrictions are applied for two CMRs within the same CMR pair configured for NCJT measurement hypothesis:

UE does not expect any uplink symbols between two CMRs within the same CMR pair.

Two CMRs are restricted with the same CDRX active time

One-to-one mapping has been agreed between M+N CSI-IM resources and M CMRs for STRP measurement hypothesis and N CMR pairs for NCJT measurement hypothesis configured in a CSI-RS resource set. Another remaining issue is the definition/value of M. Based on the agreed method to enable CMR sharing by a dedicated RRC signaling, the value of M should be derived from the RRC signaling. If the CMR sharing is enabled, M is equal to Ks. If the CMR sharing is disabled, M is equal to Ks – 2N.

For the number of CSI-IM resources

M is equal to Ks when the CMR sharing is enabled.

M is equal to Ks – 2N when the CMR sharing is disabled.

CSI measurement

For CSI measurement, a potential issue that a separate powerControlOffset shall be configured for the NCJT measurement hypothesis was proposed in the last meeting. In current specification, the powerControlOffset used for CQI calculation is configured per CSI-RS resource. The relating texts in TS 38.214[2] are shown in the following.

Clause 5.2.2 of TS 38.214[2]

The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i) where is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211], is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2.The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1.

We think it is clear and still works according to the current description that the corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1. For NCJT transmission, layers can not be mapped across different TRPs due to  non-coherent transmission between TRPs. Therefore, it implies that the “corresponding PDSCH signals” transmitted on antenna ports [3000,…,3000 + P - 1] doesn’t mean all layers of the PDSCH but the TRP-specific layers of the PDSCH mapped to CSI-RS ports within CMR of each TRP, as illustrated in Figure 2. Therefore, at least for NCJT, we think the current description in subclause 5.2.2 of TS 38.214[2] is clear and flexible enough and different powerControlOffset values can be configured to CMRs from two TRPs.

Illustration of applying current description of the ratio of EPRE to CSI-RS EPRE for MTRP.

Support no change to definition or configuration of Pc_ratio.

Non-PMI support for MTRP

Another FFS about CSI measurement is non-PMI transmission.  non-PMI transmission can simplify the complexity of UE. In non-PMI-based STRP transmission, a default rule or an RRC parameter, i.e. non-PMI-PortIndication, is used to obtain the relationship between the CSI-RS ports and rank values per CMR. For non-PMI-based MTRP transmission, the key issue is having the UE know the relationship between the CSI-RS port and rank value.

In our opinion, the legacy indication method can be reused for enhanced non-PMI CSI report, where each CMR, either for STRP hypothesis or for NCJT hypothesis, is configured with a port index indication sequence for different ranks, except the case that CMR sharing between NCJT hypothesis and STRP hypothesis is enabled. When CMR sharing is enabled, which means that a CMR is  used for both STRP hypothesis and NCJT hypothesis, the selected ports/ranks for STRP hypothesis may be different from those for NCJT hypothesis due to different interference assumptions between two hypothesis. Some options to this issue are as follows.

When non-PMI-PortIndication is configrued, separate non-PMI-PortIndication parameters are configured where one is for STRP hypothesis and the other is for NCJT hypothesis.

When non-PMI-PortIndication is not configured, an enhanced rule can be considered, e.g., the first v ports are associated with the rank v of STRP hypothesis measurement and the last k ports are associated with the rank k of NCJT hypothesis measurement.

For non-PMI CSI feedback, when CMR sharing between NCJT hypothesis and STRP hypothesis is enabled

When non-PMI-PortIndication is configrued, separate non-PMI-PortIndication parameters are configured where one is for STRP hypothesis and the other is for NCJT hypothesis.

When non-PMI-PortIndication is not configured, an enhanced rule can be considered, e.g., the first v ports are associated with the rank v of STRP hypothesis measurement and the last k ports are associated with the rank k of NCJT hypothesis measurement.

CBSR restriction and RI restriction

We have discussed the configuration of RI restriction with some alternatives proposed in the last meeting. In our opinion, different report configurations across TRPs should be considered at least for cases like TRPs in heterogeneous deployment. Therefore, we think TRP-specific CBSR and RI restriction should be considered from the perspective of flexibility. Thus our first preference is Alt 5 or Alt 6, i.e. three RI restrictions can be configured per CodebookConfig, whereas two RI restrictions are applied to two CMR groups in a CMR resource set respectively for STRP measurement hypothesis, and the third one is applied to all NCJT measurement hypotheses. In Alt 6, switching between Alt 4 and Alt 5 can be realized by configuring one or two RI restriction for STRP hypothesis besides the RI restriction for NCJT hypothesis.

For RI restriction, support Alt 5 or Alt 6 considering the flexibility.

For codebook subset restriction, support TRP-specific CBSR.

Reporting enhancement

Agreement For the UE configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis (i.e. Option 1), the bitwidth associated to X+1 CRI(s) are given as following: - Ceil(log2(N)) for X=0 - Ceil(log2(N)) in CSI associated with NCJT measurement hypothesis and Ceil(log2(M1+M2)) in CSI associated with Single-TRP measurement hypothesis for X=1 - Ceil(log2(N)) in CSI associated with NCJT measurement hypothesis and Ceil(log2(M1)) and Ceil(log2(M2)) in CSI associated with Single-TRP measurement hypothesis for X=2 - Note that M1 (M1<=K1) and M2 (M2<=K2) is the number of CMRs configured for Single-TRP measurement hypothesis in the first and second CMR groups respectively in a CMR measurement set. Agreement For the UE be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses (i.e. Option 2), - Alt 1: the first M1+M2 codepoints of CRI corresponds to M1+M2 CMRs for Single-TRP measurement hypothesis and the second N codepoints corresponds to N CMR pairs for NCJT measurement hypothesis. - Note that M1M1 (M1<=K1) and M2 (M2<=K2) is the number of CMRs configured for Single-TRP measurement hypothesis in the first and second CMR groups respectively in a CMR measurement set. Agreement To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis: - Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214. - Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214. - Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

For reporting enhancement, we have good progress. As we agreed in the previous meetings, two-part structure for a CSI report is confirmed due to subband granularity. One of the remaining issues is related to the UCI payload construction.

UCI payload construction

We can split the UCI payload construction into two stages. The first stage is how to generate a UCI sub-sequence for a CSI report and the second stage is how to combine multiple sub-sequence, each corresponding to a CSI report, to a total UCI sequence.

For the first stage, we think it is more important to discuss these viewpoints.

Viewpoint1: One CSI reporting setting contains multiple CSI reports, each CSI report corresponding to a hypothesis.

Viewpoint2: One CSI reporting setting corresponds to one CSI report that contains all measurement results of all hypotheses.

- For the second stage, the priority rule deciding the mapping order among multiple CSI reports or multiple measurement hypotheses plays an important role.

If Viewpoint1 is agreed, it is equivalent that the priority rule decides the mapping order among multiple measurement hypotheses. Modify the priority equation in Section 5.2.5 in 38.214 is needed because the current equation can not distinguish the priority of the measurement hypothesis. However, the equation is also invoked elsewhere in current specification such as CPU occupation, UE procedure for transmitting multiple PUCCHs in a slot, and so on. Some examples are shown in Table 2. Besides, Viewpoint1 contradicts some restrictions in the current specification, e.g. a UE configured with DCI format 0_1 or 0_2 does not expect to be triggered with multiple CSI reports with the same CSI-ReportConfigId, etc.

Some examples for the priority equation invoked elsewhere in current specification

| Specification description | Impact statement |
| --- | --- |
| [Clause 9.2.5, TS 38.213[3]] If a UE is configured with multiple PUCCH resources in a slot to transmit CSI reports - if the UE is not provided multi-CSI-PUCCH-ResourceList or if PUCCH resources for transmissions of CSI reports do not overlap in the slot, the UE determines a first resource corresponding to a CSI report with the highest priority [6, TS 38.214[2]] - if the first resource includes PUCCH format 2, and if there are remaining resources in the slot that do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting - if the first resource includes PUCCH format 3 or PUCCH format 4, and if there are remaining resources in the slot that include PUCCH format 2 and do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting - if the UE is provided multi-CSI-PUCCH-ResourceList and if any of the multiple PUCCH resources overlap, the UE multiplexes all CSI reports in a resource from the resources provided by multi-CSI-PUCCH-ResourceList, as described in Clause 9.2.5.2. | In the current specification, before the UE multiplexes all CSI reports in a resource, only one CSI report is carried by a PUCCH resource. If Viewpoint1 is agreed, multiple CSI reports would be carried by a PUCCH resource, which will affect the current understanding. In current understanding, only a multi-CSI PUCCH resource can carry multiple CSI reports. Thus it seems that multi-CSI-PUCCH-ResourceList must be provided for Viewpoint1 or MTRP CSI reporting on PUCCH will not be supported. Viewpoint2 does not affect the current understanding. |
| [Clause 5.2.1.6, TS 38.214[2]] For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. | For the timeline of CPU occupation, in the current specification, the beginning time of CPU occupation is the first symbol of the earliest occasion of all resources. Therefore, the beginning time of CPU occupation is set per legacy CSI report, i.e., per reporting setting. If Viewpoint1 is followed, a reporting setting can have multiple timelines for MTRP CSI acquirement. That is too complex. |
| [Clause 5.2.1.6, TS 38.214[2]] For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', - if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPU, - otherwise, OCPU=Ks, where Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement. | The OCPU is dependent on the CMR number in a CMR set. If the Viewpoint1 is employed, it means OCPU for a report is related to the specific hypothesis, i.e. OCPU is decided by the number of CMRs in a CMR group that is used for hypothesis measurement. However, Viewpoint2 still bases on the number of CMRs in the CSI-RS resource set to determine the OCPU. |

If Viewpoint2 is agreed, the legacy priority equation may not need to enhance. So the impacts shown in Table 2 is not existing. Therefore, we support that a CSI reporting setting configured with enhanced MTRP CSI reporting corresponds to a CSI report.

A CSI reporting setting configured with enhanced MTRP CSI reporting corresponds to a CSI report.

Based on Viewpoint2, the detailed design of the first stage and second stage for UCI payload construction should be considered.

For the first stage, Table 3 shows the CSI field mapping order for Part1 of a MTRP CSI report. Table 4  and Table 5 show the CSI field mapping order for Part2 of a MTRP CSI report.

CSI field mapping order of Part1 of one CSI report for the reporting Mode 1 and Mode 2, i.e. Option1 and Option2, when subband CQI and PMI are configured

|  | CSI hypothesis | CSI fields for CSI hypothesis |
| --- | --- | --- |
| CSI report Part1 | CSI for NCJT hypothesis, if reported | CRI, if reported |
|  |  | RI, if reported |
|  |  | wideband CQI for the first TB, if reported |
|  |  | subband CQI for the first TB, if reported |
|  | CSI for the first STRP hypothesis, if reported | CRI, if reported |
|  |  | RI, if reported |
|  |  | wideband CQI for the first TB, if reported |
|  |  | subband CQI for the first TB, if reported |
|  | CSI for the second STRP hypothesis, if reported | CRI, if reported |
|  |  | RI, if reported |
|  |  | wideband CQI for the first TB, if reported |
|  |  | subband CQI for the first TB, if reported |

CSI field mapping order of Part2 wideband of one CSI report for the reporting Mode 1 and Mode 2, i.e.  Option1 and Option2, when subband CQI and PMI are configured

|  | CSI hypothesis | CSI fields for CSI hypothesis |  |
| --- | --- | --- | --- |
| CSI report Part2 | CSI for NCJT hypothesis, if reported |  | 2 LIs, if reported |
|  |  |  | 2 wideband PMIs, if reported |
|  | CSI for first STRP hypothesis, if reported |  | 1 wideband CQI for the second TB, if reported |
|  |  |  | 1 LI, if reported |
|  |  |  | 1 wideband PMI, if reported |
|  | CSI for second STRP hypothesis, if reported |  | 1 wideband CQI for the second TB, if reported |
|  |  |  | 1 LI, if reported |
|  |  |  | 1 wideband PMI, if reported |

CSI fields mapping order of Part2 subband of one CSI report for the reporting Mode 1 and Mode 2, i.e.  Option1 and Option2, when subband CQI and PMI are configured

|  | CSI hypothesis | CSI fields for CSI hypothesis |
| --- | --- | --- |
| CSI report Part2 | CSI for NCJT hypothesis, if reported | 2 subband PMI of all even subband, if reported |
|  |  | 2 subband PMI of all odd subband, if reported |
|  | CSI for first STRP hypothesis, if reported | 1 subband CQI for the second TB of all even subbands, if reported |
|  |  | 1 subband PMI of all even subband, if reported |
|  |  | 1 subband CQI for the second TB of all odd subbands, if reported |
|  |  | 1 subband PMI of all odd subband, if reported |
|  | CSI for second STRP hypothesis, if reported | 1 subband CQI for the second TB of all even subbands, if reported |
|  |  | 1 subband PMI of all even subband, if reported |
|  |  | 1 subband CQI for the second TB of all odd subbands, if reported |
|  |  | 1 subband PMI of all odd subband, if reported |

For the second stage, because a MTRP CSI report is of two parts, two UCI bit sequences, a0(1), a1(1),a2(1),…,aA(1)-1(1) and a0(2), a1(2),a2(2),…,aA(2)-1(2), are generated when at least one of the CSI reports for transmission on a PUCCH/PUSCH is a MTRP CSI report.  The CSI fields of all CSI reports are mapped to the UCI bit sequence a0(1), a1(1),a2(1),…,aA(1)-1(1) and a0(2), a1(2),a2(2),…,aA(2)-1(2) according to the following tables.

Mapping order of CSI reports to UCI bit sequence a0(1), a1(1),a2(1),…,aA(1)-1(1)

| a01, a11, a21, …, aA(1)-1(1) | CSI report #1 if CSI report #1 is not of two parts, or CSI report #1, CSI Part1, if CSI report #1 is of two parts |
| --- | --- |
|  | CSI report #2 if CSI report #2 is not of two parts, or CSI report #2, CSI Part1, if CSI report #1 is of two parts |
|  | … |
|  | CSI report #n if CSI report #n is not of two parts, or CSI report #n, CSI Part1, if CSI report #1 is of two parts |

Mapping order of CSI reports to UCI bit sequence a0(2), a1(2),a2(2),…,aA(2)-1(2)

| a02, a12, a22, …, aA(2)-1(2) | CSI report #1, CSI Part2 wideband, if CSI Part2 exists for CSI report #1 |
| --- | --- |
|  | CSI report #2, CSI Part2 wideband, if CSI Part2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI Part2 wideband, if CSI Part2 exists for CSI report #n |
|  | CSI report #1, CSI Part2 subband, if CSI Part2 exists for CSI report #1 |
|  | CSI report #2, CSI Part2 subband, if CSI Part2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI Part2 subband, if CSI Part2 exists for CSI report #n |

As the description in Table 6 and Table 7, the CSI report #1, CSI report #2, …, CSI report #n correspond to the CSI reports with an increasing order of CSI report priority values according to the priority formula.

For legacy CSI reporting, UE maps all CSI reports to a UCI bit sequence for transmission on a PUSCH resource or a multi-CSI PUCCH resource. Except for the CSI report priority acquired by the priority formula, the UCI bit sequence generation is also affected by the omission rule, i.e., Table 5.2.3-1 in 38.214.

CSI omission rule

The legacy omission rule decides whether the CSI Part2 exists or which CSI quantities exist in the CSI Part2 for a CSI report. And the omission granularity is in CSI report level. Therefore two methods shown in Table 8 and Table 9 can be considered where one corresponds to omission in CSI report level and the other corresponds to omission in hypothesis level.

Method 0: Use legacy omission rule for all CSI reports, i.e., omit in order of odd subband CSI containing all hypotheses, even subband CSI containing all hypotheses.

Method 1: Apply omission order of different the hypotheses, i.e., omit in order of odd subband CSI of STRP CSIs, even subband CSI of STRP CSIs, odd subband CSI of NCJT CSI, and even subband CSI of NCJT CSI.

Omission order with a granularity of CSI report level for Part2 CSI

| Priority 0 | For CSI reports 1 to N, Part2 wideband CSI for CSI reports. |
| --- | --- |
| Priority 1 | For CSI reports 1, Part2 subband CSI of even subbands for CSI report 1. |
| Priority 2 | For CSI reports 1, Part2 subband CSI of odd subbands for CSI report 1. |
| … | … |
| Priority 2N-1 | For CSI report N, Part2 subband CSI of even subbands for CSI report N. |
| Priority 2N | For CSI report N, Part2 subband CSI of odd subbands for CSI report N. |
| Note: - Priority 0 is the highest priority and priority 2N is the lowest priority. - The CSI report n (n = 0,1,…,N) corresponds to the CSI report with the nth smallest priority value among the N CSI reports. - Each of Part2 wideband CSI, Part2 subband CSI of even subbands, Part2 subband CSI of odd subbands contains CSI quantities of all the measurement hypothesis. |  |

Omission order with a granularity of hypothesis level for Part2 CSI

| Priority 0 | For CSI reports 1 to N, Part2 wideband CSI for CSI reports. |
| --- | --- |
| Priority 1 | For CSI report 1, if CSI report 1 contains a NCJT CSI, Part2 subband CSI of even subbands for NCJT CSI. |
| Priority 2 | For CSI report 1, if CSI report 1 contains a NCJT CSI, Part2 subband CSI of odd subbands for NCJT CSI. |
| Priority 3 | For CSI report 1, if CSI report 1 contains STRP CSI(s), Part2 subband CSI of even subbands for first STRP hypothesis. |
| Priority 4 | For CSI reports 1, if CSI report 1 contains STRP CSI(s), Part2 subband CSI of odd subbands for first STRP hypothesis. |
| Priority 5 | For CSI reports 1, if CSI report 1 contains STRP CSI(s), Part2 subband CSI of even subbands for second STRP hypothesis. |
| Priority 6 | For CSI reports 1, if CSI report 1 contains STRP CSI(s), Part2 subband CSI of odd subbands for second STRP hypothesis. |
| … | … |
| Priority 6N-5 | For CSI reports N, if CSI report N contains a NCJT CSI, Part2 subband CSI of even subbands for NCJT hypothesis. |
| Priority 6N-4 | For CSI reports N, if CSI report N contains a NCJT CSI, Part2 subband CSI of odd subbands for NCJT hypothesis. |
| Priority 6N-3 | For CSI reports N, if CSI report N contains STRP CSI(s), Part2 subband CSI of even subbands for first STRP hypothesis. |
| Priority 6N-2 | For CSI reports N, if CSI report N contains STRP CSI(s), Part2 subband CSI of odd subbands for first STRP hypothesis. |
| Priority 6N-1 | For CSI reports N, if CSI report N contains STRP CSI(s), Part2 subband CSI of even subbands for second STRP hypothesis. |
| Priority 6N | For CSI reports N, if CSI report N contains STRP CSI(s), Part2 subband CSI of odd subbands for second STRP hypothesis. |
| Note: - Priority 0 is the highest priority and priority 6N is the lowest priority. - The CSI report n (n = 0,1,…,N) corresponds to the CSI report with the nth smallest priority value among the N CSI reports. |  |

  - For the sake of simplicity, we slightly prefer to use legacy priority definition and omission rule with a granularity of report level without any specification impact.

It is slightly preferred to use legacy priority definition and omission rule with a granularity of CSI report level.

Enhancement for wideband PMI and CQI

Besides subband PMI and CQI, wideband granularity is also an indispensable case, especially for reporting on PUCCH.

Only one part is reported according to the legacy design when PMI and CQI granularity are wideband. If we make all quantities of all hypotheses in one part, the UCI payload will be large, and the whole CSI report will even be dropped in certain cases. As shown in Figure 3, we compare UCI payload for Option1 and Option2 when PMI and CQI granularity are wideband with the assumptions in Table 10.

Assumptions for UCI payload calculation

Ks = 2, N = 1

Rank = 4, for STRP hypo. Rank = (2, 2), for NCJT hypo.

Legacy CRI/RI/LI bit-width definition

16 CSI-RS port

single-panel codebook: N1 = N2 = 2 O1 = O2 = 4

Comparison of UCI payload for CSI report Mode 1 and Mode 2, i.e., Option1 and Option2

According to the results, we think it is necessary to enhance the CSI report when PMI and CQI granularity are wideband. Some aspects need further study as follows:

Feedback overhead reduction

Reporting structure, e.g., two-part structure, and omission rule when two-part structure is enabled

Some restrictions for PUCCH resource, e.g., format type

Support to enhance the CSI reporting mechanism when PMI and CQI granularity are wideband.

Cat2 for non-ideal backhaul

For M-DCI based MTRP transmission in non-ideal backhaul scenario, it is difficult to coordinate transmission among TRPs due to the large latency in backhaul. From the simulation results shown in Appendix B, M-DCI based NCJT transmission with two associated CSI reporting settings can bring obvious performance gains. Therefore, MTRP CSI enhancement for non-ideal backhaul scenarios is necessary.

The following tables show the UPT gain of three schemes compared to the baseline of M-DCI based NCJT transmission without CSI exchange between TRPs.

- Scheme1 (Baseline): UE is configured to report one CSI associated with the best one among NCJT and STRP measurement hypotheses. The CSI is not exchanged between TRPs, i.e. Cat2.

- Scheme2: UE is configured to report one CSI associated with the best one among NCJT and STRP measurement hypotheses. The CSI is exchanged between TRPs with 5ms backhaul delay, i.e. Cat1 with 5ms backhaul delay.

- Scheme3: UE is configured to report one CSI associated with the best one among NCJT and STRP measurement hypotheses. The CSI is exchanged between TRPs with 50ms backhaul delay, i.e. Cat1 with 50ms backhaul delay.

For the MTRP transmission in non-ideal backhaul scenario, as described in Appendix A, each TRP is independently scheduling. Other simulation parameters can be found in Appendix D. We provide UPT comparison for FTP model 1 with RU for a baseline set to 18% and 42% for FR1 Indoor Hotspot and 15% and 27% for Dense Urban. UE only reports the best CSI. We set the same packet arrival rate (λ) for Scheme2 and Scheme3 as for Scheme1. Considerable UPT gain can be observed at 5% and 50% UPT, and mean UPT as well.

From the above tables, we observe that

Scheme1 has obvious performance gain compared to Scheme2 and Scheme3.

With RU increasing, the performance gain between Scheme1 and Scheme2/Scheme3 will increase.

Therefore, associating two reporting settings CSI-ReportConfigs which are corresponding to two TRPs can compensate the performance loss due to the delay caused by non-ideal backhaul between TRPs.

Associating two reporting settings CSI-ReportConfigs which are corresponding to two TRPs/TCI states can compensate the performance loss due to the delay caused by non-ideal backhaul between TRPs.

Indoor Hotspot with non-ideal backhaul

|  | FR1, RU for STRP | Mean UPT | 5% UPT | 50% UPT |
| --- | --- | --- | --- | --- |
| Scheme1 | 18%/42% | 0.00% | 0.00% | 0.00% |
| Scheme2 | 18% | -4.69% | -6.96% | -7.57% |
| Scheme3 |  | -21.51% | -37.50% | -29.88% |
| Scheme2 | 42% | -12.43% | -15.92% | -13.79% |
| Scheme3 |  | -35.44% | -45.29% | -38.42% |

Dense Urban with non-ideal backhaul

|  | FR1, RU for STRP | Mean UPT | 5% UPT | 50% UPT |
| --- | --- | --- | --- | --- |
| Scheme1 | 15%/27% | 0.00% | 0.00% | 0.00% |
| Scheme2 | 15% | -2.53% | -5.85% | -4.08% |
| Scheme3 |  | -10.38% | -33.49% | -14.92% |
| Scheme2 | 27% | -3.67% | -8.60% | -4.29% |
| Scheme3 |  | -16.35% | -36.95% | -21.18% |

For M-DCI based MTRP transmission, two CQIs are needed. Each CQI requires a different CPU to compute. Compared with reusing Cat1 for M-DCI, Cat2 can reuse the rule of CSI processing criteria without any further agreement, i.e., for CSI measurement associated with two reporting settings CSI-ReportConfigs for NCJT, an NCJT CSI hypothesis corresponding to a CSI-ReportConfig are assumed to occupy two CPUs and two active NZP CSI-RS resources. Occupation rule for CPU and active resources are defined per reporting setting.

For CPU occupation and active CSI-RS counting, Cat2 can reuse the rule defined for Cat1 without any further specification effort.

Another way for CSI report in non-ideal backhaul is to send the same CSI report obtained by Cat1 CSI-ReportConfig twice. This method will cause useless CSI report and increase feedback overhead significantly. Cat2 reporting scheme can divide the CSI quantities into two parts where each part corresponds to a PUCCH resource and a TRP to save the feedback overhead.

In conclusion, considering the performance, additional effort to define CPU occupation rules, and feedback overhead, we support to confirm the working assumption.

Support to confirm the work assumption in RAN1#103-e, i.e., Option1.

Considering the RAN1’s workload, we support to add some restrictions in RAN1#103-e to simplify the impact of Cat2. And, in our view, the design of Cat2 shall strive to find a commonality with the design of Cat1.

We discuss some details on CSI measurement and CSI reporting in the following subclauses.

CSI measurement

For the CMR configuration for NCJT, as shown in Figure 4, two CSI reporting settings with same field configuration except for the PUCCH resource are configured. With this configuration, as shown in Figure 13, almost all of the conclusions made for CSI measurement of Cat1 can be reused in Cat2 design, e.g., Nmax, Ks_max, CMR configuration, IMR configuration, signaling mechanism, CMR sharing, IMR sharing, etc.

Support to associate two CSI reporting settings with CMRs configuration same as Cat1 for Cat2 configuration.

CSI reporting settings for Cat2

Reporting mechanism

For CRI reporting and CRI bit width definition, Cat2 also can reuse the design of Cat1. The main difference between Cat1 and Cat2 for M-DCI is the UCI components reported to two TRPs of a CSI report.

For CRI and CRI bit width definition, Cat2 also can reuse the design of Cat1 with CSI reportings configured with CMRs same as for Cat1.

When dividing the CSI quantities, e.g., 2PMI, 2CQI, 2RI, into two parts, how to associate the CSI quantities in each part with the CSI reporting setting needs to be considered. In our opinion, dividing the CSI quantities corresponding to a NCJT hypothesis is implemented by associating the CMR group with the PUCCH resource. For example, two CMR groups are configured in each CSI reporting setting as shown in Figure 5. CMR group0 is associated with PUCCH resource0 and CMR group1 is associated with PUCCH resource1. For NCJT hypothesis reporting, same CRI indicating a NCJT hypothesis is reported in each report. The PMI, CQI, and RI corresponding to the CMR in CMR Group0 and CMR Group1 are reported on PUCCH resource0 and PUCCH resource1, respectively.

Association mechanism between the CMR group and PUCCH resource

Support to specify rules on how to divide and map the generated UCI into two associated reports in Cat2.

RRC signalling

Five preliminary RRC parameters are listed for MTRP CSI enhancement in [4] as shown in Table 13.

RRC parameters for MTRP CSI enhancement

| Sub-feature group | RAN2 Parant IE | Parameter name in the spec | Parameter name in the text | Value range | … |
| --- | --- | --- | --- | --- | --- |
| CSI-mTRP | NZP-CSI-RS-ResourceSet |  | Two CMR groups | Up to RAN2 |  |
| CSI-mTRP | TBD |  | N CMR pairs | Up to RAN2 |  |
| CSI-mTRP | TBD | sharedCMR |  | {enabled} |  |
| CSI-mTRP | CSI-ReportConfig | csi-ReportMode |  | {Mode 1, Mode 2} |  |
| CSI-mTRP | CSI-ReportConfig | numberOfSingleTRP-CSI-Mode1 |  | {0,1,2} |  |

In our opinion, CMR-related parameters should be configured in the CSI-RS resource set and the reporting-related parameters should be configured in the reporting setting. If “SharingCMR” and “N CMR pairs” are under CSI-ReportConfig, it means that all CSI-RS resource sets associated with the CSI report have the same “SharingCMR” and “N CMR pairs”, which have some risks. For example, for aperiodic CSI resource setting where the number of CSI-RS resource sets configured is not limited to S=1, configuring “SharingCMR” and “N CMR pairs” in CSI-ReportConfig means that multiple CSI-RS resource sets are configured with same configurations of “SharingCMR” and “N CMR pairs”, which has lower flexibility.

Regarding the comments that a NZP-CSI-RS-ResourceSet can be referred by two CSI-ReportConfigs with different CMR pair configurations, we think a NZP-CSI-RS-Resource can be referred by two CSI-ResourceConfigs with different CMR pairs configurations, which is allowed and flexible enough in the current specification.

For the configuration of CSI-IM-ResourceSet, defining a mapping rule between CMR/CMR pair and IMR is enough such as one-to-one mapping between IMR and CMR, and one-to-one mapping between IMR and CMR pair.

“SharingCMR” and “N CMR pairs” should be configured in NZP-CSI-RS-ResourceSet.

CSI enhancement for Partial Reciprocity

Report of W1

In previous meetings, it was agreed that the selected result of W1 is reported by combinatorial coefficients. In Rel-15 and Rel-16, combinatorial coefficients for beam selection is calculated as

icom= i=0L-1C(N-1-n(i), L-i)

where L is the number of selected beams is L which is always less than half number of N candidcate beams. However, in Rel-17, the number of selected beams can be 0.75 or 1.0 times of the number of candidate beams, which means the combinatorial coefficients table needs to be extended, as given in Table 14, to support more combinatorial coefficients if following the Rel-15/Rel-16 approach.

The total combinatorial coefficients table

| y x | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 3 | 3 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 4 | 6 | 4 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 5 | 10 | 10 | 5 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 6 | 15 | 20 | 15 | 6 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | 7 | 21 | 35 | 35 | 21 | 7 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 8 | 28 | 56 | 70 | 56 | 28 | 8 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | 9 | 36 | 84 | 126 | 126 | 84 | 36 | 9 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | 10 | 45 | 120 | 210 | 252 | 210 | 120 | 45 | 10 | 1 | 0 | 0 | 0 | 0 | 0 |
| 11 | 11 | 55 | 165 | 330 | 462 | 462 | 330 | 165 | 55 | 11 | 1 | 0 | 0 | 0 | 0 |
| 12 | 12 | 66 | 220 | 495 | 792 | 924 | 792 | 495 | 220 | 66 | 12 | 1 | 0 | 0 | 0 |
| 13 | 13 | 78 | 286 | 715 | 1287 | 1716 | 1716 | 1287 | 715 | 286 | 78 | 13 | 1 | 0 | 0 |
| 14 | 14 | 91 | 364 | 1001 | 2002 | 3003 | 3432 | 3003 | 2002 | 1001 | 364 | 91 | 14 | 1 | 0 |
| 15 | 15 | 105 | 455 | 1365 | 3003 | 5005 | 6435 | 6435 | 5005 | 3004 | 1365 | 455 | 105 | 15 | 1 |

However, with the extension of the table, the required memory for the table increases with many identical elements. Also, with increasing L, the computation complexity of combinatorial coefficients increases. For example, when the number of CSI-RS ports is 32 and K1 is 24, it means the combinatorial coefficients is calculated with 1612, and 12 iterations are needed . The operation of each iteration is a table look-up, which means the complexity of each iteration is the same. So the computation complexity is proportional to the iteration number. When the number of reported beams is larger than half of the number of candidate beams, the calculation complexity of the combinatorial coefficients of not reported beams is less than the complexity of the combinatorial coefficients of reported beams. Now that both gNB and UE know the number of reported beams and the number of candidate beams, the combinatorial coefficients of non-selected beams can be reported to derive the selected beams when the number of selected beams is larger than half of the number of candidate beams. In this way the computation iteration number is no more than N/2 for all cases. Also the combinatorial coefficients table can be shortened as Table 15.

The shortened combinatorial coefficients table

| y x | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 2 | 1 | 0 | 0 | 0 | 0 | 0 |
| 3 | 3 | 3 | 1 | 0 | 0 | 0 | 0 |
| 4 | 4 | 6 | 4 | 1 | 0 | 0 | 0 |
| 5 | 5 | 10 | 10 | 5 | 1 | 0 | 0 |
| 6 | 6 | 15 | 20 | 15 | 6 | 1 | 0 |
| 7 | 7 | 21 | 35 | 35 | 21 | 7 | 1 |
| 8 | 8 | 28 | 56 | 70 | 56 | 28 | 8 |
| 9 | 9 | 36 | 84 | 126 | 126 | 84 | 36 |
| 10 | 10 | 45 | 120 | 210 | 252 | 210 | 120 |
| 11 | 11 | 55 | 165 | 330 | 462 | 462 | 330 |
| 12 | 12 | 66 | 220 | 495 | 792 | 924 | 792 |
| 13 | 13 | 78 | 286 | 715 | 1287 | 1716 | 1716 |
| 14 | 14 | 91 | 364 | 1001 | 2002 | 3003 | 3432 |
| 15 | 15 | 105 | 455 | 1365 | 3003 | 5005 | 6435 |

For example, when the number of CSI-RS ports is 32 and K1 is 24, it means UE needs to report the selected 12 beams out of 16 candidate beams. If UE calculates the combinatorial coefficients 1612, it needs to do 12 iterations. By the approach of reporting non-selected beams, there are only non-selected 4 beams and thus only 4 iterations are enough for the combinatorial coefficients 164. Also, it only needs to look up at most 4 coloums in Table 15, while the combinatorial coefficients 1612 needs to look up at most 12 columns in Table 14.

From the perspective of gNB’s combinatorial coefficient decoding, it is obvious that the computation complexity can also be reduced by decoding from the combinatorial coefficients of non-selected beams when the number of selected beams is larger than half of the number of candidate beams.

UE reports the combinatorial coefficients of non-selected beams when the number of selected beams is larger than half of the number of candidate beams.

Report of W2

Agreement If a bitmap for indicating non-zero coefficients can be absent, down-select one Alt from the following for Rel-17 PS codebook: - Alt 1: At least for rank 1 PMI, the bitmap of indicating non-zero coefficients is not needed if Mv=1 and Beta=1. - FFS the need for Mv>1 and/or Beta<1 - Alt 2: For rank 1 /2 PMI, the bitmap(s) of indicating non-zero coefficients for corresponding layer(s) is absent if reported KNZ=K1*Mv*rank - Where KNZ is the number of non-zero coefficients - Alt 3: In addition to Alt 2, additional field is reported by UE to inform whether the bitmap of indicating non-zero coefficients for specific layer is absent if rank>1. - Alt 4: The bitmap of indicating non-zero coefficients is not needed if the number of coefficients is sufficiently small, i.e. K1Mv ≤ δ Note: If none of above Alternative is agreed in RAN1#106bis-e, the bitmap for indicating non-zero coefficient is always present by default.

In last meeting, there are four alternatives about the situations where non-zero coefficients bitmap can be absent. For Alt 1, we find that there are some opportunities that not all coefficients are reported even if Mv = 1 or beta = 1 so that the bitmap may not be absent. Now that a UE can skip some useless coefficients to report to reduce the CSI feedback similar to Rel-16, there is no need to omit the bitmap. For Alt 4, we think a UE can still skip some coefficients even if the number of coefficients is small.

On the other sides, if the bitmap is absent, all the coefficients need to be reported, which is not a common case for large CSI-RS ports number. And for the case of small CSI-RS port number, the bit number of the bitmap is not very large, which means it is not necessary to save the bits. Therefore, we think the bitmap may not be absent.

Always keep the bitmap for indicating non-zero coefficients for W2 for CSI reporting.

Report of Wf

Agreement Following working assumption is confirmed (with revision in RED): - At least for rank 1 and 2, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1. - FFS other restrictions, e.g. value(s) of N, if the value of N3 is small - FFS other restrictions, e.g. when the number of CSI-RS ports is small Agreement At least for rank 1/2 and Mv > 1, for relationship between N and Mv, support following alternative - Alt 2-1: N >= Mv, Wf is layer-common and reported by UE for N>Mv. - For Mv=2, N=2 and one value from {3, 4, 5} - RAN1 to select one value from {3, 4, 5} in RAN1#106bis-e - FFS: how to report Wf in terms of reporting mechanism and associated bits when Mv=2 and N=one value from {3, 4, 5} Note: Wf is layer-common for N=Mv Note: For all alternatives, a layer-common window/set of size N is configured. Agreement For Rel-17 PS codebook, following values of R are supported: - R = 1 and - At most one value from {2, D* NPRBSB} - FFS: which one is to be decided in RAN1#106bis if support, and applicable conditions, e.g. whether the support of this feature when Mv=1 - D is the density of CSI-RS in frequency domain and NPRBSB is the subband size in PRBs - Note that this R is optional if supported

The window size for searching FD bases will influence the number of candidate SD-FD pairs. When the number of CSI-RS ports is large, there are enough candidate SD-FD pairs to convey SD information and FD information. A single short window is enough to supplement extra FD information. When the number of CSI-RS ports is small, the candidate SD-FD pairs can only convey SD information but not enough FD information. A larger window size is needed to supplement enough FD information. In this case, a short non-consecutive window can replace the large window because the effective FD information in the window is discrete. Therefore, we think the indicating window should be consecutive for large number of CSI-RS ports while it can be consecutive and non-consecutive for small number of CSI-RS ports.

The simulation is conducted for different number of CSI-RS ports. The window size is set to 4, Mv = 2, beta = 1.0 and alpha = 1.0. Only rank 1 is considered in the simulation.

Case 1: Consecutive delay window.

Case 2: Non-consecutive delay window

The average throughput of consecutive and non-consecutive delay window

| The number of CSI-RS ports | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- |
| Average throughput of case 1 | 2.765 | 2.98 | 3.105 | 3.224 | 3.303 |
| Average throughput of case 2 | 2.88 | 3.063 | 3.189 | 3.284 | 3.335 |
| Gain between case 2 and case 1 | 4.16% | 2.79% | 2.71% | 1.86% | 0.97% |

According to the simulation results, when the number of CSI-RS ports is no more than 8, there is evident throughput gain between non-consecutive delay window and consecutive delay window. It is because that non-consecutive delay window can be equivalent to a larger delay window with more accurate delay information report. When the number of CSI-RS ports is more than 8, the gain reduces because the FD information in the precoded CSI-RS is sufficient to provide abundant FD information and a larger delay window is useless.

Note that no matter the delay window is consecutive or non-consecutive, the CSI feedback overhead is the same because the window size is the same. The average throughput of non-consecutive delay window is better than the consecutive one. A non-consecutive window can be equivalent to a larger consecutive window because gNB excludes some invalid delays. In our opinion, consecutive window is suitable to the concentrated delay distribution, while non-consecutive window is suitable to discrete delay distribution to better chase the delay distribution. Thus, whether the window is consecutive or non-consecutive can be configured or indicated by gNB.

Also, we think the window size can not be larger than N3 and N can be configured independent of N3. So when N3 is smaller than configured N, the N actually used should be min{N, N3}.

The FD bases in the window can be consecutive or non-consecutive when the number of CSI-RS ports is less than 8.

The actually used N should be min{N, N3} for the case of small N3.

As for the value of N when M = 2, the performance is evaluated with 4 CSI-RS ports and 32 CSI-RS ports. In the simulation, alpha = 1.0 and beta = 1.0. The same CSI overhead is used for different window size to eliminate the influence of Wf report. According to the simulation results, the performance gain between N = 4 and N = 5 is unconspicuous while the gain among N = 2, 3, 4 is evident. It seems that N = 4 is suitable for UE selection.

There is evident gain of N>=M in the case that the number of CSI-RS ports is 4.

When Mv = 2, the gain from N = 4 is evident.

The average throughput gain from different window length

|  | 4 CSI-RS ports | 32 CSI-RS ports |
| --- | --- | --- |
| N = 3 vs. N = 2 | 1.08% | 0.11% |
| N = 4 vs. N = 2 | 1.90% | 0.25% |
| N = 5 vs. N = 2 | 1.94% | 0.26% |

As for the report of Wf, we think a 3-bit combinatorial coefficient is suitable for the result of 42 . If UE searches the strongest beam as the tap 0 and always report the tap 0 coefficient, a 2-bit coefficient is enough for the result of the other delay location.

Support N = 4.

Wf can be reported with a 2-bit or 3-bit combinatorial coefficient.

- As for the value of R, we think it is necessary to clarify the meaning of R, whether it is the granularity of precoded CSI-RS at gNB side or the granularity of PMI report at UE side. If R is the granularity of precoded CSI-RS at gNB side, we think a larger R can provide higer accuracy to improve the frequency-domain precison. The FD bases have been precoded in CSI-RS so the larger R does not need more CSI feedback. If R is the PMI report at UE side, when Mv = 1, UE does not need to know the value of R. But when Mv > 1, UE needs to know it for the selection in the window. With the understanding of R as the granularity of precoded CSI-RS at gNB side, we perform simulation for different values of R=1, 2, D* NPRBSB assuming D = 1 and NPRBSB =8  with ideal channel estimation. In the simulation, Mv = 1 and the number of CSI-RS ports is 32. According to the simulation results, with the increasing R, at most 17% performance gain can be achieved.

However, the channel estimation is ideal in the simulation and one channel fading is estimated for each PRB, which is difficult in actual channel estimation. Considering about the accuracy of channel estimation, we slightly prefer the extra value of R is D* NPRBSB.

Slightly prefer R = D* NPRBSB for Mv > 1.

Parameter combination

Agreement Support parameter combinations represented by (alpha, Mv, beta) with K1 = alpha*P for Rel-17 PS codebook - The candidate values of alpha are {1/2, 3/4, 1} - Note that exact parameter combination will be discussed from RAN1 106bis: - based on trade-off among UPT performance, feedback overhead, and complexity - based on all supported ranks - Limit total number of parameter combinations comparable to Rel-16 eType II - Mv={1, 2} and beta = {[1/4], 1/2, 3/4, 1} are from previous agreements

For the parameter combinations, we think at most 8 combinations as same number of combination of Rel-16 regular Type II codebook need to be support. We evaluate 24 different cases with alpha = [1/2, 3/4, 1], M = [1, 2], beta = [1/4, 1/2, 3/4, 1] for 32 and 8 CSI-RS ports respectively, and window size is 4 for M=2. Only rank 1 is evaluated. The simulation results are shown in Figure 6 and Figure 7.

The average throughput gain of different parameter combinations for 32 CSI-RS ports

The average throughput gain of different parameter combinations for 8 CSI-RS ports

According to the simulation results, we divide the average throughput gain into 8 bins, in each of which we choose the point with the least CSI feedback as the optimal choice. The 8 selected combinations of (alpha,Mv,Beta) for 32 CSI-RS ports are (0.5,1,0.25), (0.75,1,0.25), (0.5,1,0.5), (1,1,0.25), (0.5,2,0.25), (0.75,2,0.25), (0.5,2,0.5), (0.75,2,0.5). And the 8 selected combinations of (alpha,Mv,Beta) for 8 CSI-RS ports are (0.75,1,0.25), (0.75,1,0.5), (0.75,1,0.75), (1,2,0.25), (0.75,2,0.5), (1,2,0.5), (1,2,0.75), (1,2,1).

From the above selected combinations, we can see the minimum value of alpha can be 0.5 for 32 CSI-RS ports, while the minimum value of alpha can be no less than 0.75 for 8 CSI-RS ports; and the beta can be at most 0.5 for 32 CSI-RS ports, while 1.0 can be used for 8 CSI-RS ports. Therefore, we think one limitation can be that the minimum of alpha increases with decreasing number of CSI-RS ports and the maximum of beta decreases with increasing number of CSI-RS ports.

The minimum of alpha increases with decreasing number of CSI-RS ports and the maximum of beta decreases with increasing number of CSI-RS ports.

Timing calibration and complexity

As we discussed in [5], there are some timing issues for UE for FDD CSI enhancement in Rel-17. There may be a timing offset between uplink channel and downlink channel even if the delay is reciprocal. Following reasons could cause timing offset:

Usually, when the UE processes timing calibration to synchronize with gNB, UE only adjusts to the reception timing to make the delay interval less than CP length. But the delay location in the uplink channel observed by gNB and the downlink channel observed by UE may be different. For FDD CSI enhancement, gNB and UE need to be aligned on the exact delay tap UE needs to estimate and feedback the amplitudes and phases, e.g. on Tap 0;

To ensure all paths can be received successfully, the UE may start receiving a few samples before the regular start point of an OFDM symbol. The UE reception timing is unknown to the gNB, thus UE would need to estimate which tap is the intended Tap 0 from gNB, e.g. tap 0 based on the tap with the strongest power.

Illustration of timing mismatch

This timing mismatch does not influence Rel-15/16 Type II (PS) codebook performance. On the one hand, the timing offset is the same for all paths, thus such delay only adds a subband specific phase offset and does not change the PMI feedback and also does not influence the frequency selectivity. On the other hand, both CSI measurement and PDSCH reception are based on the same timing offset making little difference in the sense of downlink channel timing between them. Therefore, the Rel-15/16 codebook is not sensitive to timing mismatch.

However, for Rel-17 Type II PS codebook, the influence of timing mismatch is severe. Different delay locations of each path in uplink channel and downlink channel will lead to mismatch between the FD information gNB uses to precode CSI-RS ports and the real downlink channel, which violates the assumption of delay reciprocity. As shown in Figure 9, the timing mismatch between gNB and UE is 1 which means the delay tap 0 for gNB being delay tap -1 for UE. For each CSI-RS port, gNB shifts each SD-FD basis or SD basis to delay tap 0 respectively but UE detects nothing on tap 0. While for CSI-RS port 1, there are two paths besides delay tap 0, so UE cannot search for the strongest path to find the correct delay location of the CSI-RS port.

The influence of timing mismatch

Therefore, the timing mismatch can cause the wrong coefficients to report, even zero. The simulation results are shown Figure 10. SD and FD precoding applied on CSI-RS ports which is derived with SVD method and the total number of SD-FD pairs are 32. According to the simulation results, the influence of timing mismatch is severe, which can destroy the delay reciprocity which causes significant loss in performance.

The performance loss under timing mismatch

- To solve this problem, UE can process timing calibration to counteract the timing mismatch. Usually, UE may process timing calibration on all CSI-RS ports, which means, on each CSI-RS port, an FFT should be calculated, which surely causes a lot of UE complexity. The reason why more than one CSI-RS ports are needed for timing calibration lies in two aspects: 1) tap 0 must be searched from multiple CSI-RS ports as the strongest delay locations per CSI-RS port  are different caused delay shifting by precoding on CSI-RS ports; 2) different variations of tap 0 per CSI-RS port due to uplink and downlink channel partial reciprocity need to be averaged out from multiple CSI-RS ports.

- In fact, UE only needs to process timing calibration on very few CSI-RS ports because the timing mismatch is the same on each CSI-RS port and only a few strongest CSI-RS ports are sufficient to locate the delay tap 0 or some specific tap. The simulation results about different numbers of CSI-RS ports used to process timing calibration compared to all CSI-RS ports are shown in Figure 11. The number of CSI-RS ports is 32 and the length of tap indication set is one. In the simulation, gNB maps the SD-FD pairs from the strongest to the weakest to CSI-RS ports in order. L CSI-RS ports conveying strongest SD-FD pairs are used for timing calibration.

The performance loss with different detecting ports number

- According to the simulation results, the performance loss with more than 4 or 6 strongest CSI-RS ports is similar to all 32 ports. In most cases, 4 CSI-RS ports are enough to counteract the influence of timing interference. That is, with fewer CSI-RS ports to calibrate the timing mismatch, the performance improvement is close to using all ports.

With 4 or 6 strongest CSI-RS ports (observed and mapped at gNB side) for timing calibration between gNB and UE, the system performance is similar to that of using 32 ports for timing calibration.

UE can use partial CSI-RS ports to search target tap 0 to reduce the complexity.

  - gNB can map SD-FD bases to CSI-RS ports with a predetermined order or indicating the ports for timing calibration.

Conclusions

  - To summarize, we have following observations and proposals.

- Observation 1:

Associating two reporting settings CSI-ReportConfigs which are corresponding to two TRPs/TCI states can compensate the performance loss due to the delay caused by non-ideal backhaul between TRPs.

- Observation 2:

For CPU occupation and active CSI-RS counting, Cat2 can reuse the rule defined for Cat1 without any further specification effort.

- Observation 3:

For CRI and CRI bit width definition, Cat2 also can reuse the design of Cat1 with CSI reportings configured with CMRs same as for Cat1.

- Observation 4:

There is evident gain of N>=M in the case that the number of CSI-RS ports is 4.

When Mv = 2, the gain from N = 4 is evident.

- Observation 5:

With 4 or 6 strongest CSI-RS ports (observed and mapped at gNB side) for timing calibration between gNB and UE, the system performance is similar to that of using 32 ports for timing calibration.

- Proposal 1:

The default maximum number of CMR is 2 with Rel-17 MIMO UE capability for MTRP CSI measurement.

- Proposal 2:

For CSI measurement associated with a CSI-ReportingConfig for NCJT, following restrictions are applied for two CMRs within the same CMR pair configured for NCJT measurement hypothesis:

UE does not expect any uplink symbols between two CMRs within the same CMR pair.

Two CMRs are restricted with the same CDRX active time

- Proposal 3:

For the number of CSI-IM resources

M is equal to Ks when the CMR sharing is enabled.

M is equal to Ks – 2N when the CMR sharing is disabled.

- Proposal 4:

Support no change to definition or configuration of Pc_ratio.

- Proposal 5:

For non-PMI CSI feedback, when CMR sharing between NCJT hypothesis and STRP hypothesis is enabled

When non-PMI-PortIndication is configrued, separate non-PMI-PortIndication parameters are configured where one is for STRP hypothesis and the other is for NCJT hypothesis.

When non-PMI-PortIndication is not configured, an enhanced rule can be considered, e.g., the first v ports are associated with the rank v of STRP hypothesis measurement and the last k ports are associated with the rank k of NCJT hypothesis measurement.

- Proposal 6:

For RI restriction, support Alt 5 or Alt 6 considering the flexibility.

For codebook subset restriction, support TRP-specific CBSR.

- Proposal 7:

A CSI reporting setting configured with enhanced MTRP CSI reporting corresponds to a CSI report.

- Proposal 8:

Slightly prefer to use legacy priority definition and omission rule with a granularity of CSI report level.

- Proposal 9:

Support to enhance the CSI reporting mechanism when PMI and CQI granularity are wideband.

- Proposal 10:

Support to confirm the work assumption in RAN1#103-e, i.e., Option1.

- Proposal 11:

Support to associate two CSI reporting settings with CMRs configuration same as Cat1 for Cat2 configuration.

- Proposal 12:

Support to specify rules on how to divide and map the generated UCI into two associated reports in Cat2.

Proposal 13:

“SharingCMR” and “N CMR pairs” should be configured in NZP-CSI-RS-ResourceSet.

Proposal 14:

UE reports the combinatorial coefficients of non-selected beams when the number of selected beams is larger than half of the number of candidate beams.

Proposal 15:

Always keep the bitmap for indicating non-zero coefficients for W2 for CSI reporting.

Proposal 16:

The FD bases in the window can be consecutive or non-consecutive when the number of CSI-RS ports is less than 8.

The actually used N should be min{N, N3} for the case of small N3.

Proposal 17:

Support N = 4.

Wf can be reported with a 2-bit or 3-bit combinatorial coefficient.

Proposal 18:

Slightly prefer R = D* NPRBSB for Mv > 1.

Proposal 19:

The minimum of alpha increases with decreasing number of CSI-RS ports and the maximum of beta decreases with increasing number of CSI-RS ports.

- Proposal 20:

UE can use partial CSI-RS ports to search target tap 0 to reduce the complexity.

- gNB can map SD-FD bases to CSI-RS ports with a predetermined order or indicating the ports for timing calibration.

References

Chairman’s notes for 3GPP TSG RAN WG1 #106-e Meeting

3GPP TS 38.214 V16.6.0 (2021-06)

3GPP TS 38.213 V16.6.0 (2021-06)

Post_RAN1#106-e_Rel-17_RRC FeMIMO_CSI_Rev (Mod).xlsx

R1-2007650, Further discussion and evaluation on MTRP CSI and partial reciprocity

- Appendix A: SLS simulation setup and assumptions

We conduct a performance evaluation for eMBB in FR1 4GHz carrier frequency with 10MHz BW and 15kHz SCS. MTRP transmission with non-ideal backhaul and ideal backhaul assumptions are evaluated such that independent scheduling is assumed in each TRP per cluster for non-ideal backhaul and joint scheduling is assumed per cluster for ideal backhaul. STRP (STRP) scheme is assumed as baseline. SU-MIMO is assumed for STRP, DPS, and DPS+NCJT cases. Detailed simulation assumptions can be found in Appendix D.

Scenario

In the Indoor Hotspot scenario, a TRP cluster comprises four neighboring TRPs as shown in Figure 12. Whereas in the Dense Urban scenario, a TRP cluster comprises three neighboring TRPs of a site. A UE measures the RSRP of all TRPs in the cluster, associates with a serving TRP in the cluster, and selects at most one candidate coordinating TRP in the same cluster, with the RSRP gap lower than a predefined threshold compared to the serving TRP.

TRP clustering for Indoor Hotspot

CSI calculation method

For DPS/STRP CSI, PMI and CQI are calculated as in Rel-16, where PMI is obtained by measurement over CSI-RS resource for channel measurement (CMR) of either TRP and CQI is derived from the CMR and CSI-RS resource for interference measurement (IMR).

For NCJT CSI, PMIs are obtained by measuring CMRs of each TRP. The joint equivalent MIMO channel assuming NCJT is given by He=H1W1, H2W2, where H1, H2 are estimated channels by the CMRs from the two TRPs, and W1, W2 are the precoders corresponding to the PMIs of the two TRPs. Then the CQI can be derived from per layer post-SINRs which are calculated assuming MIMO detection of the equivalent channel He and interference measured by the IMR from outside other than the two TRPs.

CSI feedback schemes

STRP transmission

UE reports the CSI to its recommended transmitting TRP.

DPS transmission

For non-ideal backhaul, the UE compares the estimated throughput for two possible DPS transmitting TRPs within the cluster and reports the CSI with maximal estimated throughput to its recommended transmitting TRP.

For ideal backhaul, UE reports the corresponding DPS CSI to each possible DPS transmitting TRPs within the cluster.

NCJT+DPS transmission

For non-ideal backhaul, the feedback method is consistent with DPS. The difference is that UE needs to compare two possible DPS CSIs with one possible NCJT CSI and select the best CSI for feedback. If UE reports NCJT CSI, rank 1 or 2 is chosen per TRP to maximize the NCJT estimated overall throughput.

For ideal backhaul, UE reports the DPS CSI and NCJT CSI to each possible DPS transmission and NCJT TRP within the cluster.

Scheduling mechanisms

STRP transmission

UE selects a serving TRP based on RSRP, and the serving TRP schedules the UE connected to the TRP according to the proportional fair algorithm.

DPS transmission

For non-ideal backhaul, the scheduler per TRP in the cluster schedules one UE which has reported its DPS CSI to the TRP according to the proportional fair algorithm. With one optimal DPS CSI to report, non-overlapping PDSCH reception from different TRPs in the time domain is achieved.

For ideal backhaul, the scheduler per cluster schedules one UE which has reported all DPS CSIs to the TRPs within the cluster according to the proportional fair algorithm. With a coordinated scheduler, non-overlapping PDSCH reception from different TRPs in the time domain is achieved.

DPS+NCJT transmission

In the non-ideal backhaul scenario, each TRP in the cluster schedules one UE which has reported its CSI, either DPS CSI or NCJT CSI, independently according to the proportional fair algorithm. With a non-ideal backhaul assumption, the scheduler of a TRP is not aware of the scheduling results of another TRP at the same time, which may result in full or partially-overlapped PDSCHs reception at the UE. One codeword per TRP is transmitted to the UE when the scheduler is NCJT.

As a result, if two TRPs happen to schedule the same UE in one subband simultaneously, the transmission layers from two TRPs to the UE can be one out of (1, 1), (1, 2), (2, 1), and (2, 2) with total transmission layers being 2, 3, or 4 since 4 Rx antenna ports are assumed at the UE.

For ideal backhaul, the scheduler per cluster schedules one UE which has reported its all CSIs to the cluster according to the proportional fair algorithm. A UE will receive a PDSCH with its layers from different TRPs in the case of NCJT scheduling.

Receiver

In the case of STRP/DPS transmission, the estimated equivalent channel measured on DMRS at the receiver can be given by

He=HW

where H is a channel from the target TRP, and W is the precoder of the target TRP. Then the per layer post-SINR can be calculated assuming MIMO detection of the equivalent channel He.

In the case of NCJT from two TRPs, the estimated joint equivalent channel measured on DMRS at the receiver can be given by

He=He1, He2

Where He1=H1W1 , He2=H2W2, and H1, H2 are channels from the two TRPs, W1, W2 are the precoders of the two TRPs. Then the per layer post-SINR can be calculated assuming MIMO detection of the joint equivalent channel He.

It is worth noting that the above simulation assumptions are irrelevant to the frequency range. The main difference between FR2 and FR1 is beam based scheduling. For the MTRP CSI simulation for FR2, additional simulation assumptions are taken into account:

Each panel of the MPUE independently accesses the optimal TRP with the RSRP gap between multiple panels lower than a predefined threshold.

UE reports the corresponding CSIs based on the optimal beam.

The scheduler schedules the UEs under one optimal beam based on the proportional fairness algorithm.

The potential problem with the above assumption for FR2 is that the number of UEs who can be served with MTRP transmission is reduced and the resource utilization decreases compared to FR1.

Figure 13 shows the comparison of UE accessing multiple TRPs in FR1 and FR2. In Indoor Hotspot scenario, the total number of dropped UEs is 5000, and the RSRP threshold for determining MTRP transmission is 6dB. Other simulation assumptions can be found in Appendix C.

Comparison of UE accessing multiple TRPs in FR1 and FR2

In Indoor Hotspot scenario, the same RSRP threshold for determining MTRP transmission may cause large differences in the number of MTRP transmission UEs between FR1 and FR2.

- Appendix B: SLS performance evaluation results

- B.1 Cat1 and Cat2 for non-ideal backhaul

For M-DCI based MTRP transmission in non-ideal backhaul scenario, it is difficult to coordinate transmission among TRPs due to the large latency in backhaul. It is reasonable for different TRPs to independently schedule and determine transmission resources. From the simulation results shown below, M-DCI based NCJT transmission with NCJT CSI enhancement can bring obvious performance gains. Therefore, MTRP CSI enhancement for non-ideal backhaul scenarios is necessary.

The following tables show the UPT gain of three schemes compared to the baseline of STRP transmission.

- Scheme1 (DPS): UE selects the DPS CSI and reports it to the selected TRP.

- Scheme2 (two STRP CSIs report to both TRPs): UE reports two STRP CSI reports to both TRPs. When NCJT is scheduled (full or partial overlap), the two STRP CSI reports are used.

- Scheme3 (DPS+ NCJT): UE selects the NCJT CSI report and reports it to both TRPs, or UE selects the DPS CSI report and reports it to the selected TRP.

For the MTRP transmission in non-ideal backhaul scenario, as described in Appendix A, each TRP is independently scheduling without CSI exchange between TRPs. Other simulation parameters can be found in Appendix D. We provide UPT comparison for FTP model 1 with RU for baseline STRP set to 16% and 38% for FR1 Indoor Hotspot and 14% and 25% for Dense Urban. UE only report the best CSI. We set the same packet arrival rate (λ) for DPS and DPS+NCJT as for STRP. Considerable UPT gain can be observed at 5% and 50% UPT, and mean UPT as well.

DPS and DPS+NCJT vs. STRP for Indoor Hotspot with non-ideal backhaul

|  | FR1, RU for STRP | Mean UPT | 5% UPT | 50% UPT |
| --- | --- | --- | --- | --- |
| STRP | 16%/38% | 0.00% | 0.00% | 0.00% |
| Scheme1 | 16% | 12.01% | 30.10% | 15.38% |
| Scheme2 |  | 41.53% | 27.61% | 25.00% |
| Scheme3 |  | 48.18% | 39.26% | 33.92% |
| Scheme1 | 38% | 25.10% | 34.89% | 33.60% |
| Scheme2 |  | 28.26% | 34.40% | 21.87% |
| Scheme3 |  | 46.26% | 55.30% | 43.80% |

DPS and DPS+NCJT vs. STRP for Dense Urban with non-ideal backhaul

|  | FR1, RU for STRP | Mean UPT | 5% UPT | 50% UPT |
| --- | --- | --- | --- | --- |
| STRP | 14%/25% | 0.00% | 0.00% | 0.00% |
| Scheme1 | 14% | 1.41% | 8.50% | 0.00% |
| Scheme2 |  | 9.20% | 3.10% | 1.96% |
| Scheme3 |  | 15.38% | 16.08% | 10.64% |
| Scheme1 | 25% | 2.21% | 7.39% | 2.95% |
| Scheme2 |  | 4.23% | 2.57% | 0.00% |
| Scheme3 |  | 9.33% | 15.98% | 4.22% |

From the above tables, we observe that

Scheme2 and Scheme3 have obvious performance gain compared to Scheme1.

Scheme3 has an obvious performance gain compared the Scheme2.

The reason for Scheme3 has some UPT gains compared with Scheme2 is contain two aspects:

MCS mismatch may often happen in Scheme2, resulting in performance degradation.

Even if TRPs schedule independently, NCJT transmission to some UEs happens, especially when the RU is lower.

The reason for the mean UPT gain of Scheme2 and Scheme3 compared with Scheme1 is that up to 4 transmission layers from two TRPs can be scheduled in NCJT while transmission layers are restricted to 1 or 2 for DPS.

For MTRP CSI enhancement Cat1, UE reports the CSI to one of the two TRPs. This would entail frequent coordination between different TRPs and thus inappropriate for practical deployment. A possible solution might be configuring two Cat1 CSI reporting settings, each one fed back to a TRP. But this is a waste of both UE computation power and reporting resources.

Cat2 is more suitable for non-ideal backhaul since UE reports the relevant CSI part to the corresponding TRPs based on legacy CSI reporting setting configuration. With minimum specification effort, MTRPMTRP transmission and reception could obtain considerable gains as shown above.

- B.2 Cat1 and Cat2 for ideal backhaul

For ideal backhaul scenario, the following tables show the UPT gain with three schemes compared to the baseline with STRP transmission.

- Scheme1 (DPS): UE reports two DPS CSI reports to the NW. The NW schedules DPS transmission according to the two DPS CSI reports.

- Scheme2 (two STRP CSIs report to both TRPs): NW schedules NCJT according to the two STRP CSI reports.

- Scheme3 (DPS+ NCJT): UE reports the DPS CSI and NCJT CSI to each possible DPS transmission and NCJT TRP within the cluster.

For the MTRP transmission, as described in Appendix A, each cluster is jointly scheduling with no backhaul delay. Other simulation parameters can be found in Appendix D. We provide UPT comparison for FTP model 1 with RUs for baseline STRP set to 16% and 38% for FR1 Indoor Hotspot, 17%, and 28% for FR2 Indoor Hotspot, and 14% and 25% for Dense Urban. We set the same packet arrival rate (λ) for DPS and DPS+NCJT as for STRP. Considerable UPT gain can be observed at 5% and 50% UPT, and mean UPT as well.

DPS and DPS+NCJT vs. STRP for Indoor Hotspot with ideal backhaul

|  | FR1 RU for STRP | Mean UPT | 5% UPT | 50% UPT |
| --- | --- | --- | --- | --- |
| STRP | 16%/38% | 0.00% | 0.00% | 0.00% |
| Scheme1 | 16% | 7.00% | 14.05% | 8.70% |
| Scheme2 |  | 35.50% | 15.37% | 13.64% |
| Scheme3 |  | 40.54% | 18.06% | 20.97% |
| Scheme1 | 38% | 3.08% | 14.75% | 4.70% |
| Scheme2 |  | 14.74% | 20.95% | 7.59% |
| Scheme3 |  | 24.09% | 23.21% | 16.42% |

DPS and DPS+NCJT vs. STRP for Dense Urban with ideal backhaul

|  | FR1 RU for STRP | Mean UPT | 5% UPT | 50% UPT |
| --- | --- | --- | --- | --- |
| STRP | 14%/25% | 0.00% | 0.00% | 0.00% |
| Scheme1 | 14% | 2.33% | 8.18% | 4.00% |
| Scheme2 |  | 8.73% | 2.47% | 1.96% |
| Scheme3 |  | 13.13% | 9.45% | 8.33% |
| Scheme1 | 25% | 2.72% | 10.27% | 4.22% |
| Scheme2 |  | 2.92% | 3.34% | -2.63% |
| Scheme3 |  | 5.67% | 8.31% | 0.00% |

From the above tables, similar conclusions can be obtained compared as in non-ideal backhaul scenario. The reason for Scheme3 has some UPT gains compared to Scheme2 is due to MCS mismatch in Scheme2.

Besides, in ideal backhaul scenario, in this evaluation, UE reports all CSIs. The scheduler has more CSI information and determines the transmission scheme for the UE. In theory, the system can achieve better transmission performance than in the non-ideal backhaul scenario. However, a more complex scheduling algorithm is also required. If a sub-optimal scheduling algorithm is used, especially in a scenario with relatively large interference, it may cause system performance degradation with increased UE scheduling opportunities.

- Appendix C: Simulation parameters

SLS assumption for MTRP enhancement

| Parameters |  | Value |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| Duplex, Waveform |  | FDD, OFDM |  |
| Multiple access |  | OFDMA |  |
| Scenario |  | Indoor hotspot (InH), Dense Urban(Macro Only) | Indoor hotspot (InH) |
| Frequency Range |  | 4GHz | 30GHz |
| Inter-BS distance |  | 20m for InH, 200m for Dense Urban |  |
| Channel model |  | According to the TR 38.901 |  |
| Antenna setup and port layouts at TRP |  | InH: 2 Tx ports: (M, N, P, Mg, Ng) = (1,1,2,1,1) Dense Urban: 4 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,2,2,1,1,1,2) (dH, dV) = (0.5, 0.8)λ | InH: 2 Tx ports: (M, N, P, Mg, Ng) = (4,4,2,1,1) |
| Antenna setup and port layouts at UE |  | 4Rx Port: (M, N, P, Mg, Ng, Mp,Np) = (1,2,2,1,1,1,2) (dH,dV) = (0.5, 0.5)λ |  |
| BS Tx power |  | 23dBm for InH, 43dBm for Dense Urban |  |
| UE antenna height & gain |  | Follow TR36.873 |  |
| UE receiver noise figure |  | 9dB |  |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |  |
|  | SCS | 15kHz | 120kHz |
| Number of RBs |  | 52 |  |
| Simulation bandwidth |  | 10 MHz | 80 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |  |
| Configuration for MTRP | Cluster | 4 neighboring TRPs for InH, 3 neighboring TRPs Dense Urban(Macro Only) |  |
|  | Maximal number of coordinating TRPs | 2 |  |
|  | Backhaul assumption | Ideal and non-ideal |  |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms - Subband PMI, subband CQI - Rank 1 or rank 2 per TRP |  |
| Traffic model |  | FTP model 1 |  |
| UE receiver |  | MMSE-IRC |  |
| Channel estimation |  | Ideal |  |

SLS assumption for CSI enhancement based on FDD reciprocity

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Urban Macro |
| Frequency Range | FR1 only, 2GHz for uplink and 2.2GHz for downlink |
| Inter-BS distance | 200m |
| Channel model | Opt. 1: The reciprocity model of DL/UL channel is based on Section 5.3 of TR 36.897 |
| Antenna setup and port layouts at gNB | (8,8,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size=8448bit |
| Numerology | 14 OFDM symbol slot SCS 15KHz |
| Simulation bandwidth | 20 MHz for 15kHz as a baseline 10 MHz for 15KHz as contract |
| Frame structure | Slot Format 0 (all downlink) for all slots |
| MIMO scheme | SU/MU-MIMO with rank adaptation is assumed |
| Rank candidate | Rank 1 as a starting point |
| MIMO layers | The maximum MU layers 8 |
| CSI feedback | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead | Companies shall provide the downlink overhead assumption |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) | 70% for SU/MU-MIMO |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| Evaluation Metric | Throughput and CSI feedback overhead as baseline metrics. |
| Baseline for performance evaluation | Rel-16 PS eTypeII Codebook with CSI-RS beamforming based on the angle information from SRS according to partial reciprocity. |
| SRS modeling for UL channel estimation | SRS periodicity with 5ms SRS error modeling in Table A.1-2 in 36.897. - Use coupling loss instead of path loss. - Delta = 9dB |