3GPP TSG RAN WG1 #106bis-e                                                R1-2108967

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	CSI feedback enhancements for Rel-17 URLLC

Agenda Item:	8.3.1.2

Document for:	Discussion and Decision

- Introduction

In RAN#106 e-meeting, CSI feedback enhancements are discussed and the following agreements are achieved [1]:

Agreement

For subband CQI reporting with more than 2 bits per subband

- Support 4-bits CQI only

Agreement

For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration.

- This feature is subject to UE capability

- FFS: Whether wideband CQI report can be omitted

In this contribution, we share our views on CSI enhancements for Rel-17 URLLC.

- Wideband CQI for 4-bit sub-band CQI report

In RAN1#106 e-meeting, 4-bit subband CQI reporting is supported in Rel-17. A remaining issue is whether wideband CQI report can be omitted.

According to the current spec as shown in Figure 1, high layer parameter csi-ReportingBand configured by gNB can indicate a subset of subbands in the bandwidth part which CSI shall be reported for. It means gNB can indicate CSI reporting only for partial subbands from a subband set. In this case, wideband CQI report is still needed to derive CQI of subbands which are configured without CSI report.

On the contrary, if gNB configures CSI reporting for all subbands, gNB can derive wideband CQI from subband CQI and 4-bit wideband CQI report overhead can be saved. When all subbands report are configured for UE, the overhead is usually not essential issue. Considering the common WB-CQI functionality, no additional optimizations for 4-bit subband CQI are preferred in Rel-17.

CSI-ReportConfig information element

…….

Fig. 1 high layer parameters on CSI-ReportConfig

Proposal 1: For 4-bit subband CQI reporting, no additional optimizations are needed in Rel-17.

- wideband CQI is reported for 4-bit subband CQI reporting.

- Conclusion

In the contribution, we discuss CSI feedback enhancement for URLLC/IIOT, and propose that,

Proposal 1: For 4-bit subband CQI reporting, no additional optimizations are needed in Rel-17.

- wideband CQI is reported for 4-bit subband CQI reporting.

- References

[1] Chairman notes RAN1#106 e-meeting