3GPP TSG RAN WG1 #106b-e	R1-2109140

e-Meeting, October 11th – 19th, 2021

Title: 	Discussion on Rel-17 UE features for UL Tx switching

Source: 	ZTE

Agenda item:	8.17.17

Document for:	Discussion/Decision

Introduction

After RAN1#106-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for UL Tx switching.

Discussion

In Rel-16, we have the following per-BC UE feature to supported option for UL Tx switching for inter-band UL CA.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 22. NR Others | 22-1 | Indicating supported option for UL Tx switching for inter-band UL CA | Indicating supported option for UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2} |

Similarly, Rel-17 also specifies two different options for UL Tx switching for inter-band UL CA. From our perspective, it is not appropriate to always require UE to support the same option for Rel-16 UL Tx switching and Rel-17 UL Tx switching for iner-band UL CA. For example, UE may support “both Option1 and Option2” for Rel-16 1Tx-2Tx UL Tx switching, but it may only support “Option1” for Rel-17 UL Tx switching,

Rel-17 UL Tx switching is enhanced from two perspectives, 1) from 1Tx-2Tx to 2Tx-2Tx switching; 2) from 2-carrier case to 3-carrier case switching. It may be ok to support the same option for 2-carrier case and 3-carrier case. But different options should be allowed for 1Tx-2Tx switching and 2Tx-2Tx switching.

Thus, we propose to introduce the following UE feature for Rel-17 2Tx-2Tx UL Tx switching for both 2-carrier case and 3-carrier case.

Proposal 1: Introduce the following UE feature for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. [NR_RF_FR1_enh] | 37-1 | Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA | Indicating supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2} | FFS | Yes | N/A |  | Per BC | N/A | N/A | N/A |  | FFS details |

Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for UL Tx switching with the following proposal.

Proposal 1: Introduce the following UE feature for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. [NR_RF_FR1_enh] | 37-1 | Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA | Indicating supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2} | FFS | Yes | N/A |  | Per BC | N/A | N/A | N/A |  | FFS details |

Reference

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, RAN WG1 #106-e, Moderators (AT&T, NTT DOCOMO, INC).