3GPP TSG RAN WG1 #106bis-e            	   	                        R1-2109191

e-Meeting, October 11th – 19th, 2021

Source:	CATT, GOHIGH

Title:	Further discussion on sidelink resource allocation enhancements for power saving

Agenda Item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

At RAN1#106-e meeting, the following conclusions and agreements were made related to resource allocation for power saving in sidelink enhancements [1].

Agreement In periodic-based partial sensing, UE monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection. Agreement Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met: - L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool - FFS: When the trigger will be received by L1 - The resource pool is (pre-)configured to enable partial sensing - Partial sensing is configured by higher layer in the UE Agreement For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing, - Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc - Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable. - FFS: If additional enhancements are needed for enabling/disabling - The triggering of re-evaluation and pre-emption checking is as in R16. Agreement When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n, - The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - On the sensing window [n+TA, n+TB] for CPS, - Details of TA and TB values based on the agreements from previous RAN1 meetings - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability - FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability - Whether the set SA is confined within a set of Y candidate slots within the RSW - UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4 - Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately. - FFS: Details on T1 Agreement For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA) - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enable.

In this contribution, the resource allocation mechanisms on power saving schemes and the impacts of SL DRX are discussed for sidelink enhancements.

Discussion on resource allocation for power saving

In the previous RAN1 meeting, periodic-based partial sensing mechanism and contiguous partial sensing mechanism were discussed, and framework on sensing occasions was generally determined. Besides that, there are remaining issues which need further discussion as follows:

- Remaining issues of Periodic-Based Partial Sensing(PBPS) Resource Allocation(RA) scheme

- Remaining issues of Contiguous Partial Sensing(CPS) RA scheme

- Remaining issues of PBPS and CPS combined RA scheme

- Details of partial sensing for re-evaluation/pre-emption

- Random selection RA

- Coexistence of full sensing, partial sensing and random selection RA

Remaining issues of Periodic-Based Partial Sensing (PBPS) RA scheme

In RAN1 #105-e meeting and RAN1 #106-e meeting, PBPS RA scheme and details are discussed. Remaining issues are mainly about sensing occasions determination, which include issues of Preserve and additional sensing occasions.

Agreement: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE Agreement: - For the set of Preserve values in periodic-based partial sensing, - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList. - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList - This is per mode 2 Tx resource pool (pre-)configuration - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx - FFS whether the monitoring can be mandatory

In this section, remaining issues of sensing occasions and Y determination are discussed, including the following aspects:

- Discussion on working assumption of  K definition

- Enhancements on SCI reservation mechanism when (pre-)configuring K

- Remaining issue on  Preserve

- Determination of Y candidate slots for periodic transmission

    - Discussion on working assumption of K definition

There has been quite a few confusions caused by the working assumption in the agreement about K, some of the confusion are even about the agreement itself.

The first issue discussed is whether the value of particular periodic sensing occasion in the (pre-)configuration denotes a specific occasion to monitor or the earliest occasion to start the monitoring. This should never come into question, because the agreement says ‘If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values’. This already answers the question that the values in the set denote a specific occasion to monitor. If the interpretation is meant for earliest occasion to start the monitoring, it would contradict with the sentence in the agreement ‘to a set of values’, as only one value is needed to indicate the earliest occasion.

The second issue is what are the agreed values could possibly be in the set. This relates the working agreement. According to the working assumption, at least two values are included in the set, the first is the one correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the second one is the last periodic sensing occasion prior to the most recent one for the given reservation periodicity.  This means with (pre-)configuration, the UE could monitor only one occasion that correspond to the most recent sensing occasion, or it could also monitor the last periodic occasion prior to the most recent one.  Note even with (pre-) configuration, the UE could still only monitor the most recent sensing occasion. For other monitoring occasion, it is still open to discuss.

One opening issue is whether the configuration is per resource pool, or per periodicity. According the agreement, ‘the UE monitors the most recent sensing occasion for a given reservation periodicity’, therefore it seems to suggest that K is per periodicity. However, we are open to further discussing this issue

    - Enhancements on SCI reservation mechanism when K is (pre)-configured

In current resource reservation mechanism, SCI can only indicate and reserve resource(s) for the next one periodic TB. For PBPS RA scheme, sensing results are limited. If PBPS for most recent one occasion failed, collision avoidance may not be achieved. Therefore, enhancements for periodic resource reservation can be considered when additional K value(s) are (pre)-configured, i.e. when UE can monitored additional sensing occasions other than most recent one sensing occasion.

Figure 1: Extension of period periodic resource reservation number by (pre-)configured K value

The number of periodic resource reservation could be enlarged with the consideration of k configuration. When additional K value(s) are (pre)-configured, UE can monitor the more prior sensing occasions. In any sensing occasion, if a SCI is decoded with indicating Prsvp_RX≠0, K value can be assumed as the extended reservation number, i.e. periodic reservation resources by a SCI indicating Prsvp_RX≠0 received in slot m would be the associated resources in slot m+Prsvp_RX,…, m+K×Prsvp_RX, as illustrated in Figure 1.

Proposal 1: In PBPS, when (pre-)configuring additional K value(s), for a decoded SCI in slot m with indicating  Prsvp_RX≠0, UE can assume the associated resources in slot  m+Prsvp_RX, … , m+K×Prsvp_RX are reserved.

    - Remaining issue of Preserve

Based on above agreement, if P_RSVP_Tx is not included in the (pre-)configured set of Preserve values, one remaining issue on sensing occasion determination is whether to monitor the occasions corresponding to P_RSVP_Tx mandatory. From our understanding, if the set of Preserve values is (pre-)configured, it means that the related periodicity values would be the most typical periodicity values for services in the resource pool. From periodic-based partial sensing aspects, this is the minimum set of values to be used for determining the sensing occasions. Whether additionally monitoring the related sensing occasions corresponding to P_RSVP_Tx can be up to UE implementation.

Proposal 2: If a set of Preserve values is (pre-)configured for periodic-based partial sensing, but P_RSVP_Tx is not included in the set, whether to monitor the P_RSVP_Tx occasions or not is up to UE implementation.

    - Determination of Y candidate slots for periodic transmission

In RAN1 #104-e meeting, the following agreements were achieved [3]:

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Large intersections in bustling downtown areas, roads around large stadiums, in such scenarios, even if the packets sent by each P-UE are small, may cause congestion due to the large number of concurrent P-UE. Therefore, it is necessary to configure the CBR for P-UE to control channel congestion.

In R16 NR V2X, CBR measured in slot n is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100*2μ slots, according to higher layer parameter timeWindowSize-CBR. For R17 P-UE, the definition of CBR in R16 maybe need to be updated to adapt to the partial sensing mechanism, because partial sensing may not be able to provide enough slots for the CBR measurement window.

Higher layer will configure a two-dimensional table which is configured transmission parameters mapped by the CBR of the resource pool and the priority of transmission. Eight priorities of transmission are defined, and CBR measurements can be mapped to 16 CBR quantization intervals. When UE sends the packets, according to the priority of its own transmission and the measured CBR value, it chooses the corresponding parameters from the two-dimensional table, and achieves the purpose of congestion control.

Part of the configuration of the transmission parameters of congestion control is the control of the physical transmission parameters, including maximum transmit power, range on number of retransmissions per TB, range of PSSCH sub-channel number and range of MCS, as shown in Figure 2. The other parameter, CR_limit, is used to determine whether the current transfer resource can be sent. If the CR_limit is exceeded the threshold, the current transfer needs to be dropped.

Figure 2: Principle of Congestion Control Mechanism

In partial sensing of LTE-V2X, the minimum number of candidate subframes Ymin  is (pre-)configured per resource pool according to the parameter minnumCandidateSF-R14. The exact value of Y is selected by UE itself with the restriction of minimum number of Ymin.

In case of congested scenario, the configured minimum number of candidate slots Ymin could be adjusted based on the conditions of channel congestion or priority, etc. As illustrated in Figure 3, when the system channel congestion is below the pre-configured threshold, a smaller value of Ymin can be configured, which can reduce the partial sensing duration and power consumption of UE; when the system channel congestion is above the pre-configured threshold, a larger value of Ymin can be configured, which can improve the reliability of transmission with the cost of performing more sensing.

Figure 3: Configurable minimum candidate slots based on channel congestion

Proposal 3: With the consideration of congestion control of UE with battery constraint, the minimum candidate slots for partial sensing could be configured based on CBR value.

If a resource pool (pre-)configured with at least partial sensing, and UE performs periodic-based partial sensing, CBR in slot n can be defined as the proportion of the number of sub-channels whose SL-RSSI exceeds the (pre-)configuration threshold in the total number of sub-channels in the resource pool with a CBR measurement window. The CBR measurement window could be M partial sensing occasions or periodic sensing occasions before slot n.

The corresponding bits of the M partial sensing occasions or periodic sensing occasions are configured by bitmap as that in the high-level configuration parameter gapCandidateSensing for partial sensing or periodic sensing.

Figure 4: Configurable minimum candidate slots based on channel congestion

Proposal 4: If UE performs periodic-based partial sensing, CBR in slot n can be measured by UE in M periodic partial sensing occasions before slot n, M periodic partial sensing occasions could be a subset of the configured partial sensing occasions.

Remaining issues of contiguous partial sensing(CPS) RA scheme

In RAN1 #106-e meeting, CPS RA scheme and details in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled are discussed. The following agreement was achieved.

Agreement When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n, - The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - On the sensing window [n+TA, n+TB] for CPS, - Details of TA and TB values based on the agreements from previous RAN1 meetings - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability - FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability - Whether the set SA is confined within a set of Y candidate slots within the RSW - UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4 - Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately. - FFS: Details on T1

In this section, remaining issues in CPS only RA scheme are discussed, including the following aspects:

- Determination of CPS window and definition of minimum CPS window size

- Determination of T1

- Restrictions of resource selection window

- Evaluations

    - Determination of CPS window and definition of minimum CPS window size

In RAN1 #106-e meeting, there is one remaining issue for further study as follows:

- FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size.

Firstly, it is necessary to define a minimum slot number of CPS window, to ensure sufficient sensing results before resource selection.

Observation 1: A minimum slot number of CPS window should be defined to ensure sufficient sensing results before resource selection.

Secondly, considering how to define a minimum CPS window size, it should be clarified that there is no definition of sensing window size in current specification. In TS 38.214 [13], sensing window is defined by a range of slots. Therefore, the minimum CPS window size can be defined as a minimum slot number of CPS window [n+ TA, n+ TB], i.e. slot number of (TB- TA+1) within CPS window should not smaller than the minimum slot number, except for TB=TA=0. It can be (pre-)configured, considering at least the requirements of priority and latency. Due to the limited range of resource reservation indication in SCI, contiguous partial sensing before resource selection should not exceed 31 logical slots. The minimum slot number within CPS window can be indicated by a higher layer parameter.

Proposal 5: The minimum slot number of CPS window (not larger than 31 logical slots) should be defined and (pre-)configured considering at least the requirements of priority and latency.

    - Determination of T1

In CPS only scheme, T1 depends on the actual value of TB. As agreed in the previous meeting, TA and TB can be negative, zero and positive. It can be classified to two cases.

- TB is negative or 0

- TB is positive

- TB is negative or 0

Regarding that multiple SL processes can be maintained simultaneously by a UE, when a packet arrives, DRX active time or contiguous partial sensing for another SL process may be ongoing. Available sensing results can be used for resource selection. If available sensing results can fulfill requirements of resource selection for the arrived packet, resource selection can be performed immediately, i.e. TB can be negative or 0. It is unnecessary to restrict UE not perform resource selection within [n+ TB, n+ TB + Tproc,0], if UE has stronger processing capability. Therefore, the restrictions of T1 is 0≤T1≤Tproc,0+Tproc,1.

Observation 2: For TB being negative or 0, restrictions of T1 is 0≤T1≤Tproc,0+Tproc,1

- TB is positive

On the other side, if there is no ongoing sensing or reception when a packet arrives, CPS can be triggered after packet arriving. In this case TB is positive and n+TB can be the reference time to determine resource selection window. Restrictions of T1 is TB≤T1≤TB+Tproc,0+Tproc,1

Observation 3: For TB being positive, restrictions of T1 is TB≤T1≤TB+Tproc,0+Tproc,1.

Proposal 6: When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n, restrictions of T1 should be defined as follows:

0≤T1≤Tproc,0+Tproc,1,             TB≤0TB≤T1≤TB+Tproc,0+Tproc,1, TB>0

    - Restrictions of resource selection window (RSW)

Considering resource selection window (RSW) for CPS only RA scheme, a new restriction should be introduced for ending time of RSW. As illustrated in Figure 5, because resource reservation in SCI should be indicated within 32 logical slots, if RSW contains more than 32 logical slots, part of candidate slots would have no sensing results for indicating reserved resources. Therefore, only candidate resources in the earlier 32 logical slots would be excluded but the later ones cannot be excluded. Then transmission resource would be likely to be selected from the non-sensed candidate resource, i.e. these resources would be nearly randomly selected.

Figure 5: Introducing a new restriction for ending of RSW

Proposal 7: In order to avoid selecting no sensed resources, candidate slot number in RSW should not exceed 32 logical slots.

Considering re-evaluation and pre-emption, other RSW restrictions should be considered. For power saving UE, in order to achieve power saving gain, there should be some enhancements for re-evaluation and pre-emption. In current specification, the end of checking window is re-evaluation/pre-emption checking time + T2, which should not exceed remaining PDB. For power saving UE, if there is no restriction for checking window, UE may perform re-evaluation/pre-emption until the PDB, it may lead to extra power consumption. In order to reduce power consumption due to extensive re-evaluation/pre-emption checking until PDB, it is necessary to restrict the time duration for re-evaluation/pre-emption checking. One is to restrict the latest checking timing for re-evaluation/pre-emption. Another is to restrict the ending time of RSW which can implicitly determine the latest checking timing for re-evaluation/pre-emption, e.g. not exceed the ending time of initial RSW. Both of them can archive power saving gain. However, the former one may be problematic for transmitting in the resources without re-evaluation/pre-emption checking after the latest checking timing, the latter one can guarantee that the resources used for transmission are checked by re-evaluation/pre-emption.

Proposal 8: For CPS only RA scheme, in order to achieve power saving gain, when performing re-evaluation/pre-emption after contiguous partial sensing based resource selection, the end of RSW due to re-evaluation/pre-emption checking should not exceed the ending time of initial RSW.

Another issue is when both periodic-based partial sensing and contiguous partial sensing are configured, whether the UE has to use one single resource selection window, or the UE can have two separate resource selection windows. In reality there are difference scenarios. The periodic partial sensing and contiguous partial sensing may target two difference traffics, or they can target one single traffic. Therefore, flexibility of resource selection window configuration is preferred.

Proposal 9:  When a resource (re)selection procedure is in a mode 2 Tx pool with reservation for another TB enabled, if UE is configured with both periodic-based and contiguous partial sensing for the resource (re)selection procedure, the sensing results of the two schemes corresponding to different traffic should be applied to the different resource selection windows.

    - Evaluations

For resource selection of aperiodic reservation transmission in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, four candidate resource allocation schemes are evaluated as follows:

- Full sensing based resource selection

- Contiguous partial sensing based resource selection with re-evaluation

  - Candidate slots of (T2 – T1 + 1) are 60 logical slots and re-evaluation checking window is not restricted

  - Candidate slots of (T2 – T1 + 1) are 32 logical slots and re-evaluation checking window does not exceed ending time of initial RSW

- Random resource selection with re-evaluation and/or pre-emption

- Random resource selection without any sensing

Simulation is performed to evaluate PRR performance and power consumption. For contiguous partial sensing based resource selection, both solutions without enhancement and with enhancement for resource selection window and re-evaluation checking window are evaluated. Simulation parameters are illustrated in Annex A. Simulation results for PRR performance and for power consumption are illustrated in Figure 6 and Figure 7 respectively.

Figure 6: PRR performances of different resource allocation schemes for aperiodic traffic

Figure 7: Power consumption CDF of different resource allocation schemes for aperiodic traffic

Table 1: Power consumption of different resource allocation schemes for aperiodic traffic

| Resource allocation schemes | Power consumption @ 99% CDF |
| --- | --- |
| Full sensing | 584700 |
| Contiguous partial sensing with 60 candidate slots | 102400 |
| Contiguous partial sensing with 32 candidate slots and re-evaluation not exceeding ending time of initial RSW | 72570 |
| Random resource selection with re-evaluation and/or pre-emption | 60830 |
| Random resource selection without any sensing | 37550 |

Based on the simulation results, it can be observed as follows:

- Full sensing based RA scheme has the best PRR performance

- RA scheme of random resource selection without sensing has the worst PRR performance

  - Comparing random selection without sensing:

    - No obvious PRR performance gain can be observed for random selection with re-evaluation but power consumption is increased.

- Contiguous partial sensing based RA can obtain PRR performance gain

  - Comparing contiguous partial sensing based RA without enhancement:

    - PRR performance gain can be observed for enhanced scheme with 32 candidate slots and re-evaluation not exceeding ending time of initial RSW

    - PRR performance of enhanced scheme is similar with that of full sensing based RA.

    - Power saving gain of enhanced scheme can be observed.

- Significant power saving gain can be observed of all the four power saving RA schemes.

Based on simulation results and analysis, there is no need to support random selection with re-evaluation. Comparing with random selection without any sensing, there is no gain but extra power consumption. Enhanced contiguous partial sensing RA can be supported because it can obtain the similar PRR performance with full sensing based RA and achieve almost 30% power saving gain with non-enhanced contiguous partial sensing based RA.

Proposal 10: For resource selection of aperiodic reservation transmission in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, support the following two resource allocation schemes for power saving:

- Random resource selection without any sensing.

- Contiguous partial sensing based RA scheme with candidate slots not exceeding 32 logical slots and re-evaluation/pre-emption checking window not later than ending time of initial RSW.

Proposal 11: Random selection with re-evaluation is not supported.

Remaining issues of PBPS and CPS combined RA scheme

For PBPS and CPS combined RA scheme, in RAN1 #106-e meeting, two cases are discussed and agreements are achieved as follows:

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA) - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enable.

The agreement is regarding under what condition the UE has to perform both performs periodic-based and contiguous partial sensing schemes when a resource (re)selection procedure triggered by periodic transmission. Note performing both will certain incur more power consumption which contradicts with the purpose the objective of the WID. In fact, there are many instances when the UE is configured, or enabled to perform PBPS and CPS, the UE should still be able to perform only PBPS if the sensing result is deemed enough. It is up to UE to decide in this case, if PBPS+CPS are both performed, or only PBPS is performed and CPS is skipped.

Note the agreement for CPS is only about required conditions, and is not a necessary condition, as it clearly says that “Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met”. This means even when all the pre-conditions (conditions described in the sub-bullet of the agreement) are met, the UE could still decide if it will perform CPS, based on other criterions.

If an aperiodic traffic is triggered when the PBPS for the previous periodic traffic is in progress, UE needs to select resources for the aperiodic service. By introducing a new Y candidate slots for the unexpected aperiodic traffic, the range of resources to be selected for the aperiodic traffic is increased and sufficient resources are provided. In addition, there could be the case where CPS is already going on for another traffic that the UE anticipates; in that case, there should be two separate resource selection operations. Whether the two resource selection operations and the new determined Y candidate slots are still in one RSW or not calls for further discussion. Therefore, while PBPS is in progress, the triggered aperiodic transmission needs a new determined Y candidate slots, and monitors corresponding CPS sensing duration between slot n and the first slot of the new Y candidate slots subject to processing constraints. How to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure   can be based on the principle of Rel-16 scheme

Observation 4: Whether the two resource selection operations and the new determined Y candidate slots are still in one RSW calls for further discussion.

Proposal 12: Y candidate slots for CPS can be separately configured from existing candidate slot for PBPS.

Random resource selection

The following agreement was made in RAN1#104-e [3] related to random resource selection:

Agreements: - Random resource selection is applicable to both periodic and aperiodic transmissions - FFS conditions for random resource selection

The remaining issue is how to define the triggering/operating conditions for random selection scheme. There are several aspects should be considered as follows:

- Conditions for random selection

- Periodically collision due to periodic transmission with random resource selection

    - Conditions for random resource selection

For a UE, whether to use random selection for resource selection can firstly consider the hardware capability of the UE, when the hardware capability of the UE does not support PSCCH reception operation, the UE should use random resource selection. For the UE with the capability of PSCCH reception or resource sensing, the power saving requirement can be considered as a condition for random resource selection. When the UE has a strict requirement on power saving, or the UE’s power is very low, or the UE enters into power saving mode for other reasons, random resource selection can be allowed to choose the sidelink resources. As for the UE without strict requirement on power saving, the channel congestion can be another factor, for example, when the value of CBR is low, the UE can be allowed to do random resource selection to reduce the latency.  Finally, whether or not the UE can use random selection also depends on the resource pool configuration.

Another potential solution is to use (pre-)configured minimum contiguous sensing duration which is discussed in section 2.2.3. If UE determines the minimum contiguous sensing duration is 0 based on CBR, it means random resource selection scheme can be allowed. This condition can be independently and co-exist with aforementioned conditions.

Proposal 13: The UE capability, requirement on power saving, resource pool configuration, congestion condition (as indicated by CBR etc.) and (pre-)configured minimum contiguous partial sensing duration can be the criteria for random resource selection.

It is important to consider the criteria for random resource selection with other considerations such as coexistence of different power saving resource selection schemes. As a result, the decision for conditions of random resource selection should be further investigated.

Proposal 14: Conditions of random selection scheme should be further investigated after schemes of periodic-based partial sensing and contiguous partial sensing are determined, considering the following aspects:

- Coexistence of different resource selection schemes.

- Congestion level

- (Pre-)configured minimum contiguous partial sensing duration

- When determined value of pre-configured minimum contiguous partial sensing duration is 0, UE can be allowed to select random resource selection scheme.

    - Periodically collision due to periodic transmission with random resource selection

The following agreement was made in RAN1#105-e [2] related to random resource selection:

Agreement: - For random resource selection, - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation. - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any) - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected. - FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions). - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

The agreements above from RAN1#104-e and RAN1#105-e support random resource selection applying to periodic transmission, one possible scenario, as shown in the Figure 8, is that a UE may collide with other UE periodically due to the random resource selection without sensing. If the UE do not have the capability of PSCCH or PSFCH reception, it can do nothing about the periodically collision. For the UE with reception or sensing capability, for instance, the type B UE which is able to receive PSFCH, could recognize whether the periodically collision happened according to the feedback information, and reduce the collision occasions by reselecting the resources.

Figure 8: Periodically collision due to periodic transmission with random resource selection

Proposal 15: The UE with reception capability of PSFCH can reselect the resource according to the HARQ feedback information to reduce periodically collision occasions.

Another way to avoid collisions of reserved resources by random selection UE without capability of sensing is to increase the priority of its reserved resources, which prevents its reservation resources from being pre-empted by other UE reservation resources. It is also necessary to avoid reserved resource by no sensing ability UE being pre-empted by random selection reserved resource of UE with sensing ability. A UE with sensing capability should reduce the number of times by utilizing random selection for resource reservation and increase the number of times it uses sensing to make resource reservations. Therefore, a UE with sensing ability should be configured with a lower priority for the resources reserved by random selection. For UEs with different reception capabilities, they are configured with different priorities for the reserved resources by random selection.

Proposal 16: For UEs with different reception capabilities, they are configured with different priorities for the reserved resources by random selection.

    - Priority threshold or levels (pre-)configured for the resource pool

The following agreements were achieved in RAN1#106-e, which are related to the aspects of random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection:

Agreements: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

When random resource selection and full/partial sensing are configured in the resource pool, for the resources reserved by UE through random resource selection, if there is a collision with the resources reserved by other resource allocation schemes, the UE without reception ability cannot avoid such resource collision by sensing or re-evaluation. By configuring a priority threshold for the resource pool, UE can reserve resources in the resource pool only when the priority of resources reserved by random selection is lower than the threshold, that is, the solution in Option 1. In this way, UE with different reception capabilities can avoid resource collisions between resources reserved by random selection via being configured different priority. When the UE with sensing capability randomly selects resources configured with a priority higher than the threshold, and the UE without sensing and re-evaluation capability randomly selects resources with a priority lower than the threshold, as described in section 2.4.2, by avoiding the coexistence of randomly selected resources reserved by UEs with different reception capabilities in the resource pool, thus reducing the probability of resource collision. However, resource pool partitioning may lead to resource pool fragmentation and lower resource utilization, which is not an effective solution. Option 1 implicitly configures the priorities of resources reserved in different resource allocation schemes, compared with Option 2, which saves signaling overhead at the physical layer. Option 7 has the same problems as Option 2. Therefore, for random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed.

Proposal 17: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed.

Details of partial sensing for re-evaluation/pre-emption

In RAN1 #106-e meeting, generic scheme of re-evaluation/pre-emption are discussed and agreements are achieved as follows:

Agreement For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing, - Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc - Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable. - FFS: If additional enhancements are needed for enabling/disabling - The triggering of re-evaluation and pre-emption checking is as in R16.

Details and potential enhancements of partial sensing for re-evaluation and pre-emption checking need to be further studied. In this section, issues are discussed as follows:

- Triggering conditions of PBPS applied for re-evaluation/pre-emption

- Restrictions of CPS applied for re-evaluation and pre-emption checking

    - Triggering conditions of PBPS applied for re-evaluation/pre-emption

Re-evaluation and pre-emption operation are introduced in NR-V2X but not specified in LTE-V2X. So PBPS applied for re-evaluation/pre-emption needs to be considered in NR Sidelink power saving mechanism. Triggering conditions of PBPS applied for re-evaluation/pre-emption should be discussed, considering re-evaluation and pre-emption for the duration that a TB arrives but resource (re)selection is not triggered.

The difference between re-evaluation and pre-emption is that whether the upcoming collided resource(s) has been signaled by SCI or not. With this difference in mind, for a periodic transmission resource, if the initial transmission(s) for first TB is not triggered for resource reselection due to re-evaluation, the re-evaluation will not perform for the following periodic transmissions since the reserved resource(s) has been signaled in current period or previous periods. As a result, there is no need to perform partial sensing for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered for the purpose of re-evaluation.

Observation 5: For the purpose of re-evaluation, periodic-based partial sensing is not performed for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

However, pre-emption operation is performed for selected and signaled resource(s) if it is collided with resource with higher priority level, it requires contiguous sensing to determine whether there is a (re-)selection trigger due to pre-emption. This design principle in full sensing mechanism should be kept consistently in partial sensing mechanism. That means, if pre-emption is enabled in a resource pool, for each arriving packet of periodic traffic, periodic-based partial sensing should be always performed, as illustrated in Figure 9(a). While pre-emption is disabled, there is no need to perform periodic-based partial sensing, as illustrated in Figure 9(b).

(a) Periodic-based partial sensing when pre-emption is enabled

(b) Periodic-based partial sensing when pre-emption is disabled

Figure 9: Periodic-based partial sensing considering pre-emption

Observation 6: For the purpose of pre-emption, whether periodic-based partial sensing should be performed or not depends on pre-emption enable or disable.

Proposal 18: For periodic traffic transmissions, if resource (re)selection is not triggered, periodic-based partial sensing should be performed based on the following rules considering pre-emption:

- When sl-PreemptionEnable-r16 is provided, periodic-based partial sensing should be performed continuously.

- When sl-PreemptionEnable-r16 is not provided, periodic-based partial sensing should not be performed.

In RAN1#106-e, it was agreed that PBPS and CPS schemes support re-evaluation and pre-emption checking, but the conditions under which schemes support re-evaluation and pre-emption checking were not discussed, which is a question worth considering. Because PBPS and CPS have different sensing occasion configuration, PBPS and CPS schemes should support re-evaluation and pre-emption checking under different conditions. For re-evaluation and pre-emption, the difference between the two partial sensing schemes is that PBPS is a discontinuous sensing operation, while CPS is continuous. In the same period of time, the duration of PBPS and CPS sensing occasion is different. PBPS can achieve the effect of energy saving due to discontinuous sensing processing, whereas the reliability of PBPS sensing results will be reduced. Continuous sensing operation of CPS will cost more energy consumption, with the advantage that the reliability of sensing results will be improved. Considering the difference of the two partial sensing modes, further study is needed for the criterion of choosing which type of partial sensing for re-evaluation and pre-emption. For example, PBPS is more appropriate for the case of periodic traffic, while CPS is more appropriate for aperiodic traffic. Other criterion could also be consideration in choosing the type of sensing.

Proposal 19:    Further study is needed for the criterion of choosing which type of partial sensing for re-evaluation and pre-emption.

    - Restrictions of CPS applied for re-evaluation and pre-emption checking

Besides PBPS, CPS should also be performed for re-evaluation/pre-emption checking. In Release 16 NR-V2X sensing based RA scheme, re-evaluation/pre-emption checking are performed at least when resource(s) are pre-selected. Same principle can be reused. In power saving RA scheme, CPS applied for re-evaluation/pre-emption checking can be triggered when resource selection is performed. From L1 aspect, the triggering time is candidate resource set reporting time.

Proposal 20: Starting time of CPS applied for re-evaluation and pre-emption checking is resource selection time for MAC layer and candidate resource set reporting time for physical layer.

As discussed in section 2.2.3, for CPS only RA scheme, in order to achieve power saving gain, when performing re-evaluation/pre-emption after contiguous partial sensing based resource selection, ending time of checking RSW window should be fixed to ending time of initial RSW. It is not limited in CPS only RA scheme but can be applied for all cases CPS applied for re-evaluation/pre-emption checking. In order to achieve power saving gain, same restrictions for RSW of re-evaluation/pre-emption checking can be applied.

Proposal 21: In order to achieve power saving gain, when performing re-evaluation/pre-emption checking, the end of checking window should be fixed to end of initial RSW.

Another issue is CPS ending time. When re-evaluation/pre-emption completed, CPS should be switched off. Ending condition of re-evaluation/pre-emption is same as the rule of re-evaluation/pre-emption in R16.

Proposal 22: The ending time of CPS applied for re-evaluation/pre-emption should be same as the ending time of re-evaluation/pre-emption checking (i.e. m-T3) which follows the rules of re-evaluation/pre-emption in R16.

Coexistence of full sensing, partial sensing and random selection RA

The following agreements were achieved in RAN1#103-e [4], which is related to configuration of Tx resource pool for sidelink mode 2:

Agreements: - In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

In LTE-V2X, resource selection mechanism of P-UE is resource pool specific configured by IE SL-P2X-ResourceSelectionConfig-r14. This IE can configure at least one of the resource selection mechanisms from partial sensing and random selection. From specification perspective, any combinations of configuration of full sensing, partial sensing and random selection for a resource pool have been supported in Rel-14 LTE-V2X.

In NR Sidelink enhancement, coexistence of full sensing, periodic-based partial sensing, contiguous partial sensing and random resource selection should be considered.

In our company's contribution R1-2100351[5] in RAN1 #104-e meeting, coexistence of full sensing, periodic-based partial sensing and random resource selection were discussed and evaluated. It can be observed in R1-2100351[5] that there is no significant performance loss for coexistence of periodic-based partial sensing and full sensing and coexistence of random resource selection and full sensing. However, because the details of periodic-based partial sensing and the schemes of contiguous partial sensing are not concluded, coexistence of periodic-based partial sensing, contiguous partial sensing and random resource selection should be further studied.

Proposal 23: Coexistence of periodic-based partial sensing, contiguous partial sensing and random resource selection should be further studied after schemes of periodic-based partial sensing and contiguous partial sensing are determined.

Discussion on the impact of SL DRX

Discontinuous reception (DRX) is a common power saving mechanism in Uu interface. When DRX mechanism is enabled, UE monitors PDCCH during the DRX active time and switches to sleep mode during the DRX inactive time. Considering the power saving requirements of some sidelink UEs, such as pedestrians, e-bikes, e-motorcycles and other devices with battery constraint, the WID [6] for R17 sidelink enhancement decided to introduce SL DRX.

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

Although DRX is mainly discussed by RAN2, RAN1 still needs to consider the potential impact of the introduction of SL DRX on sensing and resource selection, which was captured in the revised WID [7].

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

According to the revised WID, the following issues regarding the impact of SL DRX are discussed in this section:

- The impact of SL DRX on partial sensing

- The impact of SL DRX on resource selection

The impact of SL DRX on partial sensing

In RAN1 #106-e meeting, the following agreement was achieved regarding the relationship between sensing and SL DRX [1].

Agreement A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation - FFS: Other details

For the first FFS, “up to UE implementation” may lead to UE still not performing sensing during its SL DRX inactive time. The following schemes may be adopted by the power saving UE which is enabled to perform partial sensing as well as SL DRX.

Scheme 1: Partial sensing is performed independently of SL DRX.

Scheme 2: Partial sensing is performed within SL DRX active time only.

- Figure 10 and Figure 11 provides the system-level simulation results of PRR and power consumption of the above schemes. The calculations of PRR and power consumption are according to TR 37.885 [10] and the agreements of RAN1#102-e [11]. The detailed system-level simulation assumptions are summarized in Annex B.

Figure 10: The comparison of PRR performance of P2V traffic between Scheme 1 and Scheme 2

Figure 11: The comparison of power consumption of power saving UE among the three schemes

According to Figure 10, it can be observed that restricting partial sensing occasion to be within SL DRX active time will significantly degrade the PRR performance of P2V traffic. The reason is that only performing sensing within the SL DRX active time will result in insufficient sensing results, thereby increasing the possibility of resource collisions. Compared with scheme 1 where partial sensing and SL DRX are performed independently, the PRR performance loss of scheme 2 can reach 3%.

On the other hand, due to the fact that UE may also perform SL reception of PSCCH and RSRP measurement for sensing during DRX inactive time under scheme 1, the power consumption of scheme 1 is slightly higher than scheme 2, as shown in Figure 11. However, compared with full sensing, the power saving effects achieved by scheme 1 and scheme 2 are both considerable.

Observation 7: Compared with performing partial sensing independently of SL DRX, performing partial sensing only within SL DRX active time will decrease the PRR performance of P2V traffic, while the power consumption of power saving UE will not be significantly reduced.

Based on the above evaluation and observation, it is proposed that the UE behavior of SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time should be subject to the specification.

Proposal 24: The UE behaviours of SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time should be subject to the specification.

In RAN1 #106-e meeting, the following agreement was reached on the periodic-based and contiguous partial sensing [1]:

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

According to the above agreement, there are still remaining issues regarding the relationship between SL DRX and partial sensing. For P2P communication with power saving requirements on both transmitting and receiving UE, Tx UE can perform partial sensing-based resource selection while Rx UE can enable SL-DRX, in order to reduce power consumption. In this case, it is necessary to consider the relationship between the candidate resources/sensing occasion selected by Tx UE and the DRX active time of Rx UE. If Tx UE does not consider the DRX configuration of Rx UE when determining Y candidate slots and performing partial sensing-based resource selection, the transmission resources selected by the Tx UE may correspond to the DRX inactive time of the Rx UE. Due to the fact that the Rx UE cannot perform SCI reception during the DRX inactive time, packet loss inevitably occurs, which leads to the decrease of the reliability of P2P communication.

Therefore, when Tx UE performs partial sensing-based resource selection, DRX configuration of Rx UE needs to be considered by Tx UE to ensure that the selected resources for the initial transmission and retransmission(s) can be within the DRX active time of Rx UE, so that the Rx UE can receive the corresponding packets.

Proposal 25: For P2P communication, the transmission resources selected by Tx UE need to be within the DRX active time of Rx UE.

In RAN2 #113b-e meeting, the following agreements were achieved regarding the details of SL DRX timer [8]:

Agreements on details of timer

19:	Working assumption: SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource. FFS whether explicitly configured SL HARQ RTT timer may be still required. If big problem is identified next meeting, we can revisit it.

20:	The value(s) of the SL HARQ RTT Timer, when explicitly configured and not determined via SCI (if agreed to do so), is determined by UE or NW implementation.

21:	For unicast, sidelink retransmission timer can be supported for at least some cases of HARQ disabled transmissions. FFS whether HARQ RTT is supported or not.

22:	For transmissions with HARQ feedback, the RX UE starts the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission.

23:	If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

24:	For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the TX UE) the RX UE uses a configured retransmission timer.

25:	Retransmission timer can be started upon expiry of the HARQ RTT timer.

26:	The value(s) of the SL retransmission timer can be determined by UE or NW implementation.

27:	The SL active time of the RX UE includes the time in which any of its applicable sl-drx-OnDuration(s), sl-DRXInactivityTimer(s), or sl-drx-RetransmissionTimer(s) are running.

From RAN2’s above agreements, it can be observed that the Rx UE will start the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission/resource for a HARQ enabled transmission, regardless of whether the transmission is accompanied by HARQ feedback. And when the SL HARQ RTT timer is expired, the Rx UE can start a configured retransmission timer which is determined by UE or NW implementation.

From RAN1’s perspective, similar to the retransmission timer, a configured SL HARQ RTT timer is still required for cases where there is some uncertainty in the timing of a retransmission. Since the Rx UE is in DRX inactive time during the SL HARQ RTT timer running duration, SCI decoding cannot be performed during this period. If SL HARQ RTT timer is entirely derived from the SCI indication, some potential problems may occur for the pre-emption enabled scenarios. An example is shown in Figure 12. The Tx UE’s SCI indicates a later resource, but the Tx UE may reselect a resource before the indicated resource due to pre-emption. As the reselected resource is in the DRX inactive time (i.e. SL HARQ RTT timer running duration) of the Rx UE, this packet will be missed. Thus, for cases where the retransmission timing is uncertain, such as pre-emption enabled scenarios, configured SL HARQ RTT timer should be considered as a baseline.

Figure 12: Packet loss due to the SCI indication derived SL HARQ RTT timer

Observation 8: For a HARQ enabled transmission, the Rx UE will start the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission/resource, regardless of whether the transmission is accompanied by HARQ feedback.

Observation 9: The Rx UE can start a configured retransmission timer when the SL HARQ RTT timer is expired.

Proposal 26: For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the Tx UE), the Rx UE uses a configured SL HARQ RTT timer.

Based on the above observations, it can be concluded that once the transmission resource and the resource pool/PSFCH configuration are determined, the running duration of the corresponding retransmission timer can be determined. And according to RAN2’s agreement, drx-RetransmissionTimer(s)’s running duration is included in the SL DRX active time of the Rx UE. Hence, the Tx UE can take the predicable retransmission timer running duration into consideration for determining the allowable transmission time in order to ensure that Tx UE selected resources can be within DRX active time.

Figure 13: Determining the retransmission timer running duration based on the previous transmission resource

As shown in Figure 13, Tx UE sets the Y candidate resources to partially overlap with the DRX on-duration of Rx UE, ensuring that there are candidate resources within DRX active time. Tx UE can select the initial transmission resource to be within the DRX on-duration when resource selection procedure is triggered. Then, according to the resource pool/ PSFCH configuration and the DRX configuration of the Rx UE, the first running duration of drx-RetransmissionTimer can be determined. The Tx UE can take this running duration into consideration when selecting the retransmission resource(s). After selecting the first retransmission resource, the second running duration of drx-RetransmissionTimer that should be considered when selecting the remaining retransmission resource is also determined. Since this operation is related to RAN2’s specific, RAN1 should send an LS to RAN2 to inform RAN1’s consideration of sensing and resource selection.

Proposal 27: The Tx UE can take the predictable drx-RetransmissionTimer running duration of the Rx UE into consideration for determining the allowable transmission time, in order to ensure that the transmission resources selected by the Tx UE can be within Rx UE’s DRX active time.

The impact of SL DRX on resource selection

Unlike the NR Uu interface, Sidelink UE performs autonomous resource selection according to the channel sensing results. If the resources selected by the Tx UE fail to align with the DRX on-duration/active time of the Rx UE, as shown in Figure 14, packet loss may occur. Thus, it is necessary to consider the impact of DRX parameters on Tx UE's resource selection when introducing the DRX mechanism into NR sidelink enhancement.

Figure 14: Misalignment between resource selection window and DRX on-duration

In RAN2 #115-e meeting, the following agreements have already been achieved regarding the resource selection at the TX UE. An LS [9] was send to RAN1 to ask RAN1 taking the above agreements into account and inform RAN2 whether/how RAN1 intends to reflect the restriction in the following RAN2 agreement “When data is available for transmission to one or more RX UE in DRX, TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) determined by the timers maintained at the TX UE.  Details are FFS. FFS whether RAN1 or RAN2 implement this restriction.”

Agreements:

When data is available for transmission to one or more RX UE in DRX, TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) determined by the timers maintained at the TX UE.  Details are FFS. FFS whether RAN1 or RAN2 implement this restriction.

For unicast, the TX UE selects the resources for the initial transmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE. How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact.

For unicast, the TX UE can select the resources for the retransmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE.  How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact.

For broadcast, the TX UE can select the resources for the initial transmission associated with any active time supported by broadcast (i.e. on duration) at the RX UE.

For broadcast, the TX UE can select the resources for the retransmission associated with any active time supported by broadcast (i.e. on duration) at the RX UE.

According to RAN2’s agreements, “TX UE selects the resources taking into account the active time (current or future) of the RX UE(s)” means TX UE should reserve the resources which are within the active time of the RX UE, so that the transmission(s) can be successfully monitored by the RX UE. The basis for implementing the above restriction is to ensure that the resource selection window of the TX UE includes at least part of the DRX active time of the RX UE. Otherwise, TX UE may not find any candidate resource corresponding to the DRX active time of the RX UE(s), while the RX UE cannot receive expected transmission(s) during its DRX active time. In this case, TX UE can only transmit the packet in the DRX inactive time of the RX UE or drop the transmission, both of which will lead to reliability decrease.

Observation 10: If the resource selection window of the TX UE does not include any candidate resource corresponding to the DRX active time of the RX UE(s), transmission reliability cannot be guaranteed.

In order to avoid this situation, the resource selection window can be adjusted to at least partially overlap with the DRX active time of the Rx UE(s), as shown in Figure 15. With this adjustment, TX UE can always find appropriate candidate resources in the resource selection window, so that the transmissions on the reserved resources can be monitored by the RX UE(s). Considering that RAN1 is responsible for determining the resource selection window, the above-mentioned restriction (i.e. TX UE selects the resources taking into account the active time of the RX UE) can also be implemented by RAN1, by means of adjusting the resource selection window to at least partially overlap with the DRX active time of the Rx UE(s).

Figure 15: Adjust resource selection window to at least partially overlap with the DRX active time of the Rx UE

Proposal 28: RAN1 can be responsible for implementing the restriction that TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) by adjusting the resource selection window to at least partially overlap with the DRX active time of the Rx UE(s).

After the resource selection window is determined, either RAN1 or RAN2 can be responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s). Suppose that this operation is implemented by RAN1, TX UE can maintain two candidate resource sets which correspond to the candidate resources within the DRX active time of the Rx UE(s) (e.g. SB) and all candidate resources within the resource selection window (i.e. SA in TS 38.214 [13]) respectively, as shown in Figure 16. Both of the candidate resource sets can be reported to MAC after the resource exclusion procedure and MAC can take the two resource sets into consideration when performing resource selection. The initial transmission should be selected in SB, which is within the DRX active time of the RX UE, to ensure that it can be successfully monitored by the RX UE. Since the DRX active time may be extended by a former transmission (e.g. sl-drx-InactivityTimer may be triggered when an initial transmission is successfully decoded), some retransmission resources can also be selected within SA, in order to avoid resource pool congestion.

Figure 16: TX UE performs resource selection in two candidate resource sets

Proposal 29: RAN1 can be responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) by reporting two candidate resource sets, which correspond to the candidate resources within the DRX active time of the Rx UE(s) and all candidate resources (i.e. SA) within the resource selection window respectively.

From the perspective of minimizing the impact on the specification, the operation of determining the candidate resources corresponding to the DRX active time of the RX UE(s) can also be implemented by RAN2. In this case, TX UE maintains only one candidate resource set (i.e. SA in TS 38.214 [13]) and reports SA to MAC after the resource exclusion procedure described in current specification. However, due to the fact that MAC can only know the index of the candidate resources instead of the definite slots which the candidate resources are located in, PHY needs to additionally report the start time of the resource selection window. Based on the information reported by PHY, MAC can be aware of whether the candidate resources correspond to the DRX active time of the RX UE, and distinguish candidate resources belonging to and not belonging to the DRX active time of the RX UE(s). It can be up to RAN2 implementation to perform resource selection by taking into account the active time of the RX UE(s). For example, as shown in Figure 17, MAC can select some reserved resources corresponding to the DRX active time of the RX UE(s) for the initial transmission and some retransmissions. Considering that the DRX active time may be extended, some reservations on the candidate slots that may not belong to DRX active time can also be allowed.

Figure 17: TX UE performs resource selection in one candidate resource set

Proposal 30: RAN2 can be responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) based on the candidate resource set (i.e. SA) and the adjusted start time of the resource selection window reported by PHY.

Since the above two resource selection methods are mutually exclusive, further discussion may be needed for the down-selection. Hence, we have the following proposal.

Proposal 31: Further discussion is required for the down-selection of the following resource selection methods:

- RAN1 is responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) by reporting two candidate resource sets, which correspond to the candidate resources within the DRX active time of the Rx UE(s) and all candidate resources (i.e. SA) within the resource selection window respectively.

- RAN2 is responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) based on the candidate resource set (i.e. SA) and the adjusted start time of the resource selection window reported by PHY.

In the LS from RAN2, the following understanding about the TX UE is also informed to RAN1 based on RAN2 agreements:

- For unicast, the TX UE maintains a set of timers per pair of source Layer-2 ID and destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. For groupcast/broadcast, the TX UE maintains a set of timers per destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. The TX UE uses the timers as part of the criterion for determining the allowable transmission time for each RX UE.

From the above information provided by RAN2, it can be observed that Tx UE is aware of the DRX configuration of the Rx UE in any cast type, and determines the allowable transmission time for each Rx UE based on the DRX configuration.

Observation 11: Tx UE is aware of the DRX configuration of the Rx UE in any cast type, and determines the allowable transmission time for each Rx UE based on the DRX configuration.

In the case that the Tx UE has already got the DRX configuration of Rx UE(s), the following two schemes of selection window configuration may have benefits for the packet transmission. The first scheme is to have the resource selection window fully aligned with DRX on-duration of the Rx UE, that is, all the transmissions of a TB must be performed during the DRX on-duration. With the same DRX configuration, this scheme can maximize the power saving effect of the Rx UE, but it can also cause channel congestion to a certain extent. To decrease the channel congestion, another scheme is to ensure that at least the initial transmission can be made during DRX on-duration. In this case, for the reception of additional retransmission, Rx UE(s) can start sl-drx-InactivityTimer or sl-drx-RetransmissionTimer for the unicast and groupcast scenarios, or wake up at specific slots as indicated by the initial SCI for pre-emption disabled scenarios. But the above operations may also cause additional power consumption.

In the following, the system-level evaluation results of baseline and two schemes of resource selection and DRX are provided:

Baseline: Tx UE performs sensing and resource selection without any DRX-related restrictions, while Rx UE is DRX disabled.

Scheme 1: All the transmissions of a TB, including initial transmission and retransmissions, must be made during DRX on-duration of the Rx UE.

Scheme 2: Only the initial transmission of a TB must be made during DRX on-duration of the Rx UE.

- The examples of the above baseline and two schemes are illustrated in the Figure 18.

- Baseline: DRX disable

- Scheme 1: All the transmissions are made during DRX on-duration

- Scheme 2: Only the initial transmission is made during DRX on-duration

Figure 18: The baseline and two schemes of resource selection and DRX

The following comparative analysis of the above baseline and two schemes is provided with the system-level simulation results in the city broadcast scenarios, where 50% vehicle UEs transmit V2P periodic packets and the other 50% transmit V2V periodic packets as background interference. The calculation of PRR and power consumption are according to TR 37.885 [10] and the agreements of RAN1[11]. The detailed system-level simulation assumptions are summarized in Annex C.

Figure 19: The PRR of power saving UE reception for V2P broadcast services in 60km/h urban scenario

Figure 20: The power consumption of power saving UE reception for V2P broadcast services in 60km/h urban scenario

As shown in Figure 19, better PRR performance can be achieved when Rx UE is DRX disabled and Tx UE performs resource selection without DRX-related restrictions. For scheme 1 and scheme 2, DRX cycle is configured as 100ms, while the DRX on-duration in each DRX cycle is configured as 20ms or 15ms and starts at SFN 0 slot 0, i.e. drx-startoffset and drx-slotoffset are both zero. It is obviously that restricting the resource selection window to be fully aligned with DRX on-duration of the Rx UE will significantly decrease PRR due to channel congestion. Comparing to scheme 1, scheme 2 only restricts the initial transmission of a TB must be made during DRX on-duration, thus alleviating the channel congestion and improving the PRR performance by nearly 5%.

It can be noted that PRR performance of scheme 2 is also better than that of the baseline when the distance between Tx UE and Rx UE is longer than 150m. This phenomenon can be explained as that due to the restriction of scheme 2 that the initial transmission needs to be made during the DRX on-duration; the interference during the DRX off-duration is relaxed compared with that of the baseline where the interference is evenly distributed. For UEs that are near to the Tx UE, resource sensing is effective. The Tx UE can avoid resource collision based on the sensing result. For UEs that are relative far away from the Tx UE, the resource selection mechanism is close to random selection since the RSRP threshold has been raised to a relatively high value. In this case, the performance is directly related to the interference in the resource pool. Therefore, when there is a relatively long distance between the Tx UE and the Rx UE, the performance of scheme 2 will be better than the baseline.

Figure 20 shows the CDF diagram of the power consumption of the abovementioned baseline and two schemes of resource selection and DRX. Compared with the baseline, scheme 1 and scheme 2 which use the SL DRX mechanism achieved a power saving gain of up to 80%. For scheme 1, the power consumption of all PUEs is the same because all PUEs have the same wake-up time, i.e. during DRX on-duration. For scheme 2, since the PUE has additional wake-up time in addition to the DRX on-duration, the power consumption of all PUEs is greater than that of scheme 1 under the same DRX configuration. When the DRX on-duration of scheme 2 is configured shorter than that of scheme 1, such as 15ms, the average power consumption of PUE can be reduced to 3.9, which is significantly lower than that of scheme 1.

As can be seen from the above simulation results, scheme 2 can improve the power saving performance of PUEs by shortening the DRX on-duration, while reducing the loss of PRR performance. Thus, when the Rx UE is DRX enabled, ensuring that at least the initial transmission of a TB can be made during DRX on-duration of the Rx UE can achieve a trade-off between PRR and power consumption.

Observation 12: Comparing with scheme that the resource selection window for all the transmissions are restricted to be fully aligned with DRX on-duration of the Rx UE, the scheme that only initial transmission is transmitted during the DRX on-duration of Rx UE can archive better PRR performance.

Proposal 32: When the Rx UE is DRX enabled, Tx UE should ensure that at least the initial transmission can be made during DRX on-duration of the Rx UE to achieve a trade-off between Packet Reception Ratio (PRR) and power consumption.

Conclusion

In this contribution, issues and enhancements of resource allocation for power saving and SL DRX are discussed. Observations and proposals are given as follows:

Proposal 1: In PBPS, when (pre-)configuring additional K value(s), for a decoded SCI in slot m with indicating  Prsvp_RX≠0, UE can assume the associated resources in slot  m+Prsvp_RX, … , m+K×Prsvp_RX are reserved.

Proposal 2: If a set of Preserve values is (pre-)configured for periodic-based partial sensing, but P_RSVP_Tx is not included in the set, whether to monitor the P_RSVP_Tx occasions or not is up to UE implementation.

Proposal 3: With the consideration of congestion control of UE with battery constraint, the minimum candidate slots for partial sensing could be configured based on CBR value.

Proposal 4: If UE performs periodic-based partial sensing, CBR in slot n can be measured by UE in M periodic partial sensing occasions before slot n, M periodic partial sensing occasions could be a subset of the configured partial sensing occasions.

Observation 1: A minimum slot number of CPS window should be defined to ensure sufficient sensing results before resource selection.

Proposal 5: The minimum slot number of CPS window (not larger than 31 logical slots) should be defined and (pre-)configured considering at least the requirements of priority and latency.

Observation 2: For TB being negative or 0, restrictions of T1 is 0≤T1≤Tproc,0+Tproc,1

Observation 3: For TB being positive, restrictions of T1 is TB≤T1≤TB+Tproc,0+Tproc,1.

Proposal 6: When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n, restrictions of T1 should be defined as follows:

0≤T1≤Tproc,0+Tproc,1,             TB≤0TB≤T1≤TB+Tproc,0+Tproc,1, TB>0

Proposal 7: In order to avoid selecting no sensed resources, candidate slot number in RSW should not exceed 32 logical slots.

Proposal 8: For CPS only RA scheme, in order to achieve power saving gain, when performing re-evaluation/pre-emption after contiguous partial sensing based resource selection, the end of RSW due to re-evaluation/pre-emption checking should not exceed the ending time of initial RSW.

Proposal 9:  When a resource (re)selection procedure is in a mode 2 Tx pool with reservation for another TB enabled, if UE is configured with both periodic-based and contiguous partial sensing for the resource (re)selection procedure, the sensing results of the two schemes corresponding to different traffic should be applied to the different resource selection windows.

Proposal 10: For resource selection of aperiodic reservation transmission in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, support the following two resource allocation schemes for power saving:

- Random resource selection without any sensing.

- Contiguous partial sensing based RA scheme with candidate slots not exceeding 32 logical slots and re-evaluation/pre-emption checking window not later than ending time of initial RSW.

Proposal 11: Random selection with re-evaluation is not supported.

Observation 4: Whether the two resource selection operations and the new determined Y candidate slots are still in one RSW calls for further discussion.

Proposal 12: Y candidate slots for CPS can be separately configured from existing candidate slot for PBPS.

Proposal 13: The UE capability, requirement on power saving, resource pool configuration, congestion condition (as indicated by CBR etc.) and (pre-)configured minimum contiguous partial sensing duration can be the criteria for random resource selection.

Proposal 14: Conditions of random selection scheme should be further investigated after schemes of periodic-based partial sensing and contiguous partial sensing are determined, considering the following aspects:

- Coexistence of different resource selection schemes.

- Congestion level

- (Pre-)configured minimum contiguous partial sensing duration

- When determined value of pre-configured minimum contiguous partial sensing duration is 0, UE can be allowed to select random resource selection scheme.

Proposal 15: The UE with reception capability of PSFCH can reselect the resource according to the HARQ feedback information to reduce periodically collision occasions.

Proposal 16: For UEs with different reception capabilities, they are configured with different priorities for the reserved resources by random selection.

Proposal 17: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed.

Observation 5: For the purpose of re-evaluation, periodic-based partial sensing is not performed for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

Observation 6: For the purpose of pre-emption, whether periodic-based partial sensing should be performed or not depends on pre-emption enable or disable.

Proposal 18: For periodic traffic transmissions, if resource (re)selection is not triggered, periodic-based partial sensing should be performed based on the following rules considering pre-emption:

- When sl-PreemptionEnable-r16 is provided, periodic-based partial sensing should be performed continuously.

- When sl-PreemptionEnable-r16 is not provided, periodic-based partial sensing should not be performed.

Proposal 19:    Further study is needed for the criterion of choosing which type of partial sensing for re-evaluation and pre-emption.

Proposal 20: Starting time of CPS applied for re-evaluation and pre-emption checking is resource selection time for MAC layer and candidate resource set reporting time for physical layer.

Proposal 21: In order to achieve power saving gain, when performing re-evaluation/pre-emption checking, the end of checking window should be fixed to end of initial RSW.

Proposal 22: The ending time of CPS applied for re-evaluation/pre-emption should be same as the ending time of re-evaluation/pre-emption checking (i.e. m-T3) which follows the rules of re-evaluation/pre-emption in R16.

Proposal 23: Coexistence of periodic-based partial sensing, contiguous partial sensing and random resource selection should be further studied after schemes of periodic-based partial sensing and contiguous partial sensing are determined.

Observation 7: Compared with performing partial sensing independently of SL DRX, performing partial sensing only within SL DRX active time will decrease the PRR performance of P2V traffic, while the power consumption of power saving UE will not be significantly reduced.

Proposal 24: The UE behaviours of SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time should be subject to the specification.

Proposal 25: For P2P communication, the transmission resources selected by Tx UE need to be within the DRX active time of Rx UE.

Observation 8: For a HARQ enabled transmission, the Rx UE will start the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission/resource, regardless of whether the transmission is accompanied by HARQ feedback.

Observation 9: The Rx UE can start a configured retransmission timer when the SL HARQ RTT timer is expired.

Proposal 26: For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the Tx UE), the Rx UE uses a configured SL HARQ RTT timer.

Proposal 27: The Tx UE can take the predictable drx-RetransmissionTimer running duration of the Rx UE into consideration for determining the allowable transmission time, in order to ensure that the transmission resources selected by the Tx UE can be within Rx UE’s DRX active time.

Observation 10: If the resource selection window of the TX UE does not include any candidate resource corresponding to the DRX active time of the RX UE(s), transmission reliability cannot be guaranteed.

Proposal 28: RAN1 can be responsible for implementing the restriction that TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) by adjusting the resource selection window to at least partially overlap with the DRX active time of the Rx UE(s).

Proposal 29: RAN1 can be responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) by reporting two candidate resource sets, which correspond to the candidate resources within the DRX active time of the Rx UE(s) and all candidate resources (i.e. SA) within the resource selection window respectively.

Proposal 30: RAN2 can be responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) based on the candidate resource set (i.e. SA) and the adjusted start time of the resource selection window reported by PHY.

Proposal 31: Further discussion is required for the down-selection of the following resource selection methods:

- RAN1 is responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) by reporting two candidate resource sets, which correspond to the candidate resources within the DRX active time of the Rx UE(s) and all candidate resources (i.e. SA) within the resource selection window respectively.

- RAN2 is responsible for determining the candidate resources corresponding to the DRX active time of the RX UE(s) based on the candidate resource set (i.e. SA) and the adjusted start time of the resource selection window reported by PHY.

Observation 11: Tx UE is aware of the DRX configuration of the Rx UE in any cast type, and determines the allowable transmission time for each Rx UE based on the DRX configuration.

Observation 12: Comparing with scheme that the resource selection window for all the transmissions are restricted to be fully aligned with DRX on-duration of the Rx UE, the scheme that only initial transmission is transmitted during the DRX on-duration of Rx UE can archive better PRR performance.

Proposal 32: When the Rx UE is DRX enabled, Tx UE should ensure that at least the initial transmission can be made during DRX on-duration of the Rx UE to achieve a trade-off between Packet Reception Ratio (PRR) and power consumption.

References

  - 3GPP TSG RAN WG1 Meeting RAN1 #106-e Chairman's Notes, August. 2021.

  - 3GPP TSG RAN WG1 Meeting RAN1 #105-e Chairman's Notes, May. 2021.

  - 3GPP TSG RAN WG1 Meeting RAN1#104-e Chairman's Notes, Feb. 2021.

  - 3GPP TSG RAN WG1 Meeting RAN1 #103-e Chairman's Notes, Novemeber. 2020.

  - R1-2100351, “Discussion on resource allocation for power saving”, CATT, GOHIGH, RAN1 #104-e meeting.

  - RP-193257, “New WID on NR sidelink enhancement”, LG Electronics, RAN #86-E meeting.

  - RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, RAN #90-E meeting.

  - 3GPP TSG RAN WG2 Meeting RAN2 #113b-e Chairman's Notes, April. 2021.

  - R1-2108710, “LS to RAN1 on RAN2 Agreements Related to Resource Selection”, RAN2.

  - 3GPP TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR; (Release 15)”, V15.3.0.

  - 3GPP TSG RAN WG1 Meeting RAN1 #102-e Chairman's Notes, Aug. 2020.

  - 3GPP TR 36.885, “Study on LTE-based V2X Services; (Release 14)”, V14.0.0.

  - 3GPP TR 38.214, “Physical layer procedures for data (Release 16)”, V16.6.0.

- Annex A: System Level Evaluation Assumptions for SL power saving

| Parameter | Value |
| --- | --- |
| Deployment scenario | City scenario with Vehicles and Pedestrians - Vehicle UEs - Vehicle type distribution: 100% vehicle type 2. - Vehicle speed is 60 km/h in all the lanes. - Pedestrians are generated in sidewalk as defined in TR36.885 [12] |
| Channel model | Sidelink: Urban-LOS & Urban NLOS [10] |
| Spectrum allocation | Carrier frequency: 6 GHz Simulated Bandwidth:40 MHz |
| Subcarrier spacing | 30 kHz |
| Traffic model | - P2V - Periodical traffic Option 1: Traffic model for P-UE's transmission specified in TS 36.885 - The message size is fixed at 300 bytes and transmission frequency is 1 Hz - ‘100ms’ latency requirement - Aperiodic traffic Option 4: Aperiodic Model 1 specified in TR37.885 with following changes: - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms - Packet size: Uniformly random in the range between 200 bytes and 800 bytes with the quantization step of 200 bytes - Latency requirement: 250 ms or 100 ms - V2V - 50% Vehicle UEs transmit periodic packets. Model 2 (medium traffic intensity) [10] - Inter-packet arrival time: 100 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms - 50% Vehicle UEs transmit aperiodic packets. Model 1 (medium traffic intensity) [10] - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Packet size: Uniformly random in the range between 400 bytes and 2000 bytes with the quantization step of 400 bytes - Latency requirement: 50 ms |
| SCI/Data frequency resource allocation and Tx parameters | - PSCCH: 10 PRB, 3 OS, QPSK - PSSCH: - 20 PRB for 300 Bytes P2V periodic packets, 16QAM - 10 PRB per 200Bytes P2V aperiodic packets, 16QAM - 30 PRB for 800 Bytes and 1200 Bytes V2V periodic packets, 16QAM - 10 PRB per 400 Bytes for V2V aperiodic packets, 16QAM |
| TX number per TB | - 3 |
| Channel coding | PSCCH: Polar code PSSCH: LDPC |
| Antenna configuration | (Tx, Rx) = (2, 4) |
| Power saving mechanism | - Power saving UE(Pedestrian UE) - Periodic traffic - Periodically partial sensing with performing re-evaluation - Aperiodic traffic - Full sensing with performing re-evaluation - Contiguous partial sensing(31 logical slots before resource selection) with performing re-evaluation - T2 is 60 logical slots and re-evaluation checking window is not restricted - T2 is 32 logical slots and re-evaluation checking window does not exceed n + TB + T2 - Random selection with performing re-evaluation - Random selection without sensing and Re-evaluation - Vehicle UE - Full sensing with performing Re-evaluation |
| Performance metric | - PRR - Power consumption |

- Annex B: System Level Evaluation Assumptions for combined partial sensing and SL DRX

| Parameter | Value |
| --- | --- |
| Deployment scenario | City scenario with Vehicles and Pedestrians - Vehicle UEs - Vehicle type distribution: 100% vehicle type 2. - Vehicle speed is 60 km/h in all the lanes. - Pedestrians are generated in sidewalk as defined in TR36.885 [12] |
| Channel model | Sidelink: Urban-LOS & Urban NLOS [10] |
| Spectrum allocation | Carrier frequency: 6 GHz Simulated Bandwidth:40 MHz |
| Subcarrier spacing | 30 kHz |
| Traffic model | - P2V - Periodical traffic Option 1: Traffic model for P-UE's transmission specified in TS 36.885 - The message size is fixed at 300 bytes and transmission frequency is 1 Hz - ‘100ms’ latency requirement - V2V - 50% Vehicle UEs transmit periodic packets. Model 2 (medium traffic intensity) [10] - Inter-packet arrival time: 100 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms - 50% Vehicle UEs transmit aperiodic packets. Model 1 (medium traffic intensity) [10] - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Packet size: Uniformly random in the range between 400 bytes and 2000 bytes with the quantization step of 400 bytes - Latency requirement: 50 ms |
| SCI/Data frequency resource allocation and Tx parameters | - PSCCH: 10 PRB, 3 OS, QPSK - PSSCH: - 20 PRB for 300 Bytes P2V periodic packets, 16QAM - 30 PRB for 800 Bytes and 1200 Bytes V2V periodic packets, 16QAM - 10 PRB per 400 Bytes for V2V aperiodic packets, 16QAM |
| Channel coding | PSCCH: Polar code PSSCH: LDPC |
| Antenna configuration | (Tx, Rx) = (2, 4) |
| Power saving mechanism | - Power saving UE(Pedestrian UE) - Periodic traffic - Periodically partial sensing independent with SL DRX (with performing re-evaluation) - Periodically partial sensing within DRX on duration only (with performing re-evaluation) - Vehicle UE - Full sensing with performing Re-evaluation |
| DRX configuration | - Power saving UE (Pedestrian UE) - DRX cycle: 250ms - DRX on-duration: 30ms - All pedestrians use the same DRX configuration - Vehicle UE - DRX disabled |
| Performance metric | - PRR - Power consumption |

- Annex C: System Level Evaluation Assumptions for SL DRX

| Parameter | Value |  |
| --- | --- | --- |
| Deployment scenario | City: Option A scenario [10] - Vehicle speed = 60 km/h |  |
| Channel model | Sidelink: Urban-LOS & Urban NLOS [10] |  |
| Spectrum allocation | Carrier frequency: 6 GHz Simulated Bandwidth:20 MHz |  |
| Subcarrier spacing | 30 kHz |  |
| Traffic model | V2V | - 50% Vehicle UEs transmit packets - Periodic: Model 2 (medium traffic intensity [10] - Inter-packet arrival time: 100 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms |
|  | V2P | - 50% Vehicle UEs transmit packets - Option 7: Periodic Model 2 specified in TR 37.885 with following change: [10] - Inter-packet arrival time: 500ms - Latency requirement: 100 ms |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB, 3 OS - PSSCH: 30 PRB for packet size of 800 and 1200 Bytes |  |
| Data Packet Tx parameters | Periodic variable packet size evaluations: - 800 Byte packet: 16-QAM, 1 TTI (CR = 0.444) - 1200 Byte packet: 16-QAM, 1 TTI (CR = 0.667) |  |
| Channel coding | PSCCH: Polar code PSSCH: LDPC |  |
| Antenna configuration | (Tx, Rx) = (2, 4) |  |
| Transmission number per TB | - 3 |  |
| DRX configuration | - Power saving UE (Pedestrian UE) - DRX cycle: 100ms - drx-slotoffset: 0 - drx-startoffset: 0 - DRX on-duration: - Scheme 1: 20ms - Scheme 2: 15ms or 20ms - All pedestrians use the same DRX configuration - Vehicle UE - DRX disabled - V2P UEs were informed of PUE’s DRX configuration in advance |  |
| Performance metric | - PRR - Power consumption |  |