3GPP TSG RAN WG1 #106b-e                                                                           R1-2109236

e-Meeting, October 11th – 19th, 2021

# Source:             CATT

Title:                  Configuration of TRS/CSI-RS for paging enhancement

Agenda Item:    8.7.1.2

Document for:  Discussion and Decision

Introduction

In RAN1#106-e, the following was agreed for UE power saving on configuration of TRS/CSI-RS occasions for idle/inactive UEs[1].

Agreement

Support at least one of the following alternatives

- Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion.

- Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

Note:  The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication.

Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only.

Agreement

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where

- the time duration can be determined based on at least one from the following (to be down-selected):

  - Alt-1: configured by higher layer

  - Alt-2: a predefined/configured window

  - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s)

  - Alt-4: until when the UE receives another availability indication

  - A combination of alternatives or other alternatives is not precluded.

- the reference point can be determined as at least one from the following (to be down-selected):

  - Alt-1: start of next PO or DRX cycle

  - Alt-2: time location where UE receives the indication

    - Note: the time location is subject to application delay if agreed

  - Alt-3: start of current PO or DRX cycle where UE receive the indication

  - Alt-4: a time location which is configured by higher layer

  - A combination of alternatives or other alternatives is not precluded.

Agreement

For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as

  - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

In this contribution, TRS/CRS configuration details for paging enhancement for UE in RRC-Idle/Inactive mode are discussed.

Discussion

Potential TRS/CSI-RS occasion(s) for paging enhancement

TRS/CSI-RS configuration

- Higher layer signaling

The legacy TRS/CRS-RS resource/resource set configuration based on RRC signaling for RRC_CONNECTED mode UE can be considered as the framework of signaling in the SIB design  for the configuration of TRS/CSI-RS occasion(s) for RRC IDLE/Inactive mode UE(s). Legacy TRS/CRS-RS resource/resource set configuration procedure/parameters details are shown in Figure 1. We can see from Figure 1 that roughly ten bytes are required for each NZP-CSI-RS Resource configuration. Considering that one TRS Resource set contains 4 NZP-CSI-RS Resources per beam for FR1, the required signaling overhead for maximum 64 beams is significantly higher than the maximum size of SIB1 or SI message of 2976 bits as specified in TS38.331 [2]. Therefore, TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit.

Proposal 1: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit.

Figure 1:  Example of TRS/CRS-RS resources/resource sets configuration

- TRS/CSI-RS resource configuration in Time domain

Two candidates can be utilized to configure TRS/CRS-RS occasions for paging reception using SIB.

- Option1: TRS/CSI-RS resources are configured with SI without association relation with paging occasion(s)

- Option 2:TRS/CSI-RS resources  are associated with the paging occasion(s)

For option1, if  one or multiple TRS/CSI-RS resources/ resource sets can be configured with SIB signaling which TRS/CSI-RS resource configuration usually at least contains the parameters of TRS/CSI-RS pattern, resource mapping, gold sequence scrambling ID, multi-beam QCL information and other information. The payload size of TRS/CSI-RS resource configuration will create a huge SIB overhead as shown in Figure 1. gNB can configure the TRS/CSI-RS occasions for the CONNECTED mode UE to the IDLE/inactive UEs.  Once such TRS/CSI-RS resource is no longer used by CONNECTED mode UE, gNB needs to reconfigure TRS/CSI-RS resource using update of the system information by paging all IDLE/Inactive mode UEs. UE will read system information block update to acquire new TRS/CSI-RS resource configuration information which will reduce power saving gain of TRS/CSI-RS occasion(s).  Furthermore, when TRS occasion(s) are configured without association to the PO(s), e.g., TRS/CSI-RS can be configured far behind PO boundary, UE could not use such TRS/CSI-RS configuration for channel tracking which will degrade UE power saving gain significantly.

Observation 1: When CSI-RS resources are configured by SI without association with the paging occasion(s), UE might not use the TRS for channel tracking to achieve the UE power saving gain.

TRS/CSI-RS would be configured for all IDLE/Inactive UEs since the network cannot pin the exact UE camping cell within the registration area.  Since a set of TRS/CSI-RS resource is configured for all IDLE/Inactive UEs, the same set of TRS/CSI-RS resource could be UE-specifically configured for CONNECTED mode UEs since the TRS/CSI-RS resource is used for channel tracking of each beam in the cell for both CONNECTED and IDLE/Inactive UEs. The configuration of TRS/CSI-RS resource should be a steady resource configuration to be used by all CONNECTED mode UEs for channel tracking and beam management when UE is under the coverage of a given beam. The TRS/CSI-RS resource would be configured persistently as long as there are UEs under the beam coverage.   The same TRS/CSI-RS resource would be used by IDLE/Inactive UEs for channel tracking and achieving UE power saving. Thus, the TRS/CSI-RS resource configured for IDLE/Inactive UEs would be persistently present and shared with CONNECTED mode UEs without dynamic ON/OFF.

For option2, the TRS/CSI-RS resources are configured and associated with the SSB/paging occasion. As shown in Figure 2, the TRS/CSI-RS could be configured within a window period before paging occasion, which is similar to PS_offset configured for UE to decode DCI format 2_6. TRS/CSI-RS occasion could be configured within the dedicated gap between SSB and paging occasion. The configuration of TRS/CSI-RS resource grid for IDLE/Inactive UEs in SIB-X could be simplified with reduced number of parameters.

For TRS/CSI-RS resource used by CONNECTED mode UE, it is the choice of the network implementation to configure such TRS/CSI-RS resource used for both IDLE and CONNECTED mode UE. Since UE starts with IDLE mode first, a specific configuration of TRS/CSI-RS could help IDLE mode UE to reduce the power consumption. Once UE enters CONNECTED mode, gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE. It implies that a set of TRS/CSI-RS resource are transmitted for IDLE mode UEs and could be used by CONNECTED mode UEs. Considering that option 2 has lower specification efforts and higher UE power saving gain that option 1, TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s).

Figure 2: TRS/CSI-RS occasion is associated with PO/SSB

Observation 2: gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE.

Proposal 2: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) to achieve good power saving gain with low SIB signaling overhead.

- TCI state configuration

In RAN1#105-e, the QCL assumption of TRS/CSI-RS occasion(s) for IDLE/Inactive UEs was agreed to be indicated as SSB index by higher layer as follows,

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

It was agreed in RAN1#104-e that multiple RS resources can be configured for TRS/CSI-RS occasion(s) for IDLE/Inactive UEs.  While paging information is transmitted in multiple beams, multiple TRS resources configuration with one TRS/CSI-RS resource associated with one beam is needed to achieve the UE power saving. In TRS/CSI-RS configuration structure, TCI state is configured per CSI-RS resource set to support the CONNECTED mode UE with dynamic beamforming operation. Once UE is configured with the TRS resource set for channel tracking, one TRS resource set includes two CSI-RS resources in a slot or four CSI-RS resources in two consecutive slots will be configured with same TCI state. Furthermore, multiple TRS resources with same TCI state configured for different connected UEs could be provided to IDLE/Inactive mode UE for AGC/channel tracking performance improvement of IDLE/Inactive UEs. Up to 64 CSI-RS resources can be configured for one TRS/CSI-RS resources set as in TS38.331 [2]. Compared with configuration of TCI state per CSI-RS resource, one CSI-RS/TRS resource set/CSI-Resource configuration can be configured with one TCI state which can reduce unnecessary signaling overhead significantly.

Observation 3: Compared with configuration of TCI state per CSI-RS resource, one TRS resource set can be configured with one TCI state which can reduce unnecessary signaling overhead.

Proposal 3: QCL information configuration of TRS for idle/inactive UE should be configured at least per CSI-RS resource set.

- TRS/CSI-RS occasion(s) configuration procedure

It was agreed in RAN1#102-e [3] that TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported. If TRS/CSI-RS resources associated with the paging occasion(s), an example of periodic TRS/CSI-RS configuration procedure is shown as follows,

Periodic TRS/CSI-RS occasion(s) configuration procedure:

Step1) Configured parameters of TRS/CSI-RS resource from the resourceMapping and periodicityAndOffset of nzp-CSI-RS-Resource set

Step 2) SIB indicates parameters details, including

- QCL assumption of the configured TRS/CSI-RS resources associated with a SSB;

Step 3) TRS occasion(s) after the SSB is obtained based on the configured TRS/CSI-RS resource grid and periodicity in step1 and step 2

The following two alternatives can be considered to support periodic TRS/CSI-RS configuration.

- Alt. 1: TRS/CSI-RS occasion(s) corresponding to single periodic TRS CSI-RS resource/ resource set

- According to TS 38.304 [4], SFN for the Paging Frame (PF) is determined by paging cycle T and total paging frames number N={T, T/2, T/4, T/8, T/16} in T configured by NAS control signaling [6], where N is the function of T. PF frames corresponding to UEs configured with different paging cycles can be aligned in time domain as shown in Figure 3. Similar to derivation of SFN for the PF, gNB can configure TRS_offset including in the SIB X configured for TRS/CSI-RS resources UE can derive the radio frame for TRS/CSI-RS occasion using the following formula

(SFN + PF_offset + TRS_offset) mod T = (T / N)*(UE_ID mod N)

Figure 3: TRS/CSI-RS occasion(s) corresponding to single periodic TRS/CSI-RS resource/ resource set

- Alt. 2: TRS/CSI-RS occasion(s) corresponding to multiple periodic TRS CSI-RS resources/ resource sets

gNB also can configure multiple periodic TRS/CSI-RS resources/ resource sets to associate with the paging occasion(s). As shown in Figure 4, gNB can configure Ns TRS_offset for each TRS/CSI-RS resource corresponding to one PO of target paging frame, where Ns is the number of paging occasions in a  PF. From gNB perspective, TRS/CSI-RS occasion(s) correspond to multiple periodic TRS/CSI-RS resources/ resource sets, which provides more flexibility in configuration than that of Alt. 1.

Figure 4: TRS/CSI-RS occasion(s) corresponding to multiple periodic TRS CSI-RS resources/ resource sets

Proposal 4: The following procedure can be used for TRS/CSI-RS occasion(s) configuration:

Step1) Configured parameters of TRS/CSI-RS resource from the resourceMapping and periodicityAndOffset of nzp-CSI-RS-Resource set

Step 2) SIB indicates parameters details, including

- QCL assumption of the configured TRS/CSI-RS resources associated with a SSB;

Step 3) TRS occasion(s) after the SSB is obtained based on the configured TRS/CSI-RS resource grid and periodicity in step1 and step 2

Availability indication

It was agreed in RAN1 #105-e that support of at least L1 based signaling and further study supporting SIB based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.  L1 and SIB based signaling design details are discussed in this section.

SIB based signaling

The common understanding of SIB based signaling agreed in RAN1#104b-e is that existing SI update procedure is used for SIB based signaling.  The SIB-based signaling is the backbone of the TRS/CSI-RS configuration and availability indication. It was agreed in RAN1#103e that SIB signaling (SIB-X) provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).  The SIB-based signaling for TRS/CSI-RS availability indication is based on the presence/absence of the SIB-X when UE retrieve the system information.  Once the configuration of the TRS/CSI-RS occasion is retrieved by IDLE/Inactive UEs in a given cells, UE would assume TRS/CSI-RS resource would be available persistently until UE is indicated by the SI update, which the SIB-X is not present in the system information.     Compared with L1 signaling mechanism, SIB based availability indication of TRS/CSI-RS at the configured occasion(s) for IDLE/Inactive mode UE can provide affirmative resource availability.  In addition, the configuration of the L1 signaling for TRS/CSI-RS availability indication in the DCI (PEI and/or paging DCI) needs to be included in SIB-X.   If IDLE/Inactive UE needs to decode the SIB-X to get the configuration and TRS/CSI-RS resource information and L1 signaling configuration if it is present, UE could use the TRS/CSI-RS resource before PO without any additional overhead.  If UE moves to a new cell, UE would read SIB-X to retrieve the information of TRS/CSI-RS resource of new cell and L1 signaling configuration for TRS/CSI-RS availability indication. If SIB-X present at the new cell, UE could assume TRS/CSI-RS is available all the time. If gNB decides not to transmit the TRS/CSI-RS for IDLE/Inactive UEs, gNB can send the paging indication to the specific cell only for system information change on the TRS/CSI-RS availability.  There is no additional overhead.

Observation 4: Compared with L1 signaling, SIB based availability indication of TRS/CSI-RS at a given cell for IDLE/Inactive mode UE can provide affirmative TRS/CSI-RS resource availability to achieve UE power saving gain and no additional signaling overhead.

The configuration of TRS/CSI-RS resource is included in SIB-X.  If SIB-X of TRS/CSI-RS for IDLE/Inactive UEs is present, UE should assume TRS/CSI-RS is available for IDLE/Inactive mode UE in the serving cell.  Procedure of SIB based availability indication of TRS/CSI-RS is as follows.

- TRS/CSI-RS resource is configured by SIB-X

  - If SIB-X is present,

    - IDLE/Inactive UE would assume the present of TRS/CSI-RS resources for the subsequent paging occasions

      - UE would assume the TRS/CSI-RS based on the location of TRS/CSI-RS resource for AGC tuning, channel tracking and RSRP measurements

        - For explicitly indication of radio frame and the associated offset, TRS/CSI-RS location is included in the SIB-X for all UEs.

        - For implicitly indication for each UE, TRS/CSI-RS location is derived based on UE ID, SFN, and PF_offset in a formula similar to the PO formula

  - Otherwise

    - IDLE/Inactive UE would assume TRS/CSI-RS resources are not available.

If gNB would like to remove the TRS/CSI-RS resources for IDLE/Inactive UE, gNB can depopulate the SIB-X and send the paging information to all UEs for the system information update. IDLE/Inactive UE could also assume the availability of TRS/CSI-RS resource if the SIB-X for TRS/CSI-RS configuration populates in the system information without any blind decoding.   Upon receiving a SI change notification during a modification period for DRX/eDRX, the UE acquires the new SI immediately from the start of the next modification period. If SIB1 does not schedule SIB-X, UE will assume that there is no TRS/CSI-RS transmission at the configured occasion(s). Whether or not idle/inactive UE needs to read SIB other than SIB-X depends on valueTag contained in the si-SchedulingInfo in SIB1. If valueTag associated with the SIB is not changed compared with previous version, UE can use a valid stored version of the SIB without the need of reading unnecessary SIB information as specified in TS38.331 [2].

Proposal 5: The availability of TRS/CSI-RS at a given cell should be indicated to the UE by SIB-based signaling, which is indicated by the presence/absence of SIB-X.

L1 based signaling

- Validity time

In RAN1#106-e, validity time of L1-based availability indication was discussed with 4 alternatives of time duration alternatives for down selection.

  - Alt-1: configured by higher layer

  - Alt-2: a predefined/configured window

  - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s)

  - Alt-4: until when the UE receives another availability indication

Alt.1 valid time configured by higher layer is a one shot configuration with a given interval and will restrict the UE’s assumption on the availability of TRS/CSI-RS resources only within a fixed time interval. The consideration of TRS/CSI-RS resource validity interval is to assist UE in performing the channel tracking.  However, the restriction of TRS/CSI-RS resource availability time interval would degrade   the UE power saving performance.  Alt.3 will introduce additional overhead of L1-based availability indication and create the uncertainty of TRS/CSI-RS availability in achieving the UE power saving gain. There is no distinct advantage of dynamic indication of validity time of TRS/CSI-RS resource with additional overhead in DCI.  Alt.2 can only support indicating the availability of TRS/CSI-RS in a specific time window before the PO, which would not achieve the desired power saving gain by using TRS/CSI-RS for channel tracking and estimation. From UE power saving perspective, TRS/CSI-RS occasion should be configured period associated with PO with persistent.  The TRS should be located before the PDCCH-based PEI to allow UE wake up later to achieve the same level of preparation for PEI detection and achieving the power saving gain as shown in Figure 5.  The TRS/CSI-RS can be used to reduce the number of SSBs required by UE for preparation of PDCCH based PEI.

Figure 5: TRS assisted DCI based PEI reception

Power saving gain with different valid time interval of TRS/CSI-RS resource is evaluated for idle/inactive UE, as shown in Figure 6. TRS is only transmitted within the time interval with average paging rate=0.1, DRX cycle=1.28s. We can see L1 based signaling of availability indication with validity timer will degrade UE power saving gain significantly, e.g., with validity timer of 10 DRX cycle, 38% of UE power saving gain degradation is observed comparing with persistent TRS transmission without validity timer.  Therefore, validity timer with small interval will seriously degrade power saving gain but large validity timer will provide additional system overhead from viewpoint of gNB. Therefore, UE should assume an availability indication is always valid before receiving new availability indication.

Figure 6: Power saving gain with different validity timer

Proposal 6: UE should assume an availability indication is always valid before receiving new availability indication.

To support L1-based availability indication of TRS/CSI-RS at the configured occasion(s), another issue is how to determine starting point of validity time duration, i.e., reference point. The starting point of validity time should be after  the L1-based signaling of availability indication and before the next PO when UE is in preparation of PEI detection and paging DCI decoding.  The reference point of validity time duration should be after the current PO which L1-based signaling indicating the availability of TRS/CSI-RS is detected and before the next PO or DRX cycle.

Proposal 7: Reference point of validity time duration should be after the current PO which L1-based signaling indicating availability of TRS/CSI-RS is detected and before at the starting point of the next PO or DRX cycle for both paging DCI and PEI based availability indication.

- Signaling type

Whether or not both paging DCI and PEI are supported for availability indication was heavily discussed in RAN1#105-e and RAN1#106-e. For paging DCI indication, the consensus is that it should be supported at least for the case when PEI is not configured.

Proposal 8: Paging DCI based availability indication should be supported at least for the case when PEI is not configured.

When PEI is configured, the power saving gain should be evaluated for TRS/CSI-RS availability indication carried by PEI or paging DCI.  UE is required to decode the PEI before each paging occasion regardless there is a paging or not.   UE would only decode the paging DCI when it is indicated by the PEI to decode and retrieve the paging information.   The availability indication of TRS/CSI-RS resource would be the main factor in achieving IDLE/Inactive UE power saving which the UE power saving gain would be optimized when TRS/CSI-RS is persistently available. The availability indication carried by PEI or paging DCI should not affect much of UE power saving gain.  UE Power saving gains for PEI and paging DCI carrying TRS availability indication are evaluated below. In the simulation, DCI based PEI is considered in the evaluation with corresponding procedure of TRS/CSI-RS assisted PDCCH-based paging indication as shown in [6]. It is assumed that TRS availability probability is 50% and the gNB needs to wake up all UEs using PEI to receive the paging message for acquiring TRS availability indication in case when the TRS availability information changes. UE will assume that TRS/CSI-RS resource availability indication by PEI/paging DCI is persistently valid before receiving new availability indication.  Figure 7 shows the UE power saving gain performance of TRS/CSI-RS resource availability indication by PEI over that by paging DCI with different paging rates and number of paging subgroups.  We can see from Figure 7 that only negligible UE power saving gains is achieved for PEI based availability indication over that of paging DCI. The reason is that when the PEI is configured, the UE power saving gain mainly provided by PEI. For average paging rate = 0.1, the probability of waking up all UEs with PEI to receive unnecessary paging DCI is very low.    For higher paging rate, UE has high probability to avoid unnecessary UE wake up, which causes marginable UE power saving loss for availability indication carried by paging DCI. In addition, the additional bit(s) for TRS availability indication in PEI would degrade the PEI detection performance.

Observation 5: PEI based availability indication has a negligible UE power saving gain over that of paging DCI.

Figure 7: Power saving gain of PEI over paging DCI

- Indication content

In RAN1 #106-e, whether to support beam based physical layer availability/unavailability indication information for configured RS resources using a bitmap or codepoint was discussed and the following alternatives were provided,

- Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion.

- Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion.

One paging occasion is configured at each SSB in a multi-beam cell with DL common channel beam sweeping as in TS38.304 [5]. In order to achieve UE power saving in a multi-beam cell, one TRS/CSI-RS resource needs to be configured at each beam. Since TRS/CSI-RS resources are FDM/TDM’d between different beams within a cell, TRS/CSI-RS resources for all beams need to be configured together in the SIB-X with each beam having QCL assumption associated with the SSB ID in a cell. If one beam is NOT and the other beams are configured with TRS/CSI-RS resource, UE would not be able to assume the present of TRS/CSI-RS since UE does not know which beam UE is under coverage (a given SSI ID) after long deep sleep due to UE mobility. Therefore, alt.1, i.e., beam specific availability indication dose not work and all TRS/CSI-RS resources within a cell should be configured and available/ unavailable for all beams together. The subset of TRS/CSI-RS resources for the selected beams cannot help UE achieving power saving. Thus, the availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell.

Proposal 9: The availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell.

For paging DCI based availability indication, one bit of reserved bits field of paging DCI is configured for UE by SIB-X signaling to use as TRS availability indication field.  A UE needs to determine such bit field in the corresponding bit location of paging DCI to receive the indication information for the configured occasion(s), as description in Table 1.

Table 1：Example for availability indication

| The bit filed indication | Description |
| --- | --- |
| 1 | UE assume presence of all TRS/CSI-RS configured at the configured occasion(s) |
| 0 | UE assume absence of TRS/CSI-RS at the configured occasion(s) |

For PEI based availability indication (if supported), there is additional 1 bit in PDCCH-based PEI to indicate TRS availability. When one PDCCH-based PEI indicates multiple POs and TRS availability, each PO is mapped to a bit field in PDCCH-based PEI and 1-bit TRS availability indication is mapped to TRS availability indication field. Assuming that the number of POs associated with one PEI is N1, there are N1+1 bit fields in PDCCH-based PEI, as shown in Figure 8. The first N1 bit fields are used to indicate N1 POs respectively and the last one bit field is used to indicate TRS availability. PDCCH-based PEI also indicates sub-group and TRS availability in the form of bitmap. The UE needs to determine TRS availability indication field in the corresponding bit location of PDCCH-based PEI to receive the indication information for the configured occasion(s).  Further details can be found in [7].

Figure 8: The bit fields design of PDCCH-based PEI if supporting TRS availability

- L1 based signaling configuration

When L1 signaling is supported for availability indication of TRS/CSI-RS resources, the extra overhead of L1 signaling would be present all the time even network would have the TRS/CSI-RS resources available all the time for the UE.  This would increase the resource overhead and UE complexity in decoding the L1 signaling all the time. Thus, it would be more efficient to have mechanism controlled by the network to enable/disable the L1 signaling of available indication of the configured TRS/CSI-RS resources for IDLE/Inactive UEs. The most straight forward way of enable/disable the L1 signaling of TRS/CSI-RS availability indication is to include 1 bit enable/disable parameter together with the TRS/CSI-RS resource configuration in SIB-X. If the parameters enable/disable L1 signaling for TRS/CSI-RS resource availability indication is shown “disable” in SIB-X, UE would assume the TRS/CSI-RS resource is available all the time. The other alternatives of enable/disable L1 signaling is the present/absent of parameter of L1 signaling configuration in SIB-X.  If the parameter of L1 signaling configuration in the DCI (PDCCH-based PEI and/or paging DCI) is not present in SIB-X, UE does not know where to detect the L1 signaling for TRS availability indication.  Thus, UE would assume the TRS is available persistently until UE is informed of the information change in SIB-X.   gNB does not need to include the L1 signaling for the control TRS/CSI-RS resource availability.  UE does not to decode the L1 signaling for the TRS/CSI-RS resource availability indication when it is available all the time.

Proposal 10: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X.

Conclusion

In this contribution, configuration of TRS/CSI-RS for paging enhancement is discussed.  We have the following observations and proposals:

Proposal 1: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit.

Observation 1: When CSI-RS resources are configured by SI without association with the paging occasion(s), UE might not use the TRS for channel tracking to achieve the UE power saving gain.

Observation 2: gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE.

Proposal 2: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) to achieve good power saving gain with low SIB signaling overhead.

Observation 3: Compared with configuration of TCI state per CSI-RS resource, one TRS resource set can be configured with one TCI state which can reduce unnecessary signaling overhead.

Proposal 3: QCL information configuration of TRS for idle/inactive UE should be configured at least per CSI-RS resource set.

Proposal 4: The following procedure can be used for TRS/CSI-RS occasion(s) configuration:

Step1) Configured parameters of TRS/CSI-RS resource from the resourceMapping and periodicityAndOffset of nzp-CSI-RS-Resource set

Step 2) SIB indicates parameters details, including

- QCL assumption of the configured TRS/CSI-RS resources associated with a SSB;

Step 3) TRS occasion(s) after the SSB is obtained based on the configured TRS/CSI-RS resource grid and periodicity in step1 and step 2

Observation 4: Compared with L1 signaling, SIB based availability indication of TRS/CSI-RS at a given cell for IDLE/Inactive mode UE can provide affirmative TRS/CSI-RS resource availability to achieve UE power saving gain and no additional signaling overhead.

Proposal 5: The availability of TRS/CSI-RS at a given cell should be indicated to the UE by SIB-based signaling, which is indicated by the presence/absence of SIB-X.

Proposal 6: UE should assume an availability indication is always valid before receiving new availability indication.

Proposal 7: Reference point of validity time duration should be after the current PO which L1-based signaling indicating availability of TRS/CSI-RS is detected and before at the starting point of the next PO or DRX cycle for both paging DCI and PEI based availability indication.

Proposal 8: Paging DCI based availability indication should be supported at least for the case when PEI is not configured.

Observation 5: PEI based availability indication has a negligible UE power saving gain over that of paging DCI.

Proposal 9: The availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell.

Proposal 10: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X.

References

- RAN1 chairman notes, 3GPP TSG RAN WG1 #106-e.

- TS38.331v16.4.0, Radio Resource Control (RRC) protocol specification.

- RAN1 chairman notes, 3GPP TSG RAN WG1 #102-e.

- TS38.304v16.4.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- TS24.501v16.8.0, Non-Access-Stratum (NAS) protocol for 5G System (5GS).

- R1-2102641, Paging enhancement for UE power saving, CATT.

- R1-2109235, Paging enhancement for UE power saving, CATT.