3GPP TSG RAN WG1 #106bis-e		                                                      R1-2109245

e-Meeting, October 11th – 19th, 2021

Source:	CATT

Title:	Views on reusing PUSCH enhancements for Msg3

Agenda Item:	8.8.4

Document for:	Discussion and Decision

Introduction

In RAN#90-e meeting, the new WID on coverage enhancement was approved. It is well-understood that the coverage of Msg3 is a bottleneck channel in NR which needs to be enhanced. Accordingly, the following objective for Msg3 PUSCH enhancement is included in the WID. [1]

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

Additionally, three objectives for PUSCH enhancement were also approved and included in the WID:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

Considering Msg3 PUSCH is PUSCH in nature, it would be beneficial to discuss whether the solutions to enhance the coverage of normal PUSCH can also be applied to Msg3 PUSCH. In this contribution, we provide our views on whether and how to reuse PUSCH enhancements for Msg3 PUSCH.

Discussion

In coverage enhancement WID, PUSCH and Msg3 PUSCH are separately described, which are summarized in Table 1. More mechanisms for PUSCH are pursued to enhance the coverage of PUSCH. As a result, network has more options to achieve a better coverage for PUSCH, e.g. applies more than one mechanism at the same time. In the end, Msg3 PUSCH would again become the bottleneck because of less functionality to improve the coverage. It is better to consider whether the mechanism applied to PUSCH can be directly reused by Msg3 PUSCH.

Table 1 Comparison of the objectives between PUSCH and Msg3 PUSCH.

|  | Objectives for PUSCH | Objective for Msg3 PUSCH |
| --- | --- | --- |
| Detail enhancements | Enhancement on PUSCH repetition type A(typically the same target and can be considered together) | Type A PUSCH repetition |
|  | TB processing over multi-slot PUSCH(not align with Msg3 PUSCH objective) |  |
|  | Joint channel estimation(may be easily applied to Msg3 PUSCH if repetition type A is enabled) |  |

Fundamentally, Msg3 PUSCH is also one kind of PUSCH. The only difference between Msg3 PUSCH and normal PUSCH is that they are scheduled with different signaling. Theoretically, all the techniques applied to a normal PUSCH could be applied to Msg3 PUSCH.

In RAN1#106-e, the following agreements were reached [2]:

Agreement - The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered. - If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot. Agreement Do not support TBoMS for Msg3 in Rel-17 coverage enhancement WI.

It is clear that TBoMS is not supported in Msg3, and Msg3 PUSCH repetition shall be based on available slots (similar to AI 8.8.1.1). However, it is still unclear if joint channel estimation can be used for Msg3 PUSCH repetition.

In general, joint channel estimation on multiple slots requires gNB to operate channel estimation on consecutive slots. It makes sense to also take it into consideration for Msg3 PUSCH repetition in order to further enhance the coverage. The exactly same mechanisms for PUSCH can be reused for Msg3 PUSCH transmission with repetition type A.  However, from UE’s view, the actual time domain window for Msg3 cannot be determined until a UE receives relative RRC configuration, e.g. window length L of the configured TDW. From gNB’s view, the actual time domain window cannot be determined when the maximum duration is unknown if RAN4 determines that multiple values of maximum duration are specified and need to be reported. Hence, we propose that joint channel estimation would be adopted only for the UEs in RRC_CONNECTED state.

Proposal 1: Joint channel estimation can only apply to the UEs in RRC_CONNECTED state.

- Joint channel estimation cannot be applied to Msg3 PUSCH repetition during initial random access.

- Joint channel estimation can be applied to Msg3 PUSCH repetition in RRC_CONNECTED state.

Conclusion

This contribution provided our views whether to reuse the mechanisms of PUSCH for Msg3. Accordingly, we have the following proposal:

Proposal 1: Joint channel estimation can only apply to the UEs in RRC_CONNECTED state.

- Joint channel estimation cannot be applied to Msg3 PUSCH repetition during initial random access.

- Joint channel estimation can be applied to Msg3 PUSCH repetition in RRC_CONNECTED state.

References

  - RP-202928, New WID on NR coverage enhancements, China Telecom.

  - Chairman's Notes RAN1#106-e, 3GPP TSG RAN WG1 Meeting #106-e, e-Meeting, August 16th – 27th, 2021.