3GPP TSG RAN WG1 #106b-e			                                  R1-2109298

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.8.2

Source: 	CMCC

Title: 	Discussion on PUCCH enhancements

Document for:	Discussion and Decision

Introduction

In the RAN1 #106e meeting, the PUCCH enhancements was discussed focusing on the dynamic PUCCH repetition factor indication and DMRS bundling across PUCCH repetition [1]. The agreements will be listed within the corresponding sections.

In this contribution, we provide our views on the PUCCH enhancements.

Discussion

2.1 Dynamic PUCCH repetition factor indication

In the last meeting, the working assumption was confirmed to support dynamic PUCCH repetition factor indication.

Confirm the following working assumption Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication. - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor. - FFS: RRC signaling enhancement details

Based on the working assumption, the repetition factor should be configured per PUCCH resources. The PRI and the CCE index could be used to indicate both the PUCCH resources and the repetition factor. But there are still some issues about the indication mechanism. If the 2nd starting CCE position is used to indicate repetition factor 2, then all the UEs allocated with 2 repetitions would be overlap in the PDCCH and conflict.

Another issue is that, the starting CCE index are limited due to the aggregation level. For those high aggregation level such as 8 and 16, the staring position are limited. Then then candidate repetition factors configured for those PUCCH should also be limited.

Observation 1:

If the staring CCE index is used to indicate the repetition factors, there would be conflicts for the UE who share a same repetition factor.

Observation 2:

The staring CCE index would be limited due to high aggregation level.

In the last meeting, the repetition factors have been agreed as follows.

Agreement - In Rel-17, reuse the Rel-16 PUCCH repetition factors 2, 4, 8. - Do not support PUCCH repetition factor larger than 8 In Rel-17.

It was concluded that no other repetition factors would be supported beyond 2,4 and 8. Then there is still one issue about how to indicated no repetition or the repetition factor 1. Two options could be considered. One is to include the repetition factor 1 into the repetition factor set, using the PRI and starting CCE index to indicate “no repetition”. The other is to use other filed to indicate no repetition is applied for this PUCCH transmission. It should be noted that using the starting CCE index to indicate repetition factor is limited due to the aggregation level.

Proposal 1:

The mechanism to indicate no repetitions should be discussed for PUCCH. Both including the repetition factor 1 into the set and additional indication could be considered.

2.1.1 Applicability of dynamic PUCCH repetition factor indication

In the last meeting, one FL’s proposal was provided about the supported PUCCH formats.

FL Proposal 1: Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4.

Current specification only support PUCCH repetition for PUCCH format 1, 3 and 4. PUCCH format 0 and 2 are not supported. Based on current WID, only signaling for dynamic indication is supposed to be specified. Enhance the other PUCCH format with repetitions is out of the scope at least for CE. From CE’s perspective, during the SI, the PUCCH format 0 and format 2 are not the bottleneck for the coverage. Then there is also no motivation to enhance those two formats.

On the other hand, during the meeting some company mentioned that the enhancement of the PUCCH format 0 and 2 are discussed in another agenda. At least, the AI of CE should wait for the conclusion from the other agenda. In addition, it needs to further clarify motivation to enhance those formats, as the CE SI do not provide any information about that.

Proposal 2:

Currently only PUCCH format 1, 3, 4 should be supported. Format 0 and 2 needs more discussion and clarification.

2.2 DMRS bundling across PUCCH repetitions

The DMRS bundling of PUCCH DMRS is similar with the joint channel estimation of PUSCH. The coverage could be improved due to the enhancements of the accuracy of channel estimations. The PUCCH repetition under the consecutive slots could be benefit from the DMRS bundling. The PUCCH repetition number could be reduced due to the improvement of channel estimation. Similar as joint channel estimation, the DMRS bundling should consider the power consistency, phase consistency, and the use of time-frequency resources. Since the frequency hopping is usually configured for the PUCCH transmission. The combination of frequency hopping and DMRS bundling should also discussed.

In the 105e meeting, the following agreements have been reached.

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements. - Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

Since the combination of frequency hopping and joint channel estimation have been discussed in the agenda of joint channel estimation, frequency hopping pattern should at least the starting point.

Proposal 3:

For the design of frequency hopping, the DMRS bundling of PUSCH should could be the starting point of PUCCH.

2.2.1 Time domain window design details

One issue was raised during the last meeting and no conclusion had been reached.

Updated FL Question 2: the RRC configuration for DMRS bundling across PUCCH repetitions is - Option 1: per UE - Option 2: per UL BWP - Option 3: per PUCCH resource format - Option 4: per PUCCH resource

In Rel-16, the PUCCH repetitions is configured per PUCCH format. And in the current discussion, the PUCCH repetition could be dynamically indicated. Then for the DMRS bundling, at least per PUCCH format should be supported. And finer granularity (option 4) could bring more flexibility for gNB to use DMRS bundling or not. Per UE configuration is very similar to the per PUCCH format. Once the DMRS bundling is enabled, then the RRC reconfiguration are need to turn-off this function, which means the UE would always work in the mode of DMRS bundling or not. Per UL BWP configuration would require a duplicated configurations for both DMRS bunded BWP and non-DMRS bundled BWP. Then the option 4 is slightly preferred.

Proposal 4:

The option 4, per PUCCH resource configured DMRS bundling, is preferred, as this option could provide more flexibility to enable and un-enable the function.

- 2.2.1 Power control and TA

The prerequisite for enabling joint channel estimation is that the UE should keep power consistency and phase continuity during the time domain window. The TA adjustments and the TPC occurs in the time domain window would violate the condition.

As discussed in our companion paper in the JCE of PUSCH, the TA command should be ignored. And the TPC command could be accumulated within the time domain window but without taking effect. When the time domain window is closed, UEs could transmit in the updated power. This is the same issue for PUSCH JCE. PUCCH could reuse the conclusion from PUSCH JCE.

Proposal 5：

- During the time domain window, he TA command could be ignored.

- The TPC command could be accumulated but not take into practice. When the time domain window is closed, UEs could transmit in the updated power.

Proposal 6：

DMRS bundling across PUCCH repetition could reuse the conclusion of PUSCH JCE for the TA updates and TPC commands.

Conclusions

In this contribution, we provide our views on the PUCCH enhancements. The proposals are as below.

Observation 1:

If the staring CCE index is used to indicate the repetition factors, there would be conflicts for the UE who share a same repetition factor.

Observation 2:

The staring CCE index would be limited due to high aggregation level.

Proposal 1:

The mechanism to indicate no repetitions should be discussed for PUCCH. Both including the repetition factor 1 into the set and additional indication could be considered.

Proposal 2:

Currently only PUCCH format 1, 3, 4 should be supported. Format 0 and 2 needs more discussion and clarification.

Proposal 3:

For the design of frequency hopping, the DMRS bundling of PUSCH should could be the starting point of PUCCH.

Proposal 4:

The option 4, per PUCCH resource configured DMRS bundling, is preferred, as this option could provide more flexibility to enable and un-enable the function.

Proposal 5：

- During the time domain window, he TA command could be ignored.

- The TPC command could be accumulated but not take into practice. When the time domain window is closed, UEs could transmit in the updated power.

Proposal 6：

DMRS bundling across PUCCH repetition could reuse the conclusion of PUSCH JCE for the TA updates and TPC commands.

References

- [1] Draft Report of 3GPP TSG RAN WG1 #106e, e-Meeting, 16th May - 27th August 2021