3GPP TSG RAN WG1#106-bis-e	                                           	      R1-2109356

e-Meeting, October 11th – October 19th, 2021

Agenda Item:	8.3.2

Source:	NEC

Title:	Enhancements for unlicensed band URLLC/IIoT

Document for:	Discussion

- Introduction

In RAN#106-e, the following agreements have been made on enhancements for unlicensed band URLLC/IIoT [1]:

Agreement

In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI:

- The UE follows the indicated COT initiator as the following:

  - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Agreement

When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

- Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

In this contribution, we discuss the remaining issues related to URLLC transmissions on the support of UE-initiated COT for configured grant and release of UE initiated COT.

- Discussion

## 2.1 	Support of UE-initiated COT for Configured Grant

In semi-static channel access mode when a UE can operate as initiating device, it has been agreed that the type of COT determination is based on the content in the scheduling DCI. The basic operations can be achieved via gNB initiated COT. Within each COT, there can be multiple transmissions in both DL and UL. UEs served by the gNB may transmit within the gNB’s shared COT. For UE initiated FFP with configured grant, collision handling can become more complex. For example, UE1 may have a transmission via UL configured grant using UE1 initiated COT based on UE’s FFP, another high priority UE2 may also want to transmit, the high priority UE2 may not be able to transmit even at the next FFP if the start of FFP is aligned with UE1’s configured grant.

UE initiated COT is introduced to improve the support of URLLC UL configured grant transmission. We propose that gNB can explicitly release a UE initiated COT in order to support high priority URLLC transmissions. As shown in Figure 1, UE#1 is sending UL transmissions using FFP-ue1, while UE#2 would need to send a high priority UL transmission via configure grant. Even though UE#2 has a higher priority UL transmission, it can be blocked due to LBT failure, also the gNB is not aware that UE#2 has data to transmit. UE#1 may initiate a COT successfully in every FFP if its configured grant transmission is aligned with the FFP boundary. To overcome this, a higher energy detection threshold level may be configured for the URLLC UE, and UE with URLLC data may be allowed to transmit immediately, even if the channel is considered occupied otherwise. gNB can then detect this UL transmission and identify the UE. This would be helpful in reducing the access delay for URLLC service.

Figure 1 Release of UE Initiated COT for Configured Grant

Upon detecting the high priority transmission, gNB may send a PDCCH to cancel a low priority UE’s transmission and/or release the corresponding UE initiated COT (if it is not a scheduled transmission). To avoid the case in which the gNB may need to repeatedly cancel the UE initiated COT, as the UE with configured grant transmission may successfully obtain the channel again at its next FFP, it is proposed that once a UE initiated COT is released by gNB, the low priority UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

Proposal 1: Support EDT calculation based on the traffic priority to allow early transmission of high priority URLLC service.

Proposal 2: gNB may cancel a low priority UE’s transmission and/or release the corresponding UE initiated COT in order to support another high priority URLLC transmission.

Proposal 3: Once a UE initiated COT is released by gNB, the UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

# 5 Conclusion

In this contribution, we discussed issues related to UE initiated COT for FBE, Release of UE initiated COT, and proposed the following:

Proposal 1: Support EDT calculation based on the traffic priority to allow early transmission of high priority URLLC service.

Proposal 2: gNB may cancel a low priority UE’s transmission and release the corresponding UE initiated COT in order to support high priority URLLC transmission of another UE.

Proposal 3: Once a UE initiated COT is released by gNB, the UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

# References

[1] Chairman’s notes for RAN1#106-e