3GPP TSG RAN WG1 #106bis-e			R1-2109400

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.4

Source:		Xiaomi

Title:	Discussion on UE features for NTN

Document for:	Discussion

Introduction

In [1], the preliminary RAN1 UE features for Rel-17 NR NTN are provided. In this contribution, we provide our comments on these RAN1 UE features.

Comments on UE features

On timing relationship aspects

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

In RAN1 104e meeting, an agreement on the K1 extension was made as follows:

The motivation to have extension of K1 is to support the contiguous DL slots in specific TDD deployment scenario. UE should be able to support this feature when operating with some specific TDD configuration. Thus this should be added as an optional UE feature.

Proposal 1: Add the extension of K1 value as the UE optional feature specifically to TDD cases.

On HARQ aspects

26-5 is the UE feature for increasing the number of HARQ processes. The agreements were made in RAN1 102e meeting [2] on the extension of maximal HARQ process number as follows:

Agreement:

The extension of maximal HARQ process number can be considered with following assumptions:

- The maximal supported HARQ process number is up to 32.

- FFS: Support on the maximal HARQ process number is up to UE capability

- Minimizing the impacts on specification and scheduling

Currently, no agreement is made on whether it is a UE capability to support the maximal 32 HARQ processes. Thus, we suggest to set it as the [optional] feature at this stage.

Proposal 2: UE feature for increasing the number of HARQ processes is set as [optional] due to the lack of agreements.

Agreement:

- For Type-1 HARQ codebook, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, one of following options should be supported:

  - Option-1: The UE will report NACK only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH

  - Option-2: The UE will report NACK/ACK for the feedback-disabled HARQ process depending on the decoding results of corresponding PDSCH

Agreement:

For Type-1 HARQ codebook, if only DCI carrying feedback-disabled HARQ process is detected by UE, one of following options should be supported:

- Option-1: The UE’s behavior is same as the case if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE

- Option-2: The UE should skip the codebook feedback at least when the feedback is carried by PUCCH

  - FFS: the case that feedback is carried by PUSCH.

26-5 is the UE feature for HARQ codebook enhancements. For type-1 HARQ codebook, the following agreements were made till now [3]

In our understanding, if DCIs carrying the feedback-disabled and feedback-enabled HARQ processes are detected by UE, option 2 is the legacy UE behaviour. Thus to our understanding, it is still not decided yet enhancements for type-1 HARQ codebook is needed at this stage. It is suggested to remove the description of “Enhancement on Type-1 HARQ codebook in NTN” in the UE feature 26-5.

Proposal 3: Remove the description of “Enhancement on Type-1 HARQ codebook in NTN” in the UE feature 26-5.

On beam management/polarization aspects

Conclusion:

Discuss the necessity of reporting UE polarization capability considering at least following aspects,

- Deployment scenarios.

- UE implementation aspects with respect to polarization.

- Satellite implementation aspects for switching between polarization states.

- Satellite implementation aspects for realizing multiplexing of UEs having different polarization capabilities.

26-8 is the UE feature for support of polarization signalling in NR NTN. The UE polarization capability (RHCP, LHCP, Linear) is described as one component of UE feature. However, it is not concluded that it is a UE capability yet as indicated in [4]. Thus it is suggested to remove “UE polarization capability (RHCP, LHCP, Linear)” in the description.

Proposal 4: Remove the description of “UE polarization capability (RHCP, LHCP, Linear)” in the UE feature 26-8.

Conclusions

In this contribution, we present the discussion on the UE features for NR NTN. Based on our analysis, we have the following proposals:

Proposal 1: Add the extension of K1 value as the UE optional feature specifically to TDD cases.

Proposal 2: UE feature for increasing the number of HARQ processes is set as [optional] due to the lack of agreements.

Proposal 3: Remove the description of “Enhancement on Type-1 HARQ codebook in NTN” in the UE feature 26-5.

Proposal 4: Remove the description of “UE polarization capability (RHCP, LHCP, Linear)” in the UE feature 26-8.

References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderaators (AT&T, NTT DOCOMO), RAN1#106-e, August 16th – 27th, 2021

- 3GPP RAN1 102e chairman-note

- 3GPP RAN1 106e chairman-note

- 3GPP RAN1 104e chairman-note