3GPP TSG RAN WG1 #106bis-e			R1-2109450

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.11.1.2

Source: 	Hyundai Motors

Title:	Discussion on inter-UE coordination for Mode 2 enhancements

Document for:	Discussion and Decision

Introduction

In the Rel-17 WID for NR sidelink enhancement [1], an objective on resource allocation enhancement in mode2 is as follows:

2. Resource allocation enhancement: - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89.

In RAN1 #106-e [2], the following agreements were made:

Agreement For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B. - Set of resources preferred for UE-B’s transmission - Set of resources non-preferred for UE-B’s transmission Agreement For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI Agreement - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2: - A UE that sends an explicit request for inter-UE coordination information can be UE-B - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A - Working assumption At least a destination UE of a TB transmitted by UE-B can be UE A - The above feature can be enabled or disabled or controlled by (pre-)configuration - FFS: Details on how to support this, including (pre-)configuration signaling granularity - FFS: Additional details and conditions on UE-A and UE-B - Working Assumption In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2: - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B - The above feature can be enabled or disabled or controlled by (pre-)configuration - FFS: Details on how to support this, including (pre-)configuration signaling granularity - FFS: Additional details and conditions on UE-A and UE-B Agreement In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2: - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A - Working assumption At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s) - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured - FFS: Additional details and condition(s) on UE-A and UE-B - The above feature can be enabled or disabled or controlled by (pre-)configuration - FFS: Details on how to support this, including (pre-)configuration signaling granularity - FFS: Definition of expected/potential resource conflict(s) and other details (if any) Agreement In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A: - UE-B can determine resource(s) to be re-selected based on the received coordination information - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated - FFS: Other details (if any) Agreement In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any) - FFS: Other option(s), and other details (if any) - For non-preferred resource set, - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: Details including - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any) - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set - FFS: Other option(s), and other details (if any) Agreement In scheme 2, at least the following is supported to determine inter-UE coordination information: - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s): - Condition 2-A-1: - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency - FFS: Other details (if any) - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication - (Working Assumption) Condition 2-A-2: - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation - FFS: Other details (if any) - FFS: Other condition(s) - FFS: Other details (if any) Agreement In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set: - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission - Condition 1-A-1: - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold - FFS: Other details (if any) - FFS: Condition 1-A-2: - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B - FFS: Other details (if any) - FFS: Condition 1-A-3: - Resource(s) satisfying UE-B’s traffic requirement (if available) - FFS: Other details (if any) - FFS: Other condition(s) - FFS: Other details (if any) Agreement In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set: - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission - Condition 1-B-1: - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement - FFS: Other details (if any) - FFS: Condition 1-B-2: - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B - FFS: Other details (if any) - FFS: Other condition(s) - FFS: Other details (if any)

In this contribution, we discuss resource allocation enhancement in mode 2 based on inter-UE coordination.

Discussion

In RAN1 #106-e meeting, for inter UE coordination scheme 1, the following inter-UE coordination information signalling from UE-A is supported.

- Set of resources preferred for UE-B’s transmission

- Set of resources non-preferred for UE-B’s transmission

At the UE-A side, UE-A can transmit the set of resources either preferred or non-preferred for UE-B’s transmission. Thus, at the UE-B side, UE-B should determine whether the coordination information is preferred resource sets or non-preferred resource sets after receiving the coordination information sent from UE-A. For this determination, an indicator should be required in inter UE coordination scheme 1 and the SCI can include a 1-bit field for this indication. Alternatively, this indicator can be contained in the contents of the coordination information.

In both schemes, the UE-B may be received the coordination information from multiple the UE-As. In this case, the priority of inter-UE coordination information can be included in the contents of the coordination information. Using this priority, if UE-B receives multiple resource sets from multiple UE-As, UE-B will (re)select the resource for its transmission based on the higher priority inter UE coordination information.

The payload size of the inter-UE coordination information for the scheme 1 may be larger due to indicating the set of resources. Thus, for the scheme1, the SCI (e.g., 1st SCI or 2nd SCI or new 2nd SCI format) is more suitable for the container of the coordination information considering the payload size. Meanwhile, for the scheme 2, the PSFCH (or PSFCH-like format) can be used for indicating the presence of resource conflict.

Proposal 1: For inter-UE coordination scheme 1, 1-bit field is needed to indicate whether the coordination information is the set of resources preferred or non-preferred for UE-B’s transmission.

Proposal 2: For inter-UE coordination, further study for the coordination information provided by multiple UEs is needed.

Proposal 3: The SCI can be used for the container of inter-UE coordination scheme 1 and the PSFCH (or PSFCH-like format) can be used for the container of inter-UE coordination scheme 2.

Conclusion

In this contribution, the following conclusions were made:

Proposal 1: For inter-UE coordination scheme 1, 1-bit field is needed to indicate whether the coordination information is the set of resources preferred or non-preferred for UE-B’s transmission.

Proposal 2: For inter-UE coordination, further study for the coordination information provided by multiple UEs is needed.

Proposal 3: The SCI can be used for the container of inter-UE coordination scheme 1 and the PSFCH (or PSFCH-like format) can be used for the container of inter-UE coordination scheme 2.

Reference

[1] RP-201385, “WID revision: NR sidelink enhancement,” LG Electronics.

[2] R1-2108569, “Feature lead summary for AI 8.11.2.2 Inter-UE coordination for Mode 2 enhancements,” RAN1 #106-e, Moderator (LG Electronics).