3GPP TSG-RAN WG1 Meeting#106bis-e                                       R1-2109464

E-meeting, October 11th – 19th, 2021

Title:	[Draft] LS Reply to RAN2 on inter-cell beam management and multi-TRP in Rel-17

Response to:	R1-2108717/R2-2108925

Release:	Rel-17

Work Item:	NR_FeMIMO-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact Person:

Name:	Emad Farag

E-mail Address:	e.farag@samsung.com

# Overall description

RAN1 would like to thank RAN2 for the LS on “inter-cell beam management and multi-TRP in Rel-17”. In the LS, RAN2 raised a few questions. RAN1 has discussed these questions and came to the following answers and conclusions:

Regarding Q1,

RAN2 notes that WI objective 1 states " The same beam measurement/reporting mechanism will be reused for inter-cell mTRP "). RAN2 would like to understand if the entire inter-cell BM is also applicable to inter-cell mTRP? If not, which part is not applicable to mTRP and how does that work?

Inter-cell beam management is based on the Unified TCI framework. While inter-cell mTRP is based on the legacy Rel-15/Rel-16 TCI framework. Therefore, only beam measurement/reporting mechanisms are common between inter-cell beam management and inter-cell mTRP. The beam indication mechanisms are different between inter-cell beam management and inter-cell mTRP.

Regarding Q2a,

Are UL and DL always processed at the same TRP or can the UE use e.g. serving cell TRP for UL transmissions and TRP with different PCI for DL reception or vice-versa?

This is still an open point in RAN1, contingent on the resolution of the FFS in the following agreement. Note that nothing else is agreed, such restriction doesn’t apply.

Agreement

On Rel.17 beam indication enhancements for inter-cell management, the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) apply to:

- Both joint TCI and separate DL/UL TCI

- FFS: For separate DL/UL TCI, whether the indicated DL TCI and UL TCI are associated with SSBs of a same physical cell ID

Regarding Q2b,

If UE is receiving DL data from TRP with different PCI on dedicated channels, is the UE still able to receive short message (e.g. paging) and system information from serving cell TRP at the same time?

At any one time (e.g., in the same symbol), the UE can only receive channels associated with SSBs of a same physical cell ID. Switching from channels associated with SSBs of one physical cell ID to channels associated with SSBs of another physical cell ID is subject to UE latency requirements for inter-cell beam switching.

Regarding Q2c:

is the UE able to always receive CD-SSB from serving cell TRP when needed and is there any impact to RRM measurements of serving or neighbour cells?

Please see answer to question Q2b for receiving CD-SSB. There is no change in UE capability to perform RRM measurements of serving or neighboring cells.

Regarding Q2d:

Is the number of TRPs involved in the operation restricted to two (i.e. serving cell TRP and TRP with different PCI? Are there any restrictions on TRPs from which UE may send/receive data, or TRPs from which the UE is assumed to be able to make L1 measurements?

RAN1 is still discussing the maximum number of RRC configured PCIs different from the serving cell for measurement and reporting and has made the following agreement

Agreement

On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for inter-cell beam management and inter-cell mTRP, select NMAX (the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting) from the following alternatives (to be decided in RAN1#106bis-e):

- Alt1: NMAX   is up to UE capability with candidate values of 1 and X.

  - Note: X as agreed in AI 8.1.2.2

  - When NMAX is configured to be X, the UE measures up to X PCIs different from the serving cell PCI

  - Additional restriction may be added by RAN4

- Alt2. NMAX=1

The related agreement made in AI 8.1.2.2 (inter-cell mTRP) during RAN1 #106-e is provided as follows.

Agreement

Rel. 17 inter-cell MTRP, the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability

- For the report value of X, multiple candidate values including 1 is supported.

  - FFS: Which values to support other than 1.

  - Values larger than 7 are precluded

  - RAN1 needs to agree on value(s) of X other than 1

- Down-select one of the following alternatives:

  - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity

  - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB

- The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States

For AI 8.1.2.2, i.e., inter-cell mTRP operation, only one additional PCI different from the serving cell PCI can be associated with active TCI state(s) per CC. The related agreement made in RAN1 #104b-e is copied below.

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Regarding Q2e:

Is the inter-cell beam management applicable to any serving cell (i.e. PCell/PSCell/SCell)? That is, can intercell beam management or intercell mTRP be configured for SCell and/or PSCell in addition to PCell?

Yes, as long as for each cell on the serving cell, the corresponding neighboring cell with a different PCI is on the same frequency per the updated WID: “this work shall only consider intra-DU and intra-frequency cases”

Regarding Q2f:

Which signalling should be used for TCI switching for inter-cell beam management?

This based on the unified TCI framework, which uses both MAC-CE based and MAC-CE+DCI based beam indication schemes.

Regarding Q2h (question Q2g is missing!)

Is it correct understanding that such simultaneous Tx/Rx is not supported for “inter-cell beam management”, but is supported for “inter-cell mTRP”? If so, what is the difference regarding their configuration that needs to be introduced by RAN2?

Yes, for inter-cell beam management at any time data can be received from one cell and can be transmitted to one cell. For inter-cell multi-TRP data can be received from multi-cells at the same time (e.g. on different beams) using the legacy Rel-15/Rel-16 TCI framework. To enable the simultaneous Tx/Rx for inter-cell mTRP, each PCI is associated with a different CORESETPoolIndex value. The related agreement made in AI 8.1.2.2 (inter-cell mTRP) during RAN1 #106-e is:

Agreement

- For inter-cell mTRP, one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex

- FFS: The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

Regarding Q3a:

Is it assumed that TA is the same for both serving cell TRP and TRP with different PCI, or does UE maintain different TAs for each?

This is the conclusion in RAN1:

Conclusion

On Rel.17 enhancements for inter-cell beam management,

- In Rel-17, RAN1 cannot reach consensus in supporting same or different TA values across the serving cell and TRPs with different PCIs from that of the serving cell

This implies that TA is left to NW implementation using the current specification. Naturally, this implies that the same TA value is used across cells/TRPs.

Regarding Q3b:

Are there any impacts to RACH operation with inter-cell beam management? That is, is it necessary to perform RACH toward TRP with different PCI e.g. for TA, BFR, etc?

No impact on RACH.

Regarding Q3c:

When UE is configured for TRP with different PCI for a cell with UL, is there an impact to UL power control or PHR?

No impact on power control and PHR beyond what is needed to support multi-TRP in the same cell.

Regarding Q4a:

Is there a single HARQ entity handling both the serving cell TRP and TRP with different PCI?

Yes. Please note that inter-cell beam management for intra-DU scenarios.

Regarding Q4b

Can retransmission occur from different TRP than initial transmission for the same HARQ process? E.g. can initial transmission be done from serving cell TRP and retransmission from TRP with different PCI?

Yes. Please note that inter-cell beam management for intra-DU scenarios.

Regarding Q5a

Does RAN1 assume that only certain parameters can be different from the serving cell and if so, which ones?

No assumptions made in RAN1, other than having a different PCI.

Regarding Q5b

Which RRC configuration(s) need to be provided for inter-cell beam measurement and reporting?

Information about SSBs of neighboring cell with different PCI need to be provided. Further details on RRC configurations will be included in the RRC parameter list.

Regarding Q5c

Are the RRC parameters/configurations different for inter-cell mTRP and inter-cell beam management?

Inter-cell beam management uses the unified TCI framework, inter-cell mTRP uses the legacy Rel-15/Rel-16 TCI framework. RRC parameters for configuring each of these frameworks is different. Further details on RRC configurations will be included in the RRC parameter list.

# Actions

To: RAN4, RAN2 and RAN3

ACTION: 	RAN1 kindly asks RAN2 and RAN4 to take the above into consideration.

# Dates of next TSG WG1 meetings

RAN1#107-e	Novermber 11th –19th, 2021	E-meeting

RAN1#107bis-e	Januray 17th – 25th, 2022	E-meeting