GPP TSG RAN WG1 #106bis-e	        		             R1-2109534

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.4

Source: 	Samsung

Title: 	UE features for NR MBS

Document for:	Discussion and Decision

Introduction

This contribution considers UE features list for Rel-17 MBS.

Discussions

In [1], the preliminary RAN1 UE features list for Rel-17 was discussed.

Mandatory/optional

For FG 33-1, this feature should be optional. Instead, it is proposed to note that this is a basic FG for MBS.

| Features | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. | Optional without capability signalling This is a basic FG for MBS. |

Pre-requisite

For 33-3-2/3/4, pre-requisite can be either 33-1 or 33-2, but should not be both.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1 or 33-2 |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1 or 33-2 |
| 33. NR_MBS | 33-3-4 | Intra-slot TDM-ed multiple unicast PDSCH and/or multiple group-common PDSCH | - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1 or 33-2 |

Adding G-CS-RANTI for FG 33-6-1

For 33-6-1, we assume that activation DCI with G-CS-RNTI would also indicate priority. Then, G-CS-RNTI needs to be added as well.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI and G-CS-RNTI for multicast. |

Conclusions

This contribution considered the UE features list for Rel-17 MBS. The above changes are proposed.

- References

[1] R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO), RAN1#106-e, Aug., 2021.