3GPP TSG RAN WG1 #106bis-e	R1-2109535

# e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.13

Source:	Samsung

Title:	UE features for DSS

Document for:	Discussion and decision

- Introduction

This contribution discusses UE features for Rel-17 DSS WI based on the preliminary Rel-17 UE features [1].

- UE features for DSS

As discussed in our companion Tdoc [2], a Rel-16 UE can support DSS without hardware or specification impact for PDCCH monitoring if the UE is not required to monitor PDCCH on the P(S)Cell and on the sSCell in a same slot (slot is the one of smallest SCS in case of different SCS on P(S)Cell and sSCell); the only feature that the UE needs to support for DSS is scheduling the P(S)Cell from a sSCell. Such a solution can be considered a low-complexity design for DSS operation, which is the purpose for supporting Type-A UEs.

Any other description for a Type-A UE that would lead to an exception for non-simultaneous / TDM’ed PDCCH monitoring on two scheduling cells, such as for non-Type-3 CSS on P(S)Cell, will not constitute a low-complexity design for supporting DSS operation: they will be simply Type-B UEs with certain (arbitrary) restrictions/features without any benefit to UE implementation.

Observation 1: A UE can support DSS based on Rel-16 PDCCH monitoring where the UE does not monitor PDCCH for a scheduled cell from two scheduling cells in a same slot (for smallest SCS). The only requirement for a UE to support DSS is to support a scheduling SCell (sSCell) for the P(S)Cell.

Proposal 1: For Type-A UE, the only requirement is to support a scheduling SCell (sSCell) for the P(S)Cell.

Regarding whether/how to capture Type-A UEs in (RAN1) specification, there is currently no apparent reason to provide specification support for a Type-A UE. UE types may not be specified at all, and a Type-A UE can be a UE that supports DSS but does not support the requirements of a Type-B UE as they will be defined in UE features (according to corresponding specifications). For example, RAN1 specifications can refer to Type-B UEs based on potential related RRC configurations, such as RRC parameters for PDCCH monitoring allocation (0≤α≤1 and 0≤β≤1) among P(S)Cell and sSCell. In that case, RAN1 specifications can refer to Type-A UEs as ones with an sSCell configuration for P(S)Cell scheduling, but lacking configuration for Type-B-related RRC parameters.

Observation 2: RAN1 specifications may not need to refer to DSS UE Types at all.

For cross-carrier scheduling, NR has specified couple of UE capabilities in Rel-15/16 (see the details in Annex B).

- Rel-15: FG 6-10, Cross carrier scheduling for the same numerology

- Rel-16: FG 18-5, DL cross-carrier scheduling with different SCS

- Rel-16: FG 18-5b, UL cross-carrier scheduling with different SCS

In light of reusing existing capability as much as possible, combining above Rel-15/16 capabilities, the resulting Rel-17 UE feature would become as Table 1 below which includes:

- The component description is simplified.

- The description of Type A UE can be clarified further once RAN1 agrees.

- CIF is covered by legacy capabilities and the prerequisite is updated accordingly.

- FFSs are removed.

Proposal 2: Update FG 34-1/34-2 as in Table 1.

Table 1

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B) - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5, 6-10, 18-5, 18-5b |
| 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions (Type A) - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5, 6-10, 18-5, 18-5b |

- Conclusions

Based on above discussion, the observations and proposals are summarized as following:

Observation 1: A UE can support DSS based on Rel-16 PDCCH monitoring where the UE does not monitor PDCCH for a scheduled cell from two scheduling cells in a same slot (for smallest SCS). The only requirement for a UE to support DSS is to support a scheduling SCell (sSCell) for the P(S)Cell.

Proposal 1: For Type-A UE, the only requirement is to support a scheduling SCell (sSCell) for the P(S)Cell.

Observation 2: RAN1 specifications may not need to refer to DSS UE Types at all.

Proposal 2: Update FG 34-1/34-2 as in Table 1.

Table 1

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B) - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5, 6-10, 18-5, 18-5b |
| 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions (Type A) - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5, 6-10, 18-5, 18-5b |

References:

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2109518, Cross-carrier scheduling from SCell to PCell, Samsung

# Annex A: Preliminary RAN1 UE features for Rel-17 DSS [1]

- NR_DSS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 |  |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 |  |

- LTE_NR_DC_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | Temporary RS based SCell activation | SCell activation with the assistance of temporary RS - Temporary RS is aperiodic and triggered by MAC CE. | 6-5 | Yes | N/A |  | Per UE[/Per BC] | No | No |  |  | Optional with capability signalling |

# Annex B: Rel-15/16 UE features with respect to cross-carrier scheduling

Rel-15

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. CA/DC, BWP, SUL | 6-10 | Cross carrier scheduling for the same numerology | Cross carrier scheduling for the same numerology with CIF where numerologies for scheduling cell and scheduled cell are same | 6-5, 6-6 | crossCarrierScheduling-SameSCS | BandNR | n/a | n/a |  |  |

Rel-16

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-5 | DL cross-carrier scheduling with different SCS | 1. The UE supports DL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling CC and scheduled CC are different Candidate value set for component 1: {Scheduling CC of lower SCS and scheduled CC of higher SCS, Scheduling CC of higher SCS and scheduled CC of lower SCS, both} Note: Following components are applicable to CCS from lower SCS to higher SCS when the UE reports FG 18-5 - Processing one unicast DCI scheduling DL per scheduling CC slot per scheduled CC for FDD scheduling CC - Processing one unicast DCI scheduling DL per scheduling CC slot per scheduled CC for TDD scheduling CC Note: Following components are applicable to CCS from higher SCS to lower SCS when the UE reports FG 18-5 - Processing one unicast DCI scheduling DL per N consecutive scheduling CC slot per scheduled CC for FDD scheduling CC - Processing one unicast DCI scheduling DL per N consecutive scheduling CC slot per scheduled CC for TDD scheduling CC - N is based on pair of (scheduling CC SCS, scheduled CC SCS): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) | 6-5 | crossCarrierSchedulingDL-DiffSCS-r16 | CA-ParametersNR-v1610 | n/a | n/a | crossCarrierScheduling-OtherSCS | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-5b | UL cross-carrier scheduling with different SCS | 1. The UE supports UL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in UL carrier aggregation where numerologies for the scheduling CC and scheduled CC are different Candidate value set for component 1: {Scheduling CC of lower SCS and scheduled CC of higher SCS, Scheduling CC of higher SCS and scheduled CC of lower SCS, both} Note: Following components are applicable to CCS from lower SCS to higher SCS when the UE reports FG 18-5b - Processing one unicast DCI scheduling UL per scheduling CC slot per scheduled CC for FDD scheduling CC - Processing 2 unicast DCI scheduling UL per scheduling CC slot per scheduled CC for TDD scheduling CC Note: Following components are applicable to CCS from higher SCS to lower SCS when the UE reports FG 18-5b - Processing one unicast DCI scheduling UL per N consecutive scheduling CC slot per scheduled CC for FDD scheduling CC - Processing 2 unicast DCI scheduling UL per N consecutive scheduling CC slot per scheduled CC for TDD scheduling CC - N is based on pair of (scheduling CC SCS, scheduled CC SCS): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,5), (120,30), N = 8 for (120,15) | 6-6 | crossCarrierSchedulingUL-DiffSCS-r16 | CA-ParametersNR-v1610 | n/a | n/a | crossCarrierScheduling-OtherSCS | Optional with capability signalling |