3GPP TSG RAN WG1 #106bis-e                                                 R1- 2109547

e-Meeting, October 11th – 19th, 2021

Source: MediaTek Inc.

Title: Enhancements on SRS flexibility, coverage and capacity

Agenda Item: 8.1.3

Document for:  Discussion

In this contribution, we provide our views on remaining issues of R17 SRS enhancements.

# Guard period for antenna switching

RAN1#106-e

Agreement

- On the presence of guard symbols in Rel-17 for SRS antenna switching, down-select one of the following

  - Alt 1-0: Guard symbols are always-on, which is same as Rel-15

  - Alt 1-1: Guard symbols are configurable subject to UE capability

- On whether to introduce guard symbols between SRS resource sets for antenna switching, down-select one of the following

  - Alt 2-0: Do not introduce guard symbols between SRS resource sets, i.e., guard symbols only appears between SRS resources in a resource set

  - Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots

- Note: Rel-15 guard period symbols are supported if none of the above enhancements is agreed

First, in R17, SRS resource could start in any symbol in a slot, so specify minimum guard period is required if antenna switching SRS Resource Sets are mapped to consecutive slots (i.e., Alt 2-1). Second, since Rel-15 already had minimum guard symbols requirement for two SRS resources of an SRS resource set, we prefer maintain the same rule to keep it consistent (i.e., Alt 1-0).

Proposal 1:

- Support Alt 1-0: Guard symbols are always-on, which is same as Rel-15

- Support Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots

# RB offset hopping for partial frequency sounding

RAN1#106-e

Agreement

Support start RB location (Noffset) hopping in different SRS frequency hopping periods for RPFS and at least periodic/semi-persistent SRS, where Noffset Noffset  is the start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs.

- For a given SRS transmission occasion, Noffset=kF+khopping mod PFPFmSRS, BSRS , where khopping is same for all SRS occasions within a legacy FH period but changes across legacy FH periods, kF and PF are at least configured by RRC signaling (kF = {0, 1, …, PF-1}).

- Support at least one pattern for khopping in time domain, FFS detailed pattern

- Note: the legacy FH period is the period to sound the full SRS hopping bandwidth across the different subbands of mSRS, BSRS RBs each.

- This start RB location hopping is enabled or disabled by RRC signaling.

- FFS whether MAC CE or DCI can be additionally used

- When this start RB location hopping is disabled,khopping khopping is fixed to be 0 for all SRS symbols

- This start RB location hopping is UE optional.

- FFS whether start RB location hopping is also applicable on SRS occasion(s) within one FH period (e.g., when R>1) and/or on aperiodic SRS, if so, how

In previous meeting, start RB location hopping is agreed for periodic/semi-persistent SRS, and the hopping offset khopping is fixed within a legacy FH period. By this, even with partial frequency sounding, the entire bandwidth can be covered by multiple FH periods.

On the other hand, in aperiodic SRS case the symbols in one SRS resource are unlikely to cover more than one FH period. To make the scheme useful, it can use the repetition symbols (R>1) and apply different offset in each symbol within R symbols. This is illustrated in Figure 1 for PF=4, R=2, Nsymbol=8. From our view, enabling offset hopping in A-SRS is more important since P/SP-SRS has multiple sounding opportunities and it can configure a smaller NBSRS without using partial frequency sounding.

Figure 1. PF=4. RB offset hopping with symbol repetition R=2. Shadowed blocks are active bandwidth for SRS. Offset for symbol keven = 0 and kodd = 2.

To make it work for both intra FH period and inter FH period, the offset khopping can be formulated as

khopping=kintraFH+kinterFH

where kinterFH is fixed within a FH period (as in agreement) and kintraFH is function of mod(l',R) and l' is the symbol index in an SRS resource. Each term can be configured independently.

As for the hopping pattern of kintraFH and/or kinterFH, since only small number PF=2,4 need to consider, a simple table is sufficient. Table 1 is one way of implementation where for kinterFH i is the index for (i+1)-th FH period and for kintraFH i=mod(l',R).

Table 1. RB offset hopping values

| PF | modi, PF=0,…,PF-1 |
| --- | --- |
| 2 | 0, 1 |
| 4 | 0, 2, 1, 3 |

Proposal 2:

- Support RB offset hopping for A-SRS when repetition factor R>1

- RB offset hopping khopping=kintraFH+kinterFH each term corresponds to hopping offset of intra legacy FH period and inter legacy FH period, respectively.

- RB offset hopping is a fixed pattern according to Table 1

# Comb 8

RAN1#106-e

Agreement

For Comb-8 SRS in Rel-17, down-select one of the following in RAN1#106bis-e

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

Comb-8 with nSRScs,max=6 offers maximum 48 orthogonal ports, which is the same as legacy Comb-4 with nSRScs,max=12. From a consideration of available SRS resource, Comb-8 with nSRScs,max=12 can double the totoal number of ports with expense that the resolvable delay window is reduced. Since the main purpose of introducing comb-8 is to increase SRS capacity, nSRScs,max=12 is preferred while reduced delay window should be taken into account and handled in network deployment.

When the sequence length is not multiple of nSRScs,max, the derived sequences by different CSs are not necessary perfectly orthogonal. Moreover, the correlation is notable when the sequence length is short. So a proper rule to limit applicability should be considered. In order to have well correlation separation between any pair (p,q) of ports (derived by different CS), the phase ramping vectors vp and vq need to be orthogonal, where  vp=[1,ejαp,ej2αp, …,  ej(M-1)αp], αp=2πnSRScs,pnSRScs,max, and M is the sequence length. This requires

| vpHvq=n=0M-1ej(αq-αp)n=0 | (1) |
| --- | --- |

Two ways to define the rule are considered below:

- Sequence length condition
Consider configured sequence length M=4m⋅12/(KTCPF) for some integer m (as mSRS,BSRS is multiple of 4), to ensure orthogonal condition (1) holds for all p,q, p≠q, it requires nSRScs,max|M (i.e., M is multiple of nSRScs,max). We note that M is restricted to avaliable lengths defined in current specification. One can check that condition is satisfied for R15 (see in Table 2) by letting PF=1.

Table 2. R15 nSRScs,max values and length condition

| KTC | nSRScs,max | Equivalent condition for nSRScs,max¦M |
| --- | --- | --- |
| 2 | 8 | 8¦24m |
| 4 | 12 | 12¦12m |
| 8 | 6 | 6¦6m |

In R17, we note that this condition may not hold with comb-2/comb-4 since PF can be {2,4}. So we need such a rule even for comb-2/comb-4. For comb-8, the condition can be simplified to PF|m  if  nSRScs,max=6 or 2PF|m  if  nSRScs,max=12.

A simple rule is to restrict that the applicability of nSRScs,max (therefore KTC) be based on length condition nSRScs,max|M hold or not.

Observation 1: when PF>1, pair-wise orthogonality may not be gauranteed for CS nSRScs,p∈{0,…,nSRScs,max-1} even for comb-2/comb-4

- Sub-select CS nSRScs,p∈{0,…,nSRScs,max-1} 
Another way to handle the issue is to choose nSRScs,p for each port, such that for every UE, all pair p,q, p≠q  the condition (1) holds. For example, consider M=6, nSRScs,max=12 for a 4 ports SRS, instead of using nSRScs,p=0,3,6,9, using  nSRScs,p=0,2,4,6 will fulfill the requirement.

Note that with this approach, not all nSRScs,p∈{0,…,nSRScs,max-1} can be jointly use, so resource are not fully utlitized. Furthermore, this approach requires formula change in 38.211.

From above analysis, restriction based on sequence length condition is simple and preferred.

Proposal 3: For the maximum number of CSs for Comb-8, support

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

- The restriction is based on sequence length condition nSRScs,max|M

# Conclusions

Proposal 1:

- Support Alt 1-0: Guard symbols are always-on, which is same as Rel-15

- Support Alt 2-1: Introduce guard symbols between two sets mapped to consecutive slots

Proposal 2:

- Support RB offset hopping for A-SRS when repetition factor R>1

- RB offset hopping khopping=kintraFH+kinterFH each term corresponds to hopping offset of intra legacy FH period and inter legacy FH period, respectively.

- RB offset hopping is a fixed pattern according to Table 1

Observation 1: when PF>1, pair-wise orthogonality may not be gauranteed for CS ∈{0,…,nSRScs,max-1} even for comb-2/comb-4

Proposal 3: For the maximum number of CSs for Comb-8, support

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

- The restriction is based on sequence length condition nSRScs,max|M