3GPP TSG RAN WG1 #106bis-e			R1-2109715

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.9

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NB-IoT and LTE-MTC enhancements

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.17.9 regarding UE features for NB-IoT and LTE-MTC enhancements and captures the following email discussion.

[106bis-e-R17-UE-features-NB-IoT-eMTC-01] Email discussion UE features for NB-IoT and LTE-MTC enhancements – Shinya (DOCOMO) - 1st check point: October 14 - Final check point: October 19

In the preliminary RAN1 UE features list for Rel-17 LTE [1], there are following feature groups for NB-IoT and LTE-MTC enhancements.

- 1-1		16-QAM for unicast NPDSCH

- 1-2		16-QAM for unicast NPUSCH

- 1-3		14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs

- 1-4		A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only

Based on the discussions summarized in Sections 2-4, following is the suggested list of issues to be discussed and priority order considering RAN2 impact especially for capability signaling design, which are tagged and colour coded with High priority, Medium priority, or Low priority.

FL proposal of list of issues/proposals and priority:

- High priority issues (such as a certain FG is necessary or not):

  - Discuss whether to separate the capability for CQI report to support 16-QAM modulation from 1-1

  - Discuss whether FG 1-2 can be kept as “16-QAM for unicast NPUSCH”

  - Discuss whether FG 1-3 can be kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs”

  - Discuss whether FG 1-4 can be kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only”

- Medium priority issues (such as components and type that have capability signaling impacts):

  - Discuss whether separate capabilities are indicated for different deployment mode for FG 1-1

  - Discuss whether the type of FGs 1-1 and 1-2 should be per UE or per band

  - Discuss FDD/TDD differentiation for FGs 1-1 and 1-2

  - Discuss whether/how to support candidate values for reporting for component 3 in FG 1-3

  - Discuss whether the type of FG 1-3 should be per UE or per band

  - Discuss whether the type of FG 1-4 should be per UE or per band

- Low priority issues (such as components that do not have capability signaling impacts)

  - Discuss whether/how to revise any other contents in FGs 1-1 and 1-2 which do not have capability signaling impacts

  - Discuss whether/how to revise the prerequisite feature groups for FG 1-3

  - Discuss whether/how to revise any other contents in FG 1-3 which do not have capability signaling impacts

  - Discuss whether/how to revise the prerequisite feature groups for FG 1-4

  - Discuss whether/how to revise any other contents in FG 1-4 which do not have capability signaling impacts

- 1-1 to 1-2: 16-QAM for unicast NPDSCH/NPUSCH

In [1], FGs 1-1 to 1-2 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-1 | 16-QAM for unicast NPDSCH | 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-2 | 16-QAM for unicast NPUSCH | 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation. | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | Huawei, HiSilicon | For Rel-17 NB-IoT UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH, the need of FDD/TDD differentiation is [Yes] in the preliminary version. Regarding support of 16-QAM, we failed to see the difference between TDD and FDD. Furthermore, for the legacy NB-IoT, the MCS table for unicast NPDSCH and unicast NPUSCH is the same for TDD and FDD. Therefore, the need of FDD/TDD differentiation should be no for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH. Proposal 1: The need of FDD/TDD differentiation should be ‘no’ for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH. |
| --- | --- | --- |
| [3] | ZTE, Sanechips | For the 16-QAM enabling, we actually have the corresponding agreement [1] in RAN1 103e-meeting as following: Agreement For 16-QAM in NB-IoT, separate optional UE capabilities for UL and DL are supported: - The support of 16QAM in DL is indicated by an optional UE capability signaling. - The support of 16QAM in UL is indicated by an optional UE capability signaling.. It indicates that DL 16QAM and UL 16QAM are indicated as the optional UE capability. Observation 1: DL 16QAM and UL 16QAM have been agreed as the optional UE capability respectively. As discussed for 16-QAM, the DL maximum TBS is 4968 btis and the UL maximum TBS is not increased. When 16-QAM feature is enabled, it is assumed all the TBS entries including 2536 btis can be scheduled. Therefore, the R14 feature for larger maximum TBS should be supported by default when 16-QAM is enabled. Proposal 1: If 16QAM is supported, Rel-14 feature of 2536bits should be supported by default. For the power allocation of 16-QAM, the following agreements/work assumptions [2] in RAN1 106e-meeting are achieved. Confirm the working assumption: Working Assumption For the indication of 16-QAM in uplink - The “Modulation and coding scheme” field in DCI Format N0 is utilized as in legacy for scheduling QPSK. - One reserved state in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM. - The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL. Working Assumption For downlink power allocation to support 16QAM: - For inband deployments, a power ratio is signaled in addition to the signalling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”. - the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signalled - the signalling is UE specific Note: “symbols with NRS” and “symbols without NRS nor CRS” have the same power.. For the downlink standalone and guard-band deployments, it is agreed that one power ratio is signaled optionally. For the downlink inband deployments, the work assumption still requires further confirmation. For the uplink, whether/how to introduce dynamic power control and how to introduce open loop power control also need further discussion. From our understanding, the power allocation would be optional for 16-QAM. Proposal 2: Downlink power control are mandatory for 16-QAM. Proposal 3: Uplink power control are optional UE capabilities for 16-QAM. Whether to use the same UE capability to indicate the 16QAM for connected mode and PUR can be discussed. From our perspective, different parameters can used to separately enable the function. When the UE reports that the UE support the 16QAM capability, it means it can be separately enabled in the connected mode or PUR. Therefore, separate UE capability for 16-QAM for connected mode or PUR is not needed. Proposal 4: For the 16QAM related UE capability support, separate UE capability for connected mode and PUR is not needed. For NB-IoT, the legacy features are listed as following. We show our preference in the table 1. Table 1. Example NB-IoT UE features for 16QAM Combine with 16QAM; Note. R14 feature for 2-HARQ processes; support. R14 feature for larger maximum TBS; support; If 16QA is supported, R14 feature for larger maximum TBS should be supported by default.. R14 feature for SC-PTM; Not Support; WID only applied for unicast. R15 feature for Semi-persistent scheduling; Support. R15 feature for EDT; Not support; Agreement For both uplink and downlink - 16-QAM is not applied to C-RNTI from CSS. - 16-QAM is not applied to EDT.. R16 feature for MTB scheduling; Support; Agreement Support 16-QAM for multi-TB scheduling.. R16 feature for PUR; Support. Proposal 7: For the NB-IoT 16QAM features combination - R14 feature for SC-PTM and R15 feature for EDT are not supported for 16-QAM - Other features can be separate and combined enabled together with 16QAM. |
| [4] | Qualcomm | 1. NB_IOTenh4_LTE_eMTC6; 1-1; 16-QAM for unicast NPDSCH; 1. Reception of unicast NPDSCH modulated with 16-QAM 2. Downlink power allocation for 16-QAM; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE; Per Band; N/A; N/A; FFS whether separate capabilities are indicated for different deployment modes.; Optional with capability signaling. 1. NB_IOTenh4_LTE_eMTC6; 1-1a; CQI report to support 16-QAM modulation; 1. CQI report to support 16-QAM modulation; 1-1; Yes; N/A; The network cannot rely on CQI report for scheduling 16-QAM; Per Band; N/A; N/A; Optional with capability sigaling. 1. NB_IOTenh4_LTE_eMTC6; 1-2; 16-QAM for unicast NPUSCH; 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation.; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE; Per Band; N/A; N/A; Optional with capability signaling. |
| [5] | Ericsson | For 16-QAM for unicast in UL and DL: In the column “Need of FDD/TDD differentiation,” before proceeding with a “Yes”, it needs to be verified whether the agreements we have reached to support 16-QAM hold for TDD and what would be the impact in other Working Groups. For 16-QAM for unicast in UL and DL: It seems that from a RAN4 perspective there might be a need to add another Test Model (in the BS conformance specs) and NPDSCH demodulation requirements for supporting 16-QAM in TDD. Discuss whether the agreements we have reached to support 16-QAM in UL and DL hold for TDD and what are the impacts in other Working Groups Decide whether 16-QAM for unicast in UL and DL is applicable for both FDD and TDD |

Discussion

[FL1] High priority question 2-1:

- Companies are encouraged to provide views on whether to separate the capability for CQI report to support 16-QAM modulation from 1-1

| Company | Comment |
| --- | --- |
| Ericsson | The “CQI report” solution is still under discussion, perhaps is better to comeback to “question 2-1” once the solution is known. In our view, in principle the “CQI report” can remain as a component of 16-QAM for unicast NPDSCH. |
| ZTE, Sanechips | No need to separate the capability for CQI report. If 16-QAM is supported and CQI table is not supported, eNB can not appropriately schedule the PDSCH for 16-QAM. Therefore, a basic component of 16-QAM is fine with us. |
| Huawei, HiSilicon | We prefer to keep CQI report as a basic component for 16-QAM. |
| Nokia, NSB | In our view, there is no need to separate capability for CQI reporting from FG 1-1. |
| FL2 | According to the comments provided so far, all companies is fine to include the capability for CQI report to support 16-QAM modulation in FG 1-1. Therefore, following proposal is made to confirm FG 1-1 is kept as “16-QAM for unicast NPDSCH”. [FL2] High priority proposal 2-1: - FG 1-1 is kept as “16-QAM for unicast NPDSCH” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-1; 16-QAM for unicast NPDSCH; 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE; Per UE; [Yes]; N/A; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Ericsson | We are ok with it. |
| ZTE, Sanechips | We are fine with it. |
| Lenovo, MotoM | We are fine with the update from FL2 |
| Qualcomm | Although we would prefer to have a separate capability, we are OK to go with the majority view for the sake of progress. |
| Nokia, NSB | We support the FL proposal. |
| FL3 | Since no concerns/objections have been received so far (more than 24 hours from FL2), the same proposal is set for email endorsement at 1st check point (Oct 14th). High priority proposal 2-1: - FG 1-1 is kept as “16-QAM for unicast NPDSCH” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-1; 16-QAM for unicast NPDSCH; 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE; Per UE; [Yes]; N/A; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| FL4 | Following was agreed at the 1st check point (October 14) High priority proposal 2-1: - FG 1-1 is kept as “16-QAM for unicast NPDSCH” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-1; 16-QAM for unicast NPDSCH; 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE; Per UE; [Yes]; N/A; Optional with capability signaling. Let’s further discuss the contents highlighted in yellow in the next step. |

[FL1] High priority question 2-2:

- Companies are encouraged to provide views on whether FG 1-2 can be kept as “16-QAM for unicast NPUSCH”

| Company | Comment |
| --- | --- |
| Ericsson | It is unclear what is the proposed modification. FG 1-2 seems OK as it is now. |
| ZTE, Sanechips | OK with FG1-2. |
| Huawei, HiSilicon | Fine with FG 1-2. |
| Nokia, NSB | FG 1-2 can be kept as is. |
| FL2 | According to the comments provided so far, all companies is fine to keep FG 1-2 as “16-QAM for unicast NPUSCH”. Therefore, following proposal is made to confirm FG 1-2 is kept as “16-QAM for unicast NPUSCH”. [FL2] High priority proposal 2-2: - FG 1-2 is kept as “16-QAM for unicast NPUSCH” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-2; 16-QAM for unicast NPUSCH; 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation.; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE; Per UE; [Yes]; N/A; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Ericsson | We are Ok with it. |
| ZTE, Sanechips | We are fine. |
| Lenovo, MotoM | We are fine with the proposal in FL2 |
| Nokia, NSB | We support the FL proposal. |
| FL3 | Since no concerns/objections have been received so far (more than 24 hours from FL2), the same proposal is set for email endorsement at 1st check point (Oct 14th). High priority proposal 2-2: - FG 1-2 is kept as “16-QAM for unicast NPUSCH” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-2; 16-QAM for unicast NPUSCH; 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation.; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE; Per UE; [Yes]; N/A; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| FL4 | Following was agreed at the 1st check point (October 14) High priority proposal 2-2: - FG 1-2 is kept as “16-QAM for unicast NPUSCH” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-2; 16-QAM for unicast NPUSCH; 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation.; Category NB-2; Yes; N/A; The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE; Per UE; [Yes]; N/A; Optional with capability signaling. Let’s further discuss the contents highlighted in yellow in the next step. |

[FL1] Medium priority question 2-3:

- Companies are encouraged to provide views on whether separate capabilities are indicated for different deployment mode for FG 1-1

| Company | Comment |
| --- | --- |
| Ericsson | We think it is not necessary to break-down 16-QAM in DL into separate capabilities for each deployment mode. |
| ZTE, Sanechips | Not necessary. |
| Huawei, HiSilicon | We don’t it’s needed to have separate capabilities for different deployment modes. |
| Nokia, NSB | It is not necessary to have separate capabilities for different deployment modes. |
| FL2 | According to the comments provided so far, all companies don’t see the necessity to have separate capabilities for different deployment mode for FG 1-1. Therefore, following proposal is made. [FL2] Medium priority proposal 2-3: - For FG 1-1, separate capabilities are not indicated for different deployment mode |
| Ericsson | We are Ok with the proposal. |
| ZTE, Sanechips | We are OK with it. |
| Lenovo, MotoM | We are fine with proposal in FL2 |
| Qualcomm | We think it would be useful to have it per deployment mode for IODT purposes (the MCS table will be different for different deployment modes). A possible compromise is to make it “mandatory with capability signaling”, such that if the UE supports one deployment mode it supports all, but we keep IODT bits for different deployment modes. |
| Nokia, NSB | We support the FL proposal. Regarding Qualcomm’s proposal above we don’t see any difference in practice, so we don’t see that as a compromise, at least based on the short explanation above. |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| FL4 | Proposal is updated based on the comment from Qualcomm [FL4] Medium priority proposal 2-3: - For FG 1-1, select one of the following options: - Option 1: Separate capabilities are not indicated for different deployment mode - Option 2: FG 1-1 is supported as mandatory with capability signaling |
| Ericsson v013 | To Moderator: Option 2 is confusing and seems to touch upon the last column, which is not under FFS. It seems that the options under down-selection should be: - Option 1: Separate capabilities are not indicated for different deployment mode - Option 2: Separate capabilities are indicated for different deployment mode |
| Qualcomm | Our preference is to have separate capabilities for different deployment modes for IODT. |
| Nokia, NSB | Option 2 is very confusing indeed, not sure what is being asked. Hence, our comments from last round remain valid. |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study whether separate capabilities are indicated for different deployment mode for FG 1-1 toward the next RAN1 meeting considering the comments provided so far. |

[FL1] Medium priority question 2-4:

- Companies are encouraged to provide views on whether the type of FGs 1-1 and 1-2 should be per UE or per band

| Company | Comment |
| --- | --- |
| Ericsson | We would like to understand better and know more about the motivation and advantages of performing such a modification. It seems that the proposal of using a type “per band” was brought up within the context of 5G, which make us think that perhaps the motivation is different than in LTE-MTC/NB-IoT. |
| ZTE, Sanechips | Similar with legacy R15/R16 features, per UE is preferred. |
| Huawei, HiSilicon | We have no strong view, but would also like to understand the motivation of per band. |
| Nokia, NSB | We feel this should be per UE, further justification is needed to make it per band. |
| FL2 | According to the comments provided so far, all companies are fine to keep the type of FGs 1-1 and 1-2 as per UE. Therefore, following proposal is made. [FL2] Medium priority proposal 2-4: - Type of FGs 1-1 and 1-2 are per UE |
| Ericsson | Since no explanation/motivation has been provided to do something different compared to previous releases, we are Ok with the proposal |
| ZTE, Sanechips | We are OK with it. |
| Lenovo, MotoM | Per UE is preferred, we are OK with proposal in FL2. |
| Qualcomm | We think it would be important to keep the feature per band. With the introduction of IOT NTN, it is expected that most of the terrestrial features would also apply to NTN. Additionally, the support of 16-QAM is RF-dependent, so it is possible the UE can support the EVM/emission requirements in low bands but not in higher bands (e.g. 2GHz vs 700MHz) |
| Nokia, NSB | We support FL proposal. Introduction of IOT NTN has no impact on the decision of the reporting type for these features. This issue has been addressed already in Rel-16 regarding NR-U, and same solutions can be applied here, e.g. indicate to RAN2 we need a differentiation for IOT NTN. Feedback overhead and network complexity should not be overlooked by RAN1. |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| FL4 | Proposal is updated based on the comment from Qualcomm [FL4] Medium priority proposal 2-4: - For FGs 1-1 and 1-2, select one of the following options: - Option 1: type of FGs 1-1 and 1-2 are per UE - Option 2: type of FGs 1-1 and 1-2 are per band |
| Ericsson v013 | If “With the introduction of IOT NTN, it is expected that most of the terrestrial features would also apply to NTN” and that is the motivation to define the type “per band”. How about legacy features that are already “per UE”? |
| Qualcomm | For legacy features, we would need to discuss them one by one, and decide to add new capabilities for NTN IOT (similar to what was done, for example, for unlicensed in Rel-16). But this can be discussed under NTN IOT feature list discussion. What we can discuss here is that the new features should take this into account, and make them “per band”. |
| Nokia, NSB | We do not support this discussion on making features “per band” for the sake of NTN IOT differentiation at all. In our view this is not a motivation to entertain adding this signaling overhead. Hence we support Option 1 only. |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study appropriate granularity toward the next RAN1 meeting considering the comments provided so far. |

[FL1] Medium priority question 2-5:

- Companies are encouraged to provide views on FDD/TDD differentiation for FGs 1-1 and 1-2

| Company | Comment |
| --- | --- |
| Ericsson | In our view it is important that operators, UE vendors, Network vendors and other interested parties discuss the relevance of supporting 16-QAM for TDD NB-IoT, especially if supporting it results in an extra impact in any of the Working Groups involved. It seems that from a RAN4 perspective there might be a need to add another Test Model (in the BS conformance specs) and NPDSCH demodulation requirements for supporting 16-QAM in TDD. |
| ZTE, Sanechips | From RAN1 perspective, we do not see the need to differentiate TDD and FDD. |
| Huawei, HiSilicon | 16-QAM can be supported for both TDD and FDD, and there’s no need to differentiate TDD and FDD. |
| Nokia, NSB | In our understanding the feature can be supported for both FDD and TDD, no clear need for differentiation. |
| FL2 | According to the comments provided so far, almost all companies don’t think FDD/TDD differentiation is necessary for FGs 1-1 and 1-2. Therefore, following proposal is made. [FL2] Medium priority proposal 2-5: - FDD/TDD differentiation is not necessary for FGs 1-1 and 1-2 |
| Ericsson | Companies have expressed that “16-QAM can be supported for both TDD and FDD”, however we haven’t had a discussion on the support of TDD and it might have impacts in other Working Groups where the support of TDD might not be transparent. For example, we have pointed out that the support for TDD might require to add another Test Model (in the BS conformance specs) and NPDSCH demodulation requirements for supporting 16-QAM in TDD (We can perhaps ask RAN4 about any foreseen impact). Moreover, before touching upon this aspect, we have encouraged operators, UE vendors, Network vendors and other interested parties to discuss the relevance of supporting 16-QAM for TDD NB-IoT, especially if supporting it might result in extra impacts in other Working Groups. Thus, this can kept as FFS. |
| ZTE, Sanechips | Maybe we can add the “from RAN1 perspective” to address the concern from Ericsson. |
| Lenovo, MotoM | Issues are not found to support proposal in FL2, but we can keep FFS and wait the view from other WGs as E/// comments |
| Qualcomm | We propose to have this feature per band, so this is not needed. |
| Nokia, NSB | We are OK to keep this as FFS until there is more clarity from other WGs. |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| FL4 | This proposal can be discussed after some progress is made for proposal 2-4 |
| Ericsson v013 | We are OK to keep this as FFS. |

[FL2] Low priority question 2-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 1-1 and 1-2 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Ericsson | No, until new agreements will be reached. |
| Nokia, NSB | No revision needed until new agreements are made. |

- 1-3: 14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs

In [1], FG 1-3 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the Ues)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-3 | 14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues | 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and Alt-2e | 1. Category M1 2. HD-FDD 3. CE Mode A 4. Support of 10 DL HARQ processes in HD-FDD in CE mode A 5. HARQ-ACK bundling support in HD-FDD in CE mode A | Yes | N/A | The network cannot enable 14 HARQ processes for the UE | Per UE | FDD only | N/A | - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”. | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [3] | ZTE, Sanechips | For the 14-HARQ processes, the following agreement [1] in RAN1 103e-meeting is achieved. Agreement The following working assumption is confirmed Introduce a new optional UE capability to support 14 HARQ processes. Observation 2: 14-HARQ processes has been agreed as the optional UE capability. Also, for the 14-HARQ process introduction, it is agreed that the scheduling delay and HARQ-delay design would be considered. However, it seems that no need to introduce the corresponding UE capability, since the 14-HARQ processes feature is not workable if the scheduling delay and HARQ-delay design are not introduced. Proposal 5: Scheduling delay and HARQ-delay design are mandatory for 14-HARQ processes feature. Both R17 features 14-HARQ processes and 1736bits are MTC enhancements. We can discuss them together. As the following table, we show our preference for the feature combination. Table 2. Example MTC UE features for 14-HARQ processes Combine with 14HARQ-processes; Note. Rel-14 feature for new numbers of repetitions for PUSCH in CE mode A; Support. Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A; Support. Rel-14 feature on HARQ-ACK bundling in HD-FDD in CE mode A; Support; Bundling was supported for FDD case, HD-FDD case can be discussed in R17. Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B; Support. Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A; Support; When 14-HARQ processes is enabled, this feature should be supported by default.. Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A; Not support; New HARQ delay is designed and the function is overlapped.. Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B; Support. Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A; Support. Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B; Not support; If only one repetition is supported, there is no need to early terminate.. Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B; Support. Rel-16 features for multiple TB scheduling; Not support; Conclusion In Rel-17, the 14 HARQ processes feature is not supported when the multi-TB grant feature is enabled.. Rel-16 feature for PUR; Not support; 14-HARQ processes feature is only for downlink.. Rel-16 feature for resource reservation; Support. Proposal 8: For 14-HARQ processes feature combination, - Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD is not needed. - Rel-15 feature for uplink HARQ-ACK feedback is not supported. - Rel-16 feature for PUR is not supported. - Rel-14 feature for 10 downlink HARQ processes in FDD is supported by default if the 14-HARQ processes is configured. |
| --- | --- | --- |
| [4] | Qualcomm | 1. NB_IOTenh4_LTE_eMTC6; 1-3; 14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues; 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and/or Alt-2e; 1. Category M1 2. HD-FDD 3. CE Mode A [4. Support of 10 DL HARQ processes in HD-FDD in CE mode A] [5. HARQ-ACK bundling support in HD-FDD in CE mode A]; Yes; N/A; The network cannot enable 14 HARQ processes for the UE; Per Band; FDD only; N/A; - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.; Optional with capability signaling Values (which apply to support of component 5): {alt1, alt2-e, both}. |
| [5] | Ericsson | For the 14 HARQ processes feature: The “Prerequisite feature groups” column has listed: “CE mode A”. However, CE mode A support is already implied by Category M1 support, hence we suggest removing “CE mode A” as it has been done for “UE feature lists” of previous releases (See e.g., [3] and [4]). For the 14 HARQ processes feature: Remove from the “Prerequisite feature groups” column the following listed item: “CE Mode A”. For the 14 HARQ processes feature: The “Prerequisite feature groups” column has listed: “4. Support of 10 DL HARQ processes in HD-FDD in CE mode A” and “5. HARQ-ACK bundling support in HD-FDD in CE mode A”. However, deployment-wise it is not straightforward to implement and test in live networks “4. And 5.” As separate Rel-14 features, hence “.4 and .5” should not be pre-requisites for the new 14 HARQ processes feature. For the 14 HARQ processes feature: Remove from the “Prerequisite feature groups” column the following listed items: “4. Support of 10 DL HARQ processes in HD-FDD in CE mode A” and “5. HARQ-ACK bundling support in HD-FDD in CE mode A”. |

Discussion

[FL1] High priority question 3-1:

- Companies are encouraged to provide views on whether FG 1-3 can be kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues”

| Company | Comment |
| --- | --- |
| Ericsson | Yes. |
| ZTE, Sanechips | Yes |
| Huawei, HiSilicon | Yes |
| Nokia, NSB | FG 1-3 can be kept as is. |
| FL2 | According to the comments provided so far, all companies is fine to keep FG 1-3 as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues”. Therefore, following proposal is made to confirm FG 1-3 is kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues”. [FL2] High priority proposal 2-2: - FG 1-3 is kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-3; 14 HARQ processes for PDSCH for HD-FDD Cat. M1 Ues; 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and Alt-2e; 1. Category M1 2. HD-FDD 3. CE Mode A 4. Support of 10 DL HARQ processes in HD-FDD in CE mode A 5. HARQ-ACK bundling support in HD-FDD in CE mode A; Yes; N/A; The network cannot enable 14 HARQ processes for the UE; Per UE; FDD only; N/A; - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Ericsson | We are ok with it. |
| ZTE, Sanechips | We are fine. |
| Lenovo, MotoM | We are fine with proposal in FL2 |
| Qualcomm | We are OK |
| Nokia, NSB | We support FL proposal. |
| FL3 | Since no concerns/objections have been received so far (more than 24 hours from FL2), the same proposal is set for email endorsement at 1st check point (Oct 14th). High priority proposal 2-2: - FG 1-3 is kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-3; 14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs; 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and Alt-2e; 1. Category M1 2. HD-FDD 3. CE Mode A 4. Support of 10 DL HARQ processes in HD-FDD in CE mode A 5. HARQ-ACK bundling support in HD-FDD in CE mode A; Yes; N/A; The network cannot enable 14 HARQ processes for the UE; Per UE; FDD only; N/A; - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| FL4 | Following was agreed at the 1st check point (October 14) High priority proposal 2-2: - FG 1-3 is kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-3; 14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs; 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and Alt-2e; 1. Category M1 2. HD-FDD 3. CE Mode A 4. Support of 10 DL HARQ processes in HD-FDD in CE mode A 5. HARQ-ACK bundling support in HD-FDD in CE mode A; Yes; N/A; The network cannot enable 14 HARQ processes for the UE; Per UE; FDD only; N/A; - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.; Optional with capability signaling. Let’s further discuss the contents highlighted in yellow in the next step. (I notice there is a typo in the proposal index, 2-2 is duplicated, it should be 3-1) |

[FL1] Medium priority question 3-2:

- Companies are encouraged to provide views on whether/how to support candidate values for reporting for component 3 in FG 1-3, e.g.

  - {alt1, alt2-e, both}

| Company | Comment |
| --- | --- |
| Ericsson | We are fine, just to avoid misunderstandings we should replace the word “values” by “solutions”. That is: ValuesSolutions (which apply to support of component 5): {alt1, alt2-e, both} |
| ZTE, Sanechips | We are fine with {alt1, alt2-e, both} |
| Huawei, HiSilicon | We are fine with {Alt1, alt2-e, both} |
| Nokia, NSB | We don’t see a need for reporting candidate values, as we think both should be the default support for the feature. |
| FL2 | According to the comments provided so far, almost all companies are fine with {alt1, alt2-e, both} for the candidate solutions for component 3 in FG 1-3. Therefore, following proposal is made. [FL2] Medium priority proposal 3-2: - For component 3 in FG 1-3, UE reports one of {alt1, alt2-e, both} |
| Ericsson | We are ok with the proposal of the FL in view of the IODT phase. |
| ZTE, Sanechips | We are fine with it. |
| Lenovo, MotoM | We share the similar view as Nokia. There is no need to report for the feature. There is no difference to support Alt1 and Alt2-e for UE capability. |
| Qualcomm | We agree with FL2. |
| Nokia, NSB | We do not support reporting of candidate values for component 3 |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| FL4 | Proposal is updated based on the comments provided so far [FL4] Medium priority proposal 3-2: - For component 3 in FG 1-3, select one of the following options: - Option 1: UE reports one of {alt1, alt2-e, both} - Option 2: UE supports both Alt-1 and Alt-2e |
| Ericsson v013 | As we expressed it earlier, we are fine with Option-1 in view of the IODT phase. |
| Qualcomm | OK with Option 1 |
| FL5 | No concern/objection has been received so far. Therefore, the same proposal is set for email endorsement Medium priority proposal 3-2: - For component 3 in FG 1-3, select one of the following options: - Option 1: UE reports one of {alt1, alt2-e, both} - Option 2: UE supports both Alt-1 and Alt-2e |
| Nokia, NSB | We support Option 2. |
| FL | Following was agreed at the final check point (October 19) Agreement - For component 3 in FG 1-3, select one of the following options: - Option 1: UE reports one of {alt1, alt2-e, both} - Option 2: UE supports both Alt-1 and Alt-2e |

[FL1] Medium priority question 3-3:

- Companies are encouraged to provide views on whether the type of FG 1-3 should be per UE or per band

| Company | Comment |
| --- | --- |
| Ericsson | Same comment as in question 2-4. |
| ZTE, Sanechips | Similar as above question 2-4, per UE is preferred. |
| Huawei, HiSilicon | Same comment as 2-4. |
| Nokia, NSB | Same view as in question 2-4. |
| FL2 | According to the comments provided so far, all companies are fine to keep the type of FG 1-3 as per UE. Therefore, following proposal is made. [FL2] Medium priority proposal 3-3: - Type of FG 1-3 is per UE |
| Ericsson | Same comment as in question 2-4, therefore we are Ok with the proposal. |
| ZTE, Sanechips | OK with it. |
| Lenovo, MotoM | per UE is preferred. We are fine with proposal in FL2 |
| Qualcomm | Similar to the 16-QAM discussion, we think “per band” is preferred due to the possibility of this feature to apply to IOT NTN. |
| Nokia, NSB | We support FL proposal. As said before, introduction of IOT NTN has no impact on the decision of the reporting type for these features. This issue has been addressed already in Rel-16 regarding NR-U, and same solutions can be applied here, e.g. indicate to RAN2 we need a differentiation for IOT NTN. Feedback overhead and network complexity should not be overlooked by RAN1. |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| FL4 | Proposal is updated based on the comment from Qualcomm [FL4] Medium priority proposal 3-3: - For FG 1-3, select one of the following options: - Option 1: type of FG 1-3 is per UE - Option 2: type of FG 1-3 is per band |
| Ericsson v013 | Same comment as in question 2-4 (Ericsson v013) |
| Qualcomm | Per band, due to NTN issues (as mentioned above) |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study appropriate granularity toward the next RAN1 meeting considering the comments provided so far. |

[FL2] Low priority question 3-4:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 1-3

| Company | Comment |
| --- | --- |
| Ericsson | We do not see this topic as low priority since it has impacts on Inter-Operability Device Testing (IODT). We basically suggest for the 14 HARQ processes feature to remove from the “Prerequisite feature groups” column the following listed items: “4. Support of 10 DL HARQ processes in HD-FDD in CE mode A” and “5. HARQ-ACK bundling support in HD-FDD in CE mode A”. We also suggest removing from the “Prerequisite feature groups” column the following listed item: “CE Mode A”, since CE mode A support is already implied by Category M1 support, (this has been done for “UE feature lists” of previous releases (See e.g., [3] and [4] in R1-2110319). |
| FL4 | Following proposal is made based on the comments provided so far [FL4] Low priority proposal 3-4: - Following features are removed from the prerequisite feature groups for FG 1-3 - CE Mode A - Support of 10 DL HARQ processes in HD-FDD in CE mode A - HARQ-ACK bundling support in HD-FDD in CE mode A |
| Ericsson | We support FL4 proposal 3-4 |
| Qualcomm | OK |
| FL5 | No concern/objection has been received so far. Therefore, the same proposal is set for email endorsement Low priority proposal 3-4: - Following features are removed from the prerequisite feature groups for FG 1-3 - CE Mode A - Support of 10 DL HARQ processes in HD-FDD in CE mode A - HARQ-ACK bundling support in HD-FDD in CE mode A |
| FL | Following was agreed at the final check point (October 19) Agreement - Following features are removed from the prerequisite feature groups for FG 1-3 - CE Mode A - Support of 10 DL HARQ processes in HD-FDD in CE mode A - HARQ-ACK bundling support in HD-FDD in CE mode A |

[FL2] Low priority question 3-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 1-3 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Ericsson | No for the moment. |

- 1-4: A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only

In [1], FG 1-4 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the Ues)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-4 | A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only | 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED | 1. Category M1 2. HD-FDD 3. CE Mode A | Yes | N/A | The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues | Per UE | FDD only | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [2] | Huawei, HiSilicon | For Rel-17 LTE-MTC UE feature A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only, we do not see any difference between unicast PDSCH and PUR PDSCH, regarding the use of maximum DL TBS of 1736 bits. Similarly, the UE capabilities for combining max DL TBS and other features are not needed either. Therefore, we have the following proposal. Proposal 2: UE capability for combination of max DL TBS and other features is not needed. |
| --- | --- | --- |
| [3] | ZTE, Sanechips | Regarding the maximum 1736btis, there are some agreements [3] in 104e-meeting. Agreement The 1736 bits DL TBS feature is enabled by unicast RRC configuration.. The above indicates that the 1736 bits DL TBS feature can be enabled. It is nature to assume that this feature also can be disabled. Therefore, it is an optional UE capability. Proposal 6: 1736 bits DL TBS is an optional UE feature. For feature maximum 1736bits, seems no compatible issues are seen and following conclusion is made Conclusion “NOTE: It is RAN1 assumption that 1736 DL TBS feature is compatible with all other eMTC features applicable for HD-FDD Cat. M1 Ues in CE mode A. It is assumed that there’s no change to DCI formats, TBS tables and CQI tables.”. It is worth to mention that multicast and PUR also supports the 1736 bits feature according to the above conclusion. Moreover, a contribution [4] from us in 105e-meeting mentioned that and the corresponding discussion summary [5] indicates that is the consensus. Observation 3: Multicast and PUR can support the 1736 bits feature. |
| [4] | Qualcomm | 1. NB_IOTenh4_LTE_eMTC6; 1-4; A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only; 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED; 1. Category M1 2. HD-FDD 3. CE Mode A; Yes; N/A; The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues; Per Band; FDD only; N/A; Optional with capability signaling. |
| [5] | Ericsson | For the max DL TBS of 1736 bits feature: The “Prerequisite feature groups” column has listed: “CE mode A”. However, CE mode A support is already implied by Category M1 support, hence we suggest removing “CE mode A” as it has been done for “UE feature lists” of previous releases (see e.g., [3] and [4]). For the max DL TBS of 1736 bits feature: Remove from the “Prerequisite feature groups” column the following listed item: “CE Mode A”. For the max DL TBS of 1736 bits feature: It is worth noting that the support of a maximum DL TBS of 1736 bits for HD-FDD Ues in CE Mode A refers to connected mode only (See “Components” column). In our view, it seems sufficient in principle to support the larger DL TBS for connected mode features. For the max DL TBS of 1736 bits feature: Clarify whether the conclusion “It is RAN1 assumption that 1736 DL TBS feature is compatible with all other eMTC features applicable for HD-FDD Cat. M1 Ues in CE mode A” is meant to apply for “connected-mode” features or both “connected-mode and idle-mode” features. |

Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether FG 1-4 can be kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only”

| Company | Comment |
| --- | --- |
| Ericsson | Yes. |
| ZTE, Sanechips | Yes |
| Huawei, HiSilicon | Yes |
| FL2 | According to the comments provided so far, all companies is fine to keep FG 1-4 as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only”. Therefore, following proposal is made to confirm FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only”. [FL2] High priority proposal 4-1: - FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-4; A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only; 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED; 1. Category M1 2. HD-FDD 3. CE Mode A; Yes; N/A; The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues; Per UE; FDD only; N/A; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Ericsson | We are Ok with it. |
| ZTE, Sanechips | OK with it. |
| Lenovo, MotoM | We are OK |
| FL3 | Since no concerns/objections have been received so far (more than 24 hours from FL2), the same proposal is set for email endorsement at 1st check point (Oct 14th). High priority proposal 4-1: - FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-4; A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only; 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED; 1. Category M1 2. HD-FDD 3. CE Mode A; Yes; N/A; The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues; Per UE; FDD only; N/A; Optional with capability signaling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| FL4 | Following was agreed at the 1st check point (October 14) High priority proposal 4-1: - FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only” as follows 1. NB_IOTenh4_LTE_eMTC6; 1-4; A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only; 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED; 1. Category M1 2. HD-FDD 3. CE Mode A; Yes; N/A; The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues; Per UE; FDD only; N/A; Optional with capability signaling. Let’s further discuss the contents highlighted in yellow in the next step. |

[FL1] Medium priority question 4-2:

- Companies are encouraged to provide views on whether the type of FG 1-4 should be per UE or per band

| Company | Comment |
| --- | --- |
| Ericsson | Same comment as in question 2-4. |
| ZTE, Sanechips | Similar as above question 2-4, per UE is preferred. |
| Huawei, HiSilicon | Same comment as 2-4. |
| FL2 | According to the comments provided so far, all companies are fine to keep the type of FG 1-4 as per UE. Therefore, following proposal is made. [FL2] Medium priority proposal 4-2: - Type of FG 1-4 is per UE |
| Ericsson | Same comment as in question 2-4, therefore we are Ok with the proposal. |
| ZTE, Sanechips | OK with it. |
| Lenovo, MotoM | We are OK |
| Qualcomm | Similar comment as above, we would like to have a differentiation for IOT NTN, which makes “per band” a better approach. |
| Nokia, NSB | OK with it. Similar comment as above regarding IOT NTN. It is sufficient to indicate to RAN2 we need a differentiation for IOT NTN. |
| FL3 | Given that companies still have different view, this proposal is not set for email endorsement at 1st check point (Oct 14th). Moderator will update the proposal in the next round discussion |
| FL4 | Proposal is updated based on the comment from Qualcomm [FL4] Medium priority proposal 4-2: - For FG 1-4, select one of the following options: - Option 1: type of FG 1-4 is per UE - Option 2: type of FG 1-4 is per band |
| Ericsson v013 | Same comment as in question 2-4 (Ericsson v013). |
| Qualcomm | Per band due to NTN issues (same as above) |
| FL5 | Given that companies have different view and considering the remaining time in this meeting, no additional proposal is made. Companies are encouraged to study appropriate granularity toward the next RAN1 meeting considering the comments provided so far. |

[FL2] Low priority question 4-3:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 1-4

| Company | Comment |
| --- | --- |
| Ericsson | We suggest removing from the “Prerequisite feature groups” column the following listed item: “CE Mode A”, since CE mode A support is already implied by Category M1 support, (this has been done for “UE feature lists” of previous releases (See e.g., [3] and [4] in R1-2110319). RAN2 has agreed to support PUR, so it should be added as a component of FG 1-4. |
| FL4 | Following proposals are made based on the comments provided so far [FL4] Low priority proposal 4-3a: - CE Mode A is removed from the prerequisite feature groups for FG 1-4 [FL4] Low priority proposal 4-3b: - PUR is added as a component of FG 1-4 Companies are also encouraged to provide text proposal for PUR to be captured in the column of components |
| Ericsson v013 | We support proposal 4-3a, whereas proposal 4-3b for the moment can be kept as FFS to think a bit more on the “text proposal for PUR to be captured in the column of components”. |
| FL5 | No concern/objection has been received at least for proposal 4-3a. Therefore, following proposal is set for email endorsement Low priority proposal 4-3a: - CE Mode A is removed from the prerequisite feature groups for FG 1-4 |
| FL | Following was agreed at the final check point (October 19) Agreement - CE Mode A is removed from the prerequisite feature groups for FG 1-4 |

[FL2] Low priority question 4-4:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 1-4 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Ericsson | No for the moment. |

- Conclusions

Following agreements were made in this RAN1 meeting:

Agreement

FG 1-3 is kept as “14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-3 | 14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs | 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and Alt-2e | 1. Category M1 2. HD-FDD 3. CE Mode A 4. Support of 10 DL HARQ processes in HD-FDD in CE mode A 5. HARQ-ACK bundling support in HD-FDD in CE mode A | Yes | N/A | The network cannot enable 14 HARQ processes for the UE | Per UE | FDD only | N/A | - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 1-1 is kept as “16-QAM for unicast NPDSCH” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-1 | 16-QAM for unicast NPDSCH | 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 1-2 is kept as “16-QAM for unicast NPUSCH” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-2 | 16-QAM for unicast NPUSCH | 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation. | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-4 | A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only | 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED | 1. Category M1 2. HD-FDD 3. CE Mode A | Yes | N/A | The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues | Per UE | FDD only | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

- For component 3 in FG 1-3, select one of the following options:

  - Option 1: UE reports one of {alt1, alt2-e, both}

  - Option 2: UE supports both Alt-1 and Alt-2e

Agreement

- Following features are removed from the prerequisite feature groups for FG 1-3

  - CE Mode A

  - Support of 10 DL HARQ processes in HD-FDD in CE mode A

  - HARQ-ACK bundling support in HD-FDD in CE mode A

Agreement

- CE Mode A is removed from the prerequisite feature groups for FG 1-4

References

[1]	R1-2108678	Preliminary RAN1 UE features list for Rel-17 LTE	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2109151	Rel-17 UE features for NB-IoT and LTE-MTC enhancements	Huawei, HiSilicon

[3]	R1-2109340	Discussion on LTE-M and NB-IoT UE features	ZTE, Sanechips

[4]	R1-2110227	UE features for NB-IoT and LTE-MTC enhancements	Qualcomm Incorporated

[5]	R1-2110319	On UE features for NB-IoT and LTE-MTC enhancements	Ericsson