3GPP TSG RAN WG1#106-bis-e      	        	                                   R1-2109774    E-meeting October.11th-19th, 2021

Agenda Item:					    8.1.2.3

Source:	Sony

Title:	Enhancements on beam management for multi-TRP

Document for:	Discussion and decision

# Introduction

In RANP#86 meeting, the WID on beam management for multi-TRP operation (highlighted) was established as below

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In RAN1#102e, both high priority and low priority issues were identified in following agreement. In this contribution, we present our views on the high priority issues, i.e. beam management enhancement for M-TRP and beam failure recovery for M-TRP.

Agreement Study Rel.17 enhancements on beam management for multi-TRPs with following priority - High priority: - Beam measurement/reporting enhancement - Beam failure recovery for multi-TRP - Low priority - Simultaneous reception of same type of channel/RS with different QCL-TypeD Simultaneous reception of different type of channel/RS with different QCL-TypeD

# Group-based beam reporting for multi-TRP

In Rel.15/16, the group-based beam reporting was supported with L (= 1) group and up to Q (= 2) Tx beams per group. Literally, the reported group-based beams could be simultaneously received by UE and therefore it facilitates DL/UL multi-beam indication when multi-TRP/panel are deployed. However, it doesn’t put any constraint on whether the two reported beams come from either a single TRP or separated TRPs. In addition, there is no information from NW side on whether the reported Q beams can be transmitted by gNB simultaneously or not.

- : the group-based beam reporting for multi-TRP

- In Rel.15/16, the group-based beam reporting was not tailored for multi-TRP operation as illustrated in Figure 1.

In Rel.15/16, the specified group-based beam reporting (only one beam group and up to 2 Tx beams per group) seems not flexible enough in the sense of NW scheduling. The reason lies in the fact that NW is only aware of single DL Tx beam group from UE, thereby the choice for selecting multiple DL Tx beams from different TRPs is very limited.

- In Rel.15/16, the group-based beam reporting (only single group and up to 2 Tx beams per group) seems too restrictive for multi-TRP operation.

## Options (Other than Option2)

In RAN1#104e, 3 options of group-based beam reporting were thoroughly analyzed and discussed. Finally, Option 2 was agreed considering its easy usage at NW side, and Option 1 and Option 3 were under FFS as in following agreements.

Agreement (RAN1#103e) Down-select at least one of the following options for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing in RAN1 #104-e - Option 1: In a CSI-report, UE can report N>1 pair/groups and M>=1 beams per pair/group - Different beams in different pairs/groups can be received simultaneously - FFS: whether M is equal or can be different across different pair/group - Option 2: In a CSI-report, UE can report N(N>=1) pairs/groups and M (M>1) beams per pair/group - Different beams within a pair/group can be received simultaneously - Option 3: UE report M(M>=1) beams in N (N>1) CSI-reports corresponding to N report setting - Different beams in different CSI-reports can be received simultaneously - FFS: whether/how to introduce an association between different CSI-reports - FFS: whether/how to differentiate reported measurements for beams that are received simultaneously vs. beams that are not received simultaneously - Whether/how to introduce an indication along with the CSI-reports to indicate whether the beams in different CSI-reports can be received simultaneously - FFS: value of N and M in each option - FFS: Association between different beams in above options and different TRP/UE panels - FFS: Identify new use cases per option compared with R16 (including backhaul) - FFS: whether different beams in different pairs/groups/reports can be received by same spatial filter per option Agreement (RAN1#104e) For beam measurement in support of M-TRP simultaneous transmission - Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously - Support M = 2 - Support extending the maximum value of N > 1, exact value FFS - N=1 and N=2 - FFS: Other values larger than 2 - FFS: Whether the UE could report beams are received with different RX beams - Further study the support of option 1 and option 3 - The above applies at least for L1-RSRP - FFS: L1-SINR

In our view, the group-based beam reporting of Option 1 can be interpreted as on a basis of per UE panel. For example, DL Tx beam #0 and DL Tx beam #2 (from UE antenna group 1) can be received by UE simultaneously. By the definition of Option 1, it aligns well with UL multi-panel beam selection which was concluded in Rel.16 and being processed in Rel.17. Since the Tx beams are reported from each UE antenna panel, it seems convenient that the UL beam indication can be conducted with specific panel ID and it seems to be well aligned with the newly defined unified TCI for either DL and UL usage.

- : the group-based beam reporting of Option 1 and Option 2 for multi-TRP

Therefore, given these aspects of group-based beam reporting analyzed for multi-TRP, we still prefer Option 1 can be supported in addition to Option 2.

- For multi-TRP operation, additionally support Option 1 of group-based beam reporting.

## L1-SINR based beam selection

In RAN1#102e, the following agreement was established

Agreement Evaluate and study at least but not limited to the following issues for multi-beam enhancement - Issue 1: Consideration of inter-beam interference - Issue 2: For group-based reporting, increased number of groups and/or beams per group - Issue 3: UE Rx panel related beam measurement/report - NOTE: “UE panel” is used for discussion purpose only

In addition to the classic beam selection metric i.e. L1-RSRP, L1-SINR which was specified in Rel.16 is to be decided as well. Different from L1-RSRP, the metric of L1-SINR reflects the interference experienced by UE. With proper channel measurement resource (CMR) and interference measurement resource (IMR) configuration, such interference measurement may include inter-cell/intra-cell/inter-TRP interference. Same as Rel.16, we have

- For group-based beam reporting of multi-TRP, support L1-SINR as beam selection metric.

If L1-SINR is supported for beam selection among multi-TRP, one question we have in mind is how to configure CMR and IMR for UE to measure. It seems straight-forward to reuse Rel.16 L1-SINR CMR/IMR configuration rule, i.e. CMR to IMR as 1-on-1 mapping in resource level. Moreover, we have one point to consider when compared with single-TRP operation, that’s inter-TRP interference.

- If L1-SINR is supported as beam selection metric, study whether L1-SINR should reflect inter-beam interference.

# BFR enhancement for multi-TRP

In Rel.15, one BFR procedure for primary cell has been designed and specified to handle the events of beam failure in PCell. Afterwards in Rel.16, a newly designed BFR procedure for secondary cell(s) was specified to cover beam failure events of all the serving cell(s) of a UE. In addition, RAN2 further enhanced the SpCell BFR by reusing the MAC CE which was initially specified for SCell BFR. However, these procedures originally aim for the single-TRP scenario and leave the multi-TRP scenario nearly untouched.

In this section, we would like to discuss the design of TRP-specific BFR in Rel.17.

The 1st issue is a general one that is whether cell-specific BFR and TRP-specific BFR can be configured in the same CC. In our understanding, if one CC is configured and then protected either by cell-specific BFR or TRP-specific BFR, it could be more robust when compared with single BFR mechanism. For instance, if one TRP failed, a UE could transmit BFRQ to the other TRP. If both TRPs failed, TRP-specific BFR may not work, thanks to beam correspondence. But cell-specific BFR in Rel.15/16 could serve as a backup scheme, e.g. fallback to CFRA or CBRA scheme. Moreover, if both BFR mechanism configured on the same CC, it could work equivalently as partial BFR which was promised to be supported in Rel.17.

- Support TRP-specific BFR and cell-specific BFR configured on the same CC.

## TRP-specific BFRQ

Before UE sends BFRQ MAC CE to NW, it has to require UL resource to carry BFRQ by using pre-configured PUCCH-SR. In RAN1#104e, up to 2 dedicated PUCCH-SR resources in a cell group can be configured to a UE. In RAN1#104bis-e, the PUCCH-SR selection rule was discussed with following agreement achieve. In RAN105e, this issue was touched again with compromised alternatives

Agreement For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR - FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1) Agreement Select one of the following alternatives with possible modification in RAN1#106-e - Alt 2.5.2 A: - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 B: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 C: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 D: - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

When one TRP is blocked, a nature choice for UE is to transmit PUCCH-SR to another TRP which is hopefully not failed. But which one TRP will be in failure is unpredictable, so UE should be able to send PUCCH-SR to any TRP priori to BFR. If no consensus can be achieved among companies, we think the only way left is up to UE selection on which PUCCH-SR resource to use. One simple implementation would be to pick up the PUCCH-SR resource whose PUCCH occasion comes earliest in time so that NW can know the TRP-specific BFR event as early as possible.

- If PUCCH-SR resource is associated with BFD-RS set, support UE to select the PUCCH-SR resource associated with non-failed TRP. Otherwise, support UE implementation on PUCCH-SR resource selection (Alt 2.5.2 C).

From signaling perspective, the BFRQ MAC CE should provide enough information on BFR to NW. Since each TRP can be identified by a CORESETPoolIndex, it is straight forward for a UE to report such TRP “ID” to NW. Therefore, NW can know which TRP(s) a UE experiences beam failure. In addition, there is a possibility that both TRPs within a CC fail at the same time. Hence, the BFRQ should be designed to facilitate UE to convey both failed TRPs to NW in one MAC CE. This is similarly design in Rel.16 for multiple SCell BFRQ.

- For TRP-specific BFRQ, support to use CORESETPoolIndex(es) in one BFRQ MAC CE as failed TRP ID(s).

## New beam after BFRR

The last step of a BFR procedure is to recover DL/UL control channel to a new beam, if reported. In RAN#96e, the following agreement on BFRR was made

Agreement - For the case of all CORESETs with 1 activated TCI state per CORESET, after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with the failed BFD-RS set reported in the MAC-CE for TRP-specific BFR is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD-RS set - FFS: How to associate CORESET(s) with failed BFD-RS set - FFS: SCS configuration of 28 symbols - FFS: Update of QCL assumption for other DL channels/RSs, UL spatial filter/power control assumption for PUCCH, and other UL channels/RSs - FFS: the case of CORESETs with 2 activated TCI states per CORESET. - The above applies to SCell and SpCell - The above applies at least for the multi-DCI case

It is clear that the QCL assumption of all CORESETs associated with failed BFD-RS set are updated to the newly reported beam. Due to the nature of TRP-specific BFR, those CORESETs belonging to the failed TRP link are configured (or by default) with the same CORESETPoolIndex, which we think is the key parameter to associated with BFD-RS set.

Following the same rule, the reported new DL beam (if any) should be used for UE to determine new UL Tx spatial filter and power control assumption for TRP-specific PUCCH transmission.

Considering UE’s processing time and new beam retuning time, the new beam shall apply after a period of time, i.e. 28 OFDM symbols, when counting on the smallest SCS of active DL BWP carrying the DCI (for BFRR) and active BWP of CC(s) reported in the MAC CE of BFRQ.

Also same as Rel.15/16 BFR, this new beam would not directly apply to DL or UL data channel. The reason lies in the fact that either DL or UL data channel can be indicated with TCI state or spatial relation in a more dynamic way.

With above consideration, we have

- On new QCL assumption for DL and spatial relation and power control for UL after BFRR, support to reuse Rel.16 rules in a TRP-specific manner

- CORESETs with the same CORESETPoolIndex can be associated with BFD-RS set.

- Spatial relation and power control parameters should be updated accordingly for PUCCH resources.

- The SCS of new beam application gap, i.e. 28 symbols is the smallest one between DL active BWP of UL DCI (for BFRR) reception and the active BWP(s) of reported CC(s) in BFRQ.

# RS overhead issue for multi-panels/BWPs/CCs

During beam management phase, UE/TRP needs to identify suitable DL/UL beam with beam sweeping from TRP/UE. The beam sweeping is performed using CSI-RS/SRS resource set. Each CSI-RS/SRS resource in a CSI-RS resource set is corresponding to each directional beam.

Especially for UL beam management, since UE has several panels as described below, many beam sweeping procedures may happen which cause SRS resource overhead issue. In Figure 4, 8 beam sweeping procedures may be required. There are red color antenna panels and blue color antenna panels. The panels with same color face to the same direction. The panels with different colors face to different directions. More specifically, the panels facing to the same direction can provide same beam sweeping property. In current NR specification, there is a QCL relationship between reference signal resources. If there is a QCL relationship between reference signal resource set, gNB/TRP can reduce UL beam management procedures. One beam sweeping in UE side is required for 4 blue antenna panels and one beam sweeping is required for 4 red antenna panels. Totally two beam sweeping procedures are needed for below antenna panel deployment which can reduce SRS resource overhead. The QCL relationship between SRS resource sets on the different antenna panels needs to be specified to reduce SRS resource overhead.

- UE side antenna panel deployment

- Specify the QCL relationship among SRS resource sets on different directional antenna panels.

In case of the TRP having same directional antenna panels as same as UE side, the QCL relationship between SSB/CSI-RS on different directional antenna panels is needed to be specified to reduce UE’s burden in monitoring SSB/CSI-RS during beam management.

- Specify the QCL relationship among CSI-RS resource sets/SSBs on different directional antenna panels.

As same as QCL definition in multi-TRPs, we can discuss same issue for multi- BWPs/CCs. UE can handle maximum 4 BWPs and 32 CCs in Rel16 specification. When each BWP/CC requires independent beam sweeping procedure, UE burden to monitor them would happen. We think the QCL relationship among SRS-resource sets/CSI-RS resource sets/SSBs on different BWPs/CCs needs to be considered.

- Specify the QCL relationship among SRS resource sets/CSI-RS resource sets/SSBs on different BWPs/CCs (intra band).

# UE antenna panels

Though the term of “antenna panel” has been intensively used in the discussion so far and one may find following agreement in Section 8.1.1 as below

Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, for discussion purpose, a panel entity corresponds to one or more RS resources: - For CSI/beam reporting, the RS resource is an RS associated with measurement and/or reporting - For beam indication, the RS resource is a source RS for UL TX spatial filter information - Note: For one RS resource, the corresponding panel entity may vary and is controlled by the UE, and whether/how to maintain a common understanding between gNB and UE can be further discussed/decided - Note: The above does not preclude possibility that an RS resource can be mapped to multiple panels - Note: The one or more RS resources may correspond to one or more RS resource set(s) depending on further discussion/decision - Note: Specification should not be designed in such a way that the UE is required to disclose its antenna implementation

A clear definition of antenna panel is still absence, which may cause difficulty for further progress on the topics relates to multiple panel operation. So we have following

- A definition of “UE antenna panel” is needed to pave the way for the progress on multi-panel operation.

A more intensive discussion on the possible definition of antenna panel and the possible panel specific transmission has been provided in [2]. We would like to reiterate here that the definition of antenna panel may vary from implementation to implementation. However, to enable an effective multiple panel operation, we suggest that the definition of a UE-panel shall be formulated in such a way that it can be associated with one or more of the properties listed below.

- A UE-panel supports Y beams

- Whether a UE-panel supports beam correspondence (BC)

- A UE-panel supports YRX independent Rx/TX-beams (from the Y set of beams)

- Each beam from a UE-panel may support one or two polarizations when receiving/transmitting

Even for multi-TRP scenario, a panel ID explicitly configured in spatialRelationInfo or unified TCI can be beneficial for panel specific operation. Therefore, we have

- A panel ID explicitly configured in spatialRelationInfo or unified TCI can be beneficial for panel specific operation.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- For multi-TRP operation, additionally support Option 1 of group-based beam reporting.

- For group-based beam reporting of multi-TRP, support L1-SINR as beam selection metric.

- If L1-SINR is supported as beam selection metric, study whether L1-SINR should reflect inter-beam interference.

- Support TRP-specific BFR and cell-specific BFR configured on the same CC.

- If PUCCH-SR resource is associated with BFD-RS set, support UE to select the PUCCH-SR resource associated with non-failed TRP. Otherwise, support UE implementation on PUCCH-SR resource selection (Alt 2.5.2 C).

- For TRP-specific BFRQ, support to use CORESETPoolIndex(es) in one BFRQ MAC CE as failed TRP ID(s).

- On new QCL assumption for DL and spatial relation and power control for UL after BFRR, support to reuse Rel.16 rules in a TRP-specific manner

- CORESETs with the same CORESETPoolIndex can be associated with BFD-RS set.

- Spatial relation and power control parameters should be updated accordingly for PUCCH resources.

- The SCS of new beam application gap, i.e. 28 symbols is the smallest one between DL active BWP of UL DCI (for BFRR) reception and the active BWP(s) of reported CC(s) in BFRQ.

- Specify the QCL relationship among SRS resource sets on different directional antenna panels.

- Specify the QCL relationship among CSI-RS resource sets/SSBs on different directional antenna panels.

- Specify the QCL relationship among SRS resource sets/CSI-RS resource sets/SSBs on different BWPs/CCs (intra band).

- A panel ID explicitly configured in spatialRelationInfo or unified TCI can be beneficial for panel specific operation.

# Reference

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung

- R1-2005560, “Considerations on the enhancement of multi-beam operation”, Sony