3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109863

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.5.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	TRP-Rx-ARP information reporting

Document for:	Discussion & Decision

# TRP-Rx-ARP Information

In in RAN1#106-e, ARP Rx reporting was discussed and the following agreement has been reached:

Agreement: Further study and conclude whether association of UL-AOA, UL-TDOA, Multi-RTT measurements with ARP (Antenna Reference Point) information is supported at RAN1#106bis-e.

The association of UL measurments, including AoA measurements, with the ARP information shall be supported for the following reasons:

- In Rel-16, each resource or resource set of DL-PRS can be associated with an ARP positon. However, for UL positioning methods, the TRP does not provide information on the Rx-ARPs for SRS timing measurements.

- If the gNB configures multiple TRPs to overcome reporting multiple Rx-ARPs, this may results in an unmethodical procedure, especially in case of joint UL-AoA and gNB-Rx-Tx reporting. For example, multiple DL-PRS resources are associated with multiple ARPs from the same TRP, while UL-AoA and UL-Rx-Tx measurements are associated with multiple TRPs despite the fact that information is the same.

- Also related to the previous bullet, TEG reporting for the Rx timing error groups sent with RTOA or Rx-Tx measurements is defined per TRP. If a common Rx and Tx ARPs reporting per TRP is not concluded, the UL and DL missalignement might result in undesired maintaince to clarify trpRxTxTEG or trpRxTEG reporting.

Figure 1 Antenna panel configuration example with multiple Rx-ARPs.

# Conclusion

In this contribution, we discuss TRP-Rx-ARP Information reporting for UL measurements.  We propose the following:

Proposal: 	Association of UL-AOA, UL-TDOA and Multi-RTT measurement results with ARP (Antenna Reference Point) information is supported