3GPP TSG-RAN WG1 Meeting #106bis-e	R1-2109928

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.4.2

Source:	Ericsson

Title:	On UL time and frequency synchronization enhancements for NTN

Document for:	Discussion

# Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the current work item description (WID) can be found in [1]. One objective is to specify uplink (UL) time and frequency synchronization enhancements for NTN. An overview of 3GPP non-terrestrial networks can be found in [15] and an overview of state of the art in LEO satellite access can be found in [16].

TR 38.821 [3] summarizes the observations made during the Release 16 study on solutions for NR to support NTN. With regards to downlink (DL) time and frequency synchronization it was concluded that robust performance can be supported without taking any special measures on the network side. The potential impact is limited to increased UE complexity.

For the UL time and frequency synchronization it was agreed that existing preamble formats can be reused during random access in case the UE can perform pre-compensation of timing and frequency offset. The suitability of the pre-compensation option is dependent on the UE’s ability to support GNSS in the different RRC states, on the availability of accurate ephemeris data, and on the UE’s ability to make use of the GNSS and ephemeris information for performing time and frequency pre-compensation to compensate for Doppler effects.

The agreements made in RAN103-e, RAN1#104-e, RAN1#104bis-e, RAN1#105-e and RN1#106-e can be found in Appendix A.

In the next sections, we express our view on this critical functionality.

# UE transmission timing calculations for NTN

## Overview of propagation delays in NTN

To determine the UE transmission timing, it is necessary to calculate the delay of the service link and the feeder link at a given point in time. These delays can be calculated based on the distances between the UE and satellite, and GW and satellite, respectively. However, to accurately calculate the delay that a given signal has experienced (or will experience), it is the distance between the position of the transmitting node at the time of transmission to the position of the receiving node at the time of reception that determines the delay. Since the time of transmission is different from the time of reception, this does not correspond to the distance between the nodes at any given point in time. In other words, accurately determining the link delays requires a dynamic view of the signal propagation, as opposed to a static snapshot.

Figure 1 illustrates the feeder and service link delays when transmission delays are taken into account. A DL signal is sent from GW at time t0, relayed by the satellite at time t1, and used for DL synchronization by the UE at time t2. An UL signal is sent from the UE at time t3, relayed by the satellite at time t4, and received by the GW/gNB at time t5. The timing relationships can be expressed as follows:

- t1=t0+df,DL=t0+1cpGWt0-psatt1

- t2=t1+ds,DL=t1+1cpsatt1-pUEt2

- t3=t2+dUE (UE-internal delay from DL sync to UL TX)

- t4=t3+ds,UL=t3+1cpUEt3-psatt4

- t5=t4+df,UL=t4+1cpsatt4-pGWt5

where psatt, pGWt and pUEt are satellite, GW and UE position vectors, respectively, in ECI (earth-centered inertial) coordinates, df,DL, df,UL, ds,DL and ds,UL  denote the delays of the DL feeder link, UL feeder link, DL service link, and UL service link, respectively,  and c is the speed of light. Note that in ECI, even though the GW moves at the speed of up to 466 m/s due to Earth rotation, it is still relatively slow compared to satellite speed of up to ~7700 m/s.

The UE needs to derive df,DL, df,UL, ds,DL and ds,UL to calculate its transmission timing. In the following we will look closer at these delays, how they can be calculated and whether it is necessary to take the dynamics into account. Note that the detailed methods for calculation should be up to implementation. What is relevant from a specification perspective is how the delays are defined and how accurate they need to be calculated.

Figure 1: Signal timing relationships for NTN.

## DL service link delay

First consider the DL service link delay, ds,DL of the signal used by the UE to synchronize at time t2. This is determined by the following expression:

ds,DL=1cpsatt2-ds,DL-pUEt2

Here, the reception time at the UE, t2, is known while the transmission time of the satellite, t2-ds,DL, is unknown. Therefore, the satellite position needs to be propagated backwards in time to calculate the service link delay. This can be done with various numerical methods. A very accurate solution can be found using e.g. the Newton-Raphson method. Below we investigate two simplified solutions from accuracy point of view.

- A crude approximation is to ignore the satellite movement and use the satellite-UE distance at sync time t2, i.e.:

ds,DL≈1cpsatt2-pUEt2

This method will be denoted “sync time” in the figure legend below.

- A simple method to take into account the approximate movement of the satellite is to first approximate the delay based on the distance at sync time t2 and then calculate the satellite position at time t2-d1 and re-calculate ds,DL:

d1=1cpsatt2-pUEt2

ds,DL=1cpsatt2-d1-pUEt2

This method will be denoted “two-step” in the figure legend below.

To assess the performance of the methods, simulations were run. A LEO satellite with circular orbit is modelled. Orbit altitudes ranging from 600 km to 1800 km are tested. For each altitude, the satellite passes from horizon to horizon (≥10° elevation angle) above a UE and a gateway on the Earth surface. Every 10 ms the UE calculates the DL service link delay based on UE position and satellite ephemeris. The two methods described above are tested and the performance is shown in Figure 2. The reference ds,DL is calculated using the Newton-Raphson method. The blue curve corresponds to the “sync time” method. The red curve corresponds to the “two-step” method.

It can be seen that using the “sync time” method (i.e., ignoring the satellite movement during the time-of-flight from satellite to UE) results in an error of up to 0.25 µs, depending on satellite altitude. This corresponds to approximately 5.3% of the CP length for a PUSCH or PUCCH with 15 kHz SCS and 43% of the CP with 120 kHz SCS.

Also, it can be seen that the method taking into account satellite movement gives good accuracy.

This is summarized in the following observation:

When calculating the DL service link delay of a signal, ignoring the satellite movement during the signal propagation from satellite to UE will lead to a calculation error of up to 0.25 µs, which corresponds to 5% of the PUCCH/PUSCH CP length for 15 kHz SCS and 43% of the CP length for 120 kHz SCS.

Using a low-complex approximative “two-step” method is sufficient to get a good approximation of the impact of satellite movement on DL service link delay.

The algorithm used by the UE to calculate the DL service link delay can be implementation dependent but the DL service link delay should be defined in the specifications taking satellite movement during DL signal propagation into account.

As part of defining the UE-specific TA, RAN1 should define the DL service link delay taking into account satellite movement during DL signal propagation from satellite to UE.

Figure 2: Max DL service link delay estimation error for different approximative calculation methods.

## UL service link delay

Similar calculations can be made when determining the other contributions to the TA. From the UE perspective, the UL service link delay must also be determined before the UE can transmit. The UE will then target a particular UL slot at the gNB receiver (based on UL grant, etc.), i.e., the desired reception time at the gNB (t5 in Figure 1) is known. Based on this and on signaled common TA from the network, the UE can calculate the desired reception time at the satellite (t4 in Figure 1), and further the required transmission time from the UE (t3 in Figure 1). Therefore, the UL service link delay will be the solution to the following equation:

ds,UL=1cpsatt4-pUEt4-ds,UL

In this equation it is only the UE position that depends on the delay, and since the UE speed is relatively low (even when considering the impact of Earth rotation), it can probably be ignored in practice.

As part of defining the UE-specific TA, RAN1 should define the UL service link delay taking into account satellite movement during UL signal propagation from UE to satellite.

## Feeder link delay

For the feeder link delay, similar considerations can be made by the network. However, this can be done in an implementation-specific way. To enable compensation for the feeder link delay, the network could approximate the feeder link delay with a polynomial function.

# Time and frequency compensation

## Reference point for time and frequency

As per conclusion at RAN1#105-e, Doppler shift compensation for the feeder link is managed transparently by the gateway and/or satellite payload. Therefore, the focus of the discussion below is the reference point for time.

The reference point is the point in the network at which UL time slots are received at their nominal time and carriers are transmitted and received at their nominal DL and UL frequencies, respectively.  The reference point for time and frequency has been the BS antenna connector, as defined in section 4.3.1 of TS 38.104 [12], up to and including release-16. For release-17 NTN WI, there are at least two options, the BS antenna connector of the gNB (as in release-16) or the antenna connector of the satellite.

If the satellite is time reference point, then UL signals will be time aligned at satellite RX. Then all time slots will be misaligned by twice the feeder link delay. This is shown in Figure 3.

Figure 3: Impact of having the time reference for UL timing synchronization at the satellite on DL/UL framing alignment.

All time slots will be misaligned by twice the feeder link delay, if the satellite is used as reference for time requirements.

Using satellite as reference for time requirements severely affects compatibility with existing Rel-16 gNB.

If the gNB is the reference point, then UL signals will be time aligned at gNB. This is illustrated in Figure 4.

Figure 4: Impact of having the time reference for UL timing synchronization at the gNB on DL/UL framing alignment.

In our view, the reference point should be under control of the network. At least the option of having gNB as the reference point should be supported. The UE should be responsible for autonomously determining and pre-compensating the access offset required for time and frequency alignment at the satellite, while the network may configure additional time offset that is signaled to the UE. The signaled time offset is determined by the gNB and may be used to move the reference point e.g. to the gNB, i.e., compensate for the feeder link RTT. The Doppler shift of the feeder link is assumed to be handled transparently by the gateway and/or satellite payload.It should be noted that even if the location of the reference point can be flexible from a physical layer point of view, as discussed under agenda item 8.4.1 (see e.g. [5]), having the reference point in the satellite will require continuous adjustment of the DL and UL time at the gNB that is complex to maintain and will also require significant additional specification effort in other RAN groups to specify how the timing adjustment must be performed by the gNB, the accuracy levels expected for this operation and testing procedures for product validation. Also, a varying DL/UL slot misalignment at the gNB, will have significant impact to the scheduler.

The reference point for time in an NTN should be under control of the network and should at least support the option of having gNB as the reference point.

The support of a reference point for time that results in a time-varying DL/UL slot misalignment at the gNB will have significant impact to gNB implementation and result in heavy specification work in other RAN groups.

## Time synchronization

### Rel-16 time synchronization

The current (Rel-16) TA is defined as follows [2]:

TTA=(NTA+NTA,offset)×Tc

It consists of two parts:

- NTA is the timing advance which is dynamically controlled by the network. For initial access (PRACH transmission), it is zero. After initial access, it is updated through an absolute timing advance command in RAR and subsequently through timing advance commands in MAC CE.

- NTA,offset is a semi-static offset which is dependent on frequency band and LTE/NR coexistence and can be explicitly signaled by the NW.

### NTN time synchronization

At RAN1#104bis-e, the following agreement was made:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The TA for NTN UE consists of four components. These are further discussed below.

#### Closed-loop TA, NTA

At RAN1#106-e, it was agreed that the UE will apply the closed-loop TA command received in msg2/msgB as follows:

NTA=NTA_old+TA. 16.642μ, FFS: the value of NTA_old

The term NTA_old was introduced considering a potential inclusion of a TA margin. However, Ericsson’s view is that if a TA margin is needed, it can be included in the Common TA. Therefore, the NTA_old is not needed in the expression above.

When TAC (TA) in msg2/msgB is received, NTA is calculated as follows:
NTA=TA. 16.642μ

#### Fixed TA offset, NTA,offset

NTA,offset is proposed to be defined as in Rel-16 (see section 3.2.1).

The fixed TA offset, NTA,offset, should be defined as in Rel-16.

#### UE-specific TA, NTA,UE-specific

The purpose of the UE-specific TA is to compensate for the RTT of the service link. The UE-specific TA is autonomously determined by the UE based on GNSS-acquired UE position and serving satellite position from broadcast ephemeris.

As discussed in Section 2, to accurately calculate the service link delay, it is necessary to take into account that the UL and DL signals are offset in time and therefore subject to different service link delays. It is also necessary to take into account when calculating the service link delay that the satellite moves during the transmission across the service link.

NTA, UE-specific=ds,DL+ds,UL/Tc

where ds,DL and ds,UL are the DL and UL service link delays derived by the UE, respectively.

The UE calculates the UE-specific TA (in Tc units) as follows:
NTA, UE-specific=ds,DL+ds,UL/Tc
where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA.

NOTE: Whether the granularity of the UE-specific TA should be Tc or a multiple of Tc is FFS.

#### Common TA, NTA,common

The purpose of the common TA is to compensate for the RTT of the feeder link and possibly other latencies in the satellite-gNB path. The common TA varies with time. As discussed in Section 2, it is beneficial to consider UL and DL delays separately since the UL and DL signals are offset in time and therefore subject to different delays. The common delay can be approximated by a polynomial function as follows:

Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift+(t-Tcommon,base)2Dcommon,2nd+(t-Tcommon,base)3Dcommon,3rd

where:

t is the time the signal passes the satellite

Tcommon,base is a reference time of the broadcast common delay

Dcommon, base is the common one-way delay at time Tcommon,base

Dcommon, drift is the common one-way delay drift rate

Dcommon, 2nd is the common one-way delay drift rate variation

Dcommon, 3rd is the common one-way delay 3rd order term

The parameters Dcommon, base, Dcommon, drift, Dcommon, 2nd and Dcommon, 3rd are broadcast regularly by the network to allow the UE to calculate the common TA, whereas the reference (epoch) time Tcommon,base is implicit from the DL subframe in which the parameters are signaled (see section 5.1).

The network broadcasts parameters describing the (one-way) common delay by a polynomial function as follows:
Dcommont=Dcommon,base+t-Tcommon,baseDcommon,drift+(t-Tcommon,base)2Dcommon,2nd+(t-Tcommon,base)3Dcommon,3rd
where:
t is the time the signal passes the satellite
Tcommon,base is the implicit epoch time
Dcommon, base is the common one-way delay at time Tcommon,base 
Dcommon, drift is the common one-way delay drift rate
Dcommon, 2nd is the common one-way delay drift rate variation
Dcommon, 3rd is the common one-way delay 3rd order term

The common TA is then calculated as the sum of the DL and UL common delays. It should be quantized to a granularity of (64/2µ)Tc (see section 3.2.2.4.1) as follows:

NTA, common=Dcommontsat,DL+Dcommontsat,UL/(64/2µ)Tc

where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to be transmitted by the UE will be relayed by the satellite.

The UE determines the common TA (in Tc units) as follows:
NTA, common=Dcommontsat,DL+Dcommontsat,UL/(64/2µ)Tc
where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to which the UE applies the TA will be relayed by the satellite.

##### Granularity of common TA

The granularity of the signaled common delay parameters, Dcommon, base, Dcommon, drift, Dcommon, 2nd and Dcommon, 3rd as well as the granularity of the applied common TA value, NTA,common, will impact the UE transmit timing error and thereby the timing error of the signal received by the gNB. The accuracy requirements for UE transmit timing are not yet defined but the design target should be that the total timing errror can be handled by the CP also in the presence of channel time dispersion. There are several sources of error that add up to the total UE transmit timing error. Some sources of error such as DL synchronization inaccuracy, UE positioning inaccuracy and satellite ephemeris inaccuracy may be costly to reduce. To allow for more feasible requirements on these error sources, the quantization error of common TA should preferably consume only a small part of the total error budget. Therefore, the NTA,common applied by the UE should have a granularity of e.g. 64/2μ [Tc units], which corresponds to less than 1% of the normal CP length for PUCCH/PUSCH.

The granularity of the NTA,common applied by the UE should be 64/2µ to limit its impact to the total timing error budget to less than 1% of the CP length.

Dcommon, base should be broadcast with granularity (64/23)Tc to match the proposed granularity of NTA,common with the highest SCS. The maximum propagation delay between gateway and satellite is 271 ms for a GEO satellite [3]. This means that the total range of Dcommon, base of 0-271 ms can be signaled with granularity (64/23)Tc using 26 bits.

The common delay, Dcommon, base, can be signaled with granularity (64/23)Tc using 26 bits.

Based on simulations is has been found that the common delay drift rate Dcommon, drift can be in the range ±25 µs/s, the common delay drift rate variation Dcommon, 2nd can be in the range 0-0.6 µs/s2 and the 3rd order term Dcommon, 3rd can be in the range ±0.012 µs/s3 . In section 3.2.2.6.3 it is shown that these parameters can be signaled with 16, 15 and 15 bits, respectively, without significant accuracy losses.

The common delay drift rate Dcommon, drift, the common delay drift variation rate Dcommon, 2nd and the 3rd order term, Dcommon, 3rd can be signaled with 16, 15 and 15 bits, respectively.

#### TA margin

At RAN1#106-e, the following agreement was made:

Agreement:

In NTN, to avoid that the UE over pre-compensates its TA during RACH procedure, down-select one option from below:

- Option 1: PRACH transmission is delayed by minCP2,GP2

- Option 2: TA margin can be considered, and it is explicitly indicated to the UE

- Option 3: TA margin can be considered, and it is included within the Common TA

- Option 4: UE handles it via implementation

When the discussion about TA margin for PRACH was started, it was assumed that the potential over-compensation of the propagation delay may be significant. Since then it has been agreed that UE-autonomous TA estimation will be used also in RRC_CONNECTED, which requires that the timing accuracy is a fraction of the CP length of PUCCH/PUSCH, i.e., in the order of 0.1-1 µs depending on the UL SCS. There is no reason why the UE would not be able to determine also its UL pre-compensation for PRACH with the same accuracy. Therefore, the potential over-compensation is negligible and will have no significant performance impact, as shown in [10]. Therefore, there is no need to define a TA margin. Among the options in the agreement from RAN1#106-e, this mostly corresponds to Option 4.

Do not define a TA margin. The RAN4 requirements on UE TX timing accuracy are expected to ensure that the performance impact of the potential TA over-compensation of PRACH will be insignificant.

#### Simulations of TA estimation accuracy

In this section, simulations of TA estimation accuracy are presented.

##### Simulation assumptions

The simulator models and assumptions are summarized in Table 1.

Table 1: Simulator models and parameters

| Satellite orbit | Satellite orbit data from Eutelsat is used. The satellite is in a low earth orbit (LEO) with approximately 537 km altitude. The data gives the satellite position and velocity with 1 second resolution. To get higher time resolution, interpolation is used. |
| --- | --- |
| Satellite ephemeris inaccuracy | Satellite orbit determination is performed in the NTN control center (NCC). Orbit determination is the process of determining the past and present satellite orbit based on measurements of satellite position and velocity. Satellite orbit prediction is also performed in the NCC. Orbit prediction is the process of predicting the future satellite position and velocity based on the input from the orbit determination step. The inaccuracy of the orbit prediction in the NCC results in a gradually increasing ephemeris error the more time that has passed since the last orbit determination. The high-precision orbit prediction algorithms typically used for this are not implemented in the simulator. Instead, a fixed error model based on the results in [9] is used. It is assumed that orbit determination is performed every 5 minutes.Since this is a worst-case analysis, it is assumed that the orbit determination is always 5 minutes old, i.e., the max value of the position and velocity error magnitude for the ”typical” orbit determination case after 5 minutes of orbit prediction is (see table 6 in [9]): Max (99.7%) satellite position error = 3.87 m Max (99.7%) satellite velocity error = 15.66 mm/s The error vector directions are selected to get worst-case impact, which according to [9] means that the position error is in the radial direction and the velocity error is in the tangential direction. |
| Ephemeris broadcast | The satellite ephemeris, with inaccuracy as described above, is broadcast frequently by the network and read by the UE at regular intervals. The position/velocity vector format is used. |
| Common TA parameter broadcast | The Common TA parameters are broadcast at the same regular interval as the satellite ephemeris. The Common TA parameters approximately describe the feeder link RTT for the given interval by a polynomial function. Curve fitting is used to determine the common TA parameters. The ephemeris including ephemeris error as described above is used to calculate the feeder link RTT. Different degrees of the Common TA polynomial function are evaluated, ranging from linear (common TA + drift rate) to 3rd order (common TA + drift rate + drift rate variation + 3rd order term). |
| UE-specific TA calculation | The UE uses the broadcast ephemeris to perform satellite orbit prediction. The satellite ephemeris is first converted to Keplerian orbital format, using the algorithm in [14], and next the simple Keplerian propagator model in [13] is used to predict the satellite position. The UE position is assumed to be fixed. No UE position error is modelled. |
| Common TA calculation | The UE calculates and applies the common TA based on the broadcast parameters. |
| Closed-loop TA control | Closed-loop TA commands are sent at regular intervals. The granularity and range of the legacy TAC format is used. The TAC is delayed by one RTT plus an additional 10 ms before it is applied by the UE. The closed-loop TA control will (try to) compensate for errors in the estimated UE-specific TA and common TA (NTA,common+NTA,UE-specific). When the UE re-acquires ephemeris data and common TA parameters and calculates new NTA,common+NTA,UE-specific, the error may change (typically decrease), which may result in a ”jump” in the applied NTA,common+NTA,UE-specific. Since the closed-loop TA control will typically have compensated for the error before the jump, the effect can be that the closed-loop TA (NTA) compensation increases the error of NTA,common+NTA,UE-specific after the jump. To avoid this, NTA is compensated for jumps in NTA,common+NTA,UE-specific each time new parameters are acquired. TAC intervals of 100 ms and 400 ms are tested. |
| Parameter acquisition interval | The ephemeris and common TA parameters are acquired by the UE at regular intervals, ranging from 5 seconds to 30 seconds. |
| Parameter quantization | When common TA parameters are quantized, the following quantization steps are used: Parameter; Quantization step. Common delay; (64/23)Tc (≈0.271/226). Delay drift rate; 5e-5/216. Drift rate variation; 6e-7/215. 3rd order term; 6e-9/214. |
| Minimum elevation angle | 10° |
| Other | The UE is served by an earth-fixed beam during one pass of the satellite |
| Sub-carrier spacing | Different SCS ranging from 15 kHz to 120 kHz are considered. |
| Target accuracy | For feeder link RTT: 10% of CP length for the respective SCS For service link RTT: 10% of CP length for the respective SCS For total RTT: 20% of CP length for the respective SCS |

##### Effects of Common TA model order

The 99th percentile estimation errors of the feeder link RTT, service link RTT and total RTT are shown in Figure 5, Figure 6 and Figure 7, respectively. As reference, target accuracy of 10 % of the CP length for feeder link RTT and service link RTT and 20 % for the total RTT is shown in the figures.

Figure 5: 99th percentile feeder link RTT approximation error.

Figure 6: 99th percentile service link RTT approximation error.

Figure 7: 99th percentile feeder+service link RTT approximation error.

The parameter update periods required to reach the accuracy targets are summarized in Table 2.

Table 2: Required parameter update period to reach accuracy targets.

|  | SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- | --- |
| Feeder link RTT | Linear | - | - | - | - |
|  | 2nd order | 22.5 s | 17.5 s | 14 s | 11 s |
|  | 3rd order | >30 s | >30 s | >30 s | 28 s |
| Service link RTT |  | >30 s | >30 s | >30 s | >30 s |
| Total RTT | Linear | 5 s | - | - | - |
|  | 2nd order | 28 s | 22.5 s | 17.5 s | 13 s |
|  | 3rd order | >30 s | >30 s | >30 s | >30 s |

##### Effects of quantization

Figure 8 and Figure 9 shows the effects of quantization of the common TA parameters according to Table 3. The quantization results in a small degradation of feeder link RTT estimation accuracy but no visible impact on the total RTT accuracy. Since the accuracy of the total RTT is a more relevant performance measure, the field lengths can potentially be reduced somewhat compared to Table 3.

Table 3: Quantization steps for common delay parameters

| Parameter | Quantization step | Field length |
| --- | --- | --- |
| Common delay | (64/23)Tc (≈0.271/226) | 26 |
| Delay drift rate | 5e-5/216 | 16 |
| Drift rate variation | 6e-7/215 | 15 |
| 3rd order term | 6e-9/214 | 14 |

Figure 8: 99th percentile feeder link RTT approximation error with common delay parameter quantization.

Figure 9: 99th percentile feeder+service link RTT approximation error with common delay parameter quantization.

##### Impact of closed-loop TA control

Figure 10 and Figure 11 show the impact of closed-loop TA control when TA commands are sent at 400 ms and 100 ms intervals, respectively, to compensate for the residual errors of UE-specific TA and common TA estimation. It can be seen that the validity time of the parameters is significantly improved by the closed-loop TA control.

Figure 10: 99th percentile feeder+service link RTT approximation error with closed-loop TA control with 400 ms TAC interval.

Figure 11: 99th percentile feeder+service link RTT approximation error with closed-loop TA control with 100 ms TAC interval.

The parameter (common TA and ephemeris) update periods required to reach the accuracy targets are summarized in Table 4.

Table 4: Required parameter update period to reach accuracy targets on total RTT estimation with open-loop + closed-loop TA control.

|  | SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- | --- |
| 400 ms TAC interval | Linear | 11 s | 5 s | - | - |
|  | 2nd order | >30 s | >30 s | >30 s | 25 s |
|  | 3rd order | >30 s | >30 s | >30 s | >30 s |
| 100 ms TAC interval | Linear | >30 s | 17 s | 8 s | <5 s |
|  | 2nd order | >30 s | >30 s | >30 s | >30 s |
|  | 3rd order | >30 s | >30 s | >30 s | >30 s |

##### Discussion

Table 2 shows that without closed-loop TA control, characterizing the common TA with a base value + drift rate is insufficient. Only for the case of 15 kHz SCS the accuracy target can be fulfilled and only with frequent acquisition – every 5 s – of the common TA + ephemeris parameters. To manage all SCS with a reasonable update interval of the parameters, 2nd and preferably also 3rd order terms of the common TA are needed.

Table 4 shows that when closed-loop TA control is used to compensate for the residual errors in the open-loop TA control, the common TA + ephemeris parameter update interval can be significantly increased. With a 400 ms interval of the TAC, an update rate of 25 s or more of the common TA + ephemeris parameters is sufficient with only base value + drift rate + 2nd order term of common TA. Including also a 3rd order term will extend the validity time further.

The following observations are made:

Without closed-loop TA control, it is necessary to characterize the common TA with 2nd and 3rd order terms in addition to the drift rate and base value.

When closed-loop TA control is used to compensate for the residual errors in the open-loop TA control, the common TA + ephemeris parameter update interval can be significantly increased. With a 400 ms interval of TAC, an update interval of 25 seconds or more can be achieved if the common TA is characterized with a 2nd order terms in addition to the drift rate and base value.

Including a 3rd order term characterize the common TA reduces the dependence on closed-loop TAC and extends the validity time of the common TA.

The findings are summarized in the following proposals:

Confirm the working assumption that Common TA may include parameter(s) indicating timing drift.

Characterize the common TA with the following parameters:
 - Common delay
 - Drift rate
 - Drift rate variation
 - 3rd order term

## Frequency synchronization

At RAN1#103-e, it was agreed that the UE autonomously compensates for the Doppler shift of the service link. At RAN1#105-e, it was concluded that the gateway and/or satellite payload compensates for the Doppler shift of the feeder link.

One remaining issue related to frequency synchronization is whether the gNB should pre-compensate (a common part of) the Doppler shift of the service link to simplify initial DL synchronization for the UE. At RAN1#105-e (see FL summary in [6]), concerns were raised that DL frequency pre-compensation is a significant challenge to the UE and/or the gNB. Therefore, a proposal to de-prioritize support of DL frequency pre-compensation for the service link was discussed but not agreed. Also, the SI concluded that DL synchronization is possible without DL pre-compensation. Therefore, we support the proposal made by the FL at RAN1#105-e:

Deprioritize support of Common DL frequency compensation for the service link Doppler shift.

## Alternative UE-centric solution

In the previous sections, solutions for time and frequency compensation assume that the gNB/gateway is responsible for determining the feeder link compensation and instructing UE to apply additional access offsets accordingly. Alternatively, if the position of a reference point (which may or may not be the gateway) and the UL and DL carrier frequencies of the feeder link were signaled to the UE, the UE could autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link. This would simplify the time and frequency compensation procedures.

As mentioned, the reference point may or may not be the gateway. It is up to network to configure. In scenarios where it is acceptable to broadcast gateway position, the network can configure the reference point to be the position of the gateway. In scenarios where it is not acceptable to broadcast gateway position, the network can configure the reference point to be the “approximate” position of the gateway. If the reference point is not the gateway, it is up to gNB how to handle the (small) variations in timing and frequency offset.

If the position of a reference point of the feeder link and the UL and DL carrier frequencies of the feeder link are signaled to the UE, the UE can autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link, which would simplify the time and frequency compensation procedures.

Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link.

NOTE: At RAN1#105-e, an LS [8] was sent to SA1/SA3 asking for guidance on the security aspects of broadcasting information that implicitly or explicitly exposes the gateway position.

## Validity timer

It has not been decided if separate validity timers are needed for common TA and satellite ephemeris. One option is to have a common validity timer for ephemeris and common TA if the UE reads both from the same SIB. Then, the UE will need to read the SIB anyway regardless of which quantity has expired or is about to expire. However, if they are transmitted in different SIBs, we prefer a flexible design with separate validity timers for common TA and ephemeris.

Use separate validity timers for serving satellite ephemeris and common TA if they are transmitted in different SIBs, otherwise use a single validity timer for both.

To have unambiguous starting time of the validity timer(s), it is proposed to start it at the epoch time of the satellite ephemeris and common TA.

The validity timer(s) should be started at the epoch time(s) of the satellite ephemeris and common TA.

# GNSS requirements

RAN1 needs to discuss how to deal with the case of no GNSS coverage. The relevant case of interest is when the NR-NTN satellite is accessible while GNSS is out of coverage. For instance, it is reasonable to assume that neither of the two systems will provide indoor coverage. However, there may be corner cases where e.g. a mountain or a high-rise building blocks line-of-sight to a majority of the GNSS satellites but not all NTN satellites. How to deal with this case needs further discussion.

RAN1 to determine the relevance of the case of NTN coverage but no GNSS coverage.

# Satellite ephemeris

## Epoch time

At RAN1#106-e, the following agreement was made:

Agreement:

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

The epoch time is defined by a slot that is not (necessarily) the slot in which the ephemeris was sent. The purpose is to avoid the need for tight coordination between RRC that generates the SIB containing the ephemeris data, and MAC that schedules its transmission in a given slot.

SIB other than SIB1 are sent in SI messages that are scheduled within an SI window of length si-WindowLength that occurs periodically with a period of si-Periodicity.

- si-WindowLength: 5, 10, 20, 40, 80, 160, 320, 640, 1280 slots

- si-Periodicity: 8, 16, 32, 64, 128, 256, 512 radio frames (corresponding to 80 to 5120 ms)

It is proposed to couple the ephemeris to the start of the SI window of the SI message in which the ephemeris is sent.

When the ephemeris data is received from an external source such as an NCC (NTN Control Center), it may have an explicit epoch time that differs from the SI window start position. To avoid that the gNB has to propagate the ephemeris for an extended period of time, it is proposed to let the gNB signal an offset N from the start of the SI window to the epoch time. The granularity of N can be slots to give millisecond (or less) granularity of the epoch time. If even better alignment is needed, propagation of the ephemeris within a millisecond should be feasible with a very simple orbit propagation algorithm in the gNB.

Figure 12 shows an example of the SI scheduling for the case that si-Periodicity=32 radio frames and si-WindowLength=20 slots.

Figure 12: Illustration of SI scheduling and implicit epoch time.

The same epoch time definition should be used also for the common TA.

The epoch time of the serving satellite ephemeris is defined by the start of the Nth slot after the start of the SI window of the SI message containing the ephemeris, where N is optionally signaled with the ephemeris (otherwise 0).

The epoch time of the Common TA parameters is defined by the start of the Nth slot after the start of the SI window of the SI message containing the Common TA parameters, where N is optionally signaled with the Common TA parameters (otherwise 0).

To make the epoch time unambiguous it is also necessary to define a reference point. The epoch time is then the time at which the slot start that defines the epoch is transmitted from the reference point. It is proposed to use the satellite as reference point for serving satellite ephemeris epoch time.

The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the satellite transmitter.

# RRC parameters

The outcome of the intial discussion on RRC parameters related to UL synchronization is summarized in [11]. Table 5 gives our comments on the current preliminary list of parameters. Table 6 lists additional parameters that are expected to be needed when agreed.

Table 5: Comments on preliminary list of RRC parameters.

| Parameter Name | Ericsson comments |
| --- | --- |
| NTACommon | The parameter should indicate the one-way common delay instead of the common TA. For this reason it is suggested to rename it to e.g. DelayCommon. |

Table 6: Expected additional RRC parameters.

| Parameter Name | Description |
| --- | --- |
| DelayCommonDrift | The drift rate of the common delay |
| DelayCommon2nd | The drift rate variation of the common delay |
| DelayCommon3rd | The third order term of the common delay |

# Conclusions

In the previous sections, we discuss UL time and frequency synchronization enhancements for NTN. We made the following observations:

Observation 1	When calculating the DL service link delay of a signal, ignoring the satellite movement during the signal propagation from satellite to UE will lead to a calculation error of up to 0.25 µs, which corresponds to 5% of the PUCCH/PUSCH CP length for 15 kHz SCS and 43% of the CP length for 120 kHz SCS.

Observation 2	Using a low-complex approximative “two-step” method is sufficient to get a good approximation of the impact of satellite movement on DL service link delay.

Observation 3	The algorithm used by the UE to calculate the DL service link delay can be implementation dependent but the DL service link delay should be defined in the specifications taking satellite movement during DL signal propagation into account.

Observation 4	All time slots will be misaligned by twice the feeder link delay, if the satellite is used as reference for time requirements.

Observation 5	Using satellite as reference for time requirements severely affects compatibility with existing Rel-16 gNB.

Observation 6	The support of a reference point for time that results in a time-varying DL/UL slot misalignment at the gNB will have significant impact to gNB implementation and result in heavy specification work in other RAN groups.

Observation 7	The common delay, Dcommon, base, can be signaled with granularity (64/23)Tc using 26 bits.

Observation 8	The common delay drift rate Dcommon, drift, the common delay drift variation rate Dcommon, 2nd and the 3rd order term, Dcommon, 3rd can be signaled with 16, 15 and 15 bits, respectively.

Observation 9	Without closed-loop TA control, it is necessary to characterize the common TA with 2nd and 3rd order terms in addition to the drift rate and base value.

Observation 10	When closed-loop TA control is used to compensate for the residual errors in the open-loop TA control, the common TA + ephemeris parameter update interval can be significantly increased. With a 400 ms interval of TAC, an update interval of 25 seconds or more can be achieved if the common TA is characterized with a 2nd order terms in addition to the drift rate and base value.

Observation 11	Including a 3rd order term characterize the common TA reduces the dependence on closed-loop TAC and extends the validity time of the common TA.

Observation 12	If the position of a reference point of the feeder link and the UL and DL carrier frequencies of the feeder link are signaled to the UE, the UE can autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link, which would simplify the time and frequency compensation procedures.

Based on the discussion in the previous sections we propose the following:

Proposal 1	As part of defining the UE-specific TA, RAN1 should define the DL service link delay taking into account satellite movement during DL signal propagation from satellite to UE.

Proposal 2	As part of defining the UE-specific TA, RAN1 should define the UL service link delay taking into account satellite movement during UL signal propagation from UE to satellite.

Proposal 3	The reference point for time in an NTN should be under control of the network and should at least support the option of having gNB as the reference point.

Proposal 4	When TAC (TA) in msg2/msgB is received, NTA is calculated as follows: 
NTA=TA. 16.642μ

Proposal 5	The fixed TA offset, NTA,offset, should be defined as in Rel-16.

Proposal 6	The UE calculates the UE-specific TA (in Tc units) as follows: 
NTA, UE-specific=ds,DL+ds,UL/Tc
where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA.

Proposal 7	The network broadcasts parameters describing the (one-way) common delay by a polynomial function as follows: 
Dcommont=Dcommon,base+t-Tcommon,baseDcommon,drift+(t-Tcommon,base)2Dcommon,2nd+(t-Tcommon,base)3Dcommon,3rd
where:
t is the time the signal passes the satellite
Tcommon,base is the implicit epoch time 
Dcommon, base is the common one-way delay at time Tcommon,base  
Dcommon, drift is the common one-way delay drift rate 
Dcommon, 2nd is the common one-way delay drift rate variation 
Dcommon, 3rd is the common one-way delay 3rd order term

Proposal 8	The UE determines the common TA (in Tc units) as follows: 
NTA, common=Dcommontsat,DL+Dcommontsat,UL/(64/2µ)Tc
where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to which the UE applies the TA will be relayed by the satellite.

Proposal 9	The granularity of the NTA,common applied by the UE should be 64/2µ to limit its impact to the total timing error budget to less than 1% of the CP length.

Proposal 10	Do not define a TA margin. The RAN4 requirements on UE TX timing accuracy are expected to ensure that the performance impact of the potential TA over-compensation of PRACH will be insignificant.

Proposal 11	Confirm the working assumption that Common TA may include parameter(s) indicating timing drift.

Proposal 12	Characterize the common TA with the following parameters:  
- Common delay  
- Drift rate  
- Drift rate variation  
- 3rd order term

Proposal 13	Deprioritize support of Common DL frequency compensation for the service link Doppler shift.

Proposal 14	Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link.

Proposal 15	Use separate validity timers for serving satellite ephemeris and common TA if they are transmitted in different SIBs, otherwise use a single validity timer for both.

Proposal 16	The validity timer(s) should be started at the epoch time(s) of the satellite ephemeris and common TA.

Proposal 17	RAN1 to determine the relevance of the case of NTN coverage but no GNSS coverage.

Proposal 18	The epoch time of the serving satellite ephemeris is defined by the start of the Nth slot after the start of the SI window of the SI message containing the ephemeris, where N is optionally signaled with the ephemeris (otherwise 0).

Proposal 19	The epoch time of the Common TA parameters is defined by the start of the Nth slot after the start of the SI window of the SI message containing the Common TA parameters, where N is optionally signaled with the Common TA parameters (otherwise 0).

Proposal 20	The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the satellite transmitter.

# References

RP-211784, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #93-e, September 2021.

TS 38.211, Physical channels and modulation.

TR 38.821, Solutions for NR to support non-terrestrial networks.

R1-2100520, “Considerations on UL timing and frequency synchronization in NTN”, Thales, Rakuten mobile, January 2021

R1-2101296, “Further views on DL-UL timing relationship for NTN operation”, Nokia, January 2021.

R1-2106290, ”FL Summary on enhancements on UL time and frequency synchronization for NR NTN”, Moderator (Thales), May 2021

R4-2113521, ”Timing requirements”, Ericsson, May 2021

R1-2106332, ”LS on broadcast of NTN GW or gNB position”, RAN1, May 2021

R1-2106556, ”Considerations on UL timing and frequency synchronization in NTN”, Thales, August 2021

R1-2102915, ”Discussion on UL synchronization for NR-NTN”, ZTE, April 2021

R1-2108675, ”RRC parameter changes required for NR-NTN, up to RAN1 #106-e”, Moderator (Thales), August 2021

TS 38.104, Base Station (BS) radio transmission and reception, V16.5.0.

Eng, M., and René Schwarz, "Keplerian Orbit Elements → Cartesian State Vectors," available at https://downloads.rene-schwarz.com/download/M001-Keplerian_Orbit_Elements_to_Cartesian_State_Vectors.pdf.

Eng, M., and René Schwarz, "Cartesian State Vectors → Keplerian Orbit Elements," available at https://downloads.rene-schwarz.com/download/M002-Cartesian_State_Vectors_to_Keplerian_Orbit_Elements.pdf.

X. Lin, S. Rommer, S. Euler, E. A. Yavuz, and R. S. Karlsson, “5G from space: An overview of 3GPP non-terrestrial networks,” to appear in IEEE Communications Standards Magazine, March 2021. Available at arXiv preprint arXiv:2103.09156.

X. Lin, S. Cioni, G. Charbit, N. Chuberre, S. Hellsten, and J.-F. Boutillon, “On the path to 6G: Embracing the next wave of low Earth orbit satellite access,” to appear in IEEE Communications Magazine, April 2021. Available at arXiv preprint arXiv:2104.10533.

Agreements from previous RAN1 meetings

At RAN1#103-e, the following agreements were made:

Agreement:

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

Agreement:

An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.

Agreement:

- In NTN, the network may broadcast

- A common timing offset value

  - FFS details of the common timing offset

- FFS: A common timing drift rate

- Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows:

Where:

is derived from the User specific TA self-estimation

is derived at least from the common timing offset value if broadcasted by the network. The granularity of  and whether  is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed.

- depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2.

- is specified in TS 38.211 section 4.1.

- Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Working assumption:

It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

Agreement:

An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

At RAN1#104-e, the following agreements were made:

Agreement:

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

- FFS: Operation of closed loop and open loop TA control

Agreement:

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

- FFS: Details of the combination of open and closed loop TA control

Conclusion:

It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation

Agreement:

RAN1 should send an LS to RAN4 with the following questions:

Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

Agreement:

- RAN1 to support satellite ephemeris broadcast based at least on one of the following format options:

  - Option 1: Ephemeris format based on satellite position and velocity state vectors

    - FFS: Details on state vectors formats

    - FFS: Details on time reference provisioning/format

  - Option 2: Ephemeris format based on orbital elements

    - FFS: Details on orbital elements formats

    - FFS: Details on time reference provisioning/format

- FFS: Whether down-selection is needed or both options are supported

At RAN1#104bis-e, the following agreements and conclusions were made:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

Agreement:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

Conclusion:

The orbital propagator model to be used at UE side can be left to implementation.

At RAN1#105-e, the following agreements and conclusions were made:

Agreement:

Specifications should support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements.

Agreement:

RAN1 should send an LS to SA3, SA1 and possibly SA3-LI to get more inputs regarding the security/regulatory aspects if the NTN GW/gNB position is broadcast or possible to be derived by the UE with assistance information from the network, and on any aspects related to accuracy of the position.

Conclusion:

The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated by the GW and satellite-payload without any specification impacts in Release 17.

At RAN1#106-e, the following agreements and conclusions were made:

Agreement:

- A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris.

  - FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration.

- FFS: Whether the same validity duration can be applied for Common TA.

Conclusion:

Indication of common post-compensation frequency offset for Uplink is not needed.

Agreement:

Confirm the working assumption on non-extension of TAC 12-bit field in msg2 (or msgB) and that the UE follows the requirements on UL time pre-compensation for Msg1/MsgA transmission as defined by RAN4.

Agreement:

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

Agreement:

In NTN, to avoid that the UE over pre-compensates its TA during RACH procedure, down-select one option from below:

- Option 1: PRACH transmission is delayed by minCP2,GP2

- Option 2: TA margin can be considered, and it is explicitly indicated to the UE

- Option 3: TA margin can be considered, and it is included within the Common TA

- Option 4: UE handles it via implementation

Agreement:

In NR NTN, NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction as follows:

- When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as follows:

NTA=NTA_old+TA. 16.642μ, FFS: the value of NTA_old,

where, TA is the TAC field in msg2/msgB

- When TACs (TA) provided within the MAC CE is received, NTA is updated as follows:

NTA_new=NTA_old+TA-31.16.642μ,

Where, TA is the TAC field receivd in MAC CE  command

Working assumption:

Common TA may include parameter(s) indicating timing drift.

- The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.