3GPP TSG RAN WG1 #106bis-e                              R1-2109967

e-Meeting, October 11th –19th, 2021

Agenda Item:	8.2.6

Source: 	LG Electronics

Title: 	Considerations on channel access mechanism to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

The following WID objectives related to the channel access mechanism were made in RAN#90-e meeting [1]:

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access Study, and if needed specify, energy detection threshold enhancement

In this contribution, we discuss and provide views on the channel access mechanism to support NR above 52.6 GHz, in terms of the channel access mode switching, the introduction of CAPC, CWS adjustment mechanism and Cat-2 LBT to the beam-based LBT/transmission, and the ED threshold enhancement considering the relationship between the LBT beam and transmission beam(s).

- Discussion

- LBT bandwidth

Conclusion: There is no consensus in RAN1 to support the functionality of accessing a carrier if there is interference in part of the carrier in frequency. Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements)

In RAN1#106-e meeting [7], the definition of LBT bandwidth for single-carrier transmission and multi-carrier transmission was agreed and it is corresponding to Alt SC.1 and Alt CA.2 in earlier agreements. Based on the agreement, gNB/UE can transmit a transmission when performing single or multiple LBT over the channel bandwidth(s) (or BWP bandwidth(s)). However, considering the coexistence with the incumbent system (e.g., WiGig) operating in the same band, the maximum LBT bandwidth may need to be up to 2 GHz (or 2.16 GHz). Therefore, in addition to the carrier bandwidth (or BWP bandwidth), the bandwidth of multiple CCs up to 2GHz (or 2.16 GHz) can be supported considering other technology sharing the channel is absent or not.

Proposal #1: The bandwidth of multiple CCs up to 2 GHz (or 2.16 GHz) can be supported considering the coexistence with incumbent system in addition to the carrier bandwidth (or BWP bandwidth) which is already agreed upon.

- COT sharing

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed

According to the agreement in RAN1#106-e [7], if the responding device transmission starts after the maximum gap Y from the end of the initiating device transmission, a Cat-2 LBT is needed before the responding device transmission and it depends on the local regulations and a UE capability. Since Cat-2 LBT is a UE capability unlike NR-U, it may be necessary to define a UE behaviour for LBT type indication when the Cat-2 LBT capability of the UE is unknown because it is before the capability report, such as the initial access. If ChannelAccess-CPext field is 2 bits in DCI format 0_0 and 1_0 even in NR operation on 52.6 GHz, the LBT type indication is required when gNB schedules the Msg3 PUSCH or triggers the PUCCH for Msg4/MsgB by the fallback DCI.

One solution is that configuring allowed entries for DCI format 0_0 and 1_0 by only Cat-1 (Type 2C in NR-U) and Cat-3 or Cat-4 (Type 1 in NR-U) except Cat-2 (Type 2A in NR-U). Another way is to configure entries including Cat-2 LBT and to interpret differently for each UE. For example, if Cat-2 LBT is indicated to UE, the UE having Cat-2 LBT capability performs Cat-2 LBT but the UE that does not have Cat-2 LBT capability performs Cat-3 (or Cat-4 LBT).

Regarding the options for the maximum gap Y, Option 3 (gNB determines Y (for example, according to local regulation) can be supported and the CP extension indication may need to be discussed depending on the value of Y.

Proposal #2: If ChannelAccess-CPext field is kept as 2 bits in DCI format 0_0 and 1_0 for FR2-2 unlicensed band same as in NR-U, it is necessary to define UE behaviour for LBT type indication before identifying the Cat-2 LBT capability of the UE, such as initial access.

Proposal #3: Regarding the options for the maximum gap Y, Option 3 (gNB determines Y (for example, according to local regulation)) can be supported and the CP extension indication may need to be discussed depending on the value of Y.

- Rx assistance

Agreement: For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e - Scheme 1: L1-RSSI based receiver assistance - Resource used for RSSI measurement - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS - FFS: any enhancement needed for ZP-CSI-RS for this purpose (eg., ZP-CSI-RS over all REs in BWP over one or more symbols). - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval - L1-RSSI is reported in an AP-CSI report - L1-RSSI trigger in UL grant - FFS if L1-RSSI trigger can also be carried in DL grant - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed - FFS: How to indicate the measurement beam for L1-RSSI - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc - Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - FFS if the downlink data transmission can be granted with the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission, in which case, the CCA or eCCA is performed for at least the first UL PUCCH/SRS transmission - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - Scheme 3: CCA or eCCA based receiver assistance with new RTS/CTS type transmission - New RTS/CTS-like signaling introduced. - gNB sends RTS-like signaling to UE. UE performs CCA or eCCA and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed CCA or eCCA. After detecting the CTS-like signal, the data transmission happens - Scheme 4: Legacy L3-RSSI with potential enhancements - FFS potential enhancements, e.g., supporting gNB indicating the beam used for UE RSSI measurement, supporting gNB indicating new reference SCS and measurement bandwidths - Note: The schemes listed above are not mutually exclusive and should be discussed separately.

Multiple schemes for receiver assistance were captured during the RAN1#106-e meeting and it seems necessary to narrow down among the various schemes considering the remaining WI schedules.

Agreement: For NR operation with 480 kHz and/or 960 kHz SCS, only value(s) for CSI computation delay requirement 2 are to be defined. - FFS: The specific values

From the above agreement related to CSI computation delay in RAN1#106-e meeting, it was already agreed that only requirement 2 is supported for 480 kHz and/or 960 kHz SCS. Therefore, Scheme 1 is not needed because L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline.

Proposal #4: Rx assistance Scheme 1 is not needed because L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, according to the agreement made in RAN1#106 that for 480 kHz and/or 960 kHz SCS, only value(s) for CSI computation delay requirement 2 are to be defined.

The additional or new mechanism for the receiver assisted LBT is not preferred because the assistance information or feedback mechanism is already supported by the current specification. In this regard, for Scheme 2, the receiver-assistance information can be transmitted implicitly to indicate the LBT outcome by the appearance of the scheduled PUCCH/SRS/PUSCH. However, the explicit receiver-assistance information transmission by the payload bit in PUCCH/PUSCH in Scheme 2 and introduction of new RTS/CTS-like signalling in Scheme 3 are not preferred. For Scheme 4, it can be supported with possible enhancement (e.g., beam specific RSSI, L3-RSSI with supporting new SCS) to relieve the hidden node problem.

Proposal #5: For the receiver to provide assistance, the feedback mechanisms already supported by the current specification such as implicit method in Scheme 2 (appearance of the scheduled PUCCH/SRS/PUSCH) can be considered and it is not preferred to introduce the additional or new mechanism (such as added explicit payload bit in PUSCH/PUCCH or introduction of new RTS/CTS-like signalling in Scheme 3).

- Multi-beam COT

Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

In RAN1#104b-e meeting [5], the clarification on independent per-beam LBT sensing was discussed and the multiple alternatives based on each proposing company were captured. For Alt B (for both SDM and TDM case), we think that when and what conditions need to support Alt B should be clarified first. For example, only the transmitter equipped with multi-TRP may perform the per-beam LBT for different beams simultaneously.

For Alt A-2, it is equivalent to the independent single beam transmission in an each separate COT. Hence, it is not relevant to the multiplexing of multi-beam transmission. For Alt A-3, it seems that it is not aligned with the LBT procedures described in ETSI EN 302 567 regulation. For a given back-off counter, the CCA check cannot be skipped or assumed to be idle based on measurement of a different spatial channel.

For a COT with MU-MIMO (SDM) and TDM of beams transmission, the per-beam LBT sensing should be performed at the beginning of COT for all beam directions to be transmitted within a COT. In other words, the node should complete the eCCA on all of the beam(s) transmitted in COT and then can start transmission. Therefore, Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) should be supported. For a concern on the large LBT latency, the additional single wide beam (or omnidirectional LBT) of Cat-2 LBT can be used after back-to-back eCCA is finished.

Proposal #6: For a COT with MU-MIMO (SDM) and TDM of beams transmission, adopt Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) when independent per-beam LBT sensing at the start of COT.

■ Transmission with multiple candidate directions

For some broadcast signals/channels (e.g. SSB) in NR, multi-beam sweeping transmission is necessary. Depending on the further discussion on the LBT operation for directional transmission, directional CCA at transmitter side may be necessary. In that case, it should be further studied how to perform omni-directional or directional CCA for multiple-beam sweeping transmission.

In addition, if either transmitter or receiver side channel sensing is introduced per transmission direction or reception direction, it should be also discussed how to define CWS management, e.g., per-direction management or across-direction management. It can be also considered how to adjust CWS if omni-directional LBT as well as directional LBT are operated at the same time. To be specific, CWS increment/reset event in a particular direction may affect the CWS managed for other directions. For example, if the CWS for directional LBT increase occurs in a particular direction due to collision, the CWS managed for omni-directional LBT may also be affected and may be increased. Conversely, if the CWS reset event is occurred in omni-directional LBT, the CWS managed in each direction may also be affected and may be reset. Also, if the back-off counter value is also defined per-direction, the same principle can be applied to back-off counter value decrease in Type1 LBT procedure.

Proposal #7: If the directional CCA procedure is introduced the followings points can be considered:

- How to perform the CCA procedure for multiple-beam sweeping transmission

- How to define CWS management (e.g., per-direction or across-direction management)

- How to manage the back-off counter value

■ Tx Burst construction for COT sharing

If the directional LBT is performed with a specific LBT beam using a specific directional antenna, the CCA range should cover the interference range of the transmission beam(s). Within the COT acquired by the LBT beams with specific direction and ED threshold, the transmission of beams that have directions different from the CCA range (due to large transmit power than ED threshold) shall not be allowed to be multiplexed (SDM/TDM) in the COT. For example, if the COT is acquired by the directional LBT with Beam A, the transmission of beam(s) is only allowed to the beam having the same direction as Beam A in the COT. When LBT is performed with wide beam A, it may be necessary to discuss whether to allow transmission of the multiple narrow beams included in the beam A (i.e., multiple beam sweeping transmission) and whether to allow the transmission of beam other than the LBT beam in the COT.

Proposal #8: The relationship between the LBT beam with a specific direction to acquire the COT and the transmission beam(s) allowed to transmit in that COT should be defined considering the relationship between the CCA range of the LBT beam and the interference range of the transmission beam(s).

If the directional LBT is performed to transmit a beamformed transmission, it may be desirable that all DL signals/channels (or UL signals/channels) belonging to the same TX burst have QCL relationship. For example, the gNB performs directional LBT to Beam A to acquire COT and transmits DL signals across four slots. If gNB’s transmission is beamformed to Beam A for the first three slots but the last slot is redirected to Beam B, it may collide with other nodes (e.g., WiGig AP) that has transmitted in the direction of beam B after the success of directional LBT to Beam B. The same principle can be applied to the COT sharing between the gNB and UE in the COT acquired by the directional LBT. When the spatial relationship between the DL signals/channels and the UL signals/channels are associated, it is desirable that the COT acquired by directional LBT is shared between the DL signals/channels and UL signals/channels having spatial QCL relationship.

Proposal #9: It would be beneficial for coexistence that channel occupancy acquired by directional LBT is shared only for DL and UL signals/channels having spatial QCL relationship.

- Directional LBT

Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support a multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded

In RAN1#106-e meeting, there were discussions about whether/how to specify the relationship between sensing beam and transmission beam, and multiple options are captured for Alt 1 and Alt 2. Regarding this issue, the relative relationship between all applicable sensing beams and the transmission beam can be defined in RAN1 by using the beam correspondence and the QCL/TCI framework in RAN1 without RAN4 involvement. Therefore, we support Alt 2 that extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s).

Proposal #10: For the directional LBT, support Alt 2 that extends the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s).

On gNB side sensing beam selection for a DL transmission beam, supporting Option 1 (the selection of eligible sensing beam for a transmission beam is left for gNB implementation) is sufficient. Regarding UE side sensing beam selection for a UL transmission beam, the beam correspondence (BC) capability is a mandatory feature for a UE since Rel-15 NR (refer to below UE feature table). Therefore, there are two types of UE that one is satisfying BC with beam sweeping and another one is satisfying BC without beam sweeping.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| beamCorrespondenceWithoutUL-BeamSweeping Indicates how UE supports FR2 beam correspondence as specified in TS 38.101-2 [3], clause 6.6. The UE that fulfils the beam correspondence requirement without the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall set the field to supported. The UE that fulfils the beam correspondence requirement with the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall not report this field. | Band | Yes | N/A | FR2 only |

For the FFS point regarding how and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework, Option 2 should be supported. To be specific, for an indication of sensing beam for PUSCH (including CG-PUSCH) and PUCCH, the information about the sensing beam can be jointly encoded or separately indicated together with SRI or TCI indication for the transmission beam in the DCI (activation DCI for CG-PUSCH). For a sensing beam for SRS, the sensing beam can be configured to each SRS resource set/group or each SRS resource together with the configuration of QCL assumption for SRS resource.

Proposal #11: On gNB side sensing beam selection for a DL transmission beam, support Option 1 (the selection of eligible sensing beam for a transmission beam is left for gNB implementation).

Proposal #12: For the FFS point regarding how and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework, Option 2 should be supported.

- ED threshold enhancement

Working assumption: - For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

For Pout in EDT determination, we support the previous version of the working assumption without the updates (or just adopt the mean EIRP which is the definition in ETSI EN 302 567 as is). In addition, we should consider the following situation when the determining the EDT: If multiple UL transmissions are scheduled within a COT, the transmissions with an EIRP larger than the max EIRP used for the initial EDT calculation may be suddenly scheduled in the middle of the COT.

In this case, the maximum transmission power of UL transmission transmitted within the remaining COT may be limited to the max EIRP or less so as not to exceed the max EIRP used in the EDT calculation for initial COT acquisition. Alternatively, if the UL power to be transmitted within the remaining COT is larger than the max EIRP used in the EDT calculation for initial COT acquisition, UE can simply drop the corresponding UL transmission.

Proposal #13: Confirm the working assumption on Pout definition in RAN1 #104bis-e without the change and it needs to consider when the determining the EDT: If multiple UL transmissions are scheduled within a COT, the transmissions with an EIRP larger than the max EIRP used for the initial EDT calculation may be suddenly scheduled in the middle of the COT.

For the relationship between the LBT beam and the transmission beam(s), if the directional LBT is performed with a specific LBT beam (i.e., Rx beam pattern) using a specific directional antenna, the CCA range should cover the interference range of the transmission beam(s). In other words, the value of ED threshold and the CCA range should be determined considering the interference range of the transmission beam(s) in the COT. Within the COT acquired by the LBT beams with specific direction and ED threshold, the transmission of beams that have directions different from the CCA range (due to large transmit power than ED threshold) shall not be allowed to be multiplexed (SDM/TDM) in the COT. For example, if the COT is acquired by the directional LBT with relatively narrow beam A, the transmission of beam(s) is only allowed to the beam having the same direction as Beam A in the COT. When LBT is performed with wide beam A, it can allow transmission of the multiple narrow beams included in the beam A in TDM/SDM manner. In this regard, the ED threshold can be further adjusted by reflecting the relationship between the sensing beam and transmission beam and it may be closely related to the beam correspondence between Tx/Rx beams.

The beam correspondence requirements may be different depending on whether the UE capability supports beamCorrespondenceWithoutUL-BeamSweeping or not. A UE supporting this capability can satisfy requirements (for minimum peak EIRP and spherical coverage) without UL beam sweeping and beam management such as beam indication from gNB. On the other hand, for a UE not supporting the capability, the requirements (for minimum peak EIRP and spherical coverage) must be satisfied through the beam management procedure, and additionally, without beam management, a requirement relaxed by 3 dB must be satisfied. Therefore, it is necessary to adjust the ED threshold value differently depending on whether the UE supports beamCorrespondenceWithoutUL-BeamSweeping or not. To be specific, the ED threshold for a UE not supporting the capability can be lower than that for a UE supporting the capability. In addition, for the UE without this capability, the ED threshold value can be adjusted depending on the presence or absence of a beam management procedure.

Proposal #14: The ED threshold can be further adjusted by reflecting the beam correspondence capability/requirement of UE. For pseudo-omni beam, the adjustment to ED threshold is not necessary regardless of the beam correspondence capability.

- No LBT

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode - Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

In the RAN1#102-e meeting [2], it was agreed that both channel access with LBT mechanism(s) and a channel access mechanism without LBT are supported to initiate a channel occupancy for gNB and UE. In addition, in RAN1#105-e meeting [6], the UE can be configured with the operating mode between LBT mode and no-LBT mode by both cell-specific and UE-specific gNB configurations for regions where LBT is not mandated.

When the UE is operating in no-LBT mode, the PUSCH reception failure at gNB side may be occurred consecutively due to the collision with adjacent UE’s transmission in a high interference environment. In this case, performing the LBT before the channel access can be beneficial to prevent further collisions. Therefore, supporting the mechanism for operating mode switching from no-LBT mode to LBT mode should be supported additionally for the interference mitigation scheme on the no-LBT mode.

When indicating the operating mode to the UE, the gNB can also indicate whether the operating mode can be switched by UE when certain conditions are met. Alternatively, it is also possible that the UE can change the operating mode only by the gNB indication but UE can request switching to gNB when certain conditions are met. For example, UE can switch the operating mode from no-LBT mode to LBT mode only when it can assume high interference environment e.g., by estimating interference level or by using HARQ information of (consecutive) failure transmissions. Once the operating mode is switched to LBT mode, the timer can be started and the operating mode can be switched to the no-LBT mode when the timer is expired or at least one of the specific conditions (e.g., when ACKs were received consecutively for the PUSCH transmitted with LBT) for switching the operating mode is met. This switching mechanism between the LBT mode and No-LBT mode can also be beneficial for coexistence with different RAT/operator or for high traffic load scenario.

Proposal #15: For regions where LBT is not mandated, the mechanism for switching between the no-LBT mode and LBT mode should be supported and specified at least for UL, and the channel access mode switching between no-LBT mode and LBT mode can be determined e.g., based on the consecutive decoding success or failure or interference measurement.

- Short control signalling and contention exempt transmission

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

In RAN1#105-e meeting [6], it was agreed that the contention exempt short control signalling rules can be applicable to the transmission of msg1 for the 4 step RACH and msgA for the 2-step RACH for all supported SCS. Regarding the interpretation of the regulation, the proponents of Alt-2 assert the restriction of 10% over any 100ms interval should be applied from one UE perspective while the proponents of Alt-1 claim this interpretation is a misuse of the exemption that is introduced in regulation for short control signalling. Considering the number of PRACH slots and ROs for a single PRACH configuration in Table 6.3.3.2-4 in TS 38.211, the total time resources can easily far exceed the restriction of 10% over 100ms interval. Moreover, it seems that the interpretation of regulation such as Alt-2 is likely to cause coexistence issues with the incumbent system operating in the same band. Therefore, the requirement of 10% over any 100ms interval should be applied to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

Observation #1: The interpretation of regulation for 10% over any 100ms interval restitution from one UE perspective (Alt-2) is likely to cause coexistence problems with the incumbent system operating in the same band.

In the case of Alt-1, the gNB can explicitly indicate whether a short control signing rule is applicable or not to the configured msg1/msgA resources by the RACH configuration. Alternatively, it is also possible that the UE can determine whether a short control signing rule can be applied to msg1/msgA transmission by checking duty cycle for the configured ROs (or ROs and POs) resources implicitly within the observation period. Meanwhile, a reference point for the observation period (i.e., 100ms interval) may be indicated to determine whether a short control signing rule is applicable or not to the configured msg1/msgA resources. For example, SFN=0 can be considered a reference point when the reference point is not explicitly configured, and it can be assumed that the 100ms observation period to determine whether the duty cycle conditions are met is periodically configured in the time-domain.

Proposal #16: Whether a short control signing rule is applicable or not to the configured msg1/msgA resources can be explicitly indicated by the gNB or can be implicitly determined by the UE.

- CWS and CAPC

Agreement: Use the CCA check procedure in EN 302 567 (per RAN1 understanding as from RAN1 #102-e) as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work. - Whether CAPC and contention window adjustment mechanisms are introduced - Whether ED threshold change is needed, e.g., due to changes in bandwidth, beamforming gain etc. - Whether contention window range needs to be adjusted

In RAN1#103-e meeting [3], the agreement was made that the CCA check procedure in EN 302 567 as the baseline for channel access for 60GHz band when LBT is applied and whether to introduce CAPC and CWS adjustment mechanism will be further discussed during WI phase.

Because the purpose of CAPC and CWS adjustment are to prioritize high priority traffic and resolve the collision between the transmissions, the introduction of CAPC and CWS adjustment mechanism can be beneficial in highly congested scenario. Moreover, considering the fair coexistence with the incumbent system (e.g., WiGig) operating in the above 52.6GHz, it is necessary to consider the introduction of CAPC and CWS adjustment procedure. The procedures specified for the CAPC and CWS adjustment mechanism in Rel-16 NR-U can be reused as baseline for operation in the 60 GHz band.

Proposal #17: The channel access priority classes (CAPC) can be introduced for NR above 52.6 GHz to differentiate the channel access probabilities for different channels and traffic.

Proposal #18: To reduce the probability of collision, the contention window adjustment (CWS) procedure similar to Rel-16 NR-U can be adopted, and it is also necessary to discuss the relationship of the CWS and back-off counter values between wide beam LBT and independent per-beam LBT for multi-beam COT.

- Conclusions

In this contribution, the channel access mechanism to support NR in high frequency range from 52.6 GHz to 71 GHz was discussed, and the followings are proposed.

Proposal #1: The bandwidth of multiple CCs up to 2 GHz (or 2.16 GHz) can be supported considering the coexistence with incumbent system in addition to the carrier bandwidth (or BWP bandwidth) which is already agreed upon.

Proposal #2: If ChannelAccess-CPext field is kept as 2 bits in DCI format 0_0 and 1_0 for FR2-2 unlicensed band same as in NR-U, it is necessary to define UE behaviour for LBT type indication before identifying the Cat-2 LBT capability of the UE, such as initial access.

Proposal #3: Regarding the options for the maximum gap Y, Option 3 (gNB determines Y (for example, according to local regulation)) can be supported and the CP extension indication may need to be discussed depending on the value of Y.

Proposal #4: Rx assistance Scheme 1 is not needed because L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, according to the agreement made in RAN1#106 that for 480 kHz and/or 960 kHz SCS, only value(s) for CSI computation delay requirement 2 are to be defined.

Proposal #5: For the receiver to provide assistance, the feedback mechanisms already supported by the current specification such as implicit method in Scheme 2 (appearance of the scheduled PUCCH/SRS/PUSCH) can be considered but it is not preferred to introduce the additional or new mechanism (such as added explicit payload bit in PUSCH/PUCCH or introduction of new RTS/CTS-like signalling in Scheme 3).

Proposal #6: For a COT with MU-MIMO (SDM) and TDM of beams transmission, adopt Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) when independent per-beam LBT sensing at the start of COT.

Proposal #7: If the directional CCA procedure is introduced the followings points can be considered:

- How to perform the CCA procedure for multiple-beam sweeping transmission

- How to define CWS management (e.g., per-direction or across-direction management)

- How to manage the back-off counter value

Proposal #8: The relationship between the LBT beam with a specific direction to acquire the COT and the transmission beam(s) allowed to transmit in that COT should be defined considering the relationship between the CCA range of the LBT beam and the interference range of the transmission beam(s).

Proposal #9: It would be beneficial for coexistence that channel occupancy acquired by directional LBT is shared only for DL and UL signals/channels having spatial QCL relationship.

Proposal #10: For the directional LBT, support Alt 2 that extends the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s).

Proposal #11: On gNB side sensing beam selection for a DL transmission beam, support Option 1 (the selection of eligible sensing beam for a transmission beam is left for gNB implementation).

Proposal #12: For the FFS point regarding how and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework, Option 2 should be supported.

Proposal #13: Confirm the working assumption on Pout definition in RAN1 #104bis-e without the change and it needs to consider when the determining the EDT: If multiple UL transmissions are scheduled within a COT, the transmissions with an EIRP larger than the max EIRP used for the initial EDT calculation may be suddenly scheduled in the middle of the COT.

Proposal #14: The ED threshold can be further adjusted by reflecting the beam correspondence capability/requirement of UE. For pseudo-omni beam, the adjustment to ED threshold is not necessary regardless of the beam correspondence capability.

Proposal #15: For regions where LBT is not mandated, the mechanism for switching between the no-LBT mode and LBT mode should be supported and specified at least for UL, and the channel access mode switching between no-LBT mode and LBT mode can be determined e.g., based on the consecutive decoding success or failure or interference measurement.

Observation #1: The interpretation of regulation for 10% over any 100ms interval restitution from one UE perspective (Alt-2) is likely to cause coexistence problems with the incumbent system operating in the same band.

Proposal #16: Whether a short control signing rule is applicable or not to the configured msg1/msgA resources can be explicitly indicated by the gNB or can be implicitly determined by the UE.

Proposal #17: The channel access priority classes (CAPC) can be introduced for NR above 52.6 GHz to differentiate the channel access probabilities for different channels and traffic.

Proposal #18: To reduce the probability of collision, the contention window adjustment (CWS) procedure similar to Rel-16 NR-U can be adopted, and it is also necessary to discuss the relationship of the CWS and back-off counter values between wide beam LBT and independent per-beam LBT for multi-beam COT.

- References

- RP-202925, “Revised WID: Extending current NR operation to 71 GHz”, RAN#90-e, CMCC.

- RAN1#102-e Chairman’s note

- RAN1#103-e Chairman’s note

- RAN1#104-e Chairman’s note

- RAN1#104b-e Chairman’s note

- RAN1#105-e Chairman’s note

- RAN1#106-e Chairman’s note