3GPP TSG RAN WG1 #106b-e			R1-2109982

e-Meeting, October 11th – 19th , 2021

Agenda Item:	8.7.2

Source: 	LG Electronics

Title: 	Discussion on DCI-based power saving adaptation during DRX Active Time

Document for:	Discussion and decision

- Introduction

According to WID [1] on UE power saving enhancements, followings will be studied and specified in this WI;

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized

Based on WID, one of Rel-17 power saving techniques is enhancements on power saving techniques for connected-mode UE, more specifically extension(s) to Rel-16 DCI based power saving adaptation during DRX Active Time for an active BWP.

In NR, a UE performs channel estimations and blind decodes on candidates given by CORESETs and search space set configurations. The channel estimation and blind decoding are one of the main factors of UE’s power consumption on connected-mode. From the power saving perspective, it is useful to reduce unnecessary channel estimations and blind decodes. Candidate techniques for decreasing power consumption on PDCCH monitoring were narrowed down to PDCCH monitoring skipping and SS set group (SSSG) switching. Both techniques are accepted to strive for a common design for DCI-based PDCCH monitoring adaptation.

- Discussion

In this contribution, we discuss considerations for enhancements on power saving techniques for connected-mode UE.

  - Configuration of at most 2 bit indication

In the last meeting, the agreements were made that at most 2 bit indication in DCIs can be specified and package 1 of common design for DCI-based PDCCH monitoring adaptation is selected.

Agreement - At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell - FFS: the bit size of the indication is configurable - FFS: bit mapping to the PDCCH monitoring behaviour - FFS: details of indication of multiple cells case Agreement Select either package 1 or package 2 - Package 1  UE behavior after receiving PDCCH indication of monitoring adaptation can be one of the followings, - Working Assumption: Beh 1: PDCCH skipping is not activated - Beh 1A: PDCCH skipping means stopping PDCCH monitoring for a duration X - FFS the possible values for X - FFS: Whether and how to support more than one skipping duration(s) - FFS: whether to continue monitoring PDCCH scrambled by C-RNTI for Type 0/1/1A/2 CSS or not - Beh 2: stop monitoring SS sets associated with SSSG#1 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#0 (legacy behaviour) - Beh 2A: stop monitoring SS sets associated with SSSG#0 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#1 (legacy behaviour) - Working Assumption: Beh 2B(if confirmed): stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2 (if confirmed)  Note: The number of supported SSSG is left to UE feature discussion.  FFS: UE capability of supported UE behaviors  Indication of Beh 1A when SSSG(s) are not configured is supported.  Working assumption: Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported  FFS: Indication of Beh 1A when three SSSG(s) (if supported) are configured  Y bits is configured for scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) indicating PDCCH schedules data and also PDCCH monitoring adaptation - FFS how the UE behavior(s) defined above mapping to Y bits  Note: at most Y = 2  Working Assumption at most 3 SSSGs is supported to be configured. - FFS: whether or how SSSG can be configured to be monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - FFS: whether or how non-default SSSG to another non-default SSSG  FFS details of timer(s) for switching between SSSG(s) - UE fallbacks to default SSSG (i.e., SSSG#0) after timer expiration. - R16 timer for SSSG switching and the corresponding behavior is as baseline  FFS whether the timer(s) is configured per SSSG, or per BWP or other approaches.  FFS whether the skipping duration(s) is configured per SSSG, per BWP, or other approaches.  FFS PDCCH monitoring adaptation indicated by non-scheduling DCI  PDCCH based monitoring adaptation is limitedapplied to USS and type-3 CSS.

Regarding FFS point of monitoring PDCCH scrambled by C-RNTI for Type 0/1/1A/2-PDCCH CSS set of the agreement from the last meeting, we need to define ‘PDCCH skipping’ explicitly. In general understanding, PDCCH skipping means that a UE skips all the PDCCHs in DRX Active Time.

According to TS38.321, the RNTIs controlled by a DRX functionality are listed as follows:

- the MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, and AI-RNTI.

Which means some RNTIs including SI-RNTI, RA-RNTI, MsgB-RNTI, and P-RNTI are not controlled by a DRX functionality. If a UE is indicated by a DCI to shut down all the PDCCH monitoring, it is unclear whether the UE should keep monitoring PDCCH candidates for a DCI scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, and P-RNTI or not. Considering that monitoring adaptation is configured for C-DRX, it seems appropriate that a UE should monitor PDCCH candidates for a DCI scrambled by RNTIs not controlled by a DRX functionality despite of skipping duration. And, in our understanding, a UE should stop monitoring PDCCH candidates for a DCI with CRC scrambled by C-RNTI (and other RNTIs) in skipping duration.

SI-RNTI, RA-RNTI, MsgB-RNTI, and P-RNTI are used to scramble DCI format for a Type0/0A/1/2-PDCCH CSS set. According to TS38.213, UE’s monitoring PDCCH candidates for a DCI with CRC scrambled by C-RNTI (and MCS-C-RNTI, CS-RNTI) is specified as follows:

- the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

A UE monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set. It is specified for SSSG switching that a UE can be provided a group index for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. In other words, Type0/0A/1/2-PDCCH CSS sets do not belong to any group and are always be monitored by a UE.

Observation 1: Based on TS38.213 and TS38.321, legacy UE monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set for a duration.

Observation 2: If monitoring behavior of Observation 1 is applied without modification to a UE indicated to skip for a duration, it will decrease power saving efficiency of monitoring adaptation.

With regard to enhancement of Rel-16 SSSG switching for power saving, emulating PDCCH skipping functionality by supporting SSSG switching which is related to Beh 2B if SSSG#2 is an ‘empty’ group, may not be possible. Even though a UE is indicated to monitor an ‘empty’ group where no SS set(s) is configured, a UE should monitor a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set. Thus, SSSG switching to ‘empty’ group and PDCCH skipping does not indicate UE’s identical PDCCH monitoring activity.

Observation 3: Supporting SSSG switching to emulate PDCCH skipping functionality, i.e. Beh 2B, cannot be possible if PDCCH skipping shut down UE’s monitoring PDCCH candidates for a DCI with CRC scrambled by RNTIs controlled by a DRX functionality.

Thus, we need to define explicitly UE’s monitoring behaviors regarding PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set after receiving indication of PDCCH skipping. In order to follow the original intention of skipping meaning that a UE skips all the PDCCHs in DRX Active Time, a UE should stop monitoring PDCCH candidates for a DCI with CRC scrambled by C-RNTI (and other RNTIs) in skipping duration. Monitoring PDCCH candidates for a DCI scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, and P-RNTI cannot be stopped because these RNTIs are not controlled by DRX functionality. However, it has little impact on UE’s skipping. In the normal connected-mode DRX situation a UE rarely monitors PDCCH candidates for a DCI scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, and P-RNTI. Therefore, if a UE should not monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set for a duration after receiving indication of PDCCH skipping, the UE seems to stop all the PDCCH monitoring. If a situation that a UE should monitors PDCCH candidates for a DCI scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, and P-RNTI happens, it is more appropriate that a UE should monitor PDCCH candidates and put an end to the situation immediately.

Proposal 1: After receiving indication of PDCCH skipping, a UE should not monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set for a duration.

At most 2 bit indication in self-scheduling DCIs can be used for indicating switching and/or skipping, separately or jointly. Regarding bit mapping to the PDCCH monitoring behavior, joint indication of switching and skipping should be supported. Joint indication has following advantages.

If joint indication is supported, a network can choose and indicate either switching or skipping considering expected traffic after scheduling. If no data traffic is expected for a duration after scheduling, a network can send a DCI including scheduling and indication of skipping. If the network expects no or very little traffic but not wants a UE to keep monitoring PDCCH sparsely, the network can send a DCI including scheduling and indication of switching to SSSG#1 where SS sets with long periodicities are. Joint indication makes it possible to indicate monitoring adaptation flexibly by the network. The flexibility can increase power saving efficiency of monitoring adaptation with minimized latency issues.

Proposal 2: Support joint indication of PDCCH skipping and SSSG switching for bit mapping to the PDCCH monitoring behavior of at most 2 bit indication in self-scheduling DCIs.

As the simplest way of common design for SSSG switching and PDCCH monitoring skipping, we suggest to introduce a DCI field which is composed of 1-bit flag and 1-bit contents where 1-bit flag is used for distinguishing between PDCCH skipping and SSSG switching. The first bit of monitoring adaptation field in a DCI is 1-bit flag indicating PDCCH skipping if the flag is set to 0 and SSSG switching if it is set to 1, or vice versa. In case of PDCCH skipping, contents after 1-bit contents in a DCI could be used for indicating Beh 1 or Beh 1A. In case of SSSG switching, contents after 1-bit contents in a DCI could be used for indicating Beh 2 or Beh 2A. 2-bit of indication which is composed of1-bit flag and 1-bit contents can used to joint indication of monitoring adaptation. Or, only 1-bit contents of switching or skipping can be used separately.

Introducing a DCI field where UE behavior corresponding to each state is configured by RRC signalling (without 1-bit flag) could be another option. Monitoring adaptation by RRC configuration allows a UE to perform more flexible behavior. Without the constraint of the same bit allocation for skipping and switching indications, the network can assign more states to either skipping or switching. In addition to simple indication of skipping durations or SSSG indexes, more stepwise behavior can be indicated. For example, skipping can be indicated with {Beh 1 for X slots, Beh 1A for Y slots}. It means that a UE continues to monitor PDCCH during X slots after the slot the UE detects the DCI, and then the UE skips monitoring PDCCH during Y slots. Switching can be indicated with {duration A, SSSG index B}. It means that a UE start monitoring PDCCH according to SS sets with group index B, which can be one of Beh 2, Beh 2A, or Beh 2B, during A slots after the slot the UE detects the DCI and application delay. Furthermore, combined indication of skipping and switching could be configured such as {Beh 1A for Y slots, duration A, SSSG index B}. That is, a UE stops monitoring PDCCH during Y slots and after that, monitors PDCCH according to SS set with group index B during A slots.

Proposal 3: Support the following design for DCI-based PDCCH monitoring adaptation for the 2-bit joint indication:

    - 1-bit flag distinguishing between PDCCH skipping and SSSG switching

    - UE behavior corresponding to each state configured by RRC signaling

At most 2 bit can be configured for DCI formats respectively. For example, the bit fields in DCI format x_1 have relatively large margin, thus, 2 bit indication can be supported. In DCI format x_2, designed to be flexible, the size of the indication field does not need to be fixed. It can be variable at the discretion of a network. For DCI format x_2, either 2 bit or 1 bit can be supported and even 0 bit is possible. We should carefully design at most 2 bit indication field for self-scheduling DCIs considering many possibilities.

Proposal 4: Bit mapping to the monitoring behavior and/or bit size of indication of monitoring adaptation can be differently configured for each DCI format.

Proposal 5: 0, 1, and 2 bit indication of monitoring adaptation can be configured for DCI format x_2 via higher layer signal.

  - DCI format 2_6 triggering PDCCH monitoring adaptation

In the last meeting, we agreed to adopt DCI formats 0_1, 1_1, 0_2, and 1_2 as indicating PDCCH monitoring adaptation as follows:

Agreement: - PDCCH schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping for a duration is supported. - At least DCI format(s) 1-1, 0-1, 1-2 and 0-2 can be used for the indication(s)

In Rel-16, new DCI format (i.e., DCI format 2_6) was introduced to indicate whether to wake up or not on next long DRX cycle. Abbreviation “DCP” is used in RAN2 specifications for the RAN1 terminology “DCI with CRC scrambled by PS-RNTI”. It means that the UE is indicated to monitor or skip monitoring all configured SS sets in the corresponding on-duration by wake-up message from the DCI. In addition to wake-up indication, it can be advantageous to inform the UE in advance whether monitoring adaptation techniques discussed in Rel-17 are applied at the beginning of DRX Active Time.

When the network sends a wake-up message (DCP) to a UE, it can be assumed that there is specific data scheduling for the UE. The network can narrow down candidates of the SS set that the UE should monitor. Therefore, the UE doesn’t have to monitor all configured SS sets, and can monitor only e.g., a subset of them indicated by DCP when starting DRX on-duration. This subset of SS sets can be a SSSG. In this case, it can be beneficial to adjust the number of candidates of the SS sets to be monitored by the UE from the power saving perspective. Thus, DCP outside DRX Active Time can be introduced for indicating PDCCH monitoring adaptation inside DRX Active Time for power saving.

For indicating PDCCH monitoring adaptation, an additional field of DCI format 2_6 may be needed or existing field (e.g. SCell dormancy field for UEs not configured with carrier aggregation) can be reused. SSSG index or switching flag can be added and switching duration may also be needed. We need to discuss whether to support the DCI format 2_6 that indicates PDCCH monitoring adaptation inside DRX Active Time and if supported, which modification of existing DCI format 2_6 is needed.

DCI format 2_6 can be a potential DCI format indicating PDCCH monitoring adaptation inside DRX Active Time. We can reuse the field of DCI format 2_6 (i.e. wake-up indication and SCell dormancy) and network signaling overhead can be reduced. Unlikely scheduling DCIs, DCI format 2_6 for one UE has only maximum 6-bits of field, therefore it is suitable for only indicating monitoring adaptation. Since DCI format 2_6 is not monitored inside DRX Active Time in Rel-16 system, it may be good to introduce a monitoring window for Rel-17 power saving. It might help to address the potential issue regarding BD/CCE limits. Therefore, we need to discuss whether and how to define the monitoring window for DCI format 2_6 inside DRX Active Time.

Proposal 6: Support PDCCH monitoring adaptation indicated by a DCI format 2_6 inside/outside DRX Active Time.

    - Discuss whether and how to define the monitoring window for DCI format 2_6 inside DRX Active Time.

  - Application delay and HARQ/retransmission

Before a UE starts to skip PDCCH or switch SSSG, the UE needs time to decode DCI carried the signalling. There were several suggested options of application delay in the last meeting. Suggested options of application delay can be into two forms: one is time-based application simply in the form of slots which are based on Rel-16 SSSG switching and cross-slot scheduling, and the other is that the command applied after UE’s UL transmission.

For application delay, we should consider missing case of DCI indicating PDCCH monitoring adaptation. For example, a network indicated SSSG switching from index 0 to index 1 and sends a scheduling DCI on SS set in group index 1, however, a UE may not receive a DCI indicating SSSG switching and therefore cannot monitor PDCCH candidates in the SS set in group index 1. If SSSG switching is applied after UE’s UL transmission (PUSCH or ACK) which gives confirmation to the network that the UE successfully received the DCI.

In case of PDCCH skipping, missing case of DCI is not problematic. For example, a network indicated PDCCH skipping and do not transmit any DCI for a skipping duration. A UE may not receive a DCI indicating PDCCH skipping and keep monitoring PDCCH candidates in SS sets. However, it is not the problem in terms of data scheduling, it is just a little loss in terms of UE’s power consumption. The problem is how we can handle HARQ/retransmission when a UE receives PDCCH skipping command. Giving enough application delay may be the solution, but it is difficult to get an accurate reference point as well as not knowing exactly how much application delay is enough. Thus, PDCCH skipping applied after UE’s UL transmission (PUSCH or ACK) can be a suitable option.

Contrary to the previous contribution, time-based application delay is appropriate for monitoring adaptation signalled by a DCI without scheduling information. When the network sends monitoring adaptation signalling to a UE without scheduling information, it can be assumed that there is no or less data scheduling for the UE. As there is no UL transmission and scheduled data, a UE applies monitoring adaptation after a few slots required for a UE to decode a DCI. However, missing case of DCI indicating SSSG switching still be an issue.

Proposal 7: Application delay enabling the UE and network to handle missing case of DCI indicating SSSG switching should be supported.

Proposal 8: Consider different application delay for two cases:

    - Command applied after UE’s UL transmission (PUSCH or ACK) if monitoring adaptation is indicated by a DCI with scheduling information.

    - Time-based application delay if monitoring adaptation is indicated by a DCI without scheduling information.

  - Implicit PDCCH monitoring adaptation triggered by SR and RACH

A UE can transmit SR even though the UE is indicated to adapt PDCCH monitoring. In this case, several issues may arise. For example, if a UE is indicated to skip all PDCCH monitoring, there may be no SS sets for the UE to expect a UL grant. Or if a UE is indicated to monitor PDCCH according to SS sets with group index 0 where periodicities of SS sets are generally long, there can be latency issue of receiving a UL grant.

We introduce a monitoring window for a UL grant regarding SR that allows changes to UE’s current monitoring adaptation behavior. For example, a UE monitors all the SS sets in the window regardless of indicated monitoring adaptation. For maintaining power saving as much as possible, a UE monitors specific configured SS sets or SSSG not all the SS sets. We can refer to a RAR window for the configuration of the monitoring window for a UL grant regarding SR.

The situation can be similar in case of UE transmitting PRACH except that a monitoring window is already specified as a RAR window. We only introduce UE’s behavior that the UE monitors a specific SS set which is type1-PDCCH CSS set in case of CBRA or configured for detecting Msg2 RAR with C-RNTI in case of CFRA, regardless of indicated monitoring adaptation during DRX Active Time.

Proposal 9: Support implicit PDCCH monitoring adaptation triggered by SR and RACH

    - Discuss whether and how to define a monitoring window for a UL grant regarding SR

- Conclusion

In this contribution, reduced PDCCH monitoring for RedCap UEs is discussed, and followings are observed and proposed;

Observation 1: Based on TS38.213 and TS38.321, legacy UE monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set for a duration.

Observation 2: If monitoring behavior of Observation 1 is applied without modification to a UE indicated to skip for a duration, it will decrease power saving efficiency of monitoring adaptation.

Observation 3: Supporting SSSG switching to emulate PDCCH skipping functionality, i.e. Beh 2B, cannot be possible if PDCCH skipping shut down UE’s monitoring PDCCH candidates for a DCI with CRC scrambled by RNTIs controlled by a DRX functionality.

Proposal 1: After receiving indication of PDCCH skipping, a UE should not monitors PDCCH candidates for a DCI with CRC scrambled by C-RNTI in a Type0/0A/1/2-PDCCH CSS set for a duration.

Proposal 2: Support joint indication of PDCCH skipping and SSSG switching for bit mapping to the PDCCH monitoring behavior of at most 2 bit indication in self-scheduling DCIs.

Proposal 3: Support the following design for DCI-based PDCCH monitoring adaptation for the 2-bit joint indication:

    - 1-bit flag distinguishing between PDCCH skipping and SSSG switching

    - UE behavior corresponding to each state configured by RRC signaling

Proposal 4: Bit mapping to the monitoring behavior and/or bit size of indication of monitoring adaptation can be differently configured for each DCI format.

Proposal 5: 0, 1, and 2 bit indication of monitoring adaptation can be flexibly configured for DCI format x_2.

Proposal 6: Support PDCCH monitoring adaptation indicated by a DCI format 2_6 inside/outside DRX Active Time.

    - Discuss whether and how to define the monitoring window for DCI format 2_6 inside DRX Active Time.

Proposal 7: Application delay enabling the UE and network to handle missing case of DCI indicating SSSG switching should be supported.

Proposal 8: Consider different application delay for two cases:

    - Command applied after UE’s UL transmission (PUSCH or ACK) if monitoring adaptation is indicated by a DCI with scheduling information.

    - Time-based application delay if monitoring adaptation is indicated by a DCI without scheduling information.

Proposal 9: Support implicit PDCCH monitoring adaptation triggered by SR and RACH

    - Discuss whether and how to define a monitoring window for a UL grant regarding SR

- Reference

- RP-200938	Revised WID UE Power Saving Enhancements for NR