3GPP TSG RAN WG1 #106-bis-e	R1-2110262

e-Meeting, Oct 11th – 19th, 2021

Agenda item:		8.15.2

Source:	Nordic Semiconductor ASA

Title:					Timing relationship enhancements

Document for:		Discussion and Decision

# Introduction

RAN#92 approved WID on NTN IoT in [1]. One of the RAN1 objectives are

Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763: - Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763 - UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD - Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

# Discussion on TA and K_offset

In terrestrial deployments of eMTC, TA is limited to less than 1ms and for that reason guard periods for type-B half-duplex FDD operation are specified in TS 36.211 as:

For type B half-duplex FDD operation, guard periods, each referred to as a half-duplex guard subframe, are created by the UE by

- not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

- not receiving a downlink subframe immediately following an uplink subframe from the same UE.

Guard periods as defined above may not be applicable to NTN, since TA is typically > 1 ms. However, eNB should know subframes, where UE is not expected to monitor PDCCH or to receive PDSCH. In NTN, when switching from DL to UL, a guard period should start at subframe n + Koffset – 1 - TAceil, where n is a subframe where UL transmission starts as per current specification, and TAceil is timing advance rounded up to number of subframes. When switching from UL to DL, a guard period should end at subframe n + Koffset + N + 1 – TAfloor, where n is a subframe where UL transmission starts as per current specification, N is the length of the UL transmission, and TAfloor is timing advance rounded down to number of subframes.

Proposal-1: For eMTC and NB-IoT NTN, when switching from DL to UL, a guard period starts at subframe n + Koffset – 1 – TAceil, where n is a subframe where UL transmission starts as per current specification, and TAceil is timing advance rounded up to number of subframes. When switching from UL to DL, a guard period ends at subframe n + Koffset + N + 1 – TAfloor, where N is the length of the UL transmission, and TAfloor is timing advance rounded down to number of subframes.

The eNB may configure cell-specific Koffset,common that would accommodate the largest possible UE-specific TA component possible in a cell and that could be included in SIB1. A cell-specific Koffset,common is used during initial access.

Proposal-2: Cell-specific Koffset,common is at least indicated in SIB1

In MSG3, UE should report the UE-specific TA and in MSG4, an eNB configures dedicated UE-specific Koffset,UE. The eNB may use the knowledge of total TA and determine minimum Koffset, UEmin from total TA, and may configure UE-specific Koffset,UE of its choice to UE such that Koffset, UEmin < Koffset,UE.

Proposal-3: UE reports its UE-specific TA at least in MSG3 and eNB configures UE-specific Koffset,UE that is larger than total TA at least in MSG4.

# Conclusions

We have the following observations and proposals:

Proposal-1: For eMTC and NB-IoT NTN, when switching from DL to UL, a guard period starts at subframe n + Koffset – 1 – TAceil, where n is a subframe where UL transmission starts as per current specification, and TAceil is timing advance rounded up to number of subframes. When switching from UL to DL, a guard period ends at subframe n + Koffset + N + 1 – TAfloor, where N is the length of the UL transmission, and TAfloor is timing advance rounded down to number of subframes.

Proposal-2: Cell-specific Koffset,common is at least indicated in SIB1

Proposal-3: UE reports its UE-specific TA at least in MSG3 and eNB configures UE-specific Koffset,UE that is larger than total TA at least in MSG4.

References

- RP-211601, “WID on NB-IoT/eMTC support for NTN”, MediaTek Inc., Eutelsat S.A.