3GPP TSG RAN WG1 #106bis-e	R1-2110408

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

This summary includes HARQ-ACK feedback specific issues, NACK-only specific issues, HARQ-ACK feedback common issues, HARQ-ACK feedback for SPS multicast, PDSCH repetition, and CSI feedback. In each of high level issue, a sub-level list of issues are organized.

For each of listed issue, proposal(s) is/are suggested from moderator’s perspective according to the submitted individual company’s proposal(s). Companies are welcome to make comments in the table “collect views”. The proposals may be updated in subsequent rounds according to the comments collected in previous rounds so as to strive to converge to consensus. Note that moderator may only tend to collect concerns when time is right for some specific rounds, for which companies are expected to only provide concerns in the table “collect concerns” if any instead of inputting views again and again to alleviate efforts.

People can use “navigation pane” to quickly overview the organization of the summary and proposal(s) for each issue for discussion and provide views/comments into the table of “collect view”/“collect concerns” under each proposal.

Note: for all proposals FL suggests, companies are encouraged to input views, situation can be known better, so as to progress fruitfully.

# ACK/NACK-based feedback specific

## Separate HARQ-ACK codebook

Submitted Proposals

“separate codebooks for Type-1”

(FUTUREWEI) proposal 1:

  - The signalling for URLLC feature to configure the separate codebooks for unicast and multicast is applicable also when they are of the same priority and for Type-1 HARQ codebook.

“codebook for the same priority”

separate codebooks

(Huawei) Proposal 1:

  - Supporting generating separate codebooks for unicast and multicast with the same priority, especially when UE is configured with a separate PUCCH-Config for multicast from that for unicast and slot-based PUCCH and sub-slot based PUCCH are configured wherein, respectively.

(FUTUREWEI) proposal 1:

  - The signalling for URLLC feature to configure the separate codebooks for unicast and multicast is applicable also when they are of the same priority and for Type-1 HARQ codebook.

(Spreadtrum) proposal 1:

  - For type-2 HARQ-ACK codebook, when unicast and multicast are with same priorities, separate codebooks should be generated.

(OPPO) Proposal 1:

  - For the case of same priority for multicast and uncast, separate codebook can be configured for multicast and unicast.

(CATT) Proposal 17:

  - When UE is configured Type-1/ Type-2 codebooks for unicast and multicast with same priorities, if the unicast and multicast have same codebook type and same PUCCH structure configuration, UE generates a single codebook; otherwise, UE separately generates each codebook for unicast and multicast.

Single codebook

(CATT) Proposal 17:

  - When UE is configured Type-1/ Type-2 codebooks for unicast and multicast with same priorities, if the unicast and multicast have same codebook type and same PUCCH structure configuration, UE generates a single codebook; otherwise, UE separately generates each codebook for unicast and multicast.

(CMCC) proposal 4:

  - Single codebook for unicast and multicast is constructed for the case of same priority, for Type-1 and Type-2 HARQ codebook.

(Ericsson) proposal 17:

  - When the UE is configured with separate pdsch-HARQ-ACK-CodebookList and the codebook types for multicast and unicast are different, and of the same priority, the UE sends feedback for both unicast and multicast by concatenating the unicast and multicast HARQ codebooks into one single codebook. The UE uses the last PRI received in the unicast assignments corresponding to the UL feedback slot.

    - FFS: strategy to handle the case when the UE cannot find a PUCCH with enough bit capacity in the configured PUCCH resource sets for unicast.

- “PUCCH-config codebook”

- Separate config-> separate codebook

(OPPO) Proposal 2:

  - If PUCCH-config/PUCCH-ConfigurationList for unicast is also applied for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

(OPPO) Proposal 4:

  - If separate PUCCH-config/PUCCH-ConfigurationList is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

- Separate config-> same codebook

(ZTE) proposal 8:

  - If separate HARQ-ACK codebook generation for unicast and multicast is configured, UE multiplexes HARQ-ACK codebooks for unicast and multicast with the same priority in one PUCCH resource from PUCCH-config /PUCCH-ConfigurationList configured for unicast instead of multicast.

- “Configuration for codebook type ”

(LGE) Proposal 7:

  - Clarify whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast.

“Configuration for codebook”

- (CATT) Proposal 14:

  - The current mechanism for unicast (i.e. UE generates at most one HARQ-ACK information bit in a PUCCH slot) can be applied when pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast is not configured.

- (CATT) Proposal 15:

  - UE does not expect the HARQ-ACK information for unicast and multicast are transmitted in a same PUCCH in a PUCCH slot when any one of the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for unicast and pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast is not configured.

(CATT) Proposal 16:

  - To configure one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities, the current mechanism (i.e. different PUCCH-configs are configured for different priorities) can be applied without further enhancement.

(Nokia) Proposal 16:

  - RAN1 defines new multiplexing rules for the scenario when the multicast codebook type for the same priority is configured to a different type than that of the unicast, e.g., multicast uses Type-2 and unicast uses Type-1.

    - If new multiplexing rules are not defined (e.g., due to time limitation), such configuration should be avoided.

“whether multiplexing different priorities”

(Intel) Proposal 13:

  - Multiplexing across priorities is not allowed in a single PUCCH occasion. For overlapping PUCCH transmissions in time, dropping rule is used.

### Round-1

FL’s comment:

Separate PUCCH-config/PUCCH-ConfigurationList for multicast can be optionally configured to multicast as agreed. However, whether separate configurations imply separate codebooks generation or transmission is a question some companies raised up for discussion. It’s a valid open question because in Rel-16 two separate PUCCH-config configured to UE is only for the purpose of generating two separate codebooks with different priorities. Generating separate codebooks for unicast and multicast with different priorities has been agreed (with separate PUCCH-config/PUCCH-ConfigurationList configured) and generating a joint or separate codebooks especially when separate PUCCH-config/PUCCH-ConfigurationList being configured is still open for the case of the same priority.

If separate PUCCH-config/PUCCH-ConfigurationList is configured for multicast, UE can be configured to generate separate codebooks for unicast and multicast with the same priority as proposed by OPPO. More important reason to justify separate codebooks for the case of same priority is as brought up by Huawei, CATT, Ericsson, Nokia, and CMCC that different codebook type or different PUCCH structure configuration are configured for unicast or multicast as illustrated in the following figure for example:

UE may be configured with sub-slot based PUCCH for unicast and with slot-based PUCCH for multicast (if multicast does not support sub-slot based PUCCH). If the sub-slot PUCCH for unicast and slot-based PUCCH for multicast are indicated in the same slot, UE will have to multiplex the HARQ-ACK bits (with the same priority) to generate a single codebook according to the current agreements. As such, HARQ-ACK for unicast intended for sub-slot PUCCH1 may need to be transmitted on PUCCH5 within a codebook which multiplexed with HARQ-ACK for multicast, so that the low latency benefit of sub-slot PUCCH for unicast is lost.

Alternatively, instead of generating separate codebooks, to address such issues, refraining network from configuring such cases is also proposed by some companies.

#### Proposal 2.1.1-1

Support generating separate HARQ-ACK codebooks for unicast and multicast with the same priority subject to UE capability

- It applies only to the case when separate PUCCH-config/PUCCH-ConfigurationList is configured for multicast for ACK/NACK based HARQ-ACK feedback.

- It is up to gNB to configure UE to generate a single or separate codebooks for unicast and multicast with the same priority.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This is one alternative to avoid multiplexing HARQ-ACK bits with the same priority into one codebook for the case of UE being configured with different codebook Types or different PUCCH structures for unicast and multicast, for which the discussion is complex and needs more efforts. |
| Nokia, NSB | Do not support. We have agreed that multiplexing is applied in case PUCCH transmissions of multicast and unicast with the same priority collide. This should not depend on UE capability. Moreover, we have extensively defined the behavior of Type-1 and Type-2 codebook when multiplexing is made. The behavior should not be related to the PUCCH-config configuration, at all. In the end, the multiplexed feedback should be transmitted in one of the configured PUCCH-configs, which would be the one whose PUCCH resource is used based on the latest DCI. We do not think that the sub-slot / slot-based configuration is an issue, and this can be handled by gNB implementation. We have only mentioned that the gNB should either ensure that different type of multicast / unicast codebook are not scheduled to be multiplexed, or we should define multiplexing rules for that, e.g., concatenation. Moreover, since the current specification limits only 1 HARQ-ACK including PUCCH in a slot, it does not make much sense to generate separate codebooks, since the UE is not capable of transmitting those. |
| vivo | If generating separate HARQ-ACK codebooks means separate PUCCH transmission for HARQ-ACK of unicast and multicast in the same slot, we support the main bullet. We support non-overlapped HARQ-ACK PUCCHs for unicast and multicast with the same priority in the same slot. For the first sub-bullet, we think separate PUCCH-config/PUCCH-ConfigurationList and separate HARQ-ACK codebook has no relation. Even when same PUCCH-config/PUCCH-ConfigurationList is configured, separate HARQ-ACK codebook can be generated based on RRC configuration. |
| Qualcomm | It is related with P2.1.1-2. If different codebook types are configured for unicast and multicast, separate codebooks should be applied even with same priority and same PUCCH-config/PUCCH-ConfigurationList. |
| Samsung | Do not support. It will introduce another layer of complexity/testing for no apparent reason. So what if the codebooks have different Types? TB-based and CBG-based codebooks are multiplexed in a same PUCCH since Rel-15, and single/multi-PDSCHs HARQ-ACK codebooks will be multiplexed in a same PUCCH in Rel-17. Even HARQ-ACK codebooks of different priorities (can be of different Types) will be multiplexed in a same PUCCH in Rel-17. |
| Ericsson | Support. However we should discuss how to transmit the generated codebooks. |
| NTT DOCOMO | More clarification is needed on the conditions for generating separate codebooks. If different codebook type or different PUCCH structure are configured for unicast or multicast, then separate codebooks will need to be generated. But otherwise, i.e., same codebook type and same PUCCH structure, there is no need to generate separate codebooks. |
| ZTE | We understand and are ok with the intention of this proposal. But it seems the proposal itself can NOT address the issue mentioned by FL. From our perspective, the issue mentioned by FL is more like a multiplexing issue. The proposal suggests to generate two separate HARQ-ACK codebooks for unicast and multicast with the same priority but doesn’t mention anything about multiplexing. Even if we allow UE to generate two separate codebooks, multiplexing may also be needed. Thus, a complete solution is preferred. |
| Lenovo, Motorola Mobility | We don’t support this proposal. This proposal is not needed. Anyway, for different HARQ-ACK codebook types configured for unicast and multicast, it is obvious to generate separate codebooks even with same priority and same PUCCH-config/PUCCH-ConfigurationList. |
| CMCC | If the PUCCH for multicast and PUCCH for unicast is not overlapped in one slot, we support UE generates separate codebooks for unicast and multicast with the same priority. But if the PUCCH for multicast and PUCCH for unicast is overlapped in one slot, UE should generate a single codebook, especially as the proposal 2.1.1-2, the PUCCH structure needs to be same between unicast and multicast to avoid complicated multiplexing issue. |
| MediaTek | If separate codebooks for multicast and unicast are generated within one slot, it means that at least two codebooks will be transmitted in one PUCCH slot. However, the current spec supports only one HARQ codebook is transmitted in one PUCCH slot for the same priority case. If the separate CBs for multicast and unicast are generated in different slot with different PUCCH resource, it is natural behavior and ok for us. |

#### Proposal 2.1.1-2

For UE supports both unicast and multicast, UE does not expect to be configured with different codebook types nor different PUCCH structures (e.g., slot-based/sub-slot based PUCCH, different sub-slot PUCCH length) for unicast and multicast.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This is another alternative to avoid multiplexing HARQ-ACK bits with the same priority into one codebook for the case of UE being configured with different codebook Types or different PUCCH structures for unicast and multicast, for which the discussion is complex and needs more efforts. Companies can express views to support which alternative. |
| Nokia, NSB | Support. In our view, sub-slot / slot configuration issue can be handled by gNB implementation. However, in order not to delay the progress, since the limitation would not have severe impact, we can support the proposal as it is. |
| vivo | Not support. we think for UE supports both unicast and multicast, UE can be configured with different codebook types, UE just separately construct HARQ-ACK codebook for unicast and multicast and then concatenate them into one. We are fine with that UE does not expect to be configured with different PUCCH structures. |
| Qualcomm | It seems against the following RAN1 agreement: Agreement: For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast. |
| Samsung | Do not support. In addition to comments for P 2.1.1-1, it is not clear what slot/sub-slot has to do with multiplexing HARQ-ACK codebooks in a PUCCH (e.g. resource indicated by last unicast or by last unicast/multicast DCI). Also, a “slot-based” PUCCH resource can be a “sub-slot” one (e.g. starting symbol and number of symbol can be anything – up to the gNB to configured). |
| Ericsson | Agree with qualcomm. Based on the agreement, we should decide what to do in that case with the resulting generated codebooks. |
| NTT DOCOMO | It is possible that the codebook type for unicast is e-Type2 or Type3, so it should be possible to configure different codebook types for unicast and multicast regardless of PUCCH structure. |
| ZTE | We are ok with the above proposal. But again, it seems the proposal itself is still not complete, the multiplexing issue still needs to be finalized.. |
| Lenovo, Motorola Mobility | We don’t support this proposal. It is not necessary to add such configuration limitation. |
| CMCC | We only support the PUCCH structure needs to be the same. As the codebook type, we agree with vivo, UE can be configured with different types and concatenate into one. |
| MediaTek | Share the similar view with QC. |

#### Proposal 2.1.1-3

- When pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast is not configured, UE generates at most one HARQ-ACK information bit in a PUCCH slot.

- UE does not expect the HARQ-ACK information for unicast and multicast are transmitted in a same PUCCH in a PUCCH slot when any one of the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for unicast and pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast is not configured.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | Originates from CATT’s proposals. Seems straightforward but make sense to achieve agreements because it affects the UE behavior description when the RRC parameter is not configured. |
| Nokia, NSB | We support the first bullet with modifications. In our view, this agreement should only cover ACK/NACK feedback, as we are discussing NACK-only feedback specific issues in other sections. Having specified that, the first bullet is natural, but only per priority per MBS service. We have agreed that HARQ-ACK feedback for different priorities can be sent in the same slot within different PUCCHs. This can also apply herein. Regarding second bullet, and for the case where multiple MBS services are to be ACK/NACK-ed, we can define a simple concatenation rule, unless major concerns are raised. What we propose: - For the UE configured with ACK/NACK based feedback for multicast, when pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast is not configured, UE generates at most one HARQ-ACK information bit per priority per MBS in a PUCCH slot. - FFS: Multiplexing rule for different MBS services, and multiplexing rule for MBS and unicast. |
| vivo | Not support. we think the issue should be “when pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast is not configured, which type codebook is used for multicast”. |
| Qualcomm | Similar view as vivo |
| Samsung | Do not support. The first bullet is unnecessary – either pdsch-HARQ-ACK-Codebook-Multicast can be mandatory, as for unicast, or a default value (e.g. Type 2) can be defined - it will have equivalent results as the first bullet if the NW chooses to schedule MBS so that there is one HARQ-ACK bit per PUCCH, and can also allow the NW to do any scheduling it wants (a codebook is available). There is no reason for the restriction of the second sub-bullet. |
| Ericsson | The motivation for the proposal is unclear. With the constraint "UE generates at most one HARQ-ACK information bit in a PUCCH slot", what does the UE do if it has received more TBs to be acked in this slot? |
| NTT DOCOMO | We are fine with the modified proposal by Nokia. |
| ZTE | Based on our understanding, we even didn’t have such behavior for unicast. Do we have any strong motivation to define such UE behavior considering that network can always configure pdsch-HARQ-ACK-Codebook-Multicast/pdsch-HARQ-ACK-CodebookList-Multicast for UE if it supports multicast. |
| Lenovo, Motorola Mobility | The motivation and principle are not quite clear. What does “UE generates at most one HARQ-ACK information bit in a PUCCH slot.” mean? |
| CMCC | Agree with vivo, the default HARQ-ACK codebook type should be discussed, other wise it will cause large scheduling restriction. |

## Non-overlapping/sub-slot PUCCH

Submitted Proposals

“sub-slot based PUCCH”

(ZTE) Proposal 6:

  - Regarding the case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot:

- HARQ-ACK PUCCH overriding mechanism is reused for multicast with the same priority at least for the same MBS service..

- if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are determined to be in the same UL slot, then

•	The overriding/multiplexing should be used for the two HARQ-ACK PUCCHs for the same priority (regardless of whether they overlap in the time domain).

- sub-slot based PUCCH transmission for HARQ-ACK is supported.

(Spreadtrum) proposal 3:

  - Sub-slot based PUCCH transmission for HARQ-ACK is supported.

(vivo) Proposal 9: Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

(CMCC) proposal 1:

  - Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

(Ericsson) Proposal 20:

  - Sub-slot PUCCH based ACK/NACK HARQ feedback for multicast is supported which is a UE capability.

“Non-overlapping slot-based PUCCH”

(vivo) Proposal 7:

  - For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast,

    - For the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot, separate transmission of HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast is supported.

(CMCC) proposal 2:

  - If separate PUCCH(s) is configured for HARQ-ACK for multicast, the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK with the same priority index should be the same. The PUCCH structure for PUCCH-Config for multicast and unicast HARQ-ACK with different priority index can be different.

(DOCOMO) Proposal 10:

  - Support at most one PUCCH with HARQ-ACK in a slot/sub-slot per priority index per TRP.

(DOCOMO) Proposal 11:

  - When multicast transmission is performed from a single TRP and HARQ-ACK feedback for unicast and multicast is indicated as transmitted at the same slot/sub-slot, the UE multiplexes HARQ-ACK with the same priority regardless whether PUCCH resources are overlapped or not.

### Round-1

“sub-slot based PUCCH”

FL’s comment:

Regarding whether UE supports sub-slot based PUCCH with UE capability, this issue has been discussed for several meetings, although supporting it is still clearly proposed by ZTE, Spreadtrum, vivo, CMCC, and Ericsson in this meeting, I doubt it can be agreeable for this meeting due to sustain concern of supporting it from some companies.

Regarding the number of non-overlapping slot-based PUCCH in the same slot supported for multicast, this issue has been discussed also earlier together with the cases of single TRP and multiple TRP. The conclusion although not recorded is that we will focus on the single TRP scenario first and the number of non-overlapping slot-based PUCCH in the same slot is unchanged or the same as legacy, so nothing new needs to be agreed.

Based on this situation, no need to suggest the proposal for these issues. Different views are welcome.

Collect views:

| Company | Comments |
| --- | --- |

## Type-1 HARQ codebook

Submitted Proposals

“FDM-ed for more than one G-RNTI”

(ZTE) Proposal 2:

  - Type-1 HARQ-ACK codebook construction for overlapped multicast and multicast or overlapped unicast and multicast, for a PDSCH SLIV group, the number of HARQ-ACK bits generated by the UE takes the UE capability of PDSCH reception into account.

(ZTE) Proposal 3:

  - Support separate configuration of TDRA tables for unicast and multicast.

(CATT) Proposal 18:

  - when the number of services configured including unicast and multicast is greater than number of FDM-ed PDSCHs reported by UE, the enhancement on Type-1 HARQ-ACK codebook generation shall be studied.

(Nokia) Proposal 17:

  - RAN1 discusses whether FDM-ed / TDM-ed multicast transmissions of different multicast / broadcast services are allowed.

(Nokia) Proposal 18:

  - When FDM-ed transmissions of unicast and multicast / broadcast services are allowed, the UE constructs separate Type-1 HARQ-ACK sub-codebooks using Rel-15 / 16 mechanisms for each multicast / broadcast service.

(Nokia) Proposal 19:

  - The UE concatenates the constructed Type-1 sub-codebooks and sends them in the same PUCCH resource in case their HARQ-ACK feedback is scheduled for the same time instance.

(Nokia) Proposal 20:

  - The order of concatenation of the MBS sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing order of the G-RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook. MBS sub-codebooks are preceded by unicast sub-codebook, as agreed before.

(LGE) Proposal 26:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells where a CFR is configured for an interested service for multicast

(Ericsson) Proposal 18:

  - FDMed unicast and multicast capable UE can be indicated via RRC signaling whether type-1 codebook should be generated as FDMed manner or not.

“TDM-ed cases”

“intersection k1”

(vivo) Proposal 4: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 1.

  - for slot timing values K_1 in the intersection of K_1 set for unicast (termed set A) and K_1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K_1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K_1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(CATT) Proposal 19:

  - Alt 2 can be used as baseline scheme and Alt 1 can be used as enhancement scheme on TDM-ed Type-1 HARQ-ACK codebook construction.

(CMCC) Proposal 8:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is:

    - for slot timing values K_1 in the intersection of K_1 set for unicast (termed set A) and K_1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

    - for slot timing values K_1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

    - for slot timing values K_1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(Nokia) Proposal 21:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK / NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 1 from the following alternatives:

    - Alt 1:

      - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

      - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

      - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

    - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

(Lenovo) Proposal 13:

  - For Type-1 HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, determine candidate PDSCH reception occasions based on below steps:

    - if a slot is included in intersection of downlink association set (DAS) for unicast and DAS for multicast, the candidate PDSCH reception occasions in the slot are determined according to the union of the configured TDRA table for unicast and the configured TDRA table for multicast;

    - if the slot is only included in DAS for unicast, the candidate PDSCH reception occasions in the slot are determined only according to the configured TDRA table for unicast;

    - if the slot is only included in DAS for multicast, the candidate PDSCH reception occasions in the slot are determined only according to the configured TDRA table for multicast

(Apple) Proposal 6:

  - For Type-1 HARQ-ACK codebook construction for TMD-ed unicast and multicast multiplexed in the same slot, K1 set is in the intersection K1 set of unicast and multicast is based on union of the PDSCH TDRA sets, otherwise PDSCH TDRA set of unicast K1 set or multicast K1 set is applied.

(Qualcomm) Proposal 3:

  - For HARQ-ACK codebook construction of ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, prefer Alt1 to determine PDSCH reception candidate occasions.

    - Alt 1 assuming K1 set for unicast termed as set A and K1 set for multicast termed as set B

      - for slot timing values K1 in the intersection of A and B, based on union of the PDSCH TDRA sets,

      - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

      - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(Ericsson) Proposal 19:

  - Alt 1 is selected as the method to construct type-1 HARQ-ACK codebook for TDMed unicast and multicast.

“Union k1”

(ZTE) Proposal 1:

  - For Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is for slot timing values K_1 in the union of K_1 set for unicast and K_1 set for multicast, based on the union of the PDSCH TDRA sets.

(Spreadtrum) proposal 2:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, PDSCH reception candidate occasions are based on the union of the PDSCH TDRA sets for slot timing values K_1 in the union of K_1 set for unicast and K_1 set for multicast.

(OPPO) Proposal 5:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, PDSCH reception candidate occasions are based on Alt 2.

(CATT) Proposal 19:

  - Alt 2 can be used as baseline scheme and Alt 1 can be used as enhancement scheme on TDM-ed Type-1 HARQ-ACK codebook construction.

(Intel) Proposal 7:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions, Alt 2 is supported.

Others

(Intel) Proposal 6:

  - If UE is configured one CB for multicast and one CB for unicast of different priorities, the pdsch-HARQ-ACK-Codebook configuration can include a priority indicator which applies to all multicast codebooks.

(Intel) Proposal 8:

  - For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

### Round-1

FL’s Comments

Regarding FDM-ed Type-1 codebook construction for more than one G-RNTI, whether UE supports more than one G-RNTI FDM-ed scheduling needs to be first decided in AI 8.12.1.

For TDM-ed Type-1 codebook, whether the construction is based on the intersection of K1 sets or based on the union of K1 sets, the views are as follows based on the submitted proposals:

- Supportive to Alt 1 (intersection): [8, vivo, Nokia, CATT, Lenovo, Qualcomm, CMCC, Apple, Ericsson]

  - the value of K1 would be larger than that of unicast for the UE with PDSCH processing capability 2. Smaller size of Alt 1. No additional procedure complexity. (vivo)

  - TDRA and K1 set for unicast and multicast are typically different. Alt1 can reduce the overhead. (CMCC)

- Supportive to Alt 2 (union): [5, Spreadtrum, ZTE, CATT, OPPO, Intel]

  - Less spec impact and unified solution. (ZTE, Spreadtrum)

  - Payload size reduction is not a design target. (OPPO)

  - Alt 1 and Alt 2 would result to basically a same codebook. For TDD, the Type-1 HARQ-ACK codebook depends on the UL/DL configuration (e.g. DDDSU) which removes most slot timing values K1. There is no apparent reason for the TDRA tables for unicast and multicast to be (substantially) different. (Samsung)

#### Proposal 2.3.1-1 (H)

TDRA tables for unicast and multicast can be separately configured.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | Although it is obvious, it would be good to have it agreed as others proposed. |
| Nokia, NSB | Ok. |
| vivo | Ok |
| Qualcomm | If it means TDRA tables in PDSCH-Config-Multicast can be configured, separate from that of PDSCH-Config in the associated unicast BWP, we agree with the proposal. On the other hand, if TDRA tables in PDSCH-Config-Multicast is not configured, UE will use that in the associated unicast BWP by default? |
| Samsung | Does it add anything to the agreement that PDSCH-Config can be separately provided for multicast and, if not provided, the unicast one applies? |
| Ericsson | Support |
| NTT DOCOMO | Support |
| ZTE | We think this proposal is straightforward. But the default UE behavior should also be defined if TDRA tables for multicast is not configured. Thus, the following updated version is proposed by us. #### Proposal 2.3.1-1 (H) TDRA tables for unicast and multicast can be separately configured. - If TDRA table is not separately configured for multicast, the TDRA table for unicast also applies to multicast. |
| OPPO | OK |
| Spreadtrum | Support |
| Lenovo, Motorola Mobility | OK |
| CMCC | Support |
| CATT | OK |
| MediaTek | Support |

#### Proposal 2.3.1-2

The discussion of Type-1 codebook construction for more than one G-RNTI scheduled in FDM-ed is subject to whether UE supports more than one G-RNTI scheduled in FDM-ed that will be decided first in AI 8.12.1.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | - Not support. This statement is not correct, as we have mentioned in our contribution. Construction of separate Type-1 sub-codebooks is needed even if only one multicast and one unicast are allowed to be FDM-ed in a slot, since we could have the case where one MBS is FDM-ed with the unicast transmission (i.e., PDSCH of G-RNTI 1 is FDM-ed with PDSCH of C-RNTI) in one slot, and another MBS is FDM-ed with the unicast transmission (PDSCH of G-RNTI 2 is FDM-ed with PDSCH of C-RNTI) in another slot, and the corresponding HARQ-ACK transmissions are scheduled for the same slot. For such cases, construction of separate Type-1 HARQ-ACK sub-codebooks for each multicast / broadcast service (and one unicast sub-codebook, if needed) is needed. |
| vivo | - Ok |
| Qualcomm | - Similar view as Nokia that the discussion is not limited to the case that UE supports FDM-ed multiple G-RNTIs in a slot. |
| Samsung | - Same understanding as Nokia. |
| Ericsson | - Agree with Qualcomm and Nokia comments above. |
| NTT DOCOMO | - We agree with Nokia |
| ZTE | - Ok to discuss it in AI8.12.1. But the issue seems not only related to the case when more than one G-RNTI scheduled FDM-ed. Another important case is the following one, i.e., one unicast is FDM-ed with one multicast and they are TDM-ed with another multicast. |
| Lenovo, Motorola Mobility | - We are OK with it. |
| CMCC | - Ok to discuss the issue brought by Nokia. |

#### Proposal 2.3.1-3

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 1 from the following alternatives:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We prefer Alt 2. Alt 1 seems a minor optimization. |
| Nokia, NSB | Support. |
| vivo | Support |
| Qualcomm | ok |
| Samsung | Prefer Alt. 2. There will only be additional spec impact and complexity from Alt. 1 for no benefit particularly considering the single cell operation in Rel-17 for multicast. Not worth changing the Type-1 construction method to possibly save 1-2 bits, if that. Also, Alt. 1 is incomplete – for multiple G-RNTIs (also related to previous P 2.3.1-2), several iterations over the G-RNTIs are needed to get set B or several set Bs are needed – unclear which from the proposal (there will be even less benefit in such case). Overall, RAN1 should always opt for a legacy/simple approach when a modification provides no gain in practice. |
| Ericsson | OK |
| NTT DOCOMO | Support |
| ZTE | We understand that theoretically, Alt.1’s best performance is better than Alt.2. However, in practice, as also mentioned by other companies, the K1 candidate values for unicast and multicast will be the same. The practical gain of Alt.1 is marginal. However, Alt.1 will incur additional implementation impact in both UE and network side. Thus, overall we still propose Alt.2. |
| Lenovo, Motorola Mobility | We support Alt 1. |
| CMCC | Support Frist, we think alt 1 is based on current type-1 construction method not changing it, because there is only unicast service in Rel-15/16, the union K1(all K1 list are configured for unicast) and TDRA table (all TDRA tables are configured for unicast)is used. But for MBS, the K1 list are different from unicast, since the PDCCH monitoring for unicast service is always in per slot manner due to unpredictable traffic arrivals, while multicast services are periodic and predictable, the period of monitoring periodicity of multicast PDCCH can configured as multiple slots to reduce UE PDCCH monitoring power consumption. For the intersection K1, union multicast and unicast TDRA tables are used, but for other K1, it is straightforward only used multicast or unicast tables. As the multiple G-RNTI issues, we only agree a separate PDSCH-config are configured within CFR and it is necessary to limit configuring one CFR within UE active BWP, that means there is also only one TDRA table configuration which applied to multiple G-RNTIs. |

## Type-2 HARQ codebook

Submitted Proposals

“Codebook for more than one per G-RNTI”

(ZTE) Proposal 4:

  - Regarding Type-2 HARQ-ACK codebook construction for MBS, the maximum number of Type-2 HARQ-ACK sub-codebooks for multicast is determined by UE capability for MBS.

- “max number of G-RNTI”

(OPPO) Proposal 3:

  - At most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with type 2 unicast HARQ-ACK sub-codebook.

(CATT) Proposal 20:

  - Support concatenating no more than 3 Type-2 HARQ-ACK codebooks for multicast scheduling.

(CATT) Proposal 21:

  - When the number of configured G-RNTI is larger than the number of UL DAI, the association between the UL DAI and G-RNTI can be configured by RRC.

- others

(CATT) Proposal 22:

  - To achieve the common understanding on codebook size between gNB and UE, RAN1 should specify the following two aspects for generating Type-2 codebooks when UE joint an ongoing multicast transmission:

    - T0 indication: The slot that UE begins to monitor the PDCCH occasion and acquire scheduling information.

    - Missing DCI: The UE assume that DAI is not circled (e.g.  j=0) and (n-1) DCIs are missed when the DCI value is n (n is not equal 1) on T0 slot.

(CATT) Proposal 23:

  - For Type-2 HARQ-ACK feedback, nHARQ-ACK for PUCCH transmission can be modified as following:

nHARQ-ACK=nHARQ-ACK(unicast) +OACK(multicast),  in where nHARQ-ACK(unicast)  is computed as below formula which is same as R15/R16:

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c ;

    - OACK(multicast) is the total number of bit for all multicast traffic.

### Round-1

FL’s Comments

RAN2 LS in (R1-2106408) indicates RAN2 agreed that in order to receive multiple MBS services, UE needs to support multiple G-RNTIs and/or G-CS-RNTIs. And it is FFS whether this depends on UE capability.

The issue of max G-RNTI UE supports could be handled in UE feature discussion phase.

FL’s Proposal:

#### Proposal 2.4.1-1

The following proposal will be discussed in UE feature discussion:

- The maximum G-RNTI UE supports is up to UE capability reporting and the candidate values are  [2] and [3].

Collect views:

| Company | Comments |
| --- | --- |
| - Nokia, NSB | - Support to discuss the issues in UE feature discussion. |
| - vivo | - We agree that multiple G-RNTIs should be based on UE capability, and we think the candidate values can be discussed in UE feature part or FFS. |
| - Qualcomm | - Ok to discuss it in UE feature. - Also need to consider max number of G-RNTI(s)/G-CS-RNTI(s) for multicast and/or broadcast. |
| - Samsung | - Can be discussed in UE features – the number should start from 1 not [2] or [3]. |
| - Ericsson | - Support |
| - NTT DOCOMO | - Support |
| - ZTE | - OK to discuss this in UE feature session. |
| - CMCC | - Support |
| - MediaTek | - Support to discuss the issue in UE feature. |

## UCI multiplexing/prioritizing

Submitted Proposals

“last unicast DCI”

(Huawei) Proposal 2:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the last unicast DCI.

(ZTE) Proposal 7:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to Alt.1: the last DCI for unicast.

(vivo) Proposal 6:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(CMCC) Proposal 5:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission should be based on the PRI indicated in the last DCI for unicast.

(Nokia) Proposal 14:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(Ericsson) Proposal 24:

    - When multicast and unicast traffic HARQ feedback can be multiplexed into the same PUCCH resource, the specific PUCCH resource is determined by the PRI in DCI associated with unicast traffic.

“last across DCI”

(Spreadtrum) Proposal 4:

  - For ACK/NACK based HARQ-ACK feedback, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

(NEC) Proposal 7:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI across multicast and unicast.

(CATT) Proposal 24:

  - For multiplexing the HARQ-ACKs of multicast and unicast, the last DCI refers to the last DCI across unicast and multicast.

(MediaTek) Proposal 2:

  - When multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the “last DCI” refers to the last DCI across unicast and multicast.

(LGE) Proposal 21:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and dynamically scheduled unicast or for dynamically scheduled multicast PDSCHs only, UE determines a PUCCH resource based on the PRI of the last DCI scheduling either multicast or unicast.

(Apple) Proposal 5:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

(Qualcomm) Proposal 2:

  - For multiplexing of ACK/NACK feedback for unicast and multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

(DOCOMO) Proposal 9:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, support Alt 2.

    - The order is first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and is then indexed in an ascending order across PDCCH monitoring occasion indexes.

- “multiplexing for overlap or not”

(Nokia) Proposal 13:

  - Multiplexing of HARQ-ACK feedback of same priority is supported in case PUCCH transmissions are in the same slot, not only when the corresponding PUCCH resources physically overlap.

- Multiplexing for shared PUCCH-config/PUCCH-ConfigurationList:

(OPPO) Proposal 2:

  - If PUCCH-config/PUCCH-ConfigurationList for unicast is also applied for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

UL-DAI

(vivo) Proposal 8:

  - For PUCCH including multicast HARQ-ACK overlaps with PUSCH, reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and PUSCH.

    - FFS UL-DAI indication when both unicast HARQ-ACK and multicast HARQ-ACK are multiplexed on one PUSCH

### Round-1

FL’s Comments

Whether the PRI is based on the last unicast DCI or the last DCI across unicast and multicast when determining the PUCCH resources after multiplexing HARQ bits for unicast and multicast, the views are summarized as follows:

From the submitted proposals for this meeting:

- Supportive to “last unicast DCI”: [6, ZTE, vivo, Nokia, Samsung, CMCC, Ericsson]

- Supportive to “last across DCI”: [8, Spreadtrum, NEC,  CATT, Qualcomm, LGE, MediaTek, Apple, DOCOMO]

The concerns of “last unicast DCI”

- Restriction is not necessary. NW ensure orthogonal PUCCH resources for UEs. (Spreadtrum)

- DCI for multicast is group-common and it is not flexible to indicate UE-specific PUCCH resource (vivo)

- without additional judgement about whether this DCI schedules a multicast service or unicast service and But for the case that UE has multicast service only, a power control mechanism for PUCCH conveying multicast HARQ-ACK only should be supported. The same mechanism should be used for the multiplexing between unicast and multicast. (NEC)

- But Alt.1 requires a unicast PDSCH always to be scheduled. Moreover, the number of unicast DCIs shall be more or equal to four to support the scenario where at most three consecutive DCI are missing.(CATT)

The concerns of “last DCI across unicast/multicast”

- The same TPC value applying to all UEs does not make sense. (ZTE)

- If HARQ-ACK feedback for unicast multiplexes on the PUCCH resources of multicast, the UCI payload size after multiplexing is not fixed. In order to ensure the same flexibility of payload size of multicast and unicast, RRC signaling needs to configure 4 PUCCH resource sets for multicast, which brings huge PUCCH overhead. When multiplexing ACK/NACK feedback for unicast with NACK-only feedback for multicast, NACK codebook needs to multiplex on PUCCH for unicast. Alt.2 does not work if last DCI is multicast DCI. (CMCC)

- Alt. 1 is a better approach, since a later MBS scheduling at the time of unicast scheduling is not expected with high probability, so it is a corner case. Furthermore, we have provided our view on concerns regarding Alt. 1 during e-mail discussions.  (Nokia)

Taking all these concerns into account, the most important one seems the TPC value issue.

FL’s Proposal:

#### Proposal 2.5.1-1

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

Collect views:

| Company | comments |
| --- | --- |
| Nokia, NSB | Support. |
| vivo | Support |
| Qualcomm | Not support. We don’t see the reason to change the legacy behavior by using last DCI (no need to differentiate unicast or multicast). This proposal is only talking about the ACK/NACK-based multicast feedback multiplexing with unicast one. The argument on NACK-only-based multicast feedback is not valid. Regarding TPC, it is not necessary to bound the fast PC command with the PUCCH indicated by the last DCI. The TPC command in GC-DCI is not applied (under discussion in 8.12.1). But there are other legacy ways for UE-specific TPC, such as DCI format 2_2 with TPC-PUCCH-RNTI. |
| Samsung | Support. There is no such thing as “legacy behavior” for this case. The main problem is not the TPC command (that is a trivial issue in our view) but that using the multicast PRI cannot possibly work without reserving resources for each UE in advance (i.e. PRI in multicast DCI is useless when unicast HARQ-ACK is also multiplexed). For example, assuming a maximum of say 10 RBs for a unicast PUCCH, we would like to request proponents for using the last multicast DCI to describe where the PUCCH resources will be when: {UE1 UE2 UE3} transmit PUCCH with unicast+multicast HARQ-ACK in a first slot {UE4 UE5 UE6} transmit PUCCH with unicast+multicast HARQ-ACK in a second slot {UE1 UE2 UE4 UE5} transmit PUCCH with unicast+multicast HARQ-ACK in a third slot Also, to then generalize to an arbitrary number of UEs and for “last multicast DCI” for different UEs being in different slots (e.g. for different MBS) when PUCCH with HARQ-ACK for those UEs is in the same slot. |
| Ericsson | Support |
| NTT DOCOMO | Not support. In Alt 1, the gNB scheduler has to indicate a PUCCH resource considering the future resource allocation when sending a unicast DCI. It would complicate the implementation of the gNB scheduler. |
| ZTE | - We support the above proposal. - In response to CATT’s following argument, we don’t think Alt.1 requires a unicast PDSCH always to be scheduled. The main bullet of this proposal says “For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast”, if there is no unicast, we don’t need to perform multiplexing. But Alt.1 requires a unicast PDSCH always to be scheduled. Moreover, the number of unicast DCIs shall be more or equal to four to support the scenario where at most three consecutive DCI are missing.(CATT) |
| Lenovo, Motorola Mobility | - Not support. As mentioned by NTT DOCOMO, last unicast DCI leads to the scheduling constraints as gNB anyway needs to transmit a unicast DCI after the last multicast DCI. |
| CMCC | - Support |

## PUCCH resource

Submitted Proposals

“Resource sets/resources”

(ZTE) Proposal 5:

  - Regarding configuration of PUCCH resource sets for ACK/NACK feedback for multicast, PUCCH-Config/PUCCH-ConfigurationList is shared for unicast and multicast if the separate PUCCH-Config/PUCCH-ConfigurationList is not configured for multicast.

(OPPO) Proposal 13:

  - For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among UEs within the group are supported.

(OPPO) Proposal 14:

  - For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

(OPPO) Proposal 15:

  - Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

(CMCC) Proposal 3:

  - For the separate PUCCH-ConfigurationList configured for multicast for two simultaneously constructed HARQ-ACK codebooks with different priorities,

    - different PUCCH Resource IDs are configured in the two PUCCH-Config within the PUCCH-ConfigurationList.

(LGE) Proposal 1:

  - PUCCH-config for multicast can be configured per CFR or per UE’s active UL BWP associated to the CFR.

- “k1 indication”

(DOCOMO) Proposal 8:

  - A list of k1 values for DCI format 1_0 for multicast is configurable.

(LGE) Proposal 3:

  - For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

(LGE) Proposal 4:

  - For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured).

    - Different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

(LGE) Proposal 5:

  - For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

### Round-1

#### Proposal 2.6.1 (H)

- For the first DCI format used for scheduling multicast, a list of k1 (i.e., PDSCH-to-HARQ_feedback timing) values for DCI format 1_0 for multicast is configured per UE by RRC signaling.

- This applies to at least the ACK/NACK based HARQ-ACK feedback.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | For ACK/NACK based HARQ-ACK feedback, UE is expected to transmit the HARQ-ACK for multicast on PUCCH resources configured per UE specifically, so the k1 indicator in GC-DCI (including both the first and the second DCI formats) makes sense to be configurable, so that the PUCCH to be transmitted could be per UE. |
| LG | We are fine with this proposal. |
| Nokia, NSB | Ok, as it provides more flexibility. |
| vivo | This issue is also summarized in 8.12.1. we think it can be discussed in 8.12.1 |
| Qualcomm | It can be discussed in 8.12.1 |
| Samsung | Support without the sub-bullet. All field values in the first DCI format can be configured by RRC – this is not the unicast DCI format 1_0 for initial access/fallback when RRC configuration is not possible. |
| Ericsson | The issue can be resolved in 8.12.1 |
| NTT DOCOMO | Support |
| ZTE | We are generally ok with this proposal. However, based on our understanding, the key issue is whether we allow separate K1 value configuration for multicast for the second DCI format. If yes, the same separate K1 value configuration can also be configured for the first DCI format instead of configuring different K1 values for the first and the second DCI format. Thus, we propose to confirm this key issue first. |
| OPPO | OK |
| Spreadtrum | It can be discussed in AI8.12.1 |
| Lenovo, Motorola Mobility | Support. We also prefer to delete the sub-bullet as the newly configured K1 set can be also applied to NACK-only based feedback. |
| CMCC | Discuss in 8.12.1 |
| CATT | Prefer discuss this issue in 8.12.1. |
| MediaTek | Share the similar view that the issue can be discussed in AI 8.12.1 |

# NACK-only based feedback specific

## PUCCH format and HARQ bits

Submitted Proposals

“format and HARQ bits”

(Huawei) Proposal 3:

  - Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

(Huawei) Proposal 4:

  - The sequence cyclic shift for NACK-only is .

(OPPO) Proposal 6:

  - The PUCCH format used for NACK-only based HARQ-ACK feedback is semi-statically configured by RRC.

(TD Teck) Proposal 1:

  - Support PUCCH format 2/3/4 for NACK-only based HARQ-ACK feedback.

(TD Teck) Proposal 2:

  - If PUCCH format 2/3/4 is used to feed back the NACK information for an MBS session, N is configured as a part of the configuration information of the MBS session. UE maps the NACK information of 1 bit long onto a sequence of N>2 bits long. Then send the NACK information with PUCCH format 2/3/4.

(Ericsson) Proposal 5:

  - Use the PUCCH format 0 or format 1 phase rotations and for format 1 the DFT spreading as dimension in addition to OFDM-symbol and PRB

### Round-1

FL’s Comments

Not enough input and the discussion may also be subject to the discussion of more than one NACK-only based feedback are available for transmission in the same PUCCH slot in another section. TBD for now.

FL’s Proposal:

#### Proposal  3.1.1  TBD

Collect views:

| Company | Comments |
| --- | --- |

## Overlapping PUCCH/PUSCH

Submitted Proposals

“Overlap with other PUCCH/PUSCH”

Cases:

(ZTE) Proposal 11:

  - For the time domain overlapping between NACK-only PUCCH and unicast PUCCH/PUSCH, RAN1 study PUCCH multiplexing for the following scenes.

    - Unicast ACK/NACK/SR PUCCH F0/F1 overlaps with NACK-only PUCCH.

    - Unicast ACK/NACK/CSI PUCCH F2/F3/F4 overlaps with NACK-only PUCCH;

    - unicast PUSCH and NACK-only PUCCH overlap.

Solution 1: NACK-only->ACK/NACK

(ZTE) Proposal 12:

  - If HARQ-ACK PUCCH for unicast and NACK-only PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

    - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

    - NACK-only based HARQ-ACK needs to be transformed to ACK/NACK based HARQ-ACK bits.

(NEC) Proposal 6:

  - Support multiplexing between NACK-only based HARQ-ACK feedback and other UCIs/PUSCH.

(CMCC) Proposal 9:

  - When PUCCH resource for NACK-only feedback for multicast collides with PUCCH resources for HARQ-ACK feedback for unicast or other UCI for the same priority,

    - For NACK-only based feedback with low priority, NACK-only feedback for multicast can be dropped.

    - For NACK-only based feeback with high priority, UE multiplexes the NACK-only with others by replacing the NACK-only with ACK/NACK HARQ bits.

(Nokia) Proposal 12:

  - If for a UE a scheduled group-common PUCCH resource for PTM NACK-only feedback overlaps in time with a UE-specific PUCCH resource for other UCIs or a PUSCH transmission with the same priority, this UE should multiplex the PTM HARQ-ACK feedback with the other UCIs on the UE-specific PUCCH resource or with the PUSCH transmission, by treating NACK-only feedback as a 1-bit ACK / NACK feedback.

(Samsung) Proposal 7:

  - A UE reports ACK/NACK when the UE multiplexes HARQ-ACK for multicast PDSCH receptions with other UCI in a PUCCH or in a PUSCH.

(DOCOMO) Proposal 13:

  - For multiplexing/prioritizing NACK-only based feedback with unicast HARQ-ACK, SR, CSI, or PUSCH, support either of the following options:

    - Option 1-1: Drop NACK-only based feedback when a PUCCH for NACK-only based feedback overlaps with a PUCCH for unicast HARQ-ACK, SR, CSI or a PUSCH.

    - Option 2-2: Support multiplexing/prioritizing when a PUCCH for NACK-only based feedback overlaps with a PUCCH for unicast HARQ-ACK, SR, CSI or a PUSCH.

      - Multiplexing/prioritizing is applied regardless of whether NACK-only feedback is actually transmitted or not

(LGE) Proposal 11:

  - When different PUCCHs for NACK only HARQ-ACK and ACK/NACK based HARQ-ACK are overlapped, NACK only HARQ-ACK is changed to ACK/NACK based HARQ-ACK to be multiplexed with other ACK/NACK based HARQ-ACK.

(LGE) Proposal 13:

  - Discuss UE procedure for multiplexing NACK-only based HARQ-ACK and SR for PUCCH Format 0 and/or 1.

(Convida) Proposal 2:

  - When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUCCH resource allocated for a transmission (2nd transmission) using ACK/NACK HARQ feedback, the UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission.

(Convida) Proposal 3:

  - When a UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUCCH.

(Convida) Proposal 4:

  - When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUSCH transmission, the UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource.

(Convida) Proposal 5:

  - When a UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUSCH.

“more than two bits for NACK-only”

Alt 1: NACK-only->ACK/NACK

(Huawei) Proposal 5:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, when the NACK-only feedback are available with other UCI feedback on the same PUCCH resource, or when the PUCCH resources are overlapped with PUSCH transmission, the NACK-only feedback is transformed into the ACK/NACK based feedback.

(ZTE) Proposal 10:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot,

    - If sub-slot based PUCCH is supported, UE transmits multiple sub-slot based PUCCHs with each corresponding to one NACK-only feedback;

    - Otherwise, consider the following Alts

      - Alt 1: UE transforms multiple NACK-only feedback to ACK/NACK information bits, share the PUCCH resources configured to ACK/NACK based feedback for unicast or multicast;

      - Alt 4: Multiple PUCCH resources are configured, where each PUCCH resource corresponds to one codebook state

        - Up to 2^N-1 PUCCH resources are needed for NACK-only feedback for N MBS services from network perspective.

(Nokia) Proposal 3:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, Alt. 1 is used to increase PUCCH capacity of NACK-only based HARQ-ACK feedback:

    - Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(Nokia) Proposal 4:

  - In case of Alt.1, if supported, to transmit the multiplexed HARQ-ACK feedback, the UE is configured with specific dedicated PUCCH resource(s), and the UE uses the dedicated PUCCH resource(s) to transmit multiplexed NACK-only bits, rather than selecting the PUCCH resource based on the PRI field of the last DCI.

(Nokia) Proposal 5:

  - In case of Alt.1, if supported, concatenation of the converted NACK-only bits is based on the ascending order of PUCCH resource IDs that are originally scheduled for NACK-only feedback.

(LGE) Proposal 12:

  - When different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

(Apple) Proposal 4:

  - If the reported HARQ-ACK bits are larger than PUCCH payload size, UE switches to ACK/NACK based HARQ-ACK feedback mode.

Alt 2: Sub-slot PUCCH

(ZTE) Proposal 10:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot,

    - If sub-slot based PUCCH is supported, UE transmits multiple sub-slot based PUCCHs with each corresponding to one NACK-only feedback;

    - Otherwise, consider the following Alts

      - Alt 1: UE transforms multiple NACK-only feedback to ACK/NACK information bits, share the PUCCH resources configured to ACK/NACK based feedback for unicast or multicast;

      - Alt 4: Multiple PUCCH resources are configured, where each PUCCH resource corresponds to one codebook state

        - Up to 2^N-1 PUCCH resources are needed for NACK-only feedback for N MBS services from network perspective.

(CATT) Proposal 13:

  - Support sub-slot based PUCCH (Alt2) when more than one NACK-only based feedbacks are available for transmission in the same PUCCH slot.

(Ericsson) Proposal 3:

  - When more than one NACK-only based feedback need be transmitted in same PUCCH slot, both Alt 2 and Alt 4 are supported.

Alt 4: Combination of NACK-only

(ZTE) Proposal 10:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot,

    - If sub-slot based PUCCH is supported, UE transmits multiple sub-slot based PUCCHs with each corresponding to one NACK-only feedback;

    - Otherwise, consider the following Alts

      - Alt 1: UE transforms multiple NACK-only feedback to ACK/NACK information bits, share the PUCCH resources configured to ACK/NACK based feedback for unicast or multicast;

      - Alt 4: Multiple PUCCH resources are configured, where each PUCCH resource corresponds to one codebook state

        - Up to 2^N-1 PUCCH resources are needed for NACK-only feedback for N MBS services from network perspective.

(NEC) Proposal 2:

  - Support Alt-4 to transmit more than one NACK-only based HARQ-ACK feedback existing in the same PUCCH slot.

    - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

(CMCC) Proposal 8:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, HARQ-ACK bits can be multiplexed in one PUCCH.

    - Different HARQ-ACK information of a number of group-common PDSCHs are feedbacked in different PUCCHs respectively.

(Samsung) Proposal 6:

  - A UE is provided orthogonal PUCCH resources to select from according to combinations of NACK values for PDSCH/TB receptions.

(Lenovo) Proposal 12:

  - For multiple NACK-only feedback multiplexing in same slot, from per UE perspective, the PUCCH resource which corresponds to the first incorrectly decoded PDSCH is selected for transmitting the NACK-only feedback in the PUCCH slot

(DOCOMO) Proposal 12:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support Alt 4.

(Qualcomm) Proposal 5:

  - For RRC_CONNECTED UEs, when more than one NACK-only based feedback are available for transmission in the same PUCCH slot, at least support

    - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

(Ericsson) Proposal 3:

  - When more than one NACK-only based feedback need be transmitted in same PUCCH slot, both Alt 2 and Alt 4 are supported.

(Ericsson) Proposal 4:

  - Denoting the number of bits in the NACK-only codebook by N, downselect one or both from the following two variants to signal all information from the codebook in the same uplink slot:

- To convey N bits the UE transmits N PUCCH resources in the same slot, where each PUCCH resource represents one bit in the NACK-only codebook of N bits.

- To convey N bits the UE transmits one selected PUCCH resource   in a slot, from a set of M=2^N-1 configured PUCCH resources. The actually-used PUCCH resource therefore carries the required N information bits.

(Ericsson) Proposal 6:

  - A NACK-only codebook can be realized with a combination of PUCCH resources using PUCCH format 1 as the base format. Each codebook bit corresponds to an associate PUCCH resource with a specific pair of cylic shifts, symbol in the slot and an orthogonal sequence value. All the PUCCH resources used to transmit the codebook are orthogonal.

(Ericsson) Proposal 7:

  - Further study  HARQ bundling for NACK-only, where a transmitted NACK signal is associated with a subset of bits in the NACK-only codebook, where multiple UEs use the same PUCCH resource for the NACK-only signal relating to the same subset and the subset size may reduce to 1. A UE transmits the NACK signal if at least one bit of the associated subset of bits in the NACK-only codebook is cleared, i.e. indicates a PDSCH decoding failure. The gNB accordingly retransmits the transport blocks of all HARQ processes of the subset.

(Ericsson) Proposal 8:

  - The gNB has to ensure that it does not transmit more than Nmax=Z (variant a) above) or Nmax=log2(Z+1) (variant b) above) transport blocks with dl-DataToUL-ACK pointing to a uplink slot. Nmax has to be selected as the minimum of corresponding UE capability considering all UEs in NACK-only mode receiving the transport blocks.

Alt 5: NACK if any NACK

(vivo) Proposal 11:

  - When more than one NACK-only based feedback is available for transmission in the same PUCCH slot, UE transmits NACK when any of the PDSCHs is NACK.

(OPPO) Proposal 8:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, NACK-only bundling is performed to generate only 1 HARQ-ACK bit (i.e., Alt 5).

- Indirect proposal

(Spreadtrum) Proposal 5:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, not support Alt 1 and Alt 5

### Round-1

“Overlap with other PUCCH/PUSCH”

FL’s Comments

The proposals for this discussion clearly supports multiplexing the feedback for the NACK-only based with other UCI/PUSCH. There was comment in the last meeting clarity is needed whether the multiplexing is executed only when the TB is failed in decoding. The intention was whenever the PUCCH resources for the NACK-only based feedback overlaps with the resources for other UCI/PUSCH, UE will report ACK/NACK according to the outcome of the TB decoding because otherwise gNB will attempt blind decoding since gNB is not aware whether the ACK from UE is transmitted or not.

FL’s Proposal:

#### Proposal  3.2.1-1

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback for unicast or other UCI for the same priority, support UE multiplexing the NACK-only based feedback with other UCI by transforming NACK-only into the ACK/NACK HARQ bit.

- This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

- Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support. |
| Qualcomm | ok |
| Samsung | Support. Would be good to also capture PUSCH overlapping. |
| Ericsson | Support |
| NTT DOCOMO | Support |
| ZTE | We support the above proposal, which is a straightforward solution to address the multiplexing issue between NACK-only feedback and other UCI. |
| CMCC | Support |
| vivo | For the case NACK-only overlaps with PUSCH/CSI PUCCH, the proposal seems ok. But for the case NACK-only on PUCCH format 0 and it overlaps with SR, SR with multiplex NACK-only on the NACK-only PUCCH resource with different CS according to the current multiplexing rule between HARQ-ACK and SR. either more PUCCH resources reservation is needed or PUCCH resource collision is happened in this case. |

“more than one/two for NACK-only”

FL’s Comments

Regarding the 5 alternatives to support more than one NACK-only based feedback are available for transmission in the same PUCCH slot as agreed as follows,

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

  - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - Alt2: Support sub-slot based PUCCH for this case.

  - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

  - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - Alt5: NACK-only bundling

All these proposals work from analysis of the papers, the differences lie in the efficiency of the alternative and specification efforts for supporting the particular alterative. [Nokia, [10]] has a good analysis of pros/cons and standard impact for each alternative.

Alt1 and Alt4 have clear support although Alt1 proponents may have concerns for Atl4 and vice versa. Alt2 and Alt3 need UE capability to support sub-slot based PUCCH or more than one PUCCH in the same slot and for UE not having such capability the specification still needs to define UE behavior to solve this issue. In addition, Alt3 has no support for this meeting. Alt2 has support from ZTE, CATT, and also Ericsson but ZTE and Ericsson also proposed other alternatives that can be considered, e.g., Alt1/4.

The concern for Alt 4 is the specification efforts might be huge to define combination (of specific sequences) that would fit different scenarios, in addition to allocation of significant amount of PUCCH resources to represent each combination. The concern for Alt 1 is that Alt1 relies on sufficient PUCCH resources with both PUCCH-ConfigurationList for ACK/NACK and PUCCH-ConfigurationList for NACK-only based feedback configured for multicast. However, the resources configuration and usage is up to network. Network can configure both resources for ACK/NACK based and NACK-only based feedback but the real PUCCH overhead depends on whether the PUCCH resource is used or not. Both Atl1 and Alt4 have benefits in some cases and have drawback in some cases.

At this stage, for progress, we can try at least to preclude some alternatives.

FL’s Proposal:

#### Proposal  3.2.1-2

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further  decide on the following alternatives with potential down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are generally fine with this proposal. We support Alt 1 because Alt 1 can be used in various multiplexing cases. |
| Nokia, NSB | For the sake of not blocking the progress, we support the proposal. As we have extensively discussed in our contribution, Alt.4 is not applicable and requires huge specification impact. We support Alt.1, whose “presumable” problems are solvable by network implementation, as FL has indicated. Furthermore, Alt.1 will be a unified solution for NACK-only feedback multiplexing, when combined with Proposal 3.2.1-1. |
| Qualcomm | We prefer Alt4. Does Alt1 means the PUCCH resource for ACK/NACK-based multicast feedback will be configured and used in the same PUCCH slot? |
| Samsung | We support Alt. 4 Surprised to see companies heavily arguing in favor of NACK-only to control PUCCH overhead now be OK with ACK/NACK when things are worse in terms of overhead (>1 bit). The issue is not whether the NW can configure resources, it is whether there is any UL BW when 1 RB is needed for each UE, from potentially 100s of UEs, to transmit PUCCH. Alt.1 means that NACK-only is not supported if there is >1 PDSCH in a “bundling window” – e.g. is not supported at all for TDD where the UL overhead issue is worst. Companies raising spec efforts for Alt. 4 would be good to explain how PUCCH format 0 works in NR and what additional effort they envision other than extending from 2 bits to e.g. 4 bits. |
| ericsson | We are OK removing Alt3 and Alt5. We definitely wish to keep Alt4 and also prefer to keep Alt2 instead of Alt1, but can live with keeping both for the time being. So keeping Alt1, Alt2 and Alt4 would be OK. |
| NTT DOCOMO | We prefer Alt 4. Alt 1 will require orthogonal PUCCH resources among UEs, as HARQ-ACK will be transmitted using ACK/NACK based scheme. This means that when the number of UEs in a group is large, a lot of PUCCH resources will be needed. This is contrary to the main motivation to support NACK-only based feedback, which is to save PUCCH resources when the number of UEs is large. |
| ZTE | Although our preference is to support sub-slot based PUCCH for this case, we understand the concern from UE vendors. We are Ok to go with the above proposal if it is the majority view. |
| Lenovo, Motorola Mobility | We support Alt 4. Alt 1 is against the motivation to introduce NACK-only based feedback. |
| CMCC | Support the proposal and we prefer Alt4. |
| vivo | We are ok to remove alt 2 and alt 3. For Alt 1, it may cause PUCCH collision when two NACK-only are available in the same PUCCH slot. In addition, according to contributions in UE feature part, supporting of ACK/MACK and supporting of ACK/NACK may be different UE capabilities. If they are different UE capabilities, Alt 1 means if UE support NACK-only feedback, the UE has to support ACK/NACK feedback. This should be considered. For Alt 4, the combination will be too complicated when there are many NACK-only in the same PUCCH slot and too PUCCH resources are reserved. |

## PUCCH resource

Submitted Proposals

“Resource sets/resources”

(Huawei) Proposal 6:

  - A separate PUCCH-Config or PUCCH-ConfigurationList for multicast can be but is not necessarily configured for NACK-only based HARQ-ACK feedback.

(ZTE) Proposal 9:

  - Regarding configuration of PUCCH resource sets for NACK-only feedback for multicast, it shares the same PUCCH-Config/PUCCH-ConfigurationList for ACK/NACK-based codebook for multicast (or PUCCH-Config/PUCCH-ConfigurationList for ACK/NACK-based codebook for unicast) if separate configuration for NACK-only is not configured.

(OPPO) Proposal 7:

  - The PUCCH resource for NACK-only based HARQ-ACK feedback is configured per G-RNTI.

(CATT) proposal 9:

  - NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering k1, PRI and DCI CCE index to indicate the PUCCH resource.

(CATT) proposal 10:

  - Up to 32 PUCCH resources can be configured for the PUCCH resource set for NACK-only feedback in MBS.

(CATT) proposal 11:

  - The PUCCH resource set can be used by all MBS services which applying NACK-only based HARQ-ACK feedback.

(CATT) Proposal 12:

  - When separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast, pucch-ConfigCommon can be applied.

(CMCC) Proposal 7:

  - If PUCCH-ConfigurationList for NACK-only based HARQ-ACK feedback is not configured, PUCCH-ConfigurationList for unicast applies.

(Nokia) proposal 2:

  - For NACK-only based feedback for RRC_CONNECTED UEs receiving multicast, if the UE is not configured with a separate PUCCH-ConfigurationList, PUCCH-Config for unicast applies.

(Lenovo) Proposal 7:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for NACK-only based feedback can’t be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(Lenovo) Proposal 8:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for NACK-only based feedback can’t be shared with PUCCH resource configuration for ACK/NACK based feedback for multicast.

(Lenovo) Proposal 9:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(Intel) Proposal 11:

  - If separate PUCCH-ConfigurationList is not configured for NACK-only feedback for multicast, the UE can re-use the PUCCH configuration for ACK/NACK based feedback for multicast.

(Intel) Proposal 12:

  - If high priority NACK-only feedback is configured, the UE expects to be configured with a separate PUCCH-ConfigurationList for NACK-only feedback.

(Intel) Proposal 14:

  - For PUCCH resource selection the following rules can be considered for overlap involving different cases:

    - If MBS and unicast have separate PUCCH-Configuration but with the same priority index and the PUCCH overlap in time, the PUCCH carrying the multiplexed HARQ-ACK feedback is from the PUCCH-Configuration of unicast for the corresponding priority.

    - If multicast ACK/NACK based feedback and NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback.

    - If unicast ACK/NACK based feedback and multicast NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback for unicast

(Intel) Proposal 15:

  - HARQ priority rules are also applicable to feedback for MBS SPS wherein SPS configuration of priority 0/1 maps to HARQ codebook priority 0/1 respectively.

(LGE) Proposal 2:

  - Group common PUCCH resources for NACK only based HARQ-ACK are configured within UL CFR configured within UE’s active UL BWP.

    - Group common PUCCH resources are only configured by PUCCH-config for multicast.

    - UE specific PUCCH resources are configured on UE’s active BWP, as currently specified.

(LGE) Proposal 8:

  - Different group common PUCCH resources can be related to different DL RS e.g. in terms of symbol/slot and/or PRB and/or sequence for PUCCH.

(LGE) Proposal 10:

  - Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

(Apple) Proposal 2:

  - PUCCH resources for NACK-only are independently configured in UL CFR.

(Apple) Proposal 3:

  - PUCCH resource pool is defined for NACK-based feedback, UE select the PUCCH resource according to the subscribed MSB sessions.

(Qualcomm) Proposal 4:

  - For RRC_CONNECTED UEs, support separate PUCCH configuration for ACK/NACK-based and NACK-only-based multicast HARQ feedback.

(Ericsson) Proposal 9:

  - The PRI in the DCI is interpreted by UEs in NACK-only mode to select one of the PUCCH resource configurations for NACK-only.

(Ericsson) Proposal 10:

  - A UE configured with NACK only is always configured with a separate PUCCH configuration for NACK only.

- Power control for NACK-only

(OPPO) Proposal 12:

  - For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

(Nokia) proposal 8:

  - A dedicated PUCCH-PowerControl parameter set, distinct from the PUCCH-PowerControl parameter set for UE-specific PUCCH resources, is used for configuration of power control of group-common PUCCH resources for NACK-only HARQ-ACK feedback.

### Round-1

FL’s Comments

When the separate PUCCH-Config/PUCCH-ConfigurationList for multicast for NACK-only based feedback is configured, the remaining open issues (especially affecting RRC parameters) are the maximum number of PUCCH resources sets, the maximum number of PUCCH resource in the PUCCH resource set, and the payload for each PUCCH resource or the PUCCH resource set.

FL’s Proposal:

#### Proposal  3.3.1-1 (H)

For the separate PUCCH-Config/PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

  - Up to 1 PUCCH resources set in each PUCCH-Config,

  - Up to 32 PUCCH resources can be configured for the first PUCCH resource set in each PUCCH-Config.

    - FFS the payload for each PUCCH resource or PUCCH resource set.

  - The PUCCH resource set applies to all G-RNTIs configured to UE for the NACK-based feedback.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are generally fine with this proposal. We wonder how the last bullet point works for NACK-based feedback for different priorities. |
| Nokia, NSB | We support the intention of the proposal. If we agree on Alt.1 on Proposal 3.2.1-2, we need an additional resource in NACK-only PUCCH-config to transmit multiplexed NACK-only feedback. We believe that the FFS targets that purpose. We support the proposal fully, if that understanding is correct. |
| vivo | Generally fine. we are not sure what’s the motivation for the FFS |
| Qualcomm | Need more clarification on the last subbullet. Does it intentionally exclude G-CS-RNTI(s)? |
| Samsung | Shouldn’t this wait for a decision on multi-bit NACK-only (i.e. previous proposal)? Maybe this also relates to the FFS? |
| Ericsson | In our view, multiple resources are needed to create the NACK only harq response. Therefore, we support using multiple resource sets (index via PRI) and use multiple resources to generate the harq response. |
| NTT DOCOMO | We would like to wait until the details of NACK-only feedback procedure become clearer. |
| ZTE | Ok with the above proposal. |
| OPPO | Share similar view that this proposal is related to details of multi-bits NACK-only feedback, it is better to discuss this later. |
| Lenovo, Motorola Mobility | Defer to the conclusion of NACK-only multiplexing. |
| CMCC | Similar view as Samsung |
| MediaTek | Generally Ok with the proposal. |

FL’s Comments

Some companies proposed the PUCCH resource to be transmitted to carry the NACK-only based feedback is determined by the PRI and k1 indication in the scheduling DCI and the DCI CCE index. This should have been the default assumption for the discussion but it would be good to clarify it at the beginning for more further discussion.

FL’s Proposal:

#### Proposal  3.3.1-2

For NACK-only based feedback, the PUCCH resource to be used for the transmission of the NACK-only based feedback is based on the PRI and K1 indication included in the scheduling DCI and the DCI CCE index.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support the proposal. This is natural, and there is no need to change the behavior. |
| vivo | Support |
| Samsung | Why is the PUCCH resource based on K1? Even if the above is agreed, it cannot be translated to specifications. Is the intention to use the same resource determination as for unicast HARQ-ACK of 1-2 bits? |
| Ericsson | Not clear what happens when multiple PDSCH map to the same PUCCH resource based on this mechanism. |
| NTT DOCOMO | Support |
| ZTE | If the intention of this proposal is to reuse the legacy PUCCH resource determination procedure for NACK-only, we prefer to change the wording to reflect that. Proposal from ZTE: For NACK-only based feedback, the mechanism of PUCCH resource determination for the 1st PUCCH resource set is the same as that defined in Section 9.2.1 of TS38.213 for unicast. |
| Lenovo, Motorola Mobility | We don’t support this proposal. Why K1 indication and CCE are needed for determining the PUCCH resource are not clear to us. |
| CMCC | We also need discuss the multiple NACK-only feedback case. |

FL’s Comments

When the separate PUCCH-Config/PUCCH-ConfigurationList for multicast for NACK-only based feedback is not configured, UE’s behaviour also needs to be discussed, e.g., whether UE assumes NACK-only is not supported or assumes PUCCH-Config/ PUCCH-ConfigurationList for unicast applies. The decision may depend on the discussion of transmitting more than one NACK-only based feedback are available for transmission in the same PUCCH slot: for Alt1, PUCCH-Config/ PUCCH-ConfigurationList for unicast can apply for NACK-only; for Alt4, PUCCH-Config/ PUCCH-ConfigurationList for unicast may not be workable for NACK-only.

FL’s Proposal:

#### Proposal  3.3.1-3 (H)

For NACK-only based feedback for RRC_CONNECTED UE receiving multicast, UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast. If not configured,

- Alt1: NACK-only is not supported.

- Alt2: PUCCH-Config/ PUCCH-ConfigurationList for unicast applies.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We wonder if NACK-only based feedback cannot be configured together with ACK/NACK based feedback for multicast. If NACK-only based feedback cannot be configured together with ACK/NACK based feedback for multicast, this proposal seems fine. However, if NACK-only based feedback can be configured together with ACK/NACK based feedback for multicast, the PUCCH-Config/ PUCCH-ConfigurationList for ACK/NACK based feedback for multicast could also apply to NACK-only based feedback. |
| Nokia, NSB | Support Alt2. Network implementation can make unicast PUCCH-config work with NACK-only feedback, in case Alt.1 is chosen in Proposal 3.1.2-2. This is another benefit of Alt.1 over Alt.4, which would make NACK-only quite complex. |
| vivo | Between these two options, we support Alt 2. |
| Qualcomm | Need to discuss first whether to support separate configuration of NACK-only based feedback and ACK/NACK based feedback for multicast. |
| Samsung | Agree that it should first be concluded whether to use the multicast configuration for ACK/NACK. If also not provided, Alt. 2 is preferable. |
| Ericsson | Support Alt1 |
| NTT DOCOMO | We slightly prefer Alt 1. |
| ZTE | We support with Alt.2. |
| OPPO | PUCCH resource for NACK-only HARQ feedback of MBS should be separate from that for ACK/ANCK HARQ feedback of MBS. As to the 2 alternatives, we prefer Alt 1. |
| Lenovo, Motorola Mobility | We support Alt 2. |
| CMCC | Support Alt 2 |
| CATT | Not support. In our understating, the PUCCH-Config/ PUCCH-ConfigurationList for unicast is not mandatory, so Alt2 will not work when PUCCH-Config/ PUCCH-ConfigurationList for unicast is not configured. Thus, we believe the pucch-ConfigCommon which configured by SIB1 can be applied. In pucch-ConfigCommon, the type of PUCCH format in the PUCCH resource sets is PUCCH format 0 or 1, which is suitable for NACK-only based HARQ-ACK for multicast. |
| MediaTek | Share the similar view with Samsung. If the PUCCH resource for multicast ACK/NACK is configured, the NACK-only feedback back can use it firstly if PUCCH-Config/PUCCH-ConfigurationList for NACK only is not configured. |

## Others

Submitted Proposals

(ZTE) Proposal 13:

  - The DAI filed in UL scheduling DCI is used to indicate the number of PDSCHs received by the UE corresponding to NACK-only feedback.

(Ericsson) Proposal 1:

  - For NACK-only transmission of HARQ feedback for group scheduling, both semistatic and dynamic codebooks are supported

(Ericsson) Proposal 2:

  - The NACK-only codebook for a G-RNTI is constructed independently from codebooks constructed for other G-RNTIs that the UE may also receive and from the codebook that includes feedback for PDSCH scheduled by C-RNTI.

# HARQ-ACK feedback common

## Feedback priority

Submitted Proposals

“priority across M-cast and Unicast/other UCI”

(Nokia) Proposal 11:

  - For NACK-only based HARQ-ACK feedback for multicast, the prioritizing rule between the HARQ-ACK for multicast and SR / CSI, i.e., multiplexing same priority and prioritizing different priorities, can reuse Rel-16 prioritizing rule between the HARQ-ACK for unicast and SR / CSI.

    - How NACK-only is multiplexed with the HARQ-ACK for unicast and SR / CSI is discussed separately.

“priority indication”

(NEC) Proposal 1:

  - Support Alt-1 for the priority indicate of the first DCI format

    - For the first DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

(CMCC) Proposal 10:

  - The priority index is not included in the DCI format 1_0 with CRC scrambled with G-RNTI and is low priory by default.

(Nokia) Proposal 15:

  - The priority index for the first DCI format for GC-PDCCH is optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

(Samsung) Proposal 5:

  - The priority indicator field can be configured to be included in any multicast DCI format.

(Lenovo) Proposal 10:

  - The priority index is not included in the first DCI format for GC-PDCCH.

(Lenovo) Proposal 11:

  - Low priority is always assumed for the PDSCH scheduled by the first DCI format.

(MediaTek) Proposal :

  - The priority index is always low priority (i.e., no priority index field) for the first DCI format for GC-PDCCH scrambled with G-RNTI.

(Intel) Proposal 16:

  - The priority index for first DCI format is not included in DCI and is always low priority.

(Intel) Proposal 17:

  - Scheduling of priority 1 MBS is possible using only the second DCI format (DCI 1_1).

(DOCOMO) Proposal 1:

  - Introduce a priority indicator field in the first DCI format for multicast.

(Apple) Proposal 1:

  - The priority index is not included in the first DCI format for GC-PDCCH, i.e., PDSCH scheduled by first DCI foams is always low priority.

(Qualcomm) Proposal 8:

  - The priority index is not included in the first DCI format for GC-PDCCH.

    - UE assume low priority for multicast HARQ-ACK feedback indicated by the first DCI format by default.

(Ericsson) Proposal 21:

  - For the first DCI format for GC-PDCCH, there is no priority index in the DCI format and the priority is always low.

### Round-1

FL’s Comments:

Regarding the priority indication in the DCI format, the agreement was achieved in the last meeting:

Agreement:

The priority index is,

- for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format for GC-PDCCH, down-select from:

    - Alt1: Optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

    - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

The remaining open issue is whether the priority index can be included in the first DCI format by configuration.

Proponents of Alt1 and reasons:

- NEC, Nokia, Samsung, DOCOMO,

- Flexible scheduling because only two DCI formats are supported for multicast (NEC, Nokia)

- The first DCI format will include configurable fields that are not present in DCI format 1_0. There is no reason for the first DCI format to have reduced capabilities than the second DCI format particularly as deployment of the second DCI format may be more challenging for a network due to potential size alignment issues. (Samsung)

Proponents of Alt2 and reasons:

- CMCC, Lenovo, MediaTek, Intel, Apple, Qualcomm, Ericsson

- First DCI takes DCI 1_0 as the baseline, reusing the legacy rule. (CMCC, Lenovo, MediaTek, Qualcomm)

Based on the reasoning of each alternative analysed in the papers, the main argument for Alt2 is reusing the legacy rule although Alt2 has majority support. For UE receiving  multicast, UE would enter RRC_CONNECTED state and obtain the necessary configuration first. In addition, some field in the first DCI may have to be configurable anyway, e.g., k1 value for the ACK/NACK based feedback. From FL’s perspective, Alt1 would make more sense and debate is still welcome to see more views.

FL’s Proposal:

#### Proposal 4.1.1-1 (H)

- The priority index for the first DCI format for GC-PDCCH is optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support. |
| vivo | Alt 2 is preferred. We prefer to keep current rule for DCI 1_0 in terms of priority indication. |
| Qualcomm | We don’t support Alt1. Why not go with the majority view Alt2. |
| Samsung | Support. There is no “current rule” for the first DCI format and it is not DCI format 1_0. For DCI format 1_0, the priority indicator was not configured only because it is not possible to configure anything. Also, DCI format 1_2 with potentially much smaller size than DCI format 1_0 was defined. The second DCI format is primarily considered to have larger size than the first DCI format and include fields that will not be likely needed for high priority (e.g. for 2 TBs). What sense does it make to have priority only for the second, potentially much larger, DCI format? Why should the specifications prohibit a NW from including priority indicator in the first DCI format? What is the technical justification? |
| NTT DOCOMO | Support. Since ‘identifier for DCI formats’ is not needed for the first DCI format, the bit should be utilized for other purposes. |
| ZTE | Although our preference is to minimize the spec impact (Alt.2). But we are also ok to go with Alt.1. |
| OPPO | OK |
| ETRI | Support. |
| Spreadtrum | We prefer to reuse the legacy rule, where no priority indication is included in DCI 1_0. |
| Lenovo, Motorola Mobility | We don’t support this proposal. |
| CMCC | Support Alt 2 to hard-code the default low priority. |
| CATT | Prefer Alt2. |
| MediaTek | Not support. We support Alt 2 and reuse the legacy mechanism. |

## Feedback mode selection

Submitted Proposals

“RRC configured”

(Huawei) Proposal 7:

  - If PUCCH resources are configured for ACK/NACK based or NACK-only based feedback, the feedback mode is dynamically indicated by DCI. Otherwise, the feedback mode is semi-statically configured by RRC signaling.

(CMCC) Proposal 11:

  - The HARQ-ACK feedback mode for multicast is configured by RRC signalling and the configured HARQ-ACK feedback mode are applied for all G-RNTIs.

(Nokia) Proposal 9:

  - When the UE supports both ACK / NACK based and NACK-only based feedback for multicast, the UE can be configured with separate PUCCH resources for both ACK / NACK based and NACK-only feedback.

(Intel) Proposal 1:

  - For RRC_CONNECTED UEs receiving multicast supporting both ACK/NACK based and NACK-only HARQ feedback:

    - ACK/NACK based feedback is used for delivery mode with PTP or PTM Scheme 2 (if supported)

    - NACK only feedback is used for delivery modes 1 and 2 with PTM Scheme 1

    - UEs within a group receiving multicast transmission can be configured with different HARQ feedback modes.

(Intel) Proposal 5:

  - For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

- Explicit configuration: Semi-static RRC configuration of ACK/NACK or NACK only mode

- Implicit configuration:

- The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

- If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

(LGE) Proposal 9:

  - Select one of the following options to configure HARQ-ACK feedback scheme in PUCCH-config for multicast.

    - Option 1: Only one of NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in a PUCCH-config for multicast.

      - FFS: Whether/how to support ACK/NACK based HARQ-ACK if NACK only based HARQ-ACK is configured in PUCCH-config for multicast

      - FFS: Whether to support more than one PUCCH-config for multicast

    - Option 2: Both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in PUCCH-config for multicast.

      - FFS: Whether NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are can be jointly configured within a same PUCCH resource set or separately configured for different PUCCH resource sets.

(ETRI) Proposal 1:

  - Support at least RRC configuration for UEs to indicate HARQ-ACK feedback scheme to use.

(Qualcomm) Proposal 1:

  - For RRC_CONNECTED UEs, ACK/NACK and NACK-only can be configured by unicast RRC signaling.

    - -	FFS: ACK/NACK and NACK-only can be explicitly or implicitly selected via GC-PDCCH.

“Dynamic switching”

(Huawei) Proposal 7:

  - If PUCCH resources are configured for ACK/NACK based or NACK-only based feedback, the feedback mode is dynamically indicated by DCI. Otherwise, the feedback mode is semi-statically configured by RRC signaling.

(ZTE) Proposal 14:

  - Consider the following method to determine the feedback mechanism for the UE between ACK/NACK feedback and NACK-only feedback:

    - Both ACK/NACK PUCCH resources and NACK-only PUCCH resources for multicast are configured in the same PUCCH resource set.

    - PRI in DCI is used to indicate the feedback mechanism and PUCCH resources from the PUCCH resource set.

(Nokia) Proposal 10:

  - As a third option to Alt. 2, a dynamic mechanism via MAC-CE is introduced to configure which HARQ-ACK feedback scheme is used by the UE.

(Lenovo) Proposal 1:

  - Support dynamical HARQ-ACK feedback switch among Option 1 (Group NACK-only based feedback), Option 2 (UE-specific NACK-only based feedback), and Option 3 (UE-specific ACK/NACK-based feedback).

(MediaTek) Proposal 3:

  - Network can flexibly choose the HARQ-ACK mode and the HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

### Round-1

FL’s Comments

Whether RRC configures or DCI dynamically indicates the feedback mode may depend on the design details for NACK-only or the PUCCH configuration. For example, if no separate PUCCH-Config/ PUCCH-ConfigurationList configured implies no support of NACK-only, the mode selection is naturally up to network configuration by RRC. Otherwise, if UE can be configured with both PUCCH resources for ACK/NACK based and NACK-only based, dynamic indication of the feedback mode will be possible.

FL’s Proposal:

#### Proposal 4.2.1-1 (H)

If UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast, DCI indicates the feedback is ACK/NACK based or NACK-only based. Otherwise, the feedback mode is semi-statically configured by RRC signaling.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | In our view, even when the DCI indicates which feedback scheme to use in a PTM group, some UEs may individually be configured. This is needed, especially in case the gNB would like to disable the NACK-only feedback of only some UEs in bad channel conditions for them to not always occupy NACK-only resource, and assign them UE-specific ACK/NACK resources. Thus, it would be better if both RRC and DCI could configure the UEs’ HARQ-ACK feedback mechanism, just as in case of the WA for enabling/disabling. We propose: For selecting which HARQ-ACK feedback mechanism to use for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the function of group-common DCI indicating the which feedback mechanism to use. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates whether ACK/NACK based or NACK-only based HARQ-ACK feedback is used - Otherwise, which HARQ-ACK feedback mechanism to use is configured by RRC signaling. - FFS details. |
| vivo | For a certain G-RNTI, we think there is no motivation to support dynamic switching between ACK/NACK and NACK-only feedback mode. What’s more, it may bring additional issues and complexity to support DCI indication between ACK/NACK based or NACK-only based. If this proposal is for the case with multiple G-RNTI, we think the HARQ-ACK feedback mode is RRC configured per G-RNTI, and DCI indicates the feedback is ACK/NACK based or NACK-only based implicitly by G-RNTI. |
| Qualcomm | Need more clarification: Here the DCI is GC-DCI? Is it applied to the first DCI format, second DCI format, or both? Does it mean: If any UE in a multicast group is not configured with separate PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast, GC-DCI cannot dynamically indicate multicast feedback options? Again, we need to discuss whether separate PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback can be configured for multicast or not. |
| Samsung | Do not support. The benefits from introducing such functionality are unclear, especially since it can only be an optional UE feature that a portion of UEs may not support. Error cases also result from dynamic switching of the feedback mode. |
| Ericsson | Not support. If the UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast it must still be possible to RRC configure the type of HARQ feedback per G-RNTI, without using DCI indication. Similar to the DCI indication of HARQ enable/disable, a UE could however be configured to react on DCI signaling whether ACK/NACK or NACK-only feedback should be provided. A DCI bit could e.g. be configured, per UE, to switch between one of the following three pairs of states: ACK/NACK--no feedback, ACK/NACK--NACK-only, NACK-only--no feedback. |
| NTT DOCOMO | Generally fine |
| ZTE | We are OK with the above proposal. From our perspective, the PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast can be configured in the same PUCCH resource set. |
| OPPO | Agree with the proposal |
| ETRI | We are OK with the RRC signaling, but may need some more discussion for DCI indication |
| Lenovo, Motorola Mobility | OK |
| CMCC | Don’t support, we think semi-static configuration is enough. There is no motivation to support dynamic switching between two HARQ-ACK feedback modes, since NACK-only is use for huge UE number case and the join in and out of MBS group is a higher layer procedure which means the UE number in one MBS group will not change so frequently. In addition, DCI based solution needs more spec impact, e.g., which DCI is used. |
| CATT | Not support. It is more reasonable that the mode feedback is configured per G-RNTI. |
| MediaTek | #### As we commented in Proposal 3.3.1-3, even though the PUCCH-Config/ PUCCH-ConfigurationList is not configured for NACK-only case, the PUCCH resource for ACK/NACK can be used for NACK only. Thus, we suggest the proposal can be updated as following: If UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast, DCI indicates the feedback is ACK/NACK based or NACK-only based HARQ-ACK feedback for multicast. Otherwise, the feedback mode is semi-statically configured by RRC signaling. |

## Enable/disable HARQ-ACK feedback

Submitted Proposals

“Confirm WA”

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(NEC) Proposal 3

  - Confirm the main bullet of the following working assumption.

(CMCC)  Proposal 12:

  - Confirm the working assumption:

(Samsung) Proposal 1:

  - Confirm WA for HARQ-ACK enabling/disabling and remove all FFS.

    - Conclude how HARQ-ACK enabling/disabling applies for the Type-1 HARQ-ACK codebook.

    - Clarify that for SPS PDSCH, the UE follows the DCI indication as for DG PDSCH (when applicable).

    - For NACK-only, DCI-based enabling/disabling of HARQ-ACK report is not supported.

(MediaTek) Proposal 5:

  - Confirming the working assumption:

(DOCOMO) Proposal 2:

  - Confirm the working assumption on HARQ feedback enabling/disabling.

(ETRI) Proposal 2:

  - Confirm the working assumption:

“RRC signalling”

(Huawei) Proposal 8:

  - The enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by unicast RRC signaling.

(Huawei) Proposal 9:

  - If the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is not configured, whether UE feedbacks ACK/NACK or not is configured per G-RNTI by unicast RRC signalling.

(vivo) Proposal 1:

  - For indicating HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH

    - RRC signalling configures or DCI indicates the HARQ-ACK feedback option among three HARQ-ACK feedback options of no feedback, ACK/NACK feedback, NACK-only feedback.

(vivo) Proposal 2:

  - HARQ-ACK feedback option for multicast for RRC_CONNECTED UEs should be configured/indicated per g-RNTI.

    - When RRC disables HARQ-ACK feedback option indication function of group-common DCI, for RRC_CONNECTED UE receiving multicast, the HARQ-ACK feedback option is configured per g-RNTI.

    - When RRC enables HARQ-ACK feedback option indication function of group-common DCI, for RRC_CONNECTED UE receiving multicast, the HARQ-ACK feedback option is implicitly indicated by g-RNTI.

- (OPPO) Proposal 9:

  - For HARQ feedback enabling/disabling of dynamic group-common PDSCH:

    - RRC signalling for enabling/disabling the function of group-common DCI or enabling/disabling HARQ-ACK feedback is configured per G-RNTI.

    - 2 bits in group-common DCI are used to indicate whether/which HARQ feedback is enabled;

- (CATT) Proposal 4:

  - State 1(ACK/NACK enabling) and State 3 (enabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback ACK/NACK) cannot be configured simultaneously.

- (Nokia) Proposal 25:

  - RRC-based enabling / disabling of HARQ-ACK feedback per G-RNTI is used for MBS..

- (DOCOMO) Proposal 4:

  - RRC configuration for enabling/disabling HARQ-ACK feedback is per G-RNTI.

- (Qualcomm) Proposal 6:

  - For RRC_CONNECTED UEs, HARQ-ACK feedback can be enabled/disabled per G-RNTI by unicast RRC signaling.

- (Ericsson) Proposal 11:

  - Via RRC configuration each UE can be individually configured, for each G-RNTI, to use HARQ ACK/NACK, NACK-only or no HARQ feedback.

“DCI signalling”

(NEC) Proposal 4:

  - Reuse the existing field in the group-common DCI to indicate the HARQ-ACK feedback manner between ACK/NACK and NACK-only.

(NEC) Proposal 5:

  - Reuse the existing field in the group-common DCI to indicate whether ACK/NACK based HARQ-ACK feedback is enabled/disabled.

(CATT) Proposal 1:

  - For DCI indicating the HARQ ACK enabling/disabling, Alt 3 (A special state of an existing DCI field) and Alt 4(Implicit indication) can be supported.

(CMCC) Proposal 14:

  - One bit in the group-common DCI is used whether ACK/NACK based HARQ-ACK feedback is enabled/disabled.

(Samsung) Proposal 2:

  - DCI-based HARQ-ACK enabling/disabling is by a 1-bit field in first/second DCI formats for multicast.

(Lenovo) Proposal 2:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast, RRC signaling configures a non-numerical value in the K1 set and PDSCH-to-HARQ_timing indicator in the DCI indicates a numerical value or the non-numerical value for enabling or disabling the HARQ-ACK feedback.

(DOCOMO) Proposal 3:

  - For DCI indication for enabling/disabling HARQ-ACK feedback, reuse the same mechanism as sidelink scheduling, i.e. DCI format 3_0.

    - If PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled.

(Qualcomm) Proposal 7:

  - For RRC_CONNECTED UEs, dynamic enabling/disabling function of GC-DCI is implicitly indicated by HARQ process ID.

    - Enabling/disabling feedback is configured per HARQ process.

(Ericsson) Proposal 14:

  - The enable/disable indicator in the PDCCHs scrambled by same RNTI which point to the same PUCCH for HARQ feedback must be the same.

- “Applicability to NACK-only”

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(CATT) Proposal 2:

  - The enabling/disabling ACK/NACK-based HARQ-ACK feedback mechanism can be used for NACK-only based HARQ-ACK feedback.

(Nokia) Proposal 27:

  - Current WA is enhanced to also include NACK-only feedback, for which the same HARQ-ACK feedback enabling / disabling mechanisms to the ones agreed for ACK / NACK feedback apply.

(Lenovo) Proposal 3:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 can be straightforwardly extended to NACK-only based feedback and multiple G-RNTI cases.

(MediaTek) Proposal 4:

  - The ACK/NACK-based enabling/disabling HARQ-ACK feedback mechanism can be extended to apply to NACK-only based feedback enabling/disabling.

(Intel) Proposal 4:

  - For NACK-only feedback RRC configures ON/OFF with no DCI based indication.

(DOCOMO) Proposal 5:

  - DCI indication for enabling/disabling HARQ-ACK feedback also applies to NACK-only based feedback.

(ETRI) Proposal 4:

  - Support DCI indication for enabling/disabling NACK-only based HARQ-ACK feedback.

- “Applicability to HARQ-ACK codebook”

(CATT) Proposal 6:

  - When Type-1 HARQ-ACK feedback is configured, State 3 (The function of group-common DCI based indication) is not supported.

(Ericsson) Proposal 13:

  - The UE is only expected to follow the enable / disable indicator for HARQ feedback in group-common DCI when the two following conditions are fulfilled.

    - If it is configured to use the enable/disable indicator present in group common PDCCH

    - If the HARQ feedback codebook type is of type 2.

- “Applicability to retransmission”

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(CATT) Proposal 3:

  - The indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling same as initial transmission.

(Nokia) Proposal 27:

  - If HARQ-ACK feedback is enabled / disabled for a service, the UE provides / do not provide HARQ-ACK feedback also upon retransmissions.

(Lenovo) Proposal 4:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 can be supported for multicast retransmission.

(Ericsson) Proposal 15:

  - The enable/disable indicator in the group-common PDCCHs can be present for either initial transmission and retransmission as long as they are PTM transmission.

- “UE reaction”

(CATT) Proposal 5:

  - When only State 2 (RRC disabling) and State 3 (The function of group-common DCI based indication) are configured simultaneously, UE is allowed not to react to the DCI signaling but instead follow UE-specific RRC configuration for HARQ feedback.

(Intel) Proposal 3:

  - If RRC enables dynamic indication of HARQ on/off, UE always expects to be signaled with HARQ on/off in DCI i.e., there is no case where UE can ignore DCI based indication

(Ericsson) Proposal 12:

  - The presence of the Group common DCI indicator bit indicating the feedback type in the multicast DCI is RRC configured as part of the PDCCH format. Additionally, the UE is configured to monitor or ignore that bit.

    - UE configured to ignore the DCI indication for HARQ feedback type rely on the feedback type configured in RRC

    - UE configured to monitor the DCI indication for HARQ feedback type use the indicator in DCI to select one of two possible feedback types out of the possible three.

- “Applicability to SPS GC-PDSCH”

(vivo) Proposal 3:

  - For MBS for RRC_CONNECTED UEs, HARQ-ACK feedback option for SPS group-common PDSCH is configured per SPS PDSCH index.

(OPPO) Proposal 10:

  - HARQ-ACK feedback enabling/disabling of SPS group-common PDSCH is configured by RRC signaling.

(Lenovo) Proposal 6:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 can be supported for SPS group-common PDSCH transmission.

- “UE behavior”

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(MediaTek) Proposal 6:

  - The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

(MediaTek) Proposal 7:

  - Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

- “C/T-DAI issue”

(ZTE) Proposal 15:

  - Regarding DAI mismatch issue existing in the working assumption, if RAN1 prefers to address this issue via specification, network performs separate DAI counting for “PDCCH indicating HARQ-ACK enabling” and “PDCCH indicating HARQ-ACK disabling”.

(CMCC) Proposal 13

  - The HARQ-ACK of PDSCH that feedbacks in the same PUCCH should be enabled/disabled simultaneously:

(MediaTek) Proposal 6:

  - The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

(Intel) Proposal 9:

  - For Type 2 Codebook if HARQ is disabled by DCI for a GC-PDSCH, the C-DAI can for the GC-PDSCH can be ignored but the T-DAI is still indicated.

(Intel) Proposal 10:

  - When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback.

### Round-1

FL’s comments:

The working assumption was agreed to be updated as follows:

Agreement:

Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details.

- Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group

FUTUREWEI, NEC, CMCC, Samsung, MediaTek, DOCOMO, and ETRI proposed to confirm this working assumption.

However, as discussed in the last meeting, some companies are not quite sure how the working assumption is exactly going to work due to the discussion of RRC signaling and DCI signaling are still not clear yet. The discussion in this meeting can start from the RRC signaling and DCI signaling first. Once such discussion sets the same understanding of this mechanism among the group, confirming the working assumption will not be controversial hopefully.

FL’s Proposal:

#### Proposal 4.3.1-1 (H)

The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support the intention with different wording: RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback per G-RNTI. |
| Qualcomm | Need further discussion. For Rel-17 NR NTN, similar feature of enabling/disabling on HARQ feedback for downlink transmission has been supported, which is configured per HARQ process via UE specific RRC signaling. For a multicast service, the same QoS requirement should be applied to both initial and retransmission of a multicast TB using the same HARQ process. It is also desirable to enable/disable HARQ-ACK feedback per HARQ process. Especially in Rel-17, MBS could be supported by NTN and a unified solution is preferred. Based on the indicated HARQ process ID, the UE is implicitly indicated by GC-DCI whether HARQ-ACK feedback is enabled or disabled. The same way can be extended to select ACK/NACK-based or NACK-only based feedback per HARQ process. |
| Samsung | Prefer “per UE” as the conditions for enabling/disabling depend on a UE’s conditions. G-RNTI is a finer granularity and can be acceptable for progress. |
| Ericsson | Support |
| NTT DOCOMO | Support |
| ZTE | Agree with the above proposal. |
| OPPO | Fine with the proposal |
| ETRI | Support |
| Spreadtrum | Support |
| Lenovo, Motorola Mobility | We prefer per G-RNTI configuration so as a common configuration to the group of UEs. |
| CMCC | Support |
| MediaTek | Generally Ok. |

#### Proposal 4.3.1-2 (H)

If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support. |
| vivo | Support |
| Qualcomm | ok |
| Samsung | OK (same comments as before for the G-RNTI) |
| Ericsson | Support |
| NTT DOCOMO | Support |
| ZTE | Agree with the above proposal. |
| OPPO | Support the proposal |
| ETRI | Support |
| Spreadtrum | Support |
| Lenovo, Motorola Mobility | We prefer per G-RNTI configuration so as a common configuration to the group of UEs. |
| CMCC | Support |
| MediaTek | Support |

#### Proposal 4.3.1-3

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives:

- Alt1: Reuse one existing field in the group-common DCI.

- Alt2: Introduce a new field in the group-common DCI.

Collect views:

| Company | Comments |
| --- | --- |
| - LG | - We are fine with this proposal. |
| - Nokia, NSB | - Support the proposal. |
| - vivo | - We think it can be indicated by RNTI implicitly. There is no need to explicitly indicate. |
| - Qualcomm | - Is it to be applied to first DCI format, second DCI format or both? |
| - Samsung | - OK. We don’t think Alt. 1 can work as there can be UEs that do not support the feature and will probably need to use the existing field. |
| Ericsson | Support alt2. |
| NTT DOCOMO | Support. We prefer Alt1. Existing fields should be reused to minimize the size of DCI format. |
| ZTE | Ok with the above proposal. Our understanding is Alt.2. |
| ETRI | Support the proposal and prefer Alt2. |
| Spreadtrum | Support |
| Lenovo, Motorola Mobility | Support. |
| CMCC | Prefer alt 1 e.g., reuse ‘Identifier for DCI formats’ field |
| MediaTek | Support Alt 2. |

#### Proposal 4.3.1-4 (H)

Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback.

Collect views:

| Company | Comments |
| --- | --- |
| - LG | - We are fine with this proposal. |
| - Nokia, NSB | - Support. - Also, the WA should cover NACK-only feedback. |
| - Samsung | - OK. |
| - Ericsson | - Support |
| - NTT DOCOMO | - Support |
| - ZTE | - We are ok to confirm the work assumption. However, if companies prefer to address the issue of DAI mismatch, we also proposed one solution in our contribution, i.e., network performs separate DAI counting with the same DCI filed for “PDCCH indicating HARQ-ACK enabling” and “PDCCH indicating HARQ-ACK disabling. |
| - OPPO | - OK |
| - ETRI | - Support |
| - Spreadtrum | - Support |
| - CMCC | - Support |
| - CATT | - Support |
| - MediaTek | - Support |

## Retransmission

Submitted Proposals

“CBG-based retransmission”

(CATT) Proposal 28:

  - CBG based PTP retransmission can be supported in multicast retransmission when a UE is configured with CBG transmission for unicast.

(Nokia) Proposal 1:

  - CBG-based re-transmissions are not supported for PTM re-transmissions.

(Intel) Proposal 18:

  - For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

- “retransmission vs. Codebook”

(LGE) Proposal 6:

  - HARQ-ACK to PTP retransmission is treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook.

- “retransmission vs. HARQ-ACK feedback mode”

(MediaTek) Proposal 10:

  - PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

(MediaTek) Proposal 11:

  - PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

(Convida) Proposal 1:

  - PTM transmission scheme 2 should be supported for MBS retransmission.

### Round-1

FL’s Comments

Whether UE supports simultaneously PTM scheme 1 an PTP for retransmission is currently discussed in AI 8.12.1 and whether UE can dynamic switch between ACK/NACK based and NACK-only based HARQ-ACK feedback is also to be discussed. The issues related in this section can be discussed later.

Collect views:

| Company | Comments |
| --- | --- |

# HARQ-ACK feedback for multicast SPS

Submitted Proposals

- “NACK-only for SPS activation/deactivation”

(Huawei) Proposal 10:

  - For multicast SPS activation/deactivation, NACK-only based HARQ-ACK feedback is not supported.

(ZTE) Proposal 16:

  - Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

    - The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

    - The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

    - FFS: the feedback mode for deactivation PDCCH

(Spreadtrum) Proposal 8:

  - For the activation group-common PDCCH, the feedback option is ACK/NACK based feedback.

(vivo) Proposal 10:

  - NACK-only based HARQ-ACK feedback is not supported for multicast SPS activation/deactivation.

(CMCC) Proposal 15:

  - Support ACK/NACK based HARQ-ACK feedback for SPS activation/deactivation.

(Samsung) Proposal 8:

  - A gNB can configure to a UE a PUCCH resource for NACK-only feedback in response to detection of a DCI format activating/deactivating a SPS PDSCH configuration at configured slots.

(Lenovo) Proposal 14:

  - For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

(Lenovo) Proposal 15:

  - For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK for the SPS PDSCH.

(MediaTek) Proposal 8:

  - MBS SPS activation/deactivation’s feedback mechanism only support ACK/NACK based HARQ feedback mode.

(LGE) Proposal 14:

  - For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

    - UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

(ETRI) Proposal 3:

  - Support NACK-only based HARQ-ACK feedback for multicast PDSCH with PDCCH scheduling.

(Convida) Proposal 6:

  - NACK-only based HARQ-ACK feedback should be supported for MBS SPS PDSCH with PDCCH scheduling.

(Convida) Proposal 7:

  - Support HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI.

(Google) Proposal 1:

  - For SPS activation with NACK-only based HARQ feedback, consider following options.

    - Option 1: SPS is activated by RRC signaling (i.e. similar to the Type-1 configured grant)

    - Option 2: UE reports ACK/NACK based HARQ feedback to the SPS activation command in a UE specific PUCCH resource, and reports NACK-only base HARQ feedback to the SPS PDSCH transmission in group common PUCCH resource.

      - FFS: details

    - Option 3: MBS SPS is activated by a group common DCI. Handling the missing of group common DCI is based on network implementation.

(Google) Proposal 2:

  - For SPS deactivation with NACK-only based HARQ feedback, consider following options

    - Option 1: Network provides the MBS SPS transmission length information in the activating DCI. UE deactivates the SPS according to the indicated length.

    - Option 2: Apply ACK/NACK based HARQ feedback to the SPS deactivation command, and NACK-only base HARQ feedback to the SPS PDSCH transmission.

      - FFS: details

    - Option 3: MBS SPS can be deactivated by a group common DCI, handling the missing of the group common DCI missing is based on network implementation

“HARQ-ACK option for SPS multicast”

(Huawei) Proposal 12:

  - If PUCCH resources are configured for ACK/NACK based or NACK-only based feedback for multicast SPS, the feedback mode is dynamically indicated by SPS activation DCI. Otherwise, the feedback mode is semi-statically configured by RRC signaling.

(FUTUREWEI) Proposal 2:

  - For support of HARQ-ACK feedback for SPS multicast, use SPS-config configuration as a baseline for SPS multicast.

(ZTE) Proposal 17:

  - Support indicating the HARQ-ACK feedback mode (ACK/NACK or NACK-only) for multicast SPS PDSCH without PDCCH scheduling via SPS activation DCI.

(Spreadtrum) Proposal 6:

  - For SPS group-common PDSCH without PDCCH scheduling, support Alt1.

(Spreadtrum) Proposal 7:

  - For SPS group-common PDSCH without PDCCH scheduling, support activation PDCCH to enable/disable HARQ-ACK feedback based on UE’s capability.

(vivo) Proposal 3:

  - For MBS for RRC_CONNECTED UEs, HARQ-ACK feedback option for SPS group-common PDSCH is configured per SPS PDSCH index.

(OPPO) Proposal 11:

  - HARQ-ACK feedback enabling/disabling mechanism for dynamic group-common PDSCH is reused for the DCI activating/deactivating SPS configuration.

(CATT) Proposal 7:

  - For SPS group-common PDSCH, HARQ-ACK feedback for the first active SPS PDSCH is always enabled regardless of HARQ-ACK enabling/disabling configuration.

(CATT) Proposal 8:

  - If HARQ-ACK enabling/disabling is determined by DCI, the HARQ-ACK feedback of the PDSCH transmission without DCI (Case3) can be based on the DCI indication of the activation command.

(CATT) Proposal 25:

  - For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option can be configured per SPS configuration index (Alt1).

(CMCC) Proposal 16:

  - For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured per SPS configuration index.

(Nokia) Proposal 32:

  - For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is indicated in the SPS activation DCI.

(Samsung) Proposal 9:

  - The HARQ-ACK feedback option is configured per SPS configuration index.

(Lenovo) Proposal 16:

  - HARQ-ACK feedback option is indicated in the SPS activation DCI.

(Lenovo) Proposal 17:

  - One-bit identifier in the activation DCI format is reused for indicating NACK-only based feedback or ACK/NACK-based feedback.

(MediaTek) Proposal 9:

  - HARQ-ACK feedback option is indicated in the SPS activation DCI for multicast SPS PDSCH without PDCCH scheduling.

(DOCOMO) Proposal 6:

  - For selection of HARQ-ACK feedback scheme for multicast SPS PDSCH, support Alt 1.

(DOCOMO) Proposal 7:

  - HARQ-ACK feedback scheme for dynamically scheduled PDSCH for multicast is configured per priority index..

(Ericsson) Proposal 16:

  - For a UE configured to transmit multicast via SPS , configuration of the HARQ feedback option is done separately for each SPS-config.

    - ACK/NACK feedback is always transmitted regardless of the configured HARQ-ACK feedback option for a given SPS config when the UE responds to SPS activation or release, i.e. ACK-NACK feedback is always used to follow a PDCCH scrambled with G-CS-RNTI.

    - For SPS PDSCH without PDCCH, how to send HARQ feedback is according to the configuration of the feedback option for the SPS configuration.

    - If the UE is configured to monitor the DCI indicator for HARQ feedback options, for the SPS PDSCH which is associated with PDCCH activation and PDCCH deactivation, ACK/NACK feedback is always transmitted regardless of the indicator in this group-common PDCCH, i.e. ACK-NACK feedback is always used to follow a PDCCH scrambled with G-CS-RNTI. For SPS PDSCH without PDCCH, how to send HARQ feedback is according to the indicator in the group-common activation PDCCH.

(Ericsson) Proposal 22:

  - HARQ feedback option should be configured per SPS configuration even for SPS PDSCH.

    - FFS: whether SPS configurations with the same G-RNTI are expected to follow the same HARQ feedback option.

(Ericsson) Proposal 23:

  - If UE support group-common DCI based indication, HARQ feedback option can be indicated in SPS activation DCI.

Codebook related

(Huawei) Proposal 11:

  - When UE is configured with NACK-only for multicast SPS without PDCCH scheduling and when the HARQ-ACK bits for more than one multicast SPS configuration, for multicast SPS and unicast SPS, or for multicast SPS and dynamic scheduling of unicast/multicast, needs to be transmitted in the same PUCCH slot, the NACK-only based feedback for multicast SPS is transformed into the ACK/NACK based feedback.

(vivo) Proposal 5:

  - For the HARQ-ACK codebook for SPS PDSCHs, UE generates HARQ-ACK codebooks for SPS PDSCHs in the same way as that in Rel-16 regardless it is unicast SPS PDSCH or multicast SPS PDSCH.

- Further pursue NACK-only

(Nokia) Proposal 28:

  - Enhancements to SPS activation / deactivation mechanisms are needed to have reliable SPS grant in case of NACK-only feedback is used, in particular a mechanism for the gNB to be certain that all UEs have received the SPS grant would be desirable.

(Nokia) Proposal 29:

  - For NACK-only HARQ operation a mechanism should be used, in which UEs are made aware via RRC signalling that SPS might be used for an MBS and request retransmission of an SPS activation PDCCH only if they have not received it in a certain amount of time.

(Nokia) Proposal 30:

  - While gNBs can send SPS deactivation commands, that are in NACK-only mode not acknowledged by UEs, UEs can assume that SPS has been deactivated if they have not been able to decode a PDSCH for a certain period of time.

(Nokia) Proposal 31:

  - In NACK-only HARQ operation, a method is supported for UEs to check with the gNB whether an SPS (re-)activation has been sent by the gNB but missed by the UE. Options include:

    - (a) Option 1: Using a group-common uplink resource

    - (b) Option 2: Using UE-specific signalling (MAC-CE or RRC message)

    - (c) Other methods are not precluded.

(LGE) Proposal 18:

  - For group common SPS, group common PUCCH resources used for NACK only based HARQ-ACK is semi-statically configured per SPS configuration for SPS PDSCH transmissions.

- activation/deactivation related

(LGE) Proposal 15:

  - For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

(LGE) Proposal 16:

  - For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

(LGE) Proposal 17:

  - After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

    - The pre-determined time is determined by RRC and/or DCI.

- “HARQ for retransmission”

(LGE) Proposal 19:

  - For group common SPS retransmission, PUCCH resource is allocated by DCI of which CRC is scrambled by G-CS-RNTI

(LGE) Proposal 20:

  - Either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK is used for SPS PDSCH retransmission.

(LGE) Proposal 22:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, for multicast SPS and dynamically scheduled unicast, or for dynamically scheduled multicast and multicast SPS, UE determines a PUCCH resource based on one of the following options:

    - Option 1: The PRI of the last DCI with dynamic scheduling.

    - Option 2: SPS (e.g. if SPS is unicast)

    - Option 3: priority index

(LGE) Proposal 23:

  - For multiplexing HARQ-ACKs for multicast SPS and unicast SPS or for multicast SPS PDSCHs, UE determines a PUCCH resource based on one of the following options:

    - Option 1: the lowest SPS configuration index

    - Option 2: the highest priority of SPS between SPS configurations

    - Option 3: unicast SPS configuration (or multicast SPS configuration)

## Round-1

FL’s Comments

Due to the concern of indistinguishable between DCI missing and ACK skipped for NACK-only based feedback, majority supports DCI activation/deactivation only supports ACK/NACK based feedback.

Regarding the feedback mode selection between ACK/NACK and NACK-only for PDSCH without PDCCH scheduling,

Proponent of Alt1 (configurable per SPS configuration index):

- Spreadtrum, vivo, CATT, CMCC, Samsung, DOCOMO, Ericsson

Proponent of Alt2 (indicated in the activation DCI):

- ZTE, Nokia, Lenovo, MediaTek

The feedback mode selection for dynamic scheduling has not been decided yet, so the issue for SPS can be discussed later or we can try whether we can agree on the majority proposal, i.e., supporting Alt1.

FL’s Proposal:

#### Proposal 5.1-1

For multicast SPS activation/deactivation, only supports ACK/NACK based feedback.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | We do not support the proposal. We have provided an approach in our contribution to overcome the concern mentioned by companies regarding missed DCI for NACK-only feedback. With that, NACK-only feedback can also be used for SPS activation/deactivation. |
| vivo | Support |
| Qualcomm | Support |
| Samsung | It will be difficult for the network not being able to avoid having to support ACK/NACK. The overall benefits of NACK-only are also reduced. NACK-only can also be supported based on NW configuration as we describe in our Tdoc. However, we understand it is getting late and will not object to concluding if a “good” number of companies do not care about allowing a NW to operate MBS using only NACK-only. |
| Ericsson | ok |
| NTT DOCOMO | Support |
| ZTE | Ok with the above proposal. It can prevents the unnecessary ambiguity for SPS activation/deactivation. |
| Lenovo, Motorola Mobility | We prefer ACK only feedback. In case the DCI is missed, how can UE identify that and transmit NACK to gNB? |
| CMCC | Support |
| MediaTek | support |

#### Proposal 5.1-2 (H)

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured per SPS configuration index.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | If the companies think that configuring UEs individually by RRC signalling does not create overhead in case the gNB would like to change the feedback mode, we can support the proposal to not block the progress. |
| vivo | We think HARQ-ACK feedback option also includes on HARQ-ACK feedback. We the proposal can be changed as following: For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option among {ACK/NACK, NACK-only, disabled feedback} is configured per SPS configuration index. |
| Qualcomm | We are generally fine with RRC configured feedback options (ACK/NACK-based, NACK-only and no HARQ feedback) for SPS GC-PDSCH without PDCCH scheduling. However, considering Proposal 4.3.1-2, we propose to consider the following alternatives: #### Proposal 5.1-2 (H) For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by unicast RRC signaling and down select one of the following alternatives: Alt1: per SPS configuration index. Alt2: per G-CS-RNTI |
| Samsung | OK for progress with the proposal and with the modification by Vivo. |
| Ericsson | ok |
| ZTE | Based on our understanding, Alt2 (indicated in the activation DCI) won’t incur any additional specification impact other than adding one field in the DCI. However, Alt2 can offer much flexibility for the network, which is preferred from our perspective. |
| OPPO | OK |
| Spreadtrum | Fine with the proposal, also fine with the updated version from vivo. |
| Lenovo, Motorola Mobility | OK |
| CMCC | Support |
| CATT | Support |

# PDSCH repetition

Submitted Proposals

(vivo) Proposal 12:

  - For PDSCH repetition of group-common PDSCH, if a UE is configured with multiple g-RNTIs,

- The PDSCH aggregation factors for group-common PDSCHs with different g-RNITs can be separately configured.

(Nokia) Proposal 24:

  - For Config A, pdsch-AggregationFactor is per multicast / broadcast service.

(LGE) Proposal 24:

  - For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

(Qualcomm) Proposal 9:

  - For SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

  - Config A and Config B can be supported for SPS GC-PDSCH.

    - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast.

    - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast.

  - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is equal to 1.

## Round-1

FL’s Comments

A couple of proposals seem straightforward and may have been considered in the RRC parameters list so it would be good to have the proposals agreed for further more other relevant discussion.

#### Proposal 6.1-1 (H)

If configured, the pdsch-AggregationFactor for multicast is configured per G-RNTI.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support. |
| vivo | Support |
| Qualcomm | Ok |
| Samsung | OK. |
| Ericsson | ok |
| NTT DOCOMO | Support |
| ZTE | Support. |
| OPPO | OK |
| Spreadtrum | We agree with the intention of the proposal. But some clarification is needed. - The parameter pdsch-AggregationFactor is configured for SPS, and each SPS configuration is associated with G-CS-RNTI not G-RNTI. Thus, in our understanding, the proposal shall be updated as below: #### Proposal 6.1-1 (H) If configured, the pdsch-AggregationFactor for multicast is configured per G-CS-RNTI. |
| CMCC | Support |
| CATT | Support |

#### Proposal 6.1-2 (H)

For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

  - Config A or Config B can be configured to UE:

    - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast.

    - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast.

  - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is equal to 1.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| Nokia, NSB | Support. |
| vivo | - Support in principle. For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value should be the same as the default for DG PDSCH.. |
| Qualcomm | - ok |
| Samsung | - OK – but why list all individual parameters when we agree that PDSCH-Config can be separately provided for multicast? What does that mean for the rest of the parameters in PDSCH-Config? |
| Ericsson | ok |
| NTT DOCOMO | Support |
| ZTE | We are OK with the above proposal. |
| OPPO | OK |
| Spreadtrum | Support |
| CMCC | Support |
| CATT | Support |

# CSI feedback and trigger

Submitted Proposals

(CATT) Proposal 29:

  - CSI feedback enhancement for MBS can be further studied and discussed but with low priority.

(DOCOMO) Proposal 14

  - Include an aperiodic CSI report trigger in DCI format 1_1 for multicast.

(DOCOMO) Proposal 15:

  - Support aperiodic CSI reporting on PUCCH.

(Ericsson) Proposal 25:

  - The existing Rel. 15/16 framework of CSI feedback is reused for multicast/PTM.

## Round-1

FL’s Comments

Since we have agreed that existing CSI feedback can be used for multicast transmission and whether enhancement is needed needs further study, the situation does change much comparing to previous meetings.

FL does not see probability to converge to a point more than FFS, so suggest coming back later when situation changes.

Collect views:

| Company | Comments |
| --- | --- |

# Other miscellaneous proposals

Submitted Proposals

(FUTUREWEI) Proposal 3:

  - For the same service, all RRC_CONNECTED UEs receiving multicast are configured with the same MCS table.

(FUTUREWEI) Proposal 4:

  - For MBS, FBRM should be used to improve performance for small sized TBs.

(CATT) Proposal 26:

  - To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

(CATT) Proposal 27:

  - UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

(Nokia) Proposal 6:

  - A mechanism is adopted to disable HARQ-ACK feedback (and optionally CSI reporting) of the outlier UEs. This is down-selected from the following:

    - The UE detects itself that it is an outlier UE (e.g. if the reliability criteria cannot be met for a specific amount of time) and disables its own HARQ-ACK feedback mechanism.

    - The UE detects itself that it is an outlier UE and sends a request to the gNB to disable its feedback (e.g. using a specific CQI value such as CQI 0).

(Nokia) Proposal 7:

  - An outlier UE’s NACK-only HARQ-ACK feedback on group-common PUCCH resources can be re-activated by using one of the following methods:

    - The gNB can assign dedicated ACK / NACK resources to the UE and / or the UE can keep reporting CSI feedback, so that the gNB can decide when to re-activate the NACK-only based HARQ-ACK feedback of the UE.

    - The UE can be configured with a QoS criterion, such as an average BLER, and by satisfying such a criterion, the UE can either directly re-activate its feedback mechanism, or request gNB to allow the UE to utilize the group-common HARQ-ACK resource for NACK-only HARQ-ACK feedback.

(Nokia) Proposal 22:

  - RAN1 decides whether the UE produces HARQ-ACK bits for all the multicast services that the UE had indicated interest or only for services that the UE has received PDSCH(s) when constructing multiplexed Type-1 codebook for FDM-ed unicast / multicast case.

(Nokia) Proposal 23:

  - To overcome the problem of HARQ-ACK codebook size ambiguity, the constructed HARQ-ACK sub-codebooks are prepended by the UE with the information (with a known size) about which service the following sub-codebook belongs to using one of the following methods:

    - i.	The received services can be indexed, and that index can be prepended to the HARQ-ACK sub-codebook:

      - The services can be indexed based on the order of RNTIs. For instance, unicast can be of index 0, the service with minimum configured G-RNTI value is index 1, service with second minimum G-RNTI is index 2 ….

      - A mapping of RNTI to index can be directly signaled by the gNB via RRC signaling, e.g., during RNTI configuration.

      - Same indexing with the concatenation order of the sub-codebooks can be used.

(Nokia) Proposal 33:

  - On top of MIMO precoding as widely applied in NR unicast that is transparent to the receiver, non-transparent diversity techniques such as LD-CDD known from LTE are supported for group-common PDSCH transmission.

(Intel) Proposal 2:

  - For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

(LGE) Proposal 25:

  - Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

FL’s Comments

The proposals summarized in this section are either interested by only a single company or can be discussed in other section/agenda. Hence, FL does not plan to handle the proposals in this section in this meeting.

Collect views:

| Company | Comments |
| --- | --- |

# Proposals for 1st GTW session

#### Proposal 4.3.1-1-r1 (H)

The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI.

#### Proposal 4.3.1-2 (H)

If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

#### Proposal 4.3.1-4 (H)

Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback.

#### Proposal 2.3.1-1-r1 (H)

TDRA table for multicast can be separately configured from that PDSCH-Config in the associated unicast BWP .

- If TDRA table is not separately configured for multicast, the TDRA table for unicast also applies to multicast.

#### Proposal  3.2.1-1-r1

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit.

- This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

- Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

- FFS the case of PUCCH for SR.

#### Proposal  3.2.1-2

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further  decide on the following alternatives with potential down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

# References

R1-2108724	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2108805	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2108852	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2108927	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2109002	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2109068	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2109136	Discussion on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2109195	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2109304	Discussion on reliability improvement	CMCC

R1-2109317	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2109516	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2109539	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2109568	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2109634	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2109702	Discussion on mechanisms to improve reliability for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2109768	PUCCH formats for NACK-ONLY based HARQ-ACK feedback	TD Tech

R1-2109984	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2110057	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2110075	Discussion on HARQ-ACK feedback mechanism for multicast of RRC_CONNECTED UEs	ETRI

R1-2110119	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2110211	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2110250	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

R1-2110356	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

# Appendix Agreements summary for AI 8.12.2

## 102e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

## 103e

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

## 104e

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on   the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

## 104be

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

## 105e

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Agreement:

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.

Agreement:

NR supports at least the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting.

Agreement:

For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- Agreement:

- Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Conclusion:

PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Agreement:

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

## 106e

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

Agreement:

When UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks.

- FFS: How UE is configured one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities.

Agreement:

For a UE configured with Type-1 HARQ-ACK codebook,

- If UE is not configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for TDM-ed unicast and multicast.

- If UE is configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for FDM-ed unicast and multicast.

Agreement:

For UEs supporting ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the following values are unchanged compared to unicast in Rel-16:

  - The maximum number of PUCCH resources sets in each PUCCH-Config,

  - The maximum number of PUCCH resources in a PUCCH resource set in each PUCCH-Config,

  - The maximum number of UCI information bits for the first PUCCH resource set.

  - The total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList.

  - Note:

    - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

    - The case of NACK-only based is discussed separately.

Agreement:

- When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for ACK/NACK based feedback for multicast, it is applied to all G-RNTIs configured to UE.

Agreement:

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

  - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

  - FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

The priority index is,

- for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format for GC-PDCCH, down-select from:

    - Alt1: Optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

    - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

  - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - Alt2: Support sub-slot based PUCCH for this case.

  - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

  - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - Alt5: NACK-only bundling

Agreement:

When UE supports and is configured with more than one G-RNTI,

  - for Type-2 codebook construction, DAI is separately counted per G-RNTI.

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

Agreement:

Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details.

- Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group

Agreement

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, select one or more of the following alternatives:

  - Alt1: HARQ-ACK feedback option is configured per SPS configuration index.

  - Alt2: HARQ-ACK feedback option is indicated in the SPS activation DCI.

  - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.