3GPP TSG RAN WG1 Meeting #106bis-e 	R1-2110506

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 2)

Document for:	Discussion and Decision

# Introduction

- P5 (FDD CSI) at #24 at NWM

- P11 from RAN1 Email Reflector due to the issue of MS Equation

- P24 (MTRP CSI) at # 31at NWM

# Summary of CSI enhancement for FDD

Proposal 5: With regarding to parameter combinations, following 8 parameter combinations are supported in Rel-17 PS codebook:

| M | Alpha | Beta |
| --- | --- | --- |
| 1 | 1 | 1 |
| 1 | 1 | 3/4 |
| 1 | 1 | 1/2 |
| 1 | 3/4 | 1/2 |
| 2 | 1 | 3/4 |
| 2 | 1 | 1/2 |
| 2 | 1/2 | 3/4 |
| 2 | 1/2 | 1/2 |

FFS: whether further restrictions/dependences for given parameter combination(s) are needed

[FL note]

- Vivo has commented to prefer to switch the parameter combination {2, ½, 3/4} as {2, ¾, 1/2}

Proposal 11: For the priority of mapping coefficients for Rel17 PS codebook, study the following alternatives and down-select one or more alternatives in RAN1 107:

- Alt 1: Support mapping coefficients firstly across port indices, secondly across FD basis indices, and thirdly across layers, i.e. priority value is given by the priority value Pril,i,f=K1⋅M⋅l+K1⋅f+i

- Alt 2: Support mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅i+l

- Alt 3: Support mapping coefficients firstly across layers, secondly across port indices, and thirdly across FD basis indices, i.e., the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l

  - FFS port permutation function ψ(i)

Note that other solutions are not excluded.

# Summary of CSI enhancement for MTRP

Proposal 24: To confirm the order of UCI payload construction for reported CSIs, down-select one from the following:

- Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214.

  - i.e. introducing priority index corresponding to single-TRP or NCJT measurement hypothesis type. It also implies that one CSI reporting setting for NCJT measurement reporting can contain multiple CSI reports each of which corresponds to one single-TRP or NCJT measurement hypothesis

- Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

  - i.e. introducing mapping order of CSI fields in the order of MTRP CSI, the first TRP CSI, and the second TRP CSI. It also implies that one CSI reporting setting for NCJT measurement reporting contains single CSI report which corresponds multiple single-TRP and/or NCJT measurement hypotheses

  - FFS whether modifying the table of priority reporting levels for Part 2 CSI is needed for prioritized single-TRP or NCJT measurement hypothesis type for Part 2 CSI

[FL note]

- Alt 1: MTK, QC, DOCOMO, Samsung, Intel, CATT, Motorola

- Alt 4: Oppo, ZTE, Vivo, CMCC, Nokia, Apple, Spreadtrum, LGE, Nokia, Fraunhofer, Ericsson