3GPP TSG RAN WG1 Meeting #106bis-e	 R1-2110658

E-meeting, October 11th – 19th, 2021

Title:	Draft LS on triggering signalling of temporary RS for SCell activation

Response to:

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2

Source:	Huawei [RAN WG1]

To:	RAN WG2

Cc:	RAN WG4

Contact Person:

#### Name:	Frank Long

E-mail Address:	frank.longyi {@ }huawei.com

Attachments:	[RRC parameters]

1. Overall Description:

Regarding the triggering signalling of temporary RS for SCell activation, RAN1 has reached the following agreements,

Agreement For efficient activation of SCells - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS Note: Separate from the support of Option 1a, it is up to RAN4 whether or not to consider an activation time enhancement for Option 2 without requiring further RAN1 work - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding Rel-15/16 A-TRS(s) Send an LS to RAN4. The LS is endorsed in R1-2104110. Agreement To trigger temporary RS, - MAC-CE at least provides the following information: - temporary RSs are to be triggered on X out of Y (Y≥X) to-be-activated SCells, respectively, while no temporary RS is to be triggered on the other to-be-activated SCells. - The following information can be provided by RRC for temporary RS for each SCell - The number of RS bursts and the gap length between the RS bursts (Opt 2.3.3) - Triggering offset of temporary RS (Opt 2.3.4) - Triggering offset can be provided, e.g., by reusing existing CSI-RS framework - QCL information (Opt 2.3.5) - Triggering QCL information can be provided, e.g., by reusing existing CSI-RS framework - A unique temporary RS configuration index - FFS: the maximum number of temporary RS per cell/per UE Note: Reusing A-TRS triggering framework is not precluded. - Information for 0, 1, or more temporary RS can be provided for each configured SCell Agreement - Provide the functionality to be fulfilled, as well as the status about the understanding on Alt 1 and Alt 2, which could be provided by examples (including respective possible RRC parameters, if agreed, required by Alt 1 and Alt 2) to facilitate RAN2’ understanding. - Send LS to ask RAN2 to consider the following alternatives and finalize the MAC-CE or RRC signalling design, including parameters. - RAN1 only needs to focus on RRC parameters examples, if needed. - List of RAN1 endorsed RRC parameters for this issue will not be sent to RAN2 Alt 1: Bitmap approach in MAC-CE - Every Z-bit block in the bitmap corresponds to a SCell, Z>=0 - A Z-bit block indicates the temporary RS [configuration index], and a value zero indicated by the bit block means no RS resource transmitted. - The to-be-activated SCell is indicated via the C values in the legacy SCell activation/de-activation MAC CE or in the new MAC-CE Alt 2: Reuse A-TRS triggering framework - A trigger state is indicated by the MAC-CE explicitly - The association between a trigger state and temporary RS for one or multiple SCells is configured by RRC according Rel-16 A-TRS triggering framework - FFS: The value zero of the MAC-CE indication means no temporary RS is triggered by the MAC-CE for all to-be-activated SCells

Both alternatives above, i.e. Alt 1 and Alt 2 are different in term of MAC-CE design and requires different RRC signalling. Because RAN1 has no consensus on which alternative can result in a better signalling design, RAN1 did not further select one from them and thus respectfully requests RAN2 to consider the two alternatives and finalize the design of MAC-CE and RRC signalling. RAN1 has no intention to support both alternatives.

To facilitate the RAN2 discussion on RRC signalling required for either alternative, two example sets of RRC parameters are provided, respectively, as attachment. It is up to RAN2 to finalize the RRC signalling design.

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully requests RAN2 to consider the two alternatives above, i.e. Alt 1 and Alt 2, and finalize the design of MAC-CE and RRC signalling.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#107-e		11 - 19 November 2021   	Online

3GPP RAN1#107-bis-e		17 - 25 January 2022   	Online