3GPP TSG-RAN WG1 Meeting #106bis-e	R1-21xxxxx

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.9.2

Source:	Moderator (Ericsson)

Title:	Feature Lead Summary [106bis-e-LTE-Rel17-NB-IoT-eMTC-02] - final checkpoint

Document for:	Discussion and Decision

# 1	Introduction

In the Work Item (WI) on “Additional enhancements for NB-IoT and LTE-MTC” [1], one of the objectives is to specify the following enhancement for LTE-MTC:

- Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC] [RAN1]

This feature lead summary (FLS) collects companies’ views as described in [2-6], as well as the working assumption and discussions that took place during the corresponding GTW session.

Annex 1 contains the agreements reached in RAN1 #102-e [7], RAN1 #103-e [8], RAN1 #104-e [9], RAN1 #104-bis-e [10], RAN1 #105-e [11], and RAN1 #106bis-e [12]. The FLS discussed during the first GTW session can be found in [13].

# 2	FLS on 14 HARQ processes in DL in LTE-MTC

## 2.1	Potential Agreements for Endorsement

There is a number potential agreements that have been stable (some of them even from RAN1# 106-e), thus to speed-up the progress of this Rel-17 objective it is seems possible to agree on the following potential conclusion and potential agreements in a “single-shot”.

Potential Conclusion#1:

Conclusion: How to implement/describe the states, e.g., table, resulting from the joint encoding solution of Alt-2e is left up to the Editor, based on the agreements for the PDSCH scheduling delay, HARQ-ACK delay and the WA confirmed for Alt-2e.

Potential Agreement#4:

The Rel-17 14 HARQ processes feature only applies to User Specific Search Space (USS)

Potential Agreement#5:

In Rel-17, for the 14 HARQ processes feature the “HARQ-ACK process number” field uses 4-bits.

- The mapping associated to the 4-bits of this field is updated to include the newly added HARQ processes (i.e., 11th, 12th, 13th, and 14th HARQ processes).

Potential Agreement#6:

In Rel-17, for the 14 HARQ processes feature the following updates on the technical specification are to be performed.

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes.

Companies are kindly requested to provide their views below:

| Company | Please confirm that Potential Conclusion#1, Potential Agreement#4, #5, and #6 can be agreed. | Comments |
| --- | --- | --- |

## 2.2	Open issues

In subsections below refer to set of issues that require a further discussion.

### 2.2.1	“Repetition number” field: 2 bits

Background: During the GTW session it was discussed whether the “Repetition number” field can be set to zero for the 14 HARQ processes feature as to make use of it for joint-encoding purposes. The following three options were considered:

Potential Agreement#2:

In Rel-17, for the 14 HARQ processes feature the “Repetition number” field is:

        Opt-1: 0-bits when the “HARQ-ACK bundling flag” is set to 1 (i.e., 2-bits from this field become available for jointly-encoding purposes).

        Opt-2: 0-bits when the 14 HARQ processes feature is configured (i.e., 2-bits from this field become available for jointly-encoding purposes).

        Opt-3: 2-bits as in legacy.

During the GTW session there were two trends, companies that think that is important to keep usable the “Repetition number” field (as per Opt-1 or Opt-3), and companies that prefer to set the “Repetition number” field to zero-bits from the moment the 14 HARQ processes feature is configured (which means that the “Repetition number” field would remain unusable as long as the 14 HARQ processes feature is configured).

Towards the end of the GTW session there was a comment questioning whether Opt-3 will require any modification on the PDSCH scheduling delay and HARQ-ACK delay expressions, the online time of the GTW session ran-out and it was no time to answer. In relation with it, below there is an illustration showing that keeping usable the “Repetition number” field will not require any additional modification.

- PDSCH scheduling delay used along with HARQ-ACK delay configured with Alt-1

- Handling of the 11th HARQ process (associated to MPDCCH#10 and PDSCH#10) in a single HARQ process scenario without PDSCH repetition:

| subframe # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPDCCH | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH | 12 | 13 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  |  |  |  | 10 |  |  |  |  |  |  |  |
| PUCCH# HARQ-ACK |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 2 |  |  |  |  |  | 0 |  |  |  |

Note: Recall that the minimum HARQ-ACK delay that can be used encompasses 4 subframes.

Let’s focus on MPDCCH#10 & PDSCH#10:

  - The PDSCH scheduling delay is 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

  - The HARQ-ACK delay counting starts from subframe#18 and will be 2 BL/CE DL subframes + 1 subframe + 1 BL/CE UL subframe.

- Handling of the 11th HARQ processes (associated to MPDCCH#10 and PDSCH#10) in a single HARQ process scenario with PDSCH repetitions.

| subframe # | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPDCCH | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH | 12 | 13 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |  |  |  |  | 10 | 10 | 10 | 10 |  |  |  |  |
| PUCCH# HARQ-ACK |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 | 2 |  |  |  |  |  |  |  |  | 0 |

Note: Recall that the minimum HARQ-ACK delay that can be used encompasses 4 subframes.

Let’s focus on MPDCCH#10 & PDSCH#10:

  - The PDSCH scheduling delay is 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

  - The HARQ-ACK delay counting starts from subframe#21 and will be 2 BL/CE DL subframes + 1 subframe + 1 BL/CE UL subframe.

Observations from A) and B):

- We can see how the PDSCH scheduling and HARQ-ACK delay expressions do not require any modification, since indeed the required delays are the same for a single HARQ process scenario with and without PDSCH repetition. The same conclusion applies when Alt-2e is configured.

- Keeping usable the “Repetition number” will allow us to handle situations where the radio conditions deteriorate (which is an unpredictable matter) and where the only suitable choice will be using 1 HARQ process along with PDSCH repetitions (It is worth noting that performing a reconfiguration in such scenario (as per Opt-2) is likely to fail since repetitions would be needed).

- Moreover, it is important to make clear that Opt-3 does not purse any modification on the PDSCH scheduling delay and HARQ-ACK delay, and that the PDSCH repetitions case is connected to the 1 HARQ process case.

| Company | Please state your views | Comments |
| --- | --- | --- |

### 2.2.2	“HARQ-ACK delay” field: 3 bits

Background: In Rel-17, the HARQ-ACK delay (either based on Alt-1 or Alt-2e) will be indicated together with the PDSCH Scheduling delay through joint-encoding using a single DCI field. Thus, it is under discussion whether this legacy DCI field will be set to 0-bits depending on the status of the HARQ-ACK bundling flag, or if it will set to 0-bits from the moment the 14 HARQ process feature is configured.

Potential Agreement#3:

In Rel-17, for the 14 HARQ processes feature the “HARQ-ACK delay” field is:

         Opt-1: 0-bits when the "HARQ-ACK bundling flag" is set to 1 (i.e., 3-bits from this field become available for jointly-encoding purposes)

         Opt-2: 0-bits when the 14 HARQ processes feature is configured (i.e., 3-bits from this field become available for jointly-encoding purposes).

Companies are kindly requested to provide their views below:

| Company | - Please state your views | Comments |
| --- | --- | --- |

### 2.2.3	HARQ-ACK delay set(s) for Alt-2e

Background: During the GTW session, the following Working Assumption was reached:

Working Assumption For the joint encoding of “PDSCH Scheduling delay” and “HARQ-ACK delay” when Alt-2e is configured, the HARQ-ACK delay set has a size of: - Alt-C: - 12 elements: HARQ-ACK delay set = {a, b, c, d, e, f, g, h, i, j, k, l} for the PDSCH Scheduling delay expression associated to the delay of 2. - 10 elements: HARQ-ACK delay set = {o, p, q, r, s, t, u, v, x, w} for the two PDSCH Scheduling delay expressions associated to the delay of 7. - FFS: The values of {a, b, c, d, e, f, g, h, i, j, k, l}, {o, p, q, r, s, t, u, v, x, w} where some of these elements may share the same value. Note 1: The highlighted delay values in the alternatives above correspond to either range1 = {4, 5, 7, 9, 11, 13, 15, 17} or range2 = {4, 5, 6, 7, 8, 9, 10, 11}. Note 2: Only one alternative under the umbrella of either Alt-A or Alt-B or Alt-C is to be selected.

| Company | Please provide your initial views on what should be the delay values for HARQ-ACK delay sets of Alt-C. | Comments |
| --- | --- | --- |

# 3	References

RP-201306, WID: Additional enhancements for NB-IoT and LTE-MTC, RAN #88e, Electronic Meeting, June 29th-3rd, 2020.

R1-2108778, “Support of 14-HARQ processes in DL for HD-FDD MTC UEs,” Huawei, HiSilicon, RAN1 #106bis-e, October 11th – 19th, 2021.

R1-2109315, “Support of 14-HARQ processes in DL for eMTC,” Nokia, Nokia Shanghai Bell, RAN1 #106bis-e, October 11th – 19th, 2021.

R1-2109338, “Remaining issues on 14-HARQ processes in DL for eMTC,” ZTE, RAN1 #106bis-e, October 11th – 19th, 2021.

R1-2109175, “Support of 14 HARQ processes and scheduling delay,” Qualcomm Incorporated, RAN1 #106bis-e, October 11th – 19th, 2021.

R1-2110317, “Support of 14 HARQ processes in DL in LTE-MTC,” Ericsson, RAN1 #106bis-e, October 11th – 19th, 2021.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #102-e, e-Meeting, August 17th – 28th, 2020.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting, October 26th – November 13th, 2020.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #104-bis-e, e-Meeting, April 12th – 20th, 2021.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 10th – 27th, 2021.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #106-e, e-Meeting, August 16th – 27th, 2021.

R1-2110413, “Feature Lead Summary [106bis-e-LTE-Rel17-NB-IoT-eMTC-02] - first checkpoint”, Moderator (Ericsson), RAN1 #106bis-e, October 11th – 19th, 2021.

# Annex 1

## A1.1 List of agreements from RAN1 #102-e:

Agreement

Introduce a new RRC configuration parameter to enable 14 HARQ processes.

Agreement

For a UE configured with 14 HARQ processes, a PDSCH scheduling delay of 2 BL/CE DL subframes and 7 [FFS subframes type(s)] is supported at least in the PUCCH non-repetition case:

- FFS details of signaling.

- FFS other delay values to account for the presence of non-BL/CE subframes in the PUCCH non-repetition case.

- FFS if the 14 HARQ processes feature is supported in PUCCH repetition case.

Working Assumption

Introduce a new optional UE capability to support 14 HARQ processes

## A1.2 List of agreements from RAN1 #103-e:

Agreement

The following working assumption is confirmed

Introduce a new optional UE capability to support 14 HARQ processes

Agreement

The design of the 14 HARQ processes feature accounts for the presence of non-BL/CE UL and DL subframes in the PUCCH non-repetition case.

- FFS: PDSCH scheduling delays

- FFS: HARQ-ACK delays

- FFS: Configurable/dynamic set of PDSCH delays/HARQ-ACK delays

For future meetings:

Companies to further study on the impact of measurement gaps on the 14 HARQ processes feature.

Agreement

For the support of 14 HARQ processes, the solution to assign PDSCH scheduling delays should be able to minimize unnecessary waste of subframes derived from the presence of non-BL/CE DL subframes and non-BL/CE UL subframes.

- The following solutions will be further investigated:

  - The indication of subframe types for the PDSCH scheduling delay of 7 are:

    - 1 BL/CE DL subframe + 1 subframe + 3 [BL/CE UL subframes] + 1 subframe + 1 BL/CE DL subframe.

    - 1 subframe + 3 [BL/CE UL subframes] + 1 subframe + 2 BL/CE DL subframes.

  - Configurable delays including other values than 2 and 7.

- Other solutions are not precluded.

Agreement

For the support of 14 HARQ processes, the solution to assign HARQ-ACK delays should aim to maximize the number of HARQ processes that can be scheduled in presence of non-BL/CE DL subframes and non-BL/CE UL subframes.

- Different percentages of presence of non-BL/CE subframes can be analyzed as to represent typical scenarios and determine which HARQ-ACK delays should be included.

## A1.3 List of agreements from RAN1 #104-e:

Agreement

The PDSCH scheduling delay for the PUCCH non-repetition case (i.e., PUCCH repetitions = 1):

- 2 BL/CE DL subframes.

- The PDSCH scheduling delay of 7 is expressed as:

  - 1 BL/CE DL subframe + 1 subframe + [3 subframes] + 1 subframe + 1 BL/CE DL subframe.

  - 1 subframe + [3 subframes] + 1 subframe + 2 BL/CE DL subframes.

Agreement

For the 14 HARQ processes feature, when PUCCH is used with 1 repetition and there is presence of non-BL/CE UL subframes (i.e., invalid UL subframes):

- The term surrounded by brackets in Solution 1 is resolved as 3 BL/CE UL subframes.

## A1.4 List of agreements from RAN1 #104-bis-e:

Agreement

In Rel-17, for the 14 HARQ processes feature, PUCCH repetition is not supported with HARQ-ACK bundling.

Conclusion

In Rel-17, the 14 HARQ processes feature is not supported when the multi-TB grant feature is enabled.

R1-2103860	Feature Lead Summary [104b-e-LTE-Rel17_NB_IoT_eMTC-02]: 2nd check point	Moderator (Ericsson)

Agreement

In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be down-selected in RAN1#105-e from:

- Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

  - FFS: The expression consisting of different subframe types.

  - FFS: Signaling Details.

- Alt-2: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

  - FFS: The percentage of presence of non-BL/CE DL subframes and non-BL/CE UL subframes to be handled.

  - FFS: HARQ-ACK delay values and length of the HARQ-ACK delay set.

  - FFS: Signaling Details.

The following aspects will be considered towards the down-selection of one of the two alternatives (i.e., Alt-1 or Alt-2) for the HARQ-ACK delay solution:

- Total number of bits required in DCI

- Scenarios that can be handled, including:

(a) different numbers of scheduled HARQ processes per burst (including dynamically switching between more than 10 HARQ processes and 10 or less HARQ processes)

(b) different % of invalid subframes for both 10 and 40 SF long bitmaps

- Robustness against loss of DCIs

- Flexibility

- RRC signaling overhead

## A1.5 List of agreements from RAN1 #105-e:

Agreement

In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be supported with multiple solutions: Alt-1 for full flexibility and Alt-2e for support of legacy delay

Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

  - Without using more than 6 bits

  - FFS: How to minimize the overhead by using joint encoding

Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

  - The HARQ-ACK delay values and the length of the HARQ-ACK delay set will be based on

    - Alt-2e: “3 bits (same as legacy)”

    - FFS: Whether HARQ delay set is to use range1 or range2

RRC signaling will be used to configure between Alt-1 and Alt-2e

FFS: Signaling details

FFS: Joint encoding

Working Assumption

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field:

- The field uses no more than 7 bits if Alt-1 is configured.

- The field is 5 bits if Alt-2e is configured.

- FFS: Details of the joint encoding.

- FFS: Legacy DCI fields that might be re-purposed for the jointly encoded solution of Alt-1 and Alt-2e respectively.

Note: Alt-1 expresses the HARQ-ACK delay as: (y) BL/CE DL subframe + 1 subframe + (z) BL/CE UL subframes, where y = {0, 1, 2, … 11} and z = {1, 2, 3}.

Conclusion:

In Rel-17, for the 14 HARQ processes feature:

When the HARQ-ACK delay is configured to use Alt-1 “PUCCH using Repetition = 1 is postponed”, whereas when the HARQ-ACK delay is configured to use Alt-2e “PUCCH using Repetition = 1 is not postponed (legacy behavior)”.

Agreement

In Rel-17, the 14 HARQ processes feature is applicable for HD-FDD Cat M1 UEs in CE Mode A only.

For discussion in future meetings:

Whether 14 HARQ processes feature can be enabled for PDSCH repetition case

## A1.6 List of agreements from RAN1 #106-e:

Agreement

Confirm the below Working Assumption for Alt-2e with following updates

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field:

- The field is 5 bits if Alt-2e is configured.

- FFS: Details of the joint encoding.

- FFS: Legacy DCI fields that might be set to zero bits in length for the jointly encoded solution Alt-2e.

For Alt-1, it will be separate discussion based existing working assumption

Agreement

Confirm the below Working Assumption for Alt-1 with following updates

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field:

- The field is no more than 7 bits if Alt-1 is configured.

- FFS: Details of the joint encoding.

- FFS: Legacy DCI fields that might be set to zero bits in length for the jointly encoded solution Alt-1.

Note: Alt-1 expresses the HARQ-ACK delay as: (y) BL/CE DL subframe + 1 subframe + (z) BL/CE UL subframes, where y = {0, 1, 2, … 11} and z = {1, 2, 3}.

Agreement

For the PDSCH scheduling delay and HARQ-ACK delay jointly encoded in a single DCI field:

- The DCI field uses 7 bits if Alt-1 is configured.

Conclusion

How to implement/describe the states, e.g., table, resulting from the joint encoding solution of Alt-1 is left up to the Editor, based on the agreements for the PDSCH scheduling delay, HARQ-ACK delay and the WA confirmed for Alt-1.