3GPP TSG RAN WG1 #107-e		R1-2111360

e-Meeting, November 11th – 19th, 2021

Title: 	Evaluation results for XR

Source: 	CEWiT

Agenda item:	8.14.1

Document for:	Discussion and decision

# Introduction

In the previous RAN1 meetings, agreements were made on the XR traffic model, evaluation methodologies and simulation assumptions [1]- [7]. Based on these agreements, we perform system level simulations for single stream DL video stream for XR.

In this contribution, we present our evaluation results for XR capacity for Dense Urban and Urban Macro scenario. For these scenarios we evaluate DL system capacity for AR/VR and CG application. We also perform comparisons for different parameters such as SU/MU-MIMO, PDB, data rate etc., and analyse their impact of system capacity.

This contribution is an extension of our previous contribution, R1-2108869[8].

# Simulation Assumptions, Traffic Model and KPI

In this section we summarize the simulation assumptions, traffic mode and KPI for XR capacity evaluation.

## Simulation Assumptions

In the RAN1#106-e meeting[2], agreements were made to capture the performance evaluation results for capacity. Based on these agreement, we perform evaluation of the baseline XR capacity for Downlink (DL). The baseline evaluation parameters for single stream DL video were agreed in RAN1#105-e[3].  For our simulations we consider DL AR/VR and CG video stream with data rate of 30Mbps and 45Mbps at 60 fps. The agreed PDB value for AR/VR is 10ms and for CG the PDB value is 15ms. The baseline traffic models are summarised in Table 1.

Table 1 Summary of baseline single stream model

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | {30 , 45} | 60 | 10 |
|  | CG | 30 | 60 | 15 |

## Traffic Model

The various parameters for the packet generation were agreed in RAN1#104bis-e meeting[4]. The packet size follows a Truncated Gaussian distribution. The mean packet size is derived based on the date rate and the fps values.  Other required packet size parameters can be calculated using the mean value. For our evaluation purpose, we use consider the [STD, Max, Min] to be [10.5, 150, 50] % of the mean packet size.

The arrival time of the packets is quasi-periodic due the presence of jitter.  For jitter we consider the baseline option with mean 0ms, STD 2ms and range [-4, 4]ms. The detailed traffic model parameters are given in Table 2.

Table 2 Single Stream DL XR Traffic model parameters

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Data Rate | 30Mbps | 45Mbps |  |
| Packet size distribution | Truncated Gaussian |  |  |
| Packet size(mean) | 62500 Bytes | 93750 Bytes |  |
| Packet size(std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) |  |
| Packet Size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) |  |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) |  |
| Jitter | J is drawn from a truncated Gaussian distribution baseline: Mean: 0 ms; STD: 2 ms; Range: [-4, 4] ms |  |

## KPI for XR Capacity Evaluation

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

According to RAN1#103-e meeting[6] the system capacity is defined as,

For our evaluation we consider the value of X = 90, that is the system capacity is maximum number of users with at least 90 % of users being satisfied.

For calculating the percentage of satisfied users, we use the RAN1 agreement made in RAN1#104bis-e meeting [4] that states that a user is declared satisfied if more that 99 % of packets are successfully derived with the given air interface PDB of 10ms for AR/VR and 15ms for CG applications.  Here the 99% is the baseline value that was agreed in the meeting.

## Other Simulation Assumptions

While performing the system level simulations for XR capacity evaluation for FR1, we assume that users are distributed unevenly across the cells. In our simulations we consider the traffic arrival offset to be zero.

The detailed simulation assumptions are given in the Appendix.

# Evaluation Results

In this section we present our evaluation results for XR capacity. We perform system level simulations for FR1 DU and UMa scenarios for DL traffic. Impact of various parameters is also evaluated for FR1 DL DU. Capacity baseline performance is presented in section 3.1 and the impact of various parameters is presented in Section 3.2.

## Capacity Baseline Performance

The traffic model for baseline capacity is summarized on Table 1 and Table 2. The system level parameters used for simulations is given in Appendix.

### Results for Dense Urban

For DU, we consider AR/VR traffic with data rate 30Mbps and 45 Mbps and for CG we consider video traffic with data rate 30 Mbps.  For 30 Mbps AR/VR and CG traffic model, we performed simulations using MU-MIMO scheduler as well, while only SU-MIMO was used for 45 Mbps AR/VR traffic model.

Figure 1 shows the percentage of satisfied users per cell for Dense Urban scenario for AR/VR and CG applications.

Figure 1 Average users per cell vs Percentage of satisfied users for FR1 Dense Urban scenario.

Observation 1: For FR1 Dense Urban scenario, the DL system capacity for AR/VR and CG single stream traffic is summarized in Table 3.

Table 3 Capacity Results for DR1 DU.

| App | PDB | Bit Rate | MIMO | Capacity | C1=floor(Capacity) | % of UEs satisfied when #UE/cell=C1 |
| --- | --- | --- | --- | --- | --- | --- |
| AR/VR | 10ms | 30Mbps | SU-MIMO | 4.05 | 4 | 90% |
| AR/VR | 10ms | 30Mbps | MU-MIMO | 5.78 | 5 | 94% |
| AR/VR | 10ms | 45Mbps | SU-MIMO | 2.04 | 2 | 90% |
| CG | 15ms | 30Mbps | SU-MIMO | 5.57 | 5 | 94% |
| CG | 15ms | 30Mbps | MU-MIMO | >8 | 8 | 91% |

### Results for Urban Macro

For UMa, we consider AR/VR traffic with data rate 30Mbps and 45 Mbps and for CG we consider video traffic with data rate 30 Mbps. Only SU-MIMO was considered for UMa scenario.

Figure 2 shows the percentage of satisfied users per cell for UMa scenario for AR/VR and CG applications.

Figure 2 Average users per cell vs Percentage of satisfied users for FR1 Urban Macro scenario.

Observation 2: For FR1 Urban Macro scenario, the DL system capacity for AR/VR and CG single stream traffic is summarized in Table 4.

Table 4 Capacity Results for DR1 UMa.

| App | PDB | Bit Rate | MIMO | Capacity | C1=floor(Capacity) | % of UEs satisfied when #UE/cell=C1 |
| --- | --- | --- | --- | --- | --- | --- |
| AR/VR | 10ms | 30Mbps | SU-MIMO | 2.98 | 2 | 98% |
| AR/VR | 10ms | 45Mbps | SU-MIMO | 1.85 | 1 | 100% |
| CG | 15ms | 30Mbps | SU-MIMO | 4.08 | 4 | 90% |

## Impact of Various Parameters on XR Capacity

In this section we analyze the impact of various system parameters on XR capacity. Specifically, we consider the impact of data rate, PDB and MU-MIMO on XR system capacity. For all the simulations in this section, we consider FR1 DU scenario with a data rate of 30Mbps and 10ms PDB to be the baseline case.

### Impact of Data Rate

Figure 3 shows the percentage of UE’s satisfied with XR video traffic with data rates 30Mbps and 45 Mbps.

Figure 3 Impact of data rate on capacity.

It can be observed that when data rate is increased from 30 to 45 Mbps, the capacity performance decreases from 4.05 to 2.04 UE per cell, i.e., by about 49%.

Observation 3: XR system capacity decreases as data rate increases.

### Impact of PDB

Figure 5 shows the percentage of UE’s satisfied with 30Mbps XR video traffic with PDB values of 10ms and 15ms.

Figure 4 Impact of PDB on capacity.

It can be observed that when PDB is increased from 10ms to 15ms, the system capacity is increased from 4.05 to 5.57 UE per cell, i.e., by about 37.5%.

Observation 4: XR system capacity increases as PDB value increases.

### Impact of MU-MIMO

Figure 3 shows the percentage of UE’s satisfied with 30Mbps XR video traffic with SU-MIMO and MU-MIMO scheduling.

Figure 5 Impact of MU-MIMO on capacity.

It can be observed that by using MU-MIMO, the XR system capacity can be increased from 4.05 to 5.78, i.e., by about 42.7%.

Observation 5: XR system capacity increase with the use of MU-MIMO.

# Conclusions

In this contribution we presented our evaluation results for XR. Based on our system level simulation we have the following observations.

Observation 1: For FR1 Dense Urban scenario, the DL system capacity for AR/VR and CG single stream traffic is summarized in Table 3.

Table 3 Capacity Results for DR1 DU.

| App | PDB | Bit Rate | MIMO | Capacity | C1=floor(Capacity) | % of UEs satisfied when #UE/cell=C1 |
| --- | --- | --- | --- | --- | --- | --- |
| AR/VR | 10ms | 30Mbps | SU-MIMO | 4.05 | 4 | 90% |
| AR/VR | 10ms | 30Mbps | MU-MIMO | 5.78 | 5 | 94% |
| AR/VR | 10ms | 45Mbps | SU-MIMO | 2.04 | 2 | 90% |
| CG | 15ms | 30Mbps | SU-MIMO | 5.57 | 5 | 94% |
| CG | 15ms | 30Mbps | MU-MIMO | >8 | 8 | 91% |

Observation 2: For FR1 Urban Macro scenario, the DL system capacity for AR/VR and CG single stream traffic is summarized in Table 4.

Table 4 Capacity Results for DR1 UMa.

| App | PDB | Bit Rate | MIMO | Capacity | C1=floor(Capacity) | % of UEs satisfied when #UE/cell=C1 |
| --- | --- | --- | --- | --- | --- | --- |
| AR/VR | 10ms | 30Mbps | SU-MIMO | 2.98 | 2 | 98% |
| AR/VR | 10ms | 45Mbps | SU-MIMO | 1.85 | 1 | 100% |
| CG | 15ms | 30Mbps | SU-MIMO | 4.08 | 4 | 90% |

Observation 3: XR system capacity decreases as data rate increases.

Observation 4: XR system capacity increases as PDB value increases.

Observation 5: XR system capacity increase with the use of MU-MIMO.

# References

RP-201145, Revised SID on XR Evaluations for NR, RAN meeting #88e, June 29 – July 3, 2020

Chairman notes, RAN1#106bis-e, Oct 2021

Chairman notes, RAN1#106-e, Aug 2021.

Chairman notes, RAN1#105-e, May 2021.

Chairman notes, RAN1#104b-e, April 2021.

Chairman notes, RAN1#104-e, February 2021.

Chairman notes, RAN1#103-e, November 2020.

R1-2108869, Initial Evaluation Results For XR, CEWiT, RAN1#106bis-e, Oct 2021

# Appendix A: System Simulation Parameters

| Parameter | Value |
| --- | --- |
| Deployment | Dense urban with single layer of Marco layer refers to TR 38.913 Urban Macro refers to TR 38.913 |
| Channel model | For Dense urban: - Uma refers to TR 38.901 For Urban Macro: - Uma refers to TR 38.901 |
| Layout | For Dense urban: - 21 cells with wraparound, ISD = 200m For Urban Macro: - 21 cells with wraparound, ISD = 500m |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| System bandwidth | Baseline: 100 MHz |
| TDD configuration | Option 1: DDDSU (S: 10D:2F:2U) |
| BS Tx power | 49 dBm per 20 MHz |
| BS antenna parameters | For Dense Urban/Urban Macro scenario: - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) - (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna parameters | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| BS height | 25 m |
| UE height | For Dense Urban/Urban Macro scenario: - Outdoor UEs: 1.5 m - Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | Omni-directional, 0 dBi |
| Noise figure | BS: 5 dB, UE: 9dB |
| Downtilt | For Dense urban: - 12 degree For Urban Macro: - 6 degree |
| UE distribution | For Dense Urban/Urban Macro scenario: - 80% indoor, 20% outdoor |
| UE speed | 3 km/h |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| MCS | Up to 256QAM |
| Transmission scheme | Closed Loop MIMO |
| Scheduler | SU-MIMO/MU-MIMO PF |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 |
| gNB processing delay | 3 Slots |
| PDCCH overhead | 1/7 |
| Target BLER | 10% |
| Max HARQ transmission | 3 |