3GPP TSG RAN WG1 #107-e			                        R1-2111655

e-Meeting, November 11th – 19th, 2021

Source:	CAICT

Title:	Discussion on enhancements of INACTIVE mode positioning and on-demand PRS

Agenda Item:	8.5.6

Document for:	Discussion and Decision

- Introduction

In RAN1#106bis e-meeting, NR Positioning Enhancements for INACTIVE mode positioning and on demand PRS are discussed, and the following agreements are made:

- The following list of parameters is supported for UE-initiated and LMF initiated on-demand DL PRS request

  - Start/end time of DL PRS transmission

  - DL PRS resource repetition factor

  - Number of DL PRS resource symbols per DL PRS resource

  - DL-PRS CombSize N

  - Number of DL PRS frequency layers

  - ON/OFF indicator (for LMF initiated request only)

- FFS values for requested on-demand DL PRS parameters and whether parameters are resource-specific, TRP-specific, or PFL-specific

In this contribution, we share our views on other enhancements of  INACTIVE mode positioning and on-demand PRS.

- Discussions

  - NR Positioning in RRC_INACTIVE State

Without supporting RRC_IDLE/ RRC_INACTIVE mode positioning in Rel-16, UEs in RRC_IDLE/ RRC_INACTIVE mode have to first enter into RRC connected mode, which leads to increase lots of signaling overhead, additional latency and high power consumption.

For DL positioning, the DL PRS configuration for measurement can be obtained from the broadcast system information in Rel-16 specification, the UE based DL positioning can be supported. While to support the UE assisted DL positioning, the DL positioning measurement has to be reported to the LMF, to support UEs in RRC_INACTIVE state to transport the DL positioning measurement, the measurement report transmission can be specified in RAN2 UL small data transmission.

For UL positioning, the UL positioning reference signal configuration need to be informed to UEs, and the UL positioning reference signal can be SRS-pos and also can be RACH preamble, For UEs in RRC_CONNECTED state, network needs to provide the configuration information of UL positioning reference signal. While for UEs in RRC_INACTIVE state, the configuration information can be informed to the UEs by a new paging message which transmitting the UL positioning reference signal configuration, or a new random access process to help UEs to obtain the configuration information of UL positioning reference signal configuration.

Proposal 1: For UL positioning in RRC_IDLE state, to trigger the UL positioning signal transmission, a new paging message or a new random access process need to be specified.

Proposal 2: The UL positioning reference signal can be SRS-pos signal or RACH preambles.

  - On-demand PRS enhancement

For on demand PRS enhancement, the multiple sets of configuration of DL PRS can be supported considering the different positioning QoS requirements. Besides the agreed list of parameters including

- Start/end time of DL PRS transmission, DL PRS resource repetition factor, Number of DL PRS resource symbols per DL PRS resource,DL-PRS CombSizeN, Number of DL PRS frequency layers and ON/OFF indicator (for LMF initiated request only), the configuration of DL PRS can also include positioning base station information (e.g. the number of TRPs and beam directions).

2.2.1. System information broadcast

The multiple sets of DL PRS configuration can be broadcasted in the system information, and for the UE initiated on demand DL PRS, the UEs could request the required DL PRS configuration considering on the positioning QoS requirements. the UEs could request the concrete set of DL PRS configuration through message 1 (MSG1) or message 3 (MSG3) in the random access process, the UE sends MSG1 according to the SIB1 system configuration to request DL PRS configuration or sends MSG3 based on a special preamble sequence to request the required DL PRS configuration. When the request is based on MSG1, the gNB receives MSG1, then sends MSG2 transmitting the random access response confirmation information to UEs, and then broadcast the UE requested DL PRS configuration. When the request is based on MSG3, the gNB sends MSG4 transmitting confirmation to the UEs, and then broadcast the UE requested DL PRS configuration. On the other hand, When no request of one or more sets of DL PRS configuration are transmitted by any UEs, the one or more sets of DL PRS configuration can be configured as OFF state, and are not transmitted to UEs by gNBs.

2.2.2. RRC signaling request

The multiple sets of DL PRS configuration can also be requested by RRC configuration, the procedure is shown in  Figure 1.

Step 1, the UE sends the DL PRS request to the LMF.

Step 2, the LMF sends the request of UE to the serving gNB and neighboring gNB.

Step 3, the serving gNB and the neighboring gNB send response to the LMF.

Step 4: the location server sends a response to the UE.

Step 5: the serving gNB and neighboring gNB send the on-demand DL PRS to the UE.

Figure 1. the procedure of RRC signaling to request DL PRS

Proposal 3: Lists of parameters for UE / LMF initiated on-demand DL PRS request can also include participating positioning base station information (e.g. number of TRPs and beam directions).

Proposal 4: For supporting on-demand PRS, requesting mechanism can be considered to use uplink RACH channel as a candidate.

Proposal 5: For on demand PRS, the potential signaling can be system information broadcasting or RRC signaling.

- Conclusion

In this contribution, we give following proposals of enhancements for INACTIVE mode positioning and on-demand PRS:

Proposal 1: For UL positioning in RRC_IDLE state, to trigger the UL positioning signal transmission, a new paging message or a new random access process need to be specified.

Proposal 2: The UL positioning reference signal can be SRS-pos signal or RACH preambles.

Proposal 3: Lists of parameters for UE / LMF initiated on-demand DL PRS request can also include participating positioning base station information (e.g. number of TRPs and beam directions).

Proposal 4: For supporting on-demand PRS, requesting mechanism can be considered to use uplink RACH channel as a candidate.

Proposal 5: For on demand PRS, the potential signaling can be system information broadcasting or RRC signaling.

References

- 3GPP RAN1 #106bis-e meeting chairman’s notes