3GPP TSG RAN WG1 #107-e		   	R1-2111822

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.4.3

Source:	InterDigital, Inc.

Title:	Remaining issues on HARQ enhancement for NTN

Document for:	Discussion and Decision

# Introduction

In RAN1 #106bis-e, the following agreement has been made as a progress for the HARQ enhancement for NTN [1]:

Agreement:

For DCI indicating SPS PDSCH release, HARQ-ACK report is as in Rel-16.

Conclusion:

For DCI 0-0/1-0, no enhancement to support indication of more than 16 HARQ processes is considered in Rel-17.

In this contribution, we discuss on the remaining issues on HARQ enhancement for NTN.

# Discussion

## Performance enhancement for disabled HARQ feedback

Aggregated Transmission

In RAN1 #105-e, it has been agreed to focus on the aggregated transmission as a performance enhancement for the case when HARQ feedback is disabled for a HARQ process. The slot aggregation has been supported since Rel-15 and the maximum number of aggregated slots is 8. To enhance performance of PDSCH when HARQ feedback is disabled, the maximum number of aggregated slots can be increased to a higher number. Two possible values are on the table to increase the number of aggregated slots such as 16 and 32. From our perspective, it is preferred to support a higher number of aggregated slot as possible as a maximum (i.e., 32)

Proposal-1: the maximum number of aggregated slots is increased to 32

The slot aggregation number is configured as a part of PDSCH-config for a BWP which is a semi-static configuration. However, it would be beneficial if gNB can adapt the slot aggregation number based on the status of retransmission at RLC as an example which is also helpful to optimize the resource utilization.

Therefore, indicating slot aggregation number for PDSCH dynamically in DCI could improve the performance as similar to dynamic repetition number indication used in LTE eMTC/NB-IoT.

Proposal-2: slot aggregation number for a PDSCH is dynamically indicated in DCI

Although a larger slot aggregation number is introduced to enhance the PDSCH performance when HARQ feedback is disabled, it doesn’t need to be restricted to be used when HARQ feedback is disabled. For example, even for HARQ feedback is enabled, a larger slot aggregation number could provide benefit in NR NTN scenario when some of UEs are coverage limited due to other issue.

Proposal-3: a larger slot aggregation number for a PDSCH can be also used when HARQ feedback is enabled

CQI table enhancement

During SI, RAN2 has agreed that HARQ feedback can be enabled/disabled per UE per HARQ process number. In the previous RAN1 meeting, RAN1 also made an agreement that the HARQ feedback enabling/disabling per UE per HARQ process number is configured via UE-specific RRC signaling.

The HARQ feedback disabling for a HARQ process could address the issue of the UE throughput performance loss due to long round trip delay with limited HARQ process numbers, and the rely on the higher layer ARQ.

The L1 retransmission with HARQ feedback allows the scheduler to use an aggressive MCS selection for initial transmission as the retransmission could handle the failed TB reception, which in general improves the UE throughput. Therefore, in general, the PDSCH target BLER is assumed 10% (e.g., for eMBB) and its associated CQI is also derived based on the PDSCH target BLER.

When the HARQ feedback is disabled, it would be beneficial to lower the target BLER for PDSCH as no retransmission is available so that the number of retransmissions in upper layer can be reduced. Note that the retransmission in higher layer would increase the latency and resource waste significantly especially when frequent error occurs for TB transmission.

Observation-1: lowering target BLER for PDSCH when HARQ feedback is disabled is beneficial in terms of resource utilization and latency as it can reduce the number of retransmissions in higher layer

The HARQ feedback is also used for outer-loop link adaptation as the CQI feedback is typically not accurate since interference is changed dynamically, therefore the outer-loop link adaptation based on ACK/NACK has been used. However, if HARQ feedback is disabled and CQI feedback is based on the target BLER 10%, it is hard to determine MCS to meet a lower target BLER (e.g., 1%). Therefore, it would be beneficial to use a CQI table with a lower BLER target (but not as low as URLLC CQI table) when HARQ feedback is disabled.

Observation-2: use of a CQI table with a lower BLER target (e.g., 1%) could provide a better link adaptation with lower PDSCH BLER target when HARQ feedback is disabled

Therefore, a same CQI table with a different target BLER may be considered based on the HARQ feedback enabling/disabling status.

Proposal-4: a CQI table with a new target BLER (e.g., 1%) is considered when HARQ feedback is disabled

## HARQ feedback disabling restriction for MAC-CE command

The MAC-CE action time has been defined based on the timing of HARQ-ACK feedback of the corresponding PDSCH carrying MAC-CE command. Therefore, if HARQ-ACK feedback is disabled, it becomes unclear when the MAC-CE command is applied at the UE side since there is no associated HARQ-ACK feedback, resulting in ambiguity period as similar to the RRC although the period could be much shorter than RRC ambiguity period. Moreover, the reliability of the MAC-CE command is also degraded since gNB has no information whether the UE received the MAC-CE command correctly or not. As such, it has been discussed whether the PDSCH carrying MAC-CE command is transmitted only via a HARQ process with HARQ-ACK feedback enabled or leave it for the gNB implementation.

The MAC-CE command is used for (re)configuration of TCI states for beam management which is sensitive to the time latency. Therefore, it is important that UE and gNB are synchronized when the MAC-CE command is applied at the UE, otherwise significant performance loss may be observed during the ambiguity period. If the MAC-CE command is missed at the UE and gNB doesn’t know the case, the performance loss could be even worse.

Based on this observation, restricting to use MAC-CE command for the HARQ process with HARQ feedback enabled seems to be the best option without compromising reliability and performance.

Proposal-5: HARQ feedback is enabled for at least one HARQ process number

Proposal-6: the PDSCH carrying MAC-CE command is transmitted in the HARQ process with HARQ feedback enabled

Although at least one HARQ process is enabled always for MAC-CE command, it would be a separate discussion whether the restriction has to be specified or it is up to gNB implementation. Since it is also related to UE implementation, it is better to clearly capture in the specification whether a UE expects to receive a MAC-CE command in a PDSCH associated with a HARQ process number of which HARQ feedback is disabled or not.

Proposal-7: a UE is not expected to receive a MAC-CE command in a PDSCH associated with a HARQ process number of which HARQ feedback is disabled

# Summary

In this contribution, we discussed on remaining issues on the HARQ feedback enhancement for NTN. Based on the discussion, we have following observations and proposals:

Observation-1: lowering target BLER for PDSCH when HARQ feedback is disabled is beneficial in terms of resource utilization and latency as it can reduce the number of retransmissions in higher layer

Observation-2: use of a CQI table with a lower BLER target (e.g., 1%) could provide a better link adaptation with lower PDSCH BLER target when HARQ feedback is disabled

Proposal-1: the maximum number of aggregated slots is increased to 32

Proposal-2: slot aggregation number for a PDSCH is dynamically indicated in DCI

Proposal-3: a larger slot aggregation number for a PDSCH can be also used when HARQ feedback is enabled.

Proposal-4: a CQI table with a new target BLER (e.g., 1%) is considered when HARQ feedback is disabled

Proposal-5: HARQ feedback is enabled for at least one HARQ process number

Proposal-6: the PDSCH carrying MAC-CE command is transmitted in the HARQ process with HARQ feedback enabled

Proposal-7: a UE is not expected to receive a MAC-CE command in a PDSCH associated with a HARQ process number of which HARQ feedback is disabled

# References

- RAN1 #106bis-e Chairman’s note